3GPP TSG-RAN5 Meeting #95-e	R5-223087r1

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2983 | rev | - | Current version: | 16.11.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to SOR test case 6.3.1.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | PDU session establishment is not handled in the test procedure. In the preamble also, it is not handled as UE is powered on in the test procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new steps 16Aa1 and 17Aa1 to handle the PDU session establishment |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st Revision: Step 17Aa1 updated to refer to the whole test procedure from TS 38.508-1, clause 4.5.2.2-4 to cater for the case when pc_noOf_PDUsSameConnection = 0 and pc_noOf_PDUsNewConnection>0. Added Huawei, HiSilicon as co-source. |  |  |  |  |  |  |  |  |

#### 6.3.1.10	Steering of UE in roaming during mobility update registration

6.3.1.10.1	Test Purpose (TP)

(1)

with {UE being in automatic PLMN selection mode , current VPLMN not part of “PLMNs where registration was aborted due to SOR” List and not part of "User Controlled PLMN Selector with Access Technology" list and UE's USIM configured with indication that the UE is to receive Steering of Roaming information due to initial registration in a VPLMN}

ensure that {

when {UE performs mobility update Registration Procedure to cell belonging to VPLMN and chosen VPLMN does not provide Steering of Roaming info}

then {UE continues to camp on the chosen VPLMN}

}

6.3.1.10.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122, clause C.2. Unless otherwise stated these are Rel-15 requirements.

[TS 23.122, clause C.2]

2)	Upon receiving REGISTRATION REQUEST message, the VPLMN AMF executes the registration procedure as defined in subclause 4.2.2.2.2 of 3GPP TS 23.502 [63]. As part of the registration procedure:

a)	if the VPLMN AMF does not have subscription data for the UE, the VPLMN AMF invokes Nudm_SDM_Get service operation to the HPLMN UDM to get amongst other information the Access and Mobility Subscription data for the UE (see step 14b in subclause 4.2.2.2.2 of 3GPP TS 23.502 [63]); or

b)	if the VPLMN AMF already has subscription data for the UE and:

i)	the 5GS registration type IE in the received REGISTRATION REQUEST message indicates "initial registration" and the "SoR Update Indicator for Initial Registration" field in the UE context is set to 'the UDM requests the AMF to retrieve SoR information when the UE performs NAS registration type "initial registration"' as specified in table 5.2.2.2.2-1 of 3GPP TS 23.502 [63]); or

ii)	the 5GS registration type IE in the received REGISTRATION REQUEST message indicates "emergency registration" and the "SoR Update Indicator for Emergency Registration" field in the UE context is set to 'the UDM requests the AMF to retrieve SoR information when the UE performs NAS registration type "emergency registration"' as specified in table 5.2.2.2.2-1 of 3GPP TS 23.502 [63]);

then the VPLMN AMF invokes Nudm_SDM_Get service operation message to the HPLMN UDM to retrieve the steering of roaming information (see step 14b in subclause 4.2.2.2.2 of 3GPP TS 23.502 [63]);

otherwise the VPLMN AMF sends a REGISTRATION ACCEPT message without the steering of roaming information to the UE and steps 3a, 3b, 3c, 3d, 4, 5, 6 are skipped;

…

11)	If the UE has a list of available PLMNs in the area and based on this list the UE determines that there is a higher priority PLMN than the selected VPLMN and the UE is in automatic network selection mode, then the UE shall attempt to obtain service on a higher priority PLMN as specified in subclause 4.4.3.3 by acting as if timer T that controls periodic attempts has expired after the release of the N1 NAS signalling connection. If the N1 NAS signalling connection is not released after implementation dependent time, the UE may locally release the N1 signalling connection except when the UE has an established emergency PDU session (see 3GPP TS 24.501 [64]).

6.3.1.10.3	Test Description

6.3.1.10.3.1	Pre-test conditions

System Simulator:

-	Three inter-frequency multi-PLMN NR Cells as specified in TS 38.508-1 [4] Table 4.4.2-1 are configured broadcasting PLMNs as indicated in Table 6.3.1.10.3.1-1.

-	The PLMNs are identified in the test by the identifiers in Table 6.3.1.10.3.1-1. The MCC and MNC values of PLMN identifiers are specified in TS 36.523-1 [13], Table 6.0.1-1.

Table 6.3.1.10.3.1-1: PLMN identifiers

| NR Cell | PLMN names |
| --- | --- |
| NR Cell 1 | PLMN14 |
| NR Cell 12 | PLMN13 |
| NR Cell 11 | PLMN14 |

-	NR Cell 1 is set to ''Serving Cell'';

-	NR Cell 12 is set to ''Not-Suitable Cell'';

-	NR Cell 11 is set to ''Not-Suitable Cell'';

-	TAI of NR Cell 11 is set to ''TAI-2'' and TAI of NR Cell 1 is set to ''TAI-1';'

-	System Information Combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cells.

UE:

-	The UE is in Automatic PLMN selection mode.

-	USIM configuration as defined in Table 6.4.1-10 of TS 38.508-1 [4] will be used.

Preamble:

-	Ensure that the UE has cleared the RPLMN. And the UE is in Switched OFF (State 0-A) as defined in TS 38.508-1 [4] Table 4.4A.2-0.

6.3.1.10.4	Test procedure sequence

Table 6.3.1.10.4-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power on the UE. | - |  | - | - |
| 2-14 | Steps 1 to 13 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2-2 are performed on NR Cell 1. | - | - | - | - |
| 15 | The SS transmits an DLInformationTransfer message and a REGISTRATION ACCEPT message containing steering of roaming information indicating that acknowledgement is not requested from the UE for successful reception. | <-- | NR RRC: DLInformationTransfer 5G MM: REGISTRATION ACCEPT | - | - |
| 16 | The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message without SOR transparent container. | --> | NR RRC: ULInformationTransfer 5G MM: REGISTRATION COMPLETE | - | - |
| - | EXCEPTION: Step 16Aa1 is performed if pc_noOf_PDUsSameConnection > 0. | - | - | - | - |
| 16Aa1 | The generic procedure for UE-requested PDU session establishment, specified in subclause 4.5A.2, takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection. | - | - | - | - |
| 17 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| - | EXCEPTION: Step 17Aa1 is performed if pc_noOf_PDUsNewConnection > 0. | - | - | - | - |
| 17Aa1 | The generic procedure for UE-requested PDU session establishment, specified in subclause 4.5.2.2-4, takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsNewConnection and then release the RRC Connection. | - | - | - | - |
| 18 | The SS configures: - NR Cell 11 as "Serving cell" - NR Cell 1 as "Non-Suitable cell" - NR Cell 12 as "Non-Suitable cell". | - | - | - | - |
| 19 | The UE transmits an RRCSetupRequest message on NR Cell 11. | --> | NR RRC: RRCSetupRequest | - | - |
| 20 | SS transmits an RRCSetup message on NR Cell 11. | <-- | NR RRC: RRCSetup | - | - |
| 21 | The UE transmits an RRCSetupComplete message to confirm the successful completion of the connection establishment and a REGISTRATION REQUEST message indicating "mobility registration updating" on NR Cell 11. | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | - | - |
| 22 | SS sends a REGISTRATION ACCEPT message without including SOR transparent container on NR Cell 11. | <-- | NR RRC: DLInformationTransfer 5GMM: REGISTRATION ACCEPT | - | - |
| 23 | Check: Does the UE send a REGISTRATION COMPLETE without including SOR transparent container on NR Cell 11? | --> | NR RRC: ULInformationTransfer 5GMM: REGISTRATION COMPLETE | 1 | P |
| 24 | The SS transmits an RRCRelease message on NR Cell 11. | <-- | NR RRC: RRCRelease | - | - |
| 25 | The SS configures: - NR Cell 11 as "Serving cell" - NR Cell 1 as "Non-Suitable cell" - NR Cell 12 as "Serving cell". | - | - | - | - |
| 26 | Check: Does the UE transmit an RRCSetupRequest message on NR Cell 12 within timer T=6 minutes *(1+10% tolerance)? NOTE: The timer T shall be set to 396s (6 minutes *(1+10% tolerance)), where the high priority PLMN search timer is defined in EFHPPLMN. | - | - | 1 | F |
| 27 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is camped on NR Cell 11? | - | - | 1 | - |

6.3.1.10.5	Specific message contents

Table 6.3.1.10.5-1: REGISTRATION ACCEPT for NR Cell 1 (step 15, Table 6.3.1.10.4-1)

| Derivation Path: 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SOR Transparent Container | Present | The SOR transparent container carries steering of roaming information. |  |
| SoR-MAC-IAUS | Set to match the calculated SoR-MAC-IAUS as the way defined in TS 33.501 A.17 |  |  |
| CounterSOR | Value generated as per TS 33.501 Cl 6.14.2.3 |  |  |
| SOR data type | 0 | The SOR transparent container carries steering of roaming information. |  |
| List indication value | 1 | List of preferred PLMN/access technology combinations is provided |  |
| List type | 1 | The list type is a PLMN ID and access technology list |  |
| Acknowledgement (ACK) value | 0 | Acknowledgement is not requested |  |
| PLMN ID 1 | PLMN 14 |  |  |
| Access Technology Identifier 1 | NG-RAN |  |  |
| PLMN ID 2 | PLMN 13 |  |  |
| Access Technology Identifier 2 | NG-RAN |  |  |

Table 6.3.1.10.5-2: REGISTRATION COMPLETE for NR Cell 1 (step 16, Table 6.3.1.10.4-1)

| Derivation Path: 38.508-1 Table 4.7.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SOR Transparent Container | Not Present |  |  |

Table 6.3.1.10.5-3: REGISTRATION ACCEPT for NR Cell 11 (step 22, Table 6.3.1.10.4-1)

| Derivation Path: 38.508-1 Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SOR Transparent Container | Not Present |  |  |

Table 6.3.1.10.5-4: REGISTRATION COMPLETE for NR Cell 11 (step 23, Table 6.3.1.10.4-1)

| Derivation Path: 38.508-1 Table 4.7.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SOR Transparent Container | Not Present |  |  |