Title:	Discussion on test point selection for NS_48 PC3 A-MPR and ASE in FR1

Document for:	Discussion and approval

Background

The purpose of this paper is to provide an analysis of the parameters for the A-MPR test configuration table with a selection of test points that minimize testing and still achieve good test coverage for NS_48. The intention with NS_48 is to protect E-UTRA band 34/NR band n34 and frequency range 1900-1920 MHz as indicated in Table 1 below.

Table 1: Additional requirements for “NS_48” (from [4])

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA band 34 – NR band n34 | FDL_low | - | FDL_high | -50 | 1 |  |
| Frequency range | 1900 | - | 1915 | -15.5 | 5 | 1 |
| Frequency range | 1915 | - | 1920 | +1.6 | 5 | 1 |
| NOTE 1: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |

Discussion

The general agreement in RAN5 is that FR1 testing should follow what is being tested for LTE. This is also valid for test point selection for A-MPR in FR1.

The selection of test points below is partly based on approved test point analyzes for SEM [2, 3] for FR1. For NS_48 the operating band n1 (UL 1920-1980 MHz) has specific A-MPR requirements, see Table 2.

Table 2: A-MPR regions for NS_48 (from [4])

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 MHz | 1932.5≤ FC ≤ 1967.5 | ≥0 | ≥9.72 | A3 |
|  |  | ≥18.72 | <1.08 | A3 |
| 30 MHz | 1935 ≤ FC ≤ 1965 | ≥0 | ≥13.5 | A3 |
|  |  | ≥21.6 | <1.08 | A5 |
| 40 MHz | 1940 ≤ FC ≤ 1960 | ≥0, <2.88 | ≥0 | A2 |
|  |  | ≥2.88, <17.1 | ≥max (0, 12*SCS*RBend - 3.6) | A3 |
|  |  | ≥17.1, <27.36 | ≥13.5 | A4 |
|  |  | ≥27.36, <34.56 | ≥13.5 | A2 |
|  |  | ≥27.36, <34.56 | <1.08 | A3 |
|  |  | ≥34.56 | ≥0 | A1 |
| 45 MHz | 1942.5 ≤ FC ≤ 1957.5 | ≥0, <4.86 | >0 | A2 |
|  |  | ≥4.86, <19 | ≥max (0, 12*SCS*RBend - 3.6) | A4 |
|  |  | ≥19, <37.44 | ≥15.4 | A2 |
|  |  | ≥30.96, <37.44 | <1.08 | A5 |
|  |  | ≥37.44 | >0 | A1 |
| 50 MHz | 1945 ≤ FC ≤ 1955 | ≥0, <6.12 | >0 | A2 |
|  |  | ≥6.12, <20.7 | ≥max (0, 12*SCS*RBend - 3.6) | A4 |
|  |  | ≥20.7, <41.04 | ≥17.1 | A2 |
|  |  | ≥33.84, <41.04 | <1.08 | A5 |
|  |  | ≥41.04 | >0 | A1 |

The RB allocation for testing of these A-MPR regions depends on both carrier centre frequency, channel bandwidth and SCS.

Test Environment

Based on the baseline agreed in [2], the test environment for NR A-MPR measurement for FR1 can be defined as in LTE.

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for A-MPR NS_48.

Test Frequencies

UL test carrier centre frequencies are defied in Table 2 for the NS_48 regions. Select the lowest and highest carrier centre frequency for each A-MPR requirement. Lowest carrier centre frequency since this frequency is closest to the band to be protected. Lowest carrier centre frequency since this frequency is closest to the frequency range 1900-1920MHz to be protected. Highest carrier centre frequency since this frequency is closest to the band 34/n34 to be protected.

PROPOSAL 2: Select the lowest and highest carrier centre frequency for each A-MPR requirement.

Test Channel Bandwidth

The minimum requirements for NS_48 apply for channel bandwidths 25MHz, 30MHz, 40MHz, 45MHz and 50MHz.

PROPOSAL 3: If supported, select 25MHz, 30MHz, 40MHz, 45MHz and 50MHz channel bandwidth.

  - Test Subcarrier Spacing

The selection of test subcarrier spacing can be based on the agreed proposals for SEM [2], i.e. Lowest and Highest supported subcarrier spacing.

PROPOSAL 4: Select Lowest and Highest supported subcarrier spacing.

  - UL Modulation

The selection of UL modulation can be based on the agreed proposals for SEM [2], i.e. all defined UL waveforms (5 DFT-s OFDM and 4 CP-OFDM waveforms). But in order to reduce the testing workload, down selection of UL modulation can be considered. The highest modulation type need to be tested considering the high PAPR feature. But with the highest modulation, MPR might be higher than A-MPR, and in that case A-MPR is not verified properly, therefore the lowest modulation also needs to be tested. For DFT-s-OFDM QPSK is selected instead of BPSK since BPSK is optionally supported.

PROPOSAL 5: Test QPSK and 256QAM for DFT-s-OFDM and CP-OFDM.

  - Uplink RB allocation

The UL RB allocation can be based on the agreed proposals for SEM [2], i.e. Edge_1RB_left, Edge_1RB_Right and Outer_Full. These common uplink configurations are listed in [5].

A-MPR for NS_48 has some Region specific RB allocations in Table 2, where the ranges for the RB allocation is specified in terms of RBend × 12 × SCS and LCRB × 12 × SCS. Table 3 shows the corresponding RB ranges for 15/30/60 kHz SCS and proposed allocation expressed as LCRB@RBstart.

Table 3: UL RB allocation for NS_48

| Channel Bandwidth, | Carrier Center Frequency, Fc, MHz | SCS | Regions |  | Prop. Alloc (DFT-s) | Prop. Alloc (CP) | A-MPR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MHz |  |  | RBend | LCRB* |  |  |  |
| 25 MHz | 1932.5≤ FC ≤ 1967.5 | 15 | ≥0 | ≥54 | Outer_Full | Outer_Full | A3 |
|  |  | 30 | ≥0 | ≥27 | Outer_Full | Outer_Full | A3 |
|  |  | 60 | ≥0 | ≥13.5 | Outer_Full | Outer_Full | A3 |
|  |  | 15 | ≥104 | <6 | Edge_1RB_Right | Edge_1RB_Right | A3 |
|  |  | 30 | ≥52 | <3 | Edge_1RB_Right | Edge_1RB_Right | A3 |
|  |  | 60 | ≥26 | <1.5 | Edge_1RB_Right | Edge_1RB_Right | A3 |
| 30 MHz | 1935 ≤ FC ≤ 1965 | 15 | ≥0 | ≥75 | Outer_Full | Outer_Full | A3 |
|  |  | 30 | ≥0 | ≥37.5 | Outer_Full | Outer_Full | A3 |
|  |  | 60 | ≥0 | ≥18.75 | Outer_Full | Outer_Full | A3 |
|  |  | 15 | ≥120 | <6 | Edge_1RB_Right | Edge_1RB_Right | A5 |
|  |  | 30 | ≥60 | <3 | Edge_1RB_Right | Edge_1RB_Right | A5 |
|  |  | 60 | ≥30 | <1.5 | Edge_1RB_Right | Edge_1RB_Right | A5 |
| 40 MHz | 1940 ≤ FC ≤ 1960 | 15 | [0,16) | ≥0 | 16@0 | 16@0 | A2 |
|  |  | 30 | [0,8) | ≥0 | 8@0 | 8@0 | A2 |
|  |  | 60 | [0,4) | ≥0 | 4@0 | 4@0 | A2 |
|  |  | 15 | [16,95) | ≥max (0, RBend - 20) | 90@0 | 95@0 | A3 |
|  |  | 30 | [8,47.5) | ≥max (0, RBend - 10) | 48@0 | 48@0 | A3 |
|  |  | 60 | [4,23.75) | ≥max (0, RBend - 5) | 24@0 | 24@0 | A3 |
|  |  | 15 | [95,152) | ≥75 | 150@0 | 152@0 | A4 |
|  |  | 30 | [48,76) | ≥37.5 | 75@0 | 76@0 | A4 |
|  |  | 60 | [24,38) | ≥18.75 | 36@0 | 38@0 | A4 |
|  |  | 15 | [152,192) | ≥75 | 192@0 | 192@0 | A2 |
|  |  | 30 | [76,96) | ≥37.5 | 96@0 | 96@0 | A2 |
|  |  | 60 | [38,48) | ≥18.75 | 48@0 | 48@0 | A2 |
|  |  | 15 | [152,192) | <6 | 5@187 | 5@187 | A3 |
|  |  | 30 | [76,96) | <3 | 2@94 | 2@94 | A3 |
|  |  | 60 | [38,48) | <1.5 | 1@47 | 1@47 | A3 |
|  |  | 15 | ≥0 | ≥192 | Outer_Full | Outer_Full | A1 |
|  |  | 30 | ≥0 | ≥96 | Outer_Full | Outer_Full | A1 |
|  |  | 60 | ≥0 | ≥48 | Outer_Full | Outer_Full | A1 |
| 45 MHz | 1942.5 ≤ FC ≤ 1957.5 |  |  | 15 | [0,27) |  | >0 |  | 27@0 |  | 27@0 |  | A2 |  |
|  |  |  |  | 30 | [0,13.5) |  | >0 |  | 12@0 |  | 13@0 |  | A2 |  |
|  |  |  |  | 60 | [0,6.75) |  | >0 |  | 6@0 |  | 6@0 |  | A2 |  |
|  |  |  |  | 15 | [27, 105.5) |  | ≥max (0, RBend - 20) |  | 100@0 |  | 106@0 |  | A4 |  |
|  |  |  |  | 30 | [13.5, 52.7) |  | ≥max (0, RBend - 10) |  | 50@0 |  | 53@0 |  | A4 |  |
|  |  |  |  | 60 | [6.75, 26.35) |  | ≥max (0, RBend - 5) |  | 27@0 |  | 27@0 |  | A4 |  |
|  |  |  |  | 15 | [105.5, 208) |  | ≥85.5 |  | 200@0 |  | 208@0 |  | A2 |  |
|  |  |  |  | 30 | [52.7, 104) |  | ≥42.7 |  | 100@0 |  | 104@0 |  | A2 |  |
|  |  |  |  | 60 | [26.35, 52) |  | ≥21.4 |  | 50@0 |  | 52@0 |  | A2 |  |
|  |  |  |  | 15 | [172,208) |  | <6 |  | 5@203 |  | 5@203 |  | A5 |  |
|  |  |  |  | 30 | [86,104) |  | <3 |  | 2@102 |  | 2@102 |  | A5 |  |
|  |  |  |  | 60 | [43,52) |  | <1.5 |  | 1@51 |  | 1@51 |  | A5 |  |
|  |  |  |  | 15 | ≥208 |  | >0 |  | Outer_Full |  | Outer_Full |  | A1 |  |
|  |  |  |  | 30 | ≥104 |  | >0 |  | Outer_Full |  | Outer_Full |  | A1 |  |
|  |  |  |  | 60 | ≥52 |  | >0 |  | Outer_Full |  | Outer_Full |  | A1 |  |
| 50 MHz | 1945 ≤ FC ≤ 1955 | 15 | [0,34) | ≥0 | 32@0 | 34@0 | A2 |
|  |  | 30 | [0,17) | ≥0 | 18@0 | 18@0 | A2 |
|  |  | 60 | [0,8.5) | ≥0 | 9@0 | 9@0 | A2 |
|  |  | 15 | [34,115) | ≥max (0, RBend - 20) | 108@0 | 115@0 | A4 |
|  |  | 30 | [17,57.5) | ≥max (0, RBend - 10) | 54@0 | 58@0 | A4 |
|  |  | 60 | [9,28.75) | ≥max (0, RBend - 5) | 27@0 | 29@0 | A4 |
|  |  | 15 | [115,228) | ≥95 | 225@0 | 228@0 | A2 |
|  |  | 30 | [58,114) | ≥47.5 | 108@0 | 114@0 | A2 |
|  |  | 60 | [29,57) | ≥23.75 | 54@0 | 57@0 | A2 |
|  |  | 15 | [188,228) | <6 | 5@223 | 5@223 | A5 |
|  |  | 30 | [94,114) | <3 | 2@112 | 2@112 | A5 |
|  |  | 60 | [47,57) | <1.5 | 1@56 | 1@56 | A5 |
|  |  | 15 | ≥228 | ≥0 | Outer_Full | Outer_Full | A1 |
|  |  | 30 | ≥114 | ≥0 | Outer_Full | Outer_Full | A1 |
|  |  | 60 | ≥57 | ≥0 | Outer_Full | Outer_Full | A1 |

The proposals for LCRB@RBstart in Table 3 are all based on allocating as many RBs as possible located to the lower part of the channel bandwidth, i.e. select lowest RBstart.

PROPOSAL 6: Select as many RBs as possible close to the lower band edge.

2.7.	Number of Test Points

Based on the proposals the maximum number of test points for A-MPR tests for NS_48 (PC3) is:

| Network Signalling label | Environment conditions | Number SCS | Number of ChBW | Maximum Number of Modulations | Maximum Number of Steps (freq. & RB alloc.) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- | --- |
| NS_48 | 1 | 2 (L/H) | 5 (25, 30, 40, 45, 50) | 4 | 6 | 240 |

Proposal

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 NS_48 A-MPR test case definition:

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for A-MPR NS_48.

PROPOSAL 2: Select the lowest and highest carrier centre frequency for each A-MPR requirement.

PROPOSAL 3: If supported, select 25MHz, 30MHz, 40MHz, 45MHz and 50MHz channel bandwidth.

PROPOSAL 4: Select Lowest and Highest supported subcarrier spacing.

PROPOSAL 5: Test QPSK and 256QAM for DFT-s-OFDM and CP-OFDM.

PROPOSAL 6: Select as many RBs as possible close to the lower band edge.

References

- R5-180065, "Discussion about Test parameters for NR TRx TCs " (NTT DOCOMO).

- R5-180886, " Discussion on test point selection for NR SEM in FR1” (Keysight Technologies).

- R5-182019, “Discussion of NR FR1 Test Point for TX Spurious Emission test cases”, (Qualcomm Inc.)

- 3GPP TS 38.101-1, V17.5.0 (2022-03)

- 3GPP TS 38.521-1, V17.4.0 (2022-03), Table 6.1-1