3GPP TSG-RAN5 Meeting #95-e        R5-223199r1
Electronic Meeting, 9th May – 20th May 2022

Title: 	Discussion on mandatory channel bandwidths after Rel-15

Source:	        Keysight Technologies UK Ltd

Agenda Item:	5.4.17

Document for:	Discussion and Endorsement

# 1.	Introduction

Table 5.3.5-1 in 38.101-1 ([1], [2]) defines UE channel bandwidth per each NR FR1 frequency operating band. To be noticed that the support of these channel bandwidths per band is understood as mandatory unless a specific note is defined to indicate that a given channel bandwidth is optional for a given release (NOTE 4 in Table 5.3.5-1 in [1] and [2]).

Similarly, Table 5.3.5-1 in 38.101-2 ([3], [4]) defines UE channel bandwidth per each NR FR2 frequency operating band, also indicating through notes which are the channel bandwidths that are optional for a given release of the specification (NOTE 1 in Table 5.3.5-1 in [3] and [4]).

On the other hand, in 38.508-1 ([5]) section 4.3.1.0A, there are some exemptions documented for single carrier low, mid and high test channel bandwidth for NR FR1 as for NR FR2 in case some of the understood as mandatory channel bandwidths are not supported applicable to Release 15 only.

This is creating some confusion as it is not clear whether channel bandwidths not documented as optional in core specifications ([1]-[4]) are really mandatory or whether exemptions should be extended to other releases different from Rel-15.

This document reviews the background for existing channel bandwidths exemptions and proposes a way forward to clarify existing ambiguities regarding the applicable channel bandwidths to be tested.

# 2.	Background

# 2.1 Single CC

# 2.1.1 RAN plenary discussions

The topic of mandatory channel bandwidths was discussed under Rel-15 scope at RAN#78 back in 2017 for single carrier. At that time, the way forward agreed ([6]) stated the following aspects:

Observation 1: RAN plenary decided in [6] that:

- UE maximum channel bandwidth is a UE capability. (R4-1708845) ([7])

- UE shall support any Rel-15 channel bandwidth that is smaller than its UE supported maximum channel bandwidth. (R4-1706321) ([8])

- The maximum channel bandwidth supported by UE can be different for UL & DL and SCS specific. (R4-1706321) ([8])

- All bandwidths listed in TS 38.101-1 v15.0.0 Table 5.3.5-1 for each band shall be mandatory with a single CC.

Then, it was discussed again (still for single component carriers) during RAN#80 through [9]:

concluding with the way forward in [10]:

Observation 2: IOdT bits were requested with the goal to enable the UE to indicate which channel bandwidths it supported, as initial NR deployments will likely not support all the channel bandwidths and SCS combinations defined in 38.101-1 and 38.101-2.

Observation 3: At RAN plenary level the period in which these IOdT bits were applicable (definition of initial NR deployments) was not discussed.

# 2.1.2 UE capabilities

As a consequence of RAN plenary discussions described in section 2.1.1, RAN2 discussed the definition of corresponding UE capabilities and achieved first endorsement of [11] then final agreement in [12]:

Hence it can be seen that related UE capabilities for DL channel bandwidth for single CC are defined in 38.306 ([13]) as:

Related UE capabilities for UL channel bandwidth for single CC are defined in 38.306 ([13]) as:

Observation 4: UE capabilities related to channel bandwidths support for single carrier are: channelBWs-DL, channelBWs-DL-v1590, channelBWs-UL, channelBWs-UL-v1590, channelBW-90mhz, supportedBandwidthDL and supportedBandwidthUL, where channelBWs-xL inidicated the supported channel bandwidths for each SCS while supportedBandwidthxL indicates the maximum channel bandwidths supported.

Observation 5: According to supportedBandwidthxL definition, for FR1 all the bandwidths listed in TS38.101-1 Table 5.3.5-1 for each band shall be mandatory with a single CC unless indicated optional. For FR2, the set of mandatory CBW is 50,100, 200 MHz.

Observation 6: IOdT bits map with channelBWs-xL.

Observation 7: At RAN2 level, the period in which these IOdT bits were applicable (definition of initial NR deployments) was not discussed.

# 2.1.3 RAN5 related activities

Once RAN2 defined UE capabilities as described in sections 2.1.2, RAN5 worked in the addition of clarification notes to indicate how exempted channel bandwidths should be handled when defining channel bandwidths to be tested. Clarifications regarding mid channel were added in [14] while similar clarifications for low and high test channel bandwidth were added in [15]:

Table 4.3.1-1: Mid Test Channel bandwidths for each NR band, FR1 NR band / UE Mid Test Channel bandwidth. NR Band; Mid [MHz]. n1; 15. n2; 15. n3; 15. n5; 15. n7; 15. n8; 15. n12; 10. n20; 15. n25; 15. n28; 15. n34; 10. n38; 15. n39; 20. n40; 30. n41; 50. n51; 5. n66; 20. n70; 15. n71; 10. n75; 15. n76; 5. n77; 50. n78; 50. n79; 60. n80; 20. n81; 15. n82; 15. n83; 15. n84; 15. n86; 20. Note 1: For UEs where IOT bit declaration is required due to lack of channel BW support in the network, if mid channel BW is not supported by the UE, select the closest lower channel BW supported by the UE in both UL and DL. This shall apply until further updates from RAN plenary and only for Rel 15 UEs.. Table 4.3.1-2: Mid Test Channel bandwidths for each NR band, FR2 NR band / UE Mid Test Channel bandwidth. NR Band; Mid [MHz]. n257; [200]. n258; [200]. n260; [200]. NOTE 1: For UEs where IOT bit declaration is required due to lack of channel BW support in the network, if mid channel BW is not supported by the UE, select the closest lower channel BW supported by the UE in both UL and DL. This shall apply until further updates from RAN plenary and only for Rel 15 UEs..

Notes added in [13] and [14] were revised at a later stage in [15] to remove references to IOT bits:

The notes added in ([13]) were later revised in ([14]): Table 4.3.1-1: Mid Test Channel bandwidths for each NR band, FR1 NR band / UE Mid Test Channel bandwidth. NR Band; Mid [MHz]. n1; 15. n2; 15. n3; 15. n5; 15. n7; 15. n8; 15. n12; 10. n20; 15. n25; 15. n28; 15. n34; 10. n38; 15. n39; 20. n40; 30. n41; 50. n51; 5. n66; 20. n70; 15. n71; 10. n75; 15. n76; 5. n77; 50. n78; 50. n79; 60. n80; 20. n81; 15. n82; 15. n83; 15. n84; 15. n86; 20. Note 1: For UEs with limited UE channel bandwidth capability, if mid channel BW is not supported by the UE, select the closest lower channel BW supported by the UE in both UL and DL. This shall apply only for Rel 15 UEs.. Table 4.3.1-2: Mid Test Channel bandwidths for each NR band, FR2 NR band / UE Mid Test Channel bandwidth. NR Band; Mid [MHz]. n257; 100. n258; [200]. n260; [200]. NOTE 1: For UEs with limited UE channel bandwidth capability, if mid channel BW is not supported by the UE, select the closest lower channel BW supported by the UE in both UL and DL. This shall apply only for Rel 15 UEs.. (Only mid test channel bandwidth changes are shown for simplicity, but similar changes applied to low and high test channel bandwidths).

Observation 8: At RAN5 level, the period in which these IOdT bits were applicable (definition of initial NR deployments) was discussed and concluded as Rel-15 UEs.

# 2.2 Carrier aggregation, dual connectivity and supplemental uplink

# 2.2.1 RAN4 definitions

Regarding CA and SUL band combinations, RAN4 defines different bandwidths combinations sets indicating band which bandwidth combinations shall be supported in each band in the band combination for such band combo.

Keysight’s understanding is that all bandwidths’ combinations defined within a bandwidth combination set are mandatory. Otherwise, the definition of different bandwidths combination sets would make no sense if the device could indicate through Single CC capabilities which channel bandwidths (channelBWS-xL) it supports.

Observation 9: For CA and SUL, RAN4 defines different bandwidths combinations sets indicating band which bandwidth combinations shall be supported in each band in the band combination for such band combo.

Observation 10: All bandwidths’ combinations defined within a bandwidth combination set are mandatory.

Observation 11: Bandwidth combination sets might include any mandatory and optional channel bandwidth defined for each of the frequency bands within such band combination.

Regarding FR1 dual connectivity or FR1+FR2 dual connectivity, core specifications indicate in section 5.5B that the bandwidth combination sets for the corresponding NR CA configurations are applicable to dual connectivity:

In 38.101-1 ([2]): In 38.101-3 ([17]):

For EN-DC, in 38.101-3 ([17]):

- section 5.3B.1.2 defines bandwidth combination sets for Intra-band contiguous EN-DC,

- Section 5.3B.1.3 defines bandwidth combination sets for Intra-band non-contiguous EN-DC,

However, there is no definition of bandwidth combination sets for Inter-band EN-DC.

Observation 12: No definition of bandwidth combination sets for Inter-band EN-DC can be found in 38.101-3 ([17]) while such definition exists for intra-band contiguous EN-DC and intra-band non-contiguous EN-DC.

Observation 13: It is understood that applicable bandwidth combination set defined for each cell group applies to Inter-band EN-DC.

Observation 14: Similar bandwidth combination sets defined for EN-DC apply to NE-DC.

# 2.2.2 UE capabilities

Regarding CA and dual connectivity, there are additional capabilities defined for different band combinations in 38.306 related to bandwidth combination sets ([13]):

Observation 15: Devices can indicate which band combination sets are supported for each CA, DC or SUL band combo through UE capabilities supportedBandwidthCombinationSetEUTRA, supportedBandwidthCombinationSet, supportedBandwidthCombinationSetIntraEN-DC.

Observation 16: There is no UE capability where the device can indicate that it does not support a certain bandwidth combination within the bandwidth combination set it supports.

# 2.3 Operators expectations

Keysight’s understanding is that operator expectations regarding supported channel bandwidths in the devices are as currently defined in core specifications (where all channel bandwidths are mandatory unless defined as optional) with the exemptions defined in RAN5 test specifications for Rel-15 devices (applying IOdT bits defined in channelBWs-xL capabilities). Hence the only optional channel bandwidths for Rel-16 and forward devices are the ones described in core specifications.

# 3.  	Discussion

UEs declaring to be Rel-16 or later but not supporting all the bandwidths assumed mandatory for the Release declared by the UE have been found, driving therefore into conflict when running against a conformance test case implementation. Therefore, it is necessary this topic is reopened at RAN5 level so one of the following proposals can be agreed:

Proposal 1A: Limitations on mandatory channel bandwidths supported by the UE are only permitted for Rel-15 UEs in RF conformance testing for single carrier testing. For CA, DC and SUL no limitations are allowed. This is the understanding of current test specs. No further changes needed.

Proposal 1B: Limitations on mandatory channel bandwidths supported by the UE are permitted in RF conformance testing for single carrier testing, disregarding UE’s declared release. Updates will be needed to 38.508-1 to clarify this aspect. An LS to RAN2 might be required to clarify UE capabilities. For CA, DC and SUL no limitations are allowed. Corresponding CR has been submitted in [18].

Proposal 1C: RAN5 cannot reach a consensus on up to which UE release mandatory channel bandwidths exemptions are allowed for either single carrier or CA, DC or SUL testing. In this case, an LS is required to be sent to RAN4 copying RAN2 to get clarification. A draft LS has been submitted in [19].

# 4. 	Conclusion

This document reviews the background for existing channel bandwidths exemptions and proposes a way forward to clarify existing ambiguities regarding the applicable channel bandwidths to be tested. The following observations are done:

Observation 1: RAN plenary decided in [6] that:

- UE maximum channel bandwidth is a UE capability. (R4-1708845) ([7])

- UE shall support any Rel-15 channel bandwidth that is smaller than its UE supported maximum channel bandwidth. (R4-1706321) ([8])

- The maximum channel bandwidth supported by UE can be different for UL & DL and SCS specific. (R4-1706321) ([8])

- All bandwidths listed in TS 38.101-1 v15.0.0 Table 5.3.5-1 for each band shall be mandatory with a single CC.

Observation 2: IOdT bits were requested with the goal to enable the UE to indicate which channel bandwidths it supported, as initial NR deployments will likely not support all the channel bandwidths and SCS combinations defined in 38.101-1 and 38.101-2.

Observation 3: At RAN plenary level the period in which these IOdT bits were applicable (definition of initial NR deployments) was not discussed.

Observation 4: UE capabilities related to channel bandwidths support for single carrier are: channelBWs-DL, channelBWs-DL-v1590, channelBWs-UL, channelBWs-UL-v1590, channelBW-90mhz, supportedBandwidthDL and supportedBandwidthUL, where channelBWs-xL inidicated the supported channel bandwidths for each SCS while supportedBandwidthxL indicates the maximum channel bandwidths supported.

Observation 5: According to supportedBandwidthxL definition, for FR1 all the bandwidths listed in TS38.101-1 Table 5.3.5-1 for each band shall be mandatory with a single CC unless indicated optional. For FR2, the set of mandatory CBW is 50,100, 200 MHz.

Observation 6: IOdT bits map with channelBWs-xL.

Observation 7: At RAN2 level, the period in which these IOdT bits were applicable (definition of initial NR deployments) was not discussed.

Observation 8: At RAN5 level, the period in which these IOdT bits were applicable (definition of initial NR deployments) was discussed and concluded as Rel-15 UEs.

Observation 9: For CA and SUL, RAN4 defines different bandwidths combinations sets indicating band which bandwidth combinations shall be supported in each band in the band combination for such band combo.

Observation 10: All bandwidths’ combinations defined within a bandwidth combination set are mandatory.

Observation 11: Bandwidth combination sets might include any mandatory and optional channel bandwidth defined for each of the frequency bands within such band combination.

Observation 12: No definition of bandwidth combination sets for Inter-band EN-DC can be found in 38.101-3 ([17]) while such definition exists for intra-band contiguous EN-DC and intra-band non-contiguous EN-DC.

Observation 13: It is understood that applicable bandwidth combination set defined for each cell group applies to Inter-band EN-DC.

Observation 14: Similar bandwidth combination sets defined for EN-DC apply to NE-DC.

Observation 15: Devices can indicate which band combination sets are supported for each CA, DC or SUL band combo through UE capabilities supportedBandwidthCombinationSetEUTRA, supportedBandwidthCombinationSet, supportedBandwidthCombinationSetIntraEN-DC.

Observation 16: There is no UE capability where the device can indicate that it does not support a certain bandwidth combination within the bandwidth combination set it supports.

The recommendation is to agree one of the following proposals:

Proposal 1A: Limitations on mandatory channel bandwidths supported by the UE are only permitted for Rel-15 UEs in RF conformance testing for single carrier testing. For CA, DC and SUL no limitations are allowed. This is the understanding of current test specs. No further changes needed.

Proposal 1B: Limitations on mandatory channel bandwidths supported by the UE are permitted in RF conformance testing for single carrier testing, disregarding UE’s declared release. Updates will be needed to 38.508-1 to clarify this aspect. An LS to RAN2 might be required to clarify UE capabilities. For CA, DC and SUL no limitations are allowed. Corresponding CR has been submitted in [18].

Proposal 1C: RAN5 cannot reach a consensus on up to which UE release mandatory channel bandwidths exemptions are allowed for either single carrier or CA, DC or SUL testing. In this case, an LS is required to be sent to RAN4 copying RAN2 to get clarification. A draft LS has been submitted in [19].

# 5.	References

[1] 3GPP TS 38.101-1 v16.11.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 16)

[2] 3GPP TS 38.101-1 v17.5.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 17)

[3] 3GPP TS 38.101-2 v16.11.0 User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone (Release 16)

[4] 3GPP TS 38.101-2 v17.5.0 User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone (Release 17)

[5] 3GPP TS 38.508-1 v17.4.0 User Equipment (UE) conformance specification; Part1: Common test environment (Release 17)

[6] RP-172832, “WF on Channel Bandwidth Support for NR”, Qualcomm Incorporated, Sprint, Nokia, Nokia Shanghai Bell, Deutsche Telekom, Orange, Telecom Italia, BT, AT&T,

T-Mobile US, Dish Network, Verizon, RAN#78.

[7] R4-1708845, “WF on UE mandatory channel bandwidth”, Nokia, RAN4#84

[8] R4-1706321, “WF on Channel Bandwidth”, Qualcomm Incorporated, RAN4#83

[9] RP-181011, “IOdT Bits for NR”, Qualcomm Incorporated, RAN#80

[10] RP-181443, “WF on IOdT Bits for SCS and Channel Bandwidths”, Qualcomm Incorporated, RAN#80

[11] R2-1810908, “Channel bandwidth UE capabilities”, Qualcomm Incorporated, RAN2 AH Meeting #1807

[12] R2-1813437, “38.306 corrections and cleanup”, Intel Corporation, RAN2#103

[13] 3GPP TS 38.306 v17.0.0 User Equipment (UE) radio access capabilities (Release 17)

[14] R5-186677, “Resubmission of update to 38.508 for mid channel bandwidth”, Qualcomm Finland RFFE Oy, RAN5#81

[15] R5-187557, “Addition of low and high test channel bandwidth in 38.508”, Qualcomm Japan Inc, RAN5#81

[16] R5-197304, “Removing IOT bit information from test channel bandwidth tables”, Ericsson, Sprint, RAN5#84

[17] 3GPP TS 38.101-3 v17.5.0 User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios (Release 17)

[18] R5-223200, “Corrections on mandatory channel bandwidths after Rel-15”, Keysight Technologies UK Ltd, RAN5#95

[19] R5-223201, “Draft LS on Channel bandwidths exemptions”, Keysight Technologies UK Ltd, RAN5#95