3GPP TSG-RAN5 Meeting #96-e	R5-224093r2

# Online, , 15th August 2022 - 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2428 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on UE Power Limit Messages: Option 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd, Apple Portugal |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2022-07-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Message type value for POWER LIMIT test functions is not aligned with definition in TS 38.509 according to R5-224088 and R5-224089. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Redefine message type value for POWER LIMIT test functions. - Redefine parameters for ACTIVATE POWER LIMIT REQUEST message to also consider unequal bandwidth among all component carriers within the aggregated bandwidth. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UL-CA tests blocked from execution due to SCell Drop Issue |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7A.7, 4.9.32 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR depends on R5-224078, R5-224084, R5-224085, R5-224088 and R5-224089. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: CR title corrected to remove test specification redundant information. Revision 2: Merging with R5-224091. Values and conditions from R5-224091 reviewed. |  |  |  |  |  |  |  |  |

<Unchanged Sections skipped>

#### NSSAI DELETE RESPONSE

This message is only sent in the direction UE to SS, embedded in a RRC ULInformationTransfer message.

Table 4.7A.6-2: NSSAI DELETE RESPONSE

| Derivation Path: 38.509 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 0 0 1 1 1 |  |  |

### 4.7A.7	UE Power Limit Messages

Editor’s note:	The power limit request message values set by the UPLF test mode is currently applicable to equal PSD (equal channel bandwidths on all component carriers) only. Message values are pending for unequal channel bandwidths

#### -	ACTIVATE POWER LIMIT REQUEST

This Message is only sent in the direction SS to UE, embedded in a RRC DLInformationTransfer message.

- Table 4.7A.7-1: ACTIVATE POWER LIMIT REQUEST

| Derivation Path: 38.509 clause 6.11.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 0 1 1 1 0 |  |  |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 0 0 1 0 |  | TOT NR AGG BW 100 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 0 0 1 1 |  | TOT NR AGG BW 150 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 0 1 0 0 |  | TOT NR AGG BW 200 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 0 1 0 1 |  | TOT NR AGG BW 250 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 0 1 1 0 |  | TOT NR AGG BW 300 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 0 1 1 1 |  | TOT NR AGG BW 350 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 0 0 0 |  | TOT NR AGG BW 400 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 0 0 1 |  | TOT NR AGG BW 450 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 0 1 0 |  | TOT NR AGG BW 500 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 0 1 1 |  | TOT NR AGG BW 550 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 1 0 0 |  | TOT NR AGG BW 600 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 1 0 1 |  | TOT NR AGG BW 650 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 1 1 0 |  | TOT NR AGG BW 700 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 1 1 1 |  | TOT NR AGG BW 750 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 0 0 0 |  | TOT NR AGG BW 800 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 0 0 1 |  | TOT NR AGG BW 850 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 0 1 0 |  | TOT NR AGG BW 900 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 0 1 1 |  | TOT NR AGG BW 950 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 1 0 0 |  | TOT NR AGG BW 1000 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 1 0 1 |  | TOT NR AGG BW 1050 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 1 1 0 |  | TOT NR AGG BW 1100 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 1 1 1 |  | TOT NR AGG BW 1150 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 0 0 0 |  | TOT NR AGG BW 1200 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 0 0 1 |  | TOT NR AGG BW 1250 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 0 1 0 |  | TOT NR AGG BW 1300 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 0 1 1 |  | TOT NR AGG BW 1350 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 1 0 0 |  | TOT NR AGG BW 1400 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 1 0 1 |  | TOT NR AGG BW 1450 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 1 1 0 |  | TOT NR AGG BW 1500 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 1 1 1 |  | TOT NR AGG BW 1550 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 1 0 0 0 0 0 |  | TOT NR AGG BW 1600 |
| PCELL NR BANDWIDTH | 0 0 0 0 0 0 0 1 |  | PCELL NR BW 50 |
| PCELL NR BANDWIDTH | 0 0 0 0 0 0 1 0 |  | PCELL NR BW 100 |
| PCELL NR BANDWIDTH | 0 0 0 0 0 1 0 0 |  | PCELL NR BW 200 |
| PCELL NR BANDWIDTH | 0 0 0 0 1 0 0 0 |  | PCELL NR BW 400 |

| Condition | Explanation |
| --- | --- |
| TOT NR AGG BW 100 | Total NR aggregated Bandwidth equal to 100 MHz |
| TOT NR AGG BW 150 | Total NR aggregated Bandwidth equal to 150 MHz |
| TOT NR AGG BW 200 | Total NR aggregated Bandwidth equal to 200 MHz |
| TOT NR AGG BW 250 | Total NR aggregated Bandwidth equal to 250 MHz |
| TOT NR AGG BW 300 | Total NR aggregated Bandwidth equal to 300 MHz |
| TOT NR AGG BW 350 | Total NR aggregated Bandwidth equal to 350 MHz |
| TOT NR AGG BW 400 | Total NR aggregated Bandwidth equal to 400 MHz |
| TOT NR AGG BW 450 | Total NR aggregated Bandwidth equal to 450 MHz |
| TOT NR AGG BW 500 | Total NR aggregated Bandwidth equal to 500 MHz |
| TOT NR AGG BW 550 | Total NR aggregated Bandwidth equal to 550 MHz |
| TOT NR AGG BW 600 | Total NR aggregated Bandwidth equal to 600 MHz |
| TOT NR AGG BW 650 | Total NR aggregated Bandwidth equal to 650 MHz |
| TOT NR AGG BW 700 | Total NR aggregated Bandwidth equal to 700 MHz |
| TOT NR AGG BW 750 | Total NR aggregated Bandwidth equal to 750 MHz |
| TOT NR AGG BW 800 | Total NR aggregated Bandwidth equal to 800 MHz |
| TOT NR AGG BW 850 | Total NR aggregated Bandwidth equal to 850 MHz |
| TOT NR AGG BW 900 | Total NR aggregated Bandwidth equal to 900 MHz |
| TOT NR AGG BW 950 | Total NR aggregated Bandwidth equal to 950 MHz |
| TOT NR AGG BW 1000 | Total NR aggregated Bandwidth equal to 1000 MHz |
| TOT NR AGG BW 1050 | Total NR aggregated Bandwidth equal to 1050 MHz |
| TOT NR AGG BW 1100 | Total NR aggregated Bandwidth equal to 1100 MHz |
| TOT NR AGG BW 1150 | Total NR aggregated Bandwidth equal to 1150 MHz |
| TOT NR AGG BW 1200 | Total NR aggregated Bandwidth equal to 1200 MHz |
| TOT NR AGG BW 1250 | Total NR aggregated Bandwidth equal to 1250 MHz |
| TOT NR AGG BW 1300 | Total NR aggregated Bandwidth equal to 1300 MHz |
| TOT NR AGG BW 1350 | Total NR aggregated Bandwidth equal to 1350 MHz |
| TOT NR AGG BW 1400 | Total NR aggregated Bandwidth equal to 1400 MHz |
| TOT NR AGG BW 1450 | Total NR aggregated Bandwidth equal to 1450 MHz |
| TOT NR AGG BW 1500 | Total NR aggregated Bandwidth equal to 1500 MHz |
| TOT NR AGG BW 1550 | Total NR aggregated Bandwidth equal to 1550 MHz |
| TOT NR AGG BW 1600 | Total NR aggregated Bandwidth equal to 1600 MHz |
| PCELL NR BW 50 | PCC NR Bandwidth equal to 50 MHz |
| PCELL NR BW 100 | PCC NR Bandwidth equal to 100 MHz |
| PCELL NR BW 200 | PCC NR Bandwidth equal to 200 MHz |
| PCELL NR BW 400 | PCC NR Bandwidth equal to 400 MHz |

#### ACTIVATE POWER LIMIT RESPONSE

This message is only sent in the direction UE to SS, embedded in a RRC ULInformationTransfer message.

- Table 4.7A.7-2: ACTIVATE POWER LIMIT RESPONSE

| Derivation Path: 38.509 clause 6.11.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 0 1 1 1 1 |  |  |

#### DEACTIVATE POWER LIMIT REQUEST

This message is only sent in the direction SS to UE, embedded in a RRC DLInformationTransfer message.

- Table 4.7A.7-3: DEACTIVATE POWER LIMIT REQUEST

| Derivation Path: 38.509 clause 6.11.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 1 0 0 0 0 |  |  |

#### DEACTIVATE POWER LIMIT RESPONSE

This message is only sent in the direction UE to SS, embedded in a RRC ULInformationTransfer message.

- Table 4.7A.7-3: DEACTIVATE POWER LIMIT RESPONSE

| Derivation Path: 38.509 clause 6.11.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 1 0 0 0 1 |  |  |

<Unchanged sections skipped>

### 4.9.32	Test procedure to activate UE Power Limit Function (UPLF)

Editor’s note:	The power limit request message values set by the UPLF test mode is currently applicable to equal PSD (equal channel bandwidths on all component carriers) only. Message values are pending for unequal channel bandwidths

4.9.32.1	Initiation

UE is operating in FR2 in RRC_CONNECTED state with UE power limit test function activated.

4.9.32.2	Procedure

Table 4.9.32.2-1: Test procedure Sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1 | SS request UE to activate UE Power Limit Function | <-- | ACTIVATE POWER LIMIT REQUEST | - | - |
| 2 | UE confirms that UE Power Limit Function is activated | --> | ACTIVATE POWER LIMIT RESPONSE | - | - |

4.9.32.3	Specific Message contents

- Table 4.9.32.3-1: ACTIVATE POWER LIMIT REQUEST

| Derivation Path: 38.509 clause 6.11.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 1 0 1 1 1 0 |  |  |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 0 0 1 0 |  | TOT NR AGG BW 100 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 0 1 1 |  | TOT NR AGG BW 150 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 0 1 0 0 |  | TOT NR AGG BW 200 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 0 1 0 1 |  | TOT NR AGG BW 250 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 0 1 1 0 |  | TOT NR AGG BW 300 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 0 1 1 1 |  | TOT NR AGG BW 350 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 0 0 0 |  | TOT NR AGG BW 400 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 0 0 1 |  | TOT NR AGG BW 450 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 0 1 0 |  | TOT NR AGG BW 500 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 0 1 1 |  | TOT NR AGG BW 550 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 1 0 0 |  | TOT NR AGG BW 600 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 1 0 1 |  | TOT NR AGG BW 650 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 1 1 0 |  | TOT NR AGG BW 700 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 0 1 1 1 1 |  | TOT NR AGG BW 750 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 0 0 0 |  | TOT NR AGG BW 800 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 0 0 1 |  | TOT NR AGG BW 850 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 0 1 0 |  | TOT NR AGG BW 900 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 0 1 1 |  | TOT NR AGG BW 950 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 1 0 0 |  | TOT NR AGG BW 1000 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 1 0 1 |  | TOT NR AGG BW 1050 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 1 1 0 |  | TOT NR AGG BW 1100 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 0 1 1 1 |  | TOT NR AGG BW 1150 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 0 0 0 |  | TOT NR AGG BW 1200 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 0 0 1 |  | TOT NR AGG BW 1250 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 0 1 0 |  | TOT NR AGG BW 1300 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 0 1 1 |  | TOT NR AGG BW 1350 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 1 0 0 |  | TOT NR AGG BW 1400 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 1 0 1 |  | TOT NR AGG BW 1450 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 1 1 0 |  | TOT NR AGG BW 1500 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 0 1 1 1 1 1 |  | TOT NR AGG BW 1550 |
| TOTAL NR AGGREGATED BANDWIDTH | 0 0 1 0 0 0 0 0 |  | TOT NR AGG BW 1600 |
| PCELL NR BANDWIDTH | 0 0 0 0 0 0 0 1 |  | PCELL NR BW 50 |
| PCELL NR BANDWIDTH | 0 0 0 0 0 0 1 0 |  | PCELL NR BW 100 |
| PCELL NR BANDWIDTH | 0 0 0 0 0 1 0 0 |  | PCELL NR BW 200 |
| PCELL NR BANDWIDTH | 0 0 0 0 1 0 0 0 |  | PCELL NR BW 400 |

| Condition | Explanation |
| --- | --- |
| TOT NR AGG BW 100 | Total NR aggregated Bandwidth equal to 100 MHz |
| TOT NR AGG BW 150 | Total NR aggregated Bandwidth equal to 150 MHz |
| TOT NR AGG BW 200 | Total NR aggregated Bandwidth equal to 200 MHz |
| TOT NR AGG BW 250 | Total NR aggregated Bandwidth equal to 250 MHz |
| TOT NR AGG BW 300 | Total NR aggregated Bandwidth equal to 300 MHz |
| TOT NR AGG BW 350 | Total NR aggregated Bandwidth equal to 350 MHz |
| TOT NR AGG BW 400 | Total NR aggregated Bandwidth equal to 400 MHz |
| TOT NR AGG BW 450 | Total NR aggregated Bandwidth equal to 450 MHz |
| TOT NR AGG BW 500 | Total NR aggregated Bandwidth equal to 500 MHz |
| TOT NR AGG BW 550 | Total NR aggregated Bandwidth equal to 550 MHz |
| TOT NR AGG BW 600 | Total NR aggregated Bandwidth equal to 600 MHz |
| TOT NR AGG BW 650 | Total NR aggregated Bandwidth equal to 650 MHz |
| TOT NR AGG BW 700 | Total NR aggregated Bandwidth equal to 700 MHz |
| TOT NR AGG BW 750 | Total NR aggregated Bandwidth equal to 750 MHz |
| TOT NR AGG BW 800 | Total NR aggregated Bandwidth equal to 800 MHz |
| TOT NR AGG BW 850 | Total NR aggregated Bandwidth equal to 850 MHz |
| TOT NR AGG BW 900 | Total NR aggregated Bandwidth equal to 900 MHz |
| TOT NR AGG BW 950 | Total NR aggregated Bandwidth equal to 950 MHz |
| TOT NR AGG BW 1000 | Total NR aggregated Bandwidth equal to 1000 MHz |
| TOT NR AGG BW 1050 | Total NR aggregated Bandwidth equal to 1050 MHz |
| TOT NR AGG BW 1100 | Total NR aggregated Bandwidth equal to 1100 MHz |
| TOT NR AGG BW 1150 | Total NR aggregated Bandwidth equal to 1150 MHz |
| TOT NR AGG BW 1200 | Total NR aggregated Bandwidth equal to 1200 MHz |
| TOT NR AGG BW 1250 | Total NR aggregated Bandwidth equal to 1250 MHz |
| TOT NR AGG BW 1300 | Total NR aggregated Bandwidth equal to 1300 MHz |
| TOT NR AGG BW 1350 | Total NR aggregated Bandwidth equal to 1350 MHz |
| TOT NR AGG BW 1400 | Total NR aggregated Bandwidth equal to 1400 MHz |
| TOT NR AGG BW 1450 | Total NR aggregated Bandwidth equal to 1450 MHz |
| TOT NR AGG BW 1500 | Total NR aggregated Bandwidth equal to 1500 MHz |
| TOT NR AGG BW 1550 | Total NR aggregated Bandwidth equal to 1550 MHz |
| TOT NR AGG BW 1600 | Total NR aggregated Bandwidth equal to 1600 MHz |
| PCELL NR BW 50 | PCC NR Bandwidth equal to 50 MHz |
| PCELL NR BW 100 | PCC NR Bandwidth equal to 100 MHz |
| PCELL NR BW 200 | PCC NR Bandwidth equal to 200 MHz |
| PCELL NR BW 400 | PCC NR Bandwidth equal to 400 MHz |

- Table 4.9.32.3-2: ACTIVATE POWER LIMIT RESPONSE

| Derivation Path: 38.509 clause 6.11.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 1 1 1 1 |  |  |
| Skip indicator |  | 0 0 0 0 |  |  |
| Message type |  | 1 0 1 0 1 1 1 1 |  |  |