3GPP TSG-RAN5 Meeting #96-e	R5-224110r3
Electronic Meeting, 15th August – 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0765 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhanced Beam correspondence Measurement Uncertainties and test tolerances |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd, Apple Portugal |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Measurement uncertainties and test tolerances for the new Rel-16 FR2 RF enhancements test case for enhanced beam correspondence tolerance are undefined. Some of the changes in R5-223750 were not properly implemented. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define measurement uncertainties and test tolerances for enhanced beam correspondence tolerance test as proposed in R5-224109. Redo missing changes agreed in R5-223750. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specifications will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.1_1, 6.2.1.2_1, 6.6.2, F.1.2, F.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is dependent on R5-224109. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: CR title corrected to remove redundant test specification information. Revision 2: Adding Apple Portugal as co-sourcing company tafter solving the overlapping conflict with R5-225235. Revision 3: Remove 6.2.1.1_1 title change. |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES 1>>>

#### 6.2.1.1_1	UE maximum output power - EIRP and TRP (Rel16 and forward)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

-	Same as in 6.2.1.1

-

6.2.1.1_1.1	Test purpose

Same as 6.2.1.1.1

6.2.1.1_1.2	Test applicability

This test case applies to all types of NR UEs release 16 and forward supporting either SSB-based or CSI-RS based enhanced beam correspondence.

6.2.1.1_1.3	Minimum conformance requirements

Same as 6.2.1.1.3 except for the following section on multi-band relaxation factors:

For the Release 16 UEs that support multiple FR2 bands, minimum requirement for peak EIRP and EIRP spherical coverage in Tables 6.2.1.1_1.3.3-1 and 6.2.1.1_1.3.3-3 shall be decreased per band, respectively, by the peak EIRP relaxation parameter MBP,n and EIRP spherical coverage relaxation parameter MBS,n, as indicated in Table 6.2.1.1_1.3-1.

Table 6.2.1.1_1.3.1-1: UE multi-band relaxation factors for power class 3 (Rel-16 and forward)

| Band | MBP,n (dB) | MBS,n (dB) |
| --- | --- | --- |
| n257 | 0.73 | 0.73 |
| n258 | 0.6 | 0.7 |
| n259 | 0.5 | 0.4 |
| n260 | 0.51 | 0.41 |
| n261 | 0.52,4 | 0.74 |
| Note 1: n260 peak and spherical relaxations are 0 dB for UE that exclusively supports n261+n260 Note 2: n261 peak relaxation is 0 dB for UE that exclusively supports n261+n260 Note 3: n257 peak and spherical relaxations are 0 dB for UE that exclusively supports n261+n257 Note 4: n261 peak and spherical relaxations are 0 dB for UE that exclusively supports n261+n257 |  |  |

6.2.1.1_1.4	Test description

6.2.1.1_1.4.1	Initial conditions

Same as 6.2.1.1.4.1 and 6.6.1.4.3

6.2.1.1_1.4.2	Test procedure

The following cases are tested depending on UE capability:

- Test procedure if beamCorrespondenceWithoutUL-BeamSweeping is NOT supported and beamCorrespondenceSSB-based-r16 is supported:

1.1	Same as 6.2.1.1.4.2 with the exception that step 6 is skipped and measurements shall be carried out using only side conditions defined in Table 6.6.1.3.3.1.1-1

1.2	Skip to Step 7.

2.	Test procedure if beamCorrespondenceWithoutUL-BeamSweeping is NOT supported, and beamCorrespondenceCSI-RS-based-r16 is supported

2.1	Same as 6.2.1.1.4.2 with the exception that step 6 is skipped and measurements shall be carried out using only side conditions defined in Table 6.6.2.3.3-1

2.2	Skip to Step 7.

3.	Test procedure if beamCorrespondenceWithoutUL-BeamSweeping is NOT supported, beamCorrespondenceCSI-RS-based-r16 and beamCorrespondenceSSB-based-r16 are supported

3.1	 Same as 6.2.1.1.4.2 with the exception that step 6 is skipped and measurements shall be carried out using only side conditions defined in Table 6.6.1.3.3.1.1-1.

3.2	Repeat 6.2.1.1.4.2 with step 6 skipped with Tx Beam Peak direction determined using the side conditions in Table 6.6.2.3.3-1 Record the verdict (as this result will not be compared to test requirements in this test case but in a different one)..3.3	Skip to Step 7.

4.	Test procedure if beamCorrespondenceWithoutUL-BeamSweeping is supported and beamCorrespondenceSSB- based-r16 is supported:

4.1	Same as 6.2.1.1.4.2 with the exception that step 6 is skipped and measurements shall be carried out using only side conditions defined in Table 6.6.1.3.3.1.1-1.

4.2	Skip to Step 7.

5.	Test procedure if beamCorrespondenceWithoutUL-BeamSweeping is supported and beamCorrespondenceCSI-RS-based-r16 is supported:

5.1	Same as 6.2.1.1.4.2 with the exception that step 6 is skipped and measurements shall be carried out using only side conditions defined in Table 6.6.2.3.3-1

5.2	Skip to Step 7

6.	Test procedure if beamCorrespondenceWithoutUL-BeamSweeping is supported, beamCorrespondenceCSI-RS-based-r16 and beamCorrespondenceSSB-based-r16 is supported

6.1	Same as 6.2.1.1.4.2 with the exception that step 6 is skipped and measurements shall be carried out using only side conditions defined in Table 6.6.1.3.3.1.1-1.

6.2	Repeat 6.2.1.1.4.2 with step 6 skipped with Tx Beam Peak direction determined using the side conditions in Table 6.6.2.3.3-1. Record the verdict (as this result will not be compared to test requirements in this test case but in a different one).

7.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.1.

6.2.1.1_1.4.3	Message contents

Same as 6.2.1.1_1.4.3

6.2.1.1_1.5	Test requirement

Same as 6.2.1.1_1.5 except for Table 6.2.1.1_1.5_1

Table 6.2.1.1_1.5-1: UE maximum output test requirements for power class 3 (Rel-16 and forward)

| ID | FR2 bands/set | Test requirement (dB) (Note 1) |  |  |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | n257 | n258 | n259 | n260 | n261 |  |
| 1 | n257 | 22.4-TT-MBP,n |  |  |  |  |  |
| 2 | n258 |  | 22.4-TT-MBP,n |  |  |  |  |
| 3 | n259 |  |  | 18.7-TT-MBP,n |  |  |  |
| 4 | n260 |  |  |  | 20.6-TT-MBP,n |  |  |
| 5 | n261 |  |  |  |  | 22.4-TT-MBP,n |  |
| 6 | n257, n261 | 22.4-TT-MBP,n |  |  |  | 22.4-TT-MBP,n | MBP,n relaxation is 0 dB |
| 7 | n260, n261 |  |  |  | 20.6-TT-MBP,n | 22.4-TT-MBP,n | MBP,n relaxation is 0 dB |
| Note 1: MBP,n is the Multi-band Relaxation factor for the tested band. This shall fulfil the requirements in Table 6.2.1.1.3.3-5. |  |  |  |  |  |  |  |

## <<< END OF CHANGES 1>>>

## <<< START OF CHANGES 2>>>

#### 6.2.1.2_1	UE maximum output power - Spherical coverage (Rel16 and forward)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

-	Same as in 6.2.1.2

6.2.1.2_1.1	Test purpose

Same as 6.2.1.2.1.

6.2.1.2.2	Test applicability

This test case applies to all types of NR UE release 16 and forward supporting either SSB-based or CSI-RS based enhanced beam correspondence without UL beam sweeping.

6.2.1.2_1.3	Minimum conformance requirements

Same as 6.2.1.1_1.3

6.2.1.2_1.4	Test description

6.2.1.2_1.4.1	Initial conditions

Same as 6.2.1.2.4.1

6.2.1.2.4.2	Test procedure

The following cases are tested depending on UE capability:

- Test procedure if beamCorrespondenceWithoutUL-BeamSweeping is NOT supported and beamCorrespondenceSSB-based-r16 is supported:

1.1	Same as 6.2.1.2.4.2 with the exception that measurements shall be carried out using only side conditions defined in Table 6.6.1.3.3.1.1-1

1.2	End test procedure

2.	Test procedure if beamCorrespondenceWithoutUL-BeamSweeping is NOT supported, and beamCorrespondenceCSI-RS-based-r16 is supported

2.1	Same as 6.2.1.2.4.2 with the exception that measurements shall be carried out using only side conditions defined in Table 6.6.2.3.3-1

2.2	End test procedure .

3.	Test procedure if beamCorrespondenceWithoutUL-BeamSweeping is supported and beamCorrespondenceSSB- based-r16 is supported:

3.1	Same as 6.2.1.2.4.2 with the exception that measurements shall be carried out using only side conditions defined in Table 6.6.1.3.3.1.1-1

3.2	End test procedure

4.	Test procedure if beamCorrespondenceWithoutUL-BeamSweeping is supported and beamCorrespondenceCSI-RS-based-r16 is supported:

4.1	Same as 6.2.1.2.4.2 with the exception that step 7 is skipped and measurements shall be carried out using only side conditions defined in Table 6.6.2.3.3-1

4.2	End test procedure

6.2.1.2_1.4.3	Message contents

Same as 6.2.1.1_4.3 and 6.6.1.4.3.

6.2.1.2.5	Test requirement

Same as 6.2.1.2_1,5 except Table 6.2.1.2_1.5-1 below.

Table 6.2.1.2_1.5-1: UE spherical coverage for power class 3 (Rel-16 and forward)

| ID | FR2 bands/set | Test requirement (dB) (Note 1) |  |  |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | n257 | n258 | n259 | n260 | n261 |  |
| 1 | n257 | 11.5-TT-MBs,n |  |  |  |  |  |
| 2 | n258 |  | 11.5-TT-MBs,n |  |  |  |  |
| 3 | n259 |  |  | 5.8-TT-MBs,n |  |  |  |
| 4 | n260 |  |  |  | 8-TT-MBs,n |  |  |
| 5 | n261 |  |  |  |  | 11.5-TT-MBs,n |  |
| 6 | n257, n261 | 11.5-TT-MBs,n |  |  |  | 11.5-TT-MBs,n | MBs,n relaxation is 0 dB |
| 7 | n260, n261 |  |  |  | 8-TT-MBs,n | 11.5-TT-MBs,n | MBs,n relaxation is 0 dB for n260 |
| Note 1: MBs,n is the Multiband Relaxation factor for the tested band. This shall fulfil the requirements in Table 6.2.1.1.3.3-5. |  |  |  |  |  |  |  |

## <<< END OF CHANGES 2>>>

## <<< START OF CHANGES 3>>>

### 6.6.2	Enhanced Beam correspondence – EIRP

6.6.2.1	Test purpose

To verify the UE’s ability to select a suitable beam for UL transmission based on DL measurements with or without relying on UL beam sweeping within the range prescribed by the specified nominal maximum output power and beam correspondence tolerance.

6.6.2.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support either CSI-RS or SSB based beam correspondence and do not support beam correspondence without UL beam sweeping.

6.6.2.3	Minimum conformance requirements

6.6.2.3.1	Enhanced Beam correspondence for PC3

6.6.2.3.1.1	General Test Coverage Rules

The beam correspondence requirement for PC3 UEs consists of three components: UE minimum peak EIRP (as defined in clause 6.2.1.1.3.3), UE spherical coverage (as defined in clause 6.2.1.1.3.3), and beam correspondence tolerance (as defined in clause 6.6.1.3.3.2). The beam correspondence requirement is fulfilled if the UE satisfies one of the following conditions, depending on the UE’s beam correspondence capability IE beamCorrespondenceWithoutUL-BeamSweeping, as defined in TS 38.306 [26]:

If beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16 are supported, the UE shall meet the minimum peak EIRP requirement according to Table 6.2.1.1.3.3-1 and spherical coverage requirement according to Table 6.2.1.1.3.3-3 using the side conditions for SSB based enhanced beam correspondence requirements as defined in Clause 6.6.2.3.2.

-	If beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceCSI-RS-based-r16 are supported, the UE shall meet the minimum peak EIRP requirement according to Table 6.2.1.1.3.3-1 and spherical coverage requirement according to Table 6.2.1.1.3.3-3 using the side conditions for CSI-RS based enhanced beam correspondence requirements as defined in Clause 6.6.2.3.3.

If beamCorrespondenceWithoutUL-BeamSweeping is not present and beamCorrespondenceSSB-based-r16 is supported, the UE shall meet the minimum peak EIRP requirement according to Table 6.2.1.1.3.3-1 and spherical coverage requirement according to Table 6.2.1.1.3.3-3 with uplink beam sweeping using the side conditions for SSB based enhanced beam correspondence requirements as defined in Clause 6.6.2.3.2.  Such a UE shall meet the beam correspondence tolerance requirement defined in Clause 6.6.1.3.3.2 and shall support uplink beam management, as defined in TS 38.306 [14].

-	If beamCorrespondenceWithoutUL-BeamSweeping is not present and beamCorrespondenceCSI-RS-based-r16 is supported, the UE shall meet the minimum peak EIRP requirement according to Table 6.2.1.1.3.3-1 and spherical coverage requirement according to Table 6.2.1.1.3.3-3 with uplink beam sweeping using the side conditions for CSI-RS based enhanced beam correspondence requirements as defined in Clause 6.6.2.3.3. Such a UE shall meet the beam correspondence tolerance requirement defined in Clause 6.6.1.3.3.2 and shall support uplink beam management, as defined in TS 38.306 [14].

6.6.2.3.1.2	Applicability rules based on support for type of enhanced beam correspondence

For UEs supporting more than one type of beam correspondence, the following applicability rules apply:

-	If a UE meets enhanced beam correspondence requirements either based on SSB or based on CSI-RS, it is considered to have met the beam correspondence requirements based on SSB and CSI-RS.

-	For a UE supporting either SSB based or CSI-RS based enhanced beam correspondence, UE shall meet the supported enhanced beam correspondence requirements.

-	For a UE supporting both SSB based and CSI-RS based enhanced beam correspondence, the UE shall meet  both SSB based and CSI-RS based enhanced beam correspondence requirements and the following applicability rules for verifying the requirements apply:

-	The enhanced beam correspondence requirements shall be verified with the SSB based enhanced beam correspondence side conditions in clause 6.6.2.3.2

-	If the UE meets the SSB based enhanced beam correspondence requirements using the side conditions in clause 6.6.2.3.2 and meets the minimum peak EIRP requirement as defined in clause 6.2.1.1 using the CSI-RS based side conditions in clause 6.6.2.3.3, where the link direction is determined in the SSB based enhanced beam correspondence test, the UE is considered to have met both the SSB based and CSI-RS based enhanced beam correspondence requirements.

-	Otherwise, if UE does not meet the minimum peak EIRP requirement as defined in clause 6.2.1.3 using the CSI-RS based side conditions in clause 6.6.4.3.3, the enhanced beam correspondence requirements shall be further verified for the UE with the CSI-RS based enhanced beam correspondence side conditions in clause 6.6.2.3.3.

6.6.2.3.2	Side Condition for SSB based enhanced Beam Correspondence requirements

The beam correspondence requirements for beam correspondence based on SSB are only applied under the following side conditions:

-	The downlink reference signal SSB is provided and CSI-RS is not provided.

-	For beam correspondence, conditions for L1-RSRP measurements are fulfilled according to Table 6.6.1.3.3.1.1-1.

6.6.2.3.3	Side Condition for CSI-RS based enhanced Beam Correspondence requirements

The beam correspondence requirements for beam correspondence based on CSI-RS are only applied under the following side conditions:

-	The downlink reference signals including both SSB and CSI-RS are provided.

-	The reference measurement channel for beam correspondence are fulfilled according to the CSI-RS configuration in Annex A.3.

-	For beam correspondence, conditions for L1-RSRP measurements are fulfilled according to Table 6.6.1.3.3.1.1-2 and SSB signal is provided according to Table 6.6.2.3.3-1.

Table 6.6.2.3.3-1: SSB signal conditions for CSI-RS based beam correspondence requirements

| Angle of arrival | NR operating bands | Minimum SSB_RP Note 2 | SSB Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm / SCSSSB | dB |
|  |  | SCSSSB = 120 kHz |  |
| All angles Note 1 | n257 | -101,4 | ≥1 |
|  | n258 | -101,4 |  |
|  | n259 | -97,1 |  |
|  | n260 | -97,1 |  |
|  | n261 | -101,4 |  |
| NOTE 1: For UEs that support multiple FR2 bands, the Minimum SSB_RP values for all angles are increased by ΣMBS, the UE multi-band relaxation factor in dB specified in clause 6.2.1. NOTE 2: Values specified at the radiated requirements reference point to give minimum SSB Ês/Iot, with no applied noise. |  |  |  |

6.6.2.3.4	Normative reference

The normative reference for this requirement is TS 38.101-2 [3] clause 6.6.4

6.6.2.4	Test description

6.6.2.4.1	Initial conditions

Same as 6.6.1.4.1.

6.6.2.4.2	Test procedure

The following cases are tested depending on UE capability:

- Test procedure if beamCorrespondenceWithoutUL-BeamSweeping is NOT supported, uplink beam management and beamCorrespondenceSSB-based-r16 are supported:

1.1	Same as 6.6.1.4.2 with the exception that measurements shall be carried out using only side conditions defined in Table 6.6.1.3.3.1.1-1.

1.2	End test procedure.

2.	Test procedure if beamCorrespondenceWithoutUL-BeamSweeping is NOT supported, uplink beam management and beamCorrespondenceCSI-RS-based-r16 is supported

2.1	Same as 6.6.1.4.2 with the exception that measurements shall be carried out using only side conditions defined in Table 6.6.2.3.3-1.

2.2	End test procedure.

3.	Test procedure if beamCorrespondenceWithoutUL-BeamSweeping is NOT supported, uplink beam management, beamCorrespondenceCSI-RS-based-r16 and beamCorrespondenceSSB-based-r16 are supported

3.1	Same as 6.6.1.4.2 with the exception that measurements shall be carried out using only side conditions defined in Table 6.6.1.3.3.1.1-1.

3.2	If measurement performed in 6.2.1.1_1.4.2 Step 3.2 was fail, repeat test same as 6.6.1.4.2 with the exception that measurements shall be carried out using only side conditions defined in Table 6.6.2.3.3-1.

3.3	End test procedure.

6.6.2.4.3	Message contents

Same as the message contents in 6.6.1.4.3

6.6.2.5	Test requirements

The defined %-tile EIRP in measurement distribution derived within 6.6.2.4.2 (as per step 2.6 of clause 6.6.1.4.2) shall exceed the values specified in Table 6.2.1.2.5-3 in clause 6.2.1.2.5. The defined %-tile ΔEIRPBC in measurement distribution derived in step 2.7 shall not exceed the values specified in Table 6.6.1.5-1.

Table 6.6.1.5-1: UE beam correspondence tolerance for power class 3

| Operating band | Max ∆EIRPBC at 85th %-tile ∆EIRPBC CDF (dB) |
| --- | --- |
| n257 | 3.0 +TT |
| n258 | 3.0 +TT |
| n260 | 3.2 +TT |
| n261 | 3.0 +TT |
| NOTE: The requirements in this table are verified only under normal temperature conditions as defined in TS 38.508-1 [10] subclause 4.1.1. |  |

## <<< END OF CHANGES 3>>>

## <<< START OF CHANGES 4>>>

## F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty (MTSU) for transmitter tests

| Sub clause |  | Maximum Test System Uncertainty |  | Derivation of MTSU |  |
| --- | --- | --- | --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) |  | PC3 Minimum peak EIRP, Max EIRP Max Device size ≤ 30 cm ±4.89 dB (FR2a, NTC testing) ±5.09 dB (FR2b, NTC testing) ±5.17 dB (FR2a, ETC testing) ±5.37 dB (FR2b, ETC testing) |  | MTSU = 1.00 x MU (from Table B.3-1 in TR 38.903) |  |
| 6.2.1.1 UE maximum output power (TRP) |  | PC3 Max TRP Max Device size ≤ 30 cm ±4.42 dB (FR2a, NTC testing) ±4.62 dB (FR2b, NTC testing) ±4.70 dB (FR2a, ETC testing) ±4.90 dB (FR2b, ETC testing) |  | MTSU = 1.00 x MU (from Table B.3-2 in TR 38.903) |  |
| 6.2.1.1_1 UE maximum output power – EIRP (Rel-16 and forward) |  | Same as 6.2.1.1 |  |  |  |
| 6.2.1.2 UE maximum output power (Spherical coverage) |  | PC3 Max Device size ≤ 30 cm ±4.60 dB (FR2a) ±5.20 dB (FR2b) |  | MTSU = 1.00 x MU (from Table B.3-3 in TR 38.903) |  |
| 6.2.1.2_1 UE maximum output power – Spherical coverage (Rel16 and forward) |  | Same as 6.2.1.2 |  |  |  |
| 6.2.2 UE maximum output power reduction |  | PC3 Max Device size ≤ 30 cm ±4.92 dB (FR2a), NTC testing ±5.10 dB (FR2b), NTC testing ±5.20 dB (FR2a, ETC testing) ±5.38 dB (FR2b, ETC testing) |  | MTSU = 1.00 x MU (from Table B.4-1 in TR 38.903) |  |
| 6.2.3 UE maximum output power with additional requirements |  | Same as 6.2.2 |  |  |  |
| 6.2.4 Configured transmitted power |  | TBD |  |  |  |
| 6.2A.1.1.1 UE maximum output power - EIRP and TRP for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.1.2 UE maximum output power - EIRP and TRP for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.1.3 UE maximum output power - EIRP and TRP for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.1.4 UE maximum output power - EIRP and TRP for CA (5UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.1.5 UE maximum output power - EIRP and TRP for CA (6UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.1.6 UE maximum output power - EIRP and TRP for CA (7UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.1.7 UE maximum output power - EIRP and TRP for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.2.1 Spherical coverage for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.2.2 Spherical coverage for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.2.3 Spherical coverage for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.2.4 Spherical coverage for CA (5UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.2.5 Spherical coverage for CA (6UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.2.6 Spherical coverage for CA (7UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.2.7 Spherical coverage for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.2.1 UE maximum output power reduction for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  | MTSU = 1.00 x MU (from Table B.4-1 in TR 38.903) |  |
| 6.2A.2.2 UE maximum output power reduction for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.2.3 UE maximum output power reduction for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.2.4 UE maximum output power reduction for CA (5UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.2.5 UE maximum output power reduction for CA (6UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.2.6 UE maximum output power reduction for CA (7UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.2.7 UE maximum output power reduction for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2D.2 UE maximum output power reduction for UL MIMO |  | Same as 6.2.2 |  |  |  |
| 6.2D.3 UE maximum output power with additional requirements for UL MIMO |  | Same as 6.2.3 |  |  |  |
| 6.3.1 Minimum output power |  | PC1 Minimum peak EIRP, Max EIRP Max Device size ≤ 30 cm FFS (FR2a) FFS dB (FR2b) PC3 Minimum peak EIRP, Max EIRP Max Device size ≤ 30 cm ±6.15 dB (FR2a & FR2b, NTC testing ±6.41 dB (FR2a & FR2b, ETC testing)) |  | MTSU = 1.00 x MU (from Table B.7-1 in TR 38.903) |  |
| 6.3.2 Transmit OFF power |  | PC3: Max Device size ≤ 30 cm ±5.49 dB (FR2a) |  | MTSU = 1.00 x MU (from Table B.8-1 in TR 38.903) |  |
| 6.3.3.2 General ON/OFF time mask |  | ON power: Same as 6.2.1.1 (EIRP) for the respective power class OFF power: Same as 6.3.1 for the respective power class |  |  |  |
| 6.3.3.4 PRACH time mask |  | PC3: PRACH power: TBD OFF power: Max Device size ≤ 30 cm ±6.15 dB (FR2a & FR2b, NTC testing) ±6.41 dB (FR2a & FR2b, ETC testing) |  |  |  |
| 6.3.3.6 SRS time mask |  | TBD |  |  |  |
| 6.3.4.2 Absolute power tolerance |  | PC3 Max Device size ≤ 30 cm ±8.05 dB (FR2a & FR2b, NTC testing) ±8.42 dB (FR2a & FR2b, ETC testing) |  | MTSU = SQRT (UL Meas Uncer2 + DL Meas Uncer2) UL Meas Uncer: Same as 6.3.1 DL Meas Uncer: Same as 7.3.2 |  |
| 6.3.4.3 Relative power tolerance |  | PC3 Max Device size ≤ 30 cm [±1.7 dB] (FR2a) [±1.7 dB] (FR2b) |  | MTSU = 1.00 x MU (from Table B.9a.2.2-2 in TR 38.903) |  |
| 6.3.4.4 Aggregate power tolerance |  | PC3 Max Device size ≤ 30 cm ±1.4 dB (FR2a) ±1.4 dB (FR2b) |  | MTSU = 1.00 x MU (from Table B.9a.3.2-2 in TR 38.903) |  |
| 6.3A.1.1 Minimum output power for CA (2UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.2 Minimum output power for CA (3UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.3 Minimum output power for CA (4UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.4 Minimum output power for CA (5UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.5 Minimum output power for CA (6UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.6 Minimum output power for CA (7UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.7 Minimum output power for CA (8UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.3.1.1 General ON/OFF time mask for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.3.1.2 General ON/OFF time mask for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.3.1.3 General ON/OFF time mask for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.3.1.4 General ON/OFF time mask for CA (5UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.3.1.5 General ON/OFF time mask for CA (6UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.3.1.6 General ON/OFF time mask for CA (7UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.3.1.7 General ON/OFF time mask for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.4.2.1 Absolute power tolerance for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.2 Absolute power tolerance for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.3 Absolute power tolerance for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.4 Absolute power tolerance for CA (5UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.5 Absolute power tolerance for CA (6UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.6 Absolute power tolerance for CA (7UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.7 Absolute power tolerance for CA (8UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.3.1 Relative power tolerance for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz TBD Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.3.2 Relative power tolerance for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz TBD Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.3.3 Relative power tolerance for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz TBD Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.3.4 Relative power tolerance for CA (5UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.4.3.5 Relative power tolerance for CA (6UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.4.3.6 Relative power tolerance for CA (7UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.4.3.7 Relative power tolerance for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.4.4.1 Aggregate power tolerance for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.4 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.4.2 Aggregate power tolerance for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.4 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.4.3 Aggregate power tolerance for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.4 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.4.4 Aggregate power tolerance for CA (5UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.4.4.5 Aggregate power tolerance for CA (6UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.4.4.6 Aggregate power tolerance for CA (7UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.4.4.7 Aggregate power tolerance for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3D.3.1 General ON/OFF time mask for UL MIMO |  | PC3: OFF Power Max Device size ≤ 30cm ± 6.15 dB (FR2a) ± 6.15 dB (FR2b) ON Power Quiet Zone size ≤ 30cm TBD (FR2a) TBD (FR2b) |  | OFF Power MTSU = 1.00 x MU (from Table B.8-2-4 in TR 38.903) ON Power TBD |  |
| 6.3D.3.4 SRS time mask for UL MIMO |  | PC3: OFF Power Max Device size ≤ 30cm ± 6.15 dB (FR2a) ± 6.15 dB (FR2b) ON Power Quiet Zone size ≤ 30cm TBD (FR2a) TBD (FR2b) |  | OFF Power MTSU = 1.00 x MU (from Table B.8-2-4 in TR 38.903) ON Power TBD |  |
| 6.4.1 Frequency error |  | ± 0.01 ppm (NTC & ETC testing) |  | MTSU = 1.00 x MU (from B.10.1 and B.10.2 in TR 38.903) |  |
| 6.4.2.1 Error vector magnitude |  | PUSCH, PC3, FR2a: As defined in Table F.1.2-2. PUSCH, PC3, FR2b: As defined in Table F.1.2-3. Otherwise: TBD |  |  |  |
| 6.4.2.2 Carrier leakage |  | PC3 Max Device size ≤ 30 cm ±5.44 dB (FR2a) ±5.57 dB (FR2b) uplink absolute power measurement uncertainty: 6.15 dB (FR2a & FR2b, NTC testing) uplink relative power measurement uncertainty: 1.4 dB (FR2a & FR2b, NTC testing) |  | MTSU = 1.00 x MU (from Table B.11-1 in TR 38.903) |  |
| 6.4.2.3 In-band emissions |  | TBD |  |  |  |
| 6.4.2.4 EVM equalizer spectrum flatness |  | TBD |  |  |  |
| 6.4.2.5 EVM equalizer spectrum flatness for BPSK modulation |  | TBD |  |  |  |
| 6.4A.1.1 Frequency error for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.4A.1.2 Frequency error for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.4A.1.3 Frequency error for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.4A.1.4 Frequency error for CA (5UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.4A.1.5 Frequency error for CA (6UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.4A.1.6 Frequency error for CA (7UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.4A.1.7 Frequency error for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.4A.2.1.1 Error Vector magnitude for CA (2UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.2 Error Vector magnitude for CA (3UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.3 Error Vector magnitude for CA (4UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.4 Error Vector magnitude for CA (5UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.5 Error Vector magnitude for CA (6UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.6 Error Vector magnitude for CA (7UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.7 Error Vector magnitude for CA (8UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.1 Carrier leakage for CA (2UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.2 Carrier leakage for CA (3UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.3 Carrier leakage for CA (4UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.4 Carrier leakage for CA (5UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.5 Carrier leakage for CA (6UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.6 Carrier leakage for CA (7UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.7 Carrier leakage for CA (8UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.1 In-band emissions for CA (2UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.2 In-band emissions for CA (3UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.3 In-band emissions for CA (4UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.4 In-band emissions for CA (5UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.5 In-band emissions for CA (6UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.6 In-band emissions for CA (7UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.7 In-band emissions for CA (8UL CA) |  | TBD |  |  |  |
| 6.5.1 Occupied bandwidth |  | Max Device size ≤ 30cm PC3: FR2a: ±0.4 [%CBW] (BW 50MHz) ±0.4 [%CBW] (BW 100MHz) ±1.2 [%CBW] (BW 200MHz) ±1.2 [%CBW] (BW 400MHz) FR2b: ±0.4 [%CBW] (BW 50MHz) ±0.4 [%CBW] (BW 100MHz) ±1.3 [%CBW] (BW 200MHz) ±1.3 [%CBW] (BW 400MHz) FR2c: TBD |  |  |  |
| 6.5.2.1 Spectrum Emission Mask |  | Max Device size ≤ 30 cm ±4.94 dB (FR2a) ±5.32 dB (FR2b) |  | MTSU = 1.00 x MU (from Table B.16-1 in TR 38.903) |  |
| 6.5.2.3 Adjacent Channel Leakage Ratio |  | Max Device size ≤ 30cm FR2a, NTC & ETC testing: ±5.63 dB (BW ≤ 50MHz) ±6.09 dB (50MHz < BW ≤ 100MHz) ±6.09 dB (100MHz < BW ≤ 200MHz) ±6.09 dB (200MHz < BW ≤ 400MHz) FR2b, NTC & ETC testing: ±6.09 dB (BW ≤ 50MHz) ±6.09 dB (50MHz < BW ≤ 100MHz) ±6.09 dB (100MHz < BW ≤ 200MHz) ±6.09 dB (200MHz < BW ≤ 400MHz) |  | MTSU = 1.00 x MU (from Table B.17-1B in TR 38.903) |  |
| 6.5.3.1 Transmitter Spurious emissions |  | Max Device size ≤ 30 cm Maximum in-band BW ≤ 400MHz ±5.14 dB (6GHz ≤ f ≤ 12.75GHz) ±5.11 dB (12.75GHz < f ≤ 23.45GHz) ±5.41 dB (23.45GHz < f < 40.8GHz) ±7.42 dB (40.8GHz ≤ f ≤ 66GHz) ±7.72 dB (66GHz ≤ f ≤ 80GHz) |  | MTSU = 1.00 x MU (from Table B.18-1 in TR 38.903) |  |
| 6.5.3.2 Spurious emission band UE co-existence |  | Max Device size ≤ 30 cm Maximum in-band BW ≤ 400MHz Protected band n260, n261, n257: ±6.00 dB Protected frequency 23.6 GHz ≤ f ≤ 24.0 GHz:±6.00 dB Protected frequency 57 GHz ≤ f ≤ 66GHz: ±8.01 dB Protected frequency 36 GHz ≤ f ≤ 37GHz: ±6.00 dB |  | MTSU = 1.00 x MU (from Table B.18-1a in TR 38.903) |  |
| 6.5.3.3 Additional Spurious emission |  | Max Device size ≤ 30 cm Maximum in-band BW ≤ 400MHz ±5.14 dB (6GHz ≤ f ≤ 12.75GHz), NS_202 ±5.70 dB (12.75GHz < f ≤ 23.45GHz), NS_202 ±6.00 dB (23.45GHz < f < 40.8GHz), NS_202, NS_203 ±8.01 dB (40.8GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band), NS_202 |  | MTSU = 1.00 x MU (from Table B.18-1b in TR 38.903) |  |
| 6.5A.1.1 Occupied bandwidth for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Max Device size ≤ 30cm PC3: FR2a: TBD FR2b: TBD FR2c: TBD Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.1.2 Occupied bandwidth for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5A.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.1.3 Occupied bandwidth for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5A.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.1.4 Occupied bandwidth for CA (5UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5A.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.1.5 Occupied bandwidth for CA (6UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5A.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.1.6 Occupied bandwidth for CA (7UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5A.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.1.7 Occupied bandwidth for CA (8UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5A.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.2.1.1 Spectrum Emission Mask for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.2.1.2 Spectrum Emission Mask for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.2.1.3 Spectrum Emission Mask for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.2.1.4 Spectrum Emission Mask for CA (5UL CA) |  | TBD |  |  |  |
| 6.5A.2.1.5 Spectrum Emission Mask for CA (6UL CA) |  | TBD |  |  |  |
| 6.5A.2.1.6 Spectrum Emission Mask for CA (7UL CA) |  | TBD |  |  |  |
| 6.5A.2.1.7 Spectrum Emission Mask for CA (8UL CA) |  | TBD |  |  |  |
| 6.5A.2.2.1 Adjacent channel leakage ratio for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  | MTSU = 1.00 x MU (from Table B.17-1B in TR 38.309) |  |
| 6.5A.2.2.2 Adjacent channel leakage ratio for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.2.2.3 Adjacent channel leakage ratio for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.2.2.4 Adjacent channel leakage ratio for CA (5UL CA) |  | TBD |  |  |  |
| 6.5A.2.2.5 Adjacent channel leakage ratio for CA (6UL CA) |  | TBD |  |  |  |
| 6.5A.2.2.6 Adjacent channel leakage ratio for CA (7UL CA) |  | TBD |  |  |  |
| 6.5A.2.2.7 Adjacent channel leakage ratio for CA (8UL CA) |  | TBD |  |  |  |
| 6.5A.3.1.1 Transmitter Spurious emissions for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.3.1.2 Transmitter Spurious emissions for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.3.1.3 Transmitter Spurious emissions for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.3.1.4 Transmitter Spurious emissions for CA (5UL CA) |  | TBD |  |  |  |
| 6.5A.3.1.5 Transmitter Spurious emissions for CA (6UL CA) |  | TBD |  |  |  |
| 6.5A.3.1.6 Transmitter Spurious emissions for CA (7UL CA) |  | TBD |  |  |  |
| 6.5A.3.1.7 Transmitter Spurious emissions for CA (8UL CA) |  | TBD |  |  |  |
| 6.5A.3.2.1 Spurious emission band UE co-existence for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.3.2.2 Spurious emission band UE co-existence for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.3.2.3 Spurious emission band UE co-existence for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.3.2.4 Spurious emission band UE co-existence for CA (5UL CA) |  | TBD |  |  |  |
| 6.5A.3.2.5 Spurious emission band UE co-existence for CA (6UL CA) |  | TBD |  |  |  |
| 6.5A.3.2.6 Spurious emission band UE co-existence for CA (7UL CA) |  | TBD |  |  |  |
| 6.5A.3.2.7 Spurious emission band UE co-existence for CA (8UL CA) |  | TBD |  |  |  |
| 6.5A.3.3.1 Additional spurious emissions for CA (2UL CA) |  | TBD |  |  |  |
| 6.5A.3.3.2 Additional spurious emissions for CA (3UL CA) |  | TBD |  |  |  |
| 6.5A.3.3.3 Additional spurious emissions for CA (4UL CA) |  | TBD |  |  |  |
| 6.5A.3.3.4 Additional spurious emissions for CA (5UL CA) |  | TBD |  |  |  |
| 6.5A.3.3.5 Additional spurious emissions for CA (6UL CA) |  | TBD |  |  |  |
| 6.5A.3.3.6 Additional spurious emissions for CA (7UL CA) |  | TBD |  |  |  |
| 6.5A.3.3.7 Additional spurious emissions for CA (8UL CA) |  | TBD |  |  |  |
| 6.5D.2.1 Spectrum Emission Mask for UL MIMO |  | Same as 6.5.2.1 |  |  |  |
| 6.5D.2.2 Adjacent channel leakage ratio for UL MIMO |  | Same as 6.5.2.3 |  |  |  |
| 6.6.1 Beam correspondence – EIRP |  | PC3 Max Device size ≤ 30 cm 2.67 dB (FR2a, NTC testing) 3.80 dB (FR2b, NTC testing) |  | MTSU = 1.00 x MU (from Table B.18a.2-2 in TR 38.309) |  |
| 6.6.2 Enhanced Beam correspondence - EIRP |  | Same as 6.6.1 |  |  |  |
| NOTE 1: FR2a: 23.45GHz ≤ f ≤ 32.125GHz FR2b: 32.125GHz ≤ f ≤ 40.8GHz FR2c: 40.8GHz ≤ f ≤ 44.3GHz |  |  |  |  |  |

Table F.1.2-2: EVM Measurement Uncertainty (MU) for PUSCH, PC3, FR2a (23.45GHz <= f <= 32.125GHz)

| Test ID | Modulation | RB alloc. | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 2.78% | 3.85% | 5.44% | 7.69% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 3.10% | 4.16% | 5.88% | 8.99% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 2.78% | 3.85% | 5.44% | 7.69% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 3.10% | 4.16% | 5.88% | 8.99% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 3.31% | 4.50% | 6.36% | 11.21% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 3.60% | 4.73% | 6.68% | 11.21% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 4.26% | 5.96% | 8.41% | 15.84% |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 5.01% | 7.08% | 9.99% | 15.84% |
| 9 | CP-OFDM QPSK | Inner_Full | 3.60% | 4.73% | 6.68% | 11.89% |
| 10 | CP-OFDM QPSK | Outer_Full | 3.71% | 4.99% | 7.07% | 11.89% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 4.26% | 5.96% | 8.41% | 15.84% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 4.26% | 5.96% | 8.41% | 15.84% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 6.31% | 8.91% | 12.59% | 21.13% |
| 14 | CP-OFDM 64 QAM | Outer_Full | 6.31% | 8.91% | 12.59% | 21.13% |

Table F.1.2-3: EVM Measurement Uncertainty (MU) for PUSCH, PC3, FR2b (32.125GHz < f <= 40.8GHz)

| Test ID | Modulation | RB alloc. | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 3.56% | 4.83% | 6.91% | 9.65% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 4.15% | 5.69% | 8.11% | 12.50% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 3.56% | 4.83% | 6.91% | 9.65% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 4.15% | 5.69% | 8.11% | 12.50% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 4.54% | 6.26% | 8.91% | 18.06% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 5.09% | 7.19% | 10.15% | 18.06% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 6.78% | 9.58% | 13.54% | 25.50% |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 8.06% | 11.38% | 16.09% | 25.50% |
| 9 | CP-OFDM QPSK | Inner_Full | 5.09% | 7.19% | 10.15% | 19.13% |
| 10 | CP-OFDM QPSK | Outer_Full | 5.39% | 7.61% | 10.75% | 19.13% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 6.78% | 9.58% | 13.54% | 25.50% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 6.78% | 9.58% | 13.54% | 25.50% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 10.14% | 14.33% | 20.25% | 34.01% |
| 14 | CP-OFDM 64 QAM | Outer_Full | 10.14% | 14.33% | 20.25% | 34.01% |

## <<< END OF CHANGES 4>>>

## <<< START OF CHANGES 5>>>

## F.3.2	Measurement of transmitter

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Influence of noise is subtracted from MTSU before calculating the TT for lower limit Tx test cases.

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause |  | Test Tolerance (TT) |  | Formula for test requirement |  |
| --- | --- | --- | --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) |  | PC3 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 2.87 dB (FR2a, NTC) 2.87 dB (FR2b, NTC) 3.04 dB (FR2a, ETC) 3.04 dB (FR2b, ETC) Max EIRP 0 dB |  | Minimum peak EIRP TT = 0.60 x (MTSUIFF - 0.1) (FR2a) TT = 0.60 x (MTSUIFF - 0.3) (FR2b) |  |
| 6.2.1.1 UE maximum output power (TRP) |  | PC3 Max TRP IFF (Max Device size ≤ 30 cm) 2.65 dB (FR2a, NTC) 2.77 dB (FR2b, NTC) 2.82 dB (FR2a, ETC) 2.94 dB (FR2b, ETC) |  | Max TRP TT = 0.60 x MTSUIFF |  |
| 6.2.1.1_1 UE maximum output power – EIRP (Rel-16 and forward) |  | Same as 6.2.1.1 |  |  |  |
| 6.2.1.2 UE maximum output power (Spherical coverage) |  | PC1 TBD PC2 TBD PC3 IFF (Max Device size ≤ 30 cm) 2.58 dB (FR2a) 2.58 dB (FR2b) PC4 TBD |  | PC3 TT = 0.60 x (MTSUIFF - 0.3) (FR2a) TT = 0.60 x (MTSUIFF - 0.9) (FR2b) |  |
| 6.2.1.2_1 UE maximum output power – Spherical coverage (Rel16 and forward) |  | Same as 6.2.1.2 |  |  |  |
| 6.2.2 UE maximum output power reduction |  | PC3 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 3.11 dB (FR2a, NTC) 3.11 dB (FR2b, NTC) 3.30 dB (FR2a, ETC) 3.30 dB (FR2b, ETC) |  | Minimum peak EIRP TT = 0.65 x (MTSUIFF - 0.13) (FR2a) TT = 0.65 x (MTSUIFF - 0.31) (FR2b) |  |
| 6.2.3 UE maximum output power with additional requirements |  | Same as 6.2.2 |  |  |  |
| 6.2.4 Configured transmitted power |  | TBD |  |  |  |
| 6.2A.1.1.1 UE maximum output power - EIRP and TRP for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous TBD |  |  |  |
| 6.2A.1.1.2 UE maximum output power - EIRP and TRP for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous TBD |  |  |  |
| 6.2A.1.1.3 UE maximum output power - EIRP and TRP for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous TBD |  |  |  |
| 6.2A.1.1.4 UE maximum output power - EIRP and TRP for CA (5UL CA) |  | Intra-band contiguous CA, Intra-band non-contiguous CA TBD |  |  |  |
| 6.2A.1.1.5 UE maximum output power - EIRP and TRP for CA (6UL CA) |  | Intra-band contiguous CA, Intra-band non-contiguous CA TBD |  |  |  |
| 6.2A.1.1.6 UE maximum output power - EIRP and TRP for CA (7UL CA) |  | Intra-band contiguous CA, Intra-band non-contiguous CA TBD |  |  |  |
| 6.2A.1.1.7 UE maximum output power - EIRP and TRP for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.2.1 Spherical coverage for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.2.2 Spherical coverage for CA (3UL CA) |  | Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.2.3 Spherical coverage for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.2.4 Spherical coverage for CA (5UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.2.5 Spherical coverage for CA (6UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.2.6 Spherical coverage for CA (7UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.2.7 Spherical coverage for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.2.1 UE maximum output power reduction for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.2.2 UE maximum output power reduction for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.2.3 UE maximum output power reduction for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.2.4 UE maximum output power reduction for CA (5UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.2.5 UE maximum output power reduction for CA (6UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.2.6 UE maximum output power reduction for CA (7UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.2.7 UE maximum output power reduction for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2D.2 UE maximum output power reduction for UL MIMO |  | Same as 6.2.2 |  |  |  |
| 6.2D.3 UE maximum output power with additional requirements for UL MIMO |  | Same as 6.2.3 |  |  |  |
| 6.3.1 Minimum output power |  | PC3 Minimum EIRP IFF (Max Device size ≤ 30 cm) NTC 4.21 dB (FR2a 50 MHz) 2.52 dB (FR2a 100 MHz) 0.66 dB (FR2a 200 MHz) 0 dB (FR2a 400 MHz) 1.17 dB (FR2b 50 MHz) 0 dB (FR2b 100 MHz) 0 dB (FR2b 200 MHz) 0 dB (FR2b 400 MHz) ETC 4.37 dB (FR2a 50 MHz) 2.68 dB (FR2a 100 MHz) 0.82 dB (FR2a 200 MHz) 0 dB (FR2a 400 MHz) 1.33 dB (FR2b 50 MHz) 0 dB (FR2b 100 MHz) 0 dB (FR2b 200 MHz) 0 dB (FR2b 400 MHz) |  | Minimum EIRP TT = max(R, ΔSNRmr + 0.65 x (MTSUIFF – 1.0)) -R R: Relaxation needed to limit influence of TE noise to 1 dB (specified in clause 6.3.1.5) ΔSNRmr: Systematic offset due to noise when measuring at minimum requirement level (-13 dBm) FR2a 50 MHz: ΔSNRmr = 0.86 dB FR2a 100 MHz: ΔSNRmr = 1.57 dB FR2a 200 MHz: ΔSNRmr = 2.71 dB FR2a 400 MHz: ΔSNRmr = 4.35 dB FR2b 50 MHz: ΔSNRmr = 2.32 dB FR2b 100 MHz: ΔSNRmr = 3.82 dB FR2b 200 MHz: ΔSNRmr = 5.82 dB FR2b 400 MHz: ΔSNRmr = 8.21 dB |  |
| 6.3.2 Transmit OFF power |  | 0 dB |  |  |  |
| 6.3.3.2 General ON/OFF time mask |  | PC3: ON Power Same as 6.2.1.1 (EIRP) OFF Power 0 dB |  | ON Power: Same as 6.2.1.1 (EIRP) OFF Power: Same as 6.3.1 |  |
| 6.3.3.4 PRACH time mask |  | PC3: OFF Power Max Device size ≤ 30cm 0 dB ON Power Max Device size ≤ 30cm TBD (FR2a) TBD (FR2b)TBD |  | ON Power TBD |  |
| 6.3.4.2 Absolute power tolerance |  | PC3 Max Device size ≤ 30 cm ±8.05 dB (FR2a & FR2b, NTC testing) ±8.42 dB (FR2a & FR2b, ETC testing) |  | TT = MTSU |  |
| 6.3.4.3 Relative power tolerance |  | PC3 IFF (Max Device size ≤ 30 cm) [0.46 dB] (FR2a) [0.46 dB] (FR2b) |  | PC3 TT = 0.65 x (MTSUIFF – 1.0) (FR2a) TT = 0.65 x (MTSUIFF – 1.0) (FR2b) (assuming a power step ΔP = 1 dB) |  |
| 6.3.4.4 Aggregate power tolerance |  | PC3 IFF (Max Device size ≤ 30 cm) 0.26 dB (FR2a) 0.26 dB (FR2b) |  | PC3 TT = 0.65 x (MTSUIFF – 1.0) (FR2a) TT = 0.65 x (MTSUIFF – 1.0) (FR2b) (assuming a power step ΔP = 1 dB) |  |
| 6.3A.1.1 Minimum output power for CA (2UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.2 Minimum output power for CA (3UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.3 Minimum output power for CA (4UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.4 Minimum output power for CA (5UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.5 Minimum output power for CA (6UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.6 Minimum output power for CA (7UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.7 Minimum output power for CA (8UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.3.1.1 General ON/OFF time mask for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.3.1.2 General ON/OFF time mask for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.3.1.3 General ON/OFF time mask for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.3.1.4 General ON/OFF time mask for CA (5UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.3.1.5 General ON/OFF time mask for CA (6UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.3.1.6 General ON/OFF time mask for CA (7UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.3.1.7 General ON/OFF time mask for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3D.3.1 General ON/OFF time mask for UL MIMO |  | PC3: OFF Power Max Device size ≤ 30cm 0 dB ON Power Max Device size ≤ 30cm TBD (FR2a) TBD (FR2b) |  | ON Power TBD |  |
| 6.3D.3.4 SRS time mask for UL MIMO |  | PC3: OFF Power Max Device size ≤ 30cm 0 dB ON Power Max Device size ≤ 30cm TBD (FR2a) TBD (FR2b) |  | ON Power TBD |  |
| 6.3A.4.2.1 Absolute power tolerance for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.2 Absolute power tolerance for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.3 Absolute power tolerance for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.4 Absolute power tolerance for CA (5UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.5 Absolute power tolerance for CA (6UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.6 Absolute power tolerance for CA (7UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.7 Absolute power tolerance for CA (8UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.3.1 Relative power tolerance for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz TBD Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.3.2 Relative power tolerance for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz TBD Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.3.3 Relative power tolerance for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz TBD Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.3.4 Relative power tolerance for CA (5UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.4.3.5 Relative power tolerance for CA (6UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.4.3.6 Relative power tolerance for CA (7UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.4.3.7 Relative power tolerance for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.4.4.1 Aggregate power tolerance for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.4 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.4.2 Aggregate power tolerance for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.4 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.4.3 Aggregate power tolerance for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.4 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.4.4 Aggregate power tolerance for CA (5UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.4.4.5 Aggregate power tolerance for CA (6UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.4.4.6 Aggregate power tolerance for CA (7UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.4.4.7 Aggregate power tolerance for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.4.1 Frequency error |  | 0.005 ppm (NTC & ETC testing) |  | TT = 0.5 x MTSU |  |
| 6.4.2.1 Error vector magnitude |  | TBD |  | Minimum requirement + TT |  |
| 6.4.2.2 Carrier leakage |  | IFF (Max Device size ≤ 30 cm) FR2a: ±3.54 dB (BW ≤ 400MHz) FR2b: ±3.62 dB (BW ≤ 400MHz) |  | TT = 0.65 x MTSUIFF |  |
| 6.4.2.3 In-band emissions |  | TBD |  |  |  |
| 6.4.2.4 EVM equalizer spectrum flatness |  | TBD |  |  |  |
| 6.4.2.5 EVM equalizer spectrum flatness for BPSK modulation |  | TBD |  |  |  |
| 6.4A.1.1 Frequency error for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.4A.1.2 Frequency error for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.4A.1.3 Frequency error for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.4A.1.4 Frequency error for CA (5UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.4A.1.5 Frequency error for CA (6UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.4A.1.6 Frequency error for CA (7UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.4A.1.7 Frequency error for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.4A.2.1.1 Error Vector magnitude for CA (2UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.2 Error Vector magnitude for CA (3UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.3 Error Vector magnitude for CA (4UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.4 Error Vector magnitude for CA (5UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.5 Error Vector magnitude for CA (6UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.6 Error Vector magnitude for CA (7UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.7 Error Vector magnitude for CA (8UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.1 Carrier leakage for CA (2UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.2 Carrier leakage for CA (3UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.3 Carrier leakage for CA (4UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.4 Carrier leakage for CA (5UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.5 Carrier leakage for CA (6UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.6 Carrier leakage for CA (7UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.7 Carrier leakage for CA (8UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.1 In-band emissions for CA (2UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.2 In-band emissions for CA (3UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.3 In-band emissions for CA (4UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.4 In-band emissions for CA (5UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.5 In-band emissions for CA (6UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.6 In-band emissions for CA (7UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.7 In-band emissions for CA (8UL CA) |  | TBD |  |  |  |
| 6.5.1 Occupied bandwidth |  | 0 kHz |  | Minimum requirement + TT |  |
| 6.5.2.1 Spectrum Emission Mask |  | IFF (Max Device size ≤ 30 cm) 3.21 dB (FR2a) 3.46 dB (FR2b) |  | TT = 0.65 x MTSUIFF |  |
| 6.5.2.3 Adjacent Channel Leakage Ratio |  | Absolute requirement 0 dB Relative requirement IFF (Max Device size ≤ 30 cm) FR2a: ±4.66 dB (BW ≤ 50MHz) ±4.96 dB (50MHz < BW ≤ 100MHz) ±4.96 dB (100MHz < BW ≤ 200MHz) ±4.96 dB (200MHz < BW ≤ 400MHz) FR2b: ±4.96 dB (BW ≤ 50MHz) ±4.96 dB (50MHz < BW ≤ 100MHz) ±4.96 dB (100MHz < BW ≤ 200MHz) ±4.96 dB (200MHz < BW ≤ 400MHz) |  | TT = max(R, ΔSNRmr+0.65 x (MTSUIFF-1.0)) -R + TT due to metric change TT due to metric change : 1.0 dB R: Relaxation needed to limit influence of TE noise to 1 dB (specified in clause 6.5.2.3.5) ΔSNRmr: Systematic offset due to noise when measuring ACP at minimum requirement level |  |
| 6.5.3.1 Transmitter Spurious emissions |  | 0 dB |  | Minimum requirement + TT |  |
| 6.5.3.2 Spurious emission band UE co-existence |  | 0 dB |  | Minimum requirement + TT |  |
| 6.5.3.3 Additional spurious emission |  | 0 dB |  | Minimum requirement + TT |  |
| 6.5A.1.1 Occupied bandwidth for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.1.2 Occupied bandwidth for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.1.3 Occupied bandwidth for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.1.4 Occupied bandwidth for CA (5UL CA) |  | TBD |  |  |  |
| 6.5A.1.5 Occupied bandwidth for CA (6UL CA) |  | TBD |  |  |  |
| 6.5A.1.6 Occupied bandwidth for CA (7UL CA) |  | TBD |  |  |  |
| 6.5A.1.7 Occupied bandwidth for CA (8UL CA) |  | TBD |  |  |  |
| 6.5A.2.1.1 Spectrum Emission Mask for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.2.1.2 Spectrum Emission Mask for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.2.1.3 Spectrum Emission Mask for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.2.1.4 Spectrum Emission Mask for CA (5UL CA) |  | TBD |  |  |  |
| 6.5A.2.1.5 Spectrum Emission Mask for CA (6UL CA) |  | TBD |  |  |  |
| 6.5A.2.1.6 Spectrum Emission Mask for CA (7UL CA) |  | TBD |  |  |  |
| 6.5A.2.1.7 Spectrum Emission Mask for CA (8UL CA) |  | TBD |  |  |  |
| 6.5A.2.2.1 Adjacent channel leakage ratio for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  | TT = 0.65 x MTSUIFF + TT due to metric change TT due to metric change : 1.0 dB |  |
| 6.5A.2.2.2 Adjacent channel leakage ratio for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  | TT = 0.65 x MTSUIFF + TT due to metric change TT due to metric change : 1.0 dB |  |
| 6.5A.2.2.3 Adjacent channel leakage ratio for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  | TT = 0.65 x MTSUIFF + TT due to metric change TT due to metric change : 1.0 dB |  |
| 6.5A.2.2.4 Adjacent channel leakage ratio for CA (5UL CA) |  | TBD |  |  |  |
| 6.5A.2.2.5 Adjacent channel leakage ratio for CA (6UL CA) |  | TBD |  |  |  |
| 6.5A.2.2.6 Adjacent channel leakage ratio for CA (7UL CA) |  | TBD |  |  |  |
| 6.5A.2.2.7 Adjacent channel leakage ratio for CA (8UL CA) |  | TBD |  |  |  |
| 6.5A.3.1.1 Transmitter Spurious emissions for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.3.1.2 Transmitter Spurious emissions for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.3.1.3 Transmitter Spurious emissions for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5D.2.1 Spectrum Emission Mask for UL MIMO |  | Same as 6.5.2.1 |  |  |  |
| 6.5D.2.2 Adjacent channel leakage ratio for UL MIMO |  | Same as 6.5.2.3 |  |  |  |
| 6.6.1 Beam correspondence - EIRP |  | PC3 1.26 dB (FR2a, FR2b) |  | PC3 TT = 0.60 x (MTSUIFF - ΔSNRmr) ΔSNRmr: Systematic offset due to noise when measuring at minimum requirement level |  |
| 6.6.2 Enhanced Beam correspondence - EIRP |  | Same as 6.6.1 |  |  |  |
| NOTE 1: FR2a: 23.45GHz ≤ f ≤ 32.125GHz FR2b: 32.125GHz ≤ f ≤ 40.8GHz FR2c: 40.8GHz ≤ f ≤ 44.3GHz |  |  |  |  |  |

## <<< END OF CHANGES 5>>>