3GPP TSG-RAN WG5 Meeting #96-e	R5-224408r1

Electronic Meeting, 15th - 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-2 | CR | 0164 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of on-demand PosSIB followed by location information transfer in RRC_connected state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The feature of on-demand posSIB in RRC-connected state need to be tested. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added of nn-demand PosSIB followed by location information transfer in RRC_connected state |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The work item NR_pos-UEConTest will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 37.571-3 CR 0156 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Corrected the test steps and the message contents. |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

9.4.3	On-demand PosSIB followed by location information transfer / RRC_connected state

9.4.3.1	Test Purpose (TP)

(1)

with { a NAS signalling connection existing }

ensure that {

when { UE has no assistance data stored and receives a REQUEST LOCATION INFORMATION message }

then { the UE sends the DedicatedSIBRequest message indicating the requested posSIB }

}

(2)

with { a NAS signalling connection existing }

ensure that {

when { UE receives the positioning assistance data via posSIBs after sending the onDemandSIB-Request message }

then { the UE sends a PROVIDE LOCATION INFORMATION message containing a location estimate }

}

9.4.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 clauses 5.2.2.1, 5.2.2.3.5, 5.2.2.3.6 and TS 37.355 clause7.2. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.2.2.1]

The UE applies the SI acquisition procedure to acquire the AS, NAS- and positioning assistance data information. The procedure applies to UEs in RRC_IDLE, in RRC_INACTIVE and in RRC_CONNECTED.

[TS 38.331, clause 5.2.2.3.5]

The UE shall:

1>	if the UE is in RRC_CONNECTED with an active BWP not configured with common search space with the field searchSpaceOtherSystemInformation and the UE has not stored a valid version of a SIB or posSIB, in accordance with sub-clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with sub-clause 5.2.2.1:

2>	for the SI message(s) that, according to the si-SchedulingInfo or posSI-SchedulingInfo in the stored SIB1, contain at least one required SIB or requested posSIB:

3>	if onDemandSIB-Request is configured and timer T350 is not running:

4>	initiate transmission of the DedicatedSIBRequest message in accordance with 5.2.2.3.6;

4>	start timer T350 with the timer value set to the onDemandSIB-RequestProhibitTimer;

1>	else if the UE is in RRC_CONNECTED with an active BWP configured with common search space with the field searchSpaceOtherSystemInformation and the UE has not stored a valid version of a SIB or posSIB, in accordance with sub-clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with sub-clause 5.2.2.1:

…

2>	for the SI message(s) that, according to the posSI-SchedulingInfo in the stored SIB1, contain at least one requested posSIB and for which posSI-BroadcastStatus is set to broadcasting:

3>	acquire the SI message(s) as defined in sub-clause 5.2.2.3.2;

2>	for the SI message(s) that, according to the posSI-SchedulingInfo in the stored SIB1, contain at least one requested posSIB and for which posSI-BroadcastStatus is set to notBroadcasting:

3>	if onDemandSIB-Request is configured and timer T350 is not running:

4>	initiate transmission of the DedicatedSIBRequest message in accordance with 5.2.2.3.6;

4>	start timer T350 with the timer value set to the onDemandSIB-RequestProhibitTimer;

4>	acquire the requested SI message(s) corresponding to the requested posSIB(s) as defined in sub-clause 5.2.2.3.2.

[TS 38.331, clause 5.2.2.3.6]

The UE shall set the contents of DedicatedSIBRequest message as follows:

1>	if the procedure is triggered to request the required SIB(s):

2>	include requestedSIB-List in the onDemandSIB-RequestList to indicate the requested SIB(s);

1>	if the procedure is triggered to request the required posSIB(s):

2>	include requestedPosSIB-List in the onDemandSIB-RequestList to indicate the requested posSIB(s).

The UE shall submit the DedicatedSIBRequest message to lower layers for transmission.

[TS 37.355, clause 7.2]

The supported posSibType's are specified in Table 7.2-1. The GNSS Common and Generic Assistance Data IEs are defined in clause 6.5.2.2. The OTDOA Assistance Data IEs and NR DL-TDOA/DL-AoD Assistance Data IEs are defined in clause 7.4.2. The Barometric Assistance Data IEs are defined in clause 6.5.5.8. The TBS (based on MBS signals) Assistance Data IEs are defined in clause 6.5.4.8.

Table 7.2-1: Mapping of posSibType to assistanceDataElement

|  | posSibType | assistanceDataElement |
| --- | --- | --- |
| GNSS Common Assistance Data (clause 6.5.2.2) | posSibType1-1 | GNSS-ReferenceTime |
|  | posSibType1-2 | GNSS-ReferenceLocation |
|  | posSibType1-3 | GNSS-IonosphericModel |
|  | posSibType1-4 | GNSS-EarthOrientationParameters |
|  | posSibType1-5 | GNSS-RTK-ReferenceStationInfo |
|  | posSibType1-6 | GNSS-RTK-CommonObservationInfo |
|  | posSibType1-7 | GNSS-RTK-AuxiliaryStationData |
|  | posSibType1-8 | GNSS-SSR-CorrectionPoints |
| GNSS Generic Assistance Data (clause 6.5.2.2) | posSibType2-1 | GNSS-TimeModelList |
|  | posSibType2-2 | GNSS-DifferentialCorrections |
|  | posSibType2-3 | GNSS-NavigationModel |
|  | posSibType2-4 | GNSS-RealTimeIntegrity |
|  | posSibType2-5 | GNSS-DataBitAssistance |
|  | posSibType2-6 | GNSS-AcquisitionAssistance |
|  | posSibType2-7 | GNSS-Almanac |
|  | posSibType2-8 | GNSS-UTC-Model |
|  | posSibType2-9 | GNSS-AuxiliaryInformation |
|  | posSibType2-10 | BDS-DifferentialCorrections |
|  | posSibType2-11 | BDS-GridModelParameter |
|  | posSibType2-12 | GNSS-RTK-Observations |
|  | posSibType2-13 | GLO-RTK-BiasInformation |
|  | posSibType2-14 | GNSS-RTK-MAC-CorrectionDifferences |
|  | posSibType2-15 | GNSS-RTK-Residuals |
|  | posSibType2-16 | GNSS-RTK-FKP-Gradients |
|  | posSibType2-17 | GNSS-SSR-OrbitCorrections |
|  | posSibType2-18 | GNSS-SSR-ClockCorrections |
|  | posSibType2-19 | GNSS-SSR-CodeBias |
|  | posSibType2-20 | GNSS-SSR-URA |
|  | posSibType2-21 | GNSS-SSR-PhaseBias |
|  | posSibType2-22 | GNSS-SSR-STEC-Correction |
|  | posSibType2-23 | GNSS-SSR-GriddedCorrection |
|  | posSibType2-24 | NavIC-DifferentialCorrections |
|  | posSibType2-25 | NavIC-GridModelParameter |
| OTDOA Assistance Data (clause 7.4.2) | posSibType3-1 | OTDOA-UE-Assisted |
| Barometric Assistance Data (clause 6.5.5.8) | posSibType4-1 | Sensor-AssistanceDataList |
| TBS Assistance Data (clause 6.5.4.8) | posSibType5-1 | TBS-AssistanceDataList |
| NR DL-TDOA/DL-AoD Assistance Data (clauses 6.4.3, 7.4.2) | posSibType6-1 | NR-DL-PRS-AssistanceData |
|  | posSibType6-2 | NR-UEB-TRP-LocationData |
|  | posSibType6-3 | NR-UEB-TRP-RTD-Info |

9.4.3.3	Test description

9.4.3.3.1	Pre-test conditions

System Simulator:

-	Sub-test 23, sub-test 24 and sub-test 25: NR Cell 1 and system information combination NR-1 as defined in TS 38.508-1 [30] clause 4.4.3.1.2.

-	Satellite signals (sub-test case 25): as specified in 8.2.1.

-	MBS signals (Sub-test 23): as specified in 8.2.4.

-	Sub-test 20: NR Cell 1 and NR Cell 2 as specified in 8.2.10 and system information combination NR-2 as defined in TS 38.508-1 [30] clause 4.4.3.1.2.

-	Sub-test 21: NR Cell 1, NR Cell 2 and NR Cell 3 as specified in 8.2.11 and system information combination NR-4 as defined in TS 38.508-1 [30] clause 4.4.3.1.2..

UE:

The UE shall begin the test with no assistance data stored.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [30], subclause 4.4A on NR Cell 1.

-	Then the SS sends the RESET UE POSITIONING STORED INFORMATION message to the UE to clear the stored assistance data in the UE.

Related PICS/PIXIT Statements:

-

9.4.3.3.2	Test procedure sequence

This test case includes sub-test cases dependent on the positioning method(s) supported by the UE. Each sub-test case is identified by a Sub-Test Case Number as defined in Table 9.4.3.3.2-1 below:

Table 9.4.3.3.2-1: Sub-test case numbers

| Sub-Test Case Number | Supported Positioning Methods |
| --- | --- |
| 20 | UE supporting DL-AoD (Rel-16 onwards) |
| 21 | UE supporting DL-TDOA (Rel-16 onwards) |
| 23 | UE supporting MBS(2) (Rel-16 onwards) |
| 24 | UE supporting Sensor (Rel-16 onwards) |
| 25 | UE supporting GNSS(1) (Rel-15 onwards) |
| NOTE 1: The GNSS combination of GPS, GLONASS, Galileo, BDS supported by the UE |  |

Table 9.4.3.3.2-2: Test Configuration

| Test Configuration | Network Deployment Type | Test Implementation |
| --- | --- | --- |
| B | NG-RAN NR |  |

Table 9.4.3.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1a1 to 1a2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. |  |  |  |  |
| 1a1 | IF sub-test 20 or 21 THEN The SS sends an RRCReconfiguration message as in Table 8.3.1-1 to configure the measurement gap. | <-- | RRC RRCReconfiguration | - | - |
| 1a2 | The UE sends an RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete | - | - |
| 2 | The SS sends a LPP message of type Request Capabilities. | <-- | DLInformationTransfer (LPP REQUEST CAPABILITIES) | - | - |
| 3 | The UE sends a LPP message of type Provide Capabilities including the UE positioning capabilities. | --> | ULInformationTransfer (LPP PROVIDE CAPABILITIES) | - | - |
| 4 | IF the UE LPP message at step 3 includes an acknowledgment request THEN SS sends a LPP Acknowledgement response. | <-- | DLInformationTransfer (LPP ACKNOWLEDGEMENT) | - | - |
| 5 | The SS sends a LPP message of type Request Location Information including a request for location estimate and location measurements. | <-- | DLInformationTransfer (LPP REQUEST LOCATION INFORMATION) | - | - |
| 6 | Check: Does the UE send the DedicatedSIBRequest message indicating the requested posSIBs within 5 S? | --> | RRC: DedicatedSIBRequest | 1 | P |
| - | EXCEPTION: In parallel to events described in step 6 steps 7a and 7b may take place. |  |  |  |  |
| 7a | The UE sends the LPP message of type REQUEST ASSISTANCE DATA message | --> | ULInformationTransfer (LPP REQUEST ASSISTANCE DATA) |  |  |
| 7b | The SS does not respond to the LPP REQUEST ASSISTANCE DATA message. |  |  |  |  |
| 8 | The requested posSIBs that includes the needed posSystemInformation are broadcasted. | <-- | RRC: SYSTEM INFORMATION (BCCH) |  |  |
| 9 | Check: Does the UE send an LPP message of type Provide Location Information including location estimate? | --> | ULInformationTransfer (LPP PROVIDE LOCATION INFORMATION) | 2 | P |
| 10 | IF the UE LPP message at step 9 includes an acknowledgment request THEN SS sends a LPP Acknowledgement response. | <-- | DLInformationTransfer (LPP ACKNOWLEDGEMENT) | - | - |

9.4.3.3.3	Specific message contents

Table 9.4.3.3.3-1: RESET UE POSITIONING STORED INFORMATION (Preamble)

| Derivation Path: 38.509 clause 6.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE Positioning Technology |  | Sub-test 20: 0 0 0 0 1 0 0 0 Sub-test 21: 0 0 0 0 0 1 1 1 Sub-test 23: 0 0 0 0 0 0 1 0 Sub-test 24: 0 0 0 0 0 1 0 1 Sub-test 25: 0 0 0 0 0 0 0 0 | Sub-test 20: DL-AoD Sub-test 21: DL-TDOA Sub-test 23: MBS Sub-test 24: Sensor Sub-test 25: GNSS |  |

Table 9.4.3.3.3-2: SIB1 (Preamble)

Derivation Path: TS 38.508-1, Table 4.6.1-28 with condition posSIB.

Table 9.4.3.3.3-3: PosSI-SchedulingInfo (SIB1, Table 9.4.3.3.3-2)

| Derivation Path: TS 38.508-1, Table 4.6.2a-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PosSI-SchedulingInfo-r16 ::= SEQUENCE { |  |  |  |  |
| posSchedulingInfoList-r16 SEQUENCE (SIZE (1..maxSI-Message)) OF PosSchedulingInfo-r16 { |  | 2 entries | The size of PosSchedulingInfo-r16 is 1 for Sub-tests 20, 21, 23 and 24. If Sub-test 25 and UE supports X GNSS systems, the size of PosSchedulingInfo-r16 is (X+1) and the PosSchedulingInfo-r16 needs to be broadcasted X times, once for each different GNSS. |  |
| PosSchedulingInfo-r16[1] SEQUENCE { |  |  | entry 1 |  |
| offsetToSI-Used-r16 |  | Not present |  |  |
| posSI-Periodicity-r16 |  | rf32 |  |  |
| posSI-BroadcastStatus-r16 |  | notBroadcasting |  |  |
| posSIB-MappingInfo-r16 SEQUENCE (SIZE (1..maxSIB)) OF PosSIB-Type-r16 { |  | 3 entries for Sub-test 25, 1 entry for Sub-tests 20, 21, 23 and 24 | PosSibType-n = posSibType1-1, posSibType1-2, posSibType1-3 | Sub-test 25 |
|  |  |  | PosSibType-n = posSibType4-1 | Sub-test 24 |
|  |  |  | PosSibType-n = posSibType5-1 | Sub-test 23 |
|  |  |  | PosSibType-n = posSibType6-1 | Sub-test 20 or Sub-test 21 |
| PosSIB-Type-r16[n] SEQUENCE { |  |  | entry n |  |
| encrypted-r16 |  | Not present |  |  |
| gnss-id-r16 |  | Not present |  |  |
| sbas-id-r16 |  | Not present |  |  |
| posSibType-r16 |  | PosSibType-n | The posSibType-r16 is depending on the sub–test cases and entry number n |  |
| areaScope-r16 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| PosSchedulingInfo-r16[2] SEQUENCE { |  |  | entry 2 If the UE supports X GNSS systems, the PosSchedulingInfo-r16 needs to be broadcasted X times, once for each different GNSS. | Sub-test 25 |
| offsetToSI-Used-r16 |  | Not present |  |  |
| posSI-Periodicity-r16 |  | rf32 |  |  |
| posSI-BroadcastStatus-r16 |  | notBroadcasting |  |  |
| posSIB-MappingInfo-r16 SEQUENCE (SIZE (1..maxSIB)) OF PosSIB-Type-r16 { |  | 6 entries for Sub-test 25 | posSibType2-1, posSibType2-3, posSibType2-6, posSibType2-7, posSibType2-8, posSibType2-9 |  |
| PosSIB-Type-r16[m] SEQUENCE { |  |  | entry m |  |
| encrypted-r16 |  | Not present |  |  |
| gnss-id-r16 |  | Set depending on the GNSS supported by the UE |  |  |
| sbas-id-r16 |  | Not present |  |  |
| posSibType-r16 |  | PosSibType-m, | The posSibType-r16 is depending on the entry number m |  |
| areaScope-r16 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.4.3.3.3-4: RRCReconfiguration (Preamble, step 7 as defined in TS 38.508-1 [30] Table 4.5.4.2-3)

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| onDemandSIB-Request-r16 CHOICE SEQUENCE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| onDemandSIB-RequestProhibitTimer-r16 |  | s5 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.4.3.3.3-5: DLInformationTransfer (step 2, step 4, step 5 and step 10, Table 9.4.3.3.2-3)

| Derivation Path: 38.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DLInformationTransfer ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| dlInformationTransfer SEQUENCE { |  |  |  |  |
| dedicatedNAS-Message OCTET STRING |  | Set according to Table 9.4.3.3.3-6 | DL NAS TRANSPORT |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.4.3.3.3-6: DL NAS TRANSPORT (DLInformationTransfer, Table 9.4.3.3.3-4)

| Derivation Path: 24.501 Table 8.2.11.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended Protocol discriminator |  | 01111110 | 5GS mobility management messages |  |
| Security header type |  | 0000 | Plain 5GS NAS message |  |
| Spare half octet |  | 0000 | Downlink generic NAS transport |  |
| DL NAS TRANSPORT message identity |  | 01101000 | DL NAS transport |  |
| Payload container type |  | 0011 | LTE Positioning Protocol (LPP) message container |  |
| Spare half octet |  | 0000 |  |  |
| Payload container |  | Step 2: Set according to Table 8.4-2 | LPP Request Capabilities. |  |
|  |  | Step 5: Set according to Table 9.4.3.3.3-7 | LPP Request Location Information |  |
|  |  | Steps 4 and 10: Set according to Table 9.4.3.3.3-8 | LPP Acknowledgement |  |
| Additional information |  | Present | Routing Identifier/Correlation ID |  |

Table 9.4.3.3.3-7: LPP Request Location Information (DL NAS TRANSPORT, Table 9.4.3.3.3-5)

| Derivation Path: Table 8.4-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 8.4-3 with the following exceptions: |  |  |  |  |
| locationInformationType |  | locationMeasurementsPreferred |  |  |

Table 9.4.3.3.3-8: LPP Acknowledgement (DL NAS TRANSPORT, Table 9.4.3.3.3-5)

| Derivation Path: 37.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| transactionID |  | Not present |  |  |
| endTransaction |  | TRUE |  |  |
| sequenceNumber |  | Not present |  |  |
| acknowledgement SEQUENCE { |  |  |  |  |
| ackRequested |  | FALSE |  |  |
| ackIndicator |  | (0..255) | Contains the same value of the sequenceNumber field in step 3 or 9, Table 9.4.3.3.2- |  |
| } |  |  |  |  |
| lpp-MessageBody |  | Not present. |  |  |
| } |  |  |  |  |

Table 9.4.3.3.3-9: DedicatedSIBRequest (step 6, Table 9.4.3.3.2-3)

| Derivation Path: TS 38.508-1, Table 4.6.1-2A |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DedicatedSIBRequest-r16 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| dedicatedSIBRequest-r16 SEQUENCE { |  |  |  |  |
| onDemandSIB-RequestList-r16 SEQUENCE { |  |  |  |  |
| requestedSIB-List-r16 |  | Not checked |  |  |
| requestedPosSIB-List-r16 SEQUENCE (SIZE (1..maxOnDemandPosSIB-r16)) OF PosSIB-ReqInfo-r16 { |  | 1 entry for Sub-tests 20, 21, 23 and 24; 9 entries for Sub-test 25 | The size of PosSIB-ReqInfo-r16 is 1 for Sub-tests 20, 21, 23 and 24. If Sub-test 25 and UE supporting X GNSS systems, the size of PosSIB-ReqInfo-r16 is (6X+3). |  |
| PosSIB-ReqInfo-r16[n] SEQUENCE { |  |  | entry n |  |
| gnss-id-r16 |  | Not present |  | Sub-test 20, 21, 23, 24 and 25 posSibType1-x |
|  |  | Set depending on the GNSS supported by the UE |  | Sub-test 25 posSibType2-x |
| sbas-id-r16 |  | Not checked |  |  |
| posSibType-r16 |  | PosSibType-n | PosSibType-n = posSibType1-1, posSibType1-2, posSibType1-3, posSibType2-1, posSibType2-3, posSibType2-6, posSibType2-7, posSibType2-8, posSibType2-9 | Sub-test 25 |
|  |  |  | PosSibType-n = posSibType4-1 | Sub-test 24 |
|  |  |  | PosSibType-n = posSibType5-1 | Sub-test 23 |
|  |  |  | PosSibType-n = posSibType6-1 | Sub-test 20 or Sub-test 21 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.4.3.3.3-10: ULInformationTransfer (steps 3, 7a and 9, Table 9.4.3.3.2-3)

| Derivation Path: 38.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ULInformationTransfer ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ulInformationTransfer SEQUENCE { |  |  |  |  |
| dedicatedNAS-Message OCTET STRING |  | Set according to Table 9.4.3.3.3-11 | UL NAS TRANSPORT |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.4.3.3.3-11: UL NAS TRANSPORT (ULInformationTransfer, Table 9.4.3.3.3-10)

| Derivation Path: 24.501 Table 8.2.10.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended Protocol discriminator |  | 01111110 | 5GS mobility management messages |  |
| Security header type |  | 0000 | Plain 5GS NAS message |  |
| Spare half octet |  | 0000 |  |  |
| UL NAS TRANSPORT message identity |  | 01100111 | UL NAS TRANSPORT |  |
| Payload container type |  | 0011 | LTE Positioning Protocol (LPP) message container |  |
| Spare half octet |  | 0000 |  |  |
| Payload container |  | Step 3: Set according to Table 9.4.3.3.3-12 | LPP Provide Capabilities |  |
|  |  | Step 7a: Set according to Table 9.4.3.3.3-13 | LPP Request Assistance Data |  |
|  |  | Step 9: Set according to Table 9.4.3.3.3-14 | LPP Provide Location Information |  |
| Additional information |  | Present | The UE includes the Routing Identifier received in the Additional Information IE of the DOWNLINK GENERIC NAS TRANSPORT message (step 2 Table Table 9.4.3.3.2-3) |  |

Table 9.4.3.3.3-12: LPP Provide Capabilities (UL NAS TRANSPORT, Table 9.4.3.3.3-11)

| Derivation Path: 37.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| transactionID SEQUENCE { |  |  |  |  |
| initiator |  | locationServer |  |  |
| transactionNumber |  | (0..255) | Contains the same value as the corresponding field in the LPP Request Capabilities message in step 12, Table 9.4.1.3.2-3 |  |
| } |  |  |  |  |
| endTransaction |  | TRUE |  |  |
| sequenceNumber |  | (0..255) |  |  |
| acknowledgement SEQUENCE { |  | Present, or not present |  |  |
| ackRequested |  | TRUE |  |  |
| ackIndicator |  | Not present |  |  |
| } |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities-r9 SEQUENCE { |  |  |  |  |
| commonIEsProvideCapabilities |  | Dependent on UE capabilities | Rel-14 onwards |  |
| a-gnss-ProvideCapabilities |  | Dependent on UE capabilities |  |  |
| sensor-ProvideCapabilities-r13 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| tbs-ProvideCapabilities-r13 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-DL-AoD-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| nr-DL-TDOA-ProvideCapabilities-r16 |  | Dependent on UE capabilities | Rel-16 onwards |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.4.3.3.3-13: LPP Request Assistance Data (UL NAS TRANSPORT, Table 9.4.3.3.3-11)

| Derivation Path: 37.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| transactionID SEQUENCE { |  |  |  |  |
| initiator |  | targetDevice |  |  |
| transactionNumber |  | (0..255) |  |  |
| } |  |  |  |  |
| endTransaction |  | FALSE |  |  |
| sequenceNumber |  | (0..255) |  |  |
| acknowledgement SEQUENCE { |  | Present or not present |  |  |
| ackRequested |  | TRUE |  |  |
| ackIndicator |  | Not present |  |  |
| } |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| requestAssistanceData SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| requestAssistanceData-r9 SEQUENCE { |  |  |  |  |
| commonIEsRequestAssistanceData |  | Present or not present |  |  |
| a-gnss-RequestAssistanceData |  | Present for sub-test 25 | Rel-15 onwards |  |
| otdoa-RequestAssistanceData |  | Not present |  |  |
| epdu-RequestAssistanceData |  | Not present |  |  |
| sensor-RequestAssistanceData-r14 |  | Present for sub-test 24 | Rel-16 onwards |  |
| tbs-RequestAssistanceData-r14 |  | Present for sub-test 23 | Rel-16 onwards |  |
| wlan-RequestAssistanceData-r14 |  | Not present |  |  |
| nr-Multi-RTT-RequestAssistanceData-r16 |  | Not present |  |  |
| nr-DL-AoD-RequestAssistanceData-r16 |  | Present for sub-test 20 | Rel-16 onwards |  |
| nr-DL-TDOA-RequestAssistanceData-r16 |  | Present for sub-test 21 | Rel-16 onwards |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.4.3.3.3-14: LPP Provide Location Information (UL NAS TRANSPORT, Table 9.4.3.3.3-11)

| Derivation Path: 37.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| transactionID SEQUENCE { |  |  |  |  |
| initiator |  | locationServer |  |  |
| transactionNumber |  | (0..255) | Contains the same value as the corresponding field in LPP Request Location Information message in step 15, Table 9.4.1.3.2-3 |  |
| } |  |  |  |  |
| endTransaction |  | TRUE |  |  |
| sequenceNumber |  | (0..255) |  |  |
| acknowledgement SEQUENCE { |  | present, or not present |  |  |
| ackRequested |  | TRUE |  |  |
| ackIndicator |  | Not present |  |  |
| } |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideLocationInformation-r9 SEQUENCE { |  |  |  |  |
| commonIEsProvideLocationInformation |  | May be present. Any value acceptable |  |  |
| a-gnss-ProvideLocationInformation SEQUENCE { |  | Present for sub-test 25 |  |  |
| gnss-SignalMeasurementInformation |  | Present. Any value acceptable |  |  |
| gnss-LocationInformation |  | May be present |  |  |
| gnss-Error |  | Not present |  |  |
| } |  |  |  |  |
| sensor-ProvideLocationInformation-r13 SEQUENCE { |  | Present for sub-test 24 | Rel-16 onwards |  |
| sensor-MeasurementInformation-r13 |  | Present. Any value acceptable |  |  |
| sensor-Error-r13 |  | Not present |  |  |
| sensor-MotionInformation-r15 |  | May be present |  |  |
| } |  |  |  |  |
| tbs-ProvideLocationInformation-r13 SEQUENCE { |  | Present for sub-test 23 | Rel-16 onwards |  |
| tbs-MeasurementInformation-r13 |  | Present. Any value acceptable |  |  |
| tbs-Error-r13 |  | Not present |  |  |
| } |  |  |  |  |
| nr-DL-AoD-ProvideLocationInformation-r16 SEQUENCE { |  | Present for sub-test 20 | Rel-16 onwards |  |
| nr-DL-AoD-SignalMeasurementInformation-r16 |  | Present. Any value acceptable |  |  |
| nr-DL-AoD-Error-r16 |  | Not present |  |  |
| } |  |  |  |  |
| nr-DL-TDOA-ProvideLocationInformation-r16 SEQUENCE { |  | Present for sub-test 21 | Rel-16 onwards |  |
| nr-DL-TDOA-SignalMeasurementInformation-r16 |  | Present. Any value acceptable |  |  |
| nr-DL-TDOA-Error-r16 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.4.3.3.3-15: SystemInformation – Sub-tests 20, 21, 23 and 24 (step 8, Table 9.4.3.3.2-3)

Derivation Path: TS 38.508-1, Table 4.6.1-29 with condition posSIB.

Table 9.4.3.3.3-16: SystemInformation – Sub-test 25, message 1 (step 8, Table 9.4.3.3.2-3)

Derivation Path: TS 38.508-1, Table 4.6.1-29 with condition posSIB and posSib1-x.

Table 9.4.3.3.3-17: SystemInformation – Sub-test 25, message 2 (step 8, Table 9.4.3.3.2-3)

Derivation Path: TS 38.508-1, Table 4.6.1-29 with condition posSIB and posSib2-x.

Table 9.4.3.3.3-18: PosSystemInformation-r16-IEs (step 8, Table 9.4.3.3.2-3)

| Derivation Path: TS 38.508-1, Table 4.6.2a-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PosSystemInformation-r16-IEs ::= SEQUENCE { |  |  |  |  |
| posSIB-TypeAndInfo-r16 SEQUENCE (SIZE (1..maxSIB)) OF CHOICE { |  | 3 entries |  | Sub-test 25 posSib1-x |
| posSib1-1-r16 |  | SIBpos as defined in Table 9.4.3.3.3-19 | entry 1 |  |
| posSib1-2-r16 |  | SIBpos as defined in Table 9.4.3.3.3-19 | entry 2 |  |
| posSib1-3-r16 |  | SIBpos as defined in Table 9.4.3.3.3-19 | entry 3 |  |
| posSIB-TypeAndInfo-r16 SEQUENCE (SIZE (1..maxSIB)) OF CHOICE { |  | 6 entries |  | Sub-test 25, posSib2-x |
| posSib2-1-r16 |  | SIBpos as defined in Table 9.4.3.3.3-19 | entry 1 |  |
| posSib2-3-r16 |  | SIBpos as defined in Table 9.4.3.3.3-19 | entry 2 |  |
| posSib2-6-r16 |  | SIBpos as defined in Table 9.4.3.3.3-19 | entry 3 |  |
| posSib2-7-r16 |  | SIBpos as defined in Table 9.4.3.3.3-19 | entry 4 |  |
| posSib2-8-r16 |  | SIBpos as defined in Table 9.4.3.3.3-19 | entry 5 |  |
| posSib2-9-r16 |  | SIBpos as defined in Table 9.4.3.3.3-19 | entry 6 |  |
| posSIB-TypeAndInfo-r16 SEQUENCE (SIZE (1..maxSIB)) OF CHOICE { |  | 1 entry |  | Sub-test 24 |
| posSib4-1-r16 |  | SIBpos as defined in Table 9.4.3.3.3-19 | entry 1 |  |
| posSIB-TypeAndInfo-r16 SEQUENCE (SIZE (1..maxSIB)) OF CHOICE { |  | 1 entry |  | Sub-test 23 |
| posSib5-1-r16 |  | SIBpos as defined in Table 9.4.3.3.3-19 | entry 1 |  |
| posSIB-TypeAndInfo-r16 SEQUENCE (SIZE (1..maxSIB)) OF CHOICE { |  | 1 entry |  | Sub-test 20, Sub-test 21 |
| posSib6-1-r16 |  | SIBpos as defined in Table 9.4.3.3.3-19 | entry 1 |  |
| } |  |  |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |

Table 9.4.3.3.3-19: SIBpos (PosSystemInformation-r16-IEs, Table 9.4.3.3.3-18)

| Derivation Path: TS 38.508-1, Table 4.6.2a-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIBpos-r16 ::= SEQUENCE { |  |  |  |  |
| assistanceDataSIB-Element-r16 |  | OCTET STRING containing AssistanceDataSIBelement |  |  |
| } |  |  |  |  |

Table 9.4.3.3.3-20: AssistanceDataSIBelement

| Derivation Path: TS 37.355, clause 7.4.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| AssistanceDataSIBelement-r15 ::= SEQUENCE { |  |  |  |  |
| valueTag-r15 |  | Not present |  |  |
| expirationTime-r15 |  | Not present |  |  |
| cipheringKeyData-r15 |  | Not present |  |  |
| segmentationInfo-r15 |  | Not present |  | Not present when the posSIB is not segmented |
| segmentationInfo-r15 SEQUENCE { |  |  |  | Present when the posSIB is segmented |
| segmentationOption-r15 |  | octet-string-seg |  |  |
| assistanceDataSegmentType-r15 |  | notLastSegment |  | Used when posSIB is segmented and the segment is not the last segment |
|  |  | lastSegment |  | Used when posSIB is not segmented, or posSIB is segmented and the segment is the last segment |
| assistanceDataSegmentNumber-r15 |  | index of the segmented posSIB |  |  |
| } |  |  |  |  |
| assistanceDataElement-r15 |  | OCTET STRING containing LPP IEs (GNSS-ReferenceTime, GNSS-ReferenceLocation, GNSS-IonosphericModel,GNSS-TimeModelList, GNSS-NavigationModel, GNSS-AcquisitionAssistance, GNSS-Almanac, GNSS-UTC-Model, GNSS-AuxiliaryInformation ) as defined in TS 37.571-5 [12] and mapped according to the content of posSibType-r16 in Table 9.4.1.3.3-3. |  | Sub-test 25 |
|  |  | OCTET STRING containing sensor-AssistanceDataList as defined in Table 8.4-1 |  | Sub-test 24 |
|  |  | OCTET STRING containing tbs-AssistanceDataList as defined in Table 8.4-1 |  | Sub-test 23 |
|  |  | OCTET STRING containing NR-DL-PRS-AssistanceData as defined in Table 8.4.1.6-1. |  | Sub-test 20, Sub-test 21 |
| } |  |  |  |  |