3GPP TSG-RAN5 Meeting #96-e	R5-224549r1

# Electronic Meeting, 15th – 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1906 | rev | - | Current version: | 17.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR SA FR1 RRM TC 6.5.6.1.1 - BWP switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In 6.5.6.1.1, test configurations assume that NR SpCell and NR SCell use the same duplex mode/SCS/CBW. This means that this TC is not applicable to FDD+TDD or 15kHz+30kHz CA BCs. It is neccessary to extend CA TC to support the PCC and SCC using different SCS combinations. Instead of defining new test configurations to traverse all duplex+SCS combinations, we suggest allowing the SpCC and SCC to choose their own test configuration independently to minimize impact to RAN4/RAN5 specs. To test a FDD+TDD CA BC, test configuration 1 and 2 are chosen for Cell 2 and Cell 3 respectively. Then test parameters for Cell 2 and Cell 3 shall be independently chosen as follows: Parameter; Unit; Cell2; Cell3. Frequency Range; FR1; FR1. Duplex mode; Config 1,4; FDD; FDD. Config 2,3,5,6; TDD; TDD. TDD configuration; Config 1,4; Not Applicable; Not Applicable. Config 2,5; TDDConf.1.1; TDDConf.1.1. Config 3,6; TDDConf.2.1; TDDConf.2.1. |
| Summary of change: |  | - Notes is added to test configuration tables of CA test cases to indicate that PCC/SCC can choose its test configuration independently. - Editor’s notes are added in involved test cases to note that TT analysis for SpCC+SCC = 15kHz + 30kHz / 30kHz + 15kHz SCS are still missing and need to be added in future. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.6.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: To correct copy and paste mistake in test configuration for NR SCell. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

6.5.6.1.1	NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX

Editor’s Note:	TT analysis for test configuration with SpCC SCS = 15kHz + SCC SCS = 30kHz or SpCC SCS = 30kHz + SCC SCS = 15kHz are still missing.

6.5.6.1.1.1	Test purpose

The purpose of this test is to verify the DL BWP switch delay requirement defined in TS 38.133 [6] clause 8.6, and interruption requirement on other active serving cell defined in TS 38.133 [6] clause 8.2.2.2.5.

6.5.6.1.1.2	Test applicability

This test applies to all types of NR UE release 15 onwards supporting BWP adaptation of at least 2 BWPs, DCI and timer-based active BWP switching delay Type1 or Type2 and 2DL CA.

6.5.6.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.6.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.6.1.1.

6.5.6.1.1.4	Test description

6.5.6.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.5.6.1.1.4.1-1.

Table 6.5.6.1.1.4.1-1: Supported test configurations for NR SA FR1-FR1 DCI-based DL active BWP switch with SCell in non-DRX

| Config | Description |  |  |
| --- | --- | --- | --- |
|  | NR PCell Note 3: |  | NR SCell Note 3: |
| 6.5.6.1.1-1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |  | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 6.5.6.1.1-2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |  | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 6.5.6.1.1-3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |  | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, Note 3: Test configuration for NR SpCC and test configuration for NR SCC shall be chosen independently. |  |  |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.6.1.1.4.1-2.

Table 6.5.6.1.1.4.1-2: Initial conditions for NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.6.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The general test parameter settings are set up according to Table 6.5.6.1.1.4.1-3.

2. Message contents are defined in clause 6.5.6.1.1.4.3.

3. The test scenario comprises of one NR PCell (Cell 1) and one NR SCell (Cell 2). Cell 1 and Cell 2 are configured according to Annex C.1.2 and C.1.3.

Table 6.5.6.1.1.4.1-3: General test parameters for DL BWP switch in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2 | Two NR radio channels are used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active SCell |  | Cell 2 | SCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | For both PCell and SCell |
| bwp-InactivityTimer | ms | 200 |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC. |
| Cell2 timing offset to cell1 | s | 3 | Time alignment error as specified in TS 38.104 [28] clause 6.5.3.1. |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 |  |
| T3 | s | 0.2 |  |

6.5.6.1.1.4.2	Test procedure

The test consists of 3 successive time periods, with durations of T1, T2, and T3, respectively.

PDCCHs indicating new transmissions shall be sent continuously on SCell (Cell 2) to ensure that the UE would have ACK/NACK sending except for the time duration when BWP is switching on Cell 2 and the time duration of T2.

PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1) to ensure that the UE will have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), and Cell 2 (SCell) on radio channel 2 (SCC).

-	UE is configured with 2 different UE-specific downlink bandwidth parts for SCell, BWP-1 and BWP-2, in Cell 2 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

-	UE is configured with 1 UE-specific downlink bandwidth parts the same as initial BWP for PCell, BWP-0 in Cell 1 before starting the test.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in SCell.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-0 in PCell.

-	UE is configured with a bwp-InactivityTimer timer value for SCell.

All cells have constant signal levels throughout the test.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	The SS shall configure SCell (Cell 2) on the SCC as per TS 38.508-1 [14] clause 7.5.1. Set the parameters according to Tables 6.5.6.1.1.4.1-3 and 6.5.6.1.1.5-1. Propagation conditions are set according to Annex C clauses C.2.2.

3. 	The SS shall transmit an RRCReconfiguration message releasing the dedicated configuration of the initialDownlinkBWP and the initialUplinkBWP of Cell 2. This message also configures 2 different UE-specific bandwidth parts for Cell 2, BWP-1 and BWP-2 and indicates BWP-1 as the active DL BWP using firstActiveDownlinkBWP-Id, according to Table 6.5.6.1.1.4.3-2.

4.	The SS shall send a DCI format 1_1 command for SCell DL BWP switch.

5.	The UE shall receive the DCI format 1_1 command in SCell’s slot # denoted i, then T1 starts and the UE switch its bandwidth part from BWP-1 to BWP-2:

a) If the UE starts to report valid ACK/NACK for SCell on PCell from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+k1)

and

b) If the UE starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+ Interruption length + k1)

and

c) If the number of consecutive missing ACK/NACK for PCell is no more than 1

Then, the number of successful subtests is increased by one. Otherwise, count a fail for the test, switch off/on the UE and go to step 1.

6.	If the UE sends valid ACK/NACK for the SCell on BWP-2, T2 starts. During T2, the SS shall not transmit DCI format for PDSCH reception on SCell.

7.	T3 starts from the first slot #j of the DL subframe immediately after the slot wherein bwp-InactivityTimer timer expires and the SS restarts to send DCI format for PDSCH reception on SCell. Then, the UE shall switch its bandwidth part from BWP-2 back to the default bandwidth part – BWP-1:

a) If the UE starts to report valid ACK/NACK for SCell on PCell from the first UL slot that occurs after the beginning of DL slot (j+TBWPswitchDelay+k1)

and

b) If the UE starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot (j+TBWPswitchDelay+ Interruption length + k1)

and

c) If the number of consecutive missing ACK/NACK for PCell is no more than 1.

Then, the number of successful subtests is increased by one. Otherwise, count a fail for the test, switch off/on the UE and go to step 1.

8.	Repeat steps 4-7 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

The SS verifies the DL BWP switch time in SCell by counting the slots from the time when the BWP switch command is received or bwp-InactivityTimer timer expires till an ACK/NACK is received.

The SS verifies that potential interruption to PCell is carried out in the correct time span by monitoring ACK/NACK sent in PCell during BWP switch of SCell, respectively.

Interruption length is defined in TS 38.133 [6] Table 8.2.2.2.5-1-1.

If all subtests pass, the test passes. If one subtest fails, the test fails.

6.5.6.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.5.6.1.1.4.3-1: Common Exception messages for NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |

Table 6.5.6.1.1.4.3-1A: RRCReconfiguration (Step 3)

| Derivation Path: TS 38.508-1 [14], Table 4.8.1-1B |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | Table 6.5.6.1.1.4.3-1B |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.5.6.1.1.4.3-1B: CellGroupConfig (Table 6.5.6.1.1.4.3-1A)

| Derivation Path: TS 38.508-1 [14], Table 4.8.1-2 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellConfigDedicated | ServingCellConfig | Table 6.5.6.1.1.4.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.6.1.1.4.3-2: ServingCellConfig (Table 6.5.6.1.1.4.3-1B)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| pdsch-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| radioLinkMonitoringConfig CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 2 entries |  |  |
| BWP-Downlink[1] | BWP-Downlink with condition BWP1 | entry 1 Table 6.5.6.1.1.4.3-3 |  |
| BWP-Downlink[2] | BWP-Downlink with condition BWP2 | entry 2 Table 6.5.6.1.1.4.3-3 |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | According to BWP-1 |  |
| bwp-InactivityTimer | ms200 |  |  |
| defaultDownlinkBWP-Id | 1 | According to BWP-1 |  |
| } |  |  |  |

Table 6.5.6.1.1.4.3-3: BWP-Downlink (Table 6.5.6.1.1.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Downlink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 | BWP-1 | BWP1 |
|  | 2 | BWP-2 | BWP2 |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters | RIV defined in TS 38.214 [9] that corresponds to DLBWP.1.1 |  | BWP1 |
|  | RIV defined in TS 38.214 [9] that corresponds to DLBWP.1.3 |  | BWP2 |
| } |  |  |  |
| } |  |  |  |

Table 6.5.6.1.1.4.3-4: Void

Table 6.5.6.1.1.4.3-5: Void

Table 6.5.6.1.1.4.3-6: PDSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-103 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-TimeDomainResourceAllocationList ::= SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation { | 4 entries |  |  |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 |  |
| k0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 72 | S=2, L=6 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[3] SEQUENCE { |  | entry 3 |  |
| k0 | TBWPswitchDelay | Defined in Table 6.5.6.1.0.1-1 | The DCI indicating BWP switch |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[4] SEQUENCE { |  | entry 4 |  |
| k0 | 1 |  | First DCI right after DCI-based BWP switch |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.6.1.1.4.3-7: Void

6.5.6.1.1.5	Test requirements

Tables 6.5.6.1.1.4.1-3 and 6.5.6.1.1.5-1 define the primary level settings including test tolerances.

Table 6.5.6.1.1.5-1: NR Cell specific test parameters for DL BWP switch in SA

| Parameter |  |  | Unit | Cell 1 |  | Cell2 |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  | FR1 |  | FR1 |
| Duplex mode |  | Config 1 |  | FDD |  | FDD |
|  |  | Config 2,3 |  | TDD |  | TDD |
| TDD configuration |  | Config 1 |  | Not Applicable |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |  | TDDConf.2.1 |
| BWchannel |  | Config 1,2 |  | Note 7 |  | Note 7 |
|  |  | Config 3 |  | Note 7 |  | Note 7 |
| BWoccupied |  | Config 1,2 | RB | 52 Note 5 |  | 52 Note 5 |
|  |  | Config 3 |  | 106 Note 6 |  | 106 Note 6 |
| Active BWP ID |  |  |  | 0 |  | 1, 2 |
| Initial DL BWP Configuration |  |  |  | DLBWP.0.2Note4 |  |  |
| Initial UL BWP Configuration |  |  |  | ULBWP.0.2Note4 | N.A. |  |
| Active DL BWP-0 Configuration |  |  |  | DLBWP.0.2Note4 | N.A. |  |
| Active DL BWP-1 Configuration |  |  |  | N.A. |  | DLBWP.1.1Note4 |
| Active DL BWP-2 Configuration |  |  |  | N.A. |  | DLBWP.1.3Note4 |
| Active UL BWP-0 Configuration |  |  |  | ULBWP.0.2Note4 |  | N.A. |
| Active UL BWP-1 Configuration |  |  |  | N.A. |  | N.A. |
| Active UL BWP-2 Configuration |  |  |  | N.A. |  | N.A. |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  | SR.1.1 FDD |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |
|  |  | Config 3 |  | SR.2.1 TDD |  | SR.2.1 TDD |
| RMSI CORESET parameters |  | Config 1 |  | CR.1.1 FDD |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |  | CR.2.1 TDD |
| Dedicated CORESET parameters |  | Config 1 |  | CCR.1.2 FDD |  | CCR.1.2 FDD |
|  |  | Config 2 |  | CCR.1.2 TDD |  | CCR.1.2 TDD |
|  |  | Config 3 |  | CCR.2.4 TDD |  | CCR.2.4 TDD |
| TRS Configuration |  | Config 1 |  | TRS.1.1 FDD |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |
| OCNG Patterns |  | Config 1,2 |  | OP.1 Note 5 |  |  |
|  |  | Config 3 |  | OP.1Note 6 |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x2 Low |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| NocNote 2 | Config 1,2 |  | dBm/SCS | -104 |  | -104 |
|  | Config 3 |  |  | -101 |  | -101 |
| NocNote 2 |  |  | dBm/15KHz | -104 |  | -104 |
| SS-RSRP Note 3 | Config 1,2 |  | dBm/SCS | -87 |  | -87 |
|  | Config 3 |  |  | -84 |  | -84 |
| Ês/Iot |  |  | dB | 17 |  | 17 |
| Ês/Noc |  |  | dB | 17 |  | 17 |
| IoNote3 |  | Config 1,2 | dBm/ 9.36MHz | -58.96 |  | -58.96 |
|  |  | Config 3 | dBm/ 38.16MHz | -52.86 |  | -52.86 |
| Propagation Condition |  |  |  | AWGN |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled within BWoccupied. Note 3 SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [8]. Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |  |  |  |

During T1, the UE shall start to send the ACK/NACK for SCell on PCell from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+k1).

During T3, the UE shall start to send the ACK/NACK for SCell on PCell from the first UL slot that occurs after the beginning of DL slot (j+TBWPswitchDelay+k1).

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [9].

Depending on UE capability bwp-SwitchingDelay [13], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in TS 38.133 [6] Table 8.6.2-1.

All of the above test requirements shall be fulfilled in order for the observed SCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

During T1 and T3, the start time of PCell interruption during SCell active BWP switch shall not happen outside the BWP switch delay.

The interruption of PCell shall not be longer than the interruption duration specified for active BWP switch in TS 38.133 [6] clause 8.2.2.2.5.

All of the above test requirements shall be fulfilled in order for the observed SCell active BWP switch interruption to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1, T3 if there are no uplink resources for reporting the ACK/NACK in the first UL slot that occurs after the beginning of DL slot (i+ TBWPswitchDelay+k1), (j+ TBWPswitchDelay+k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK/NACK.

<End of modified section 1>