3GPP TSG-RAN5 Meeting #96-e	R5-224556r1

# Electronic Meeting, 15th – 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2471 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NR SL message contents |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - “-r16” suffix is removed from IE names to align with guildlines in PRD 13. - SL-ResourcePoolID and SL-CBR used in Table 4.6.3-76A and Table 4.6.3-80A are not specified. - In table 4.6.6-7 sl-CSI-SchedulingRequestId-r16 is inserted in the wrong line. - IE type of sl-SyncConfigList-r16 in Table 4.6.6-10 and Table 4.6.6-11 is wrong. - Field sl-MeasObjectListCommon-r16 in Table 4.6.6-13 is still unspecified. - According to RAN1 agreements SL-RSRP measurement is performed based on PSSCH-RSRP. So it’s not correct to set field frequencyInfoSL-r16 in Table 4.6.6-16 as ACFCN of SL SSB. - IE type of SL-PSSCH-TxConfigList in Table 4.6.6-19 is incorrect. - Copy and paste mistake in Table 4.6.6-24 - To add missing fields in Table 4.6.6-25. - IE type of SL-SyncConfig in Table 4.6.6-31 is wrong. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - “-r16” suffix is removed from IE names. - SL-ResourcePoolID and SL-CBR in Table 4.6.3-76A and Table 4.6.3-80A are hardcoded. - Location of sl-CSI-SchedulingRequestId-r16 is corrected. - IE type of sl-SyncConfigList-r16 in Table 4.6.6-10 and Table 4.6.6-11 is corrected. - Field sl-MeasObjectListCommon-r16 in Table 4.6.6-13 is specified. - Field frequencyInfoSL-r16 in Table 4.6.6-16 is set as the ACFCN of DMRS used for SL-RSRP measurement - IE type of SL-PSSCH-TxConfigList in Table 4.6.6-19 is corrected. - Copy and paste mistake in Table 4.6.6-24 is corrected. - Field sl-CBR-PriorityTxConfigList-v1650 is added in Table 4.6.6-25. Field sl-ThresPSSCH-RSRP-List-r16 is renamed as sl-Thres-RSRP-List-r16 to align with 38.331 - IE type of SL-SyncConfig in Table 4.6.6-31 is corrected. Furthermore, sl-SSB-TimeAllocation2-r16 is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: Updated according to comments received. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

–	RRCReconfigurationSidelink

Table 4.6.1A-3: RRCReconfigurationSidelink

| Derivation Path: TS 38.331 [6], clause 6.6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfigurationSidelink ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier-r16 |  | RRC-TransactionIdentifier |  | RX |
|  |  | (0..3) |  | TX |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfigurationSidelink-r16 SEQUENCE { |  |  |  |  |
| slrb-ConfigToAddModList-r16 |  | Not present |  |  |
| slrb-ConfigToAddModList-r16 SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SLRB-Config-r16 { |  | 1 entry |  | SL_DRB |
| SLRB-Config-r16[1] SEQUENCE { |  |  | entry 1 |  |
| slrb-PC5-ConfigIndex-r16 |  | Not checked |  | TX |
|  |  | 1 |  | RX |
| sl-SDAP-ConfigPC5-r16 |  | Set according to parameter given in test case |  | TX |
| sl-SDAP-ConfigPC5-r16 SEQUENCE { |  |  |  | RX |
| sl-MappedQoS-FlowsToAddList-r16 SEQUENCE (SIZE (1.. maxNrofSL-QFIsPerDest-r16)) OF SL-PQFI-r16 { |  | 1 entry |  |  |
| SL-PQFI-r16[1] |  | 1 | entry 1 |  |
| } |  |  |  |  |
| sl-MappedQoS-FlowsToReleaseList-r16 |  | Not present |  |  |
| sl-SDAP-Header-r16 |  | absent |  |  |
| } |  |  |  |  |
| sl-PDCP-ConfigPC5-r16 |  | Set according to parameter given in test case |  | TX |
| sl-PDCP-ConfigPC5-r16 SEQUENCE { |  |  |  | RX |
| sl-PDCP-SN-Size-r16 |  | len12bits |  |  |
| sl-OutOfOrderDelivery-r16 |  | Not present |  |  |
| } |  |  |  |  |
| sl-RLC-ConfigPC5-r16 |  | Set according to parameter given in test case |  | TX |
| sl-RLC-ConfigPC5-r16 CHOICE { |  |  |  | RX |
| sl-AM-RLC-r16 SEQUENCE { |  |  |  |  |
| sl-SN-FieldLengthAM-r16 |  | size12 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-MAC-LogicalChannelConfigPC5-r16 |  | Set according to parameter given in test case |  | TX |
| sl-MAC-LogicalChannelConfigPC5-r16 SEQUENCE { |  |  |  | RX |
| sl-LogicalChannelIdentity-r16 |  | LogicalChannelIdentity with condition DRB1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| slrb-ConfigToReleaseList-r16 |  | Not present |  |  |
| sl-MeasConfig-r16 |  | Not present |  |  |
| sl-MeasConfig-r16 CHOICE { |  |  |  | SL_MEAS |
| setup |  | Set according to parameter given in test case |  | TX |
| setup SEQUENCE { |  |  |  | RX |
| sl-MeasObjectToRemoveList-r16 |  | Not present |  |  |
| sl-MeasObjectToAddModList-r16 |  | SL-MeasObjectList |  |  |
| sl-ReportConfigToRemoveList-r16 |  | Not present |  |  |
| sl-ReportConfigToAddModList-r16 |  | SL-ReportConfigList |  |  |
| sl-MeasIdToRemoveList-r16 |  | Not present |  |  |
| sl-MeasIdToAddModList-r16 |  | SL-MeasIdList |  |  |
| sl-QuantityConfig-r16 |  | SL-QuantityConfig |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-CSI-RS-Config-r16 |  | Not present |  |  |
| sl-CSI-RS-Config-r16 CHOICE { |  |  |  | SL_CSI |
| setup |  | Set according to parameter given in test case |  | TX |
| setup SEQUENCE { |  |  |  | RX |
| sl-CSI-RS-FreqAllocation-r16 CHOICE { |  |  |  |  |
| sl-OneAntennaPort-r16 |  | 000000000001 |  |  |
| } |  |  |  |  |
| sl-CSI-RS-FirstSymbol-r16 |  | 6 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-ResetConfig-r16 |  | Not present |  |  |
| sl-LatencyBoundCSI-Report-r16 |  | Not present |  |  |
|  |  | 160 |  | SL_CSI |
| lateNonCriticalExtension |  | Not checked |  | TX |
|  |  | Not present |  | RX |
| nonCriticalExtension |  | Not checked |  | TX |
|  |  | Not present |  | RX |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SL_DRB | To provide peer UE SL DRB related configuration via PC5 RRC |
| SL_MEAS | To provide peer UE SL RSRP measurement and reporting related configuration via PC5 RRC |
| SL_CSI | To provide peer UE SL CSI reporting related configuration via PC5 RRC |
| TX | UE transmits and NR-SS-UE receives. |
| RX | UE receives and NR-SS-UE transmits. |

<End of modified section 1>

<Start of modified section 2>

–	UECapabilityInformationSidelink

Table 4.6.1A-7: UECapabilityInformationSidelink

| Derivation Path: TS 38.331 [6], clause 6.6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UECapabilityInformationSidelink ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier-r16 |  | RRC-TransactionIdentifier |  | TX |
|  |  | Set to the same value as the rrc-TransactionIdentifier-r16 field in corresponding UECapabilityEnquirySidelink message |  | RX |
| criticalExtensions CHOICE { |  |  |  |  |
| ueCapabilityInformationSidelink-r16 SEQUENCE { |  |  |  |  |
| accessStratumReleaseSidelink-r16 |  | rel16 |  |  |
| pdcp-ParametersSidelink-r16 |  | Not checked |  | TX |
| pdcp-ParametersSidelink-r16 SEQUENCE { |  |  |  | RX |
| outOfOrderDeliverySidelink-r16 |  | supported |  |  |
| } |  |  |  |  |
| rlc-ParametersSidelink-r16 |  | Not checked |  | TX |
| rlc-ParametersSidelink-r16 SEQUENCE { |  |  |  | RX |
| am-WithLongSN-Sidelink-r16 |  | supported |  |  |
| um-WithLongSN-Sidelink-r16 |  | supported |  |  |
| } |  |  |  |  |
| supportedBandCombinationListSidelinkNR-r16 |  | Not checked |  | TX |
| supportedBandCombinationListSidelinkNR-r16 SEQUENCE (SIZE (1..maxBandComb)) OF BandCombinationParametersSidelinkNR-r16 { |  | 1 entry |  | RX |
| BandCombinationParametersSidelinkNR-r16[1] SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParametersSidelink-r16 { |  | 1 entry | entry 1 |  |
| BandParametersSidelink-r16[1] SEQUENCE { |  |  | entry 1 |  |
| freqBandSidelink-r16 |  | first FreqBandIndicatorNR contained in frequencyBandListFilterSidelink-r16 of corresponding UECapabilityEnquirySidelink message |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| supportedBandListSidelink-r16 |  | Not checked |  | TX |
| supportedBandListSidelink-r16 SEQUENCE (SIZE (1..maxBands)) OF BandSidelinkPC5-r16 { |  | 1 entry |  | RX |
| BandSidelinkPC5-r16[1] SEQUENCE { |  |  | entry 1 |  |
| freqBandSidelink-r16 |  | first FreqBandIndicatorNR contained in frequencyBandListFilterSidelink-r16 of corresponding UECapabilityEnquirySidelink message |  |  |
| sl-Reception-r16 SEQUENCE { |  |  |  |  |
| harq-RxProcessSidelink-r16 |  | n16 |  |  |
| pscch-RxSidelink-r16 |  | value1 |  |  |
| scs-CP-PatternRxSidelink-r16 CHOICE { |  |  |  |  |
| fr1-r16 SEQUENCE { |  |  |  |  |
| scs-15kHz-r16 |  | 0101011000000000 |  |  |
| scs-30kHz-r16 |  | 0101011000000000 |  |  |
| scs-60kHz-r16 |  | 0101011000000000 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| extendedCP-RxSidelink-r16 |  | supported |  |  |
| } |  |  |  |  |
| sl-Tx-256QAM-r16 |  | supported |  |  |
| lowSE-64QAM-MCS-TableSidelink-r16 |  | supported |  |  |
| csi-ReportSidelink-r16 SEQUENCE { |  |  |  |  |
| csi-RS-PortsSidelink-r16 |  | p2 |  |  |
| } |  |  |  |  |
| rankTwoReception-r16 |  | supported |  |  |
| sl-openLoopPC-RSRP-ReportSidelink-r16 |  | supported |  |  |
| sl-Rx-256QAM-r16 |  | supported |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| appliedFreqBandListFilter-r16 |  | Not checked |  | TX |
|  |  | Set to the same value as the frequencyBandListFilterSidelink-r16 of corresponding UECapabilityEnquirySidelink message |  | RX |
| lateNonCriticalExtension |  | Not checked |  | TX |
|  |  | Not present |  | RX |
| nonCriticalExtension |  | Not checked |  | TX |
|  |  | Not present |  | RX |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| TX | UE transmits and NR-SS-UE receives. |
| RX | UE receives and NR-SS-UE transmits. |

<End of modified section 1>

<Start of modified section 2>

–	SIB12

Table 4.6.2-14: SIB12

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB12-r16 ::= SEQUENCE { |  |  |  |
| segmentNumber-r16 | index of the segment contained in segmentContainer-r16 |  |  |
| segmentType-r16 | notLastSegment |  | NOT_LAST_SEG |
|  | lastSegment |  | LAST_SEG |
| segmentContainer-r16 | OCTET STRING (CONTAINING SIB12-IEs or segment of SIB12-IEs) | Note 1 |  |
| } |  |  |  |
| Note 1: Size of the SIB12-IEs-r16 or segment of SIB12-IEs-r16 contained in segmentContainer-r16 shall not exceed the maximum TBS size of the SI. For signalling test cases, the size is defined in TS 38.523-3[23] clause 7.3.3.2. |  |  |  |

| Condition | Explanation |
| --- | --- |
| NOT_LAST_SEG | Used when SIB12-IEs is segmented and the segment contained in segmentContainer-r16 is not the last segment |
| LAST_SEG | Used when SIB12-IEs is not segmented, or SIB12-IEs is segmented and the segment contained in segmentContainer-r16 is the last segment |

Table 4.6.2-14A: SIB12-IEs

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB12-IEs-r16 ::= SEQUENCE { |  |  |  |
| sl-ConfigCommonNR-r16 SEQUENCE { |  |  |  |
| sl-FreqInfoList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigCommon-r16 { | 1 entry |  |  |
| SL-FreqConfigCommon-r16[1] | SL-FreqConfigCommon | entry 1 |  |
| } |  |  |  |
| sl-UE-SelectedConfig-r16 | SL-UE-SelectedConfig |  |  |
| sl-NR-AnchorCarrierFreqList-r16 | Not present |  |  |
| sl-EUTRA-AnchorCarrierFreqList-r16 | Not present |  |  |
| sl-RadioBearerConfigList-r16 SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SL-RadioBearerConfig-r16 { | 1 entry |  |  |
| SL-RadioBearerConfig-r16[1] | SL-RadioBearerConfig | entry 1 |  |
| } |  |  |  |
| sl-RLC-BearerConfigList-r16 SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-BearerConfig-r16 { | 1 entry |  |  |
| SL-RLC-BearerConfig-r16[1] | SL-RLC-BearerConfig | entry 1 |  |
| } |  |  |  |
| sl-MeasConfigCommon-r16 | SL-MeasConfigCommon |  |  |
| sl-CSI-Acquisition-r16 | Not present |  |  |
| sl-OffsetDFN-r16 | Not present |  |  |
| t400-r16 | ms1000 |  |  |
| sl-MaxNumConsecutiveDTX-r16 | Not present |  |  |
| sl-SSB-PriorityNR-r16 | 1 |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

<End of modified section 2>

<Start of modified section 3>

–	MeasObjectNR-SL

Table 4.6.3-76A: MeasObjectNR-SL

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR-SL-r16 ::= SEQUENCE { |  |  |  |
| tx-PoolMeasToRemoveList-r16 | Not present |  |  |
| tx-PoolMeasToAddModList-r16 SEQUENCE (SIZE (1..maxNrofSL-PoolToMeasureNR-r16)) OF SL-ResourcePoolID-r16{ | 1 entry |  |  |
| SL-ResourcePoolID-r16[1] | 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section 3>

<Start of modified section 4>

–	MeasResultsSL

Table 4.6.3-80A: MeasResultsSL

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResultsSL-r16 ::= SEQUENCE { |  |  |  |
| measResultsListSL-r16 CHOICE { |  |  |  |
| measResultNR-SL-r16 SEQUENCE { |  |  |  |
| measResultListCBR-NR-r16 SEQUENCE (SIZE (1.. maxNrofSL-PoolToMeasureNR-r16)) OF MeasResultCBR-NR-r16 { | 1 entry |  |  |
| MeasResultCBR-NR-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-poolReportIdentity-r16 | 1 |  |  |
| sl-CBR-ResultsNR-r16 | (0..100) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

–	SL-BWP-Config

Table 4.6.6-1: SL-BWP-Config

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SL-BWP-Config-r16 ::= SEQUENCE { |  |  |  |  |
| sl-BWP-Id | BWP-Id |  |  |
| sl-BWP-Generic-r16 SEQUENCE { |  |  |  |
| sl-BWP-r16 | BWP |  |  |
| sl-LengthSymbols-r16 | sym14 | All symbols in slot are used for SL |  |
|  | sym12 | All symbols in slot are used for SL | EXTENDED |
| sl-StartSymbol-r16 | sym0 |  |  |
| sl-PSBCH-Config-r16 | Not present |  |  |
| sl-TxDirectCurrentLocation-r16 | Not present |  |  |
| } |  |  |  |
| sl-BWP-PoolConfig-r16 | SL-BWP-PoolConfig with condition SCHEDULING |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| EXTENDED | When cyclicPrefix is configured in sl-BWP-r16, i.e. extended CP is used |

–	SL-BWP-ConfigCommon

Table 4.6.6-2: SL-BWP-ConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-BWP-ConfigCommon-r16 ::= SEQUENCE { |  |  |  |  |
| sl-BWP-Generic-r16 SEQUENCE { |  |  |  |  |
| sl-BWP-r16 |  | BWP |  |  |
| sl-LengthSymbols-r16 |  | sym14 | All symbols in slot are used for SL |  |
|  |  | sym12 | All symbols in slot are used for SL | EXTENDED |
| sl-StartSymbol-r16 |  | sym0 |  |  |
| sl-PSBCH-Config-r16 |  | Not present |  |  |
| sl-TxDirectCurrentLocation-r16 |  | Not present |  |  |
| } |  |  |  |  |
| sl-BWP-PoolConfigCommon-r16 |  | SL-BWP-PoolConfigCommon with RXPOOL and SELECTED |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| EXTENDED | When cyclicPrefix is configured in sl-BWP-r16, i.e. extended CP is used |

–	SL-BWP-PoolConfig

Table 4.6.6-3: SL-BWP-PoolConfig

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment |  | Condition |
| SL-BWP-PoolConfig-r16 ::= SEQUENCE { |  |  |  |  |
| sl-RxPool-r16 | Not present |  |  |  |
| sl-TxPoolSelectedNormal-r16 | Not present |  |  |  |
| sl-TxPoolSelectedNormal-r16 SEQUENCE { |  |  | SELECTED |  |
| sl-PoolToReleaseList-r16 | Not present |  |  |  |
| sl-PoolToAddModList-r16 SEQUENCE (SIZE (1..maxNrofTXPool-r16)) OF SL-ResourcePoolConfig-r16 { | 1 entry |  |  |  |
| SL-ResourcePoolConfig-r16[1] SEQUENCE { |  | entry 1 |  |  |
| sl-ResourcePoolID-r16 | 1 | Index of the resource pool for normal case |  |  |
| sl-ResourcePool-r16 | SL-ResourcePool |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-TxPoolScheduling-r16 | Not present |  |  |  |
| sl-TxPoolScheduling-r16 SEQUENCE { |  |  | SCHEDULING |  |
| sl-PoolToReleaseList-r16 | Not present |  |  |  |
| sl-PoolToAddModList-r16 SEQUENCE (SIZE (1..maxNrofTXPool-r16)) OF SL-ResourcePoolConfig-r16 { | 1 entry |  |  |  |
| SL-ResourcePoolConfig-r16[1] SEQUENCE { |  | entry 1 |  |  |
| sl-ResourcePoolID-r16 | 1 | Index of the resource pool used for normal case |  |  |
| sl-ResourcePool-r16 | SL-ResourcePool |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-TxPoolExceptional-r16 | Not present |  |  |  |
| sl-TxPoolExceptional-r16 SEQUENCE { |  |  | EXCEPTIONAL |  |
| sl-ResourcePoolID-r16 | 2 | Index of the pool used during exceptional cases |  |  |
| sl-ResourcePool-r16 | SL-ResourcePool with condition EXCEPTIONAL |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCHEDULING | To configure Tx resource pool for Mode 1 SL transmission |
| SELECTED | To configure Tx resource pool for Mode 2 SL transmission |
| EXCEPTIONAL | To configure Tx resource pool for exceptional cases such as HO, re-establishment etc. |

–	SL-BWP-PoolConfigCommon

Table 4.6.6-4: SL-BWP-PoolConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-BWP-PoolConfigCommon-r16 ::= SEQUENCE { |  |  |  |  |
| sl-RxPool-r16 |  | Not present |  |  |
| sl-RxPool-r16 SEQUENCE (SIZE (1..maxNrofRXPool-r16)) OF SL-ResourcePool-r16 { |  | 1 entry |  | RXPOOL |
| SL-ResourcePool-r16[1] |  | SL-ResourcePool | entry 1 |  |
| } |  |  |  |  |
| sl-TxPoolSelectedNormal-r16 |  | Not present |  |  |
| sl-TxPoolSelectedNormal-r16 SEQUENCE (SIZE (1..maxNrofTXPool-r16)) OF SL-ResourcePoolConfig-r16 { |  | 1 entry |  | SELECTED |
| SL-ResourcePoolConfig-r16[1] SEQUENCE { |  |  | entry 1 |  |
| sl-ResourcePoolID-r16 |  | 1 | Index of the resource pool for normal case |  |
| sl-ResourcePool-r16 |  | SL-ResourcePool |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-TxPoolExceptional-r16 |  | Not present |  |  |
| sl-TxPoolExceptional-r16 SEQUENCE { |  |  |  | EXCEPTIONAL |
| sl-ResourcePoolID-r16 |  | 2 | Index of the resource pool used for normal case |  |
| sl-ResourcePool-r16 |  | SL-ResourcePool with condition EXCEPTIONAL |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| RXPOOL | To configure Rx resource pool |
| SELECTED | To configure Tx resource pool for Mode 2 SL transmission |
| EXCEPTIONAL | To configure Tx resource pool for exceptional cases such as HO, re-establishment etc. |

<End of modified section 3>

<Start of modified section 4>

–	SL-ConfigDedicatedNR

Table 4.6.6-7: SL-ConfigDedicatedNR

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-ConfigDedicatedNR-r16 ::= SEQUENCE { |  |  |  |  |
| sl-PHY-MAC-RLC-Config-r16 |  | Not present |  |  |
| sl-PHY-MAC-RLC-Config-r16 SEQUENCE { |  |  |  | SCHEDULING, SELECTED |
| sl-ScheduledConfig-r16 |  | Not present |  |  |
| sl-ScheduledConfig-r16 CHOICE { |  |  |  | SCHEDULING |
| setup |  | SL-ScheduledConfig |  |  |
| } |  |  |  |  |
| sl-UE-SelectedConfig-r16 |  | Not present |  |  |
| sl-UE-SelectedConfig-r16 CHOICE { |  |  |  | SELECTED |
| setup |  | SL-UE-SelectedConfig |  |  |
| } |  |  |  |  |
| sl-FreqInfoToReleaseList-r16 |  | Not present |  |  |
| sl-FreqInfoToAddModList-r16 |  | Not present |  |  |
| sl-FreqInfoToAddModList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfig-r16 { |  | 1 entry |  | SCHEDULING, SELECTED |
| SL-FreqConfig-r16[1] |  | SL-FreqConfig | entry 1 |  |
| } |  |  |  |  |
| sl-RLC-BearerToReleaseList-r16 |  | Not present |  |  |
| sl-RLC-BearerToAddModList-r16 |  | Not present |  |  |
| sl-RLC-BearerToAddModList-r16 SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-BearerConfig-r16 { |  | 1 entry |  | SL_DRB |
| SL-RLC-BearerConfig-r16[1] |  | SL-RLC-BearerConfig | entry 1 |  |
| } |  |  |  |  |
| sl-MaxNumConsecutiveDTX-r16 |  | Not present |  |  |  |  |  |
| sl-CSI-Acquisition-r16 |  | Not present |  |  |  |  |  |
|  |  | enabled |  |  |  | SL_CSI_REPORT |  |
| sl-CSI-SchedulingRequestId-r16 |  | Not present |  |  |
| sl-SSB-PriorityNR-r16 |  | 1 |  |  |
| networkControlledSyncTx-r16 |  | off | UE doesn't send SL SSB by default |  |
| } |  |  |  |  |
| sl-RadioBearerToReleaseList-r16 |  | Not present |  |  |
| sl-RadioBearerToAddModList-r16 |  | Not present |  |  |
| sl-RadioBearerToAddModList-r16 SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SL-RadioBearerConfig-r16 { |  | 1 entry |  | SL_DRB |
| SL-RadioBearerConfig-r16[1] |  | SL-RadioBearerConfig |  |  |
| } |  |  |  |  |
| sl-MeasConfigInfoToReleaseList-r16 |  | Not present |  |  |
| sl-MeasConfigInfoToAddModList-r16 |  | Not present |  |  |
| sl-MeasConfigInfoToAddModList-r16 SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-MeasConfigInfo-r16 { |  | 1 entry |  | SL_MEAS |
| SL-MeasConfigInfo-r16[1] |  | SL-MeasConfigInfo |  |  |
| } |  |  |  |  |
| t400-r16 |  | ms2000 |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCHEDULING | To configure UE performing mode 1 SL transmission |
| SELECTED | To configure UE performing mode 2 SL transmission |
| SL_DRB | To establish SL DRB |
| SL_MEAS | To configure UE performing measurements based on RS sent by peer UE |
| SL_CSI_REPORT | To configure UE performing SL CSI reporting |

<End of modified section 4>

<Start of modified section 5>

–	SL-DestinationIdentity

Table 4.6.6-9: SL-DestinationIdentity

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-DestinationIdentity-r16 |  | 24 bits destination ID chosen by SS |  |  |

–	SL-FreqConfig

Table 4.6.6-10: SL-FreqConfig

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-FreqConfig-r16 ::= SEQUENCE { |  |  |  |  |
| sl-Freq-Id-r16 |  | 1 |  |  |
| sl-SCS-SpecificCarrierList-r16 SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier { |  | 1 entry |  |  |
| SCS-SpecificCarrier[1] |  | SCS-SpecificCarrier with condition SL_PointA | entry 1 | ` |
| } |  |  |  |  |
| sl-AbsoluteFrequencyPointA-r16 |  | ARFCN-ValueNR with condition SL_PointA |  |  |
| sl-AbsoluteFrequencySSB-r16 |  | ARFCN-ValueNR with condition SL_SSB |  |  |
| frequencyShift7p5khzSL-r16 |  | Not present |  |  |
| valueN-r16 |  | 0 |  |  |
| sl-BWP-ToReleaseList-r16 |  | Not present |  |  |
| sl-BWP-ToAddModList-r16 SEQUENCE (SIZE (1..maxNrofSL-BWPs-r16)) OF SL-BWP-Config-r16 { |  | 1 entry |  |  |
| SL-BWP-Config-r16[1] |  | SL-BWP-Config | entry 1 |  |
| } |  |  |  |  |
| sl-SyncConfigList-r16 SEQUENCE (SIZE (1..maxSL-SyncConfig-r16)) OF SL-SyncConfig-r16 { |  | 1 entry |  |  |
| SL-SyncConfig-r16[1] |  | SL-SyncConfig | entry 1 |  |
| } |  |  |  |  |
| sl-SyncPriority-r16 |  | gnss |  |  |
| } |  |  |  |  |

–	SL-FreqConfigCommon

Table 4.6.6-11: SL-FreqConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-FreqConfigCommon-r16 ::= SEQUENCE { |  |  |  |  |
| sl-SCS-SpecificCarrierList-r16 SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier { |  | 1 entry |  |  |
| SCS-SpecificCarrier[1] |  | SCS-SpecificCarrier with condition SL_PointA | entry 1 |  |
| } |  |  |  |  |
| sl-AbsoluteFrequencyPointA-r16 |  | ARFCN-ValueNR with condition SL_PointA |  |  |
| sl-AbsoluteFrequencySSB-r16 |  | ARFCN-ValueNR with condition SL_SSB |  |  |
| frequencyShift7p5khzSL-r16 |  | Not present |  |  |
| valueN-r16 |  | 0 |  |  |
| sl-BWP-List-r16 SEQUENCE (SIZE (1..maxNrofSL-BWPs-r16)) OF SL-BWP-ConfigCommon-r16 { |  | 1 entry |  |  |
| SL-BWP-ConfigCommon-r16[1] |  | SL-BWP-ConfigCommon |  |  |
| } |  |  |  |  |
| sl-SyncPriority-r16 |  | gnss |  |  |
| sl-NbAsSync-r16 |  | Not present |  |  |
| sl-SyncConfigList-r16 SEQUENCE (SIZE (1..maxSL-SyncConfig-r16)) OF SL-SyncConfig-r16 |  | 1 entry |  |  |
| SL-SyncConfig-r16[1] |  | SL-SyncConfig | entry 1 |  |
| } |  |  |  |  |
| } |  |  |  |  |

<End of modified section 5>

<Start of modified section 6>

–	SL-MeasConfigCommon

Table 4.6.6-13: SL-MeasConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-MeasConfigCommon-r16 ::= SEQUENCE { |  |  |  |  |
| sl-MeasObjectListCommon-r16 |  | SL-MeasObjectList |  |  |
| sl-ReportConfigListCommon-r16 |  | SL-ReportConfigList with condition PERIODICAL |  |  |
| sl-MeasIdListCommon-r16 |  | SL-MeasIdList |  |  |
| sl-QuantityConfigCommon-r16 |  | SL-QuantityConfig |  |  |
| } |  |  |  |  |

–	SL-MeasConfigInfo

Table 4.6.6-14: SL-MeasConfigInfo

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-MeasConfigInfo-r16 ::= SEQUENCE { |  |  |  |  |
| sl-DestinationIndex-r16 |  | 0 | The first destination UE reported in sl-TxResourceReqList in SidelinkUEInformationNR |  |
| sl-MeasConfig-r16 SEQUENCE { |  |  |  |  |
| sl-MeasObjectToRemoveList-r16 |  | Not present |  |  |
| sl-MeasObjectToAddModList-r16 |  | SL-MeasObjectList |  |  |
| sl-ReportConfigToRemoveList-r16 |  | Not present |  |  |
| sl-ReportConfigToAddModList-r16 |  | SL-ReportConfigList with condition PERIODICAL |  |  |
| sl-MeasIdToRemoveList-r16 |  | Not present |  |  |
| sl-MeasIdToAddModList-r16 |  | SL-MeasIdList |  |  |
| sl-QuantityConfig-r16 |  | SL-QuantityConfig |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

–	SL-MeasIdList

Table 4.6.6-15: SL-MeasIdList

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-MeasIdList-r16 ::= SEQUENCE (SIZE (1..maxNrofSL-MeasId-r16)) OF SL-MeasIdInfo-r16 |  | 1 entry |  |  |
| SL-MeasIdInfo-r16[1] SEQUENCE { |  |  | entry 1 |  |
| sl-MeasId-r16 |  | 1 |  |  |
| sl-MeasObjectId-r16 |  | 1 |  |  |
| sl-ReportConfigId-r16 |  | 1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

–	SL-MeasObjectList

Table 4.6.6-16: SL-MeasObjectList

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-MeasObjectList-r16 ::= SEQUENCE (SIZE (1..maxNrofSL-ObjectId-r16)) OF SL-MeasObjectInfo-r16 { |  | 1 entry |  |  |
| SL-MeasObjectInfo-r16[1] SEQUENCE { |  |  | entry 1 |  |
| sl-MeasObjectId-r16 |  | 1 |  |  |
| sl-MeasObject-r16 SEQUENCE { |  |  |  |  |
| frequencyInfoSL-r16 |  | ARFCN-ValueNR of the DMRS used for SL-RSRP measurement Note: How to set frequencyInfoSL-r16 is FFS. |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

<End of modified section 6>

<Start of modified section 7>

–	SL-PSSCH-TxConfigList

Table 4.6.6-19: SL-PSSCH-TxConfigList

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment |  | Condition |
| SL-PSSCH-TxConfigList-r16 ::= SEQUENCE (SIZE (1..maxPSSCH-TxConfig-r16)) OF SL-PSSCH-TxConfig-r16 { |  | 1 entry |  |  |  |
| SL-PSSCH-TxConfig-r16[1] SEQUENCE { |  |  | entry 1 |  |  |
| sl-TypeTxSync-r16 |  |  | Not present | Applicable for all synchronization reference types |  |  |
| sl-ThresUE-Speed-r16 |  |  | kmph60 |  |  |  |
| sl-ParametersAboveThres-r16 SEQUENCE { |  |  |  |  |  |  |
| sl-MinMCS-PSSCH-r16 |  |  | 0 |  |  |  |
| sl-MaxMCS-PSSCH-r16 |  |  | 15 |  |  |  |
| sl-MinSubChannelNumPSSCH-r16 |  |  | 1 |  |  |  |
| sl-MaxSubchannelNumPSSCH-r16 |  |  | 27 |  |  |  |
| sl-MaxTxTransNumPSSCH-r16 |  |  | 4 |  |  |  |
| sl-MaxTxPower-r16 |  |  | Not present |  |  |  |
| } |  |  |  |  |  |  |
| sl-ParametersBelowThres-r16 SEQUENCE { |  |  |  |  |  |  |
| sl-MinMCS-PSSCH-r16 |  |  | 16 |  |  |  |
| sl-MaxMCS-PSSCH-r16 |  |  | 26 |  |  |  |
| sl-MinSubChannelNumPSSCH-r16 |  |  | 1 |  |  |  |
| sl-MaxSubchannelNumPSSCH-r16 |  |  | 27 |  |  |  |
| sl-MaxTxTransNumPSSCH-r16 |  |  | 4 |  |  |  |
| sl-MaxTxPower-r16 |  |  | Not present |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |

<End of modified section 7>

<Start of modified section 8>

–	SL-RadioBearerConfig

Table 4.6.6-23: SL-RadioBearerConfig

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-RadioBearerConfig-r16 ::= SEQUENCE { |  |  |  |  |
| slrb-Uu-ConfigIndex-r16 |  | SLRB-Uu-ConfigIndex |  |  |
| sl-SDAP-Config-r16 |  | SL-SDAP-Config |  |  |
| sl-PDCP-Config-r16 |  | SL-PDCP-Config |  |  |
| sl-TransRange-r16 |  | Not present |  |  |
| } |  |  |  |  |

–	SL-ReportConfigList

Table 4.6.6-24: SL-ReportConfigList (Thres)

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SL-ReportConfigList-r16 ::= SEQUENCE (SIZE (1..maxNrofSL-ReportConfigId-r16)) OF SL-ReportConfigInfo-r16 |  | 1 entry |  |  |  |  |  |
| SL-ReportConfigInfo-r16[1] SEQUENCE { |  |  |  | entry 1 |  |  |  |
| sl-ReportConfigId-r16 |  | 1 |  |  |  |  |
| sl-ReportConfig-r16 SEQUENCE { |  |  |  |  |  |  |
| sl-ReportType-r16 CHOICE { |  |  |  |  |  |  |
| sl-Periodical-r16 SEQUENCE { |  |  |  |  | PERIODICAL |  |
| sl-ReportInterval-r16 |  | ReportInterval |  |  |  |  |
| sl-ReportAmount-r16 |  | infinity |  |  |  |  |
| sl-ReportQuantity-r16 CHOICE { |  |  |  |  |  |  |
| sl-RSRP-r16 |  | true |  |  |  |  |
| } |  |  |  |  |  |  |
| sl-RS-Type-r16 |  | dmrs |  |  |  |  |
| } |  |  |  |  |  |  |
| sl-EventTriggered-r16 SEQUENCE { |  |  |  |  | EVENT_S1, EVENT_S2 |  |
| sl-EventId-r16 CHOICE { |  |  |  |  |  |  |
| eventS1-r16 SEQUENCE { |  |  |  |  | EVENT_S1 |  |
| s1-Threshold-r16 CHOICE { |  |  |  |  |  |  |
| sl-RSRP-r16 |  | Thres + 156 |  | Thres is the actual value of event S1 thresold (in dBm) |  |  |
| } |  |  |  |  |  |  |
| sl-ReportOnLeave-r16 |  | false |  |  |  |  |
| sl-Hysteresis-r16 |  | Hysteresis |  |  |  |  |
| sl-TimeToTrigger-r16 |  | TimeToTrigger |  |  |  |  |
| } |  |  |  |  |  |  |
| eventS2-r16 SEQUENCE { |  |  |  |  | EVENT_S2 |  |
| s2-Threshold-r16 CHOICE { |  |  |  |  |  |  |
| sl-RSRP-r16 |  | Thres + 156 |  | Thres is the actual value of event S2 thresold (in dBm) |  |  |
| } |  |  |  |  |  |  |
| sl-ReportOnLeave-r16 |  | false |  |  |  |  |
| sl-Hysteresis-r16 |  | Hysteresis |  |  |  |  |
| sl-TimeToTrigger-r16 |  | TimeToTrigger |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| sl-ReportInterval-r16 |  | ReportInterval |  |  |  |  |
| sl-ReportAmount-r16 |  | r2 |  |  |  |  |
| sl-ReportQuantity-r16 CHOICE { |  |  |  |  |  |  |
| sl-RSRP-r16 |  | true |  |  |  |  |
| } |  |  |  |  |  |  |
| sl-RS-Type-r16 |  | dmrs |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| EVENT_S1 | Configuration of event S1 triggered reprting |
| EVENT_S2 | Configuration of event S2 triggered reprting |
| PERIODICAL | Configuration of periodical reporting |

–	SL-ResourcePool

Table 4.6.6-25: SL-ResourcePool

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |  |
| sl-PSCCH-Config-r16 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| sl-TimeResourcePSCCH-r16 |  | n2 |  |  |
| sl-FreqResourcePSCCH-r16 |  | n10 |  |  |
| sl-DMRS-ScrambleID-r16 |  | 0 |  |  |
| sl-NumReservedBits-r16 |  | 2 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-PSSCH-Config-r16 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| sl-PSSCH-DMRS-TimePatternList-r16 SEQUENCE (SIZE (1..3)) OF INTEGER (2..4) { |  | 2 entries |  |  |
| INTEGER[1] |  | 2 | entry 1 |  |
| INTEGER[2] |  | 4 | entry 2 |  |
| } |  |  |  |  |
| sl-BetaOffsets2ndSCI-r16 SEQUENCE (SIZE (4)) OF SL-BetaOffsets-r16 { |  | 4 entries |  |  |
| SL-BetaOffsets-r16[k, k=1..4] |  | 1 | entry k |  |
| } |  |  |  |  |
| sl-Scaling-r16 |  | f1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-PSFCH-Config-r16 |  | Not present |  |  |
| sl-PSFCH-Config-r16 CHOICE { |  |  |  | SL_HARQ |
| setup SEQUENCE { |  |  |  |  |
| sl-PSFCH-Period-r16 |  | sl4 |  |  |
| sl-PSFCH-RB-Set-r16 |  | bitstring of length n, The leftmost p*floor(m/p)*floor(n/m) bits are set to "1" and the rest are set to "0" | Note 1, Note 2, Note 3 |  |
| sl-NumMuxCS-Pair-r16 |  | n2 |  |  |
| sl-MinTimeGapPSFCH-r16 |  | sl2 |  |  |
| sl-PSFCH-HopID-r16 |  | Not present | Default frequency hopping ID 0 is used |  |
| sl-PSFCH-CandidateResourceType-r16 |  | startSubCH |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-SyncAllowed-r16 SEQUENCE { |  |  |  |  |
| gnss-Sync-r16 |  | true |  |  |
| gnbEnb-Sync-r16 |  | true |  |  |
| ue-Sync-r16 |  | true |  |  |
| } |  |  |  |  |
| sl-SubchannelSize-r16 |  | n10 |  |  |
| dummy |  | Not present |  |  |
| sl-StartRB-Subchannel-r16 |  | 0 | start RB of BWP |  |
| sl-NumSubchannel-r16 |  | floor(n/m) | Note 1, Note 2 |  |
| sl-Additional-MCS-Table-r16 |  | Not present |  |  |
| sl-ThreshS-RSSI-CBR-r16 |  | 0 | actual value is -112 dBm |  |
| sl-TimeWindowSizeCBR-r16 |  | slot100 |  |  |
| sl-TimeWindowSizeCR-r16 |  | slot1000 |  |  |
| sl-PTRS-Config-r16 |  | Not present |  |  |
| sl-UE-SelectedConfigRP-r16 SEQUENCE { |  |  |  |  |
| sl-CBR-PriorityTxConfigList-r16 |  | SL-CBR-PriorityTxConfigList |  |  |
| sl-Thres-RSRP-List-r16 |  | SL-Thres-RSRP-List |  |  |
| sl-MultiReserveResource-r16 |  | Not present |  |  |
| sl-MaxNumPerReserve-r16 |  | n2 |  |  |
| sl-SensingWindow-r16 |  | ms100 |  |  |
| sl-SelectionWindowList-r16 SEQUENCE (SIZE (8)) OF SL-SelectionWindowConfig-r16 { |  | 8 entries |  |  |
| SL-SelectionWindowConfig-r16[k, k=1..8] SEQUENCE { |  |  | entry k |  |
| sl-Priority-r16 |  | k |  |  |
| sl-SelectionWindow-r16 |  | n10 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-ResourceReservePeriodList-r16 |  | Not present |  |  |
| sl-RS-ForSensing-r16 |  | pssch |  |  |
| sl-CBR-PriorityTxConfigList-v1650 |  | Not present |  |  |
| } |  |  |  |  |
| sl-RxParametersNcell-r16 |  | Not present |  |  |
| sl-ZoneConfigMCR-List-r16 |  | Not present |  |  |
| sl-FilterCoefficient-r16 |  | fc4 |  |  |
| sl-RB-Number-r16 |  | 10*floor(n/10) | Note 1 |  |
| sl-PreemptionEnable-r16 |  | Not present |  |  |
| sl-PriorityThreshold-UL-URLLC-r16 |  | Not present |  |  |
| sl-PriorityThreshold-r16 |  | Not present |  |  |
| sl-X-Overhead-r16 |  | Not present | Default value n0 is used |  |
| sl-PowerControl-r16 |  | Not present |  |  |
| sl-TxPercentageList-r16 SEQUENCE (SIZE (8)) OF SL-TxPercentageConfig-r16 { |  | 8 entries |  |  |
| SL-TxPercentageConfig-r16[k, k=1..8] SEQUENCE { |  |  | entry k |  |
| sl-Priority-r16 |  | k |  |  |
| sl-TxPercentage-r16 |  | p50 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-MinMaxMCS-List-r16 |  | Not present |  |  |
| sl-TimeResource-r16 |  | 1111111100 | First 8 of every 10 logical slots |  |
|  |  | 0000000011 | Last 2 of every 10 logical slots | EXCEPTIONAL |
| } |  |  |  |  |
| Note 1: n is the bandwidth of active SL BWP (in RB). n is determined in Table 4.3.1.0D-1 and Table 4.3.1.0D-2 for FR1 and FR2 respectively. Note 2: m is subchannel size configured by sl-SubchannelSize-r16. Note 3: p is the PSFCH period configured by sl-PSFCH-Period-r16 |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SL_HARQ | To enable SL HARQ feedback |
| EXCEPTIONAL | For exceptional resource pool |

–	SL-RLC-BearerConfig

Table 4.6.6-26: SL-RLC-BearerConfig

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-RLC-BearerConfig-r16 ::= SEQUENCE { |  |  |  |  |
| sl-RLC-BearerConfigIndex-r16 |  | SL-RLC-BearerConfigIndex |  |  |
| sl-ServedRadioBearer-r16 |  | SLRB-Uu-ConfigIndex |  |  |
| sl-RLC-Config-r16 |  | SL-RLC-Config with condition AM |  |  |
| sl-MAC-LogicalChannelConfig-r16 |  | SL-LogicalChannelConfig |  |  |
| } |  |  |  |  |

<End of modified section 8>

<Start of modified section 9>

–	SL-RLC-Config

Table 4.6.6-28: SL-RLC-Config

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-RLC-Config-r16 ::= CHOICE { |  |  |  |  |
| sl-AM-RLC-r16 SEQUENCE { |  |  |  | AM |
| sl-SN-FieldLengthAM-r16 |  | size12 |  |  |
| sl-T-PollRetransmit-r16 |  | ms80 |  |  |
| sl-PollPDU-r16 |  | p32768 |  |  |
| sl-PollByte-r16 |  | kB750 |  |  |
| sl-MaxRetxThreshold-r16 |  | t8 |  |  |
| } |  |  |  |  |
| sl-UM-RLC-r16 SEQUENCE { |  |  |  | UM |
| sl-SN-FieldLengthUM-r16 |  | size6 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| AM | RLC AM |
| UM | RLC UM |

–	SL-ScheduledConfig

Table 4.6.6-29: SL-ScheduledConfig

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment |  | Condition |
| SL-ScheduledConfig-r16 ::= SEQUENCE { |  |  |  |  |  |
| sl-RNTI-r16 | RNTI-Value |  |  |  |  |
| mac-MainConfigSL-r16 SEQUENCE { |  |  |  |  |  |
| sl-BSR-Config-r16 | BSR-Config |  |  |  |  |
| ul-PrioritizationThres-r16 | Not present |  |  |  |  |
| sl-PrioritizationThres-r16 | Not present |  |  |  |  |
| } |  |  |  |  |  |
| sl-CS-RNTI-r16 | Not present |  |  |  |  |
|  | RNTI-Value which is different with sl-RNTI-r16 |  | SL_CG |  |  |
| sl-PSFCH-ToPUCCH-r16 | Not present |  |  |  |  |
| sl-PSFCH-ToPUCCH-r16 SEQUENCE (SIZE (1..8)) OF INTEGER { | 1 entry |  | SL HARQ_VIA_UU |  |  |
| INTEGER[1] | 4 | entry 1 |  |  |  |
| } |  |  |  |  |  |
| sl-ConfiguredGrantConfigList-r16 | Not present |  |  |  |  |
| sl-ConfiguredGrantConfigList-r16 SEQUENCE { |  |  | SL_CG |  |  |
| sl-ConfiguredGrantConfigToReleaseList-r16 | Not present |  |  |  |  |
| sl-ConfiguredGrantConfigToAddModList-r16 SEQUENCE (SIZE (1..maxNrofCG-SL-r16)) OF SL-ConfiguredGrantConfig-r16 { | 1 entry |  |  |  |  |
| SL-ConfiguredGrantConfig-r16[1] | SL-ConfiguredGrantConfig | entry 1 |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| sl-DCI-ToSL-Trans-r16 SEQUENCE (SIZE (1..8)) OF INTEGER { | 1 entry |  |  |  |  |
| INTEGER[1] | 4 | entry 1 |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SL_CG | To provide UE SL configured grant |
| SL HARQ_VIA_UU | to report HARQ-ACK information that the UE generates based on PSFCH reception via PUCCH/PUSCH |

<End of modified section 9>

<Start of modified section 10>

–	SL-SyncConfig

Table 4.6.6-31: SL-SyncConfig

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SL-SyncConfig-r16 ::= SEQUENCE { |  |  |  |  |
| sl-SyncRefMinHyst-r16 | dB3 |  |  |  |  |
| sl-SyncRefDiffHyst-r16 | dB3 |  |  |  |  |
| sl-filterCoefficient-r16 | fc4 |  |  |  |  |
| sl-SSB-TimeAllocation1-r16 SEQUENCE { |  |  |  |  |  |
| sl-NumSSB-WithinPeriod-r16 | n1 |  |  | SCS15 |  |
|  | n2 |  |  | SCS30, SCS60, SCS120 |  |
| sl-TimeOffsetSSB-r16 | 0 |  |  |  |  |
| sl-TimeInterval-r16 | Not present |  |  | SCS15 |  |
|  | 1 |  |  | SCS30, SCS60, SCS120 |  |
| } |  |  |  |  |  |
| sl-SSB-TimeAllocation2-r16 SEQUENCE { |  |  |  |  |  |  |  |  |  |
| sl-NumSSB-WithinPeriod-r16 |  | n1 |  |  |  |  | SCS15 |  |  |
|  |  | n2 |  |  |  |  | SCS30, SCS60, SCS120 |  |  |
| sl-TimeOffsetSSB-r16 |  | 80 |  |  |  |  |  |  |  |
| sl-TimeInterval-r16 |  | Not present |  |  |  |  | SCS15 |  |  |
|  |  | 1 |  |  |  |  | SCS30, SCS60, SCS120 |  |  |
| } |  |  |  |  |  |
| sl-SSB-TimeAllocation3-r16 | Not present |  |  |  |  |
| sl-SSID-r16 | Arbitrarily chosen by SS from the range 1 to 335 |  |  |  |  |
| txParameters-r16 SEQUENCE { |  |  |  |  |  |
| syncTxThreshIC-r16 | Not present |  |  |  |  |
| syncTxThreshOoC-r16 | 4 | actual threshold is -100 dBm |  |  |  |
| syncInfoReserved-r16 | 00 |  |  |  |  |
| } |  |  |  |  |  |
| gnss-Sync-r16 | Not present |  |  |  |  |
| } |  |  |  |  |

<End of modified section 10>

<Start of modified section 11>

–	SL-UE-SelectedConfig

Table 4.6.6-35: SL-UE-SelectedConfig

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-UE-SelectedConfig-r16 ::= SEQUENCE { |  |  |  |  |
| sl-PSSCH-TxConfigList-r16 |  | SL-PSSCH-TxConfigList |  |  |
| sl-ProbResourceKeep-r16 |  | v0 |  |  |
| sl-ReselectAfter-r16 |  | n9 |  |  |
| sl-CBR-CommonTxConfigList-r16 |  | SL-CBR-CommonTxConfigList |  |  |
| ul-PrioritizationThres-r16 |  | Not present |  |  |
| sl-PrioritizationThres-r16 |  | Not present |  |  |
| } |  |  |  |  |

<End of modified section 11>

<Start of modified section 12>

4.10.1	Pre-configuration for V2X

–	SL-PreconfigurationNR

Table 4.10.1-1: SL-PreconfigurationNR

| Derivation Path: TS 38.331 [6], clause 9.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-PreconfigurationNR-r16 ::= SEQUENCE { |  |  |  |  |
| sidelinkPreconfigNR-r16 SEQUENCE { |  |  |  |  |
| sl-PreconfigFreqInfoList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigCommon-r16 { |  | 1 entry |  |  |
| SL-FreqConfigCommon-r16[1] |  | SL-FreqConfigCommon | entry 1 |  |
| } |  |  |  |  |
| sl-PreconfigNR-AnchorCarrierFreqList-r16 |  | Not present |  |  |
| sl-PreconfigEUTRA-AnchorCarrierFreqList-r16 |  | Not present |  |  |
| sl-RadioBearerPreConfigList-r16 SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SL-RadioBearerConfig-r16 { |  | 1 entry |  |  |
| SL-RadioBearerConfig-r16[1] |  | SL-RadioBearerConfig | entry 1 |  |
| } |  |  |  |  |
| sl-RLC-BearerPreConfigList-r16 SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-BearerConfig-r16 { |  | 1 entry |  |  |
| SL-RLC-BearerConfig-r16[1] |  | SL-RLC-BearerConfig | entry 1 |  |
| } |  |  |  |  |
| sl-MeasPreConfig-r16 |  | Not present |  |  |
| sl-OffsetDFN-r16 |  | Not present |  |  |
| t400-r16 |  | ms1000 |  |  |
| sl-MaxNumConsecutiveDTX-r16 |  | n32 |  |  |
| sl-SSB-PriorityNR-r16 |  | 1 |  |  |
| sl-PreconfigGeneral-r16 SEQUENCE { |  |  |  |  |
| sl-TDD-Configuration-r16 |  | Not present |  |  |
| reservedBits-r16 |  | 00 |  |  |
| } |  |  |  |  |
| sl-UE-SelectedPreConfig-r16 SEQUENCE { |  |  |  |  |
| sl-PSSCH-TxConfigList-r16 |  | SL-PSSCH-TxConfigList for Preconfiguration |  |  |
| sl-ProbResourceKeep-r16 |  | v0 |  |  |
| sl-ReselectAfter-r16 |  | n9 |  |  |
| sl-CBR-CommonTxConfigList-r16 |  | Not present |  |  |
| ul-PrioritizationThres-r16 |  | Not present |  |  |
| sl-PrioritizationThres-r16 |  | Not present |  |  |
| } |  |  |  |  |
| sl-CSI-Acquisition-r16 |  | Not present |  |  |
| sl-RoHC-Profiles-r16 |  | Not present |  |  |
| sl-MaxCID-r16 |  | Not present | default value 15 is used |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 4.10.1-2: SL-PSSCH-TxConfigList for Preconfiguration (Table 4.10.1-1)

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment |  | Condition |
| SL-PSSCH-TxConfigList-r16 ::= SEQUENCE (SIZE (1..maxPSSCH-TxConfig-r16)) OF SL-PSSCH-TxConfig-r16 { |  | 1 entry |  |  |  |
| sl-TypeTxSync-r16 |  |  | Not present | Applicable for all synchronization reference types |  |  |
| sl-ThresUE-Speed-r16 |  |  | kmph60 |  |  |  |
| sl-ParametersAboveThres-r16 SEQUENCE { |  |  |  |  |  |  |
| sl-MinMCS-PSSCH-r16 |  |  | 0 |  |  |  |
| sl-MaxMCS-PSSCH-r16 |  |  | 26 |  |  |  |
| sl-MinSubChannelNumPSSCH-r16 |  |  | 1 |  |  |  |
| sl-MaxSubchannelNumPSSCH-r16 |  |  | 27 |  |  |  |
| sl-MaxTxTransNumPSSCH-r16 |  |  | 4 |  |  |  |
| sl-MaxTxPower-r16 |  |  | Not present |  |  |  |
| } |  |  |  |  |  |  |
| sl-ParametersBelowThres-r16 SEQUENCE { |  |  |  |  |  |  |
| sl-MinMCS-PSSCH-r16 |  |  | 0 |  |  |  |
| sl-MaxMCS-PSSCH-r16 |  |  | 26 |  |  |  |
| sl-MinSubChannelNumPSSCH-r16 |  |  | 1 |  |  |  |
| sl-MaxSubchannelNumPSSCH-r16 |  |  | 27 |  |  |  |
| sl-MaxTxTransNumPSSCH-r16 |  |  | 4 |  |  |  |
| sl-MaxTxPower-r16 |  |  | Not present |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |

<End of modified section 12>