3GPP TSG-RAN5 Meeting #96-e	R5-224620r1

# Online,  , 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0783 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to connection diagram of spurious emissions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Connection diagram between SS and UE in TS 38.508-1 Annex A is FFS for Spurious emissions test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Spurious emissions test cases: - Editor’s note was corrected. - TE diagram was defined as Figure A.3.3.1.1 in TS 38.508-1. - UE diagram was defined as Figure A.3.4.1.1 in TS 38.508-1. Editorial correction: - “CA n259” was corrected to “CA_n259” in Table 6.5A.3.2.0-1. - Continuous lines above 66 GHz ≤ f ≤ 80GHz were corrected to broken lines in Table 7.9.4.2-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spurious emissions test cases will not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3, 6.5A.3.2, 6.5.A.3.3, 7.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Changes in 6.5A.3.1 were removed to avoid overlapping with R5-224370 |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

6.5.3	Spurious emissions

Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products, but exclude out of band emissions. The spurious emission limits are specified in terms of general requirements in line with SM.329 [7] and NR operating band requirement to address UE co-existence. Spurious emissions are measured as TRP.

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.5.3.1	Transmitter Spurious emissions

Editor’s Note: This clause is complete for Band n257, n258, n260 and n261 and PC3. The following aspects of the clause are for future consideration:

-	TRP Measurement uncertainty is TBD for above 80 GHz.

-	Test procedure only includes the testing of smartphone and  is FFS for laptop and FWA.

6.5.3.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

6.5.3.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

6.5.3.1.3	Minimum conformance requirements

Unless otherwise stated, the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Table 6.5.3.1.3-1 starting from the edge of the assigned NR channel bandwidth. The spurious emission limits in Table 6.5.3.1.3-2 apply for all transmitter band configurations (NRB) and channel bandwidths. The requirement is verified in beam locked mode with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid).

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 6.5.3.1.3-1: Boundary between NR out of band and spurious emission domain

| Channel bandwidth | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| OOB boundary FOOB (MHz) | 100 | 200 | 400 | 800 |

The spurious emission limits in table 6.5.3.1.3-2 apply for all transmitter band configurations (RB) and channel bandwidths.

Table 6.5.3.1.3-2: Spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |  |
| 12.75 GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band in GHz | -13 dBm | 1 MHz |  |

The normative reference for this requirement is TS 38.101-2 subclause 6.5.3.

6.5.3.1.4	Test description

6.5.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.5.3.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.3.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Low range, High range (NOTE 2) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | 120kHz |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | - | Modulation | RB allocation (NOTE 1) |
| 1 |  | DFT-s -OFDM QPSK | Inner_Full for PC2, PC3 and PC4 Inner_Full_Region1 for PC1 |
| 2 |  | DFT-s -OFDM QPSK | Inner_1RB for PC2, PC3 and PC4 Inner_Partial for PC1 (NOTE 3) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: When testing Low range test only in Frequency Range lower than (FUL_low – ΔfOOB) and when testing High range test only in Frequency Range higher than (FUL_high + ΔfOOB). NOTE 3: When testing Low range configure uplink RB to Inner_1RB_Left for PC2, PC3 and PC4 or Inner_Partial_Left_Region1 for PC1 and when testing High range configure uplink RB to Inner_1RB_Right for PC2, PC3 and PC4 or Inner_Partial_Right_Region1 for PC1. |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement channels are set according to Table 6.5.3.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.5.3.1.4.3.

<<Unchanged sections skipped>>

6.5.3.2	Spurious emission band UE co-existence

Editor’s note: The following aspects are either missing or not yet determined:

- TRP Measurement uncertainty is TBD for PC1, PC2 and PC4.

- Test procedure only includes the testing of smartphone and  is FFS for laptop and FWA.

6.5.3.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference when in co-existence with protected bands in terms of transmitter spurious emissions.

6.5.3.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

6.5.3.2.3	Minimum conformance requirements

This clause specifies the requirements for the specified NR band, for co-existence with protected bands. The requirement is verified in beam locked mode with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid).

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

The spurious emission UE co-existence limits in Table 6.5.3.2.3-1 apply for all transmitter band configurations (RB) and channel bandwidths.

Table 6.5.3.2.3-1: Spurious emissions UE co-existence limits

| NR Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band/frequency range | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n257 | NR Band n260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
|  | Frequency range | 23600 | - | 24000 | 1 | 200 | 3 |
| n258 | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n259 | NR Band 257 | FDL_low | - | FDL_high | -5 | 100 | n259 |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | Frequency range | 36000 | - | 37000 | 7 | 1000 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n260 | NR Band 257 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| n261 | NR Band 260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| NOTE 1: FDL_low and FDL_high refer to each NR frequency band specified in Table 5.2-1. NOTE 2: Void. NOTE 3: The protection of frequency range 23600-24000 MHz is meant for protection of satellite passive services. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-2 subclause 6.5.3.1.

6.5.3.2.4	Test description

6.5.3.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.5.3.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.3.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Low range, High range (NOTE 2) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | 120kHz |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | - | Modulation | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM QPSK | Inner_Full for PC2, PC3 and PC4 Inner_Full_Region1 for PC1 |
| 2 |  | DFT-s-OFDM QPSK | Inner_1RB for PC2, PC3 and PC4 Inner_Partial for PC1 (NOTE 3) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: When testing Low range test only in Frequency Range lower than (FUL_low – ΔfOOB) and when testing High range test only in Frequency Range higher than (FUL_high + ΔfOOB). NOTE 3: When testing Low range configure uplink RB to Inner_1RB_Left for PC2, PC3 and PC4 or Inner_Partial_Left_Region1 for PC1 and when testing High range configure uplink RB to Inner_1RB_Right for PC2, PC3 and PC4 or Inner_Partial_Right_Region1 for PC1. |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement channels are set according to Table 6.5.3.2.4.1-1

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.5.3.2.4.3.

<<Unchanged sections skipped>>

6.5.3.3	Additional spurious emissions

Editor’s note: The following aspects are either missing or not yet determined:

- Test procedure only includes the testing of smartphone and  is FFS for laptop and FWA.

6.5.3.3.1	Test purpose

Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

6.5.3.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

6.5.3.3.3	Minimum conformance requirements

The additional spurious emission limits in Table 6.5.3.3.3-2 through Table 6.5.3.3.3-3 apply for all transmitter band configurations (RB) and channel bandwidths. The requirement is verified in beam locked mode with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid).

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 6.5.3.3.3-1: Void

When "NS_202" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.3-2.

Table 6.5.3.3.3-2: Additional spurious emissions (NS_202) test limits

| Frequency Range | Maximum Level | Measurement bandwidth |
| --- | --- | --- |
| 7.25 GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band | -10 dBm | 100 MHz |
| 23.6 GHz  f 24.0 GHz | +1 dBm | 200 MHz |
| NOTE 1: This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1.3-1 from the edge of the channel bandwidth. The protection of frequency range 23600 - 24000 MHz is meant for protection of satellite passive services. |  |  |

When "NS_203" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.3-3. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1.3-1 from the edge of the channel bandwidth.

Table 6.5.3.3.3-3: Additional spurious emissions (NS_203) test limits

| Frequency band (GHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 23.6 f 24.0 | +1 | 200 MHz |

The normative reference for this requirement is TS 38.101-2 subclause 6.5.3.2.

6.5.3.3.4	Test description

6.5.3.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.5.3.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.3.3.4.1-1: Test Configuration Table for NS_202

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Low range, High range (NOTE 2) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | 120kHz |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | - | Modulation | RB allocation (NOTE 1) |
| 1 (NOTE 5) |  | DFT-s-OFDM QPSK | Inner_Full |
| 2 |  | DFT-s-OFDM QPSK | Inner_1RB_Left for PC2, PC3 and PC4 Inner_Partial for PC1 (NOTE 3) |
| 3 (NOTE 4) |  | DFT-s-OFDM 64QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: When testing Low range test only in Frequency Range lower than (FUL_low – ΔfOOB) and when testing High range test only in Frequency Range higher than (FUL_high + ΔfOOB). NOTE 3: When testing Low range configure uplink RB to Inner_1RB_Left for PC2, PC3 and PC4 or Inner_Partial_Left_Region1 for PC1 and when testing High range configure uplink RB to Inner_1RB_Right for PC2, PC3 and PC4 or Inner_Partial_Right_Region1 for PC1. NOTE 4: Test ID only applicable to PC1 NOTE 5: Test ID not applicable to PC1. |  |  |  |

Table 6.5.3.3.4.1-2: Test Configuration Table for NS_203

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  | Low range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |  |
| Test ID | Frequency | Channel Bandwidth | Downlink Configuration | Uplink Configuration |  |
|  |  |  | - | Modulation | RB allocation (NOTE 1) |
| 1 | Default | Default |  | DFT-s-OFDM QPSK | Inner_Full |
| 2 | Default | Default |  | DFT-s-OFDM QPSK | Inner_1RB_Left for PC2, PC3 and PC4 Inner_Partial_Left_Region1 for PC1 |
| 3 (NOTE 2) | Low range + Channel Bandwidth (NOTE 3) | Default |  | DFT-s-OFDM QPSK | Inner_Partial_Left_Region1 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: Test ID only applicable to PC1. NOTE 3: Test frequency for test ID 3 is sepecified in Table 6.2.3.4.1-4. |  |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement channels are set according to Table 6.5.3.3.4.1-1

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.5.3.3.4.3.

<<Unchanged sections skipped>>

6.5A.3	Spurious emissions for CA

6.5A.3.1	General spurious emissions for CA

6.5A.3.1.0	Minimum conformance requirements

The normative reference for this requirement is TS 38.101-2 [3] clause 6.5A.3.

6.5A.3.1.0.0	General

This clause specifies the spurious emission requirements for carrier aggregation. The requirement is verified in beam locked mode with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid). The TX beam peak direction used for CA testing is the [same as that found for single carrier scenario in clause 6.5.3].

In case the CA configuration consists of a single UL CC, spurious emissions requirements defined in subclause 6.5.3 apply. Spurious emissions requirements do not apply at any frequency where IBE requirements of clause 6.4A.2.3 apply.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

6.5A.3.1.0.1	Spurious emissions for intra-band contiguous UL CA

For intra-band contiguous UL carrier aggregation, the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) from the edge of the UL aggregated channel bandwidth, where FOOB is defined as the twice the UL aggregated channel bandwidth. For frequencies ΔfOOB greater than FOOB, the spurious emission requirements in Table 6.5.3.1.3-2 are applicable.

6.5A.3.1.0.2	Spurious emissions for intra-band non-contiguous UL CA

TBD

6.5A.3.1.1	General spurious emissions for CA (2UL CA)

Editor’s note: The following aspects are either missing or not yet determined:

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

- 	Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, and 4.

-	Connection diagram between SS and UE in TS 38.508-1 [10] Annex A is FFS.

-	Test procedure only includes the testing of smartphone and  is FFS for laptop and FWA.

6.5A.3.1.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

6.5A.3.1.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 2UL CA.

6.5A.3.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5A.3.1.0.

6.5A.3.1.1.4	Test description

6.5A.3.1.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.5A.3.1.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5A.3.1.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  | Low range, High range (NOTE 2) |  |
| Test CC combination setting as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Highest aggregated BW of the CA configuration |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |
| Test ID | CC | Downlink Configuration | UL Modulation | UL RB allocation (NOTE 1) |
| 1 | PCC | - | DFT-s-OFDM QPSK | Outer_Full |
|  | SCCs |  | DFT-s-OFDM QPSK | Outer_Full |
| 2 | PCC |  | DFT-s-OFDM QPSK | Inner_1RB for PC2, PC3 and PC4 Inner_Partial for PC1 (NOTE 3) |
|  | SCCs |  | DFT-s-OFDM QPSK | Inner_1RB for PC2, PC3 and PC4 Inner_Partial for PC1 (NOTE 3) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: When testing Low range test only in Frequency Range lower than (FUL_low – ΔfOOB) and when testing High range test only in Frequency Range higher than (FUL_high + ΔfOOB). NOTE 3: When testing Low range configure uplink RB to Inner_1RB_Left for PC2, PC3 and PC4 or Inner_Partial_Left_Region1 for PC1 and when testing High range configure uplink RB to Inner_1RB_Right for PC2, PC3 and PC4 or Inner_Partial_Right_Region1 for PC1. NOTE 4: The number of DL CCs shall be configured the same as the number of UL CCs. The requirements are appliable as per 5.3A.4 "The requirements are applicable only when Uplink CCs are configured within the frequency range between lower edge of lowest downlink component carrier and upper edge of highest downlink component carrier". |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure [TBD] for TE diagram and Figure [TBD] for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement channels are set according to Table 6.5A.3.1.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.5A.3.1.1.4.3

<<Unchanged sections skipped>>

6.5A.3.1.2	General spurious emissions for CA (3UL CA)

Editor’s note: The following aspects are either missing or not yet determined:

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

-	Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, and 4.

-	Connection diagram between SS and UE in TS 38.508-1 [10] Annex A is FFS.

-	Test procedure only includes the testing of smartphone and  is FFS for laptop and FWA.

6.5A.3.1.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

6.5A.3.1.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 3UL CA.

6.5A.3.1.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5A.3.1.0.

6.5A.3.1.2.4	Test description

Same test description as in clause 6.5A.3.1.1.4.

6.5A.3.1.2.5	Test Requirements

The test requirement is the same as in clause 6.5A.3.1.1.5.

6.5A.3.1.3	General spurious emissions for CA (4UL CA)

Editor’s note: The following aspects are either missing or not yet determined:

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

-	Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, and 4.

-	Connection diagram between SS and UE in TS 38.508-1 [10] Annex A is FFS.

-	Test procedure only includes the testing of smartphone and  is FFS for laptop and FWA.

6.5A.3.1.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

6.5A.3.1.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 4UL CA.

6.5A.3.1.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5A.3.1.0.

6.5A.3.1.3.4	Test description

Same test description as in clause 6.5A.3.1.1.4.

6.5A.3.1.3.5	Test Requirements

The test requirement is the same as in clause 6.5A.3.1.1.5.

6.5A.3.1.4	General spurious emissions for CA (5UL CA)

Editor’s note: The following aspects are either missing or not yet determined:

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

-	Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, and 4.

-	Connection diagram between SS and UE in TS 38.508-1 [10] Annex A is FFS.

-	Test procedure only includes the testing of smartphone and  is FFS for laptop and FWA.

6.5A.3.1.4.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

6.5A.3.1.4.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 5UL CA.

6.5A.3.1.4.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5A.3.1.0.

6.5A.3.1.4.4	Test description

Same test description as in clause 6.5A.3.1.1.4.

6.5A.3.1.4.5	Test Requirements

The test requirement is the same as in clause 6.5A.3.1.1.5

6.5A.3.1.5	General spurious emissions for CA (6UL CA)

Editor’s note: The following aspects are either missing or not yet determined:

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

-	Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, and 4.

-	Connection diagram between SS and UE in TS 38.508-1 [10] Annex A is FFS.

-	Test procedure only includes the testing of smartphone and  is FFS for laptop and FWA.

6.5A.3.1.5.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

6.5A.3.1.5.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 6UL CA.

6.5A.3.1.5.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5A.3.1.0.

6.5A.3.1.5.4	Test description

Same test description as in clause 6.5A.3.1.1.4.

6.5A.3.1.5.5	Test Requirements

The test requirement is the same as in clause 6.5A.3.1.1.5

6.5A.3.1.6	General spurious emissions for CA (7UL CA)

Editor’s note: The following aspects are either missing or not yet determined:

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

-	Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, and 4.

-	Connection diagram between SS and UE in TS 38.508-1 [10] Annex A is FFS.

-	Test procedure only includes the testing of smartphone and  is FFS for laptop and FWA.

6.5A.3.1.6.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

6.5A.3.1.6.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 7UL CA.

6.5A.3.1.6.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5A.3.1.0.

6.5A.3.1.6.4	Test description

Same test description as in clause 6.5A.3.1.1.4.

6.5A.3.1.6.5	Test Requirements

The test requirement is the same as in clause 6.5A.3.1.1.5

6.5A.3.1.7	General spurious emissions for CA (8UL CA)

Editor’s note: The following aspects are either missing or not yet determined:

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

-	Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, and 4.

-	Connection diagram between SS and UE in TS 38.508-1 [10] Annex A is FFS.

-	Test procedure only includes the testing of smartphone and  is FFS for laptop and FWA.

6.5A.3.1.7.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

6.5A.3.1.7.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 8UL CA.

6.5A.3.1.7.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5A.3.1.0.

6.5A.3.1.7.4	Test description

Same test description as in clause 6.5A.3.1.1.4.

6.5A.3.1.7.5	Test Requirements

The test requirement is the same as in clause 6.5A.3.1.1.5

6.5A.3.2	Spurious emission band UE co-existence for UL CA

This clause specifies the requirements for the specified carrier aggregation configurations for coexistence with protected bands. The requirement is verified in beam locked mode with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid). The TX beam peak direction used for CA testing is the [same as that found for single carrier scenario in clause 6.5.3].

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

6.5A.3.2.0	Minimum conformance requirements

For intra-band contiguous carrier aggregation, the requirements in Table 6.5A.3.2.0-1 apply.

Table 6.5A.3.2.0-1: Spurious emissions UE co-existence CA limits

| CA band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band / frequency range | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n257 | NR Band n260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | Frequency range | 23600 | - | 24000 | 1 | 200 | 2 |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| CA_n258 |  |  |  |  |  |  |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| CA_n259 | NR Band 257 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | Frequency range | 36000 | - | 37000 | 7 | 1000 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| CA_n260 | NR Band 257 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| CA_n261 | NR Band 260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| NOTE 1: FDL_low and FDL_high refer to each NR frequency band specified in Table 5.2-1 NOTE 2: The protection of frequency range 23600-24000MHz is meant for protection of satellite passive services. |  |  |  |  |  |  |  |

6.5A.3.2.1	Spurious emission band UE co-existence for CA (2UL CA)

Editor’s note: The following aspects are either missing or not yet determined:

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

-	Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, and 4.

-	Test procedure only includes the testing of smartphone and  is FFS for laptop and FWA.

6.5A.3.2.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference when in co-existence with protected bands in terms of transmitter spurious emissions.

6.5A.3.2.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 2UL CA.

6.5A.3.2.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5A.3.2.0.

6.5A.3.2.1.4	Test description

6.5A.3.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.5A.3.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5A.3.2.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  | Low range, High range (NOTE 2) |  |
| Test CC combination setting as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Maximum aggregated BW (contiguous CA) |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |
| Test ID | CC | Downlink Configuration | UL Modulation | UL RB allocation (NOTE 1) |
| 1 | PCC | - | DFT-s-OFDM QPSK | Outer_Full |
|  | SCCs |  | DFT-s-OFDM QPSK | Outer_Full |
| 2 | PCC |  | DFT-s-OFDM QPSK | Inner_1RB for PC2, PC3 and PC4 Inner_Partial for PC1 (NOTE 3) |
|  | SCCs |  | DFT-s-OFDM QPSK | Inner_1RB for PC2, PC3 and PC4 Inner_Partial for PC1 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: When testing Low range test only in Frequency Range lower than (FUL_low – ΔfOOB) and when testing High range test only in Frequency Range higher than (FUL_high + ΔfOOB). NOTE 3: When testing Low range configure uplink RB to Inner_1RB_Left for PC2, PC3 and PC4 or Inner_Partial_Left_Region1 for PC1 and when testing High range configure uplink RB to Inner_1RB_Right for PC2, PC3 and PC4 or Inner_Partial_Right_Region1 for PC1. NOTE 4: For a FR2 band under test, if the protected band frequency range in Table 6.5A.3.2.0-1 is only on lower or only higher frequency region with respect to the FR2 band under test then it is sufficient to test only Low range or High range frequencies, otherwise test at both Low range and High range. NOTE 5: Number of DL CCs shall be configured the same as number of UL CCs. The requirements are appliable as per 5.3A.4: "The requirements are applicable only when Uplink CCs are configured within the frequency range between lower edge of lowest downlink component carrier and upper edge of highest downlink component carrier". |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement channels are set according to Table 6.5A.3.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.5A.3.2.1.4.3.

<<Unchanged sections skipped>>

6.5A.3.2.2	Spurious emission band UE co-existence for CA (3UL CA)

Editor’s note: The following aspects are either missing or not yet determined:

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

-	Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, and 4.

-	Test procedure only includes the testing of smartphone and  is FFS for laptop and FWA.

6.5A.3.2.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference when in co-existence with protected bands in terms of transmitter spurious emissions.

6.5A.3.2.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 3UL CA.

6.5A.3.2.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5A.3.2.0.

6.5A.3.2.2.4	Test description

Same test description as in clause 6.5A.3.2.1.4.

6.5A.3.2.2.5	Test Requirements

The test requirement is the same as in clause 6.5A.3.2.1.5.

6.5A.3.2.3	Spurious emission band UE co-existence for CA (4UL CA)

Editor’s note: The following aspects are either missing or not yet determined:

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

-	Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, and 4.

-	Test procedure only includes the testing of smartphone and  is FFS for laptop and FWA.

6.5A.3.2.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference when in co-existence with protected bands in terms of transmitter spurious emissions.

6.5A.3.2.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 4UL CA.

6.5A.3.2.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5A.3.2.0.

6.5A.3.2.3.4	Test description

Same test description as in clause 6.5A.3.2.1.4.

6.5A.3.2.3.5	Test Requirements

The test requirement is the same as in clause 6.5A.3.2.1.5.

6.5A.3.2.4	Spurious emission band UE co-existence for CA (5UL CA)

Editor’s note: The following aspects are either missing or not yet determined:

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

-	Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, and 4.

-	Test procedure only includes the testing of smartphone and  is FFS for laptop and FWA.

6.5A.3.2.4.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference when in co-existence with protected bands in terms of transmitter spurious emissions.

6.5A.3.2.4.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 5UL CA.

6.5A.3.2.4.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5A.3.2.0.

6.5A.3.2.4.4	Test description

Same test description as in clause 6.5A.3.2.1.4.

6.5A.3.2.4.5	Test Requirements

The test requirement is the same as in clause 6.5A.3.2.1.5.

6.5A.3.2.5	Spurious emission band UE co-existence for CA (6UL CA)

Editor’s note: The following aspects are either missing or not yet determined:

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

-	Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, and 4.

-	Test procedure only includes the testing of smartphone and  is FFS for laptop and FWA.

6.5A.3.2.5.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference when in co-existence with protected bands in terms of transmitter spurious emissions.

6.5A.3.2.5.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 6UL CA.

6.5A.3.2.5.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5A.3.2.0.

6.5A.3.2.5.4	Test description

Same test description as in clause 6.5A.3.2.1.4.

6.5A.3.2.5.5	Test Requirements

The test requirement is the same as in clause 6.5A.3.2.1.5.

6.5A.3.2.6	Spurious emission band UE co-existence for CA (7UL CA)

Editor’s note: The following aspects are either missing or not yet determined:

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

-	Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, and 4.

-	Test procedure only includes the testing of smartphone and  is FFS for laptop and FWA.

6.5A.3.2.6.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference when in co-existence with protected bands in terms of transmitter spurious emissions.

6.5A.3.2.6.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 7UL CA.

6.5A.3.2.6.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5A.3.2.0.

6.5A.3.2.6.4	Test description

Same test description as in clause 6.5A.3.2.1.4.

6.5A.3.2.6.5	Test Requirements

The test requirement is the same as in clause 6.5A.3.2.1.5.

6.5A.3.2.7	Spurious emission band UE co-existence for CA (8UL CA)

Editor’s note: The following aspects are either missing or not yet determined:

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

-	Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, and 4.

-	Test procedure only includes the testing of smartphone and  is FFS for laptop and FWA.

6.5A.3.2.7.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference when in co-existence with protected bands in terms of transmitter spurious emissions.

6.5A.3.2.7.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 8UL CA.

6.5A.3.2.7.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5A.3.2.0.

6.5A.3.2.7.4	Test description

Same test description as in clause 6.5A.3.2.1.4.

6.5A.3.2.7.5	Test Requirements

The test requirement is the same as in clause 6.5A.3.2.1.5.

6.5A.3.3	Additional spurious emissions for CA

6.5A.3.3.0	Minimum conformance requirements

The additional spurious emission for CA limits in Table 6.5A.3.3.0-2 and Table 6.5A.3.3.0-3 apply for all transmitter band configurations (RB) and channel bandwidths. The requirement is verified in beam locked mode with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid).

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 6.5A.3.3.0-1: Void

When " CA_NS_202" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.0-2.

Table 6.5A.3.3.0-2: Additional spurious emissions for (CA_NS_202) test limits

| Frequency Range | Maximum Level | Measurement bandwidth |
| --- | --- | --- |
| 7.25 GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band | -10 dBm | 100 MHz |
| 23.6 GHz ≤ f ≤ 24.0 GHz | +1 dBm | 200 MHz |

When "CA_NS_203" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.0-3. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5A.3.2.0-1 from the edge of the channel bandwidth.

Table 6.5A.3.3.0-3: Additional spurious emissions (CA_NS_203) test limits

| Frequency band (GHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 23.6 f 24.0 | +1 | 200 MHz |

The normative reference for this requirement is TS 38.101-2 subclause 6.5A.3.2.

6.5A.3.3.1	Additional spurious emissions for CA (2UL CA)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

- 	Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, and 4.

- 	TP analysis for CA is FFS (identify lowest MPR w/form, RB allocation for multiple carrier or PCC only, 1RB location if RB allocated for multiple carrier).

-	Test procedure only includes the testing of smartphone and  is FFS for laptop and FWA.

6.5A.3.3.1.1	Test purpose

Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

6.5A.3.3.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 2UL CA.

6.5A.3.3.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5A.3.3.0.

6.5A.3.3.1.4	Test description

6.5A.3.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.5A.3.3.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5A.3.3.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  | Low range, High range (NOTE 2) |  |
| Test CC combination setting as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for the CA Configuration across bandwidth combination sets supported by the UE. |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | 120kHz |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | - | Modulation | RB allocation (NOTE 1) |
| 1 |  | FFS | FFS |
| 2 |  | FFS | FFS |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: When testing Low range test only in Frequency Range lower than (FUL_low – ΔfOOB) and when testing High range test only in Frequency Range higher than (FUL_high + ΔfOOB). NOTE 3: Number of DL CCs shall be configured the same as number of UL CCs. The requirements are appliable as per 5.3A.4: "The requirements are applicable only when Uplink CCs are configured within the frequency range between lower edge of lowest downlink component carrier and upper edge of highest downlink component carrier". |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement channels are set according to Table 6.5A.3.3.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.5A.3.3.1.4.3.

<<Unchanged sections skipped>>

6.5A.3.3.2	Additional spurious emissions for CA (3UL CA)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

-	Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, and 4.

-	TP analysis for CA is FFS (identify lowest MPR w/form, RB allocation for multiple carrier or PCC only, 1RB location if RB allocated for multiple carrier).

-	Test procedure only includes the testing of smartphone and  is FFS for laptop and FWA.

6.5A.3.3.2.1	Test purpose

Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

6.5A.3.3.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 3UL CA.

6.5A.3.3.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5A.3.3.0.

6.5A.3.3.2.4	Test description

Same test description as in clause 6.5A.3.3.1.4.

6.5A.3.3.2.5	Test requirement

The test requirement is the same as in clause 6.5A.3.3.1.5

6.5A.3.3.3	Additional spurious emissions for CA (4UL CA)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

-	Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, and 4.

-	TP analysis for CA is FFS (identify lowest MPR w/form, RB allocation for multiple carrier or PCC only, 1RB location if RB allocated for multiple carrier).

-	Test procedure only includes the testing of smartphone and  is FFS for laptop and FWA.

6.5A.3.3.3.1	Test purpose

Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

6.5A.3.3.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 4UL CA.

6.5A.3.3.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5A.3.3.0.

6.5A.3.3.3.4	Test description

Same test description as in clause 6.5A.3.3.1.4.

6.5A.3.3.3.5	Test requirement

The test requirement is the same as in clause 6.5A.3.3.1.5.

6.5A.3.3.4	Additional spurious emissions for CA (5UL CA)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

-	Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, and 4.

-	TP analysis for CA is FFS (identify lowest MPR w/form, RB allocation for multiple carrier or PCC only, 1RB location if RB allocated for multiple carrier).

-	Test procedure only includes the testing of smartphone and  is FFS for laptop and FWA.

6.5A.3.3.4.1	Test purpose

Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

6.5A.3.3.4.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 5UL CA.

6.5A.3.3.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5A.3.3.0.

6.5A.3.3.4.4	Test description

Same test description as in clause 6.5A.3.3.1.4.

6.5A.3.3.4.5	Test requirement

The test requirement is the same as in clause 6.5A.3.3.1.5.

6.5A.3.3.5	Additional spurious emissions for CA (6UL CA)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

-	Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, and 4.

-	TP analysis for CA is FFS (identify lowest MPR w/form, RB allocation for multiple carrier or PCC only, 1RB location if RB allocated for multiple carrier).

-	Test procedure only includes the testing of smartphone and  is FFS for laptop and FWA.

6.5A.3.3.5.1	Test purpose

Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

6.5A.3.3.5.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 6UL CA.

6.5A.3.3.5.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5A.3.3.0.

6.5A.3.3.5.4	Test description

Same test description as in clause 6.5A.3.3.1.4.

6.5A.3.3.5.5	Test requirement

The test requirement is the same as in clause 6.5A.3.3.1.5.

6.5A.3.3.6	Additional spurious emissions for CA (7UL CA)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

-	Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, and 4.

-	TP analysis for CA is FFS (identify lowest MPR w/form, RB allocation for multiple carrier or PCC only, 1RB location if RB allocated for multiple carrier).

-	Test procedure only includes the testing of smartphone and  is FFS for laptop and FWA.

6.5A.3.3.6.1	Test purpose

Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

6.5A.3.3.6.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 7UL CA.

6.5A.3.3.6.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5A.3.3.0.

6.5A.3.3.6.4	Test description

Same test description as in clause 6.5A.3.3.1.4.

6.5A.3.3.6.5	Test requirement

The test requirement is the same as in clause 6.5A.3.3.1.5.

6.5A.3.3.7	Additional spurious emissions for CA (8UL CA)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

-	Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, and 4.

-	TP analysis for CA is FFS (identify lowest MPR w/form, RB allocation for multiple carrier or PCC only, 1RB location if RB allocated for multiple carrier).

-	Test procedure only includes the testing of smartphone and  is FFS for laptop and FWA.

6.5A.3.3.7.1	Test purpose

Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

6.5A.3.3.7.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 8UL CA.

6.5A.3.3.7.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5A.3.3.0.

6.5A.3.3.7.4	Test description

Same test description as in clause 6.5A.3.3.1.4.

6.5A.3.3.7.5	Test requirement

The test requirement is the same as in clause 6.5A.3.3.1.5

<<Unchanged sections skipped>>

7.9	Spurious emissions

Editor's note:	Following aspects are either missing or not yet determined:

- The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2, and 4.

- Test procedure only includes the testing of smartphone and  is FFS for laptop and FWA.

7.9.1	Test purpose

Test verifies the UE's spurious emissions meet the requirements described in clause 7.9.3.

Excess spurious emissions increase the interference to other systems.

7.9.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

7.9.3	Minimum conformance requirements

The spurious emissions power is the power of emissions generated or amplified in a receiver. The spurious emissions power level is measured as TRP.

The power of any narrow band CW spurious emission shall not exceed the maximum level specified in Table 7.9.3-1. The requirement is verified in beam locked mode with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid).

Table 7.9.3-1: General receiver spurious emission requirements

| Frequency range | Measurement bandwidth | Maximum level | NOTE |
| --- | --- | --- | --- |
| 30MHz f < 1GHz | 100 kHz | -57 dBm | 1 |
| 1GHz f 2nd harmonic of the upper frequency edge of the DL operating band in GHz | 1 MHz | -47 dBm |  |
| NOTE 1: Unused PDCCH resources are padded with resource element groups with power level given by PDCCH as defined in Annex C.3.1. |  |  |  |

The normative reference for this requirement is TS 38.101-2 [3] clause 7.9.

7.9.4	Test description

7.9.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 7.9.4.1-1. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.9.4.1-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  | Highest |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Highest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | Mod'n | RB allocation |  | Mod'n | RB allocation |
| 1 | - | - |  | - | - |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3.2.4.1-1. |  |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C , and uplink signals according to Annex G.

4.	The DL and UL Reference Measurement channels are set according to Table 7.9.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message content are defined in clause 7.9.4.3.

7.9.4.2	Test procedure

1.	Select any of the three Alignment Options (1, 2, or 3) from Tables N.2-1 through N.2-3 to mount the DUT inside the QZ.

2.	If the re-positioning concept is applied, position the device in DUT Orientation 1 if the maximum beam peak direction is within zenith angular range 0o≤≤90o for the alignment option selected in step 1; position the device in DUT Orientation 2 (either Options 1 or 2) if the maximum beam peak direction is within zenith angular range 90o<≤180o for DUT Orientation 1 for the alignment option selected in step 1. If the re-positioning concept is not applied, position the device in DUT Orientation 1.

3.	Set the UE in the Inband Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1 using the uplink configuration in section 6.2.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 3) for the UE Tx beam selection to complete.

4.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

5.	Measure the spurious emissions as per steps outlined below with an exception to the procedure in Annex K if the re-positioning concept is applied (NOTE 4). Step (a) is optional and applicable only if SNR (test requirement level in Table 7.9.5-1 minus offset value minus noise floor of the test system) ≥ 0 dB is guaranteed.

(a)	Perform coarse TRP measurements to identify spurious emission frequencies and corresponding power level according to the procedures in Annex K, using coarse TRP measurement grid selection criteria as per Table M.4.5-3 in Annex M. The measurement is completed in both polarizations  and  over frequency range and measurement bandwidth according to Table 7.9.5-1. Optionally, a larger and non-constant measurement bandwidth than that of Table 7.9.5-1 may be applied. The measurement period shall capture the active time slots. For each spurious emission frequency with coarse TRP identified to be less than an offset dB from the TRP limit according to Table 7.9.5-1, continue with fine TRP procedures according to step (b).

The offset value shall be the TRP measurement uncertainty at 95% confidence level including the effect of coarse grid measurement uncertainty element, excluding the influence of noise. Different coarse TRP grids and corresponding offset values may be used for different frequencies. The coarse TRP grid and offset values used shall be recorded in the test report.

Table 7.9.4.2-1: Typical offset values for coarse TRP measurement step 7(a)

| Grid |  | Frequency Range |  | Offset Value |  |
| --- | --- | --- | --- | --- | --- |
| Constant Density |  | 6 GHz f < 12.75 GHz |  | 5.25 |  |
|  |  | 12.75 GHz ≤ f ≤ 23.45GHz |  | 5.21 |  |
|  |  | 23.45 GHz ≤ f ≤ 40.8GHz |  | 5.49 |  |
|  |  | 40.8 GHz ≤ f ≤ 66GHz |  | 7.31 |  |
|  |  | 66 GHz ≤ f ≤ 80GHz |  | 7.61 |  |
| Constant-Step Size |  | 6 GHz f < 12.75 GHz |  | 5.38 |  |
|  |  | 12.75 GHz ≤ f ≤ 23.45GHz |  | 5.34 |  |
|  |  | 23.45 GHz ≤ f ≤ 40.8GHz |  | 5.62 |  |
|  |  | 40.8 GHz ≤ f ≤ 66GHz |  | 7.43 |  |
|  |  | 66 GHz ≤ f ≤ 80GHz |  | 7.73 |  |
| NOTE 1: These offset values are the upper limit values when fine TRP measurement uncertainty of the test system is same as maximum test system uncertainty in Annex F and when using the coarse measurement grid with minimum number of points as specified in Table M.4.5-3. NOTE 2: It is allowed to use the offset values derived based on test system’s actual measurement uncertainty budget and denser measurement grid as specified in Table M.4.5-3. |  |  |  |  |  |

(b)	Measure fine TRP measurements according to procedures in Annex K, using fine TRP measurement grid selection criteria as per Table M.4.5-3 in Annex M, for each of the spurious emission frequency identified in step (a). Apply a measurement bandwidth according to Table 7.9.5-1.

6.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

NOTE 1:	The frequency range defined in Table 7.9.5-1 may be split into ranges. For each range a different test system, e.g. antenna and/or chamber, may be used. To pass the test case all verdicts of the frequency ranges must pass.

NOTE 2: 	Void.

NOTE 3:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.1.

NOTE 4:	If the (in-band) beam peak is within 0o≤≤90o: perform first hemispherical TRP scan (0o≤≤90o) in DUT Orientation 1 and second hemispherical TRP scan (90o>≥0o) in DUT Orientation 2. If the (in-band) beam peak is within 90o<≤180o: perform first hemispherical TRP scan (0o≤≤90o) in DUT Orientation 2 and second hemispherical TRP scan (90o>≥0o) in DUT Orientation 1. The DUT with UBF activated needs to be re-positioned during the test.

NOTE 5:	The coarse TRP measurement grid and corresponding offset dB value referred in step 5(a) above, for some valid grids can be found in TR 38.903[20] section B.18.

7.9.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.

7.9.5	Test requirement

The measured spurious emissions derived in step 5, shall not exceed the maximum level specified in Table 7.9.5-1.

Table 7.9.5-1: General receiver spurious emission requirements (Band n257, n258, n260, n261)

| Frequency range | Measurement bandwidth | Maximum level | NOTE |
| --- | --- | --- | --- |
| 6GHz f < 20GHz | 1 MHz | -47 + 10.2 dBm | 1 |
| 20GHz f < 40GHz | 1 MHz | -47 + 17.2 dBm | 1 |
| 40GHz f 2nd harmonic of the upper frequency edge of the DL operating band in GHz | 1 MHz | -47 + 33.1 dBm | 1 |
| NOTE 1: Unused PDCCH resources are padded with resource element groups with power level given by PDCCH as defined in Annex C.3.1. |  |  |  |

Table 7.9.5-2: Void

<<End of change>>