3GPP TSG-RAN5 Meeting #96-e	R5-224671r1

# Electronic Meeting, 15th – 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0444 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR IMS TC 6.7-Authentication with MAC Parameter Invalid |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The presence of registration procedure and IMS PDU session establishment procedure as defined in steps 2-19a1 of TS38.508-1 table 4.5.2.2-2 are currently not reflected in test procedure sequence. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add steps 1A-1V. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the TC or a non-conformant UE may pass the TC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: Add Note to reference the NAS signalling. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

6.7	Authentication / MAC Parameter Invalid / Only two consecutive invalid challenges / 5GS

6.7.1	Test Purpose (TP)

(1)

with { UE starting registration procedure }

ensure that {

when { UE receiving invalid MAC parameter }

then { UE sends another REGISTER request without challenge response AUTS and populates a new Security-Client header }

}

(2)

with { UE having responded to invalid MAC parameter }

ensure that {

when { UE receives another invalid MAC parameter }

then { UE sends another REGISTER request without challenge response AUTS and populates a new Security-Client header }

}

(3)

Void

6.7.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.229 clause 5.1.1.5.3]:

If, in a 401 (Unauthorized) response, either the MAC or SQN is incorrect the UE shall respond with a further REGISTER indicating to the S-CSCF that the challenge has been deemed invalid as follows:

-	in the case where the UE deems the MAC parameter to be invalid the subsequent REGISTER request shall contain no "auts" Authorization header field parameter and an empty "response" Authorization header field parameter, i.e. no authentication challenge response;

…

Whenever the UE detects any of the above cases, the UE shall:

-	send the REGISTER request using an existing set of security associations, if available (see 3GPP TS 33.203 [16]);

-	populate a new Security-Client header field within the REGISTER request and associated contact address, set to specify the security mechanisms it supports, the IPsec layer algorithms for integrity and confidentiality protection it supports and the parameters needed for the new security association setup. These parameters shall contain new values for spi_uc, spi_us and port_uc; and

-	not create a temporary set of security associations.

[TS 24.229 clause 5.1.1.5.12]:

A UE shall only respond to two consecutive invalid challenges and shall not automatically attempt authentication after receiving two consecutive invalid challenges. The UE may attempt to register with the network again after an implementation specific time.

6.7.3	Test description

6.7.3.1	Pre-test conditions

System Simulator:

-	SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE.

-	SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [16] clause 6.1 and RFC 3310 [15].

- 	SS is listening to SIP default port 5060 for both UDP and TCP protocols.

-	1 NR Cell

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	The UE is configured to register for IMS after switch on.

-	The UE is switched off.

Preamble:

-	None.

6.7.3.2	Test procedure sequence

Table 6.7.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. |  |  |  |  |
| 2 | UE sends initial registration for IMS services. | --> | REGISTER |  |  |
| 3 | SS responds with an invalid AKAv1-MD5 authentication challenge with an invalid MAC value. | <-- | 401 Unauthorized |  |  |
| 4 | Check: does the UE send a REGISTER request: - contains no AUTS directive and an empty response directive, i.e. no authentication challenge response - UE populates a new Security-Client header set to specify the security mechanism it supports, the IPsec layer algorithms it supports and the parameters needed for the new security association setup | --> | REGISTER | 1 | P |
| 5 | SS responds with an invalid AKAv1-MD5 authentication challenge with an invalid MAC value. | <-- | 401 Unauthorized |  |  |
| 6 | Check: does the UE send another REGISTER request: - contains no AUTS directive and an empty response directive, i.e. no authentication challenge response - UE populates a new Security-Client header set to specify the security mechanism it supports, the IPsec layer algorithms it supports and the parameters needed for the new security association setup | --> | REGISTER | 2 | P |
| 7 | Void |  |  |  |  |
| 8 | Void |  |  |  |  |
| 9-15 | Steps 2-8 from Clause A.2 are performed. |  |  |  |  |
| NOTE: The associated NR/5GC signalling is specified in TS 38.508 [21] generic procedure 4.5.2. |  |  |  |  |  |

6.7.3.3	Specific message contents

Table 6.7.3.3-1: REGISTER (step 2, table 6.7.3.2-1)

Derivation Path: TS 34.229-1 [2], Table in subclause A.1.1, Condition A1

Table 6.7.3.3-2: 401 Unauthorized for REGISTER (step 3/5, table 6.7.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in subclause A.1.2, Condition A1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| WWW-Authenticate |  |  |  |  |
| nonce |  | Base 64 encoding of RAND and AUTN, generated using invalid MAC value |  |  |

Table 6.7.3.3-3: REGISTER (step 4/6, table 6.7.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in subclause A.1.1, Condition A1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| CSeq |  |  |  |  |
| value |  | must be incremented from previous REGISTER |  |  |
| Call-ID |  |  |  |  |
| callid |  | same value as in REGISTER at Step 2 |  |  |
| Authorization |  |  |  |  |
| response |  | present, but empty |  |  |
| auts |  | not present |  |  |
| Security-Client |  |  |  |  |
| spi-c |  | new SPI number of the inbound SA at the protected client port, shall be different from previously used number(s) |  |  |
| spi-s |  | new SPI number of the inbound SA at the protected server port, shall be different from previously used number(s) |  |  |
| port-c |  | new protected client port, shall be different from previously used number(s) |  |  |

<End of modified section 1>