3GPP TSG-RAN5 Meeting #96-e	R5-224789r1

# Electronic Meeting, 15th – 26th Aug. 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1825 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 6.4G.2.4 EVM equalizer spectrum flatness for Tx Diversity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_TxD-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 specified requirements of EVM equalizer spectrum flatness for TxD UE as measurement per antenna. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Creating new test case 6.4G.2.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TxD requirements can’t be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4G.2.4(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: Correcting numbering of Figure 6.4G.2.4.5-1 The undefined section Annex E.XXX is changed to TBD Removing redundant paragraph in 6.4G.2.4.5 Correcting information in coversheet |  |  |  |  |  |  |  |  |

<Start of Changes>

6.4G.2.4	EVM equalizer spectrum flatness for Tx Diversity

Editor’s Note: The test case is not completed. The following aspects are either missing or not yet determined:

- Tests for Power Class 3 are FFS.

- Update to Annex E is FFS.

6.4G.2.4.1	Test purpose

Same test purpose as in 6.4.2.4.1.

6.4G.2.4.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 UE reporting Tx diversity release 15 and forward.

6.4G.2.4.3	Minimum conformance requirements

For UE supporting Tx diversity, the EVM Equalizer Spectrum Flatness requirements specified in Table 6.4.2.4.3-1 and Table 6.4.2.4.3-2 which are defined in clause 6.4.2.4.3. The composite EVM equalizer equalizer EC(f) is defined as

EC(f)=P1 ∙|EC1f|+P2 ∙|EC2(f)|P1+P2

where

ECn(f) represents equalizer coefficient for each antenna connector,  ，f is the allocated subcarriers within the transmission bandwidth ((|F|=12*);

P1 and P2 denote the linear power measured at each antenna connector respectively.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4G.2.4.

6.4G.2.4.4	Test description

Same test description as specified in clause 6.4.2.4.4 with following exceptions:

Step 3 of Test procedure as in 6.4.2.4.4.2 is replaced by:

3.	Measure spectrum flatness for each antenna connector using Global In-Channel Tx-Test (Annex E). For TDD, only slots consisting of only UL symbols are under test.

4.	Measure the mean power at each antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbols with transient periods are not under test.

5.	Calculate the composite EVM using the values measured in step 3 and step 4 as in Annex TBD.

6.4G.2.4.5	Test requirement

Each of the n spectrum flatness functions, shall derive four ripple results in Annex E.4.4.1. The derived results shall not exceed the values in Figure 6.4G.2.4.5-1:

For normal conditions, the peak-to-peak variation of the EVM equalizer coefficients contained within the frequency range of the uplink allocation shall not exceed the maximum ripple specified in Table 6.4G.2.4.5-1. For uplink allocations contained within both Range 1 and Range 2, the coefficients evaluated within each of these frequency ranges shall meet the corresponding ripple requirement and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 6.4 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 8.4 dB (see Figure 6.4G.2.4.5-1).

For extreme conditions, the EVM equalizer spectral flatness shall not exceed the values specified in Table 6.4G.2.4.5-2. For uplink allocations contained within both Range 1 and Range 2, the coefficients evaluated within each of these frequency ranges shall meet the corresponding ripple requirement and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 7.4 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 11.4 dB (see Figure 6.4G.2.4.5-1).

Table 6.4G.2.4.5-1: Requirements for EVM equalizer spectrum flatness (normal conditions)

| Frequency range | Maximum ripple [dB] |
| --- | --- |
| FUL_Meas – FUL_Low ≥ 3 MHz and FUL_High – FUL_Meas ≥ 3 MHz (Range 1) | 4 + TT (p-p) |
| FUL_Meas – FUL_Low < 3 MHz or FUL_High – FUL_Meas < 3 MHz (Range 2) | 8 + TT (p-p) |
| NOTE 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated NOTE 2: FUL_Low and FUL_High refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 3: Test tolerance TT = 1.4 dB. |  |

Table 6.4G.2.4.5-2: Minimum requirements for EVM equalizer spectrum flatness (extreme conditions)

| Frequency range | Maximum Ripple [dB] |
| --- | --- |
| FUL_Meas – FUL_Low ≥ 5 MHz and FUL_High – FUL_Meas ≥ 5 MHz (Range 1) | 4 + TT (p-p) |
| FUL_Meas – FUL_Low < 5 MHz or FUL_High – FUL_Meas < 5 MHz (Range 2) | 12 + TT (p-p) |
| NOTE 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated NOTE 2: FUL_Low and FUL_High refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 3: Test tolerance TT = 1.4 dB. |  |

Figure 6.4G.2.4.5-1: The test requirements for EVM equalizer spectral flatness with the maximum allowed variation of the coefficients indicated for unshaped modulations (the ETC test requirements are within brackets)

<End of Changes>