Title: 	Discussion on test point selection for Maximum Power Reduction for CA in FR1

Source: 	Huawei

- Introduction

As agreed in RAN5#78, each test case would require specific analysis to conform its test configuration table based on the study for Frequency, Channel BW, SCS, RB allocation (including modulations).

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for Maximum Power Reduction for CA test case in FR1.

- Discussion

Maximum Power Reduction for inter-band CA FR1 is defined in Transmitter power for CA section in [1] where its scope is described as follows:

For inter-band carrier aggregation with uplink assigned to two NR bands, the requirements in subclause 6.2.2.3 apply for each uplink component carrier.

According to table 6.2A.1.3-1 in [2], UE only support power class 3 for 2 UL CA. The test points selection for MPR of single carrier for power class 3 is extracted as below:

Table 6.2.2.4.1-1: Test Configuration Table for Power Class 3

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 13 | Default | Reduction (MPR) test case | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 23 | Low |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Left |
| 33 | High |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Right |
| 43 | Default |  | DFT-s-OFDM PI/2 BPSK | Outer Full |
| 54 | Default |  | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 64 | Low |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Left |
| 74 | High |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Right |
| 84 | Default |  | DFT-s-OFDM PI/2 BPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 10 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 13 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 14 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 15 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 16 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 17 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 18 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 19 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 20 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 21 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 22 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 23 | Default |  | CP-OFDM QPSK | Inner Full |
| 24 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 25 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM QPSK | Outer Full |
| 27 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 28 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 29 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 30 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 31 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 32 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 33 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 34 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 35 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 36 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with PI/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n40, n41, n77, n78 and n79. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n77, n78 and n79. |  |  |  |  |

We also use the test point choice from the relevant LTE test case from 36.521-1 (6.2.3A) as inspiration：

Table 6.2.3A.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| ID | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_ alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 100 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 |  |  |
| 2 | 100 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 |  |  |
| 3 | 100 | 25 |  | QPSK | 125 | P_100@0 | S_25@0 |  |  |
| 4 | 100 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 |  |  |
| 5 | 100 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 |  |  |
| 6 | 100 | 25 |  | 16QAM | 125 | P_100@0 | S_25@0 |  |  |
| 7 | 100 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 |  |  |
| 8 | 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 |  |  |
| 9 | 75 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 |  |  |
| 10 | 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 |  |  |
| 11 | 75 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 |  |  |
| 12 | 75 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 |  |  |
| 13 | 75 | 75 |  | 16QAM | 150 | P_75@0 | S_75@0 |  |  |
| 14 | 75 | 75 |  | QPSK | 2 | P_1@0 | S_1@74 |  |  |
| 15 | 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 |  |  |
| 16 | 100 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 |  |  |
| 17 | 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 |  |  |
| 18 | 100 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 |  |  |
| 19 | 100 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 |  |  |
| 20 | 100 | 50 |  | 16QAM | 150 | P_100@0 | S_50@0 |  |  |
| 21 | 100 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 |  |  |
| 22 | 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 |  |  |
| 23 | 100 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 |  |  |
| 24 | 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 |  |  |
| 25 | 100 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 |  |  |
| 26 | 100 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 |  |  |
| 27 | 100 | 75 |  | 16QAM | 175 | P_100@0 | S_75@0 |  |  |
| 28 | 100 | 75 |  | QPSK | 2 | P_1@0 | S_1@74 |  |  |
| 29 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 |  |  |
| 30 | 100 | 100 |  | QPSK | 100 | P_100@0 | S_0@0 |  |  |
| 31 | 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 |  |  |
| 32 | 100 | 100 |  | 16QAM | 18 | P_18@0 | S_0@0 |  |  |
| 33 | 100 | 100 |  | 16QAM | 100 | P_100@0 | S_0@0 |  |  |
| 34 | 100 | 100 |  | 16QAM | 200 | P_100@0 | S_100@0 |  |  |
| 35 | 100 | 100 |  | QPSK | 2 | P_1@0 | S_1@99 |  |  |
| 36 | 50 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 |  |  |
| 37 | 50 | 25 |  | QPSK | 50 | P_50@0 | S_0@0 |  |  |
| 38 | 50 | 25 |  | QPSK | 75 | P_50@0 | S_25@0 |  |  |
| 39 | 50 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 |  |  |
| 40 | 50 | 25 |  | 16QAM | 50 | P_50@0 | S_0@0 |  |  |
| 41 | 50 | 25 |  | 16QAM | 75 | P_50@0 | S_25@0 |  |  |
| 42 | 50 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 |  |  |
| 43 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 |  |  |
| 44 | 50 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 |  |  |
| 45 | 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 |  |  |
| 46 | 50 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 |  |  |
| 47 | 50 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 |  |  |
| 48 | 50 | 50 |  | 16QAM | 100 | P_50@0 | S_50@0 |  |  |
| 49 | 50 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 |  |  |
| NOTE 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. NOTE 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

For intra-band UL contiguous CA, the scope of MPR requirement in 38.101-1 v17.6.0 is described as below :

For intra-band contiguous carrier aggregation the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2A.1.1.-1 with contiguous RB allocation is specified in Table 6.2A.2.1-1 for UE power class 3 CA bandwidth classes B and C. The MPR with contiguous RB allocation is specified in Table 6.2A.2.1-1a for power class 2 CA bandwidth classes B and C when the signalling is absent for dualPA-Architecture IE, and for power class 2 CA bandwidth classe C when the signalling is indicated for dualPA-Architecture IE. The MPR with non-contiguous RB allocation is specified in Table 6.2A.2.1-1b for power class 2 CA bandwidth classes B and C with TxD supported.

In case the modulation format is different on different component carriers then the MPR is determined by the rules applied to higher order of those modulations.

Table 6.2A.2.1-1: Contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | QPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | 16QAM | 1.5 | 3.5 | 2.5 | 7 |
|  | 64QAM | 3.0 | 4.0 | 5 | 7 |
|  | 256QAM | 5.5 | 6.0 | 7 | 7.5 |
| CP-OFDM | QPSK | 2.0 | 4.0 | 3.5 | 8 |
|  | 16QAM | 2.5 | 4.0 | 3.5 | 8 |
|  | 64QAM | 3.5 | 4.0 | 5 | 8 |
|  | 256QAM | 6.5 | 6.5 | 7 | 8 |

Table 6.2A.2.1-1a: Contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 2.0 | 4.01 | 2.5 | 7 |
|  | QPSK | 2.0 | 4.01 | 2.5 | 7 |
|  | 16QAM | 2.5 | 4.01 | 2.5 | 7 |
|  | 64QAM | 3.0 | 4.51 | 5 | 7 |
|  | 256QAM | 5.5 | 6.0 | 7 | 7.5 |
| CP-OFDM | QPSK | 2.5 | 5.01 | 3.5 | 8 |
|  | 16QAM | 3.0 | 5.01 | 3.5 | 8 |
|  | 64QAM | 3.5 | 5.01 | 5 | 8 |
|  | 256QAM | 6.5 | 6.5 | 7 | 8 |
| NOTE 1: When 1 RB or 2 RB are allocated at the lower edge of lowest CC or upper edge of upper CC, MPR for outer is 5.5 dB. |  |  |  |  |  |

Table 6.2A.2.1-1b: Contiguous RB allocation for Power Class 2 with dual Tx2

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 3.0 | 5.01 | 3.5 | 8 |
|  | QPSK | 3.0 | 5.01 | 3.5 | 8 |
|  | 16QAM | 3.5 | 5.01 | 3.5 | 8 |
|  | 64QAM | 4.0 | 5.51 | 6 | 8 |
|  | 256QAM | 6.5 | 7.0 | 8 | 8.5 |
| CP-OFDM | QPSK | 3.0 | 5.51 | 4.0 | 8.5 |
|  | 16QAM | 3.5 | 5.51 | 4.0 | 8.5 |
|  | 64QAM | 4.0 | 5.51 | 5.5 | 8.5 |
|  | 256QAM | 7.0 | 7.0 | 7.5 | 8.5 |
| NOTE 1: When 1 RB or 2 RB are allocated at the lower edge of lowest CC or upper edge of upper CC, MPR for outer is 5.5 dB. NOTE 2: UE indicating TxD supported |  |  |  |  |  |

For intra-band contiguous carrier aggregation the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2A.1.1-1 with non-contiguous RB allocation is specified in Table 6.2A.2.1-2 for UE power class 3 CA bandwidth classes B and C. The MPR with non-contiguous RB allocation is specified in Table 6.2A.2.1-3 for power class 2 CA bandwidth classes B and C when the signalling is absent for dualPA-Architecture IE, and for power class 2 CA bandwidth classe C when the signalling is indicated for dualPA-Architecture IE. The MPR with non-contiguous RB allocation is specified in Table 6.2A.2.1-4 for power class 2 CA bandwidth classes B and C with TxD supported.

Table 6.2A.2.1-2: non-contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer11 | Outer22 | inner | Outer11 | Outer22 |
| DFT-s-OFDM | Pi/2 BPSK | 2 | 5.5 | 11.5 | 2.5 | 6 | 13 |
|  | QPSK | 2 | 5.5 |  | 2.5 | 6 |  |
|  | 16QAM | 2.5 | 5.5 |  | 3 | 6 |  |
|  | 64QAM | 4.5 | 6 |  | 5 | 6 |  |
|  | 256QAM | 6 | 6.5 |  | 6.5 | 6.5 |  |
| CP-OFDM | QPSK | 2.5 | 6.5 | 12 | 3.5 | 7 | 14 |
|  | 16QAM | 3 | 7 |  | 3.5 | 7 |  |
|  | 64QAM | 5 | 7 |  | 5 | 7 |  |
|  | 256QAM | 7.5 | 7.5 |  | 7.5 | 7.5 |  |
| NOTE 1: Outer 1 MPR is reduced by 3dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz |  |  |  |  |  |  |  |

Table 6.2A.2.1-3: non-contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer12 | Outer23 | Inner | Outer12 | Outer23 |
| DFT-s-OFDM | Pi/2 BPSK | 31 | 6.5 | 13 | 31 | 7.5 | 13.5 |
|  | QPSK | 31 | 6.5 |  | 31 | 7.5 |  |
|  | 16QAM | 31 | 6.5 |  | 31 | 7.5 |  |
|  | 64QAM | 5 | 6.5 |  | 5 | 7.5 |  |
|  | 256QAM | 6.5 | 7 |  | 6.5 | 7.5 |  |
| CP-OFDM | QPSK | 3.51 | 7 | 14 | 3.51 | 8 | 14.5 |
|  | 16QAM | 3.51 | 7 |  | 3.51 | 8 |  |
|  | 64QAM | 5 | 7 |  | 5 | 8 |  |
|  | 256QAM | 7.5 | 7.5 |  | 7.5 | 8 |  |
| NOTE 1: the allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. NOTE 2: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 3: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz |  |  |  |  |  |  |  |

Table 6.2A.2.1-4: non-contiguous RB allocation for Power Class 2 with dual Tx4

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer12 | Outer23 | Inner | Outer12 | Outer23 |
| DFT-s-OFDM | Pi/2 BPSK | 41 | 7.5 | 14 | 41 | 8.5 | 14.5 |
|  | QPSK | 41 | 7.5 |  | 41 | 8.5 |  |
|  | 16QAM | 41 | 7.5 |  | 41 | 8.5 |  |
|  | 64QAM | 6 | 7.5 |  | 6 | 8.5 |  |
|  | 256QAM | 7.5 | 8 |  | 7.5 | 8.5 |  |
| CP-OFDM | QPSK | 4.51 | 8 | 15 | 4.51 | 9 | 15.5 |
|  | 16QAM | 4.51 | 8 |  | 4.51 | 9 |  |
|  | 64QAM | 6 | 8 |  | 6 | 9 |  |
|  | 256QAM | 8.5 | 8.5 |  | 8.5 | 9 |  |
| NOTE 1: the allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. NOTE 2: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 3: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 4: UE indicating TxD supported |  |  |  |  |  |  |  |

We also list the Minimum requirement and Test point choice from the relevant LTE test case from 36.521-1 (6.2.3A.1) as inspiration：

For intra-band contiguous carrier aggregation the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2.2A.0-2 due to higher order modulation and contiguously allocated transmissions (resource blocks) is specified in Table 6.2.3A.1.3-1. In case the modulation format is different on different component carriers then the MPR is determined by the rules applied to higher order of those modulations.

Table 6.2.3A.1.3-1: Maximum Power Reduction (MPR) for Power Class 3

| Modulation | CA bandwidth Class B and C |  |  |  |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 25 RB + 50 RB | 50 RB + 50 RB | 25 RB + 100 RB | 50 RB + 100 RB | 75 RB + 75 RB | 75 RB + 100 RB | 100 RB + 100 RB |  |
| QPSK | > 8 and ≤ 25 | > 12 and ≤ 50 | > 8 and ≤ 25 | > 12 and ≤ 50 | > 16 and ≤ 75 | > 16 and ≤ 75 | > 18 and ≤ 100 | ≤ 1 |
| QPSK | > 25 | > 50 | > 25 | > 50 | > 75 | > 75 | > 100 | ≤ 2 |
| 16 QAM | ≤ 8 | ≤ 12 | ≤ 8 | ≤ 12 | ≤ 16 | ≤ 16 | ≤ 18 | ≤ 1 |
| 16 QAM | > 8 and ≤ 25 | > 12 and ≤ 50 | > 8 and ≤ 25 | > 12 and ≤ 50 | > 16 and ≤ 75 | > 16 and ≤ 75 | > 18 and ≤ 100 | ≤ 2 |
| 16 QAM | > 25 | > 50 | > 25 | > 50 | > 75 | > 75 | > 100 | ≤ 3 |

Table 6.2.3A.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Low and High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| ID | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_ alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 100 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 |  |  |
| 2 | 100 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 |  |  |
| 3 | 100 | 25 |  | QPSK | 125 | P_100@0 | S_25@0 |  |  |
| 4 | 100 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 |  |  |
| 5 | 100 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 |  |  |
| 6 | 100 | 25 |  | 16QAM | 125 | P_100@0 | S_25@0 |  |  |
| 7 | 100 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 |  |  |
| 8 | 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 |  |  |
| 9 | 75 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 |  |  |
| 10 | 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 |  |  |
| 11 | 75 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 |  |  |
| 12 | 75 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 |  |  |
| 13 | 75 | 75 |  | 16QAM | 150 | P_75@0 | S_75@0 |  |  |
| 14 | 75 | 75 |  | QPSK | 2 | P_1@0 | S_1@74 |  |  |
| 15 | 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 |  |  |
| 16 | 100 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 |  |  |
| 17 | 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 |  |  |
| 18 | 100 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 |  |  |
| 19 | 100 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 |  |  |
| 20 | 100 | 50 |  | 16QAM | 150 | P_100@0 | S_50@0 |  |  |
| 21 | 100 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 |  |  |
| 22 | 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 |  |  |
| 23 | 100 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 |  |  |
| 24 | 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 |  |  |
| 25 | 100 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 |  |  |
| 26 | 100 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 |  |  |
| 27 | 100 | 75 |  | 16QAM | 175 | P_100@0 | S_75@0 |  |  |
| 28 | 100 | 75 |  | QPSK | 2 | P_1@0 | S_1@74 |  |  |
| 29 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 |  |  |
| 30 | 100 | 100 |  | QPSK | 100 | P_100@0 | S_0@0 |  |  |
| 31 | 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 |  |  |
| 32 | 100 | 100 |  | 16QAM | 18 | P_18@0 | S_0@0 |  |  |
| 33 | 100 | 100 |  | 16QAM | 100 | P_100@0 | S_0@0 |  |  |
| 34 | 100 | 100 |  | 16QAM | 200 | P_100@0 | S_100@0 |  |  |
| 35 | 100 | 100 |  | QPSK | 2 | P_1@0 | S_1@99 |  |  |
| 36 | 25 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 |  |  |
| 37 | 25 | 50 |  | QPSK | 25 | P_25@0 | S_0@0 |  |  |
| 38 | 25 | 50 |  | QPSK | 75 | P_25@0 | S_50@0 |  |  |
| 39 | 25 | 50 |  | 16QAM | 8 | P_8@0 | S_0@0 |  |  |
| 40 | 25 | 50 |  | 16QAM | 25 | P_25@0 | S_0@0 |  |  |
| 41 | 25 | 50 |  | 16QAM | 75 | P_25@0 | S_50@0 |  |  |
| 42 | 25 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 |  |  |
| 43 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 |  |  |
| 44 | 50 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 |  |  |
| 45 | 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 |  |  |
| 46 | 50 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 |  |  |
| 47 | 50 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 |  |  |
| 48 | 50 | 50 |  | 16QAM | 100 | P_50@0 | S_50@0 |  |  |
| 49 | 50 | 50 |  | QPSK | 2 | P_1@0 | S_1@49 |  |  |
| 50 | 50 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 |  |  |
| 51 | 50 | 25 |  | QPSK | 50 | P_50@0 | S_0@0 |  |  |
| 52 | 50 | 25 |  | QPSK | 75 | P_50@0 | S_25@0 |  |  |
| 53 | 50 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 |  |  |
| 54 | 50 | 25 |  | 16QAM | 50 | P_50@0 | S_0@0 |  |  |
| 55 | 50 | 25 |  | 16QAM | 75 | P_50@0 | S_25@0 |  |  |
| 56 | 50 | 25 |  | QPSK | 2 | P_1@0 | S_1@24 |  |  |
| NOTE 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. NOTE 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

For intra-band non-contiguous UL CA, the scope of MPR requirement in TS 38.101-1 is described as below :

For intra-band non-contiguous CA, the allowed Maximum Power Reduction (MPR) for the maximum output power is specified into 2 types: MPR to meet -30dBm/MHz and -13dBm/MHz. The UE determins the MPR type as follows:

If AND( FIM3,low_block,low > SEM-13,low ,  FIM3,high_block,high < SEM-13,high )

MPRIM3 defined in Clause 6.2A.2.2.2

Else If AND( FIM3,low_block,low > SEM-25,low ,  FIM3,high_block,high < SEM-25,high )

MPRIM3 defined in Clause 6.2A.2.2.1

Else

MPRIM3 defined in Clause 6.2A.2.2.1

where

-	FIM3,high_block,high = (2 * Fhigh_alloc,high_edge ) – Flow_alloc,low_edge

-	FIM3,low_block,low = (2 * Flow_alloc,low_edge) – Fhigh_alloc,high_edge

-	Flow_alloc,low_edge is the lowermost frequency of the lower transmission bandwidth allocation.

-	Flow_alloc,high_edge is the uppermost frequency of the lower transmission bandwidth allocation.

-	Fhigh_alloc,low_edge is the lowermost frequency of the upper transmission bandwidth allocation.

-	Fhigh_alloc,high_edge is the uppermost frequency of the upper transmission bandwidth allocation.

-	SEM-13,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.2.2.

-	SEM-13,high = Threshold frequency where upper spectral emission mask above the upper channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.2.2.

-	SEM-25,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -25 dBm / MHz to -30 dBm / MHz, as specified in Clause 6.5A.2.2.2.

-	SEM-25,high = Threshold frequency where upper spectral emission mask above the upper channel drops from -25 dBm / MHz to -30 dBm / MHz, as specified in Clause 6.5A.2.2.2.

6.2A.2.2.1	MPRIM3 to meet -30dBm/MHz

MPR in this clause is for intra-band non-contiguous CA power class 3 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	15; 	0 ≤ B < 1.08

14.5; 	1.08 ≤ B < 2.16

13.5; 	2.16 ≤ B < 3.24

12.5;       3.24 ≤ B < 5.04

11.5; 	5.04≤ B < 10.08

10.5; 	10.08 ≤ B < 16.38

10;        16.38 ≤ B < 21.78

9; 	      21.78 ≤ B

Where:

B=(LCRB_alloc, 1* 12* SCS1 + LCRB_alloc,2 * 12 * SCS2)/1,000,000

6.2A.2.2.2	MPRIM3 to meet -13dBm/MHz

MPR in this clause is for intra-band non-contiguous CA power class 3 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MA

Where MA is defined as follows

MA = 	9	;	 0 ≤ B < 0.54

8	;	 0.54 ≤ B < 1.08

7	; 	1.08 ≤ B < 2.16

6.5	; 	2.16 ≤ B < 3.24

5.5	; 	3.24 ≤ B < 5.4

4	; 	5.4 ≤ B

Where:

B=(LCRB_alloc, 1* 12* SCS1 + LCRB_alloc,2 * 12 * SCS2)/1,000,000

We also list the Minimum requirement and Test point choice from the relevant LTE test case from TS 36.521-1 (6.2.3A.3) as inspiration：

For intra-band non-contiguous carrier aggregation with one uplink carrier, the requirements in subclause 6.2.3 apply. For intra-band non-contiguous carrier aggregation with two uplink carriers MPR is specified for E-UTRA CA configurations with a maximum possible WGAP ≤ 35 MHz; the allowed MPR is

MPR = CEIL {MN, 0.5}

where MN is defined as follows

MN = -0.125 N + 18.25; 2 ≤ N ≤ 50

-0.0333 N + 13.67; 50 < N ≤ 200

where N= NRB_alloc is the number of allocated resource blocks. E-UTRA CA configurations with a maximum possible Wgap > 35 MHz and their corresponding MPR are intended to form part of a later release.

Table 6.2.3A.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Refer to test points A: Low Wgap, Maximum Wgap |  |  |  |  |
| Test CC Combination setting (NRB) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Refer to test point Test only test points with Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | Wgap [MHz] | DL Allocation |  | UL Allocation |  |  |  |
|  | PCC NRB | SCCs NRB |  | CC MOD | PCC & SCC RB allocation | CC MOD | PCC NRB_alloc (LCRB @ RBstart) | CC MOD | SCC NRB_alloc (LCRB @ RBstart) |
| Test Parameters for CA_4A-4A Configurations |  |  |  |  |  |  |  |  |  |
| 1 | 25 | 25 | 35 | N/A | N/A | 16QAM | P_1@0 | 16QAM | S_1@24 |
| 2 | 25 | 25 | 35 | N/A | N/A | 16QAM | P_25@0 | 16QAM | S_25@0 |
| 3 | 25 | 25 | 5 | N/A | N/A | 16QAM | P_1@0 | 16QAM | S_1@24 |
| 4 | 25 | 25 | 5 | N/A | N/A | 16QAM | P_25@0 | 16QAM | S_25@0 |
| 5 | 100 | 100 | 5 | N/A | N/A | 16QAM | P_1@0 | 16QAM | S_1@99 |
| 6 | 100 | 100 | 5 | N/A | N/A | 16QAM | P_25@0 | 16QAM | S_25@75 |
| 7 | 100 | 100 | 5 | N/A | N/A | 16QAM | P_100@0 | 16QAM | S_100@0 |

Following subclauses introduce study for test environment, test frequencies, test channel bandwidth, test subcarrier spacing and uplink modulation and RB allocation aspects.

  - Test Environment

Based on the baseline agreed in [3], test environment for MPR measurement for CA can be defined as in LTE regardless of number of CCs.

Proposal 1: Define Test Environment as ‘Normal, TL/VL, TL/VH, TH/VL, TH/VH’ for MPR measurement for CA in FR1.

  - Test Frequencies

In LTE, MPR for CA is tested for Low and High range for PCC and SCC. For NR it is proposed to adopt the same selection of test frequencies.

Proposal 2: For inter-band CA, select Low range for PCC and SCC and High range for PCC and SCC as test frequencies for MPR measurement for CA in FR1; For intra-band CA, select Low range for the aggregated BW, and High range for the aggregated BW.

  - Test Channel Bandwidths

In LTE, Lowest NRB_agg and Highest NRB_agg are tested for MPR for CA of all configurations. It is proposed to adopt the same selection for NR testing.

Proposal 3: Select the Lowest NRB_agg and Highest NRB_agg Channel Bandwidth as test channel bandwidths for MPR measurement for CA in FR1

  - Test Subcarrier Spacing

Lowest and Highest SCS are tested in MPR for single carrier case in FR1, it’s reasonable to select the same SCS for CA test cases regardless of number of CCs.

Proposal 4: Select Lowest and Highest SCS supported for UE for MPR measurement for CA in FR1.

  - Uplink Modulation and RB allocation

    - Inter-band CA

In the MPR testing for inter-band CA, the selection of test point for uplink modulation and RB allocation should be based on the Minimum conformance requirements in [2], which is defined as below:

For inter-band carrier aggregation with uplink assigned to two NR bands, the requirements in subclause 6.2.2 apply for each uplink component carrier.

It is proposed to select the same uplink configuration as single carrier testing on each UL CC for MPR inter-band CA testing.

In addition, in LTE there is a test point for each aggregated BW verifying the minimum MPR on PCC and maximum MPR on SCC. This approach can be followed in NR.

Proposal 5: For MPR inter-band CA testing, it is proposed to select the same uplink modulation and RB allocation as single carrier testing on each UL CC. In addition for each aggregated BW a test point verifying the minimum MPR on PCC and maximum MPR on SCC shall be defined.

    - Intra-band contiguous CA (contiguous RB allocation)

In the MPR testing for For intra-band contiguous UL CA, the selection of test point for uplink modulation and RB allocation should be based on the Minimum conformance requirements in [2], which is defined as below:

In case the modulation format is different on different component carriers then the MPR is determined by the rules applied to higher order of those modulations.

Table 6.2A.2.1-1: Contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | QPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | 16QAM | 1.5 | 3.5 | 2.5 | 7 |
|  | 64QAM | 3.0 | 4.0 | 5 | 7 |
|  | 256QAM | 5.5 | 6.0 | 7 | 7.5 |
| CP-OFDM | QPSK | 2.0 | 4.0 | 3.5 | 8 |
|  | 16QAM | 2.5 | 4.0 | 3.5 | 8 |
|  | 64QAM | 3.5 | 4.0 | 5 | 8 |
|  | 256QAM | 6.5 | 6.5 | 7 | 8 |

Table 6.2A.2.1-1a: Contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 2.0 | 4.01 | 2.5 | 7 |
|  | QPSK | 2.0 | 4.01 | 2.5 | 7 |
|  | 16QAM | 2.5 | 4.01 | 2.5 | 7 |
|  | 64QAM | 3.0 | 4.51 | 5 | 7 |
|  | 256QAM | 5.5 | 6.0 | 7 | 7.5 |
| CP-OFDM | QPSK | 2.5 | 5.01 | 3.5 | 8 |
|  | 16QAM | 3.0 | 5.01 | 3.5 | 8 |
|  | 64QAM | 3.5 | 5.01 | 5 | 8 |
|  | 256QAM | 6.5 | 6.5 | 7 | 8 |
| NOTE 1: When 1 RB or 2 RB are allocated at the lower edge of lowest CC or upper edge of upper CC, MPR for outer is 5.5 dB. |  |  |  |  |  |

Table 6.2A.2.1-1b: Contiguous RB allocation for Power Class 2 with dual Tx2

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 3.0 | 5.01 | 3.5 | 8 |
|  | QPSK | 3.0 | 5.01 | 3.5 | 8 |
|  | 16QAM | 3.5 | 5.01 | 3.5 | 8 |
|  | 64QAM | 4.0 | 5.51 | 6 | 8 |
|  | 256QAM | 6.5 | 7.0 | 8 | 8.5 |
| CP-OFDM | QPSK | 3.0 | 5.51 | 4.0 | 8.5 |
|  | 16QAM | 3.5 | 5.51 | 4.0 | 8.5 |
|  | 64QAM | 4.0 | 5.51 | 5.5 | 8.5 |
|  | 256QAM | 7.0 | 7.0 | 7.5 | 8.5 |
| NOTE 1: When 1 RB or 2 RB are allocated at the lower edge of lowest CC or upper edge of upper CC, MPR for outer is 5.5 dB. NOTE 2: UE indicating TxD supported |  |  |  |  |  |

Table 6.2A.2.1-2: non-contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer11 | Outer22 | inner | Outer11 | Outer22 |
| DFT-s-OFDM | Pi/2 BPSK | 2 | 5.5 | 11.5 | 2.5 | 6 | 13 |
|  | QPSK | 2 | 5.5 |  | 2.5 | 6 |  |
|  | 16QAM | 2.5 | 5.5 |  | 3 | 6 |  |
|  | 64QAM | 4.5 | 6 |  | 5 | 6 |  |
|  | 256QAM | 6 | 6.5 |  | 6.5 | 6.5 |  |
| CP-OFDM | QPSK | 2.5 | 6.5 | 12 | 3.5 | 7 | 14 |
|  | 16QAM | 3 | 7 |  | 3.5 | 7 |  |
|  | 64QAM | 5 | 7 |  | 5 | 7 |  |
|  | 256QAM | 7.5 | 7.5 |  | 7.5 | 7.5 |  |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz |  |  |  |  |  |  |  |

Table 6.2A.2.1-3: non-contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer12 | Outer23 | Inner | Outer12 | Outer23 |
| DFT-s-OFDM | Pi/2 BPSK | 31 | 6.5 | 13 | 31 | 7.5 | 13.5 |
|  | QPSK | 31 | 6.5 |  | 31 | 7.5 |  |
|  | 16QAM | 31 | 6.5 |  | 31 | 7.5 |  |
|  | 64QAM | 5 | 6.5 |  | 5 | 7.5 |  |
|  | 256QAM | 6.5 | 7 |  | 6.5 | 7.5 |  |
| CP-OFDM | QPSK | 3.51 | 7 | 14 | 3.51 | 8 | 14.5 |
|  | 16QAM | 3.51 | 7 |  | 3.51 | 8 |  |
|  | 64QAM | 5 | 7 |  | 5 | 8 |  |
|  | 256QAM | 7.5 | 7.5 |  | 7.5 | 8 |  |
| NOTE 1: the allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. NOTE 2: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 3: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz |  |  |  |  |  |  |  |

Table 6.2A.2.1-4: non-contiguous RB allocation for Power Class 2 with dual Tx4

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer12 | Outer23 | Inner | Outer12 | Outer23 |
| DFT-s-OFDM | Pi/2 BPSK | 41 | 7.5 | 14 | 41 | 8.5 | 14.5 |
|  | QPSK | 41 | 7.5 |  | 41 | 8.5 |  |
|  | 16QAM | 41 | 7.5 |  | 41 | 8.5 |  |
|  | 64QAM | 6 | 7.5 |  | 6 | 8.5 |  |
|  | 256QAM | 7.5 | 8 |  | 7.5 | 8.5 |  |
| CP-OFDM | QPSK | 4.51 | 8 | 15 | 4.51 | 9 | 15.5 |
|  | 16QAM | 4.51 | 8 |  | 4.51 | 9 |  |
|  | 64QAM | 6 | 8 |  | 6 | 9 |  |
|  | 256QAM | 8.5 | 8.5 |  | 8.5 | 9 |  |
| NOTE 1: the allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. NOTE 2: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 3: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 4: UE indicating TxD supported |  |  |  |  |  |  |  |

Aiming to cover the MPR requirement in above table as much as possible in respect to modulations and RB allocations, the combinations of modulation and RB allocation given in tables above in [2] should be tested.

Since the MPR requirement for higher order of modulations among CCs are applied, selecting same modulations for PCC and SCC is enough for testing coverage. In LTE, MPR for Intra-band contiguous CA is tested for the combinations of modulation and the largest RB allocation for each MPR minimum conformance requirement. The same rule should be followed for NR FR1. Since MPR requirement are defined for inner and outer separately, it’s supported to test Inner Full and Outer Full for Intra-band contiguous UL CA with contiguous RB allocation.

Proposal 6: For intra-band contiguous UL CA with contiguous RB allocation, it is proposed to test the combinations of modulations and Inner Full / Outer Full RB allocations in table 6.2A.2.0.4-1.

Proposal 7: For intra-band contiguous UL CA with non-contiguous RB allocation, it is proposed to test the combinations of modulations and Inner / Outer 1 / Outer 2 RB allocations in table 6.2A.2.0.4-2.

Proposal 7a: The definition of Inner and Outer allocation is the same for PC3 and PC2, the same combinations of modulation and RB allocation are used for both PC3 and PC2.

    - Intra-band non-contiguous CA

In the MPR testing for For intra-band non-contiguous UL CA, the selection of test point for uplink modulation and RB allocation should be based on the Minimum conformance requirements in [2], which is defined as below:

For intra-band non-contiguous CA, the allowed Maximum Power Reduction (MPR) for the maximum output power is specified into 2 types: MPR to meet -30dBm/MHz and -13dBm/MHz. The UE determins the MPR type as follows:

If AND( FIM3,low_block,low > SEM-13,low ,  FIM3,high_block,high < SEM-13,high )

MPRIM3 defined in Clause 6.2A.2.2.2

Else If AND( FIM3,low_block,low > SEM-25,low ,  FIM3,high_block,high < SEM-25,high )

MPRIM3 defined in Clause 6.2A.2.2.1

Else

MPRIM3 defined in Clause 6.2A.2.2.1

where

-	FIM3,high_block,high = (2 * Fhigh_alloc,high_edge ) – Flow_alloc,low_edge

-	FIM3,low_block,low = (2 * Flow_alloc,low_edge) – Fhigh_alloc,high_edge

-	Flow_alloc,low_edge is the lowermost frequency of the lower transmission bandwidth allocation.

-	Flow_alloc,high_edge is the uppermost frequency of the lower transmission bandwidth allocation.

-	Fhigh_alloc,low_edge is the lowermost frequency of the upper transmission bandwidth allocation.

-	Fhigh_alloc,high_edge is the uppermost frequency of the upper transmission bandwidth allocation.

-	SEM-13,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.2.2.

-	SEM-13,high = Threshold frequency where upper spectral emission mask above the upper channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.2.2.

-	SEM-25,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -25 dBm / MHz to -30 dBm / MHz, as specified in Clause 6.5A.2.2.2.

-	SEM-25,high = Threshold frequency where upper spectral emission mask above the upper channel drops from -25 dBm / MHz to -30 dBm / MHz, as specified in Clause 6.5A.2.2.2.

6.2A.2.2.1	MPRIM3 to meet -30dBm/MHz

MPR in this clause is for intra-band non-contiguous CA power class 3 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	15; 	0 ≤ B < 1.08

14.5; 	1.08 ≤ B < 2.16

13.5; 	2.16 ≤ B < 3.24

12.5;       3.24 ≤ B < 5.04

11.5; 	5.04≤ B < 10.08

10.5; 	10.08 ≤ B < 16.38

10;        16.38 ≤ B < 21.78

9; 	      21.78 ≤ B

Where:

B=(LCRB_alloc, 1* 12* SCS1 + LCRB_alloc,2 * 12 * SCS2)/1,000,000

6.2A.2.2.2	MPRIM3 to meet -13dBm/MHz

MPR in this clause is for intra-band non-contiguous CA power class 3 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MA

Where MA is defined as follows

MA = 	9	;	 0 ≤ B < 0.54

8	;	 0.54 ≤ B < 1.08

7	; 	1.08 ≤ B < 2.16

6.5	; 	2.16 ≤ B < 3.24

5.5	; 	3.24 ≤ B < 5.4

4	; 	5.4 ≤ B

Where:

B=(LCRB_alloc, 1* 12* SCS1 + LCRB_alloc,2 * 12 * SCS2)/1,000,000

We can observe that the MPR requirements are specified into two types in terms of SEM requirements. Once the condition “FIM3,low_block,low > SEM-13,low ,  FIM3,high_block,high < SEM-13,high” is met, the requirements in 6.2A.2.2.2 applies. Otherwise the requirements in 6.2A.2.2.1 applies.

Observation 1: Two test configuration tables need to be introduced for different MPR minimum requirements.

The definitions in requirements for intra-band non-contiguous 2UL CA in 6.2A.2.2.2 is shown in the following photograph.

Fig.1 Intra-band non-contiguous 2 UL CA definitions

X is the distance from SEM-13,low to the lower edge of lower channel bandwidth. And Y is the distance from SEM-13,high to the higher edge of upper channel bandwidth.

- Condition “FIM3,low_block,low > SEM-13,low” can be detailed as below:

FIM3,low_block,low > SEM-13,low

- FIM3,low_block,low - SEM-13,low >0

- [(2 * Flow_alloc,low_edge) – Fhigh_alloc,high_edge] – [FCC1 -0.5*CBW1 -X] >0

- [2*(FCC1-0.5*NRB1*SCS1*12*0.001) - (FCC2+0.5*NRB2*SCS2*12*0.001)] – [FCC1 -0.5*CBW1 -X] >0

- [2FCC1-FCC2-FCC1] + [-(NRB1*SCS1*12*0.001) – (0.5*NRB2*SCS2*12*0.001) + 0.5*CBW1 +X] >0

- [FCC1-FCC2 +X] > (NRB1*SCS1*12*0.001) + (0.5*NRB2*SCS2*12*0.001) - 0.5*CBW1

- Condition “FIM3,high_block,high < SEM-13,high” can be detailed as below:

FIM3,high_block,high < SEM-13,high

- FIM3,high_block,high - SEM-13,high <0

- [(2 * Fhigh_alloc,high_edge ) – Flow_alloc,low_edge] - [FCC2+0.5*CBW2+Y] <0

- [2*(FCC2+0.5*NRB2*SCS2*12*0.001) - (FCC1-0.5*NRB1*SCS1*12*0.001)] - [FCC2+0.5*CBW2+Y] <0

- [2FCC2-FCC1-FCC2] + [(NRB2*SCS2*12*0.001) + (0.5*NRB1*SCS1*12*0.001) - 0.5*CBW2-Y] <0

- [FCC1-FCC2+Y] > (0.5*NRB1*SCS1*12*0.001) + (NRB2*SCS2*12*0.001) - 0.5*CBW2

Observation 2: When condition “[FCC1-FCC2 +X] > (NRB1*SCS1*12*0.001) + (0.5*NRB2*SCS2*12*0.001) - 0.5*CBW1” and “[FCC1-FCC2+Y] > (0.5*NRB1*SCS1*12*0.001) + (NRB2*SCS2*12*0.001) - 0.5*CBW2” are met, MPR requirements in TS 38.101-1 6.2A.2.2.2 MPRIM3 to meet -13dBm/MHz should apply. Otherwise MPR requirements in TS 38.101-1 6.2A.2.2.1 MPRIM3 to meet -30dBm/MHz should apply.

According to TS 38.101-1 6.5A.2.2.2, the SEM requirements for intra-band non-contiguous CA is extracted as below.

6.5A.2.2.2	Spectrum emission mask for intra-band non-contiguous CA

For intra-band non-contiguous carrier aggregation the spectrum emission mask requirement is defined as a composite spectrum emissions mask. Composite spectrum emission mask applies to frequencies up to ΔfOOB starting from the edges of the sub-blocks. Composite spectrum emission mask is defined as follows

a)	Composite spectrum emission mask is a combination of individual sub-block spectrum emissions masks

b)	In case the sub-block consist of one component carrier the sub-lock general spectrum emission mask is defined in subclause 6.5.2.1

c)	If for some frequency sub-block spectrum emission masks overlap then spectrum emission mask allowing higher power spectral density applies for that frequency

d)	If for some frequency a sub-block spectrum emission mask overlaps with the sub-block bandwidth of another sub-block, then the emission mask does not apply for that frequency.

When signalling for dualPA-Architecture IE is absent, carrier leakage or I/Q image may land inside the gap spectrum between 2 UL CCs when UL CCs are synchronized with frequencies in the gap.

Observation 3: When each sub-block consist of one component carrier, the SEM requirements in 6.5.2.1 applies for each UL carrier.

The SEM requirements in TS 38.101-1 6.5.2.1 is extracted as below:

Table 6.5.2.2-1: General NR spectrum emission mask

| Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Measurement bandwidth |
| ± 0-1 | -13 | -13 | -13 | -13 | -13 | -13 | -13 |  |  |  |  |  |  | 1 % channel bandwidth |
| ± 0-1 |  |  |  |  |  |  |  | -24 | -24 | -24 | -24 | -24 | -24 | 30 kHz |
| ± 1-5 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| ± 5-6 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 |  |
| ± 6-10 | -25 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 10-15 |  | -25 |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 15-20 |  |  | -25 |  |  |  |  |  |  |  |  |  |  |  |
| ± 20-25 |  |  |  | -25 |  |  |  |  |  |  |  |  |  |  |
| ± 25-30 |  |  |  |  | -25 |  |  |  |  |  |  |  |  |  |
| ± 30-35 |  |  |  |  |  | -25 |  |  |  |  |  |  |  |  |
| ± 35-40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 40-45 |  |  |  |  |  |  | -25 |  |  |  |  |  |  |  |
| ± 45-50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 50-55 |  |  |  |  |  |  |  | -25 |  |  |  |  |  |  |
| ± 55-60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 60-65 |  |  |  |  |  |  |  |  | -25 |  |  |  |  |  |
| ± 65-70 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 70-75 |  |  |  |  |  |  |  |  |  | -25 |  |  |  |  |
| ± 75-80 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 80-85 |  |  |  |  |  |  |  |  |  |  | -25 |  |  |  |
| ± 85-90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 90-95 |  |  |  |  |  |  |  |  |  |  |  | -25 |  |  |
| ± 95-100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 100-105 |  |  |  |  |  |  |  |  |  |  |  |  | -25 |  |

As per the definition of SEM-13,low and SEM-13,high :

-	SEM-13,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.2.2.

-	SEM-13,high = Threshold frequency where upper spectral emission mask above the upper channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.2.2.

It’s clearly that X and Y is associated with channel bandwidth of each carrier. The values for X and Y can be summarized in table 1.

Table 1: Values specified for X and Y for each channel bandwidth

| CBW | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value | 6 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |

Observation 4：X and Y values are specified in table 1.

Proposal 8: Test configuration tables 6.2A.2.1.4.1-3a and 6.2A.2.1.4.1-3b will be introduced into TS 38.521-1 respectively for “MPRIM3 to meet -13dBm/MHz” and “MPRIM3 to meet -30dBm/MHz”.

The MPRIM3 to meet -13dBm/MHz requirements is defined as:

MPR=MA Where MA is defined as follows

MA = 	9	;		0 ≤ B < 0.54

8	;		0.54 ≤ B < 1.08

7	; 		1.08 ≤ B < 2.16

6.5	; 		2.16 ≤ B < 3.24

5.5	; 		3.24 ≤ B < 5.4

4	; 		5.4 ≤ B

Where:

B=(LCRB_alloc, 1* 12* SCS1 + LCRB_alloc,2 * 12 * SCS2)/1,000,000

It’s clearly the MPR values varies with the RB allocations on both carriers. The test points should cover all the ranges of B. It’s reasonable to allocate RBs on both CCs ensuring the calculated B being the median of each range.

In LTE, 1@0 on PCC and 1@NRB2-1 on SCC RB allocation, and Full RB allocation on both CCs are configured. It’s reasonable to configure these two RB allocations in NR as well to cover the most critical scenario. Considering the MPR requirements are associated with LCRB, Outer_Full RB allocation for each CC should be included in the testing of last range of B.

Table 2 RB allocations for MPRIM3 to meet -13dBm/MHz

| Categories of B | Median of B | Total RB allocations = LCRB_alloc, 1 + LCRB_alloc, 2 |  |  |
| --- | --- | --- | --- | --- |
|  |  | SCS = 15kHz | SCS = 30kHz | SCS = 60kHz |
| 0 ≤ B < 0.54 | 0.27 | Edge_1RB_Left for PCC, Edge_1RB_Right for SCC |  |  |
|  |  | 16=8+8 | 8=4+4 | 4=2+2 |
| 0.54 ≤ B < 1.08 | 0.81 | 46 =23+23 | 24= 12+12 | 12 = 6+6 |
| 1.08 ≤ B < 2.16 | 1.62 | 90 =45+45 | 46= 23+23 | 24=12+12 |
| 2.16 ≤ B < 3.24 | 2.7 | 150 =75+75 | 76 =38+38 | 38 =19+19 |
| 3.24 ≤ B < 5.4 | 4.32 | 240 = 120+120 | 120 = 60+60 | 60=30+30 |
| 5.4 ≤ B | 5.4 | 300 = 150+150 | 150 =75+75 | 75= 38+38 |
|  |  | Outer_Full for PCC and SCC |  |  |

Following the same rule, the RB allocations for MPRIM3 to meet -30dBm/MHz is specified in table 3:

Table 3 RB allocations for MPRIM3 to meet -30dBm/MHz

| Categories of B | Median of B | Total RB allocations = LCRB_alloc, 1 + LCRB_alloc, 2 |  |  |
| --- | --- | --- | --- | --- |
|  |  | SCS = 15kHz | SCS = 30kHz | SCS = 60kHz |
| 0 ≤ B < 1.08 | 0.54 | Edge_1RB_Left for PCC, Edge_1RB_Right for SCC |  |  |
|  |  | 30 = 15+15 | 15 =8+8 | 8 =4+4 |
| 1.08 ≤ B < 2.16 | 1.62 | 90 = 45+45 | 45 =23+23 | 24=12+12 |
| 2.16 ≤ B < 3.24 | 2.7 | 150 =75+75 | 75 =38+38 | 38 =19+19 |
| 3.24 ≤ B < 5.04 | 4.14 | 230 =115+115 | 115 =58+58 | 58 =29+29 |
| 5.04≤ B < 10.08 | 7.56 | 420 =210+210 | 210=105+105 | 105 =53+53 |
|  |  | Outer_Full for PCC and SCC |  |  |

In LTE, MPR for Intra-band non-contiguous CA is tested for all of modulations for each MPR minimum conformance requirement. The same rule should be followed for NR FR1. It’s supposed to test the combinations of modulations RB allocations in table 2 or table 3 for Intra-band non-contiguous UL CA.

Proposal 9: For intra-band non-contiguous UL CA, it is proposed to test the combinations of modulations and RB allocations in table 2 for MPRIM3 to meet -13dBm/MHz.

Proposal 10: For intra-band non-contiguous UL CA, it is proposed to test the combinations of modulations and RB allocations in table 3 for MPRIM3 to meet -30dBm/MHz.

  - Number of Test Points

Based on above proposals, the total test points of MPR for CA are tabulated below.

Table 2.6-1 Total test points of MPR for inter-band CA

| Environment conditions | Test Frequencies | Test ChBW | Test SCS | Test ID | Total Test Points |
| --- | --- | --- | --- | --- | --- |
| 5 | 2 | 2 | 2 | 36 | 1440 |

Table 2.6-2 Total test points of MPR for intra-band contiguous CA (contiguous RB allocation)

| Environment conditions | Test Frequencies | Test ChBW | Test SCS | Test ID | Total Test Points |
| --- | --- | --- | --- | --- | --- |
| 5 | 2 | 2 | 2 | 18 | 720 |

Table 2.6-3 Total test points of MPR for intra-band contiguous CA (non-contiguous RB allocation)

| Environment conditions | Test Frequencies | Test ChBW | Test SCS | Test ID | Total Test Points |
| --- | --- | --- | --- | --- | --- |
| 5 | 2 | 2 | 2 | 27 | 1080 |

Table 2.6-4 Total test points of MPR for intra-band non-contiguous CA

| Environment conditions | Test Frequencies | Test ChBW | Test SCS | Test ID | Total Test Points |
| --- | --- | --- | --- | --- | --- |
| 5 | 2 | 2 | 2 | 63 | 2520 |

- Conclusion

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 Maximum Power Reduction test case definition:

Proposal 1: Define Test Environment as ‘Normal, TL/VL, TL/VH, TH/VL, TH/VH’ for MPR measurement for CA in FR1.

Proposal 2: Select Low range for PCC and SCC and High range for PCC and SCC as test frequencies for MPR measurement for CA in FR1

Proposal 3: Select the Lowest NRB_agg and Highest NRB_agg Channel Bandwidth as test channel bandwidths for MPR measurement for CA in FR1

Proposal 4: Select Lowest and Highest SCS supported for UE for MPR measurement for CA in FR1.

Proposal 5: For MPR inter-band CA testing, it is proposed to select the same uplink modulation and RB allocation as single carrier testing on each UL CC. In addition for each aggregated BW a test point verifying the minimum MPR on PCC and maximum MPR on SCC shall be defined.

Proposal 6: For intra-band contiguous UL CA with contiguous RB allocation, it is proposed to test the combinations of modulations and Inner Full / Outer Full RB allocations in table 6.2A.2.0.4-1.

Proposal 7: For intra-band contiguous UL CA with non-contiguous RB allocation, it is proposed to test the combinations of modulations and Inner / Outer 1 / Outer 2 RB allocations in table 6.2A.2.0.4-2.

Proposal 7a: The definition of Inner and Outer allocation is the same for PC3 and PC2, the same combinations of modulation and RB allocation are used for both PC3 and PC2.

Proposal 8: Test configuration tables 6.2A.2.1.4.1-3a and 6.2A.2.1.4.1-3b will be introduced into TS 38.521-1 respectively for “MPRIM3 to meet -13dBm/MHz” and “MPRIM3 to meet -30dBm/MHz”.

Proposal 9: For intra-band non-contiguous UL CA, it is proposed to test the combinations of modulations and RB allocations in table 2 for MPRIM3 to meet -13dBm/MHz.

Proposal 10: For intra-band non-contiguous UL CA, it is proposed to test the combinations of modulations and RB allocations in table 3 for MPRIM3 to meet -30dBm/MHz.

- References

[1] 3GPP TS 38.521-1 V17.3.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone; (Release 16)

[2] 	3GPP TS 38.101-1 v16.10.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 16)

[3]	R5-180065, “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.

[4] 	R5-192024,” Discussion on the waveform selection for UL MIMO tests”, Huawei, RAN5#82, Athens, Feb.2019