3GPP TSG-RAN WG5 Meeting # 96-e	R5-224892r1

# Electronic Meeting, 15th – 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 0654 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Ref sensitivity TP selection for DC_5A_n78A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Austria RFFE GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Reference sensitivity test points selection for DC_5A_n78A is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test points selections for DC_5A_n78A. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will not be aligned with the core specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-224892r1 changes Added combos DC_5_n78 to table D.2.8-1 |  |  |  |  |  |  |  |  |

5

<< Start of changes >>

### 4.3.3	Test point analysis per EN-DC configuration

#### 4.3.3.1	Reference sensitivity test cases for EN-DC

This clause contains information on test point analysis and status for FR1 EN-DC test cases in TS 38.521-3 [4] clause 7. The analyses are performed per EN-DC configuration in Table 4.3.3.1-1, Table 4.3.3.1-2 and Table 4.3.3.1-3.

Table 4.3.3.1-1: Reference Sensitivity test cases per EN-DC configuration (2CC)

| Band or band configuration | Single UL | Refsens exception, victim band (NOTE 2) | Requirement coverage (NOTE 2) | Comments |
| --- | --- | --- | --- | --- |
| DC_(n)XAA |  |  |  |  |
| 5 | Only single UL requirements defined | 5 | Exception | Added at RAN5#92-e |
| 12 | Only single UL requirements defined | 12 | Exception | Added at RAN5#92-e |
| 38 | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| 41 | Yes | N/A | NE | Added at RAN5#88-e |
| 48 | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| 71 | Yes | 71 | Exception | Added at RAN5#88-e |
| DC_XA_nXA |  |  |  |  |
| DC_2A_n2A | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_3A_n3A | Yes | 3 | Exception | Added at RAN5#88-e |
| DC_5A_n5A | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_7A_n7A | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_41A_n41A | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_48A_n48A | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_66A_n66A | Only single UL requirements defined | N/A | NE | Added at RAN5#88-e |
| DC_71A_n71A | Only single UL requirements defined | N/A | NE | Added at RAN5#92-e |
| DC_XA-nYA (NOTE 1) |  |  |  |  |
| DC_1A_n3A | Yes | 1 (IMD3) CBI | NE IMD3 CBI, CBI avoid | Added at RAN5#89-e |
| DC_1A-n5A | No | 1 (IMD4) | NE IMD4 | Added at RAN5#94-e |
| DC_1A-n7A | No | N/A | NE | Added at RAN5#94-e |
| DC_1A_n8A | No | 1 (IMD4) | NE IMD4 | Added at RAN5#95-e |
| DC_1A_n78A | No | 1 (IMD4) | NE IMD4 | Added at RAN5#88-e |
| DC_2A_n66A | No | 3 (IMD3) n66 (IMD5) | NE IMD3 IMD5 | Added at RAN5#94-e |
| DC_2A_n71A | No | 2 (HD3) n71 (HM) | NE HD HM | Added at RAN5#93-e |
| DC_2A_n77A | No | n77 (HD) 2 (HM) 2 (IMD2) 2 (IMD4) 2 (IMD5) | NE HD, HD avoid HM, HM avoid IMD2 PC2&PC3 IMD4 PC2&PC3 IMD5 PC3 | Added at RAN5#94-e |
| DC_2A_n78A | No | n78 (HD2) 2 (IMD2) 2 (IMD4) | NE HD IMD2 IMD4 | Added at RAN5#94-e |
| DC_3A_n1A | Yes | 1 (IMD3) CBI | NE IMD3 CBI, CBI avoid | Added at RAN5#89-e |
| DC_3A_n5A | Yes | 3 (IMD4) n5 (IMD2) | NE IMD2 IMD4 | Added at RAN5#94-e |
| DC_3A_n28A | No | N/A | NE | Added at RAN5#95-e |
| DC_3A_n78A | Yes | 3 (IMD2) 3 (IMD4) 3 (HD) n78 (HM) | NE IMD2 PC2&PC3 IMD4 PC2&PC3 HD HM, HM avoid | Added at RAN5#89-e |
| DC_5A_n66A | Yes | 5 (IMD2) | NE (anchor agnostic) IMD2 | Added at RAN5#93-e |
| DC_5A_n77A | No | n77 (HD) 5 (IMD4) 5 (IMD5) | NE HD, HD avoid IMD4 PC2&PC3 IMD5 PC3 | Added at RAN5#94-e |
| DC_5A_n78A | No | n78 (HD4) B5 (IMD4) | NE HD, HD avoid IMD4 | Added at RAN5#96-e |
| DC_7A_n1A | No | N/A | NE | Added at RAN5#90-e |
| DC_7A_n3A | No | 7 (IMD4¦3*fB3-1*fB7¦) | NE, IMD4 | Added at RAN5#90-e |
| DC_7A_n5A | Yes | n5 (IMD3) | NE, IMD3 | Added at RAN5#94-e |
| DC_7A_n8A | No | N/A | NE | Added at RAN5#95-e |
| DC_7A_n28A | No | N/A | NE | Added at RAN5#94-e |
| DC_7A_n78A | No | CBI | NE, CBI | Added at RAN5#94-e |
| DC_8A_n1A | No | n1 (IMD4 ¦2*fB1-2*fB8¦) | NE, IMD4 | Added at RAN5#90-e |
| DC_8A_n3A | No | 8 (IMD4 ¦3*fB8-1*fB3¦), n3 (IMD5 ¦4*fB8-1*fB3¦) | NE, IMD4, IMD5 | Added at RAN5#90-e |
| DC_8A_n20A | Yes | 8 (IMD3) n20 (IMD3) | NE IMD3 | Added at RAN5#94-e |
| DC_8A_n28A | No | N/A | NE | Added at RAN5#95-e |
| DC_8A_n77A | No | 8 (HD4), 8 (IMD4) | NE HD, HD avoid IMD4 | Added at RAN5#92-e |
| DC_11A_n77A | No | N/A | NE | Added at RAN5#91-e |
| DC_11A_n78A | No | N/A | NE | Added at RAN5#91-e |
| DC_11A_n79A | No | 11 (HM) | NE HM, HM avoid | Added at RAN5#92-e |
| DC_13A_n77A | No | n77 (HD) 13 (HM) 13 (IMD5) | NE HD, HD avoid HM, HM avoid IMD5 PC2&PC3 | Added at RAN5#94-e |
| DC_19A_n79A | No | 19 (HM ¦5*fB19DL-1*fB79¦), | NE, HM, HM avoid | Added at RAN5#90-e |
| DC_20A_n1A | No | N/A | NE | Added at RAN5#90-e |
| DC_20A_n3A | No | n3 (IMD4), 20 (IMD4) | NE IMD4 | Added at RAN5#89-e |
| DC_20A_n7A | DC_20_n7 | 20 (IMD3) | NE IMD3 | Added at RAN5#94-e |
| DC_20A_n8A | DC_20_n8 | 20 (IMD3) n8 (IMD3) | NE IMD3 | Added at RAN5#95-e |
| DC_20A_n78A | No | 20 (IMD4), n78 (HD) | NE IMD4, HD, HD avoid | Added at RAN5#88-e |
| DC_25A_n41A | No | N/A | NE | Added at RAN5#91-e |
| DC_26A_n41A | No | n41 (HD3), 26 (IMD3) | NE HD, HD avoid IMD | Added at RAN5#92-e |
| DC_26A_n77A | No | n77 (HD4), 26 (IMD4) | NE HD, HD avoid IMD | Added at RAN5#92-e |
| DC_26A_n78A | No | n78 (HD4), 26 (IMD4) | NE HD, HD avoid IMD | Added at RAN5#92-e |
| DC_26A_n79A | No | 26 (HM ¦4*fB26DL-1*fB79¦) | NE HM, HM avoid | Added at RAN5#92-e |
| DC_28A_n5A | No | CBI | NE CBI, CBI avoid | Added at RAN5#94-e |
| DC_28A_n7A | No | N/A | NE | Added at RAN5#94-e |
| DC_28A_n77A | No | n77 (HD), n77 (HM), 28 (IMD5) | NE (HD avoid), HD, HM, IMD5 | Added at RAN5#92-e |
| DC_28A_n78A | No | n78 (HD), 28 (IMD5) | NE (HD avoid), HD, IMD5 | Added at RAN5#92-e |
| DC_40A_n1A | No | N/A | NE | Added at RAN5#89-e |
| DC_40A_n78A | No | 40 (HM) | NE HM, HM avoid | Added at RAN5#89-e |
| DC_40A_n79A | No | N/A | NE | Added at RAN5#94-e |
| DC_41A_n77A (NOTE 3) | No | 41 (HM ¦3*fB41DL-2*fB77¦), 41 (CBI), n77 (CBI) | NE HM CBI | Added at RAN5#92-e |
| DC_41A_n78A (NOTE 3) | No | 41 (HM ¦3*fB41DL-2*fB77¦), 41 (CBI), n78 (CBI) | NE HM CBI | Added at RAN5#92-e |
| DC_42A_n77A | Only single UL requirements defined | N/A | NE | Added at RAN5#91-e |
| DC_48A_n66A | No | 48 (HD2) 66 (IMD5) | NE (anchor agnostic) HD2 IMD5 | Added at RAN5#92-e |
| DC_66A_n2A | DC_66_n2 | n2 (IMD3) 66 (IMD5) | NE (anchor agnostic) IMD3 IMD5 | Added at RAN5#93-e |
| DC_66A_n71A | No | 66 (IMD4) | NE (anchor agnostic) IMD4 | Added at RAN5#93-e |
| DC_66A_n77A | No | n77 (HD) 66 (IMD2) 66 (IMD5) | NE HD, HD avoid IMD2 PC2&PC3 IMD5 PC2&PC3 | Added at RAN5#94-e |
| DC_66A_n78A | No | n78 (HD2) 66 (IMD5) | NE (anchor agnostic) HD2 IMD5 | Added at RAN5#93-e |
| NOTE 1: If single UL is allowed in a DC_XA_nYA configuration the IMD requirements does not apply for UEs supporting UE capability singleUL-Transmission. NOTE 2: Notations used NE: Non-exception as defined in TS 38.101-3 [7], meaning standalone LTE in TS 36.101 [8] and NR in 38.101-1 [5] requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.1 HM: RX Harmonic Mixing, as defined in TS 38.101-3 [7], 7.3B.2.3.2 IMD: Intermodulation Distortion, as defined in TS 38.101-3 [7], 7.3B.2.3.5 CBI: Cross Band Isolation, as defined in TS 38.101-3 [7], 7.3B.2.3.4 HD avoid: single carrier requirements apply with aggressor of UL Harmonic Distortion active. HM avoid: single carrier requirements apply with aggressor of RX Harmonic Mixing active. CBI avoid: single carrier requirements apply with aggressor of Cross Band Isolation active. NOTE 3: HM cannot be avoided with aggressor still active since CBI exception always exists. Therefore, no HM avoid test point is added. |  |  |  |  |

<< Unchanged sections skipped>>

## D.2.8	Test Frequencies selections for EN-DC

In E-UTRA DL CA and UL CA reference sensitivity testing, Low range and High range are selected for intra-band CA testing. Mid-range is selected for inter band E-UTRA CA testing.

In standalone NR CA reference sensitivity testing, Low and High Range are tested as default for intra-band and Mid-range for inter-band. For CA combinations containing intra-band configuration in an inter-band CA configuration, low and high range are tested for intra-band CA and mid channel for inter-band without intra-band component.

The same principle shall apply for EN-DC testing.

For EN-DC configurations affected by exceptions, the test frequency cannot be freely chosen. One test frequency per exception requirement is sufficient to test the requirement. This is indicated in Table D.2.8-1 following the frequency relation formulas defined in clause D.2.3. Exceptions in TS 38.101-3 clause 7.3A.5 (2UL intermodulation) are not specified since the test frequency and channel bandwidth is already specified in TS 38.101-1.

Table D.2.8-1: Test frequency selection per band pair for exceptions avoidable by test frequency setting (UL harmonics, Rx mixing, cross band isolation)

| Band set (Note 2) | Frequency | Channel BW [MHz] | Exception type | Comments |
| --- | --- | --- | --- | --- |
| 1A / n3A | Mid/Mid | Highest/Highest | - | Non-Exception. |
|  | Low/High | 20 / 40 | Cross band isolation | Exception in TS 38.101-3 table 7.3B.2.3.4-1 |
| 2A/71A | DL1980 MHz/ UL665.5 MHz | 20/5 | UL Harmonic Interference | Exception Note 11 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | DL1980 MHz/ UL673 MHz | 20/10 | UL Harmonic Interference | Exception Note 12 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | UL1881 MHz/UL Low | 20/20 | Receiver Harmonic Mixing | Exception Note 4 in TS 38.101-3 Table 7.3B.2.3.2-1 |
| 2A/n77A | UL 1860 MHz / DL 3720 MHz | 20/100 | UL Harmonic Interference | Exception Note 2, 13 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | UL 1860 MHz / DL 3700 MHz | 20/20 | UL Harmonic Interference | Exception Note 3 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | DL Mid (1960) / UL 3920 | 20/100 | Receiver harmonic mixing HM2 | Receiver Harmonic Mixing Exception in TS38.101-3 Table 7.3B.2.3.2-1 |
|  | UL 1860 MHz / DL 3400 MHz |  |  | HD avoid |
|  | DL 1950 / UL 4000 | 20/100 |  | HM avoid |
| 2A/78A | DL 3740 MHz/ UL 1870 MHZ | Highest (20 MHz) / Highest (100 MHz) | UL Harmonic Interference HD2 | Exception Note 2, 13 of TS38.101-3 table 7.3B.2.3.1-1 |
|  | DL 3740 MHz/ UL 1885 MHz | 20 MHz) / Highest (20 MHz) | UL Harmonic Interference HD2 | Exception Note 3 of TS38.101-3 table 7.3B.2.3.1-1 |
| 3A / n1A | Mid/Mid | Highest/Highest | - | Non-Exception. |
|  | High/Low | 20 / 20 | Cross band isolation | Exception in TS 38.101-3 table 7.3B.2.3.4-1 |
| 3A / n41A |  |  |  | CMCC CR R5-205929 |
| 3A / n78A | Mid/High | Highest/Highest | - | Non-Exception. |
|  | Mid/3495 | Highest/Highest | UL Harmonic Interference | Exception Note 2 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | Mid/3525 | 20/20 | UL Harmonic Interference | Exception Note 3 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | Mid /3685 | 20/20 | Receiver Harmonic Mixing | Receiver Harmonic Mixing Exception in TS 38.101-3 Table 7.3B.2.3.2-1 |
| 5A/n77A | UL 829 MHz / DL 3316 MHz | 10/100 | UL Harmonic Interference | Exception Note 6, 7, 17 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | UL 829 MHz / DL 4145 MHz | 10/100 | UL Harmonic Interference | Exception Note 4, 5, 17 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | UL 840 MHz/DL 3600 MHz | 10/100 |  | HD avoid |
| 5A/n78A | UL 840 MHz / DL 3360 MHz | 10/100 | UL Harmonic Interference | Exception Note 6, 7, 17 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | UL 840 MHz / DL 3560.01 MHz | 10/100 |  | HD avoid |
|  | UL 844 MHz/UL 3421.005 MHz | 5/10 | Dual UL intermodulation | Exception in TS 38.101-3 Table 7.3B.2.3.5.1-1 |
| 7A / n78A | High/Low | 20/100 | Cross band isolation | Exception in TS 38.101-3 table 7.3B.2.3.4-1 |
| 8A / n77A | Mid (UL 897.5 MHz) / 3590 | Highest (10 MHz) / Highest (100 MHz) | UL Harmonic Interference HD4 | Exception Note 7 of TS38.101-3 table 7.3B.2.3.1-1 |
|  | Mid (UL 897.5 MHz) / 3520 | Highest (10 MHz) / Highest (100 MHz) |  | Non-Exception. Not overlapping interference since BWINT=40 MHz, FINT = (40+100)/2 = 70 |
| 11A / n79A | Mid (DL 1485.9 MHz) / 4457.7 | Highest (10 MHz) / Highest (100 MHz) | Receiver Harmonic Mixing HM3 | Receiver Harmonic Mixing Exception in TS38.101-3 Table 7.3B.2.3.2-1 |
|  | Mid (DL 1485.9 MHz) / 4512.7 MHz | Highest (10 MHz) / Highest (100 MHz) |  | Non-Exception. Not overlapping interference since BWINT=100 MHz, FINT = (100+10)/2 = 55 |
| 13A/n77A | UL 782 MHz / DL 3910 MHz | 10/100 | UL Harmonic Interference | Exception Note 4, 5 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | DL Mid (751) / UL 3755 | 10/100 | Receiver harmonic mixing | Receiver Harmonic Mixing Exception in TS38.101-3 Table 7.3B.2.3.2-1 |
|  | UL 782 MHz /DL 3600 MHz | 10/100 |  | HD avoid |
|  | DL 751 / UL 3950 | 10/100 |  | HM avoid |
| 19A / n79A | DL 884 MHz / Low (4420.02 MHz) | 10 MHz / 40 MHz | Receiver Harmonic Mixing HM3 | Receiver Harmonic Mixing Exception in TS38.101-3 Table 7.3B.2.3.2-1 |
|  | DL 884 MHz / Low (4445.02 MHz) | 10 MHz / 40 MHz |  | Non-Exception. Not overlapping interference since BWINT=40 MHz, FINT = (40+10)/2 = 25 |
| 20A / n78A | Mid/Mid | Highest/Highest | - | Non-Exception. |
|  | Mid/3388 | Highest/Highest | UL Harmonic Interference | Exception Note 7 of TS 38.101-3 table 7.3B.2.3.1-1 |
| 26A / n41A | High (UL 841.5 MHz) / 2524.5 MHz) | Highest (15 MHz) / 50 MHz | UL Harmonic Interference HD3 | Exception Note 9 of TS38.101-3 table 7.3B.2.3.1-1 |
|  | High (UL 841.5 MHz) / 2572 MHz | Highest (15 MHz) / 50 MHz |  | Non-Exception. Not overlapping interference since BWINT=45 MHz, FINT = (45+50)/2 = 47.5 |
| 26A / n77A 26A / n78A | High (UL 841.5 MHz) / 3366 MHz | Highest (15 MHz) / Highest (100 MHz) | UL Harmonic Interference HD4 | Exception Note 7 of TS38.101-3 table 7.3B.2.3.1-1 |
|  | High (UL 841.5 MHz) / 3446 MHz | Highest (15 MHz) / Highest (100 MHz) |  | Non-Exception. Not overlapping interference since BWINT=60 MHz, FINT = (60+100)/2 = 80 |
| 26A / n79A | High (DL 886.5 MHz) / 4432.5 MHz) | Highest (15 MHz) / 60 MHz | Receiver Harmonic Mixing HM5 | Receiver Harmonic Mixing Exception in TS38.101-3 Table 7.3B.2.3.2-1 |
|  | High (DL 886.5 MHz) / 4470.5 MHz) | Highest (15 MHz) / 60 MHz |  | Non-Exception. Not overlapping interference since BWINT=60 MHz, FINT = (60+15)/2 = 37.5 |
| 40A / n78A | Low/Mid | Highest/Highest | - | Non-Exception. |
|  | Mid/3525 | 20/20 | Receiver Harmonic Mixing | Receiver Harmonic Mixing Exception Note 8 in TS38.101-3 Table 7.3B.2.3.2-1 |
| 41A / n77A 41A / n78A | Low (DL 2593 MHz) / 3750 MHz) | Highest (20 MHz) / Highest (100 MHz) | Receiver Harmonic Mixing HM4 | Receiver Harmonic Mixing Exception in TS38.101-3 Table 7.3B.2.3.2-1 |
|  | Mid (DL 2593 MHz) / 3974.5 MHz) | Highest (20 MHz) / Highest (100 MHz) | Cross band isolation | Not overlapping HM interference since BWINT=240 MHz, FINT = (240+100)/2 = 170 |
|  | Mid / Mid | Highest / Highest | - | Non-Exception only possible with 1UL in band 41 |
| 48A/n66A | DL 3557,6 MHz/ UL 1778.8 MHZ | Highest (20 MHz) / Highest (400 MHz) | UL Harmonic Interference HD2 | Exception Note 2, 13 of TS38.101-3 table 7.3B.2.3.1-1 |
|  | DL 3581.0 MHz/ UL 1777.5 MHz | Highest (20 MHz) / Highest (400 MHz) | UL Harmonic Interference HD2 | Exception Note 3 of TS38.101-3 table 7.3B.2.3.1-1 |
| 66A/n77A | UL 1720 MHz / DL 3440 MHz | 20/100 | UL Harmonic Interference | Exception Note 2, 13 of TS38.101-3 table 7.3B.2.3.1-1 |
|  | UL 1720 MHz / DL 3420 MHz | 20/20 | UL Harmonic Interference | Exception Note 3 of TS38.101-3 table 7.3B.2.3.1-1 |
|  | UL 1720 MHz / DL 3600 MHz | 20/100 |  | HD avoid |
| 66A/n78A | DL 3510 MHz/ UL 1755 MHZ | Highest (20 MHz) / Highest (100 MHz) | UL Harmonic Interference HD2 | Exception Note 2, 13 of TS38.101-3 table 7.3B.2.3.1-1 |
|  | DL 3480 MHz/ UL 1755 MHz | Highest (20 MHz) / Highest (100 MHz) | UL Harmonic Interference HD2 | Exception Note 3 of TS38.101-3 table 7.3B.2.3.1-1 |
| NOTE 1: This selection is used in test case 7.3B.2.3 unless otherwise stated NOTE 2: Aggressor band in bold |  |  |  |  |  |

For EN-DC configurations affected by IMD exceptions where the exception is avoided, the test frequency and bandwidth are FFS.

<< End of changes >>