3GPP TSG-RAN5 Meeting #96-e	R5-225057r1

# Electronic, 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3192 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of test case 11.4.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test ,5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | After step 5 the UE has added the TAC of NR Cell 11 to the "list of 5GS forbidden TAs for roaming" and NR Cell 1 is the best (and only suitable) cell according to absolute priority reselection rules. In step 14 the UE is expected to camp on NR cell 1 that is on the same frequency as NR CEll 11 according to 38.508-1/table 4.4.2-1. But according to TS 38.304/5.2.4.4: " If the highest ranked cell or best cell according to absolute priority reselection rules is an intra-frequency or inter-frequency cell which is not suitable due to being part of the "list of 5GS forbidden TAs for roaming", the UE shall not consider this cell and other cells on the same frequency as candidates for reselection for a maximum of 300 seconds." |  |  |  |  |  |  |  |  |
| Summary of change: |  | NR Cell 11 has been replaced with NR Cell 23 which is on a different frequency. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will fail conformant UEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-225057->R5-225057r1 - WI on coversheet corrected - cell 11 is used change to cell 23 is reverted - NR cell 11 is turned off in step 13 |  |  |  |  |  |  |  |  |

### 11.4.5	5GMM-REGISTERED.LIMITED-SERVICE / 5GMM-IDLE / Emergency call establishment and release / Handling of 5GS forbidden tracking areas for roaming

11.4.5.1	Test Purpose (TP)

(1)

with { UE in 5GMM-REGISTERED.LIMITED-SERVICE state and 5GMM-IDLE mode }

ensure that {

when { UE is requested to make an Emergency call }

then { UE establishes the Emergency call }

}

(2)

with { UE in 5GMM-REGISTERED.LIMITED-SERVICE state and 5GMM-IDLE mode having established an Emergency call }

ensure that {

when { UE is requested to release the Emergency call }

then { UE releases the Emergency call, and, the UE considers the current cell as belonging to 5GS forbidden tracking areas for roaming }

}

11.4.5.2	Conformance requirement

References: The conformance requirements covered in the present TC are specified in: TS 24.501 [22], subclause 5.5.1.3.5, TS 23.122 [38], subclause 3.4.2. Unless otherwise stated these are Rel-15 requirements.

[TS 24.501, subclause 5.5.1.3.5]

#15	(No suitable cells in tracking area).

The UE shall set the 5GS update status to 5U3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.2.2). The UE shall reset the registration attempt counter and shall enter the state 5GMM-REGISTERED.LIMITED-SERVICE. The UE shall search for a suitable cell in another tracking area according to 3GPP TS 38.304 [28].

The UE shall store the current TAI in the list of "5GS forbidden tracking areas for roaming" and shall remove the current TAI from the stored TAI list, if present.

If the UE is operating in single-registration mode, the UE shall handle the EMM parameters EMM state, EPS update status and tracking area updating attempt counter as specified in 3GPP TS 24.301 [15] for the case when the normal tracking area updating procedure is rejected with the EMM cause with the same value.

[TS 23.122, subclause 3.4.2]

The MS is not allowed to request 5GS services except emergency services when camped on a cell of a TA of which belongs to the list of "5GS forbidden tracking areas for regional provision of service".

11.4.5.3	Test description

11.4.5.3.1	Pre-test conditions

System Simulator:

-	2 NR Cells

-	NR Cell 1 and NR Cell 11 as defined in TS 38.508-1 [4], Table 4.4.2-3.

-	On all cells when active: System information combination NR-1 as defined in TS 38.508-1 [4], subclause 4.4.3.1.2. SIB1 indicates ims-EmergencySupport.

UE:

None.

Preamble:

-	Cell configuration " in accordance with TS 38.508-1 [4], Table 4.4.2-3:

-	NR Cell 1 "Serving cell"

-	NR Cell 11 "Non-Suitable "Off" cell"

-	The UE is in test state 1N-A as defined in TS 38.508-1 [4], subclause 4.4A.2 on NR Cell 1

-	During the initial registration:

-	In the list of tracking areas provided by the AMF (IE 'TAI list') contains only the TAI of NR Cell 1.

11.4.5.3.2	Test procedure sequence

Table 11.4.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - NR Cell 11 as "Serving cell" - NR Cell 1 as "Non-Suitable "Off" cell". | - | - | - | - |
| - | EXCEPTION: The following messages are to be observed on NR Cell 11 unless explicitly stated otherwise. | - | - | - | - |
| 2 | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 3 | SS transmit an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 4 | The UE transmits an RRCSetupComplete message to confirm the successful completion of the connection establishment and a REGISTRATION REQUEST message indicating "mobility registration updating" is sent to update the registration of the actual tracking area. | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | - | - |
| 5 | SS sends a REGISTRATION REJECT message containing 5GMM cause value = #15 (No suitable cells in tracking area). | <-- | NR RRC: DLInformationTransfer 5GMM: REGISTRATION REJECT | - | - |
| 6 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 7 | Make the UE attempt an IMS emergency call dialling an emergency number e.g. 112 or 911). (NOTE 1) | - | - | - | - |
| 8 | Check: Does the UE performs Generic Test Procedure for IMS Emergency call establishment without IMS Emergency registration as specified in TS 38.508-1 [4], subclause 4.9.12? | - | - | 1 | P |
| 9 | Make the UE release the emergency call. (NOTE 1) | - | - | - | - |
| 10 | Void | - | - | - | - |
| 10A-10C | Steps 1-3 of the Generic test procedure for IMS MO Emergency call release as specified in TS 38.508-1 [4], subclause 4.9.12A take place. | - | - | - | - |
| 11 | Void | - | - | - | - |
| 12 | Start Timer T1=5 seconds. NOTE: This is an arbitrary value to wait for UE to initiate deregistration. | - | - | - | - |
| - | EXCEPTION: In parallel to step 12A1 below, the steps specified in Tables 11.4.5.3.2-2, and 11.4.5.3.2-3 may take place | - | - | - | - |
| 12A1 | Timer T1=5 seconds expires | - | - | - | - |
| 12a1-12b1 | Void | - | - | - | - |
| 12A | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 12B | Make the UE attempt an IMS non-emergency call. (NOTE 2) | - | - | - | - |
| 12C | Check: Does the UE transmit an RRCSetupRequest message for initial registration procedure on NR Cell 11 within the next 10 seconds? | --> | NR RRC: RRCSetupRequest | 2 | F |
| 13 | The SS configures: - NR Cell 1 as "Suitable neighbour intra-frequency cell" - NR Cell 11 as "Non-suitable cell". | - | - | - | - |
| 14 | Check: Does the UE perform on NR Cell 1 the Registration procedure for initial registration as specified in TS 38.508-1 [4] subclause 4.5.2.2-2, 'connected without release'? | - | - | 2 | P |
| NOTE 1: This could be done by e.g. MMI or AT command. NOTE 2: This could be done by e.g. MMI or AT command. |  |  |  |  |  |

Table 11.4.5.3.2-2: Parallel Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits a DEREGISTRATION REQUEST message with De-registration type IE set to "Normal de-registration". | --> | NR RRC: ULInformationTransfer 5GMM: DEREGISTRATION REQUEST | - | - |
| 2 | The SS transmits a DEREGISTRATION ACCEPT message. | <-- | NR RRC: DLInformationTransfer 5GMM: DEREGISTRATION ACCEPT | - | - |

Table 11.4.5.3.2-3: Parallel Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits a PDU SESSION RELEASE REQUEST message. | --> | NR RRC: ULInformationTransfer PDU SESSION RELEASE REQUEST | - | - |
| 2 | The SS transmits a PDU SESSION RELEASE COMMAND message. | <-- | NR RRC: DLInformationTransfer NR NAS: PDU SESSION RELEASE COMMAND | - | - |
| 3 | Check: Does the UE transmit a PDU SESSION RELEASE COMPLETE message? | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION RELEASE COMPLETE | - | - |

11.4.5.3.3	Specific message contents

Table 11.4.5.3.3-1: REGISTRATION REJECT (step 5, Table 11.4.5.3.2-1)

| Derivation path: TS 38.508-1 [4] table 4.7.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GMM cause | ‘00001111’B | #15 (No suitable cells in tracking area). |  |