3GPP TSG-RAN5 Meeting #96-e	R5-225115r1

# Electronic Meeting, 15th – 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1899 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of UL MIMO A-MPR test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Region of Test ID 13 in Table 6.2D.3.4.1-10 is incorrectly stated as A2. Test ID 13 has an allocation of 16@0, i.e. RBend=15 with an equivalent frequency of 2.7 MHz. Based on Table 6.2.3.3.20-1 this is an A1 allocation and the associated A-MPR is 5 dB (Table 6.2.3.3.20-2). The Tolerance in the test requirement Tables 6.2D.3.5-x is not correct since here TLOW(PCMAX_L,c) of Table 6.2D.4.3-1 applies. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Region of Test ID 13 has been corrected in Table 6.2D.3.4.1-10. The test requirement in Tables 6.2D.3.5-x have been updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test requirement remains incorrect and a conformant UE can fail the test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2D.3.4.1, 6.2D.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: removed change to Table 6.2D.3.5-1 in order to resolve overlap with CR R5-224829. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

### 6.2D.3	UE additional maximum output power reduction for UL MIMO

6.2D.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated a unique with network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band (the IE freqBandIndicatorNR) and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2D.1.3-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

6.2D.3.2	Test applicability

The requirements of this test apply in test case 6.5D.2.3 Additional Spectrum Emission mask for UL MIMO for network signalling value NS_03, NS_03U, NS_04 to all types of NR UE release 15 and forward that support UL MIMO.

The requirements of this test apply in test case 6.5D.2.4.2 Adjacent channel leakage ratio for network signalling values NS_03U, NS_05U, and NS_100 to all types of NR Power Class 2 and 3 UE release 15 and forward.

The requirements of this test apply in test case 6.5D.3.3 Additional Spurious Emissions for network signalling value NS_04 to all types of NR UE release 15 and forward that support UL-MIMO.

6.2D.3.3	Minimum conformance requirements

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the A-MPR values specified in subclause 6.2.3.3 shall apply to the maximum output power specified in Table 6.2D.1.3-1. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1.3-2. For UE supporting UL MIMO, the maximum output power is measured as the sum of the maximum output power at each UE antenna connector. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For the UE maximum output power modified by A-MPR, the power limits specified in subclause 6.2D.4.3 apply.

If UE is configured for transmission on single-antenna port, the requirements in subclause 6.2.3.3 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2D.3.

6.2D.3.4	Test description

6.2D.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in tables 6.2D.3.4.1-1 to 6.2D.3.4.1-11. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Table 6.2D.3.4.1-1: Test Configuration table for NS_04

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | (See Freq column) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_04 |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |  |
| Test ID | Freq |  |  | N/A | Modulation (NOTE 2) |  | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 2 | 2496 + 3/2 × BWChannel 6 MHz |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 3 | 2496 + BWChannel /2 + |  |  |  | CP-OFDM QPSK |  | Inner Full |
| 4 | MAX(10 MHz, 0.25 × BWChannel) |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 5 | High |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Right |
| 6 | High |  |  |  | CP-OFDM QPSK |  | Inner Full |
| 7 | High |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 8 | Low |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 9 | 2496 + 3/2 × BWChannel 6 MHz |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 10 | 2496 + BWChannel /2 + |  |  |  | CP-OFDM 16 QAM |  | Inner Full |
| 11 | MAX(10 MHz, 0.25 × BWChannel) |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 12 | High |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Right |
| 13 | High |  |  |  | CP-OFDM 16 QAM |  | Inner Full |
| 14 | High |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 15 | Low |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 16 | 2496 + 3/2 × BWChannel 6 MHz |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 17 | 2496 + BWChannel /2 + MAX(10 MHz, 0.25 × BWChannel) |  |  |  | CP-OFDM 64 QAM |  | Outer Full |
| 18 | High |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Right |
| 19 | High |  |  |  | CP-OFDM 64 QAM |  | Outer Full |
| 20 | Low |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 21 | 2496 + 3/2 × BWChannel 6 MHz |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 22 | 2496 + BWChannel /2 + MAX(10 MHz, 0.25 × BWChannel) |  |  |  | CP-OFDM 256 QAM |  | Outer Full |
| 23 | High |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Right |
| 24 | High |  |  |  | CP-OFDM 256 QAM |  | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-2: Test Configuration table for NS_35

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range, High range |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest, Highest |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest, Highest |
| A-MPR test parameters for NS_35 |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  |  | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 2 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 3 | Default | N/A | CP-OFDM QPSK | Outer Full |
| 4 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 5 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 6 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 7 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 8 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 9 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 10 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 11 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 12 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |  |

Table 6.2D.3.4.1-3: Test Configuration table for NS_05

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Use uplink carrier center frequency (Fc) as specified in test parameters |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest unless otherwise specified in test parameters. |  |  |  |  |  |
| A-MPR test parameters for NS_05 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  |  | RB allocation (Note 1) |  |  |  |  |
|  |  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz |  | SCS 60 kHz |  |
| 1 | 1922.5 | 5 | 15 |  | A3 | CP-OFDM | QPSK |  | Outer_Full |  |  |  |  |
| 2 | 1925 | 10 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |  |
| 3 | 1925 | 10 | Default |  | A7 |  | QPSK |  | 42@10 |  | 18@5 |  | 8@3 |
| 4 | 1925 | 10 | Default |  | A2 |  | QPSK |  | 6@40 |  | 3@20 |  | 1@10 |
| 5 | 1935 | 10 | Default |  | A4 |  | QPSK |  | Outer_Full |  |  |  |  |
| 6 | 1927.5 | 15 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |  |
| 7 | 1927.5 | 15 | Default |  | A7 |  | QPSK |  | 60@19 |  | 28@10 |  | 12@5 |
| 8 | 1927.5 | 15 | Default |  | A2 |  | QPSK |  | 6@56 |  | 3@28 |  | 1@14 |
| 9 | 1932.5 | 15 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |  |
| 10 | 1932.5 | 15 | Default |  | A2 |  | QPSK |  | 6@68 |  | 3@34 |  | 1@17 |
| 11 | 1942.5 | 15 | Default |  | A5 |  | QPSK |  | Outer_Full |  |  |  |  |
| 12 | 1930 | 20 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |  |
| 13 | 1930 | 20 | Default |  | A7 |  | QPSK |  | 78@28 |  | 37@14 |  | 17@7 |
| 14 | 1930 | 20 | Default |  | A2 |  | QPSK |  | 6@76 |  | 3@38 |  | 1@19 |
| 15 | 1950 | 20 | Default |  | A6 |  | QPSK |  | Outer_Full |  |  |  |  |
| 16 | 1922.5 | 5 | 15 |  | A3 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 17 | 1925 | 10 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 18 | 1925 | 10 | Default |  | A7 |  | 16 QAM |  | 42@10 |  | 18@5 |  | 8@3 |
| 19 | 1925 | 10 | Default |  | A2 |  | 16 QAM |  | 6@40 |  | 3@20 |  | 1@10 |
| 20 | 1935 | 10 | Default |  | A4 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 21 | 1927.5 | 15 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 22 | 1927.5 | 15 | Default |  | A7 |  | 16 QAM |  | 60@19 |  | 28@10 |  | 12@5 |
| 23 | 1927.5 | 15 | Default |  | A2 |  | 16 QAM |  | 6@56 |  | 3@28 |  | 1@14 |
| 24 | 1932.5 | 15 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 25 | 1932.5 | 15 | Default |  | A2 |  | 16 QAM |  | 6@68 |  | 3@34 |  | 1@17 |
| 26 | 1942.5 | 15 | Default |  | A5 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 27 | 1930 | 20 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 28 | 1930 | 20 | Default |  | A7 |  | 16 QAM |  | 78@28 |  | 37@14 |  | 17@7 |
| 29 | 1930 | 20 | Default |  | A2 |  | 16 QAM |  | 6@76 |  | 3@38 |  | 1@19 |
| 30 | 1950 | 20 | Default |  | A6 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 31 | 1922.5 | 5 | 15 |  | A3 |  | 64 QAM |  | Outer_Full |  |  |  |  |
| 32 | 1925 | 10 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |  |
| 33 | 1925 | 10 | Default |  | A7 |  | 64 QAM |  | 42@10 |  | 18@5 |  | 8@3 |
| 34 | 1925 | 10 | Default |  | A2 |  | 64 QAM |  | 6@40 |  | 3@20 |  | 1@10 |
| 35 | 1927.5 | 15 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |  |
| 36 | 1927.5 | 15 | Default |  | A7 |  | 64 QAM |  | 60@19 |  | 28@10 |  | 12@5 |
| 37 | 1927.5 | 15 | Default |  | A2 |  | 64 QAM |  | 6@56 |  | 3@28 |  | 1@14 |
| 38 | 1932.5 | 15 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |  |
| 39 | 1932.5 | 15 | Default |  | A2 |  | 64 QAM |  | 6@68 |  | 3@34 |  | 1@17 |
| 40 | 1930 | 20 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |  |
| 41 | 1930 | 20 | Default |  | A7 |  | 64 QAM |  | 78@28 |  | 37@14 |  | 17@7 |
| 42 | 1930 | 20 | Default |  | A2 |  | 64 QAM |  | 6@76 |  | 3@38 |  | 1@19 |
| 43 | 1922.5 | 5 | 15 |  | A3 |  | 256 QAM |  | Outer_Full |  |  |  |  |
| 44 | 1925 | 10 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |  |
| 45 | 1925 | 10 | Default |  | A7 |  | 256 QAM |  | 42@10 |  | 18@5 |  | 8@3 |
| 46 | 1927.5 | 15 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |  |
| 47 | 1927.5 | 15 | Default |  | A7 |  | 256 QAM |  | 60@19 |  | 28@10 |  | 12@5 |
| 48 | 1932.5 | 15 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |  |
| 49 | 1930 | 20 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |  |
| 50 | 1930 | 20 | Default |  | A7 |  | 256 QAM |  | 78@28 |  | 37@14 |  | 17@7 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-4: Test Configuration table for NS_05U

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Use uplink carrier center frequency (Fc) as specified in test parameters |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest unless otherwise specified in test parameters. |  |  |  |  |  |
| A-MPR test parameters for NS_05U |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  |  | RB allocation (Note 1) |  |  |  |  |
|  |  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz |  | SCS 60 kHz |  |
| 1 | 1922.5 | 5 | 15 |  | A3 | CP-OFDM | QPSK |  | Outer_Full |  |  |  |  |
| 2 | 1925 | 10 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |  |
| 3 | 1925 | 10 | Default |  | A7 |  | QPSK |  | 42@10 |  | 18@5 |  | 8@3 |
| 4 | 1925 | 10 | Default |  | A2 |  | QPSK |  | 6@40 |  | 3@20 |  | 1@10 |
| 5 | 1935 | 10 | Default |  | A4 |  | QPSK |  | Outer_Full |  |  |  |  |
| 6 | 1927.5 | 15 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |  |
| 7 | 1927.5 | 15 | Default |  | A7 |  | QPSK |  | 60@19 |  | 28@10 |  | 12@5 |
| 8 | 1927.5 | 15 | Default |  | A2 |  | QPSK |  | 6@56 |  | 3@28 |  | 1@14 |
| 9 | 1932.5 | 15 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |  |
| 10 | 1932.5 | 15 | Default |  | A2 |  | QPSK |  | 6@68 |  | 3@34 |  | 1@17 |
| 11 | 1942.5 | 15 | Default |  | A5 |  | QPSK |  | Outer_Full |  |  |  |  |
| 12 | 1930 | 20 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |  |  |
| 13 | 1930 | 20 | Default |  | A7 |  | QPSK |  | 78@28 |  | 37@14 |  | 17@7 |
| 14 | 1930 | 20 | Default |  | A2 |  | QPSK |  | 6@76 |  | 3@38 |  | 1@19 |
| 15 | 1950 | 20 | Default |  | A6 |  | QPSK |  | Outer_Full |  |  |  |  |
| 16 | 1922.5 | 5 | 15 |  | A3 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 17 | 1925 | 10 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 18 | 1925 | 10 | Default |  | A7 |  | 16 QAM |  | 42@10 |  | 18@5 |  | 8@3 |
| 19 | 1925 | 10 | Default |  | A2 |  | 16 QAM |  | 6@40 |  | 3@20 |  | 1@10 |
| 20 | 1935 | 10 | Default |  | A4 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 21 | 1927.5 | 15 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 22 | 1927.5 | 15 | Default |  | A7 |  | 16 QAM |  | 60@19 |  | 28@10 |  | 12@5 |
| 23 | 1927.5 | 15 | Default |  | A2 |  | 16 QAM |  | 6@56 |  | 3@28 |  | 1@14 |
| 24 | 1932.5 | 15 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 25 | 1932.5 | 15 | Default |  | A2 |  | 16 QAM |  | 6@68 |  | 3@34 |  | 1@17 |
| 26 | 1942.5 | 15 | Default |  | A5 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 27 | 1930 | 20 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 28 | 1930 | 20 | Default |  | A7 |  | 16 QAM |  | 78@28 |  | 37@14 |  | 17@7 |
| 29 | 1930 | 20 | Default |  | A2 |  | 16 QAM |  | 6@76 |  | 3@38 |  | 1@19 |
| 30 | 1950 | 20 | Default |  | A6 |  | 16 QAM |  | Outer_Full |  |  |  |  |
| 31 | 1922.5 | 5 | 15 |  | A3 |  | 64 QAM |  | Outer_Full |  |  |  |  |
| 32 | 1925 | 10 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |  |
| 33 | 1925 | 10 | Default |  | A7 |  | 64 QAM |  | 42@10 |  | 18@5 |  | 8@3 |
| 34 | 1925 | 10 | Default |  | A2 |  | 64 QAM |  | 6@40 |  | 3@20 |  | 1@10 |
| 35 | 1935 | 10 | Default |  | A4 |  | 64 QAM |  | Outer_Full |  |  |  |  |
| 36 | 1927.5 | 15 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |  |
| 37 | 1927.5 | 15 | Default |  | A7 |  | 64 QAM |  | 60@19 |  | 28@10 |  | 12@5 |
| 38 | 1927.5 | 15 | Default |  | A2 |  | 64 QAM |  | 6@56 |  | 3@28 |  | 1@14 |
| 39 | 1932.5 | 15 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |  |
| 40 | 1932.5 | 15 | Default |  | A2 |  | 64 QAM |  | 6@68 |  | 3@34 |  | 1@17 |
| 41 | 1942.5 | 15 | Default |  | A5 |  | 64 QAM |  | Outer_Full |  |  |  |  |
| 42 | 1930 | 20 | Default |  | A1 |  | 64 QAM |  | Outer_Full |  |  |  |  |
| 43 | 1930 | 20 | Default |  | A7 |  | 64 QAM |  | 78@28 |  | 37@14 |  | 17@7 |
| 44 | 1930 | 20 | Default |  | A2 |  | 64 QAM |  | 6@76 |  | 3@38 |  | 1@19 |
| 45 | 1950 | 20 | Default |  | A6 |  | 64 QAM |  | Outer_Full |  |  |  |  |
| 46 | 1922.5 | 5 | 15 |  | A3 |  | 256 QAM |  | Outer_Full |  |  |  |  |
| 47 | 1925 | 10 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |  |
| 48 | 1925 | 10 | Default |  | A7 |  | 256 QAM |  | 42@10 |  | 18@5 |  | 8@3 |
| 49 | 1935 | 10 | Default |  | A4 |  | 256 QAM |  | Outer_Full |  |  |  |  |
| 50 | 1927.5 | 15 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |  |
| 51 | 1927.5 | 15 | Default |  | A7 |  | 256 QAM |  | 60@19 |  | 28@10 |  | 12@5 |
| 52 | 1932.5 | 15 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |  |
| 53 | 1942.5 | 15 | Default |  | A5 |  | 256 QAM |  | Outer_Full |  |  |  |  |
| 54 | 1930 | 20 | Default |  | A1 |  | 256 QAM |  | Outer_Full |  |  |  |  |
| 55 | 1930 | 20 | Default |  | A7 |  | 256 QAM |  | 78@28 |  | 37@14 |  | 17@7 |
| 56 | 1950 | 20 | Default |  | A6 |  | 256 QAM |  | Outer_Full |  |  |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-5: Test Configuration table for NS_48

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Low range, High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 25 MHz, 30MHz, 40MHz, 50MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_48 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 25 | Default |  | CP-OFDM | QPSK | Outer_Full (A3) |  |  |  |
| 2 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 3 | Default | 30 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |
| 4 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A5) |  |  |  |
| 5 | Default | 40 | Default |  |  | QPSK | 16@0 (A2) |  | 8@0 (A2) | 4@0 (A2) |
| 6 | Default | 40 | Default |  |  | QPSK | 95@0 (A3) |  | 48@0 (A3) | 24@0 (A3) |
| 7 | Default | 40 | Default |  |  | QPSK | 152@0 (A4) |  | 76@0 (A4) | 38@0 (A4) |
| 8 | Default | 40 | Default |  |  | QPSK | 192@0 (A2) |  | 96@0 (A2) | 48@0 (A2) |
| 9 | Default | 40 | Default |  |  | QPSK | 5@187 (A3) |  | 2@94 (A3) | 1@47 (A3) |
| 10 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 11 | Default | 50 | Default |  |  | QPSK | 34@0 (A2) |  | 18@0 (A2) | 9@0 (A2) |
| 12 | Default | 50 | Default |  |  | QPSK | 115@0 (A4) |  | 58@0 (A4) | 29@0 (A4) |
| 13 | Default | 50 | Default |  |  | QPSK | 228@0 (A2) |  | 114@0 (A2) | 57@0 (A2) |
| 14 | Default | 50 | Default |  |  | QPSK | 5@223 (A5) |  | 2@112 (A5) | 1@56 (A5) |
| 15 | Default | 50 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 16 | Default | 25 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |
| 17 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |  |
| 18 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |
| 19 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A5) |  |  |  |
| 20 | Default | 40 | Default |  |  | 256 QAM | 16@0 (A2) |  | 8@0 (A2) | 4@0 (A2) |
| 21 | Default | 40 | Default |  |  | 256 QAM | 95@0 (A3) |  | 48@0 (A3) | 24@0 (A3) |
| 22 | Default | 40 | Default |  |  | 256 QAM | 152@0 (A4) |  | 76@0 (A4) | 38@0 (A4) |
| 23 | Default | 40 | Default |  |  | 256 QAM | 192@0 (A2) |  | 96@0 (A2) | 48@0 (A2) |
| 24 | Default | 40 | Default |  |  | 256 QAM | 5@187 (A3) |  | 2@94 (A3) | 1@47 (A3) |
| 25 | Default | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| 26 | Default | 50 | Default |  |  | 256 QAM | 34@0 (A2) |  | 18@0 (A2) | 9@0 (A2) |
| 27 | Default | 50 | Default |  |  | 256 QAM | 115@0 (A4) |  | 58@0 (A4) | 29@0 (A4) |
| 28 | Default | 50 | Default |  |  | 256 QAM | 228@0 (A2) |  | 114@0 (A2) | 57@0 (A2) |
| 29 | Default | 50 | Default |  |  | 256 QAM | 5@223 (A5) |  | 2@112 (A5) | 1@56 (A5) |
| 30 | Default | 50 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-6: Test Configuration table for NS_49

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Low range, High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 25 MHz, 30MHz, 40MHz, 50MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_49 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 25 | Default |  | CP-OFDM | QPSK | Outer_Full (A3) |  |  |  |
| 2 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 3 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Left (A3) |  |  |  |
| 4 | Default | 30 | Default |  |  | QPSK | 20@0 (A1) |  | 10@0 (A1) | 5@0 (A1) |
| 5 | Default | 30 | Default |  |  | QPSK | 36@0 (A5) |  | 18@0 (A5) | 9@0 (A5) |
| 6 | Default | 30 | Default |  |  | QPSK | 80@0 (A3) |  | 40@0 (A3) | 20@0 (A3) |
| 7 | Default | 30 | Default |  |  | QPSK | 120@0 (A4) |  | 60@0 (A4) | 30@0 (A4) |
| 8 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 9 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A5) |  |  |  |
| 10 | Default | 40 | Default |  |  | QPSK | 40@0 (A1) |  | 20@0 (A1) | 10@0 (A1) |
| 11 | Default | 40 | Default |  |  | QPSK | 5@53 (A5) |  | 2@27 (A5) | 1@14 (A5) |
| 12 | Default | 40 | Default |  |  | QPSK | 100@0 (A4) |  | 50@0 (A4) | 25@0 (A4) |
| 13 | Default | 40 | Default |  |  | QPSK | 159@33 (A2) |  | 79@17 (A2) | 39@9 (A2) |
| 14 | Default | 40 | Default |  |  | QPSK | 5@187 (A5) |  | 2@94 (A5) | 1@47 (A5) |
| 15 | Default | 40 | Default |  |  | QPSK | 192@0 (A1) |  | 96@0 (A1) | 48@0 (A1) |
| 16 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 17 | Default | 50 | Default |  |  | QPSK | 5@75 (A5) |  | 2@38 (A5) | 1@19 (A5) |
| 18 | Default | 50 | Default |  |  | QPSK | 5@215 (A5) |  | 2@108 (A5) | 1@54 (A5) |
| 19 | Default | 50 | Default |  |  | QPSK | 175@45 (A2) |  | 87@23 (A2) | 43@12 (A2) |
| 20 | Default | 50 | Default |  |  | QPSK | 220@0 (A1) |  | 110@0 (A1) | 55@0 (A1) |
| 21 | Default | 50 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 22 | Default | 25 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |
| 23 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |  |
| 24 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_Left (A3) |  |  |  |
| 25 | Default | 30 | Default |  |  | 256 QAM | 20@0 (A1) |  | 10@0 (A1) | 5@0 (A1) |
| 26 | Default | 30 | Default |  |  | 256 QAM | 36@0 (A5) |  | 18@0 (A5) | 9@0 (A5) |
| 27 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A3) |  | 40@0 (A3) | 20@0 (A3) |
| 28 | Default | 30 | Default |  |  | 256 QAM | 120@0 (A4) |  | 60@0 (A4) | 30@0 (A4) |
| 29 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 30 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A5) |  |  |  |
| 31 | Default | 40 | Default |  |  | 256 QAM | 40@0 (A1) |  | 20@0 (A1) | 10@0 (A1) |
| 32 | Default | 40 | Default |  |  | 256 QAM | 5@53 (A5) |  | 2@27 (A5) | 1@14 (A5) |
| 33 | Default | 40 | Default |  |  | 256 QAM | 100@0 (A4) |  | 50@0 (A4) | 25@0 (A4) |
| 34 | Default | 40 | Default |  |  | 256 QAM | 159@33 (A2) |  | 79@17 (A2) | 39@9 (A2) |
| 35 | Default | 40 | Default |  |  | 256 QAM | 5@187 (A5) |  | 2@94 (A5) | 1@47 (A5) |
| 36 | Default | 40 | Default |  |  | 256 QAM | 192@0 (A1) |  | 96@0 (A1) | 48@0 (A1) |
| 37 | Default | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| 38 | Default | 50 | Default |  |  | 256 QAM | 5@75 (A5) |  | 2@38 (A5) | 1@19 (A5) |
| 39 | Default | 50 | Default |  |  | 256 QAM | 5@215 (A5) |  | 2@108 (A5) | 1@54 (A5) |
| 40 | Default | 50 | Default |  |  | 256 QAM | 175@45 (A2) |  | 87@23 (A2) | 43@12 (A2) |
| 41 | Default | 50 | Default |  |  | 256 QAM | 220@0 (A1) |  | 110@0 (A1) | 55@0 (A1) |
| 42 | Default | 50 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-7: Test Configuration table for NS_03, NS_03U and NS_100

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_03, NS_03U and NS_100 |  |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |  |
|  |  |  |  |  | Modulation |  |  | RB allocation (Note 1) |
| 1 | Low | Default | Default | N/A for A-MPR test cases | CP- OFDM | QPSK |  | Edge_1RB_Left |
| 2 | High | Default | Default |  |  | QPSK |  | Edge_1RB_Right |
| 3 | Default | Default | Default |  |  | QPSK |  | Outer_Full |
| 4 | Low | Default | Default |  |  | 16 QAM |  | Edge_1RB_Left |
| 5 | High | Default | Default |  |  | 16 QAM |  | Edge_1RB_Right |
| 6 | Default | Default | Default |  |  | 16 QAM |  | Outer_Full |
| 7 | Low | Default | Default |  |  | 64 QAM |  | Edge_1RB_Left |
| 8 | High | Default | Default |  |  | 64 QAM |  | Edge_1RB_Right |
| 9 | Default | Default | Default |  |  | 64 QAM |  | Outer_Full |
| 10 | Low | Default | Default |  |  | 256 QAM |  | Edge_1RB_Left |
| 11 | High | Default | Default |  |  | 256 QAM |  | Edge_1RB_Right |
| 12 | Default | Default | Default |  |  | 256 QAM |  | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-8: Test Configuration table for NS_46

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 15MHz, 20MHz, 25 MHz, 50MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_46 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc | Ch BW | SCS |  | Uplink Configuration |  |  |  |  |  |
|  | (MHz) | (MHz) | (kHz) |  | Modulation |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  | (Note 2) |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 25 | Default |  |  | QPSK | 90@43 (A3) |  | 45@20 (A3) | 23@8 (A3) |
| 2 | Default | 25 | Default |  |  | 256QAM | 90@43 (A3) |  | 45@20 (A3) | 23@8 (A3) |
| 3 | Default | 50 | Default |  |  | QPSK | Edge_1RB_Left (A4) |  |  |  |
| 4 | Default | 50 | Default |  |  | 64QAM | Edge_1RB_Left (A4) |  |  |  |
| 5 | Default | 50 | Default | Downlink Configuration | CP-OFDM | QPSK | 120@0 (A5) |  | 60@0 (A5) | 30@0 (A5) |
| 6 | Default | 50 | Default |  |  | 64QAM | 120@0 (A5) |  | 60@0 (A5) | 30@0 (A5) |
| 7 | Default | 50 | Default |  |  | QPSK | 176@0 (A6) |  | 88@0 (A6) | 44@0 (A6) |
| 8 | Default | 50 | Default |  |  | 64QAM | 176@0 (A6) |  | 88@0 (A6) | 44@0 (A6) |
| 9 | Default | 50 | Default |  |  | QPSK | 220@0 (A7) |  | 110@0 (A7) | 55@0 (A7) |
| 10 | Default | 50 | Default |  |  | 256QAM | 220@0 (A7) |  | 110@0 (A7) | 55@0 (A7) |
| 11 | Default | 50 | Default |  |  | QPSK | Outer_Full (A8) |  |  |  |
| 12 | Default | 50 | Default |  |  | 256QAM | Outer_Full (A8) |  |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-9: Test Configuration table for NS_21

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest, Highest |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |  |  |
| A-MPR test parameters for NS_21 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink | Uplink Configuration |  |  |  |  |  |
|  |  |  |  | Configuration | Modulation |  |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  | (Note 2) |  |  | SCS 15 kHz | SCS 30 kHz |  |
| 1 | Low | Default | Default |  |  | QPSK |  | Edge_1RB_Left |  |  |
| 2 | High | Default | Default |  |  | QPSK |  | Edge_1RB_Right |  |  |
| 3 | Default | Default | Default |  |  | QPSK |  | Outer_Full |  |  |
| 4 | Default | 10 MHz | Default |  |  | QPSK |  | 4@0 |  | 2@0 |
| 5 | Default | 10 MHz | Default |  |  | QPSK |  | 4@48 |  | 2@22 |
| 6 | Low | Default | Default |  |  | 16 QAM |  | Edge_1RB_Left |  |  |
| 7 | High | Default | Default |  |  | 16 QAM |  | Edge_1RB_Right |  |  |
| 8 | Default | Default | Default | N/A | CP-s OFDM | 16 QAM |  | Outer_Full |  |  |
| 9 | Default | 10 MHz | Default |  |  | 16 QAM |  | 4@0 |  | 2@0 |
| 10 | Default | 10 MHz | Default |  |  | 16 QAM |  | 4@48 |  | 2@22 |
| 11 | Low | Default | Default |  |  | 64 QAM |  | Edge_1RB_Left |  |  |
| 12 | High | Default | Default |  |  | 64 QAM |  | Edge_1RB_Right |  |  |
| 13 | Default | Default | Default |  |  | 64 QAM |  | Outer_Full |  |  |
| 14 | Default | 10 MHz | Default |  |  | 64 QAM |  | 4@0 |  | 2@0 |
| 15 | Default | 10 MHz | Default |  |  | 64 QAM |  | 4@48 |  | 2@22 |
| 16 | Low | Default | Default |  |  | 256 QAM |  | Edge_1RB_Left |  |  |
| 17 | High | Default | Default |  |  | 256 QAM |  | Edge_1RB_Right |  |  |
| 18 | Default | Default | Default |  |  | 256 QAM |  | Outer_Full |  |  |
| 19 | Default | 10 MHz | Default |  |  | 256 QAM |  | 4@0 |  | 2@0 |
| 20 | Default | 10 MHz | Default |  |  | 256 QAM |  | 4@48 |  | 2@22 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-10: Test Configuration table for NS_44

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Defined for each test ID |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Defined for each test ID |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_44 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc | Ch BW | SCS | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  | (MHz) | (MHz) | (kHz) |  | Modulation |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  | (Note 2) |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | 2602.5 | 25 | Default |  | CP-OFDM | 64 QAM | 100@0 (A3) |  | 50@0 (A3) | 25@0 (A3) |
| 2 | 2602.5 | 25 | Default |  |  | 64 QAM | 39@94 (A3) |  | 19@46 (A3) | 9@22 (A3) |
| 3 | 2602.5 | 25 | Default |  |  | 64 QAM | Outer_Full (A6) |  |  |  |
| 4 | 2602.5 | 25 | Default |  |  | 256 QAM | 100@0 (A3) |  | 50@0 (A3) | 25@0 (A3) |
| 5 | 2602.5 | 25 | Default | N/A |  | 256 QAM | 39@94 (A3) |  | 19@46 (A3) | 9@22 (A3) |
| 6 | 2602.5 | 25 | Default |  |  | 256 QAM | Outer_Full (A6) |  |  |  |
| 7 | 2600 | 30 | Default |  |  | 64 QAM | 120@0 (A3) |  | 60@0 (A3) | 30@0 (A3) |
| 8 | 2600 | 30 | Default |  |  | 64 QAM | 69@91 (A5) |  | 34@44 (A5) | 17@21 (A5) |
| 9 | 2600 | 30 | Default |  |  | 64 QAM | Outer_Full (A5) |  |  |  |
| 10 | 2600 | 30 | Default |  |  | 256 QAM | 120@0 (A3) |  | 60@0 (A3) | 30@0 (A3) |
| 11 | 2600 | 30 | Default |  |  | 256 QAM | 69@91 (A5) |  | 34@44 (A5) | 17@21 (A5) |
| 12 | 2600 | 30 | Default |  |  | 256 QAM | Outer_Full (A5) |  |  |  |
| 13 | 2595 | 40 | Default |  |  | 64 QAM | 16@0 (A1) |  | 8@0 (A2) | 4@0 (A2) |
| 14 | 2595 | 40 | Default |  |  | 64 QAM | 80@0 (A2) |  | 40@0 (A2) | 20@0 (A2) |
| 15 | 2595 | 40 | Default |  |  | 64 QAM | 130@0 (A3) |  | 65@0 (A3) | 33@0 (A3) |
| 16 | 2595 | 40 | Default |  |  | 64 QAM | 180@0 (A4) |  | 90@0 (A4) | 45@0 (A4) |
| 17 | 2595 | 40 | Default |  |  | 64 QAM | Outer_Full (A5) |  |  |  |
| 18 | 2595 | 40 | Default |  |  | 256 QAM | 16@0 (A2) |  | 8@0 (A2) | 4@0 (A2) |
| 19 | 2595 | 40 | Default |  |  | 256 QAM | 80@0 (A2) |  | 40@0 (A2) | 20@0 (A2) |
| 20 | 2595 | 40 | Default |  |  | 256 QAM | 130@0 (A3) |  | 65@0 (A3) | 33@0 (A3) |
| 21 | 2595 | 40 | Default |  |  | 256 QAM | 180@0 (A4) |  | 90@0 (A4) | 45@0 (A4) |
| 22 | 2595 | 40 | Default |  |  | 256 QAM | Outer_Full (A5) |  |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-11: Test Configuration table for NS_27

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies |  |  |  |  |  |  | Refer to uplink carrier centre frequency (Fc) in test parameters. Associated parameters defined in Table 6.2D.3.4.1-9a and 6.2D.3.4.1-9b. |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Refer to test parameters (15, 20, 40 MHz) |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest (Note 3) |  |  |  |
| A-MPR test parameters for NS_27 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc | Ch BW | SCS | Downlink | Uplink Configuration |  |  |  |  |  |
|  | (MHz) | (MHz) | (kHz) | Configuration | Modulation |  |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  | (Note 2) |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | 3557.52 | 15 | Default |  |  | QPSK |  | Edge_1RB_Left (A3) |  |  |
| 2 | 3557.52 | 15 | Default |  |  | QPSK |  | Outer_Full (A3) |  |  |
| 3 | 3692.49 | 15 | Default |  |  | QPSK |  | Edge_1RB_Right (A3) |  |  |
| 4 | 3692.49 | 15 | Default |  |  | QPSK |  | Outer_Full (A3) |  |  |
| 5 | 3562.5 | 15 | Default |  |  | QPSK |  | Edge_1RB_Left (A4) |  |  |
| 6 | 3562.5 | 15 | Default |  |  | QPSK |  | Outer_Full (2) |  |  |
| 7 | 3687.48 | 15 | Default |  |  | QPSK |  | Edge_1RB_Right (A4) |  |  |
| 8 | 3687.48 | 15 | Default |  |  | QPSK |  | Outer_Full (2) |  |  |
| 9 | 3560.01 | 20 | Default |  |  | QPSK |  | Edge_1RB_Left (A5) |  |  |
| 10 | 3560.01 | 20 | Default |  |  | QPSK |  | Outer_Full (A5) |  |  |
| 11 | 3690 | 20 | Default |  |  | QPSK |  | Edge_1RB_Right (A5) |  |  |
| 12 | 3690 | 20 | Default |  |  | QPSK |  | Outer_Full (A5) |  |  |
| 13 | 3570 | 20 | Default |  |  | QPSK |  | Edge_1RB_Left (A6) |  |  |
| 14 | 3570 | 20 | Default |  |  | QPSK |  | Outer_Full (2) |  |  |
| 15 | 3679.98 | 20 | Default |  |  | QPSK |  | Edge_1RB_Right (A6) |  |  |
| 16 | 3679.98 | 20 | Default |  |  | QPSK |  | Outer_Full (2) |  |  |
| 17 | 3570 | 40 | Default | N/A |  | QPSK |  | Edge_1RB_Left (A7) |  |  |
| 18 | 3570 | 40 | Default |  |  | QPSK |  | 153@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 19 | 3570 | 40 | Default |  |  | QPSK |  | 99@69 (A1) | 49@34 (A1) | 24@16 (A1) |
| 20 | 3570 | 40 | Default |  |  | QPSK |  | Edge_1RB_Right (A7) |  |  |
| 21 | 3679.98 | 40 | Default |  |  | QPSK |  | Edge_1RB_Right (A7) |  |  |
| 22 | 3679.98 | 40 | Default |  |  | QPSK |  | 137@0 (A2) | 68@0 (A2) | 35@0 (A2) |
| 23 | 3679.98 | 40 | Default |  |  | QPSK |  | 99@38 (A1) | 49@18 (A1) | 24@9 (A1) |
| 24 | 3679.98 | 40 | Default |  |  | QPSK |  | Edge_1RB_Left (A7) |  |  |
| 25 | 3600 | 40 | Default |  |  | QPSK |  | Edge_1RB_Left (A8) |  |  |
| 26 | 3600 | 40 | Default |  |  | QPSK |  | Outer_Full (4.5) |  |  |
| 27 | 3649.98 | 40 | Default |  |  | QPSK |  | Edge_1RB_Right (A8) |  |  |
| 28 | 3649.98 | 40 | Default |  |  | QPSK |  | Outer_Full (4.5) |  |  |
| 29 | 3557.52 | 15 | Default |  |  | 16 QAM |  | Edge_1RB_Left (A3) |  |  |
| 30 | 3557.52 | 15 | Default |  |  | 16 QAM |  | Outer_Full (A3) |  |  |
| 31 | 3692.49 | 15 | Default |  |  | 16 QAM |  | Edge_1RB_Right (A3) |  |  |
| 32 | 3692.49 | 15 | Default |  |  | 16 QAM |  | Outer_Full (A3) |  |  |
| 33 | 3562.5 | 15 | Default |  |  | 16 QAM |  | Edge_1RB_Left (A4) |  |  |
| 34 | 3562.5 | 15 | Default |  |  | 16 QAM |  | Outer_Full (2) |  |  |
| 35 | 3687.48 | 15 | Default |  |  | 16 QAM |  | Edge_1RB_Right (A4) |  |  |
| 36 | 3687.48 | 15 | Default |  |  | 16 QAM |  | Outer_Full (2) |  |  |
| 37 | 3560.01 | 20 | Default |  |  | 16 QAM |  | Edge_1RB_Left (A5) |  |  |
| 38 | 3560.01 | 20 | Default |  |  | 16 QAM |  | Outer_Full (A5) |  |  |
| 39 | 3690 | 20 | Default |  |  | 16 QAM |  | Edge_1RB_Right (A5) |  |  |
| 40 | 3690 | 20 | Default |  |  | 16 QAM |  | Outer_Full (A5) |  |  |
| 41 | 3570 | 20 | Default |  |  | 16 QAM |  | Edge_1RB_Left (A6) |  |  |
| 42 | 3570 | 20 | Default |  |  | 16 QAM |  | Outer_Full (2) |  |  |
| 43 | 3679.98 | 20 | Default |  |  | 16 QAM |  | Edge_1RB_Right (A6) |  |  |
| 44 | 3679.98 | 20 | Default |  |  | 16 QAM |  | Outer_Full (2) |  |  |
| 45 | 3570 | 40 | Default |  |  | 16 QAM |  | Edge_1RB_Left (A7) |  |  |
| 46 | 3570 | 40 | Default |  |  | 16 QAM |  | 153@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 47 | 3570 | 40 | Default |  |  | 16 QAM |  | 99@69 (A1) | 49@34 (A1) | 24@16 (A1) |
| 48 | 3570 | 40 | Default |  |  | 16 QAM |  | Edge_1RB_Right (A7) |  |  |
| 49 | 3679.98 | 40 | Default |  |  | 16 QAM |  | Edge_1RB_Right (A7) |  |  |
| 50 | 3679.98 | 40 | Default |  |  | 16 QAM |  | 137@0 (A2) | 68@0 (A2) | 35@0 (A2) |
| 51 | 3679.98 | 40 | Default |  |  | 16 QAM |  | 99@38 (A1) | 49@18 (A1) | 24@9 (A1) |
| 52 | 3679.98 | 40 | Default |  |  | 16 QAM |  | Edge_1RB_Left (A7) |  |  |
| 53 | 3600 | 40 | Default |  |  | 16 QAM |  | Edge_1RB_Left (A8) |  |  |
| 54 | 3600 | 40 | Default |  | CP-OFDM | 16 QAM |  | Outer_Full (4.5) |  |  |
| 55 | 3649.98 | 40 | Default |  |  | 16 QAM |  | Edge_1RB_Right (A8) |  |  |
| 56 | 3649.98 | 40 | Default |  |  | 16 QAM |  | Outer_Full (4.5) |  |  |
| 57 | 3557.52 | 15 | Default |  |  | 64 QAM |  | Edge_1RB_Left (A3) |  |  |
| 58 | 3557.52 | 15 | Default |  |  | 64 QAM |  | Outer_Full (A3) |  |  |
| 59 | 3692.49 | 15 | Default |  |  | 64 QAM |  | Edge_1RB_Right (A3) |  |  |
| 60 | 3692.49 | 15 | Default |  |  | 64 QAM |  | Outer_Full (A3) |  |  |
| 61 | 3562.5 | 15 | Default |  |  | 64 QAM |  | Edge_1RB_Left (A4) |  |  |
| 62 | 3562.5 | 15 | Default |  |  | 64 QAM |  | Outer_Full (2) |  |  |
| 63 | 3687.48 | 15 | Default |  |  | 64 QAM |  | Edge_1RB_Right (A4) |  |  |
| 64 | 3687.48 | 15 | Default |  |  | 64 QAM |  | Outer_Full (2) |  |  |
| 65 | 3560.01 | 20 | Default |  |  | 64 QAM |  | Edge_1RB_Left (A5) |  |  |
| 66 | 3560.01 | 20 | Default |  |  | 64 QAM |  | Outer_Full (A5) |  |  |
| 67 | 3690 | 20 | Default |  |  | 64 QAM |  | Edge_1RB_Right (A5) |  |  |
| 68 | 3690 | 20 | Default |  |  | 64 QAM |  | Outer_Full (A5) |  |  |
| 69 | 3570 | 20 | Default |  |  | 64 QAM |  | Edge_1RB_Left (A6) |  |  |
| 70 | 3570 | 20 | Default |  |  | 64 QAM |  | Outer_Full (2) |  |  |
| 71 | 3679.98 | 20 | Default |  |  | 64 QAM |  | Edge_1RB_Right (A6) |  |  |
| 72 | 3679.98 | 20 | Default |  |  | 64 QAM |  | Outer_Full (2) |  |  |
| 73 | 3570 | 40 | Default |  |  | 64 QAM |  | Edge_1RB_Left (A7) |  |  |
| 74 | 3570 | 40 | Default |  |  | 64 QAM |  | 153@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 75 | 3570 | 40 | Default |  |  | 64 QAM |  | 99@69 (A1) | 49@34 (A1) | 24@16 (A1) |
| 76 | 3570 | 40 | Default |  |  | 64 QAM |  | Edge_1RB_Right (A7) |  |  |
| 77 | 3679.98 | 40 | Default |  |  | 64 QAM |  | Edge_1RB_Right (A7) |  |  |
| 78 | 3679.98 | 40 | Default |  |  | 64 QAM |  | 137@0 (A2) | 68@0 (A2) | 35@0 (A2) |
| 79 | 3679.98 | 40 | Default |  |  | 64 QAM |  | 99@38 (A1) | 49@18 (A1) | 24@9 (A1) |
| 80 | 3679.98 | 40 | Default |  |  | 64 QAM |  | Edge_1RB_Left (A7) |  |  |
| 81 | 3600 | 40 | Default |  |  | 64 QAM |  | Edge_1RB_Left (A8) |  |  |
| 82 | 3600 | 40 | Default |  |  | 64 QAM |  | Outer_Full (4.5) |  |  |
| 83 | 3649.98 | 40 | Default |  |  | 64 QAM |  | Edge_1RB_Right (A8) |  |  |
| 84 | 3649.98 | 40 | Default |  |  | 64 QAM |  | Outer_Full (4.5) |  |  |
| 85 | 3557.52 | 15 | Default |  |  | 256 QAM |  | Edge_1RB_Left (A3) |  |  |
| 86 | 3557.52 | 15 | Default |  |  | 256 QAM |  | Outer_Full (A3) |  |  |
| 87 | 3692.49 | 15 | Default |  |  | 256 QAM |  | Edge_1RB_Right (A3) |  |  |
| 88 | 3692.49 | 15 | Default |  |  | 256 QAM |  | Outer_Full (A3) |  |  |
| 89 | 3562.5 | 15 | Default |  |  | 256 QAM |  | Edge_1RB_Left (A4) |  |  |
| 90 | 3562.5 | 15 | Default |  |  | 256 QAM |  | Outer_Full (2) |  |  |
| 91 | 3687.48 | 15 | Default |  |  | 256 QAM |  | Edge_1RB_Right (A4) |  |  |
| 92 | 3687.48 | 15 | Default |  |  | 256 QAM |  | Outer_Full (2) |  |  |
| 93 | 3560.01 | 20 | Default |  |  | 256 QAM |  | Edge_1RB_Left (A5) |  |  |
| 94 | 3560.01 | 20 | Default |  |  | 256 QAM |  | Outer_Full (A5) |  |  |
| 95 | 3690 | 20 | Default |  |  | 256 QAM |  | Edge_1RB_Right (A5) |  |  |
| 96 | 3690 | 20 | Default |  |  | 256 QAM |  | Outer_Full (A5) |  |  |
| 97 | 3570 | 20 | Default |  |  | 256 QAM |  | Edge_1RB_Left (A6) |  |  |
| 98 | 3570 | 20 | Default |  |  | 256 QAM |  | Outer_Full (2) |  |  |
| 99 | 3679.98 | 20 | Default |  |  | 256 QAM |  | Edge_1RB_Right (A6) |  |  |
| 100 | 3679.98 | 20 | Default |  |  | 256 QAM |  | Outer_Full (2) |  |  |
| 101 | 3570 | 40 | Default |  |  | 256 QAM |  | Edge_1RB_Left (A7) |  |  |
| 102 | 3570 | 40 | Default |  |  | 256 QAM |  | 153@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 103 | 3570 | 40 | Default |  |  | 256 QAM |  | 99@69 (A1) | 49@34 (A1) | 24@16 (A1) |
| 104 | 3570 | 40 | Default |  |  | 256 QAM |  | Edge_1RB_Right (A7) |  |  |
| 105 | 3679.98 | 40 | Default |  |  | 256 QAM |  | Edge_1RB_Right (A7) |  |  |
| 106 | 3679.98 | 40 | Default |  |  | 256 QAM |  | 137@0 (A2) | 68@0 (A2) | 35@0 (A2) |
| 107 | 3679.98 | 40 | Default |  |  | 256 QAM |  | 99@38 (A1) | 49@18 (A1) | 24@9 (A1) |
| 108 | 3679.98 | 40 | Default |  |  | 256 QAM |  | Edge_1RB_Left (A7) |  |  |
| 109 | 3600 | 40 | Default |  |  | 256 QAM |  | Edge_1RB_Left (A8) |  |  |
| 110 | 3600 | 40 | Default |  |  | 256 QAM |  | Outer_Full (4.5) |  |  |
| 111 | 3649.98 | 40 | Default |  |  | 256 QAM |  | Edge_1RB_Right (A8) |  |  |
| 112 | 3649.98 | 40 | Default |  |  | 256 QAM |  | Outer_Full (4.5) |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: For FR1 bands where highest supported SCS is 60 kHz the highest tested SCS is limited to 30 kHz as carrier with SCS=60 kHz cannot be used as PCell. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-11a: Test frequencies for NS_27 (SCS=15 kHz, ΔFRaster = 15kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] | kSSB | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA(SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 79 | 3557.52 | 637168 | 3550.41 | 636694 | 0 | 30 | 7884 | 636960 | 2 | 0 | 0 (2) | 2 |
|  |  | 3562.5 | 637500 | 3555.39 | 637026 |  |  | 7888 | 637344 | 6 | 0 | 1 (6) | 6 |
|  |  | 3687.48 | 645832 | 3680.37 | 645358 |  |  | 7975 | 645696 | 2 | 2 | 1 (6) | 8 |
|  |  | 3692.49 | 646166 | 3685.38 | 645692 |  |  | 7978 | 645984 | 4 | 2 | 0 (2) | 4 |
| 20 | 106 | 3560.01 | 637334 | 3550.47 | 636698 |  |  | 7885 | 637056 | 10 | 3 | 1 (6) | 9 |
|  |  | 3570 | 638000 | 3560.46 | 637364 |  |  | 7891 | 637632 | 4 | 0 | 0 (2) | 2 |
|  |  | 3679.98 | 645332 | 3670.44 | 644696 |  |  | 7968 | 645024 | 4 | 1 | 1 (6) | 7 |
|  |  | 3690 | 646000 | 3680.46 | 645364 |  |  | 7975 | 645696 | 8 | 1 | 1 (6) | 7 |
| 40 | 216 | 3570 | 638000 | 3550.56 | 636704 |  |  | 7885 | 637056 | 4 | 3 | 1 (6) | 9 |
|  |  | 3600 | 640000 | 3580.56 | 638704 |  |  | 7905 | 638976 | 8 | 0 | 0 (2) | 2 |
|  |  | 3649.98 | 643332 | 3630.54 | 642036 |  |  | 7940 | 642336 | 0 | 3 | 0 (2) | 5 |
|  |  | 3679.98 | 645332 | 3660.54 | 644036 |  |  | 7961 | 644352 | 4 | 0 | 1 (6) | 6 |
| NOTE 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. NOTE 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-11b: Test frequencies for NS_27 (SCS=30 kHz, ΔFRaster = 30 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA[ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] | kSSB | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA(SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 38 | 3557.52 | 637168 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
|  |  | 3562.5 | 637500 | 3555.66 | 637044 |  |  | 7888 | 637344 | 12 | 0 | 2 (2) | 4 |
|  |  | 3687.48 | 645832 | 3680.64 | 645376 |  |  | 7975 | 645696 | 8 | 0 | 3 (3) | 6 |
|  |  | 3692.49 | 646166 | 3685.65 | 645710 |  |  | 7978 | 645984 | 10 | 0 | 1 (1) | 2 |
| 20 | 51 | 3560.01 | 637334 | 3550.83 | 636722 |  |  | 7885 | 637056 | 22 | 0 | 3 (3) | 6 |
|  |  | 3570 | 638000 | 3560.82 | 637388 |  |  | 7891 | 637632 | 4 | 0 | 0 (0) | 0 |
|  |  | 3679.98 | 645332 | 3670.8 | 644720 |  |  | 7968 | 645024 | 16 | 0 | 2 (2) | 4 |
|  |  | 3690 | 646000 | 3680.82 | 645388 |  |  | 7975 | 645696 | 20 | 0 | 2 (2) | 4 |
| 40 | 106 | 3570 | 638000 | 3550.92 | 636728 |  |  | 7885 | 637056 | 16 | 0 | 3 (3) | 6 |
|  |  | 3600 | 640000 | 3580.92 | 638728 |  |  | 7905 | 638976 | 8 | 0 | 0 (0) | 0 |
|  |  | 3649.98 | 643332 | 3630.9 | 642060 |  |  | 7940 | 642336 | 12 | 0 | 1 (1) | 2 |
|  |  | 3679.98 | 645332 | 3660.9 | 644060 |  |  | 7961 | 644352 | 4 | 0 | 2 (2) | 4 |
| NOTE 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. NOTE 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Editor’s note: The following lines belong at the end of subclause 6.2D.3.4.1. As new tables are added to this section, these lines should always follow the tables

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2 and uplink signals according Annex G.0, G.1, G.2 and G.3.0.

4.	The UL Reference Measurement channels are set according to the applicable Table 6.2D.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2D.3.4.3.

6.2D.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2D.3.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE. Allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at each transmit antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms over all active uplink slots and in the uplink symbols. For TDD slots only slots consisting of only UL symbols are under.

6.2D.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.1 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO, with the following exceptions for each network signalling value.

6.2D.3.4.3.1	Message contents exceptions for network signalling value "NS_04"

1.	Information element additionalSpectrumEmission is set to NS_04. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.1-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_04"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_04) |  |  |

6.2D.3.4.3.2	Message contents exceptions for network signalling value "NS_35"

1.	Information element additionalSpectrumEmission is set to NS_35. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.2-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_35" and NR band n71

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_35) |  |  |

6.2D.3.4.3.3	Message contents exceptions for network signalling value "NS_05"

1.	Information element additionalSpectrumEmission is set to NS_05. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.3-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_05"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_05) |  |  |

6.2D.3.4.3.4	Message contents exceptions for network signalling value "NS_05U"

1.	Information element additionalSpectrumEmission is set to NS_05U. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.4-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_05U"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_05U) |  |  |

6.2D.3.4.3.5	Message contents exceptions for network signalling value "NS_48"

1.	Information element additionalSpectrumEmission is set to NS_48. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.5-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_48"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 4 (NS_48) |  |  |

6.2D.3.4.3.6	Message contents exceptions for network signalling value "NS_49"

1.	Information element additionalSpectrumEmission is set to NS_49. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.6-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_49"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 5 (NS_49) |  |  |

6.2D.3.4.3.7	Message contents exceptions for network signalling value "NS_100"

1.	Information element additionalSpectrumEmission is set to NS_100. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.7-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_100"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_100) |  | not for band n65 |
|  | 2 (NS_100) |  | for band n65 |

6.2D.3.4.3.8	Message contents exceptions for network signalling value “NS_03”

1.	Information element additionalSpectrumEmission is set to NS_03. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.8-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_03" and NR band n2, n25 and n66

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_03) |  |  |

Table 6.2D.3.4.3.8-2: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_03" and NR band n70

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_03) |  |  |

6.2D.3.4.3.9	Message contents exceptions for network signalling value "NS_03U"

1.	Information element additionalSpectrumEmission is set to NS_03U. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.9-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_03U"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 3 (NS_03U) |  | for band n2, n25, n66 |

6.2D.3.4.3.10	Message contents exceptions for network signalling value "NS_46"

1.	Information element additionalSpectrumEmission is set to NS_46. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.10-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_46"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_46) |  | for band n7 |

6.2D.3.4.3.11	Message contents exceptions for network signalling value "NS_21"

1.	Information element additionalSpectrumEmission is set to NS_21. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.11-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_21"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_21) |  |  |

6.2D.3.4.3.12	Message contents exceptions for network signalling value "NS_44"

1.	Information element additionalSpectrumEmission is set to NS_44. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.12-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_44"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_44) |  |  |

6.2D.3.4.3.13	Message contents exceptions for network signalling value "NS_27"

1.	Information element additionalSpectrumEmission is set to NS_27. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2D.3.4.3.13-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_27"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_27) |  |  |

6.2D.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from table 6.2D.3.5-1 to table 6.2D.3.5-14. The allowed A-MPR values specified in table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2.2.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2D.1.3-1 apply.

Table 6.2D.3.5-0: Test Tolerance (UE additional maximum output power reduction)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

Table 6.2D.3.5-1: UE Power Class 2 test requirements (NS_04) for band n41

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | [7.5] | 0 | 18.5 | 5 | 3 | 28+TT | [13.5-TT] |
| 2 | 26 | 0 | [7.5] | 0 | 18.5 | 5 | 3 | 28+TT | [13.5-TT] |
| 3 | 26 | 0 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 4 | 26 | 0 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 5 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 6 | 26 | 1.5 | 0 | 0 | 24.5 | 3 | 3 | 28+TT | 21.3-TT |
| 7 | 26 | 3 | 0 | 0 | 23 | 3 | 3 | 28+TT | 20.0-TT |
| 8 | 26 | 0 | [7.5] | 0 | 18.5 | 5 | 3 | 28+TT | [13.5-TT] |
| 9 | 26 | 0 | [7.5] | 0 | 18.5 | 5 | 3 | 28+TT | [13.5-TT] |
| 10 | 26 | 0 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 11 | 26 | 0 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 12 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 13 | 26 | 2 | 0 | 0 | 24 | 3 | 3 | 28+TT | 21.0-TT |
| 14 | 26 | 3 | 0 | 0 | 23 | 3 | 3 | 28+TT | 20.0-TT |
| 15 | 26 | 0 | [7.5] | 0 | 18.5 | 5 | 3 | 28+TT | [13.5-TT] |
| 16 | 26 | 0 | [7.5] | 0 | 18.5 | 5 | 3 | 28+TT | [13.5-TT] |
| 17 | 26 | 0 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 18 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 19 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 20 | 26 | 0 | [10] | 0 | 16 | 5 | 3 | 28+TT | [11.0-TT] |
| 21 | 26 | 0 | [10] | 0 | 16 | 5 | 3 | 28+TT | [11.0-TT] |
| 22 | 26 | 0 | 7.5 | 0 | 18.5 | 5 | 3 | 28+TT | 13.5-TT |
| 23 | 26 | 6.5 | 0 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 24 | 26 | 6.5 | 0 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.0 dB for CP-OFDM 256 QAM and decreased by 1.5 dB for other modulations. NOTE 3: TT=0.7 for BWchannel ≤ 40 MHz; TT=1.0 for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-2: UE Power Class 3 test requirements (NS_04) for band n41

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 2 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 3 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 4 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 5 | 23 | 3.0 | 0 | 0 | 20.0 | 6 | 3 | 25+TT | 14.0-TT |
| 6 | 23 | 1.5 | 0 | 0 | 21.5 | 5 | 3 | 25+TT | 16.5-TT |
| 7 | 23 | 3.0 | 0 | 0 | 20.0 | 6 | 3 | 25+TT | 14.0-TT |
| 8 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 9 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 10 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 11 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 12 | 23 | 3.0 | 0 | 0 | 20.0 | 6 | 3 | 25+TT | 14.0-TT |
| 13 | 23 | 2.0 | 0 | 0 | 21.0 | 5 | 3 | 25+TT | 16.0-TT |
| 14 | 23 | 3.0 | 0 | 0 | 20.0 | 6 | 3 | 25+TT | 14.0-TT |
| 15 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 16 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 17 | 23 | 0 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 18 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 19 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 20 | 23 | 0 | 8 | 0 | 15.0 | 6 | 3 | 25+TT | 9.0-TT |
| 21 | 23 | 0 | 8 | 0 | 15.0 | 6 | 3 | 25+TT | 9.0-TT |
| 22 | 23 | 0 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 23 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 24 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.0 dB for CP-OFDM 256 QAM and decreased by 1.5 dB for other modulations. NOTE 3: TT=0.7 for BWchannel ≤ 40 MHz; TT=1.0 for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-3: UE Power Class 3 test requirements (NS_35) for band n71

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0 - TT |
| 2 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0 - TT |
| 3 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0 - TT |
| 4 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0 - TT |
| 5 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0 - TT |
| 6 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0 - TT |
| 7 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5 - TT |
| 8 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5 - TT |
| 9 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5 - TT |
| 10 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5 - TT |
| 11 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5 - TT |
| 12 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-4: UE Power Class 3 test requirements (NS_05) for band n1

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 23 |  | 0 |  | 3.0 |  | 7.5 |  | 0 |  | 15.5 |  | 6 |  | 2 |  | 25+TT |  | 9.5-TT |  |
| 2 |  | 23 |  | 0 |  | 3.0 |  | 10 |  | 0 |  | 13 |  | 6 |  | 2 |  | 25+TT |  | 7-TT |  |
| 3 |  | 23 |  | 0 |  | 3.0 |  | 6 |  | 0 |  | 17 |  | 5 |  | 2 |  | 25+TT |  | 12-TT |  |
| 4 |  | 23 |  | 0 |  | 3.0 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 5 |  | 23 |  | 0 |  | 3.0 |  | 3.5 |  | 0 |  | 19.5 |  | 5 |  | 2 |  | 25+TT |  | 14.5-TT |  |
| 6 |  | 23 |  | 0 |  | 3.0 |  | 10 |  | 0 |  | 13 |  | 6 |  | 2 |  | 25+TT |  | 7-TT |  |
| 7 |  | 23 |  | 0 |  | 3.0 |  | 6 |  | 0 |  | 17 |  | 5 |  | 2 |  | 25+TT |  | 12-TT |  |
| 8 |  | 23 |  | 0 |  | 3.0 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 9 |  | 23 |  | 0 |  | 3.0 |  | 10 |  | 0 |  | 13 |  | 6 |  | 2 |  | 25+TT |  | 7-TT |  |
| 10 |  | 23 |  | 0 |  | 3.0 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 11 |  | 23 |  | 0 |  | 3.0 |  | 3.5 |  | 0 |  | 19.5 |  | 5 |  | 2 |  | 25+TT |  | 14.5-TT |  |
| 12 |  | 23 |  | 0 |  | 3.0 |  | 10 |  | 0 |  | 13 |  | 6 |  | 2 |  | 25+TT |  | 7-TT |  |
| 13 |  | 23 |  | 0 |  | 3.0 |  | 6 |  | 0 |  | 17 |  | 5 |  | 2 |  | 25+TT |  | 12-TT |  |
| 14 |  | 23 |  | 0 |  | 3.0 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 15 |  | 23 |  | 0 |  | 3.0 |  | 3.5 |  | 0 |  | 19.5 |  | 5 |  | 2 |  | 25+TT |  | 14.5-TT |  |
| 16 |  | 23 |  | 0 |  | 3.0 |  | 7.5 |  | 0 |  | 15.5 |  | 6 |  | 2 |  | 25+TT |  | 9.5-TT |  |
| 17 |  | 23 |  | 0 |  | 3.0 |  | 10 |  | 0 |  | 13 |  | 6 |  | 2 |  | 25+TT |  | 7-TT |  |
| 18 |  | 23 |  | 0 |  | 3.0 |  | 6 |  | 0 |  | 17 |  | 5 |  | 2 |  | 25+TT |  | 12-TT |  |
| 19 |  | 23 |  | 0 |  | 3.0 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 20 |  | 23 |  | 0 |  | 3.0 |  | 3.5 |  | 0 |  | 19.5 |  | 5 |  | 2 |  | 25+TT |  | 14.5-TT |  |
| 21 |  | 23 |  | 0 |  | 3.0 |  | 10 |  | 0 |  | 13 |  | 6 |  | 2 |  | 25+TT |  | 7-TT |  |
| 22 |  | 23 |  | 0 |  | 3.0 |  | 6 |  | 0 |  | 17 |  | 5 |  | 2 |  | 25+TT |  | 12-TT |  |
| 23 |  | 23 |  | 0 |  | 3.0 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 24 |  | 23 |  | 0 |  | 3.0 |  | 10 |  | 0 |  | 13 |  | 6 |  | 2 |  | 25+TT |  | 7-TT |  |
| 25 |  | 23 |  | 0 |  | 3.0 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 26 |  | 23 |  | 0 |  | 3.0 |  | 3.5 |  | 0 |  | 19.5 |  | 5 |  | 2 |  | 25+TT |  | 14.5-TT |  |
| 27 |  | 23 |  | 0 |  | 3.0 |  | 10 |  | 0 |  | 13 |  | 6 |  | 2 |  | 25+TT |  | 7-TT |  |
| 28 |  | 23 |  | 0 |  | 3.0 |  | 6 |  | 0 |  | 17 |  | 5 |  | 2 |  | 25+TT |  | 12-TT |  |
| 29 |  | 23 |  | 0 |  | 3.0 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 30 |  | 23 |  | 0 |  | 3.0 |  | 3.5 |  | 0 |  | 19.5 |  | 5 |  | 2 |  | 25+TT |  | 14.5-TT |  |
| 31 |  | 23 |  | 0 |  | 3.5 |  | 8 |  | 0 |  | 15 |  | 6 |  | 2 |  | 25+TT |  | 9-TT |  |
| 32 |  | 23 |  | 0 |  | 3.5 |  | 11 |  | 0 |  | 12 |  | 6 |  | 2 |  | 25+TT |  | 6-TT |  |
| 33 |  | 23 |  | 0 |  | 3.5 |  | 6 |  | 0 |  | 17 |  | 5 |  | 2 |  | 25+TT |  | 12-TT |  |
| 34 |  | 23 |  | 0 |  | 3.5 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 35 |  | 23 |  | 0 |  | 3.5 |  | 11 |  | 0 |  | 12 |  | 6 |  | 2 |  | 25+TT |  | 6-TT |  |
| 36 |  | 23 |  | 0 |  | 3.5 |  | 6 |  | 0 |  | 17 |  | 5 |  | 2 |  | 25+TT |  | 12-TT |  |
| 37 |  | 23 |  | 0 |  | 3.5 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 38 |  | 23 |  | 0 |  | 3.5 |  | 11 |  | 0 |  | 12 |  | 6 |  | 2 |  | 25+TT |  | 6-TT |  |
| 39 |  | 23 |  | 0 |  | 3.5 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 40 |  | 23 |  | 0 |  | 3.5 |  | 11 |  | 0 |  | 12 |  | 6 |  | 2 |  | 25+TT |  | 6-TT |  |
| 41 |  | 23 |  | 0 |  | 3.5 |  | 6 |  | 0 |  | 17 |  | 5 |  | 2 |  | 25+TT |  | 12-TT |  |
| 42 |  | 23 |  | 0 |  | 3.5 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 43 |  | 23 |  | 0 |  | 6.5 |  | 10 |  | 0 |  | 13 |  | 6 |  | 2 |  | 25+TT |  | 7-TT |  |
| 44 |  | 23 |  | 0 |  | 6.5 |  | 13 |  | 0 |  | 10 |  | 7 |  | 2 |  | 25+TT |  | 3-TT |  |
| 45 |  | 23 |  | 0 |  | 6.5 |  | 6 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 46 |  | 23 |  | 0 |  | 6.5 |  | 13 |  | 0 |  | 10 |  | 7 |  | 2 |  | 25+TT |  | 3-TT |  |
| 47 |  | 23 |  | 0 |  | 6.5 |  | 6 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 48 |  | 23 |  | 0 |  | 6.5 |  | 13 |  | 0 |  | 10 |  | 7 |  | 2 |  | 25+TT |  | 3-TT |  |
| 49 |  | 23 |  | 0 |  | 6.5 |  | 13 |  | 0 |  | 10 |  | 7 |  | 2 |  | 25+TT |  | 3-TT |  |
| 50 |  | 23 |  | 0 |  | 6.5 |  | 6 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-5: UE Power Class 3 test requirements (NS_05U) for band n1

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 23 |  | 0 |  | 3.0 |  | 7.5 |  | 0 |  | 15.5 |  | 6 |  | 2 |  | 25+TT |  | 9.5-TT |  |
| 2 |  | 23 |  | 0 |  | 3.0 |  | 10 |  | 0 |  | 13 |  | 6 |  | 2 |  | 25+TT |  | 7-TT |  |
| 3 |  | 23 |  | 0 |  | 3.0 |  | 6 |  | 0 |  | 17 |  | 5 |  | 2 |  | 25+TT |  | 12-TT |  |
| 4 |  | 23 |  | 0 |  | 3.0 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 5 |  | 23 |  | 0 |  | 3.0 |  | 4 |  | 0 |  | 19 |  | 5 |  | 2 |  | 25+TT |  | 14-TT |  |
| 6 |  | 23 |  | 0 |  | 3.0 |  | 10 |  | 0 |  | 13 |  | 6 |  | 2 |  | 25+TT |  | 7-TT |  |
| 7 |  | 23 |  | 0 |  | 3.0 |  | 6 |  | 0 |  | 17 |  | 5 |  | 2 |  | 25+TT |  | 12-TT |  |
| 8 |  | 23 |  | 0 |  | 3.0 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 9 |  | 23 |  | 0 |  | 3.0 |  | 10 |  | 0 |  | 13 |  | 6 |  | 2 |  | 25+TT |  | 7-TT |  |
| 10 |  | 23 |  | 0 |  | 3.0 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 11 |  | 23 |  | 0 |  | 3.0 |  | 4 |  | 0 |  | 19 |  | 5 |  | 2 |  | 25+TT |  | 14-TT |  |
| 12 |  | 23 |  | 0 |  | 3.0 |  | 10 |  | 0 |  | 13 |  | 6 |  | 2 |  | 25+TT |  | 7-TT |  |
| 13 |  | 23 |  | 0 |  | 3.0 |  | 6 |  | 0 |  | 17 |  | 5 |  | 2 |  | 25+TT |  | 12-TT |  |
| 14 |  | 23 |  | 0 |  | 3.0 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 15 |  | 23 |  | 0 |  | 3.0 |  | 4 |  | 0 |  | 19 |  | 5 |  | 2 |  | 25+TT |  | 14-TT |  |
| 16 |  | 23 |  | 0 |  | 3.0 |  | 7.5 |  | 0 |  | 15.5 |  | 6 |  | 2 |  | 25+TT |  | 9.5-TT |  |
| 17 |  | 23 |  | 0 |  | 3.0 |  | 10 |  | 0 |  | 13 |  | 6 |  | 2 |  | 25+TT |  | 7-TT |  |
| 18 |  | 23 |  | 0 |  | 3.0 |  | 6 |  | 0 |  | 17 |  | 5 |  | 2 |  | 25+TT |  | 12-TT |  |
| 19 |  | 23 |  | 0 |  | 3.0 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 20 |  | 23 |  | 0 |  | 3.0 |  | 4 |  | 0 |  | 19 |  | 5 |  | 2 |  | 25+TT |  | 14-TT |  |
| 21 |  | 23 |  | 0 |  | 3.0 |  | 10 |  | 0 |  | 13 |  | 6 |  | 2 |  | 25+TT |  | 7-TT |  |
| 22 |  | 23 |  | 0 |  | 3.0 |  | 6 |  | 0 |  | 17 |  | 5 |  | 2 |  | 25+TT |  | 12-TT |  |
| 23 |  | 23 |  | 0 |  | 3.0 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 24 |  | 23 |  | 0 |  | 3.0 |  | 10 |  | 0 |  | 13 |  | 6 |  | 2 |  | 25+TT |  | 7-TT |  |
| 25 |  | 23 |  | 0 |  | 3.0 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 26 |  | 23 |  | 0 |  | 3.0 |  | 4 |  | 0 |  | 19 |  | 5 |  | 2 |  | 25+TT |  | 14-TT |  |
| 27 |  | 23 |  | 0 |  | 3.0 |  | 10 |  | 0 |  | 13 |  | 6 |  | 2 |  | 25+TT |  | 7-TT |  |
| 28 |  | 23 |  | 0 |  | 3.0 |  | 6 |  | 0 |  | 17 |  | 5 |  | 2 |  | 25+TT |  | 12-TT |  |
| 29 |  | 23 |  | 0 |  | 3.0 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 30 |  | 23 |  | 0 |  | 3.0 |  | 4 |  | 0 |  | 19 |  | 5 |  | 2 |  | 25+TT |  | 14-TT |  |
| 31 |  | 23 |  | 0 |  | 3.5 |  | 8 |  | 0 |  | 15 |  | 6 |  | 2 |  | 25+TT |  | 9-TT |  |
| 32 |  | 23 |  | 0 |  | 3.5 |  | 11 |  | 0 |  | 12 |  | 6 |  | 2 |  | 25+TT |  | 6-TT |  |
| 33 |  | 23 |  | 0 |  | 3.5 |  | 6 |  | 0 |  | 17 |  | 5 |  | 2 |  | 25+TT |  | 12-TT |  |
| 34 |  | 23 |  | 0 |  | 3.5 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 35 |  | 23 |  | 0 |  | 3.5 |  | 4 |  | 0 |  | 19 |  | 5 |  | 2 |  | 25+TT |  | 14-TT |  |
| 36 |  | 23 |  | 0 |  | 3.5 |  | 11 |  | 0 |  | 12 |  | 6 |  | 2 |  | 25+TT |  | 6-TT |  |
| 37 |  | 23 |  | 0 |  | 3.5 |  | 6 |  | 0 |  | 17 |  | 5 |  | 2 |  | 25+TT |  | 12-TT |  |
| 38 |  | 23 |  | 0 |  | 3.5 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 39 |  | 23 |  | 0 |  | 3.5 |  | 11 |  | 0 |  | 12 |  | 6 |  | 2 |  | 25+TT |  | 6-TT |  |
| 40 |  | 23 |  | 0 |  | 3.5 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 41 |  | 23 |  | 0 |  | 3.5 |  | 4 |  | 0 |  | 19 |  | 5 |  | 2 |  | 25+TT |  | 14-TT |  |
| 42 |  | 23 |  | 0 |  | 3.5 |  | 11 |  | 0 |  | 12 |  | 6 |  | 2 |  | 25+TT |  | 6-TT |  |
| 43 |  | 23 |  | 0 |  | 3.5 |  | 6 |  | 0 |  | 17 |  | 5 |  | 2 |  | 25+TT |  | 12-TT |  |
| 44 |  | 23 |  | 0 |  | 3.5 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 45 |  | 23 |  | 0 |  | 3.5 |  | 4 |  | 0 |  | 19 |  | 5 |  | 2 |  | 25+TT |  | 14-TT |  |
| 46 |  | 23 |  | 0 |  | 6.5 |  | 10 |  | 0 |  | 13 |  | 6 |  | 2 |  | 25+TT |  | 7-TT |  |
| 47 |  | 23 |  | 0 |  | 6.5 |  | 13 |  | 0 |  | 10 |  | 7 |  | 2 |  | 25+TT |  | 3-TT |  |
| 48 |  | 23 |  | 0 |  | 6.5 |  | 6.5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 49 |  | 23 |  | 0 |  | 6.5 |  | 6.5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 50 |  | 23 |  | 0 |  | 6.5 |  | 13 |  | 0 |  | 10 |  | 7 |  | 2 |  | 25+TT |  | 3-TT |  |
| 51 |  | 23 |  | 0 |  | 6.5 |  | 6.5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 52 |  | 23 |  | 0 |  | 6.5 |  | 13 |  | 0 |  | 10 |  | 7 |  | 2 |  | 25+TT |  | 3-TT |  |
| 53 |  | 23 |  | 0 |  | 6.5 |  | 6.5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 54 |  | 23 |  | 0 |  | 6.5 |  | 13 |  | 0 |  | 10 |  | 7 |  | 2 |  | 25+TT |  | 3-TT |  |
| 55 |  | 23 |  | 0 |  | 6.5 |  | 6.5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 56 |  | 23 |  | 0 |  | 6.5 |  | 6.5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-6: UE Power Class 3 test requirements (NS_48) for band n1

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 23 |  | 0 |  | 3 |  | 4.5 |  | 0 |  | 18.5 |  | 5 |  | 2 |  | 25+TT |  | 13.5-TT |  |
| 2 |  | 23 |  | 0 |  | 3 |  | 4.5 |  | 0 |  | 18.5 |  | 5 |  | 2 |  | 25+TT |  | 13.5-TT |  |
| 3 |  | 23 |  | 0 |  | 3 |  | 4.5 |  | 0 |  | 18.5 |  | 5 |  | 2 |  | 25+TT |  | 13.5-TT |  |
| 4 |  | 23 |  | 0 |  | 3 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 5 |  | 23 |  | 0 |  | 3 |  | 7 |  | 0 |  | 16 |  | 5 |  | 2 |  | 25+TT |  | 11-TT |  |
| 6 |  | 23 |  | 0 |  | 3 |  | 4.5 |  | 0 |  | 18.5 |  | 5 |  | 2 |  | 25+TT |  | 13.5-TT |  |
| 7 |  | 23 |  | 0 |  | 3 |  | 5.5 |  | 0 |  | 17.5 |  | 5 |  | 2 |  | 25+TT |  | 12.5-TT |  |
| 8 |  | 23 |  | 0 |  | 3 |  | 7 |  | 0 |  | 16 |  | 5 |  | 2 |  | 25+TT |  | 11-TT |  |
| 9 |  | 23 |  | 0 |  | 1.5 |  | 4.5 |  | 0 |  | 18.5 |  | 5 |  | 2 |  | 25+TT |  | 13.5-TT |  |
| 10 |  | 23 |  | 0 |  | 3 |  | 11 |  | 0 |  | 12 |  | 6 |  | 2 |  | 25+TT |  | 6-TT |  |
| 11 |  | 23 |  | 0 |  | 3 |  | 7 |  | 0 |  | 16 |  | 5 |  | 2 |  | 25+TT |  | 11-TT |  |
| 12 |  | 23 |  | 0 |  | 3 |  | 5.5 |  | 0 |  | 17.5 |  | 5 |  | 2 |  | 25+TT |  | 12.5-TT |  |
| 13 |  | 23 |  | 0 |  | 3 |  | 7 |  | 0 |  | 16 |  | 5 |  | 2 |  | 25+TT |  | 11-TT |  |
| 14 |  | 23 |  | 0 |  | 1.5 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 15 |  | 23 |  | 0 |  | 3 |  | 11 |  | 0 |  | 12 |  | 6 |  | 2 |  | 25+TT |  | 6-TT |  |
| 16 |  | 23 |  | 0 |  | 6.5 |  | 4.5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 17 |  | 23 |  | 0 |  | 6.5 |  | 4.5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 18 |  | 23 |  | 0 |  | 6.5 |  | 4.5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 19 |  | 23 |  | 0 |  | 6.5 |  | 5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 20 |  | 23 |  | 0 |  | 6.5 |  | 7 |  | 0 |  | 16 |  | 5 |  | 2 |  | 25+TT |  | 11-TT |  |
| 21 |  | 23 |  | 0 |  | 6.5 |  | 4.5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 22 |  | 23 |  | 0 |  | 6.5 |  | 5.5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 23 |  | 23 |  | 0 |  | 6.5 |  | 7 |  | 0 |  | 16 |  | 5 |  | 2 |  | 25+TT |  | 11-TT |  |
| 24 |  | 23 |  | 0 |  | 6.5 |  | 4.5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 25 |  | 23 |  | 0 |  | 6.5 |  | 11 |  | 0 |  | 12 |  | 6 |  | 2 |  | 25+TT |  | 6-TT |  |
| 26 |  | 23 |  | 0 |  | 6.5 |  | 7 |  | 0 |  | 16 |  | 5 |  | 2 |  | 25+TT |  | 11-TT |  |
| 27 |  | 23 |  | 0 |  | 6.5 |  | 5.5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 28 |  | 23 |  | 0 |  | 6.5 |  | 7 |  | 0 |  | 16 |  | 5 |  | 2 |  | 25+TT |  | 11-TT |  |
| 29 |  | 23 |  | 0 |  | 6.5 |  | 5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 30 |  | 23 |  | 0 |  | 6.5 |  | 11 |  | 0 |  | 12 |  | 6 |  | 2 |  | 25+TT |  | 6-TT |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-7: UE Power Class 3 test requirements (NS_49) for band n1

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 23 |  | 0 |  | 3 |  | 4.5 |  | 0 |  | 18.5 |  | 5 |  | 2 |  | 25+TT |  | 13.5-TT |  |
| 2 |  | 23 |  | 0 |  | 3 |  | 4.5 |  | 0 |  | 18.5 |  | 5 |  | 2 |  | 25+TT |  | 13.5-TT |  |
| 3 |  | 23 |  | 0 |  | 3 |  | 4.5 |  | 0 |  | 18.5 |  | 5 |  | 2 |  | 25+TT |  | 13.5-TT |  |
| 4 |  | 23 |  | 0 |  | 3 |  | 11 |  | 0 |  | 12 |  | 6 |  | 2 |  | 25+TT |  | 6-TT |  |
| 5 |  | 23 |  | 0 |  | 3 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 6 |  | 23 |  | 0 |  | 3 |  | 4.5 |  | 0 |  | 18.5 |  | 5 |  | 2 |  | 25+TT |  | 13.5-TT |  |
| 7 |  | 23 |  | 0 |  | 3 |  | 5.5 |  | 0 |  | 17.5 |  | 5 |  | 2 |  | 25+TT |  | 12.5-TT |  |
| 8 |  | 23 |  | 0 |  | 3 |  | 7 |  | 0 |  | 16 |  | 5 |  | 2 |  | 25+TT |  | 11-TT |  |
| 9 |  | 23 |  | 0 |  | 3 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 10 |  | 23 |  | 0 |  | 3 |  | 11 |  | 0 |  | 12 |  | 6 |  | 2 |  | 25+TT |  | 6-TT |  |
| 11 |  | 23 |  | 0 |  | 1.5 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 12 |  | 23 |  | 0 |  | 3 |  | 5.5 |  | 0 |  | 17.5 |  | 5 |  | 2 |  | 25+TT |  | 12.5-TT |  |
| 13 |  | 23 |  | 0 |  | 3 |  | 7 |  | 0 |  | 16 |  | 5 |  | 2 |  | 25+TT |  | 11-TT |  |
| 14 |  | 23 |  | 0 |  | 1.5 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 15 |  | 23 |  | 0 |  | 3 |  | 11 |  | 0 |  | 12 |  | 6 |  | 2 |  | 25+TT |  | 6-TT |  |
| 16 |  | 23 |  | 0 |  | 3 |  | 11 |  | 0 |  | 12 |  | 6 |  | 2 |  | 25+TT |  | 6-TT |  |
| 17 |  | 23 |  | 0 |  | 1.5 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 18 |  | 23 |  | 0 |  | 1.5 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 19 |  | 23 |  | 0 |  | 3 |  | 7 |  | 0 |  | 16 |  | 5 |  | 2 |  | 25+TT |  | 11-TT |  |
| 20 |  | 23 |  | 0 |  | 3 |  | 11 |  | 0 |  | 12 |  | 6 |  | 2 |  | 25+TT |  | 6-TT |  |
| 21 |  | 23 |  | 0 |  | 3 |  | 11 |  | 0 |  | 12 |  | 6 |  | 2 |  | 25+TT |  | 6-TT |  |
| 22 |  | 23 |  | 0 |  | 6.5 |  | 4.5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 23 |  | 23 |  | 0 |  | 6.5 |  | 4.5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 24 |  | 23 |  | 0 |  | 6.5 |  | 4.5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 25 |  | 23 |  | 0 |  | 6.5 |  | 11 |  | 0 |  | 12 |  | 6 |  | 2 |  | 25+TT |  | 6-TT |  |
| 26 |  | 23 |  | 0 |  | 6.5 |  | 5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 27 |  | 23 |  | 0 |  | 6.5 |  | 4.5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 28 |  | 23 |  | 0 |  | 6.5 |  | 5.5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 29 |  | 23 |  | 0 |  | 6.5 |  | 7 |  | 0 |  | 16 |  | 5 |  | 2 |  | 25+TT |  | 11-TT |  |
| 30 |  | 23 |  | 0 |  | 6.5 |  | 5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 31 |  | 23 |  | 0 |  | 6.5 |  | 11 |  | 0 |  | 12 |  | 6 |  | 2 |  | 25+TT |  | 6-TT |  |
| 32 |  | 23 |  | 0 |  | 6.5 |  | 5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 33 |  | 23 |  | 0 |  | 6.5 |  | 5.5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 34 |  | 23 |  | 0 |  | 6.5 |  | 7 |  | 0 |  | 16 |  | 5 |  | 2 |  | 25+TT |  | 11-TT |  |
| 35 |  | 23 |  | 0 |  | 6.5 |  | 5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 36 |  | 23 |  | 0 |  | 6.5 |  | 11 |  | 0 |  | 12 |  | 6 |  | 2 |  | 25+TT |  | 6-TT |  |
| 37 |  | 23 |  | 0 |  | 6.5 |  | 11 |  | 0 |  | 12 |  | 6 |  | 2 |  | 25+TT |  | 6-TT |  |
| 38 |  | 23 |  | 0 |  | 6.5 |  | 5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 39 |  | 23 |  | 0 |  | 6.5 |  | 5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 40 |  | 23 |  | 0 |  | 6.5 |  | 7 |  | 0 |  | 16 |  | 5 |  | 2 |  | 25+TT |  | 11-TT |  |
| 41 |  | 23 |  | 0 |  | 6.5 |  | 11 |  | 0 |  | 12 |  | 6 |  | 2 |  | 25+TT |  | 6-TT |  |
| 42 |  | 23 |  | 0 |  | 6.5 |  | 11 |  | 0 |  | 12 |  | 6 |  | 2 |  | 25+TT |  | 6-TT |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-8: UE Power Class 3 test requirements (NS_100) for band n1

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 |  | 23 |  | 3 |  | 4 |  | 0 |  | 19 |  | 5 |  | 2 |  | 25+TT |  | 14-TT |
| 3 |  | 23 |  | 3 |  | 4 |  | 0 |  | 19 |  | 5 |  | 2 |  | 25+TT |  | 14-TT |
| 4, 5 |  | 23 |  | 3 |  | 4 |  | 0 |  | 19 |  | 5 |  | 2 |  | 25+TT |  | 14-TT |
| 6 |  | 23 |  | 3 |  | 4 |  | 0 |  | 19 |  | 5 |  | 2 |  | 25+TT |  | 14-TT |
| 7, 8 |  | 23 |  | 3.5 |  | 4 |  | 0 |  | 19 |  | 5 |  | 2 |  | 25+TT |  | 14-TT |
| 9 |  | 23 |  | 3.5 |  | 4 |  | 0 |  | 19 |  | 5 |  | 2 |  | 25+TT |  | 14-TT |
| 10, 11 |  | 23 |  | 6.5 |  | 6.5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |
| 12 |  | 23 |  | 6.5 |  | 6.5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-9: UE Power Class 3 test requirements (NS_100) for band n2, n3, n25

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 3 | 23 | 3 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 4, 5 | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 6 | 23 | 3 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 7, 8 | 23 | 3.5 | 4 | 1.5 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 9 | 23 | 3.5 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 10, 11 | 23 | 6.5 | 6.5 | 1.5 | 15 | 6 | 2 | 25+TT | 9-TT |
| 12 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-10: UE Power Class 3 test requirements (NS_03/NS_03U) for band n66, n70

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2 |  | NS_03, NS_03U |  | 23 |  | 3 |  | 4 |  | 0 |  | 19 |  | 5 |  | 2 |  | 25+TT |  | 14-TT |  |
| 3 |  | NS_03, NS_03U |  | 23 |  | 3 |  | 4 |  | 0 |  | 19 |  | 5 |  | 2 |  | 25+TT |  | 14-TT |  |
| 4,5 |  | NS_03, NS_03U |  | 23 |  | 3 |  | 4 |  | 0 |  | 19 |  | 5 |  | 2 |  | 25+TT |  | 14-TT |  |
| 6 |  | NS_03, NS_03U |  | 23 |  | 3 |  | 4 |  | 0 |  | 19 |  | 5 |  | 2 |  | 25+TT |  | 14-TT |  |
| 7,8 |  | NS_03, NS_03U |  | 23 |  | 3.5 |  | 4 |  | 0 |  | 19 |  | 5 |  | 2 |  | 25+TT |  | 14-TT |  |
| 9 |  | NS_03, NS_03U |  | 23 |  | 3.5 |  | 4 |  | 0 |  | 19 |  | 5 |  | 2 |  | 25+TT |  | 14-TT |  |
| 10,11 |  | NS_03, NS_03U |  | 23 |  | 6.5 |  | 6.5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 12 |  | NS_03, NS_03U |  | 23 |  | 6.5 |  | 6.5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-11: UE Power Class 3 test requirements (NS_03/NS_03U) for band n2, n25

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2 |  | NS_03, NS_03U |  | 23 |  | 3 |  | 4 |  | 1.5 |  | 17.5 |  | 5 |  | 2 |  | 25+TT |  | 12.5-TT |  |
| 3 |  | NS_03, NS_03U |  | 23 |  | 3 |  | 4 |  | 0 |  | 19 |  | 5 |  | 2 |  | 25+TT |  | 14-TT |  |
| 4,5 |  | NS_03, NS_03U |  | 23 |  | 3 |  | 4 |  | 1.5 |  | 17.5 |  | 5 |  | 2 |  | 25+TT |  | 12.5-TT |  |
| 6 |  | NS_03, NS_03U |  | 23 |  | 3 |  | 4 |  | 0 |  | 19 |  | 5 |  | 2 |  | 25+TT |  | 14-TT |  |
| 7,8 |  | NS_03, NS_03U |  | 23 |  | 3.5 |  | 4 |  | 1.5 |  | 17.5 |  | 5 |  | 2 |  | 25+TT |  | 12.5-TT |  |
| 9 |  | NS_03, NS_03U |  | 23 |  | 3.5 |  | 4 |  | 0 |  | 19 |  | 5 |  | 2 |  | 25+TT |  | 14-TT |  |
| 10,11 |  | NS_03, NS_03U |  | 23 |  | 6.5 |  | 6.5 |  | 1.5 |  | 15 |  | 6 |  | 2 |  | 25+TT |  | 9-TT |  |
| 12 |  | NS_03, NS_03U |  | 23 |  | 6.5 |  | 6.5 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-12: UE Power Class 3 test requirements (NS_46) for band n7

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 23 |  | 0 |  | 3 |  | 6 |  | 0 |  | 17 |  | 5 |  | 2 |  | 25+TT |  | 12-TT |  |
| 2 |  | 23 |  | 0 |  | 6.5 |  | 6 |  | 0 |  | 16.5 |  | 5 |  | 2 |  | 25+TT |  | 11.5-TT |  |
| 3 |  | 23 |  | 0 |  | 3 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 4 |  | 23 |  | 0 |  | 3.5 |  | 5 |  | 0 |  | 18 |  | 5 |  | 2 |  | 25+TT |  | 13-TT |  |
| 5 |  | 23 |  | 0 |  | 3 |  | 3.5 |  | 0 |  | 19.5 |  | 5 |  | 2 |  | 25+TT |  | 14.5-TT |  |
| 6 |  | 23 |  | 0 |  | 3.5 |  | 3.5 |  | 0 |  | 19.5 |  | 5 |  | 2 |  | 25+TT |  | 14.5-TT |  |
| 7 |  | 23 |  | 0 |  | 3 |  | 5.5 |  | 0 |  | 17.5 |  | 5 |  | 2 |  | 25+TT |  | 12.5-TT |  |
| 8 |  | 23 |  | 0 |  | 3.5 |  | 5.5 |  | 0 |  | 17.5 |  | 5 |  | 2 |  | 25+TT |  | 12.5-TT |  |
| 9 |  | 23 |  | 0 |  | 3 |  | 7 |  | 0 |  | 16 |  | 5 |  | 2 |  | 25+TT |  | 11-TT |  |
| 10 |  | 23 |  | 0 |  | 6.5 |  | 7 |  | 0 |  | 16 |  | 5 |  | 2 |  | 25+TT |  | 11-TT |  |
| 11 |  | 23 |  | 0 |  | 3 |  | 11 |  | 0 |  | 12 |  | 6 |  | 2 |  | 25+TT |  | 6-TT |  |
| 12 |  | 23 |  | 0 |  | 6.5 |  | 11 |  | 0 |  | 12 |  | 6 |  | 2 |  | 25+TT |  | 6-TT |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-13: UE Power Class 3 test requirements (NS_21) for band n30

| Test ID | ChBw (MHz) | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) |  | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2 | 5 | 23 | 3 | 0 | 0 |  | 20 | 6 | 2 | 25+TT | 14-TT |
|  | 10 | 23 | 3 | 6 | 0 |  | 14 | 6 | 2 | 25+TT | 8-TT |
| 3 | 5 | 23 | 3 | 0 | 0 |  | 20 | 6 | 2 | 25+TT | 14-TT |
|  | 10 | 23 | 3 | 5.5 | 0 |  | 14.5 | 6 | 2 | 25+TT | 8.5-TT |
| 4,5 | 10 | 23 | 3 | 4 | 0 |  | 16 | 5 | 2 | 25+TT | 11-TT |
| 6,7 | 5 | 23 | 3 | 0 | 0 |  | 20 | 6 | 2 | 25+TT | 14-TT |
|  | 10 | 23 | 3 | 6 | 0 |  | 14 | 6 | 2 | 25+TT | 8-TT |
| 8 | 5 | 23 | 3 | 0 | 0 |  | 20 | 6 | 2 | 25+TT | 14-TT |
|  | 10 | 23 | 3 | 5.5 | 0 |  | 14.5 | 6 | 2 | 25+TT | 8.5-TT |
| 9,10 | 10 | 23 | 3 | 4 | 0 |  | 16 | 5 | 2 | 25+TT | 11-TT |
| 11,12 | 5 | 23 | 3.5 | 0 | 0 |  | 19.5 | 5 | 2 | 25+TT | 14.5-TT |
|  | 10 | 23 | 3.5 | 6 | 0 |  | 13.5 | 6 | 2 | 25+TT | 7.5-TT |
| 13 | 5 | 23 | 3.5 | 0 | 0 |  | 19.5 | 5 | 2 | 25+TT | 14.5-TT |
|  | 10 | 23 | 3.5 | 5.5 | 0 |  | 14 | 6 | 2 | 25+TT | 8-TT |
| 14,15 | 10 | 23 | 3.5 | 4 | 0 |  | 15.5 | 6 | 2 | 25+TT | 9.5-TT |
| 16,17 | 5 | 23 | 6.5 | 0 | 0 |  | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
|  | 10 | 23 | 6.5 | 6 | 0 |  | 10.5 | 7 | 2 | 25+TT | 3.5-TT |
| 18 | 5 | 23 | 6.5 | 0 | 0 |  | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 19,20 | 10 | 23 | 6.5 | 5.5 | 0 |  | 11 | 6 | 2 | 25+TT | 5-TT |
|  | 10 | 23 | 6.5 | 4 | 0 |  | 12.5 | 6 | 2 | 25+TT | 6.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-14: UE Power Class 3 test requirements (NS_44) for band n38

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 2 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 3 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 5 | 2 | 25+TT | 14.5-TT |
| 4 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 5 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 6 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 7 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 8 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 9 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 5 | 2 | 25+TT | 14.5-TT |
| 10 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 11 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 12 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 13 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 14 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 2 | 25+TT | 13-TT |
| 16 | 23 | 0 | 3.5 | 8 | 0 | 15 | 6 | 2 | 25+TT | 9-TT |
| 17 | 23 | 0 | 3.5 | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| 18 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 19 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 20 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 21 | 23 | 0 | 6.5 | 8 | 0 | 15 | 6 | 2 | 25+TT | 9-TT |
| 22 | 23 | 0 | 6.5 | 12 | 0 | 11 | 6 | 2 | 25+TT | 5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-15: UE Power Class 3 test requirements (NS_27) for band n48

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 23 |  | 0 |  | 3 |  | 6 |  | 0 |  | 17 |  | 5 |  | 3 |  | 25+TT |  | 12-TT |  |
| 2 |  | 23 |  | 0 |  | 3 |  | 6 |  | 0 |  | 17 |  | 5 |  | 3 |  | 25+TT |  | 12-TT |  |
| 3 |  | 23 |  | 0 |  | 3 |  | 6 |  | 0 |  | 17 |  | 5 |  | 3 |  | 25+TT |  | 12-TT |  |
| 4 |  | 23 |  | 0 |  | 3 |  | 6 |  | 0 |  | 17 |  | 5 |  | 3 |  | 25+TT |  | 12-TT |  |
| 5 |  | 23 |  | 0 |  | 3 |  | 4 |  | 0 |  | 19 |  | 5 |  | 3 |  | 25+TT |  | 14-TT |  |
| 6 |  | 23 |  | 0 |  | 3 |  | 2 |  | 0 |  | 20 |  | 6 |  | 3 |  | 25+TT |  | 14-TT |  |
| 7 |  | 23 |  | 0 |  | 3 |  | 4 |  | 0 |  | 19 |  | 5 |  | 3 |  | 25+TT |  | 14-TT |  |
| 8 |  | 23 |  | 0 |  | 3 |  | 2 |  | 0 |  | 20 |  | 6 |  | 3 |  | 25+TT |  | 14-TT |  |
| 9 |  | 23 |  | 0 |  | 3 |  | 6 |  | 0 |  | 17 |  | 5 |  | 3 |  | 25+TT |  | 12-TT |  |
| 10 |  | 23 |  | 0 |  | 3 |  | 6 |  | 0 |  | 17 |  | 5 |  | 3 |  | 25+TT |  | 12-TT |  |
| 11 |  | 23 |  | 0 |  | 3 |  | 6 |  | 0 |  | 17 |  | 5 |  | 3 |  | 25+TT |  | 12-TT |  |
| 12 |  | 23 |  | 0 |  | 3 |  | 6 |  | 0 |  | 17 |  | 5 |  | 3 |  | 25+TT |  | 12-TT |  |
| 13 |  | 23 |  | 0 |  | 3 |  | 4 |  | 0 |  | 19 |  | 5 |  | 3 |  | 25+TT |  | 14-TT |  |
| 14 |  | 23 |  | 0 |  | 3 |  | 2 |  | 0 |  | 20 |  | 6 |  | 3 |  | 25+TT |  | 14-TT |  |
| 15 |  | 23 |  | 0 |  | 3 |  | 4 |  | 0 |  | 19 |  | 5 |  | 3 |  | 25+TT |  | 14-TT |  |
| 16 |  | 23 |  | 0 |  | 3 |  | 2 |  | 0 |  | 20 |  | 6 |  | 3 |  | 25+TT |  | 14-TT |  |
| 17 |  | 23 |  | 0 |  | 3 |  | 11.5 |  | 0 |  | 11.5 |  | 6 |  | 3 |  | 25+TT |  | 5.5-TT |  |
| 18 |  | 23 |  | 0 |  | 3 |  | 7 |  | 0 |  | 16 |  | 5 |  | 3 |  | 25+TT |  | 11-TT |  |
| 19 |  | 23 |  | 0 |  | 3 |  | 5.5 |  | 0 |  | 17.5 |  | 5 |  | 3 |  | 25+TT |  | 12.5-TT |  |
| 20 |  | 23 |  | 0 |  | 3 |  | 11.5 |  | 0 |  | 11.5 |  | 6 |  | 3 |  | 25+TT |  | 5.5-TT |  |
| 21 |  | 23 |  | 0 |  | 3 |  | 11.5 |  | 0 |  | 11.5 |  | 6 |  | 3 |  | 25+TT |  | 5.5-TT |  |
| 22 |  | 23 |  | 0 |  | 3 |  | 7 |  | 0 |  | 16 |  | 5 |  | 3 |  | 25+TT |  | 11-TT |  |
| 23 |  | 23 |  | 0 |  | 3 |  | 5.5 |  | 0 |  | 17.5 |  | 5 |  | 3 |  | 25+TT |  | 12.5-TT |  |
| 24 |  | 23 |  | 0 |  | 3 |  | 11.5 |  | 0 |  | 11.5 |  | 6 |  | 3 |  | 25+TT |  | 5.5-TT |  |
| 25 |  | 23 |  | 0 |  | 3 |  | 4 |  | 0 |  | 19 |  | 5 |  | 3 |  | 25+TT |  | 14-TT |  |
| 26 |  | 23 |  | 0 |  | 3 |  | 4.5 |  | 0 |  | 18.5 |  | 5 |  | 3 |  | 25+TT |  | 13.5-TT |  |
| 27 |  | 23 |  | 0 |  | 3 |  | 4 |  | 0 |  | 19 |  | 5 |  | 3 |  | 25+TT |  | 14-TT |  |
| 28 |  | 23 |  | 0 |  | 3 |  | 4.5 |  | 0 |  | 18.5 |  | 5 |  | 3 |  | 25+TT |  | 13.5-TT |  |
| 29 |  | 23 |  | 0 |  | 3 |  | 6 |  | 0 |  | 17 |  | 5 |  | 3 |  | 25+TT |  | 12-TT |  |
| 30 |  | 23 |  | 0 |  | 3 |  | 6 |  | 0 |  | 17 |  | 5 |  | 3 |  | 25+TT |  | 12-TT |  |
| 31 |  | 23 |  | 0 |  | 3 |  | 6 |  | 0 |  | 17 |  | 5 |  | 3 |  | 25+TT |  | 12-TT |  |
| 32 |  | 23 |  | 0 |  | 3 |  | 6 |  | 0 |  | 17 |  | 5 |  | 3 |  | 25+TT |  | 12-TT |  |
| 33 |  | 23 |  | 0 |  | 3 |  | 4 |  | 0 |  | 19 |  | 5 |  | 3 |  | 25+TT |  | 14-TT |  |
| 34 |  | 23 |  | 0 |  | 3 |  | 2 |  | 0 |  | 20 |  | 6 |  | 3 |  | 25+TT |  | 14-TT |  |
| 35 |  | 23 |  | 0 |  | 3 |  | 4 |  | 0 |  | 19 |  | 5 |  | 3 |  | 25+TT |  | 14-TT |  |
| 36 |  | 23 |  | 0 |  | 3 |  | 2 |  | 0 |  | 20 |  | 6 |  | 3 |  | 25+TT |  | 14-TT |  |
| 37 |  | 23 |  | 0 |  | 3 |  | 6 |  | 0 |  | 17 |  | 5 |  | 3 |  | 25+TT |  | 12-TT |  |
| 38 |  | 23 |  | 0 |  | 3 |  | 6 |  | 0 |  | 17 |  | 5 |  | 3 |  | 25+TT |  | 12-TT |  |
| 39 |  | 23 |  | 0 |  | 3 |  | 6 |  | 0 |  | 17 |  | 5 |  | 3 |  | 25+TT |  | 12-TT |  |
| 40 |  | 23 |  | 0 |  | 3 |  | 6 |  | 0 |  | 17 |  | 5 |  | 3 |  | 25+TT |  | 12-TT |  |
| 41 |  | 23 |  | 0 |  | 3 |  | 4 |  | 0 |  | 19 |  | 5 |  | 3 |  | 25+TT |  | 14-TT |  |
| 42 |  | 23 |  | 0 |  | 3 |  | 2 |  | 0 |  | 20 |  | 6 |  | 3 |  | 25+TT |  | 14-TT |  |
| 43 |  | 23 |  | 0 |  | 3 |  | 4 |  | 0 |  | 19 |  | 5 |  | 3 |  | 25+TT |  | 14-TT |  |
| 44 |  | 23 |  | 0 |  | 3 |  | 2 |  | 0 |  | 20 |  | 6 |  | 3 |  | 25+TT |  | 14-TT |  |
| 45 |  | 23 |  | 0 |  | 3 |  | 11.5 |  | 0 |  | 11.5 |  | 6 |  | 3 |  | 25+TT |  | 5.5-TT |  |
| 46 |  | 23 |  | 0 |  | 3 |  | 7 |  | 0 |  | 16 |  | 5 |  | 3 |  | 25+TT |  | 11-TT |  |
| 47 |  | 23 |  | 0 |  | 3 |  | 5.5 |  | 0 |  | 17.5 |  | 5 |  | 3 |  | 25+TT |  | 12.5-TT |  |
| 48 |  | 23 |  | 0 |  | 3 |  | 11.5 |  | 0 |  | 11.5 |  | 6 |  | 3 |  | 25+TT |  | 5.5-TT |  |
| 49 |  | 23 |  | 0 |  | 3 |  | 11.5 |  | 0 |  | 11.5 |  | 6 |  | 3 |  | 25+TT |  | 5.5-TT |  |
| 50 |  | 23 |  | 0 |  | 3 |  | 7 |  | 0 |  | 16 |  | 5 |  | 3 |  | 25+TT |  | 11-TT |  |
| 51 |  | 23 |  | 0 |  | 3 |  | 5.5 |  | 0 |  | 17.5 |  | 5 |  | 3 |  | 25+TT |  | 12.5-TT |  |
| 52 |  | 23 |  | 0 |  | 3 |  | 11.5 |  | 0 |  | 11.5 |  | 6 |  | 3 |  | 25+TT |  | 5.5-TT |  |
| 53 |  | 23 |  | 0 |  | 3 |  | 4 |  | 0 |  | 19 |  | 5 |  | 3 |  | 25+TT |  | 14-TT |  |
| 54 |  | 23 |  | 0 |  | 3 |  | 4.5 |  | 0 |  | 18.5 |  | 5 |  | 3 |  | 25+TT |  | 13.5-TT |  |
| 55 |  | 23 |  | 0 |  | 3 |  | 4 |  | 0 |  | 19 |  | 5 |  | 3 |  | 25+TT |  | 14-TT |  |
| 56 |  | 23 |  | 0 |  | 3 |  | 4.5 |  | 0 |  | 18.5 |  | 5 |  | 3 |  | 25+TT |  | 13.5-TT |  |
| 57 |  | 23 |  | 0 |  | 3.5 |  | 6 |  | 0 |  | 17 |  | 5 |  | 3 |  | 25+TT |  | 12-TT |  |
| 58 |  | 23 |  | 0 |  | 3.5 |  | 6 |  | 0 |  | 17 |  | 5 |  | 3 |  | 25+TT |  | 12-TT |  |
| 59 |  | 23 |  | 0 |  | 3.5 |  | 6 |  | 0 |  | 17 |  | 5 |  | 3 |  | 25+TT |  | 12-TT |  |
| 60 |  | 23 |  | 0 |  | 3.5 |  | 6 |  | 0 |  | 17 |  | 5 |  | 3 |  | 25+TT |  | 12-TT |  |
| 61 |  | 23 |  | 0 |  | 3.5 |  | 4 |  | 0 |  | 19 |  | 5 |  | 3 |  | 25+TT |  | 14-TT |  |
| 62 |  | 23 |  | 0 |  | 3.5 |  | 2 |  | 0 |  | 19.5 |  | 5 |  | 3 |  | 25+TT |  | 14.5-TT |  |
| 63 |  | 23 |  | 0 |  | 3.5 |  | 4 |  | 0 |  | 19 |  | 5 |  | 3 |  | 25+TT |  | 14-TT |  |
| 64 |  | 23 |  | 0 |  | 3.5 |  | 2 |  | 0 |  | 19.5 |  | 5 |  | 3 |  | 25+TT |  | 14.5-TT |  |
| 65 |  | 23 |  | 0 |  | 3.5 |  | 6 |  | 0 |  | 17 |  | 5 |  | 3 |  | 25+TT |  | 12-TT |  |
| 66 |  | 23 |  | 0 |  | 3.5 |  | 6 |  | 0 |  | 17 |  | 5 |  | 3 |  | 25+TT |  | 12-TT |  |
| 67 |  | 23 |  | 0 |  | 3.5 |  | 6 |  | 0 |  | 17 |  | 5 |  | 3 |  | 25+TT |  | 12-TT |  |
| 68 |  | 23 |  | 0 |  | 3.5 |  | 6 |  | 0 |  | 17 |  | 5 |  | 3 |  | 25+TT |  | 12-TT |  |
| 69 |  | 23 |  | 0 |  | 3.5 |  | 4 |  | 0 |  | 19 |  | 5 |  | 3 |  | 25+TT |  | 14-TT |  |
| 70 |  | 23 |  | 0 |  | 3.5 |  | 2 |  | 0 |  | 19.5 |  | 5 |  | 3 |  | 25+TT |  | 14.5-TT |  |
| 71 |  | 23 |  | 0 |  | 3.5 |  | 4 |  | 0 |  | 19 |  | 5 |  | 3 |  | 25+TT |  | 14-TT |  |
| 72 |  | 23 |  | 0 |  | 3.5 |  | 2 |  | 0 |  | 19.5 |  | 5 |  | 3 |  | 25+TT |  | 14.5-TT |  |
| 73 |  | 23 |  | 0 |  | 3.5 |  | 11.5 |  | 0 |  | 11.5 |  | 6 |  | 3 |  | 25+TT |  | 5.5-TT |  |
| 74 |  | 23 |  | 0 |  | 3.5 |  | 7 |  | 0 |  | 16 |  | 5 |  | 3 |  | 25+TT |  | 11-TT |  |
| 75 |  | 23 |  | 0 |  | 3.5 |  | 5.5 |  | 0 |  | 17.5 |  | 5 |  | 3 |  | 25+TT |  | 12.5-TT |  |
| 76 |  | 23 |  | 0 |  | 3.5 |  | 11.5 |  | 0 |  | 11.5 |  | 6 |  | 3 |  | 25+TT |  | 5.5-TT |  |
| 77 |  | 23 |  | 0 |  | 3.5 |  | 11.5 |  | 0 |  | 11.5 |  | 6 |  | 3 |  | 25+TT |  | 5.5-TT |  |
| 78 |  | 23 |  | 0 |  | 3.5 |  | 7 |  | 0 |  | 16 |  | 5 |  | 3 |  | 25+TT |  | 11-TT |  |
| 79 |  | 23 |  | 0 |  | 3.5 |  | 5.5 |  | 0 |  | 17.5 |  | 5 |  | 3 |  | 25+TT |  | 12.5-TT |  |
| 80 |  | 23 |  | 0 |  | 3.5 |  | 11.5 |  | 0 |  | 11.5 |  | 6 |  | 3 |  | 25+TT |  | 5.5-TT |  |
| 81 |  | 23 |  | 0 |  | 3.5 |  | 4 |  | 0 |  | 19 |  | 5 |  | 3 |  | 25+TT |  | 14-TT |  |
| 82 |  | 23 |  | 0 |  | 3.5 |  | 4.5 |  | 0 |  | 18.5 |  | 5 |  | 3 |  | 25+TT |  | 13.5-TT |  |
| 83 |  | 23 |  | 0 |  | 3.5 |  | 4 |  | 0 |  | 19 |  | 5 |  | 3 |  | 25+TT |  | 14-TT |  |
| 84 |  | 23 |  | 0 |  | 3.5 |  | 4.5 |  | 0 |  | 18.5 |  | 5 |  | 3 |  | 25+TT |  | 13.5-TT |  |
| 85 |  | 23 |  | 0 |  | 6.5 |  | 0 |  | 0 |  | 16.5 |  | 5 |  | 3 |  | 25+TT |  | 11.5-TT |  |
| 86 |  | 23 |  | 0 |  | 6.5 |  | 0 |  | 0 |  | 16.5 |  | 5 |  | 3 |  | 25+TT |  | 11.5-TT |  |
| 87 |  | 23 |  | 0 |  | 6.5 |  | 0 |  | 0 |  | 16.5 |  | 5 |  | 3 |  | 25+TT |  | 11.5-TT |  |
| 88 |  | 23 |  | 0 |  | 6.5 |  | 0 |  | 0 |  | 16.5 |  | 5 |  | 3 |  | 25+TT |  | 11.5-TT |  |
| 89 |  | 23 |  | 0 |  | 6.5 |  | 0 |  | 0 |  | 16.5 |  | 5 |  | 3 |  | 25+TT |  | 11.5-TT |  |
| 90 |  | 23 |  | 0 |  | 6.5 |  | 2 |  | 0 |  | 16.5 |  | 5 |  | 3 |  | 25+TT |  | 11.5-TT |  |
| 91 |  | 23 |  | 0 |  | 6.5 |  | 0 |  | 0 |  | 16.5 |  | 5 |  | 3 |  | 25+TT |  | 11.5-TT |  |
| 92 |  | 23 |  | 0 |  | 6.5 |  | 2 |  | 0 |  | 16.5 |  | 5 |  | 3 |  | 25+TT |  | 11.5-TT |  |
| 93 |  | 23 |  | 0 |  | 6.5 |  | 0 |  | 0 |  | 16.5 |  | 5 |  | 3 |  | 25+TT |  | 11.5-TT |  |
| 94 |  | 23 |  | 0 |  | 6.5 |  | 0 |  | 0 |  | 16.5 |  | 5 |  | 3 |  | 25+TT |  | 11.5-TT |  |
| 95 |  | 23 |  | 0 |  | 6.5 |  | 0 |  | 0 |  | 16.5 |  | 5 |  | 3 |  | 25+TT |  | 11.5-TT |  |
| 96 |  | 23 |  | 0 |  | 6.5 |  | 0 |  | 0 |  | 16.5 |  | 5 |  | 3 |  | 25+TT |  | 11.5-TT |  |
| 97 |  | 23 |  | 0 |  | 6.5 |  | 0 |  | 0 |  | 16.5 |  | 5 |  | 3 |  | 25+TT |  | 11.5-TT |  |
| 98 |  | 23 |  | 0 |  | 6.5 |  | 2 |  | 0 |  | 16.5 |  | 5 |  | 3 |  | 25+TT |  | 11.5-TT |  |
| 99 |  | 23 |  | 0 |  | 6.5 |  | 0 |  | 0 |  | 16.5 |  | 5 |  | 3 |  | 25+TT |  | 11.5-TT |  |
| 100 |  | 23 |  | 0 |  | 6.5 |  | 2 |  | 0 |  | 16.5 |  | 5 |  | 3 |  | 25+TT |  | 11.5-TT |  |
| 101 |  | 23 |  | 0 |  | 6.5 |  | 11.5 |  | 0 |  | 11.5 |  | 6 |  | 3 |  | 25+TT |  | 5.5-TT |  |
| 102 |  | 23 |  | 0 |  | 6.5 |  | 7 |  | 0 |  | 16 |  | 5 |  | 3 |  | 25+TT |  | 11-TT |  |
| 103 |  | 23 |  | 0 |  | 6.5 |  | 0 |  | 0 |  | 16.5 |  | 5 |  | 3 |  | 25+TT |  | 11.5-TT |  |
| 104 |  | 23 |  | 0 |  | 6.5 |  | 11.5 |  | 0 |  | 11.5 |  | 6 |  | 3 |  | 25+TT |  | 5.5-TT |  |
| 105 |  | 23 |  | 0 |  | 6.5 |  | 11.5 |  | 0 |  | 11.5 |  | 6 |  | 3 |  | 25+TT |  | 5.5-TT |  |
| 106 |  | 23 |  | 0 |  | 6.5 |  | 7 |  | 0 |  | 16 |  | 5 |  | 3 |  | 25+TT |  | 11-TT |  |
| 107 |  | 23 |  | 0 |  | 6.5 |  | 0 |  | 0 |  | 16.5 |  | 5 |  | 3 |  | 25+TT |  | 11.5-TT |  |
| 108 |  | 23 |  | 0 |  | 6.5 |  | 11.5 |  | 0 |  | 11.5 |  | 6 |  | 3 |  | 25+TT |  | 5.5-TT |  |
| 109 |  | 23 |  | 0 |  | 6.5 |  | 0 |  | 0 |  | 16.5 |  | 5 |  | 3 |  | 25+TT |  | 11.5-TT |  |
| 110 |  | 23 |  | 0 |  | 6.5 |  | 4.5 |  | 0 |  | 16.5 |  | 5 |  | 3 |  | 25+TT |  | 11.5-TT |  |
| 111 |  | 23 |  | 0 |  | 6.5 |  | 0 |  | 0 |  | 16.5 |  | 5 |  | 3 |  | 25+TT |  | 11.5-TT |  |
| 112 |  | 23 |  | 0 |  | 6.5 |  | 4.5 |  | 0 |  | 16.5 |  | 5 |  | 3 |  | 25+TT |  | 11.5-TT |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied.

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

< End of changes >