3GPP TSG RAN Meeting #96-e	 	R5-225122r1

Electronic Meeting, 15th – 26th August 2021

Source:		Rohde & Schwarz

Title:	On the feasibility of the PHR method

Agenda Item:		5.4.17

Document for:	Discussion and Endorsement

# Introduction

At the previous meeting, RAN5 has been requested to study the feasibility of the PHR method (Option 1C) in order to resolve the Scell drop issue in FR2 UL CA test cases and quickly unblock UL CA testing [1]. In this contribution, we present the results of our analysis.

# Discussion

While we have estimated the impact of the PHR method on test system implementation, a test of the method with hardware has yet not been possible. Therefore, we cannot provide a definite answer whether the PHR method can be implemented with a report periodicity of 10ms.

We note that the TE needs to send the TPC commands in advance since the UE requires sometime between receiving a TPC command and sending an updated PHR report. Based on the k2 configuration in PUSCH-ConfigCommon for an SCS of 120 kHz in TS 38.508-1 [2], the delay is 1 ms. Thus, the TE has less than 9 ms, to decode the PHR report and to send TPC commands based on its content.

Observation 1: There is a time lag between the two events of the UE receiving a TPC command and UE sending an updated PHR report.

In general, there should not be any power changes during a RF measurement, as they can have detrimental effects on the measurement result and in addition some test requirements are not valid close to or during power changes. Therefore, no TPC commands shall be sent by the TE during the measurement.

Observation 2: During measurements, TPC commands shall not be sent in order to ensure accurate measurement results.

Considering observation 2, it is required that the PHR method terminates after the PHR for each CC is at its target level before the measurement can start. Thus, the PHR method is based on the assumption that the PHR report remains constant when no TPC commands are sent. Otherwise, when the PHR fluctuates although no TPC commands are sent, the Scell drop cannot always be avoided.

Observation 3: In the PHR method it is assumed that the PHR does not change its value when path loss update is disabled by alpha=0 and the UE does not receive a TPC command.

Due to observation 3, it is not necessary to evaluate each report when setting the PHR values of the individual carriers to their target values.

Observation 4: It is not necessary to evaluate each PHR report sent by the UE when setting the PHR values of the individual carriers to their target values.

In order to quickly unblock UL CA testing, it is beneficial to reduce the complexity of the implementation of the PHR method. Therefore, we propose to leave it up to TE implementation whether to analyze all PHR reports or to skip some PHR reports, i.e., to adjust the cell powers with any delay between the decoding of the PHR report and the transmission of TPC commands. After the TE has sent a TPC command, it checks one of the following PHR reports and decides whether and which TPC command is sent until the PHR values for each carrier are at their target values. In the end, it is only important that the TE implementation achieves the correct PHR, the exact details of analyzing the reports and sending the TPC commands can be left up to implementation.

In the following, we outline the adaptations of the test procedure compared to Ericsson’s draft CR with changes highlighted by change marks.

6.2A.2.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 and Annex C.3.0 for all downlink physical channels

2.	The SS shall configure SCC as per TS 38.508-1 [10] subclause 5.5.1 Procedure to configure SCC(s) for NR RF CA testing. Message contents are defined in subclause 6.2A.2.1.4.3.

3.	Apply the test step based on the 5G NR UE Release:

3a.	For Release 16 and forward 5G NR UEs supporting the UPLF test mode: SS applies a backoff on the PCell power by activating the UE Power Limit Function (UPLF).  The ACTIVATE POWER LIMIT REQUEST procedure is performed as specified in TS 38.508-1 [10] clause 4.9.32 using condition ‘NR FR2 2CA’. UE shall transmit ACTIVATE POWER LIMIT RESPONSE to SS. Go to step 4.

3b.	For Release 15 5G NR UEs and 5G NR UEs not supporting the UPLF test mode: No action

3c. For testing single CC MPR requirement: No action

4.  SS activates SCC by sending the activation MAC CE (Refer TS 38.321, clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[25], clause 9.3).

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2A.2.1.4.1-1 to Table 6.2A.2.1.4.1-7. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

7.	Apply the test step based on the 5G NR UE Release:

7a. For Release 16 and forward 5G NR UEs supporting the UPLF test mode: Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 msec starting from the first TPC command in this step to ensure that the UE transmits at its maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

7b. For Release 15 5G NR UEs and 5G NR UEs not supporting the UPLF test mode: Send uplink power control commands in uplink scheduling information to the UE per CCuntil the Power Headroom Report (PHR) from the UE for each CC is at the target value according to Table 6.2A.2.1.4.2-1.

Table 6.2A.2.1.4.2-1: Power target values for test procedure using PHR

| BW ratio (Note 1) | Xmax [dB] (Note 2) | Target PHR | ΔPHR [dB] (Note 3) | BW combination examples |
| --- | --- | --- | --- | --- |
| 1/2 | 3.0 | POWER_HEADROOM_36 (3  PH  4) | 1 | 2CC equal BW |
| 1/3 | 4.8 | POWER_HEADROOM_38 (5  PH  6) | 1.2 | 2CC 50+100 MHz CC1 |
| 2/3 | 1.8 | POWER_HEADROOM_35 (2  PH  3) | 1.2 | 2CC 50+100 MHz CC2 |
| 1/5 | 7.0 | POWER_HEADROOM_40 (7  PH  8) | 1.0 | 2CC 50+200 MHz CC1 |
| 4/5 | 1.0 | POWER_HEADROOM_34 (1  PH  2) | 1.0 | 2CC 50+200 MHz CC2 |
| 1/9 | 9.5 | POWER_HEADROOM_43 (10  PH  11) | 1.5 | 2CC 50+400 MHz CC1 |
| 8/9 | 0.5 | POWER_HEADROOM_34 (1  PH  2) | 1.5 | 2CC 50+400 MHz CC2 |
| Note 1: The BW ratio is the ratio of BW of the CC over the total Aggregated BW Note 1: Xmax = 10log(BW ratio) Note 3: ΔPHR is the worst case UE output power decrease due to Xmax and 1 dB reporting granularity of PHR according to TS38.133 [x]. At the target PHR, the UE Power Headroom is within the range Xmax≤PH<Xmax+ΔPHR |  |  |  |  |

7c. For testing single CC MPR requirement: Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 msec starting from the first TPC command in this step to ensure that the UE transmits at its maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

…

Proposal 1: Allow to skip PHR reports and update the test procedure as outlined above.

In the following, we want to compare the alternative approach of applying test mode to the PHR method:

In the definition of UE power limit function UPLF [3, 4], the UE shall apply a configured power backoff on the primary component carrier. However, it is not specified how the power is distributed on the secondary component carriers. Thus, it is up to the UE implementation, how the power is distributed among the PCC and each SCC and an equal PSD of all CCs is not ensured.

Observation 5: In the UE power limit function (UPLF), it is not specified how the UE distributes the power on the SCCs. Thus, the power for a specific SCC is completely unknown.

For several FR2 testcases, e.g. EVM, ACLR and OBW the MU and testability is governed by the lowest power of the individual CCs. For OBW, in addition the power imbalance between CCs has a significant impact on the measurement uncertainty. Without a lower bound on the power of individual CCs the MU analysis cannot proceed.

As shown in discussion paper [5], equal PSD for unequal channel BW is achievable by using carrier specific Xmax.

Observation 6: The PHR method can ensure equal PSD of all CCs.

Consequently the MU analysis for several CA TCs, e.g. EVM, ACLR and OBW is enabled. Since the power for each of the CCs is higher than in the test mode approach, the MU is reduced for the PHR method in comparison to the test mode approach. Thus the testability for the PHR method is better than that for test mode.

Observation 7: The PHR method enables MU analysis for CA TCs such as EVM, ACLR and OBW with a reduced MU and increased testability compared to the test mode method.

Due to these advantages, we propose to apply the PHR method as baseline method in the test procedure for UE Release 15 and further releases.

Proposal 2: Apply the PHR method as baseline method for UE release 15 and further releases.

In summary, it is not required to evaluate each report sent by the UE. Thus, the method’s feasibility is independent on timing requirements when the test procedure is updated as proposed. In addition the PHR method has advantages with respect to MU and testability compared to test mode approach.

# Conclusion

In summary, the feasibility of the PHR method has been assessed.

Observation 1: There is a time lag between the two events of the UE receiving a TPC command and UE sending an updated PHR report.

Observation 2: During measurements, TPC commands shall not be sent in order to ensure accurate measurement results.

Observation 3: In the PHR method it is assumed that the PHR does not change its value when path loss update is disabled by alpha=0 and the UE does not receive a TPC command.

Observation 4: It is not necessary to evaluate each PHR report sent by the UE when setting the PHR values of the individual carriers to their target values.

Proposal 1: Allow to skip PHR reports and update the test procedure as outlined above.

Observation 5: In the UE power limit function (UPLF), it is not specified how the UE distributes the power on the SCCs. Thus, the power for a specific CC is completely unknown.

Observation 6: The PHR method can ensure equal PSD of all CCs with sufficient accuracy.

Observation 7: The PHR method enables MU analysis for CA TCs such as EVM, ACLR and OBW with a reduced MU and increased testability compared to the test mode method.

Proposal 2: Apply the PHR method as baseline method for UE release 15 and further releases.

# References

- R5-223633, New method for preventing SCell drop in RAN5 FR2 UL CA test cases, Ericsson, 3GPP TSG RAN WG5 Meeting #95-e, May 2022

- TS 38.508-1, V17.5.0, 3GPP TSG RAN WG5, June 2022

- R5-224080, Rel-16 Addition of new test function to limit Pcell power: Option 1, Keysight Technologies UK Ltd, Apple Portugal, 3GPP TSG RAN WG5 Meeting #96-e, August 2022

- R5-224079, Rel-16 Pcell power limit test function proposal including unequal bandwidth case, Keysight Technologies UK Ltd, Apple Portugal, 3GPP TSG RAN WG5 Meeting #96-e, August 2022

- R5-225191, New method for preventing SCell drop in RAN5 FR2 UL CA test cases, Ericsson, 3GPP TSG RAN WG5 Meeting #96-e, August 2022