3GPP TSG-RAN5 Meeting #96-e	R5-225125r1

# Online, , 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3194 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC RRC test case 8.1.1.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - For step 1 : The SS transmits one IP packet to the UE on the DRB associated with the default PDU session on Cell 1. Note 2: Default PDU session type is set in the PICS pc_APN_Default_Configuration The setting of pc_APN_Default_Configuration is currently used to assign a DNN name in the case that the UE does not provide one, it does not guarantee that UE has established a PDU session of the particular type. - In preamble, IP PDU delay is set to 5 secs for TLB mode B and SS sends IP packet at step 1. UE tries to loop it back to SS after step 5 but it cannot be transmitted successfully as T302 (waittime) is running. Hence, the PDU may or may not be transmitted after step 6. Updated the test procedure accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Changed “default PDU session” to “first PDU session” and voided note 2. - Updated the test procedure to make the data loopback from the UE as optional |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case may not execute correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated with TTCN CR R5s220851 r1-> Updated the test procedure to make the data loopback from the UE as optional |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 8.1.1.2.3	RRC connection establishment / RRC Reject with wait time

8.1.1.2.3.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state and has sent an RRCSetupRequest message }

ensure that {

when { UE receives an RRCReject message including an IE waitTime }

then { UE doesn't re-send RRCSetupRequest message before the waitTime is expired }

}

8.1.1.2.3.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331 clause 5.3.15.2.

[TS 38.331, clause 5.3.15.2]

The UE shall:

1>	stop timer T300, if running;

1>	stop timer T319, if running;

1>	stop timer T302, if running;

1>	reset MAC and release the default MAC Cell Group configuration;

1>	if waitTime is configured in the RRCReject:

2>	start timer T302, with the timer value set to the waitTime;

1>	if RRCReject is received in response to a request from upper layers:

2>	inform the upper layer that access barring is applicable for all access categories except categories '0' and '2';

1>	if RRCReject is received in response to an RRCSetupRequest:

2>	inform upper layers about the failure to setup the RRC connection, upon which the procedure ends;

1>	else if RRCReject is received in response to an RRCResumeRequest or an RRCResumeRequest1:

2>	if resume is triggered by upper layers:

3>	inform upper layers about the failure to resume the RRC connection;

2>	if resume is triggered due to an RNA update:

3>	set the variable pendingRnaUpdate to true;

2>	discard the current KgNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key derived in accordance with 5.3.13.3;

2>	suspend SRB1, upon which the procedure ends;

The RRC_INACTIVE UE shall continue to monitor paging while the timer T302 is running.

8.1.1.2.3.3	Test Description

8.1.1.2.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cell.

UE:

-	None.

Preamble:

-	The UE is in 5GS state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3 and Test Loop Function (On) with UE test loop mode B is established.

8.1.1.2.3.3.2	Test procedure sequence

Table 8.1.1.2.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits one IP packet to the UE on the DRB associated with the first PDU session on NR Cell 1. | - | - | - | - |
| 2 | Wait for 1 second after the IP packet has been transmitted in step 1. (Note 1) | - | - | - | - |
| 3 | The SS transmits an RRCRelease message on NR Cell 1. | <-- | NR RRC: RRCRelease | - | - |
| 4-4A | Steps 1-2 of the NR RRC_CONNECTED procedure in table 4.5.4.2-3 in TS 38.508-1[4] are performed. | - | - | - | - |
| 5 | The SS responds with RRCReject message with IE waitTime set to 10s. | <-- | NR RRC: RRCReject | - | - |
| 6 | Check: Does the UE transmit an RRCSetupRequest message while timer T302 is running? | --> | NR RRC: RRCSetupRequest | 1 | F |
| 6A | The SS starts timer Timer_1 = 10 s | - | - | - | - |
| - | EXCEPTION: Steps 15a1 to 15b1 describe a transaction that depends on the UE behaviour; the "lower case letter" identifies a step sequence that takes place if a specific behaviour happens. (Note 3) | - | - | - | - |
| 7-14 | Void | - | - | - | - |
| 15a1 | Check: Does UE transmit RRCSetupRequest message for sending the IP packet received in step 1 after timer T302 expires? | - | NR RRC: RRCSetupRequest | 1 | P |
| 15a2-15a6 | Steps 3 to 7 of the NR RRC_CONNECTED procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed. | - | - | - | - |
| - | EXCEPTION: Steps 15a7 and 15a8 can occur in any order. | - | - | - | - |
| 15a7 | The UE transmits an RRCReconfigurationComplete message on Cell 1. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 15a8 | The UE loops back the IP packet received in step 1 on the DRB associated with the first PDU session on NR Cell 1. | - | - | - | - |
| 15a9 | The SS transmits an RRCRelease message on NR Cell 1. | <-- | NR RRC: RRCRelease | - | - |
| 15b1 | The SS waits for Timer_1 expiry | - | - | - | - |
| Note 1: The 1 second delay is used to secure that the UE has received and forwarded the IP Packet transmitted by the SS in step 1 to the UE test loop function before the RRCRelease message is sent by the SS in step 3. Note 2: Void |  |  |  |  |  |

8.1.1.2.3.3.3	Specific message contents

Table 8.1.1.2.3.3.3-1: RRCReject (step 5, table 8.1.1.2.3.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReject ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReject SEQUENCE { |  |  |  |
| waitTime | 10 | 10 seconds |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.1.2.3.3.3-2: CLOSE UE TEST LOOP (Preamble, Table 8.1.1.2.3.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.7A-3, condition UE TEST LOOP MODE B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | '0000 0101'B | 5 seconds |  |

Table 8.1.1.2.3.3.3-3: SERVICE REQUEST (Step 15a3, Table 8.1.1.2.3.3.2-1)

| Derivation Path: TS 38.508-1[4], Table 4.7.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Service type | '0001'B | data |  |