3GPP TSG-RAN5 Meeting #96-e	R5-225189r2

# Electronic Meeting, 15th – 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0380 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update PICS for RedCap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1, There is no PICS for HD-FDD for RedCap UE, and RF test need this PICS |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Add new PICS to 38.508-2 for RedCap UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no PICS for HD-FDD for RedCap UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | 1st revision: - replace the ‘-‘ with ‘_’ in the PICS 2nd revision Set DL 256QAM to optional for RedCap UE as per 38.306 4.2.7.10. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

A.4.3.2	Physical Layer Baseline Implementation Capabilities

Table A.4.3.2-1: UE Physical Layer Baseline Implementation Capabilities

| Item | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | M | If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support PDSCH reception based on semi-persistent scheduling | 38.306, 4.2.7.10 | Rel-15 | pc_downlinkSPS | No |  |  |
| 2 | Support 256QAM for PDSCH for FR1 | 38.306, 4.2.7.10 | Rel-15 | pc_pdsch_256QAM_FR1 | CY |  | Mandatory for non-RedCap UEs and optional for RedCap UEs. |
| 3 | Support 256QAM for PDSCH for at least one NR FR2 band | 38.306, 4.2.7.2 | Rel-15 | pc_pdsch_256QAM_FR2 | No |  |  |
| 4 | Support 256QAM for PUSCH for at least one NR FR1 band | 38.306, 4.2.7.2 | Rel-15 | pc_pusch_256QAM_FR1 | No |  |  |
| 4a | Support 256QAM for PUSCH for at least one NR FR2 band | 38.306, 4.2.7.2 | Rel-15 | pc_pusch_256QAM_FR2 | No |  |  |
| 5 | Support receiving PDSCH using PDSCH mapping type A with less than seven symbols | 38.306, 4.2.7.10 | Rel-15 | pc_pdsch_MappingTypeA | Yes | Yes |  |

<End of modified section 1>

<Start of modified section 2>

A.4.3.12	RedCap Capabilities

Table A.4.3.12-1: RedCap Capabilities

| Item | UE Sidelink Capabilities | Ref. | Release | Mnemonic | M | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support of 16 DRBs for RedCap UEs. | 38.306, 4.2.21.2 | Rel-17 | pc_16DRB_RedCap _r17 | No |  |  |
| 2 | Support of RedCap. | 38.306, 4.2.21.2 | Rel-17 | pc_RedCap_r17 | No |  |  |
| 3 | Support of 18 bit length of PDCP sequence for RedCap UEs. | 38.306, 4.2.21.3 | Rel-17 | pc_longSN_RedCap_r17 | No |  |  |
| 4 | Support of AM DRB with 18 bit length of RLC sequence number for RedCap UEs. | 38.306, 4.2.21.4 | Rel-17 | pc_ amWithLongSN_RedCap_r17 | No |  |  |
| 5 | Support of Half-duplex FDD operation (instead of full-duplex FDD operation) type A for RedCap UE. | 38.306 4.2.21.6.1 | Rel-17 | pc_HD_FDD_RedCap_r17 | No |  |  |

<End of modified section 2>