3GPP TSG RAN WG5 Meeting #96-e	R5-225201r1

Electronic Meeting, 15th Aug 2022 – 26th Aug 2022

Agenda Item:	6.6.6.6

Source:	Ericsson

Title:	NG.114 MNO provisioning

Document for:	Endorsement

# 1	Introduction

NG.114 [1] includes “Annex C MNO provisioning and Late Customization”. E.g., an open market device according NG.114 [1], C.1

“1. Supports non-roaming and roaming cases;

2. Has a default configuration suitable for many MNOs; and

3. Can be configured to the MNO’s needs.”

# 2	Discussion

NG.114 main and MNO provisioning

The main body of NG.114 [1] includes normative requirements and mandatory parts of annex C. MNO provisioning is defined in annex C.

This defines the UE behaviour and are part of all UEs.

Observation 2-1:

Configuration parameters according to NG.114 [1], C.3

“The UE must use the default value for each parameter in Table C.3-1 unless configured differently as described in Annex C.2.”

If not configured differently, only the default value is required and used. This is not clarified in the IMS test cases.

Specifically, for EVS, the EVS Configuration A2: br=5.9-24.4; bw=nb-swb is default according to Table C.3-1 Configuration parameters and their default values

| EVS/Br | 5.9-24.4 | Section 15.2 of 3GPP TS 26.114 [16] (/<X>/Speech/<X>/EVS/Br) and section 5 of 3GPP TS 26.441 [56] (Table 1) | 3.2.2.3 |
| --- | --- | --- | --- |
| EVS/Bw | nb-swb | Section 15.2 of 3GPP TS 26.114 [16] (/<X>/Speech/<X>/EVS/Bw) and section 5 of 3GPP TS 26.441 [56] (Table 1) | 3.2.2.3 |

Proposal 2-1:

It’s proposed to clarify the only required and used EVS default configuration for each IMS test case.

# 3	Proposal

It’s proposed to implement the proposal 2-1.

# 4	References

[1] GSMA PRD NG.114: "IMS Profile for Voice, Video and Messaging over 5GS, Version 2.0".