3GPP TSG RAN WG4 Meeting #103-e		 R4-2211129

# Electronic Meeting, May 9th – 20th, 2022

Title:	Reply LS on Additional RF requirements for NS_03U, NS_05U and NS_43U

Response to:	R4-2207624 (R5-221613)

Release:	Release-15

Work Item:	5GS_NR_LTE-UEConTest

Source:	TSG RAN WG4

To:	TSG RAN WG5

Cc:	None

Contact person:	Gene Fong

<gfong@qti.qualcomm.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	R4-2210811

# 1	Overall description

RAN4 would like to thank RAN5 for the LS on additional requirements for NS_03U, NS_05U, and NS_43U.  Indeed, RAN4 agrees with RAN5 that the 38.101-1 specification is not fully clear on the requirements imposed by the signaling of the above-mentioned NS values.  RAN4 would like to inform RAN5 that the intent of the NS value NS_xxU is to apply the requirements of NS_xx in addition to the requirement on UTRA ACLR.  For example, as listed in Table 6.2.3.1-1 of 38.101-1 (snippet shown below), the requirement for NS_05U is specified in clauses 6.5.3.3.4 and 6.5.2.4.2.

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 (NOTE 4) | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | Table 5.3.2-1 | N/A |
| NS_03 | 6.5.2.3.3 | n2, n25, n66, n70, n86 |  |  | Clause 6.2.3.7 |
| NS_03U | 6.5.2.3.3, 6.5.2.4.2 | n2, n25, n66, n86 |  |  | Clause 6.2.3.7 |
| NS_04 | 6.5.2.3.2, 6.5.3.3.1 | n41 | 10, 15, 20, 40, 50, 60 80, 90, 100 |  | Clause 6.2.3.2 |
| NS_05 | 6.5.3.3.4 | n1, n84 | 5, 10, 15, 20 (NOTE 2) |  | Clause 6.2.3.4 |
| NS_05U | 6.5.3.3.4, 6.5.2.4.2 | n1, n84 | 5, 10, 15, 20 |  | Clause 6.2.3.4 |
| … |  |  |  |  |  |
| NS_43 | 6.5.3.3.5 | n8, n81 | 5, 10, 15 |  | Clause 6.2.3.6 |
| NS_43U | 6.5.3.3.5, 6.5.2.4.2 | n8, n81 | 5, 10, 15 |  | Clause 6.2.3.6 |

Clause 6.5.3.3.4 contains the same requirements as applicable for NS_05, but clause 6.5.2.4.2 contains additionally a specification for UTRA ACLR intended to be signaled in deployments where UTRA may be present in the same frequency range.

RAN4 has agreed on corrections to the specification to indicate clearly that the requirements in clause 6.5.3.3.4 apply to both NS_05 and NS_05U.  The same correction is made for NS_03U and NS_43U.

# 2	Actions

To TSG WG5

ACTION: 	RAN4 kindly requests RAN5 to take the above into consideration in their further work.

# 3	Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #104-e			August 15 – 26, 2022			Electronic meeting

TSG RAN WG4 Meeting #104-bis-e		October 10 – 19, 2022			Electronic meeting