3GPP TSG-RAN5 Meeting #97	R5-227384

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2057 | rev | - | Current version: | 17.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additional requirements and A-MPR for NS_21 and n30 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Portugal |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 core specifications have implemented the regulatory requirements for Canadian WCS according to the the ISED-Canada clarification of the RSS-195 regulation in RP-221047 |  |  |  |  |  |  |  |  |
| Summary of change: |  | The network signalling value NS_21 is updated with the associated requirements and A-MPR for Canadian WCS. The A-MPR for 5MHz is introduced to clause 6.2.3.3.14. Applicability to NS_21 is removed from Clause 6.5.2.3.3. The WCS emission requirements for SEM are covered in a new clause 6.5.2.3.9. A new modified MPR behaviour bit is defined in Annex J.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Implementations for band n30 with Canadian regulatory requirements will remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3, 6.5.3.3, 6.5.2.3.9, Annex J.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Alignment with core spec TS 38.101-1 v17.7.0 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

6.2.3.3.14	A-MPR for NS_21

Table 6.2.3.3.14-1: A-MPR for "NS_21"

| Channel Bandwidth (MHz) | Modulation/Waveform |  | Region A1a RBstart ≤ 1.44MHz/12/SCS LCRB ≤ [0.54] MHz/12/SCS | Region A1b RBstart ≤ 1.44MHz/12/SCS LCRB > [0.54] MHz/12/SCS LCRB ≤ 2.16MHz/12/SCS | Region A2 LCRB > 5.4MHz/12/SCS | Region A3b RBend ≥ 7.74MHz/12/SCS LCRB > [0.54] MHz/12/SCS LCRB ≤ 2.16MHz/12/SCS | Region A3a RBend ≥ 7.74MHz/12/SCS LCRB ≤ [0.54] MHz/12/SCS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer/Inner |  | Outer | Outer/Inner |  |
| 10 | DFT-s-OFDM | PI/2 BPSK | 6 | 3 | 4 | 3 | 6 |
|  |  | QPSK | 6 | 3 | 4 | 3 | 6 |
|  |  | 16 QAM | 6 | 3 | 4 | 3 | 6 |
|  |  | 64 QAM | 6 | 3 | 4 | 3 | 6 |
|  |  | 256 QAM | 6 | 3 | 4 | 3 | 6 |
|  | CP-OFDM | QPSK | 6 | 4 | 5.5 | 4 | 6 |
|  |  | 16 QAM | 6 | 4 | 5.5 | 4 | 6 |
|  |  | 64 QAM | 6 | 4 | 5.5 | 4 | 6 |
|  |  | 256 QAM | 6 | 4 | 5.5 | 4 | 6 |

Table 6.2.3.14-2: A-MPR for "NS_21" (applicable to UEs Release 17 and forward indicating modifiedMPR-Behavior)

| Channel Bandwidth (MHz) | Modulation/Waveform |  | Region A1a RBstart ≤ 0.36MHz/12/SCS LCRB ≤ 0.54 MHz/12/SCS | Region A1b RBstart ≤ 0.36MHz/12/SCS LCRB > 0.54 MHz/12/SCS LCRB ≤ 2.52MHz/12/SCS | Region A2 LCRB > 2.52MHz/12/SCS | Region A3b RBend ≥ 3.96MHz/12/SCS LCRB > 0.54 MHz/12/SCS LCRB ≤ 2.52MHz/12/SCS | Region A3a RBend ≥ 3.96MHz/12/SCS LCRB ≤ 0.54 MHz/12/SCS |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer/Inner |  | Outer | Outer/Inner |  |
| 5 |  | DFT-s-OFDM |  | PI/2 BPSK | ≤ 4.0 |  | ≤ 2.0 |  | ≤ 1.5 |  | 2.0 |  | 4.0 |  |
|  |  |  |  | QPSK | ≤ 4.5 |  | ≤ 2.5 |  | ≤ 2.0 |  | ≤ 2.5 |  | ≤ 4.5 |  |
|  |  |  |  | 16 QAM | ≤ 4.5 |  | ≤ 2.5 |  | ≤ 2.5 |  | ≤ 2.5 |  | ≤ 4.5 |  |
|  |  |  |  | 64 QAM | ≤ 4.5 |  | ≤ 2.5 |  | ≤ 2.5 |  | ≤ 2.5 |  | ≤ 4.5 |  |
|  |  |  |  | 256 QAM | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 4.5 |  |
|  |  | CP-OFDM |  | QPSK | ≤ 4.5 |  | ≤ 4.0 |  | ≤ 4.0 |  | ≤ 4.0 |  | ≤ 4.5 |  |
|  |  |  |  | 16 QAM | ≤ 4.5 |  | ≤ 4.0 |  | ≤ 4.0 |  | ≤ 4.0 |  | ≤ 4.5 |  |
|  |  |  |  | 64 QAM | ≤ 4.5 |  | ≤ 4.0 |  | ≤ 4.0 |  | ≤ 4.0 |  | ≤ 4.5 |  |
|  |  |  |  | 256 QAM | ≤ 6.5 |  | ≤ 6.5 |  | ≤ 6.5 |  | ≤ 6.5 |  | ≤ 6.5 |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.14.

< Unchanged sections are skipped >

6.5.2.3.3.3	Requirements for network signalling value "NS_03"

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_03" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.2.3.3.3-1.

Table 6.5.2.3.3.3-1: Additional requirements for "NS_03"

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | Measurement bandwidth |  |
| 0-1 | -13 | -13 | -13 | -13 | -13 | 1 % of channel BW |  |
| 1-6 | -13 | -13 | -13 | -13 | -13 | 1 MHz |  |
| 6-10 | -25 | -13 | -13 | -13 | -13 | 1 MHz |  |
| 10-15 |  | -25 | -13 | -13 | -13 | 1 MHz |  |
| 15-20 |  |  | -25 | -13 | -13 | 1 MHz |  |
| 20-25 |  |  |  | -25 | -13 | 1 MHz |  |
| 25-40 |  |  |  |  | -13 | 1 MHz |  |
| 40-45 |  |  |  |  | -25 | 1 MHz |  |

< Unchanged sections are skipped >

##### 6.5.2.3.9	Requirements for network signalling value "NS_21"

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_21" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.2.3.9-1.

Table 6.5.2.3.9-1: Additional requirements for "NS_21" (applicable to UEs Release 17 and forward indicating modifiedMPR-Behavior)

| ΔfOOB MHz | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  | Measurement bandwidth |
| --- | --- | --- | --- |
|  | 5 | 10 |  |
| 0-1 | -13 | -13 | 1 MHz |
| 1-6 | -13 | -13 | 1 MHz |
| 6-10 | -25 | -13 | 1 MHz |
| 10-15 |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

< Unchanged sections>

Annex J (normative):
ModifiedMPR-Behavior

# J.1	Indication of modified MPR behavior

This annex contains the definitions of the bits in the field modifiedMPR-Behavior indicated per supported NR band in the IE RF-Parameters [7] by a UE supporting an MPR or A-MPR modified in a given version of this specification. A modified MPR or A-MPR behaviour can apply to a supported NR band in stand-alone operation (including CA and NN-DC operation) or in non-standalone operation with the said NR band as part of an EN-DC or NE-DC band combination.

NOTE 1:	In the present release, the modifiedMPR-Behavior is indicated [7] by an 8-bit bitmap per supported NR band.

<End of changes>