3GPP TSG-RAN5 Meeting #97	R5-226049r1

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3221 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates for NE-DC RRC test case 8.2.1.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160, Bureau Veritas, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2022-10-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | With the upgrade to baseline definitions in TS 38.331 version 17.2.0, a set of IEs remain not checked or missing. There’s NOTE 12 defined in n77 that the band has defined dedicate frequency range to be used in USA and Canada according to the spectrum defined by FCC certification and ISED authorization. n7712; 3300 MHz – 4200 MHz; 3300 MHz – 4200 MHz; TDD. NOTE 12: In the USA this band is restricted to 3450 – 3550 MHz and 3700 – 3980 MHz. In Canada this band is restricted to 3450 – 3650 MHz and 3650 – 3980 MHz. Currently, the UE capability test cases is not check these element and need to be updated accordingly. |
| Summary of change: |  | Added check of features depending on existing parameters pc_dl_SchedulingOffset_PDSCH_TypeA pc_NR_dl_DedicatedMessageSegmentation pc_twoPUCCH_group Created Tables 8.2.1.1.2.3.3-4a and 8.2.1.1.2.3.3-4b from Table 8.2.1.1.2.3.3-4 to improve readability. Fixed unbalanced branches in Table 8.2.1.1.2.3.3-4. Updated extendedBand-n77-r16 to “Checked” point in Table 8.2.1.1.2.3.3-4: UE-NR-Capability (Table 8.2.1.1.2.3.3-3). Added check point to extendedBand-n77-2-r17 in Table 8.2.1.1.2.3.3-4: UE-NR-Capability (Table 8.2.1.1.2.3.3-3). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Several capability IEs will be missed in Audit test case 8.2.1.1.2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact. See R5s221179 Other related WICs TEI15_Test, 5GS_NR_LTE-UEConTest Associated new ICS added in TS38.508-2 CR 0401 (R5-226757) and CR 0402 (R5-226759) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: Removed Tables 8.2.1.1.2.3.3-4a and 8.2.1.1.2.3.3-4b to avoid side effects with other CRs raised in the same Tables and contents moved back to original positions. Merged contents from R5-226996 and R5-226997 to include check of extendedBand-n77-r16 and extendedBand-n77-2-r17 respectively |  |  |  |  |  |  |  |  |

##### 8.2.1.1.2	UE capability transfer / Success / NE-DC

8.2.1.1.2.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with NE-DC, and, MCG(s) and SCG }

ensure that {

when { UE receives an UECapabilityEnquiry message that includes nr and eutra-nr }

then { UE transmits an UECapabilityInformation message including UE radio access capability information corresponding to the ue-CapabilityRequest variable }

}

8.2.1.1.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clause 5.6.1.3, 5.6.1.4, 5.7.7.2 and 5.7.7.3.

[TS 38.331, clause 5.6.1.3]

The UE shall set the contents of UECapabilityInformation message as follows:

1>	if the ue-CapabilityRAT-RequestList contains a UE-CapabilityRAT-Request with rat-Type set to nr:

2>	include in the ue-CapabilityRAT-ContainerList a UE-CapabilityRAT-Container of the type UE-NR-Capability and with the rat-Type set to nr;

2>	include the supportedBandCombinationList, featureSets and featureSetCombinations as specified in clause 5.6.1.4;

1>	if the ue-CapabilityRAT-RequestList contains a UE-CapabilityRAT-Request with rat-Type set to eutra-nr:

2> if the UE supports (NG)EN-DC or NE-DC:

3>	include in the ue-CapabilityRAT-ContainerList a UE-CapabilityRAT-Container of the type UE-MRDC-Capability and with the rat-Type set to eutra-nr;

3>	include the supportedBandCombinationList and featureSetCombinations as specified in clause 5.6.1.4;

1>	if the ue-CapabilityRAT-RequestList contains a UE-CapabilityRAT-Request with rat-Type set to eutra:

2>	if the UE supports E-UTRA:

3>	include in the ue-CapabilityRAT-ContainerList a ue-CapabilityRAT-Container of the type UE-EUTRA-Capability and with the rat-Type set to eutra as specified in TS 36.331 [10], clause 5.6.3.3, according to the capabilityRequestFilter, if received;

1>	if the ue-CapabilityRAT-RequestList contains a UE-CapabilityRAT-Request with rat-Type set to utra-fdd:

2>	if the UE supports UTRA-FDD:

3>	include the UE radio access capabilities for UTRA-FDD within a ue-CapabilityRAT-Container and with the rat-Type set to utra-fdd;

1>	if the RRC message segmentation is enabled based on the field rrc-SegAllowed received, and the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 38.323 [5]:

2>	initiate the UL message segment transfer procedure as specified in clause 5.7.7;

1>	else:

2>	submit the UECapabilityInformation message to lower layers for transmission, upon which the procedure ends.

[TS 38.331, clause 5.6.1.4]

The UE invokes the procedures in this clause if the NR or E-UTRA network requests UE capabilities for nr, eutra-nr or eutra. This procedure is invoked once per requested rat-Type (see clause 5.6.1.3 for capability enquiry by the NR network; see TS 36.331 [10], clause 5.6.3.3 for capability enquiry by the E-UTRA network). The UE shall ensure that the feature set IDs are consistent across feature sets, feature set combinations and band combinations in all three UE capability containers that the network queries with the same fields with the same values, i.e. UE-CapabilityRequestFilterNR, UE-CapabilityRequestFilterCommon and fields in UECapabilityEnquiry message (i.e. requestedFreqBandsNR-MRDC, requestedCapabilityNR, eutra-nr-only flag, and requestedCapabilityCommon) as defined in TS 36.331, where applicable.

NOTE 1:	Capability enquiry without frequencyBandListFilter is not supported.

NOTE 2:	In EN-DC, the gNB needs the capabilities for RAT types nr and eutra-nr and it uses the featureSets in the UE-NR-Capability together with the featureSetCombinations in the UE-MRDC-Capability to determine the NR UE capabilities for the supported MRDC band combinations. Similarly, the eNB needs the capabilities for RAT types eutra and eutra-nr and it uses the featureSetsEUTRA in the UE-EUTRA-Capability together with the featureSetCombinations in the UE-MRDC-Capability to determine the E-UTRA UE capabilities for the supported MRDC band combinations. Hence, the IDs used in the featureSets must match the IDs referred to in featureSetCombinations across all three containers. The requirement on consistency implies that there are no undefined feature sets and feature set combinations.

NOTE 3:	If the UE cannot include all feature sets and feature set combinations due to message size or list size constraints, it is up to UE implementation which feature sets and feature set combinations it prioritizes.

The UE shall:

1>	compile a list of "candidate band combinations" according to the filter criteria in capabilityRequestFilterCommon (if included), only consisting of bands included in frequencyBandListFilter, and prioritized in the order of frequencyBandListFilter (i.e. first include band combinations containing the first-listed band, then include remaining band combinations containing the second-listed band, and so on), where for each band in the band combination, the parameters of the band do not exceed maxBandwidthRequestedDL, maxBandwidthRequestedUL, maxCarriersRequestedDL, maxCarriersRequestedUL, ca-BandwidthClassDL-EUTRA or ca-BandwidthClassUL-EUTRA, whichever are received;

1>	for each band combination included in the list of "candidate band combinations":

2>	if the network (E-UTRA) included the eutra-nr-only field, or

2>	if the requested rat-Type is eutra:

3>	remove the NR-only band combination from the list of "candidate band combinations";

NOTE 4:	The (E-UTRA) network may request capabilities for nr but indicate with the eutra-nr-only flag that the UE shall not include any NR band combinations in the UE-NR-Capability. In this case the procedural text above removes all NR-only band combinations from the candidate list and thereby also avoids inclusion of corresponding feature set combinations and feature sets below.

2>	if it is regarded as a fallback band combination with the same capabilities of another band combination included in the list of "candidate band combinations", and

2>	if this fallback band combination is generated by releasing at least one SCell or uplink configuration of SCell or SUL according to TS 38.306 [26]:

3>	remove the band combination from the list of "candidate band combinations";

NOTE 5:	Even if the network requests (only) capabilities for nr, it may include E-UTRA band numbers in the frequencyBandListFilter to ensure that the UE includes all necessary feature sets needed for subsequently requested eutra-nr capabilities. At this point of the procedure the list of "candidate band combinations" contains all NR- and/or E-UTRA-NR band combinations that match the filter (frequencyBandListFilter) provided by the NW and that match the eutra-nr-only flag (if RAT-Type nr is requested by E-UTRA). In the following, this candidate list is used to derive the band combinations, feature set combinations and feature sets to be reported in the requested capability container.

1>	if the requested rat-Type is nr:

2>	include into supportedBandCombinationList as many NR-only band combinations as possible from the list of "candidate band combinations", starting from the first entry;

3>	if srs-SwitchingTimeRequest is received:

4>	if SRS carrier switching is supported;

5>	include srs-SwitchingTimesListNR for each band combination;

4>	set srs-SwitchingTimeRequested to true;

2>	include, into featureSetCombinations, the feature set combinations referenced from the supported band combinations as included in supportedBandCombinationList according to the previous;

2>	compile a list of "candidate feature set combinations" referenced from the list of "candidate band combinations" excluding entries (rows in feature set combinations) with same or lower capabilities;

2>	if uplinkTxSwitchRequest is received:

3>	include into supportedBandCombinationList-UplinkTxSwitch as many NR-only band combinations that supported UL TX switching as possible from the list of "candidate band combinations", starting from the first entry;

4>	if srs-SwitchingTimeRequest is received:

5>	if SRS carrier switching is supported;

6>	include srs-SwitchingTimesListNR for each band combination;

5>	set srs-SwitchingTimeRequested to true;

3>	include, into featureSetCombinations, the feature set combinations referenced from the supported band combinations as included in supportedBandCombinationList-UplinkTxSwitch according to the previous;

NOTE 6:	This list of "candidate feature set combinations" contains the feature set combinations used for NR-only as well as E-UTRA-NR band combinations. It is used to derive a list of NR feature sets referred to from the feature set combinations in the UE-NR-Capability and from the feature set combinations in a UE-MRDC-Capability container.

2>	include into featureSets the feature sets referenced from the "candidate feature set combinations" and may exclude the feature sets with the parameters that exceed any of maxBandwidthRequestedDL, maxBandwidthRequestedUL, maxCarriersRequestedDL or maxCarriersRequestedUL, whichever are received;

1>	else, if the requested rat-Type is eutra-nr:

2>	include into supportedBandCombinationList and/or supportedBandCombinationListNEDC-Only as many E-UTRA-NR band combinations as possible from the list of "candidate band combinations", starting from the first entry;

3>	if srs-SwitchingTimeRequest is received:

4>	if SRS carrier switching is supported;

5>	include srs-SwitchingTimesListNR and srs-SwitchingTimesListEUTRA for each band combination;

4>	set srs-SwitchingTimeRequested to true;

2>	include, into featureSetCombinations, the feature set combinations referenced from the supported band combinations as included in supportedBandCombinationList according to the previous;

2>	if uplinkTxSwitchRequest is received:

3>	include into supportedBandCombinationList-UplinkTxSwitch as many E-UTRA-NR band combinations that supported UL TX switching as possible from the list of "candidate band combinations", starting from the first entry;

4>	if srs-SwitchingTimeRequest is received:

5>	if SRS carrier switching is supported;

6>	include srs-SwitchingTimesListNR and srs-SwitchingTimesListEUTRA for each band combination;

5>	set srs-SwitchingTimeRequested to true;

3>	include, into featureSetCombinations, the feature set combinations referenced from the supported band combinations as included in supportedBandCombinationList-UplinkTxSwitch according to the previous;

1>	else (if the requested rat-Type is eutra):

2>	compile a list of "candidate feature set combinations" referenced from the list of "candidate band combinations" excluding entries (rows in feature set combinations) with same or lower capabilities;

NOTE 7:	This list of "candidate feature set combinations" contains the feature set combinations used for E-UTRA-NR band combinations. It is used to derive a list of E-UTRA feature sets referred to from the feature set combinations in a UE-MRDC-Capability container.

2>	include into featureSetsEUTRA (in the UE-EUTRA-Capability) the feature sets referenced from the "candidate feature set combinations" and may exclude the feature sets with the parameters that exceed ca-BandwidthClassDL-EUTRA or ca-BandwidthClassUL-EUTRA, whichever are received;

1>	include the received frequencyBandListFilter in the field appliedFreqBandListFilter of the requested UE capability, except if the requested rat-Type is nr and the network included the eutra-nr-only field;

1>	if the network included ue-CapabilityEnquiryExt:

2>	include the received ue-CapabilityEnquiryExt in the field receivedFilters;

[TS 38.331, clause 5.7.7.2]

A UE capable of UL RRC message segmentation in RRC_CONNECTED will initiate the procedure when the following conditions are met:

1>	if the RRC message segmentation is enabled based on the field rrc-SegAllowed received, and

1>	if the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 38.323 [5];

Upon initiating the procedure, the UE shall:

1>	initiate transmission of the ULDedicatedMessageSegment message as specified in 5.7.7.3;

[TS 38.331, clause 5.7.7.3]

The UE shall segment the encoded RRC PDU based on the maximum supported size of a PDCP SDU specified in TS 38.323 [5]. UE shall minimize the number of segments and set the contents of the ULDedicatedMessageSegment messages as follows:

1>	F or each new UL DCCH message, set the segmentNumber to 0 for the first message segment and increment the segmentNumber for each subsequent RRC message segment;

1>	set rrc-MessageSegmentContainer to include the segment of the UL DCCH message corresponding to the segmentNumber;

1>	if the segment included in the rrc-MessageSegmentContainer is the last segment of the UL DCCH message:

2>	set the rrc-MessageSegmentType to lastSegment;

1>	else:

2>	set the rrc-MessageSegmentType to notLastSegment;

1>	submit all the ULDedicatedMessageSegment messages generated for the segmented RRC message to lower layers for transmission in ascending order based on the segmentNumber, upon which the procedure ends.

8.2.1.1.2.3	Test description

8.2.1.1.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the PCell and EUTRA Cell 1 is the PS Cell.

UE:

-	None

Preamble:

-	The UE is in state RRC_CONNECTED in NE-DC mode according to TS 38.508-1 [4] clause 4.5.4.2 with MCG(s) and SCG.

8.2.1.1.2.3.2	Test procedure sequence

Table 8.2.1.1.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a UECapabilityEnquiry message including nr and eutra-nr with rrc-segAllowed-r16 set to enabled. | <-- | NR RRC: UECapabilityEnquiry | - | - |
| - | EXCEPTION: The "lower case letter" identifies a step sequence that takes place depending on the size of UE capability message. IF pc_NR_UL_Segmentation and UE’s encoded UECapabilityInformation message >= Max PDCP SDU size then steps 2a1 and 2a2 will be performed else step 2b1 will be performed | - | - | - | - |
| - | EXCEPTION: Step 2a1 is repeated a maximum of 16 times or till rrc-MessageSegmentType-r16 IE within the ULDedicatedMessageSegment-r16 message indicate rrc-MessageSegmentType-r16 = lastSegment. | - | - | - | - |
| 2a1 | Check: Does the UE send segments of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment-r16 message? | --> | NR RRC: ULDedicatedMessageSegment | - | - |
| 2a2 | Check: Is the received UE radio access capability information as per the ue-CapabilityRequest variable? | - | - | 1 | P |
| 2b1 | Check: Does the UE transmit a UECapabilityInformation message including UE radio access capability information as per the ue-CapabilityRequest variable? | --> | NR RRC: UECapabilityInformation | 1 | P |

8.2.1.1.2.3.3	Specific message contents

Table 8.2.1.1.2.3.3-1: UECapabilityEnquiry (step 1, Table 8.2.1.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-31 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueCapabilityEnquiry SEQUENCE { |  |  |  |
| ue-CapabilityRAT-RequestList SEQUENCE (SIZE (1.. maxRAT-CapabilityContainers)) OF UE-CapabilityRAT-Request { | 2 entries |  |  |
| UE-CapabilityRAT-Request[1] SEQUENCE { |  | entry 1 |  |
| rat-Type | nr |  |  |
| } |  |  |  |
| UE-CapabilityRAT-Request[2] SEQUENCE { |  |  |  |
| rat-Type | eutra-nr | entry 2 |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| ue-CapabilityEnquiryExt | OCTET STRING Containing UECapabilityEnquiry-v1560-IEs (Table 8.2.1.1.2.3.3-2) |  |  |
| } |  |  |  |
| criticalExtensionsFuture |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.1.1.2.3.3-2: UECapabilityEnquiry-v1560-IEs (Table 8.2.1.1.2.3.3-1)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| UECapabilityEnquiry-v1560-IEs ::= SEQUENCE { |  |  |  |
| capabilityRequestFilterCommon | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rrc-SegAllowed-r16 | enabled |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.1.1.2.3.3-3: UECapabilityInformation (steps2a2 and 2b1, Table 8.2.1.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityInformation ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueCapabilityInformation SEQUENCE { |  |  |  |
| ue-CapabilityRAT-ContainerList SEQUENCE (SIZE (0..maxRAT-CapabilityContainers)) OF UE-CapabilityRAT-Container { | 2 entries |  |  |
| UE-CapabilityRAT-Container[1] SEQUENCE { |  | entry 1 |  |
| rat-Type | nr |  | pc_NR_FDD or pc_NR_TDD |
| ueCapabilityRAT-Container | UE-NR-Capability | Encoded as per TS 38.331 [12] clause 5.6.1 | pc_NR_FDD or pc_NR_TDD |
| } |  |  |  |
| UE-CapabilityRAT-Container[2] SEQUENCE { |  | entry 2 |  |
| rat-Type | eutra-nr |  | pc_NE_DC |
| ueCapabilityRAT-Container | UE-MRDC-Capability | Encoded as per TS 38.331 [12] clause 5.6.1 | pc_NE_DC |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| criticalExtensionsFuture | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.1.1.2.3.3-4: UE-NR-Capability (Table 8.2.1.1.2.3.3-3)

| Derivation Path: TS 38.331 [12], clause 6.3.3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| UE-NR-Capability ::= SEQUENCE { |  |  |  |  |  |  |  |
| accessStratumRelease |  | Checked |  | "rel-15" or higher |  |  |  |
| pdcp-Parameters SEQUENCE { |  |  |  |  |  |  |  |
| supportedROHC-Profiles SEQUENCE { |  |  |  |  |  |  |  |
| profile0x0000 |  | Not checked |  |  |  |  |  |
| profile0x0001 |  | Not checked |  |  |  |  |  |
| profile0x0002 |  | Not checked |  |  |  |  |  |
| profile0x0003 |  | Not checked |  |  |  |  |  |
| profile0x0004 |  | Not checked |  |  |  |  |  |
| profile0x0006 |  | Not checked |  |  |  |  |  |
| profile0x0101 |  | Not checked |  |  |  |  |  |
| profile0x0102 |  | Not checked |  |  |  |  |  |
| profile0x0103 |  | Not checked |  |  |  |  |  |
| profile0x0104 |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| maxNumberROHC-ContextSessions |  | Not checked |  |  |  |  |  |
| uplinkOnlyROHC-Profiles |  | Not checked |  |  |  |  |  |
| continueROHC-Context |  | Not checked |  |  |  |  |  |
| outOfOrderDelivery |  | Checked |  |  |  | pc_outOfOrderDelivery |  |
| shortSN |  | Checked |  |  |  | pc_shortSN |  |
| pdcp-DuplicationSRB |  | Checked |  |  |  | pc_pdcp_DuplicationSplitSRB |  |
| pdcp-DuplicationMCG-OrSCG-DRB |  | Checked |  |  |  | pc_pdcp_DuplicationMCG_OrSCG_DRB |  |
| drb-IAB-r16 |  | Not checked |  |  |  |  |  |
| non-DRB-IAB-r16 |  | Not checked |  |  |  |  |  |
| extendedDiscardTimer-r16 |  | Not checked |  |  |  |  |  |
| continueEHC-Context-r16 |  | Not checked |  |  |  |  |  |
| ehc-r16 |  | Checked |  |  |  | pc_NR_ehc_r16 |  |
| maxNumberEHC-Contexts-r16 |  | Not checked |  |  |  |  |  |
| jointEHC-ROHC-Config-r16 |  | Not checked |  |  |  |  |  |
| pdcp-DuplicationMoreThanTwoRLC-r16 |  | Checked |  |  |  | pc_pdcp_DuplicationMoreThanTwoRLC_r16 |  |
| longSN-RedCap-r17 |  | Not checked |  |  |  |  |  |
| udc-r17 |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| rlc-Parameters SEQUENCE { |  |  |  |  |  |  |  |
| am-WithShortSN |  | Checked |  |  |  | pc_am_WithShortSN |  |
| um-WithShortSN |  | Checked |  |  |  | pc_um_WithShortSN |  |
| um-WithLongSN |  | Checked |  |  |  | pc_um_WIthLongSN |  |
| extendedT-PollRetransmit-r16 |  | Not checked |  |  |  |  |  |
| extendedT-StatusProhibit-r16 |  | Not checked |  |  |  |  |  |
| am-WithLongSN-RedCap-r17 |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| mac-Parameters SEQUENCE { |  |  |  |  |  |  |  |
| mac-ParametersCommon SEQUENCE { |  |  |  |  |  |  |  |
| lcp-Restriction |  | Checked |  |  |  | pc_lcp_Restriction |  |
| dummy |  | Not checked |  |  |  |  |  |
| lch-ToSCellRestriction |  | Not checked |  |  |  |  |  |
| recommendedBitRate |  | Checked |  |  |  | pc_recommendedBitRate |  |
| recommendedBitRateQuery |  | Checked |  |  |  | pc_recommendedBitRateQuery |  |
| recommendedBitRateMultiplier-r16 |  | Not checked |  |  |  |  |  |
| preEmptiveBSR-r16 |  | Not checked |  |  |  |  |  |
| autonomousTransmission-r16 |  | Not checked |  |  |  |  |  |
| lch-PriorityBasedPrioritization-r16 |  | Checked |  |  |  | pc_lch_PriorityBasedPrioritization_r16 |  |
| lch-ToConfiguredGrantMapping-r16 |  | Not checked |  |  |  |  |  |
| lch-ToGrantPriorityRestriction-r16 |  | Checked |  |  |  | pc_lch_ToGrantPriorityRestriction_r16 |  |
| singlePHR-P-r16 |  | Not checked |  |  |  |  |  |
| ul-LBT-FailureDetectionRecovery-r16 |  | Not checked |  |  |  |  |  |
| tdd-MPE-P-MPR-Reporting-r16 |  | Not checked |  |  |  |  |  |
| lcid-ExtensionIAB-r16 |  | Not checked |  |  |  |  |  |
| spCell-BFR-CBRA-r16 |  | Not checked |  |  |  |  |  |
| srs-ResourceId-Ext-r16 |  | Not checked |  |  |  |  |  |
| enhancedUuDRX-forSidelink-r17 |  | Not checked |  |  |  |  |  |
| mg-ActivationRequestPRS-Meas-r17 |  | Not checked |  |  |  |  |  |
| mg-ActivationCommPRS-Meas-r17 |  | Not checked |  |  |  |  |  |
| intraCG-Prioritization-r17 |  | Not checked |  |  |  |  |  |
| jointPrioritizationCG-Retx-Timer-r17 |  | Not checked |  |  |  |  |  |
| survivalTime-r17 |  | Not checked |  |  |  |  |  |
| lcg-ExtensionIAB-r17 |  | Not checked |  |  |  |  |  |
| harq-FeedbackDisabled-r17 |  | Not checked |  |  |  |  |  |
| uplink-Harq-ModeB-r17 |  | Not checked |  |  |  |  |  |
| sr-TriggeredBy-TA-Report-r17 |  | Not checked |  |  |  |  |  |
| extendedDRX-CycleInactive-r17 |  | Not checked |  |  |  |  |  |
| simultaneousSR-PUSCH-DiffPUCCH-groups-r17 |  | Not checked |  |  |  |  |  |
| lastTransmissionUL-r17 |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| mac-ParametersXDD-Diff SEQUENCE { |  |  |  |  |  |  |  |
| skipUplinkTxDynamic |  | Checked (NOTE 4) |  |  |  | pc_skipUplinkTxDynamic |  |
| logicalChannelSR-DelayTimer |  | Checked (NOTE 5) |  |  |  | pc_logicalChannelSR_DelayTimer |  |
| longDRX-Cycle |  | Checked (NOTE 6) |  |  |  | pc_longDRX_Cycle |  |
| shortDRX-Cycle |  | Checked (NOTE 7) |  |  |  | pc_shortDRX_Cycle |  |
| multipleSR-Configurations |  | Not checked |  |  |  |  |  |
| multipleConfiguredGrants |  | Checked (NOTE 12) |  |  |  | pc_multipleConfiguredGrants_r16 |  |
| secondaryDRX-Group-r16 |  | Not checked |  |  |  |  |  |
| enhancedSkipUplinkTxDynamic-r16 |  | Not checked |  |  |  |  |  |
| enhancedSkipUplinkTxConfigured-r16 |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| phy-Parameters SEQUENCE { |  |  |  |  |  |  |  |
| phy-ParametersCommon SEQUENCE { |  |  |  |  |  |  |  |
| csi-RS-CFRA-ForHO |  | Checked |  |  |  | pc_csi_RS_CFRA_ForHO |  |
| dynamicPRB-BundlingDL |  | Not checked |  |  |  |  |  |
| sp-CSI-ReportPUCCH |  | Not checked |  |  |  |  |  |
| sp-CSI-ReportPUSCH |  | Not checked |  |  |  |  |  |
| nzp-CSI-RS-IntefMgmt |  | Not checked |  |  |  |  |  |
| type2-SP-CSI-Feedback-LongPUCCH |  | Not checked |  |  |  |  |  |
| precoderGranularityCORESET |  | Not checked |  |  |  |  |  |
| dynamicHARQ-ACK-Codebook |  | supported |  |  |  |  |  |
| semiStaticHARQ-ACK-Codebook |  | Not checked |  |  |  |  |  |
| spatialBundlingHARQ-ACK |  | Checked |  |  |  | pc_spatialBundlingHARQ_ACK |  |
| dynamicBetaOffsetInd-HARQ-ACK-CSI |  | Not checked |  |  |  |  |  |
| pucch-Repetition-F1-3-4 |  | Not checked |  |  |  |  |  |
| ra-Type0-PUSCH |  | Checked |  |  |  | pc_ra_Type0_PUSCH |  |
| dynamicSwitchRA-Type0-1-PDSCH |  | Checked |  |  |  | pc_dynamicSwitchRA_Type0_1_PDSCH |  |
| dynamicSwitchRA-Type0-1-PUSCH |  | Checked |  |  |  | pc_dynamicSwitchRA_Type0_1_PUSCH |  |
| pdsch-MappingTypeA |  | supported |  |  |  | pc_pdsch_MappingTypeA |  |
| pdsch-MappingTypeB |  | Checked |  |  |  | pc_pdsch_MappingTypeB |  |
| interleavingVRB-ToPRB-PDSCH |  | Checked |  |  |  | pc_interleavingVRB_ToPRB_PDSCH |  |
| interSlotFreqHopping-PUSCH |  | Not checked |  |  |  |  |  |
| type1-PUSCH-RepetitionMultiSlots |  | Not checked |  |  |  |  |  |
| type2-PUSCH-RepetitionMultiSlots |  | Not checked |  |  |  |  |  |
| pusch-RepetitionMultiSlots |  | Checked |  |  |  | pc_pusch_RepetitionMultiSlots |  |
| pdsch-RepetitionMultiSlots |  | Checked |  |  |  | pc_pdsch_RepetitionMultiSlots |  |
| downlinkSPS |  | Checked |  |  |  | pc_downlinkSPS |  |
| configuredUL-GrantType1 |  | Checked |  |  |  | pc_configuredUL_GrantType1 |  |
| configuredUL-GrantType2 |  | Checked |  |  |  | pc_configuredUL_GrantType2 |  |
| pre-EmptIndication-DL |  | Not checked |  |  |  |  |  |
| cbg-TransIndication-DL |  | Not checked |  |  |  |  |  |
| cbg-TransIndication-UL |  | Not checked |  |  |  |  |  |
| cbg-FlushIndication-DL |  | Not checked |  |  |  |  |  |
| dynamicHARQ-ACK-CodeB-CBG-Retx-DL |  | Not checked |  |  |  |  |  |
| rateMatchingResrcSetSemi-Static |  | Not checked |  |  |  |  |  |
| rateMatchingResrcSetDynamic |  | Not checked |  |  |  |  |  |
| bwp-SwitchingDelay |  | Checked |  | 38.306, 4.2.7.10: It is mandatory to report type 1 or type 2. This capability is not applicable to IAB-MT |  | pc_bwp_SwitchingDelay_Type1 or pc_bwp_SwitchingDelay_Type2 |  |
| dummy |  | Not checked |  |  |  |  |  |
| maxNumberSearchSpaces |  | n10 |  |  |  |  |  |
| rateMatchingCtrlResrsSetDynamic |  | Not checked |  |  |  |  |  |
| maxLayersMIMO-Indication |  | Not checked |  |  |  |  |  |
| spCellPlacement |  | Not checked |  |  |  |  |  |
| twoStepRACH-r16 |  | Not checked |  |  |  |  |  |
| dci-Format1-2And0-2-r16 |  | Checked |  |  |  | pc_dci_Format1_2And0_2_r16 |  |
| monitoringDCI-SameSearchSpace-r16 |  | Not checked |  |  |  |  |  |
| type2-CG-ReleaseDCI-0-1-r16 |  | Not checked |  |  |  |  |  |
| type2-CG-ReleaseDCI-0-2-r16 |  | Not checked |  |  |  |  |  |
| sps-ReleaseDCI-1-1-r16 |  | Not checked |  |  |  |  |  |
| sps-ReleaseDCI-1-2-r16 |  | Not checked |  |  |  |  |  |
| csi-TriggerStateNon-ActiveBWP-r16 |  | Not checked |  |  |  |  |  |
| separateSMTC-InterIAB-Support-r16 |  | Not checked |  |  |  |  |  |
| separateRACH-IAB-Support-r16 |  | Not checked |  |  |  |  |  |
| ul-flexibleDL-SlotFormatSemiStatic-IAB-r16 |  | Not checked |  |  |  |  |  |
| ul-flexibleDL-SlotFormatDynamics-IAB-r16 |  | Not checked |  |  |  |  |  |
| dft-S-OFDM-WaveformUL-IAB-r16 |  | Not checked |  |  |  |  |  |
| dci-25-AI-RNTI-Support-IAB-r16 |  | Not checked |  |  |  |  |  |
| t-DeltaReceptionSupport-IAB-r16 |  | Not checked |  |  |  |  |  |
| guardSymbolReportReception-IAB-r16 |  | Not checked |  |  |  |  |  |
| harqACK-CB-SpatialBundlingPUCCH-Group-r16 |  | Not checked |  |  |  |  |  |
| crossSlotScheduling-r16 |  | Not checked |  |  |  |  |  |
| maxNumberSRS-PosPathLossEstimateAllServingCells-r16 |  | Not checked |  |  |  |  |  |
| extendedCG-Periodicities-r16 |  | Not checked |  |  |  |  |  |
| extendedSPS-Periodicities-r16 |  | Not checked |  |  |  |  |  |
| codebookVariantsList-r16 |  | Not checked |  |  |  |  |  |
| pusch-RepetitionTypeA-r16 |  | Not checked |  |  |  |  |  |
| dci-DL-PriorityIndicator-r16 |  | Checked |  |  |  | pc_dci_DL_PriorityIndicator_r16 |  |
| dci-UL-PriorityIndicator-r16 |  | Checked |  |  |  | pc_dci_UL_PriorityIndicator_r16 |  |
| maxNumberPathlossRS-Update-r16 |  | Not checked |  |  |  |  |  |
| type2-HARQ-ACK-Codebook-r16 |  | Not checked |  |  |  |  |  |
| maxTotalResourcesForAcrossFreqRanges-r16 |  | Not checked |  |  |  |  |  |
| harqACK-separateMultiDCI-MultiTRP-r16 |  | Not checked |  |  |  |  |  |
| harqACK-jointMultiDCI-MultiTRP-r16 |  | Not checked |  |  |  |  |  |
| bwp-SwitchingMultiCCs-r16 |  | Not checked |  |  |  |  |  |
| targetSMTC-SCG-r16 |  | Not checked |  |  |  |  |  |
| supportRepetitionZeroOffsetRV-r16 |  | Not checked |  |  |  |  |  |
| cbg-TransInOrderPUSCH-UL-r16 |  | Not checked |  |  |  |  |  |
| bwp-SwitchingMultiDormancyCCs-r16 |  | Not checked |  |  |  |  |  |
| supportRetx-Diff-CoresetPool-Multi-DCI-TRP-r16 |  | Not checked |  |  |  |  |  |
| pdcch-MonitoringAnyOccasionsWithSpanGapCrossCarrierSch-r16 |  | Not checked |  |  |  |  |  |
| newBeamIdentifications2PortCSI-RS-r16 |  | Not checked |  |  |  |  |  |
| pathlossEstimation2PortCSI-RS-r16 |  | Not checked |  |  |  |  |  |
| mux-HARQ-ACK-withoutPUCCH-onPUSCH-r16 |  | Not checked |  |  |  |  |  |
| guardSymbolReportReception-IAB-r17 |  | Not checked |  |  |  |  |  |
| restricted-IAB-DU-BeamReception-r17 |  | Not checked |  |  |  |  |  |
| recommended-IAB-MT-BeamTransmission-r17 |  | Not checked |  |  |  |  |  |
| case6-TimingAlignmentReception-IAB-r17 |  | Not checked |  |  |  |  |  |
| case7-TimingAlignmentReception-IAB-r17 |  | Not checked |  |  |  |  |  |
| dl-tx-PowerAdjustment-IAB-r17 |  | Not checked |  |  |  |  |  |
| desired-ul-tx-PowerAdjustment-r17 |  | Not checked |  |  |  |  |  |
| fdm-SoftResourceAvailability-DynamicIndication-r17 |  | Not checked |  |  |  |  |  |
| updated-T-DeltaRangeRecption-r17 |  | Not checked |  |  |  |  |  |
| slotBasedDynamicPUCCH-Rep-r17 |  | Not checked |  |  |  |  |  |
| sps-HARQ-ACK-Deferral-r17 |  | Not checked |  |  |  |  |  |
| unifiedJointTCI-commonUpdate-r17 |  | Not checked |  |  |  |  |  |
| mTRP-PDCCH-singleSpan-r17 |  | Not checked |  |  |  |  |  |
| supportedActivatedPRS-ProcessingWindow-r17 |  | Not checked |  |  |  |  |  |
| cg-TimeDomainAllocationExtension-r17 |  | Not checked |  |  |  |  |  |
| ta-BasedPDC-TN-NonSharedSpectrumChAccess-r17 |  | Not checked |  |  |  |  |  |
| directionalCollisionDC-IAB-r17 |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| phy-ParametersXDD-Diff SEQUENCE { |  |  |  |  |  |  |  |
| dynamicSFI |  | Not checked |  |  |  |  |  |
| twoPUCCH-F0-2-ConsecSymbols |  | Not checked |  |  |  |  |  |
| twoDifferentTPC-Loop-PUSCH |  | Not checked |  |  |  |  |  |
| twoDifferentTPC-Loop-PUCCH |  | Not checked |  |  |  |  |  |
| dl-SchedulingOffset-PDSCH-TypeA |  | Checked (NOTE 17) |  |  |  | pc_dl_SchedulingOffset_PDSCH_TypeA |  |
| dl-SchedulingOffset-PDSCH-TypeB |  | Not checked |  |  |  |  |  |
| ul-SchedulingOffset |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| phy-ParametersFRX-Diff SEQUENCE { |  |  |  |  |  |  |  |
| dynamicSFI |  | Not checked |  |  |  |  |  |
| dummy1 |  | Not checked |  |  |  |  |  |
| twoFL-DMRS |  | Not checked |  |  |  |  |  |
| dummy2 |  | Not checked |  |  |  |  |  |
| dummy3 |  | Not checked |  |  |  |  |  |
| supportedDMRS-TypeDL |  | Not checked |  |  |  |  |  |
| supportedDMRS-TypeUL |  | Not checked |  |  |  |  |  |
| semiOpenLoopCSI |  | Not checked |  |  |  |  |  |
| csi-ReportWithoutPMI |  | Not checked |  |  |  |  |  |
| csi-ReportWithoutCQI |  | Not checked |  |  |  |  |  |
| onePortsPTRS |  | Not checked |  |  |  |  |  |
| twoPUCCH-F0-2-ConsecSymbols |  | Not checked |  |  |  |  |  |
| pucch-F2-WithFH |  | supported |  |  |  |  |  |
| pucch-F3-WithFH |  | supported |  |  |  |  |  |
| pucch-F4-WithFH |  | Not checked |  |  |  |  |  |
| freqHoppingPUCCH-F0-2 |  | Not checked |  |  |  |  |  |
| freqHoppingPUCCH-F1-3-4 |  | Not checked |  |  |  |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot |  | Not checked |  |  |  |  |  |
| uci-CodeBlockSegmentation |  | Not checked |  |  |  |  |  |
| onePUCCH-LongAndShortFormat |  | Not checked |  |  |  |  |  |
| twoPUCCH-AnyOthersInSlot |  | Not checked |  |  |  |  |  |
| intraSlotFreqHopping-PUSCH |  | Not checked |  |  |  |  |  |
| pusch-LBRM |  | Not checked |  |  |  |  |  |
| pdcch-BlindDetectionCA |  | Not checked |  |  |  |  |  |
| tpc-PUSCH-RNTI |  | Not checked |  |  |  |  |  |
| tpc-PUCCH-RNTI |  | Not checked |  |  |  |  |  |
| tpc-SRS-RNTI |  | Not checked |  |  |  |  |  |
| absoluteTPC-Command |  | Not checked |  |  |  |  |  |
| twoDifferentTPC-Loop-PUSCH |  | Not checked |  |  |  |  |  |
| twoDifferentTPC-Loop-PUCCH |  | Not checked |  |  |  |  |  |
| pusch-HalfPi-BPSK |  | Not checked |  |  |  |  |  |
| pucch-F3-4-HalfPi-BPSK |  | Not checked |  |  |  |  |  |
| almostContiguousCP-OFDM-UL |  | Not checked |  |  |  |  |  |
| sp-CSI-RS |  | Not checked |  |  |  |  |  |
| sp-CSI-IM |  | Not checked |  |  |  |  |  |
| tdd-MultiDL-UL-SwitchPerSlot |  | Not checked |  |  |  |  |  |
| multipleCORESET |  | Not checked |  |  |  |  |  |
| csi-RS-IM-ReceptionForFeedback |  | Not checked |  | CSI-RS-IM-ReceptionForFeedback |  |  |  |
| csi-RS-ProcFrameworkForSRS |  | Not checked |  | CSI-RS-ProcFrameworkForSRS |  |  |  |
| csi-ReportFramework |  | Not checked |  | CSI-ReportFramework |  |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot SEQUENCE { |  |  |  |  |  |  |  |
| sameSymbol |  | Not checked |  |  |  |  |  |
| diffSymbol |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| mux-SR-HARQ-ACK-PUCCH |  | Not checked |  |  |  |  |  |
| mux-MultipleGroupCtrlCH-Overlap |  | Not checked |  |  |  |  |  |
| dl-SchedulingOffset-PDSCH-TypeA |  | Checked (NOTE 18) |  |  |  | pc_dl_SchedulingOffset_PDSCH_TypeA |  |
| dl-SchedulingOffset-PDSCH-TypeB |  | Not checked |  |  |  |  |  |
| ul-SchedulingOffset |  | Not checked |  |  |  |  |  |
| dl-64QAM-MCS-TableAlt |  | Not checked |  |  |  |  |  |
| ul-64QAM-MCS-TableAlt |  | Not checked |  |  |  |  |  |
| cqi-TableAlt |  | Not checked |  |  |  |  |  |
| oneFL-DMRS-TwoAdditionalDMRS-UL |  | Not checked |  |  |  |  |  |
| twoFL-DMRS-TwoAdditionalDMRS-UL |  | Not checked |  |  |  |  |  |
| oneFL-DMRS-ThreeAdditionalDMRS-UL |  | Not checked |  |  |  |  |  |
| pdcch-BlindDetectionNRDC |  | Not checked |  |  |  |  |  |
| mux-HARQ-ACK-PUSCH-DiffSymbol |  | Not checked |  |  |  |  |  |
| type1-HARQ-ACK-Codebook-r16 |  | Not checked |  |  |  |  |  |
| enhancedPowerControl-r16 |  | Not checked |  |  |  |  |  |
| simultaneousTCI-ActMultipleCC-r16 |  | Not checked |  |  |  |  |  |
| simultaneousSpatialRelationMultipleCC-r16 |  | Not checked |  |  |  |  |  |
| cli-RSSI-FDM-DL-r16 |  | Not checked |  |  |  |  |  |
| cli-SRS-RSRP-FDM-DL-r16 |  | Not checked |  |  |  |  |  |
| maxLayersMIMO-Adaptation-r16 |  | Not checked |  |  |  |  |  |
| aggregationFactorSPS-DL-r16 |  | Not checked |  |  |  |  |  |
| maxTotalResourcesForOneFreqRange-r16 |  | Not checked |  |  |  |  |  |
| csi-ReportFrameworkExt-r16 |  | Not checked |  |  |  |  |  |
| twoTCI-Act-servingCellInCC-List-r16 |  | Not checked |  |  |  |  |  |
| cri-RI-CQI-WithoutNon-PMI-PortInd-r16 |  | Not checked |  |  |  |  |  |
| cqi-4-BitsSubbandTN-NonSharedSpectrumChAccess-r17 |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| phy-ParametersFR1 SEQUENCE { |  |  |  |  |  |  |  |
| pdcchMonitoringSingleOccasion |  | Not checked |  |  |  |  |  |
| scs-60kHz |  | Not checked |  |  |  |  |  |
| pdsch-256QAM-FR1 |  | Checked |  |  |  | pc_pdsch_256QAM_FR1 |  |
| pdsch-RE-MappingFR1-PerSymbol |  | Not checked |  |  |  |  |  |
| pdsch-RE-MappingFR1-PerSlot |  | Not checked |  |  |  |  |  |
| pdcch-MonitoringSingleSpanFirst4Sym-r16 |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| phy-ParametersFR2 SEQUENCE { |  |  |  |  |  |  |  |
| dummy |  | Not checked |  |  |  |  |  |
| pdsch-RE-MappingFR2-PerSymbol |  | Not checked |  |  |  |  |  |
| pCell-FR2 |  | Not checked |  |  |  |  |  |
| pdsch-RE-MappingFR2-PerSlot |  | Not checked |  |  |  |  |  |
| defaultSpatialRelationPathlossRS-r16 |  | Not checked |  |  |  |  |  |
| spatialRelationUpdateAP-SRS-r16 |  | Not checked |  |  |  |  |  |
| maxNumberSRS-PosSpatialRelationsAllServingCells-r16 |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| rf-Parameters SEQUENCE { |  |  |  |  |  |  |  |
| supportedBandListNR SEQUENCE (SIZE (1..maxBands)) OF BandNR { |  |  |  |  |  |  |  |
| BandNR[i] SEQUENCE { |  |  |  |  |  |  |  |
| bandNR |  | Checked |  | Checked for 'maxBands' entries of FreqBandIndicatorNR[i] |  | pc_nrBandx ('x' being the band number/type related PICS listed in TS 38.508-2) |  |
| modifiedMPR-Behaviour |  | Not checked |  |  |  |  |  |
| mimo-ParametersPerBand |  | Not checked |  | MIMO-ParametersPerBand[i] |  |  |  |
| extendedCP |  | Not checked |  |  |  |  |  |
| multipleTCI |  | supported |  |  |  |  |  |
| bwp-WithoutRestriction |  | Checked |  |  |  | pc_bwp_WithoutRestriction |  |
| bwp-SameNumerology |  | Not checked |  |  |  |  |  |
| bwp-DiffNumerology |  | Not checked |  |  |  |  |  |
| crossCarrierScheduling-SameSCS |  | Not checked |  |  |  |  |  |
| pdsch-256QAM-FR2 |  | Checked |  | Note 8 |  | pc_pdsch_256QAM_FR2 |  |
| pusch-256QAM |  | Checked |  | Note 9 |  | pc_pusch_256QAM_FR1OR pc_pusch_256QAM_FR2 |  |
| ue-PowerClass |  | Not checked |  |  |  |  |  |
| rateMatchingLTE-CRS |  | Not checked |  |  |  |  |  |
| channelBWs-DL-v1530 CHOICE { |  |  |  |  |  |  |  |
| fr1 SEQUENCE { |  |  |  |  |  |  |  |
| scs-15kHz |  | Not checked |  |  |  |  |  |
| scs-30kHz |  | Not checked |  |  |  |  |  |
| scs-60kHz |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| fr2 SEQUENCE { |  |  |  |  |  |  |  |
| scs-60kHz |  | Not checked |  |  |  |  |  |
| scs-120kHz |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| channelBWs-UL-v1530[i] CHOICE { |  |  |  |  |  |  |  |
| fr1 SEQUENCE { |  |  |  |  |  |  |  |
| scs-15kHz |  | Not checked |  |  |  |  |  |
| scs-30kHz |  | Not checked |  |  |  |  |  |
| scs-60kHz |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| fr2 SEQUENCE { |  |  |  |  |  |  |  |
| scs-60kHz |  | Not checked |  |  |  |  |  |
| scs-120kHz |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| maxUplinkDutyCycle-PC2-FR1 |  | Not checked |  |  |  |  |  |
| pucch-SpatialRelInfoMAC-CE |  | Not checked |  |  |  |  |  |
| powerBoosting-pi2BPSK |  | Not checked |  |  |  |  |  |
| maxUplinkDutyCycle-FR2 |  | Not checked |  |  |  |  |  |
| channelBWs-DL-v1590 |  | Not checked |  |  |  |  |  |
| channelBWs-UL-v1590 |  | Not checked |  |  |  |  |  |
| asymmetricBandwidthCombinationSet |  | Not checked |  |  |  |  |  |
| sharedSpectrumChAccessParamsPerBand-r16 |  | Not checked |  |  |  |  |  |
| cancelOverlappingPUSCH-r16 |  | Not checked |  |  |  |  |  |
| multipleRateMatchingEUTRA-CRS-r16 |  | Not checked |  |  |  |  |  |
| overlapRateMatchingEUTRA-CRS-r16 |  | Not checked |  |  |  |  |  |
| pdsch-MappingTypeB-Alt-r16 |  | Not checked |  |  |  |  |  |
| oneSlotPeriodicTRS-r16 |  | Not checked |  |  |  |  |  |
| olpc-SRS-Pos-r16 |  | Not checked |  |  |  |  |  |
| spatialRelationsSRS-Pos-r16 |  | Not checked |  |  |  |  |  |
| simulSRS-MIMO-TransWithinBand-r16 |  | Not checked |  |  |  |  |  |
| channelBW-DL-IAB-r16 |  | Not checked |  |  |  |  |  |
| channelBW-UL-IAB-r16 |  | Not checked |  |  |  |  |  |
| rasterShift7dot5-IAB-r16 |  | Not checked |  |  |  |  |  |
| ue-PowerClass-v1610 |  | Not checked |  |  |  |  |  |
| condHandover-r16 |  | Checked |  |  |  | pc_condHandover_r16 |  |
| condHandoverFailure-r16 |  | Checked |  |  |  | pc_condHandoverFailure_r16 |  |
| condHandoverTwoTriggerEvents-r16 |  | Checked |  |  |  | pc_condHandoverTwoTriggerEvents_r16 |  |
| condPSCellChange-r16 |  | Checked |  |  |  | pc_condPSCellChange_r16 |  |
| condPSCellChangeTwoTriggerEvents-r16 |  | Not checked |  |  |  |  |  |
| mpr-PowerBoost-FR2-r16 |  | Not checked |  |  |  |  |  |
| activeConfiguredGrant-r16 |  | Not checked |  |  |  |  |  |
| jointReleaseConfiguredGrantType2-r16 |  | Not checked |  |  |  |  |  |
| sps-r16 |  | Not checked |  |  |  |  |  |
| jointReleaseSPS-r16 |  | Not checked |  |  |  |  |  |
| simulSRS-TransWithinBand-r16 |  | Not checked |  |  |  |  |  |
| trs-AdditionalBandwidth-r16 |  | Not checked |  |  |  |  |  |
| handoverIntraF-IAB-r16 |  | Not checked |  |  |  |  |  |
| simulTX-SRS-AntSwitchingIntraBandUL-CA-r16 |  | Not checked |  |  |  |  |  |
| sharedSpectrumChAccessParamsPerBand-v1630 |  | Not checked |  |  |  |  |  |
| handoverUTRA-FDD-r16 |  | Not checked |  |  |  |  |  |
| enhancedUL-TransientPeriod-r16 |  | Not checked |  |  |  |  |  |
| sharedSpectrumChAccessParamsPerBand-v1640 |  | Not checked |  |  |  |  |  |
| type1-PUSCH-RepetitionMultiSlots-v1650 |  | Not checked |  |  |  |  |  |
| type2-PUSCH-RepetitionMultiSlots-v1650 |  | Not checked |  |  |  |  |  |
| pusch-RepetitionMultiSlots-v1650 |  | Not checked |  |  |  |  |  |
| configuredUL-GrantType1-v1650 |  | Not checked |  |  |  |  |  |
| configuredUL-GrantType2-v1650 |  | Not checked |  |  |  |  |  |
| sharedSpectrumChAccessParamsPerBand-v1650 |  | Not checked |  |  |  |  |  |
| enhancedSkipUplinkTxConfigured-v1660 |  | Not checked |  |  |  |  |  |
| enhancedSkipUplinkTxDynamic-v1660 |  | Not checked |  |  |  |  |  |
| maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 |  | Checked |  |  |  | pc_nrBandx_maxUplinkDutyCycle_PC1dot5_MPE_FR1_r16 ('x' being the band number related PICS listed in Table A.4.3.1-4h of TS 38.508-2) |  |
| txDiversity-r16 |  | Not checked |  |  |  |  |  |
| pdsch-1024QAM-FR1-r17 |  | Not checked |  |  |  |  |  |
| ue-PowerClass-v1700 |  | Not checked |  |  |  |  |  |
| fr2-2-AccessParamsPerBand-r17 |  | Not checked |  |  |  |  |  |
| rlm-Relaxation-r17 |  | Not checked |  |  |  |  |  |
| bfd-Relaxation-r17 |  | Not checked |  |  |  |  |  |
| cg-SDT-r17 |  | Not checked |  |  |  |  |  |
| locationBasedCondHandover-r17 |  | Not checked |  |  |  |  |  |
| timeBasedCondHandover-r17 |  | Not checked |  |  |  |  |  |
| eventA4BasedCondHandover-r17 |  | Not checked |  |  |  |  |  |
| mn-InitiatedCondPSCellChangeNRDC-r17 |  | Not checked |  |  |  |  |  |
| sn-InitiatedCondPSCellChangeNRDC-r17 |  | Not checked |  |  |  |  |  |
| pdcch-SkippingWithoutSSSG-r17 |  | Not checked |  |  |  |  |  |
| sssg-Switching-1BitInd-r17 |  | Not checked |  |  |  |  |  |
| sssg-Switching-2BitInd-r17 |  | Not checked |  |  |  |  |  |
| pdcch-SkippingWithSSSG-r17 |  | Not checked |  |  |  |  |  |
| searchSpaceSetGrp-switchCap2-r17 |  | Not checked |  |  |  |  |  |
| uplinkPreCompensation-r17 |  | Not checked |  |  |  |  |  |
| uplink-TA-Reporting-r17 |  | Not checked |  |  |  |  |  |
| max-HARQ-ProcessNumber-r17 |  | Not checked |  |  |  |  |  |
| type2-HARQ-Codebook-r17 |  | Not checked |  |  |  |  |  |
| type1-HARQ-Codebook-r17 |  | Not checked |  |  |  |  |  |
| type3-HARQ-Codebook-r17 |  | Not checked |  |  |  |  |  |
| ue-specific-K-Offset-r17 |  | Not checked |  |  |  |  |  |
| multiPDSCH-SingleDCI-FR2-1-SCS-120kHz-r17 |  | Not checked |  |  |  |  |  |
| multiPUSCH-SingleDCI-FR2-1-SCS-120kHz-r17 |  | Not checked |  |  |  |  |  |
| parallelPRS-MeasRRC-Inactive-r17 |  | Not checked |  |  |  |  |  |
| nr-UE-TxTEG-ID-MaxSupport-r17 |  | Not checked |  |  |  |  |  |
| prs-ProcessingRRC-Inactive-r17 |  | Not checked |  |  |  |  |  |
| prs-ProcessingWindowType1A-r17 |  | Not checked |  |  |  |  |  |
| prs-ProcessingWindowType1B-r17 |  | Not checked |  |  |  |  |  |
| prs-ProcessingWindowType2-r17 |  | Not checked |  |  |  |  |  |
| srs-AllPosResourcesRRC-Inactive-r17 |  | Not checked |  |  |  |  |  |
| olpc-SRS-PosRRC-Inactive-r17 |  | Not checked |  |  |  |  |  |
| spatialRelationsSRS-PosRRC-Inactive-r17 |  | Not checked |  |  |  |  |  |
| maxNumberPUSCH-TypeA-Repetition-r17 |  | Not checked |  |  |  |  |  |
| puschTypeA-RepetitionsAvailSlot-r17 |  | Not checked |  |  |  |  |  |
| tb-ProcessingMultiSlotPUSCH-r17 |  | Not checked |  |  |  |  |  |
| tb-ProcessingRepMultiSlotPUSCH-r17 |  | Not checked |  |  |  |  |  |
| maxDurationDMRS-Bundling-r17 |  | Not checked |  |  |  |  |  |
| pusch-RepetitionCRC-r17 |  | Not checked |  |  |  |  |  |
| sharedSpectrumChAccessParamsPerBand-v1710 |  | Not checked |  |  |  |  |  |
| parallelMeasurementWithoutRestriction-r17 |  | Not checked |  |  |  |  |  |
| maxNumber-NGSO-SatellitesWithinOneSMTC-r17 |  | Not checked |  |  |  |  |  |
| k1-RangeExtension-r17 |  | Not checked |  |  |  |  |  |
| aperiodicCSI-RS-FastScellActivation-r17 |  | Not checked |  |  |  |  |  |
| aperiodicCSI-RS-AdditionalBandwidth-r17 |  | Not checked |  |  |  |  |  |
| bwp-WithoutCD-SSB-OrNCD-SSB-RedCap-r17 |  | Not checked |  |  |  |  |  |
| halfDuplexFDD-TypeA-RedCap-r17 |  | Not checked |  |  |  |  |  |
| posSRS-RRC-Inactive-OutsideInitialUL-BWP-r17 PosSRS-RRC-Inactive-OutsideInitialUL-BWP-r17 |  | Not checked |  |  |  |  |  |
| channelBWs-DL-SCS-480kHz-FR2-2-r17 |  | Not checked |  |  |  |  |  |
| channelBWs-UL-SCS-480kHz-FR2-2-r17 |  | Not checked |  |  |  |  |  |
| channelBWs-DL-SCS-960kHz-FR2-2-r17 |  | Not checked |  |  |  |  |  |
| channelBWs-UL-SCS-960kHz-FR2-2-r17 |  | Not checked |  |  |  |  |  |
| ul-GapFR2-r17 |  | Not checked |  |  |  |  |  |
| oneShotHARQ-feedbackTriggeredByDCI-1-2-r17 |  | Not checked |  |  |  |  |  |
| oneShotHARQ-feedbackPhy-Priority-r17 |  | Not checked |  |  |  |  |  |
| enhancedType3-HARQ-CodebookFeedback-r17 |  | Not checked |  |  |  |  |  |
| triggeredHARQ-CodebookRetx-r17 |  | Not checked |  |  |  |  |  |
| ue-OneShotUL-TimingAdj-r17 |  | Not checked |  |  |  |  |  |
| pucch-Repetition-F0-2-r17 |  | Not checked |  |  |  |  |  |
| cqi-4-BitsSubbandNTN-SharedSpectrumChAccess-r17 |  | Not checked |  |  |  |  |  |
| mux-HARQ-ACK-DiffPriorities-r17 |  | Not checked |  |  |  |  |  |
| ta-BasedPDC-NTN-SharedSpectrumChAccess-r17 |  | Not checked |  |  |  |  |  |
| ack-NACK-FeedbackForMulticastWithDCI-Enabler-r17 |  | Not checked |  |  |  |  |  |
| maxNumberG-RNTI-r17 |  | Not checked |  |  |  |  |  |
| dynamicMulticastDCI-Format4-2-r17 |  | Not checked |  |  |  |  |  |
| maxModulationOrderForMulticast-r17 |  | Not checked |  |  |  |  |  |
| dynamicSlotRepetitionMulticastTN-NonSharedSpectrumChAccess-r17 |  | Not checked |  |  |  |  |  |
| dynamicSlotRepetitionMulticastNTN-SharedSpectrumChAccess-r17 |  | Not checked |  |  |  |  |  |
| nack-OnlyFeedbackForMulticastWithDCI-Enabler-r17 |  | Not checked |  |  |  |  |  |
| ack-NACK-FeedbackForSPS-MulticastWithDCI-Enabler-r17 |  | Not checked |  |  |  |  |  |
| maxNumberG-CS-RNTI-r17 |  | Not checked |  |  |  |  |  |
| re-LevelRateMatchingForMulticast-r17 |  | Not checked |  |  |  |  |  |
| pdsch-1024QAM-2MIMO-FR1-r17 |  | Not checked |  |  |  |  |  |
| prs-MeasurementWithoutMG-r17 |  | Not checked |  |  |  |  |  |
| maxNumber-LEO-SatellitesPerCarrier-r17 |  | Not checked |  |  |  |  |  |
| prs-ProcessingCapabilityOutsideMGinPPW-r17 |  | Not checked |  |  |  |  |  |
| srs-SemiPersistent-PosResourcesRRC-Inactive-r17 |  | Not checked |  |  |  |  |  |
| channelBWs-DL-SCS-120kHz-FR2-2-r17 |  | Not checked |  |  |  |  |  |
| channelBWs-UL-SCS-120kHz-FR2-2-r17 |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| supportedBandCombinationList SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination { |  |  |  |  |  |  |  |
| BandCombinationSEQUENCE { |  |  |  |  |  |  |  |
| bandList SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters { |  |  |  |  |  |  |  |
| BandParameters CHOICE { |  |  |  |  |  |  |  |
| eutra SEQUENCE { |  |  |  |  |  |  |  |
| bandEUTRA |  | Not checked |  | FreqBandIndicatorEUTRA |  |  |  |
| ca-BandwidthClassDL-EUTRA |  | Not checked |  | CA-BandwidthClassEUTRA |  |  |  |
| ca-BandwidthClassUL-EUTRA |  | Not checked |  | CA-BandwidthClassEUTRA |  |  |  |
| } |  |  |  |  |  |  |  |
| nr SEQUENCE { |  |  |  |  |  |  |  |
| bandNR |  | Not checked |  | FreqBandIndicatorNR |  |  |  |
| ca-BandwidthClassDL-NR |  | Not checked |  | CA-BandwidthClassNR |  |  |  |
| ca-BandwidthClassUL-NR |  | Not checked |  | CA-BandwidthClassNR |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| featureSetCombination |  | Not checked |  | FeatureSetCombinationId |  |  |  |
| ca-ParametersEUTRA |  | Not checked |  | CA-ParametersEUTRA |  |  |  |
| ca-ParametersNR |  | Not checked |  | CA-ParametersNR |  |  |  |
| mrdc-Parameters |  | Not checked |  | MRDC-Parameters |  |  |  |
| supportedBandwidthCombinationSet |  | Not checked |  |  |  |  |  |
| powerClass-v1530 |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| appliedFreqBandListFilter |  | Not checked |  | FreqBandList |  |  |  |
| supportedBandCombinationList-v1540 SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1540 { |  |  |  | BandCombinationList-v1540 |  |  |  |
| BandCombination-v1540 SEQUENCE { |  |  |  |  |  |  |  |
| bandList-v1540 SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1540 { |  | Not checked |  | BandParameters-v1540 |  |  |  |
| BandParameters-v1540 SEQUENCE { |  |  |  |  |  |  |  |
| srs-CarrierSwitch CHOICE { |  |  |  |  |  |  |  |
| nr SEQUENCE { |  |  |  | NR bands |  |  |  |
| srs-SwitchingTimesListNR SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingTimeNR |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| eutra SEQUENCE { |  |  |  | EUTRA bands |  |  |  |
| srs-SwitchingTimesListEUTRA SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingTimeEUTRA |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| srs-TxSwitch-v1540 SEQUENCE { |  |  |  |  |  |  |  |
| supportedSRS-TxPortSwitch |  | Not checked |  |  |  |  |  |
| txSwitchImpactToRx |  | Not checked |  |  |  |  |  |
| txSwitchWithAnotherBand |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| ca-ParametersNR-v1540[i] |  | Not checked |  | CA-ParametersNR-v1540 |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| srs-SwitchingTimeRequested |  | Not checked |  |  |  |  |  |
| supportedBandCombinationList-v1550 |  | Not checked |  | BandCombinationList-v1550 |  |  |  |
| supportedBandCombinationList-v1560 |  | Not checked |  | BandCombinationList-v1560 |  |  |  |
| supportedBandCombinationList-v1570 |  | Not checked |  | BandCombinationList-v1570 |  |  |  |
| supportedBandCombinationList-v1580 |  | Not checked |  | BandCombinationList-v1580 |  |  |  |
| supportedBandCombinationList-v1590 |  | Not checked |  | BandCombinationList-v1590 |  |  |  |
| supportedBandCombinationList-v15a0 |  | Not checked |  | BandCombinationList-v15a0 |  |  |  |
| supportedBandCombinationList-v1610 SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1610 { |  |  |  | BandCombinationList-v1610 |  |  |  |
| BandCombination-v1610 SEQUENCE { |  |  |  |  |  |  |  |
| bandList-v1610 SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1610 { |  |  |  |  |  |  |  |
| ca-ParametersNR-v1610 |  | Checked |  | CA-ParametersNR-v1610 (Table 8.2.1.1.2.3.3-9) |  |  |  |
| ca-ParametersNRDC-v1610 |  | Not checked |  | CA-ParametersNRDC-v1610 |  |  |  |
| powerClass-v1610 |  | Not checked |  |  |  |  |  |
| powerClassNRPart-r16 |  | Not checked |  |  |  |  |  |
| featureSetCombinationDAPS-r16 |  | Not checked |  | FeatureSetCombinationId |  |  |  |
| mrdc-Parameters-v1620 |  | Not checked |  | MRDC-Parameters-v1620 |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| supportedBandCombinationListSidelinkEUTRA-NR-r16 |  | Not checked |  | BandCombinationListSidelinkEUTRA-NR-r16 |  |  |  |
| supportedBandCombinationList-UplinkTxSwitch-r16 |  | Not checked |  | BandCombinationList-UplinkTxSwitch-r16 |  |  |  |
| supportedBandCombinationList-v1630 |  | Not checked |  | BandCombinationList-v1630 |  |  |  |
| supportedBandCombinationListSidelinkEUTRA-NR-v1630 |  | Not checked |  | BandCombinationListSidelinkEUTRA-NR-v1630 |  |  |  |
| supportedBandCombinationList-UplinkTxSwitch-v1630 |  | Not checked |  | BandCombinationList-UplinkTxSwitch-v1630 |  |  |  |
| supportedBandCombinationList-v1640 |  | Not checked |  | BandCombinationList-v1640 |  |  |  |
| supportedBandCombinationList-UplinkTxSwitch-v1640 |  | Not checked |  | BandCombinationList-UplinkTxSwitch-v1640 |  |  |  |
| supportedBandCombinationList-v1650 |  | Not checked |  | BandCombinationList-v1650 |  |  |  |
| supportedBandCombinationList-UplinkTxSwitch-v1650 |  | Not checked |  | BandCombinationList-UplinkTxSwitch-v1650 |  |  |  |
| extendedBand-n77-r16 |  | Checked |  |  |  | pc_extendedBand_n77_r16 |  |
| supportedBandCombinationList-UplinkTxSwitch-v1670 |  | Not checked |  | BandCombinationList-UplinkTxSwitch-v1670 |  |  |  |
| supportedBandCombinationList-v1680 |  | Not checked |  | BandCombinationList-v1680 |  |  |  |
| supportedBandCombinationList-v1690 |  | Not checked |  | BandCombinationList-v1690 |  |  |  |
| supportedBandCombinationList-UplinkTxSwitch-v1690 |  | Not checked |  | BandCombinationList-UplinkTxSwitch-v1690 |  |  |  |
| supportedBandCombinationList-v1700 |  | Not checked |  | BandCombinationList-v1700 |  |  |  |
| supportedBandCombinationList-UplinkTxSwitch-v1700 |  | Not checked |  | BandCombinationList-UplinkTxSwitch-v1700 |  |  |  |
| supportedBandCombinationListSL-RelayDiscovery-r17 |  | Not checked |  |  |  |  |  |
| supportedBandCombinationListSL-NonRelayDiscovery-r17 |  | Not checked |  |  |  |  |  |
| supportedBandCombinationListSidelinkEUTRA-NR-v1710 |  | Not checked |  | BandCombinationListSidelinkEUTRA-NR-v1710 |  |  |  |
| sidelinkRequested-r17 |  | Not checked |  |  |  |  |  |
| extendedBand-n77-2-r17 |  | Checked |  |  |  |  | pc_extendedBand_n77_2_r17 |  |
| supportedBandCombinationList-v1720 |  | Not checked |  | BandCombinationList-v1720 |  |  |  |
| supportedBandCombinationList-UplinkTxSwitch-v1720 |  | Not checked |  | BandCombinationList-UplinkTxSwitch-v1720 |  |  |  |
| } |  |  |  |  |  |  |  |
| measAndMobParameters SEQUENCE { |  |  |  |  |  |  |  |
| measAndMobParametersCommon SEQUENCE { |  |  |  |  |  |  |  |
| supportedGapPattern |  | Not checked |  |  |  |  |  |
| ssb-RLM |  | supported |  |  |  |  |  |
| ssb-AndCSI-RS-RLM |  | Not checked |  |  |  |  |  |
| eventB-MeasAndReport |  | Checked |  |  |  | pc_eventB_MeasAndReport |  |
| handoverFDD-TDD |  | Not checked |  |  |  |  |  |
| eutra-CGI-Reporting |  | Checked |  |  |  | pc_eutra_CGI_Reporting |  |
| nr-CGI-Reporting |  | Checked |  |  |  | pc_nr_CGI_Reporting |  |
| independentGapConfig |  | Checked |  |  |  | pc_independentGapConfig |  |
| periodicEUTRA-MeasAndReport |  | Checked |  |  |  | pc_periodicEUTRA_MeasAndReport |  |
| handoverFR1-FR2 |  | Not checked |  |  |  |  |  |
| maxNumberCSI-RS-RRM-RS-SINR |  | Not checked |  |  |  |  |  |
| nr-CGI-Reporting-ENDC |  | Not checked |  |  |  |  |  |
| eutra-CGI-Reporting-NEDC |  | Not checked |  |  |  |  |  |
| eutra-CGI-Reporting-NRDC |  | Not checked |  |  |  |  |  |
| nr-CGI-Reporting-NEDC |  | Not checked |  |  |  |  |  |
| nr-CGI-Reporting-NRDC |  | Not checked |  |  |  |  |  |
| reportAddNeighMeasForPeriodic-r16 |  | Not checked |  |  |  |  |  |
| condHandoverParametersCommon-r16 |  | Not checked |  |  |  |  |  |
| nr-NeedForGap-Reporting-r16 |  | Not checked |  |  |  |  |  |
| supportedGapPattern-NRonly-r16 |  | Not checked |  |  |  |  |  |
| supportedGapPattern-NRonly-NEDC-r16 |  | Not checked |  |  |  |  |  |
| maxNumberCLI-RSSI-r16 |  | Not checked |  |  |  |  |  |
| maxNumberCLI-SRS-RSRP-r16 |  | Not checked |  |  |  |  |  |
| maxNumberPerSlotCLI-SRS-RSRP-r16 |  | Not checked |  |  |  |  |  |
| mfbi-IAB-r16 |  | Not checked |  |  |  |  |  |
| dummy |  | Not checked |  |  |  |  |  |
| nr-CGI-Reporting-NPN-r16 |  | Checked |  |  |  | pc_nr_CGI_Reporting_NPN_r16 |  |
| idleInactiveEUTRA-MeasReport-r16 |  | Not checked |  |  |  |  |  |
| idleInactive-ValidityArea-r16 |  | Not checked |  |  |  |  |  |
| eutra-AutonomousGaps-r16 |  | Not checked |  |  |  |  |  |
| eutra-AutonomousGaps-NEDC-r16 |  | Not checked |  |  |  |  |  |
| eutra-AutonomousGaps-NRDC-r16 |  | Not checked |  |  |  |  |  |
| pcellT312-r16 |  | Not checked |  |  |  |  |  |
| supportedGapPattern-r16 |  | Not checked |  |  |  |  |  |
| concurrentMeasGap-r17 |  | Not checked |  |  |  |  |  |
| nr-NeedForGapNCSG-Reporting-r17 |  | Not checked |  |  |  |  |  |
| eutra-NeedForGapNCSG-Reporting-r17 |  | Not checked |  |  |  |  |  |
| ncsg-MeasGapPerFR-r17 |  | Not checked |  |  |  |  |  |
| ncsg-MeasGapPatterns-r17 |  | Not checked |  |  |  |  |  |
| ncsg-MeasGapNR-Patterns-r17 |  | Not checked |  |  |  |  |  |
| preconfiguredUE-AutonomousMeasGap-r17 |  | Not checked |  |  |  |  |  |
| preconfiguredNW-ControlledMeasGap-r17 |  | Not checked |  |  |  |  |  |
| handoverFR1-FR2-2-r17 |  | Not checked |  |  |  |  |  |
| handoverFR2-1-FR2-2-r17 |  | Not checked |  |  |  |  |  |
| independentGapConfigPRS-r17 |  | Not checked |  |  |  |  |  |
| rrm-RelaxationRRC-ConnectedRedCap-r17 |  | Not checked |  |  |  |  |  |
| parallelMeasurementGap-r17 |  | Not checked |  |  |  |  |  |
| condHandoverWithSCG-NRDC-r17 |  | Not checked |  |  |  |  |  |
| gNB-ID-LengthReporting-r17 |  | Not checked |  |  |  |  |  |
| gNB-ID-LengthReporting-ENDC-r17 |  | Not checked |  |  |  |  |  |
| gNB-ID-LengthReporting-NEDC-r17 |  | Not checked |  |  |  |  |  |
| gNB-ID-LengthReporting-NRDC-r17 |  | Not checked |  |  |  |  |  |
| gNB-ID-LengthReporting-NPN-r17 |  | Not checked |  |  |  |  |  |
| parallelSMTC-r17 |  | Not checked |  |  |  |  |  |
| concurrentMeasGapEUTRA-r17 |  | Not checked |  |  |  |  |  |
| serviceLinkPropDelayDiffReporting-r17 |  | Not checked |  |  |  |  |  |
| ncsg-SymbolLevelScheduleRestrictionInter-r17 |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| measAndMobParametersXDD-Diff SEQUENCE { |  |  |  |  |  |  |  |
| intraAndInterF-MeasAndReport |  | Checked (NOTE 1) |  |  |  | pc_intraAndInterF-MeasAndReport |  |
| eventA-MeasAndReport |  | Checked (NOTE 2) |  |  |  | pc_eventA_MeasAndReport |  |
| handoverInterF |  | Checked (NOTE 15) |  |  |  | pc_ handoverInterF |  |
| handoverLTE-EPC |  | Not checked |  |  |  |  |  |
| handoverLTE-5GC |  | Not checked |  |  |  |  |  |
| sftd-MeasNR-Neigh |  | Checked (NOTE 13) |  |  |  | pc_SFTD_MeasNR_Neigh_FDD and/or pc_SFTD_MeasNR_Neigh_TDD |  |
| sftd-MeasNR-Neigh-DRX |  | Not checked |  |  |  |  |  |
| dummy |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| measAndMobParametersFRX-Diff SEQUENCE { |  |  |  |  |  |  |  |
| ss-SINR-Meas |  | Checked (NOTE 10) |  |  |  | pc_ss_SINR_Meas |  |
| csi-RSRP-AndRSRQ-MeasWithSSB |  | Checked (NOTE 3) |  |  |  | pc_csi-RSRP-AndRSRQ-MeasWithSSB |  |
| csi-RSRP-AndRSRQ-MeasWithoutSSB |  | Checked (NOTE 11) |  |  |  | pc_csi_RSRP_AndRSRQ_MeasWithoutSSB |  |
| csi-SINR-Meas |  | Not checked |  |  |  |  |  |
| csi-RS-RLM |  | Not checked |  |  |  |  |  |
| handoverInterF |  | Checked (NOTE 14) |  |  |  | pc_ handoverInterF |  |
| handoverLTE-EPC |  | Not checked |  |  |  |  |  |
| handoverLTE-5GC |  | Not checked |  |  |  |  |  |
| maxNumberResource-CSI-RS-RLM |  | Not checked |  |  |  |  |  |
| simultaneousRxDataSSB-DiffNumerology |  | Not checked |  |  |  |  |  |
| nr-AutonomousGaps-r16 |  | Not checked |  |  |  |  |  |
| nr-AutonomousGaps-ENDC-r16 |  | Not checked |  |  |  |  |  |
| nr-AutonomousGaps-NEDC-r16 |  | Not checked |  |  |  |  |  |
| nr-AutonomousGaps-NRDC-r16 |  | Not checked |  |  |  |  |  |
| dummy |  | Not checked |  |  |  |  |  |
| cli-RSSI-Meas-r16 |  | Not checked |  |  |  |  |  |
| cli-SRS-RSRP-Meas-r16 |  | Not checked |  |  |  |  |  |
| interFrequencyMeas-NoGap-r16 |  | Not checked |  |  |  |  |  |
| simultaneousRxDataSSB-DiffNumerology-Inter-r16 |  | Not checked |  |  |  |  |  |
| idleInactiveNR-MeasReport-r16 |  | Not checked |  |  |  |  |  |
| idleInactiveNR-MeasBeamReport-r16 |  | Not checked |  |  |  |  |  |
| increasedNumberofCSIRSPerMO-r16 |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| fdd-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |  |  |  |  |
| phy-ParametersXDD-Diff SEQUENCE { |  |  |  |  |  |  |  |
| dynamicSFI |  | Not checked |  |  |  |  |  |
| twoPUCCH-F0-2-ConsecSymbols |  | Not checked |  |  |  |  |  |
| twoDifferentTPC-Loop-PUSCH |  | Not checked |  |  |  |  |  |
| twoDifferentTPC-Loop-PUCCH |  | Not checked |  |  |  |  |  |
| dl-SchedulingOffset-PDSCH-TypeA |  | Checked (NOTE 17) |  |  |  | pc_dl_SchedulingOffset_PDSCH_TypeA |  |
| dl-SchedulingOffset-PDSCH-TypeB |  | Not checked |  |  |  |  |  |
| ul-SchedulingOffset |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| mac-ParametersXDD-Diff SEQUENCE { |  |  |  |  |  |  |  |
| skipUplinkTxDynamic |  | Checked (NOTE 4) |  |  |  | pc_skipUplinkTxDynamic |  |
| logicalChannelSR-DelayTimer |  | Checked (NOTE 5) |  |  |  | pc_logicalChannelSR_DelayTimer |  |
| longDRX-Cycle |  | Checked (NOTE 6) |  |  |  | pc_longDRX_Cycle |  |
| shortDRX-Cycle |  | Checked (NOTE 7) |  |  |  | pc_shortDRX_Cycle |  |
| multipleSR-Configurations |  | Not checked |  |  |  |  |  |
| multipleConfiguredGrants |  | Checked (NOTE 12) |  |  |  | pc_multipleConfiguredGrants_r16 |  |
| } |  |  |  |  |  |  |  |
| measAndMobParametersXDD-Diff SEQUENCE { |  |  |  |  |  |  |  |
| intraAndInterF-MeasAndReport |  | Checked (NOTE 1) |  |  |  | pc_intraAndInterF-MeasAndReport |  |
| eventA-MeasAndReport |  | Checked (NOTE 2) |  |  |  | pc_eventA_MeasAndReport |  |
| handoverInterF |  | Checked (NOTE 15) |  |  |  | pc_ handoverInterF |  |
| handoverLTE-EPC |  | Not checked |  |  |  |  |  |
| handoverLTE-5GC |  | Not checked |  |  |  |  |  |
| sftd-MeasNR-Neigh |  | Checked (NOTE 13) |  |  |  | pc_SFTD_MeasNR_Neigh_FDD and/or pc_SFTD_MeasNR_Neigh_TDD |  |
| sftd-MeasNR-Neigh-DRX |  | Not checked |  |  |  |  |  |
| dummy |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| tdd-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |  |  |  |  |
| phy-ParametersXDD-Diff SEQUENCE { |  |  |  |  |  |  |  |
| dynamicSFI |  | Not checked |  |  |  |  |  |
| twoPUCCH-F0-2-ConsecSymbols |  | Not checked |  |  |  |  |  |
| twoDifferentTPC-Loop-PUSCH |  | Not checked |  |  |  |  |  |
| twoDifferentTPC-Loop-PUCCH |  | Not checked |  |  |  |  |  |
| dl-SchedulingOffset-PDSCH-TypeA |  | Checked (NOTE 17) |  |  |  | pc_dl_SchedulingOffset_PDSCH_TypeA |  |
| dl-SchedulingOffset-PDSCH-TypeB |  | Not checked |  |  |  |  |  |
| ul-SchedulingOffset |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| mac-ParametersXDD-Diff SEQUENCE { |  |  |  |  |  |  |  |
| skipUplinkTxDynamic |  | Checked (NOTE 4) |  |  |  | pc_skipUplinkTxDynamic |  |
| logicalChannelSR-DelayTimer |  | Checked (NOTE 5) |  |  |  | pc_logicalChannelSR_DelayTimer |  |
| longDRX-Cycle |  | Checked (NOTE 6) |  |  |  | pc_longDRX_Cycle |  |
| shortDRX-Cycle |  | Checked (NOTE 7) |  |  |  | pc_shortDRX_Cycle |  |
| multipleSR-Configurations |  | Not checked |  |  |  |  |  |
| multipleConfiguredGrants |  | Checked (NOTE 12) |  |  |  | pc_multipleConfiguredGrants_r16 |  |
| } |  |  |  |  |  |  |  |
| measAndMobParametersXDD-Diff SEQUENCE { |  |  |  |  |  |  |  |
| intraAndInterF-MeasAndReport |  | Checked (NOTE 1) |  |  |  | pc_intraAndInterF-MeasAndReport |  |
| eventA-MeasAndReport |  | Checked (NOTE 2) |  |  |  | pc_eventA_MeasAndReport |  |
| handoverInterF |  | Checked (NOTE 15) |  |  |  | pc_ handoverInterF |  |
| handoverLTE-EPC |  | Not checked |  |  |  |  |  |
| handoverLTE-5GC |  | Not checked |  |  |  |  |  |
| sftd-MeasNR-Neigh |  | Checked (NOTE 13) |  |  |  | pc_SFTD_MeasNR_Neigh_FDD and/or pc_SFTD_MeasNR_Neigh_TDD |  |
| sftd-MeasNR-Neigh-DRX |  | Not checked |  |  |  |  |  |
| dummy |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| fr1-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |  |  |  |  |
| phy-ParametersFRX-Diff SEQUENCE { |  |  |  |  |  |  |  |
| dynamicSFI |  | Not checked |  |  |  |  |  |
| dummy1 |  | Not checked |  |  |  |  |  |
| twoFL-DMRS |  | Not checked |  |  |  |  |  |
| dummy2 |  | Not checked |  |  |  |  |  |
| dummy3 |  | Not checked |  |  |  |  |  |
| supportedDMRS-TypeDL |  | Not checked |  |  |  |  |  |
| supportedDMRS-TypeUL |  | Not checked |  |  |  |  |  |
| semiOpenLoopCSI |  | Not checked |  |  |  |  |  |
| csi-ReportWithoutPMI |  | Not checked |  |  |  |  |  |
| csi-ReportWithoutCQI |  | Not checked |  |  |  |  |  |
| onePortsPTRS |  | Not checked |  |  |  |  |  |
| twoPUCCH-F0-2-ConsecSymbols |  | Not checked |  |  |  |  |  |
| pucch-F2-WithFH |  | Not checked |  |  |  |  |  |
| pucch-F3-WithFH |  | Not checked |  |  |  |  |  |
| pucch-F4-WithFH |  | Not checked |  |  |  |  |  |
| freqHoppingPUCCH-F0-2 |  | Not checked |  |  |  |  |  |
| freqHoppingPUCCH-F1-3-4 |  | Not checked |  |  |  |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot |  | Not checked |  |  |  |  |  |
| uci-CodeBlockSegmentation |  | Not checked |  |  |  |  |  |
| onePUCCH-LongAndShortFormat |  | Not checked |  |  |  |  |  |
| twoPUCCH-AnyOthersInSlot |  | Not checked |  |  |  |  |  |
| intraSlotFreqHopping-PUSCH |  | Not checked |  |  |  |  |  |
| pusch-LBRM |  | Not checked |  |  |  |  |  |
| pdcch-BlindDetectionCA |  | Not checked |  |  |  |  |  |
| tpc-PUSCH-RNTI |  | Not checked |  |  |  |  |  |
| tpc-PUCCH-RNTI |  | Not checked |  |  |  |  |  |
| tpc-SRS-RNTI |  | Not checked |  |  |  |  |  |
| absoluteTPC-Command |  | Not checked |  |  |  |  |  |
| twoDifferentTPC-Loop-PUSCH |  | Not checked |  |  |  |  |  |
| twoDifferentTPC-Loop-PUCCH |  | Not checked |  |  |  |  |  |
| pusch-HalfPi-BPSK |  | Not checked |  |  |  |  |  |
| pucch-F3-4-HalfPi-BPSK |  | Not checked |  |  |  |  |  |
| almostContiguousCP-OFDM-UL |  | Not checked |  |  |  |  |  |
| sp-CSI-RS |  | Not checked |  |  |  |  |  |
| sp-CSI-IM |  | Not checked |  |  |  |  |  |
| tdd-MultiDL-UL-SwitchPerSlot |  | Not checked |  |  |  |  |  |
| multipleCORESET |  | Not checked |  |  |  |  |  |
| csi-RS-IM-ReceptionForFeedback |  | Not checked |  |  |  |  |  |
| csi-RS-ProcFrameworkForSRS |  | Not checked |  |  |  |  |  |
| csi-ReportFramework |  | Not checked |  |  |  |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot SEQUENCE { |  |  |  |  |  |  |  |
| sameSymbol |  | Not checked |  |  |  |  |  |
| diffSymbol |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| mux-SR-HARQ-ACK-PUCCH |  | Not checked |  |  |  |  |  |
| mux-MultipleGroupCtrlCH-Overlap |  | Not checked |  |  |  |  |  |
| dl-SchedulingOffset-PDSCH-TypeA |  | Checked (NOTE 18) |  |  |  | pc_dl_SchedulingOffset_PDSCH_TypeA |  |
| dl-SchedulingOffset-PDSCH-TypeB |  | Not checked |  |  |  |  |  |
| ul-SchedulingOffset |  | Not checked |  |  |  |  |  |
| dl-64QAM-MCS-TableAlt |  | Not checked |  |  |  |  |  |
| ul-64QAM-MCS-TableAlt |  | Not checked |  |  |  |  |  |
| cqi-TableAlt |  | Not checked |  |  |  |  |  |
| oneFL-DMRS-TwoAdditionalDMRS-UL |  | Not checked |  |  |  |  |  |
| twoFL-DMRS-TwoAdditionalDMRS-UL |  | Not checked |  |  |  |  |  |
| oneFL-DMRS-ThreeAdditionalDMRS-UL |  | Not checked |  |  |  |  |  |
| pdcch-BlindDetectionNRDC |  | Not checked |  |  |  |  |  |
| mux-HARQ-ACK-PUSCH-DiffSymbol |  | Not checked |  |  |  |  |  |
| type1-HARQ-ACK-Codebook-r16 |  | Not checked |  |  |  |  |  |
| enhancedPowerControl-r16 |  | Not checked |  |  |  |  |  |
| simultaneousTCI-ActMultipleCC-r16 |  | Not checked |  |  |  |  |  |
| simultaneousSpatialRelationMultipleCC-r16 |  | Not checked |  |  |  |  |  |
| cli-RSSI-FDM-DL-r16 |  | Not checked |  |  |  |  |  |
| cli-SRS-RSRP-FDM-DL-r16 |  | Not checked |  |  |  |  |  |
| maxLayersMIMO-Adaptation-r16 |  | Not checked |  |  |  |  |  |
| aggregationFactorSPS-DL-r16 |  | Not checked |  |  |  |  |  |
| maxTotalResourcesForOneFreqRange-r16 |  | Not checked |  |  |  |  |  |
| csi-ReportFrameworkExt-r16 |  | Not checked |  |  |  |  |  |
| twoTCI-Act-servingCellInCC-List-r16 |  | Not checked |  |  |  |  |  |
| cri-RI-CQI-WithoutNon-PMI-PortInd-r16 |  | Not checked |  |  |  |  |  |
| cqi-4-BitsSubbandTN-NonSharedSpectrumChAccess-r17 |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| measAndMobParametersFRX-Diff SEQUENCE { |  |  |  |  |  |  |  |
| ss-SINR-Meas |  | Checked (NOTE 10) |  |  |  | pc_ss_SINR_Meas |  |
| csi-RSRP-AndRSRQ-MeasWithSSB |  | Checked (NOTE 3) |  |  |  | pc_csi-RSRP-AndRSRQ-MeasWithSSB |  |
| csi-RSRP-AndRSRQ-MeasWithoutSSB |  | Checked (NOTE 11) |  |  |  | pc_csi_RSRP_AndRSRQ_MeasWithoutSSB |  |
| csi-SINR-Meas |  | Not checked |  |  |  |  |  |
| csi-RS-RLM |  | Not checked |  |  |  |  |  |
| handoverInterF |  | Checked (NOTE 14) |  |  |  | pc_ handoverInterF |  |
| handoverLTE-EPC |  | Not checked |  |  |  |  |  |
| handoverLTE-5GC |  | Not checked |  |  |  |  |  |
| maxNumberResource-CSI-RS-RLM |  | Not checked |  |  |  |  |  |
| simultaneousRxDataSSB-DiffNumerology |  | Not checked |  |  |  |  |  |
| nr-AutonomousGaps-r16 |  | Not checked |  |  |  |  |  |
| nr-AutonomousGaps-ENDC-r16 |  | Not checked |  |  |  |  |  |
| nr-AutonomousGaps-NEDC-r16 |  | Not checked |  |  |  |  |  |
| nr-AutonomousGaps-NRDC-r16 |  | Not checked |  |  |  |  |  |
| dummy |  | Not checked |  |  |  |  |  |
| cli-RSSI-Meas-r16 |  | Not checked |  |  |  |  |  |
| cli-SRS-RSRP-Meas-r16 |  | Not checked |  |  |  |  |  |
| interFrequencyMeas-NoGap-r16 |  | Not checked |  |  |  |  |  |
| simultaneousRxDataSSB-DiffNumerology-Inter-r16 |  | Not checked |  |  |  |  |  |
| idleInactiveNR-MeasReport-r16 |  | Not checked |  |  |  |  |  |
| idleInactiveNR-MeasBeamReport-r16 |  | Not checked |  |  |  |  |  |
| increasedNumberofCSIRSPerMO-r16 |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| fr2-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |  |  |  |  |
| phy-ParametersFRX-Diff SEQUENCE { |  | Not checked |  |  |  |  |  |
| dynamicSFI |  | Not checked |  |  |  |  |  |
| dummy1 |  | Not checked |  |  |  |  |  |
| twoFL-DMRS |  | Not checked |  |  |  |  |  |
| dummy2 |  | Not checked |  |  |  |  |  |
| dummy3 |  | Not checked |  |  |  |  |  |
| supportedDMRS-TypeDL |  | Not checked |  |  |  |  |  |
| supportedDMRS-TypeUL |  | Not checked |  |  |  |  |  |
| semiOpenLoopCSI |  | Not checked |  |  |  |  |  |
| csi-ReportWithoutPMI |  | Not checked |  |  |  |  |  |
| csi-ReportWithoutCQI |  | Not checked |  |  |  |  |  |
| onePortsPTRS |  | Not checked |  |  |  |  |  |
| twoPUCCH-F0-2-ConsecSymbols |  | Not checked |  |  |  |  |  |
| pucch-F2-WithFH |  | Not checked |  |  |  |  |  |
| pucch-F3-WithFH |  | Not checked |  |  |  |  |  |
| pucch-F4-WithFH |  | Not checked |  |  |  |  |  |
| freqHoppingPUCCH-F0-2 |  | Not checked |  |  |  |  |  |
| freqHoppingPUCCH-F1-3-4 |  | Not checked |  |  |  |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot |  | Not checked |  |  |  |  |  |
| uci-CodeBlockSegmentation |  | Not checked |  |  |  |  |  |
| onePUCCH-LongAndShortFormat |  | Not checked |  |  |  |  |  |
| twoPUCCH-AnyOthersInSlot |  | Not checked |  |  |  |  |  |
| intraSlotFreqHopping-PUSCH |  | Not checked |  |  |  |  |  |
| pusch-LBRM |  | Not checked |  |  |  |  |  |
| pdcch-BlindDetectionCA |  | Not checked |  |  |  |  |  |
| tpc-PUSCH-RNTI |  | Not checked |  |  |  |  |  |
| tpc-PUCCH-RNTI |  | Not checked |  |  |  |  |  |
| tpc-SRS-RNTI |  | Not checked |  |  |  |  |  |
| absoluteTPC-Command |  | Not checked |  |  |  |  |  |
| twoDifferentTPC-Loop-PUSCH |  | Not checked |  |  |  |  |  |
| twoDifferentTPC-Loop-PUCCH |  | Not checked |  |  |  |  |  |
| pusch-HalfPi-BPSK |  | Not checked |  |  |  |  |  |
| pucch-F3-4-HalfPi-BPSK |  | Not checked |  |  |  |  |  |
| almostContiguousCP-OFDM-UL |  | Not checked |  |  |  |  |  |
| sp-CSI-RS |  | Not checked |  |  |  |  |  |
| sp-CSI-IM |  | Not checked |  |  |  |  |  |
| tdd-MultiDL-UL-SwitchPerSlot |  | Not checked |  |  |  |  |  |
| multipleCORESET |  | Not checked |  |  |  |  |  |
| csi-RS-IM-ReceptionForFeedback |  | Not checked |  |  |  |  |  |
| csi-RS-ProcFrameworkForSRS |  | Not checked |  |  |  |  |  |
| csi-ReportFramework |  | Not checked |  |  |  |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot SEQUENCE { |  |  |  |  |  |  |  |
| sameSymbol |  | Not checked |  |  |  |  |  |
| diffSymbol |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| mux-SR-HARQ-ACK-PUCCH |  | Not checked |  |  |  |  |  |
| mux-MultipleGroupCtrlCH-Overlap |  | Not checked |  |  |  |  |  |
| dl-SchedulingOffset-PDSCH-TypeA |  | Checked (NOTE 18) |  |  |  | pc_dl_SchedulingOffset_PDSCH_TypeA |  |
| dl-SchedulingOffset-PDSCH-TypeB |  | Not checked |  |  |  |  |  |
| ul-SchedulingOffset |  | Not checked |  |  |  |  |  |
| dl-64QAM-MCS-TableAlt |  | Not checked |  |  |  |  |  |
| ul-64QAM-MCS-TableAlt |  | Not checked |  |  |  |  |  |
| cqi-TableAlt |  | Not checked |  |  |  |  |  |
| oneFL-DMRS-TwoAdditionalDMRS-UL |  | Not checked |  |  |  |  |  |
| twoFL-DMRS-TwoAdditionalDMRS-UL |  | Not checked |  |  |  |  |  |
| oneFL-DMRS-ThreeAdditionalDMRS-UL |  | Not checked |  |  |  |  |  |
| pdcch-BlindDetectionNRDC |  | Not checked |  |  |  |  |  |
| mux-HARQ-ACK-PUSCH-DiffSymbol |  | Not checked |  |  |  |  |  |
| type1-HARQ-ACK-Codebook-r16 |  | Not checked |  |  |  |  |  |
| enhancedPowerControl-r16 |  | Not checked |  |  |  |  |  |
| simultaneousTCI-ActMultipleCC-r16 |  | Not checked |  |  |  |  |  |
| simultaneousSpatialRelationMultipleCC-r16 |  | Not checked |  |  |  |  |  |
| cli-RSSI-FDM-DL-r16 |  | Not checked |  |  |  |  |  |
| cli-SRS-RSRP-FDM-DL-r16 |  | Not checked |  |  |  |  |  |
| maxLayersMIMO-Adaptation-r16 |  | Not checked |  |  |  |  |  |
| aggregationFactorSPS-DL-r16 |  | Not checked |  |  |  |  |  |
| maxTotalResourcesForOneFreqRange-r16 |  | Not checked |  |  |  |  |  |
| csi-ReportFrameworkExt-r16 |  | Not checked |  |  |  |  |  |
| twoTCI-Act-servingCellInCC-List-r16 |  | Not checked |  |  |  |  |  |
| cri-RI-CQI-WithoutNon-PMI-PortInd-r16 |  | Not checked |  |  |  |  |  |
| cqi-4-BitsSubbandTN-NonSharedSpectrumChAccess-r17 |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| measAndMobParametersFRX-Diff SEQUENCE { |  |  |  |  |  |  |  |
| ss-SINR-Meas |  | Checked (NOTE 10) |  |  |  | pc_ss_SINR_Meas |  |
| csi-RSRP-AndRSRQ-MeasWithSSB |  | Checked (NOTE 3) |  |  |  | pc_csi-RSRP-AndRSRQ-MeasWithSSB |  |
| csi-RSRP-AndRSRQ-MeasWithoutSSB |  | Checked (NOTE 11) |  |  |  | pc_csi_RSRP_AndRSRQ_MeasWithoutSSB |  |
| csi-SINR-Meas |  | Not checked |  |  |  |  |  |
| csi-RS-RLM |  | Not checked |  |  |  |  |  |
| handoverInterF |  | Checked (NOTE 14) |  |  |  | Checked (NOTE 14) |  |
| handoverLTE-EPC |  | Not checked |  |  |  |  |  |
| handoverLTE-5GC |  | Not checked |  |  |  |  |  |
| maxNumberResource-CSI-RS-RLM |  | Not checked |  |  |  |  |  |
| simultaneousRxDataSSB-DiffNumerology |  | Not checked |  |  |  |  |  |
| nr-AutonomousGaps-r16 |  | Not checked |  |  |  |  |  |
| nr-AutonomousGaps-ENDC-r16 |  | Not checked |  |  |  |  |  |
| nr-AutonomousGaps-NEDC-r16 |  | Not checked |  |  |  |  |  |
| nr-AutonomousGaps-NRDC-r16 |  | Not checked |  |  |  |  |  |
| dummy |  | Not checked |  |  |  |  |  |
| cli-RSSI-Meas-r16 |  | Not checked |  |  |  |  |  |
| cli-SRS-RSRP-Meas-r16 |  | Not checked |  |  |  |  |  |
| interFrequencyMeas-NoGap-r16 |  | Not checked |  |  |  |  |  |
| simultaneousRxDataSSB-DiffNumerology-Inter-r16 |  | Not checked |  |  |  |  |  |
| idleInactiveNR-MeasReport-r16 |  | Not checked |  |  |  |  |  |
| idleInactiveNR-MeasBeamReport-r16 |  | Not checked |  |  |  |  |  |
| increasedNumberofCSIRSPerMO-r16 |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| featureSets SEQUENCE { |  |  |  |  |  |  |  |
| featureSetsDownlink SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink { |  |  |  |  |  |  |  |
| FeatureSetDownlink SEQUENCE { |  | Not checked |  |  |  |  |  |
| featureSetListPerDownlinkCC |  | Not checked |  |  |  |  |  |
| intraBandFreqSeparationDL |  | Not checked |  |  |  |  |  |
| scalingFactor |  | Not checked |  |  |  |  |  |
| crossCarrierScheduling-OtherSCS |  | Not checked |  |  |  |  |  |
| scellWithoutSSB |  | Not checked |  |  |  |  |  |
| csi-RS-MeasSCellWithoutSSB |  | Not checked |  |  |  |  |  |
| dummy1 |  | Not checked |  |  |  |  |  |
| type1-3-CSS |  | Not checked |  |  |  |  |  |
| pdcch-MonitoringAnyOccasions |  | Not checked |  |  |  |  |  |
| dummy2 |  | Not checked |  |  |  |  |  |
| ue-SpecificUL-DL-Assignment |  | Not checked |  |  |  |  |  |
| searchSpaceSharingCA-DL |  | Not checked |  |  |  |  |  |
| timeDurationForQCL SEQUENCE { |  |  |  |  |  |  |  |
| scs-60kHz |  | Not checked |  |  |  |  |  |
| scs-120kHz |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| pdsch-ProcessingType1-DifferentTB-PerSlot SEQUENCE { |  |  |  |  |  |  |  |
| scs-15kHz |  | Not checked |  |  |  |  |  |
| scs-30kHz |  | Not checked |  |  |  |  |  |
| scs-60kHz |  | Not checked |  |  |  |  |  |
| scs-120kHz |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| dummy3 |  | Not checked |  |  |  |  |  |
| dummy4 |  | Not checked |  |  |  |  |  |
| dummy5 |  | Not checked |  |  |  |  |  |
| dummy6 |  | Not checked |  |  |  |  |  |
| dummy7 |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| featureSetsDownlinkPerCC SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC { |  |  |  |  |  |  |  |
| FeatureSetDownlinkPerCCSEQUENCE { |  |  |  |  |  |  |  |
| supportedSubcarreirSpacingDL |  | Not checked |  | SubcarrierSpacing |  |  |  |
| supportedBandwidthDL |  | Not checked |  | SupportedBandwidth |  |  |  |
| channelBW-90mhz |  | Not checked |  |  |  |  |  |
| maxNumberMIMO-LayersPDSCH |  | Not checked |  | MIMO-LayersDL |  |  |  |
| supportedModulationOrderDL |  | Not checked |  | ModulationOrder |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| featureSetsUplink SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink { |  |  |  |  |  |  |  |
| FeatureSetUplink SEQUENCE { |  |  |  |  |  |  |  |
| featureSetListPerUplinkCC |  | Not checked |  |  |  |  |  |
| scalingFactor |  | Not checked |  |  |  |  |  |
| crossCarrierScheduling-OtherSCS |  | Not checked |  |  |  |  |  |
| intraBandFreqSeparationUL |  | Not checked |  |  |  |  |  |
| searchSpaceSharingCA-UL |  | Not checked |  |  |  |  |  |
| dummy1 |  | Not checked |  |  |  |  |  |
| supportedSRS-Resources |  | Not checked |  | SRS-Resources |  |  |  |
| twoPUCCH-Group |  | Checked |  |  |  | pc_twoPUCCH_group |  |
| dynamicSwitchSUL |  | Checked |  |  |  | pc_dynamicSwitch_SUL |  |
| simultaneousTxSUL-NonSUL |  | Not checked |  |  |  |  |  |
| pusch-ProcessingType1-DifferentTB-PerSlot SEQUENCE { |  |  |  |  |  |  |  |
| scs-15kHz |  | Not checked |  |  |  |  |  |
| scs-30kHz |  | Not checked |  |  |  |  |  |
| scs-60kHz |  | Not checked |  |  |  |  |  |
| scs-120kHz |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| dummy2 |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| featureSetsUplinkPerCC SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC { |  |  |  |  |  |  |  |
| FeatureSetUplinkPerCCSEQUENCE { |  |  |  |  |  |  |  |
| supportedSubcarrierSpacingUL |  | Not checked |  | SubcarrierSpacing |  |  |  |
| supportedBandwidthUL |  | Not checked |  | SupportedBandwidth |  |  |  |
| channelBW-90mHz |  | Not checked |  |  |  |  |  |
| mimo-CB-PUSCH SEQUENCE { |  |  |  |  |  |  |  |
| maxNumberMIMO-LayersCB-PUSCH |  | Not checked |  |  |  |  |  |
| maxNumberSRS-ResourcePerSet |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| maxNumberMIMO-LayersNonCB-PUSCH |  | Not checked |  |  |  |  |  |
| supportedModulationOrderUL |  | Not checked |  | ModulationOrder |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| featureSetsDownlink-v1540 SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1540 { |  |  |  |  |  |  |  |
| FeatureSetDownlink-v1540 SEQUENCE { |  |  |  |  |  |  |  |
| oneFL-DMRS-TwoAdditionalDMRS-DL |  | Not checked |  |  |  |  |  |
| additionalDMRS-DL-Alt |  | Not checked |  |  |  |  |  |
| twoFL-DMRS-TwoAdditionalDMRS-DL |  | Not checked |  |  |  |  |  |
| oneFL-DMRS-ThreeAdditionalDMRS-DL |  | Not checked |  |  |  |  |  |
| pdcch-MonitoringAnyOccasionsWithSpanGap SEQUENCE { |  |  |  |  |  |  |  |
| scs-15kHz |  | Not checked |  |  |  |  |  |
| scs-30kHz |  | Not checked |  |  |  |  |  |
| scs-60kHz |  | Not checked |  |  |  |  |  |
| scs-120kHz |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| pdsch-SeparationWithGap |  | Not checked |  |  |  |  |  |
| pdsch-ProcessingType2 SEQUENCE { |  |  |  |  |  |  |  |
| scs-15kHz |  | Not checked |  |  |  |  |  |
| scs-30kHz |  | Not checked |  |  |  |  |  |
| scs-60kHz |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| pdsch-ProcessingType2-Limited SEQUENCE { |  |  |  |  |  |  |  |
| differentTB-PerSlot-SCS-30kHz |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| dl-MCS-TableAlt-DynamicIndication |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| featureSetsUplink-v1540 SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v1540 { |  |  |  |  |  |  |  |
| FeatureSetUplink-v1540 SEQUENCE { |  |  |  |  |  |  |  |
| zeroSlotOffsetAperiodicSRS |  | Not checked |  |  |  |  |  |
| pa-PhaseDiscontinuityImpacts |  | Not checked |  |  |  |  |  |
| pusch-SeparationWithGap |  | Not checked |  |  |  |  |  |
| pusch-ProcessingType2 SEQUENCE { |  |  |  |  |  |  |  |
| scs-15kHz |  | Not checked |  |  |  |  |  |
| scs-30kHz |  | Not checked |  |  |  |  |  |
| scs-60kHz |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| ul-MCS-TableAlt-DynamicIndication |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| featureSetsUplinkPerCC-v1540 SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC-v1540 { |  |  |  |  |  |  |  |
| FeatureSetUplinkPerCC-v1540 SEQUENCE { |  |  |  |  |  |  |  |
| mimo-NonCB-PUSCH SEQUENCE { |  |  |  |  |  |  |  |
| maxNumberSRS-ResourcePerSet |  | Not checked |  |  |  |  |  |
| maxNumberSimultaneousSRS-ResourceTx |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| featureSetsDownlink-v15a0 SEQUENCE (SIZE (1.. maxDownlinkFeatureSets)) OF FeatureSetDownlink-v15a0 |  | Not checked |  |  |  |  |  |
| featureSetsDownlink-v1610 SEQUENCE (SIZE (1.. maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1610 |  | Checked |  | FeatureSetDownlink-v1610 (Table 8.2.1.1.2.3.3-7) |  |  |  |
| featureSetsUplink-v1610 SEQUENCE (SIZE (1.. maxUplinkFeatureSets)) OF FeatureSetUplink-v1610 |  | Checked |  | FeatureSetUplink-v1610 (Table 8.2.1.1.2.3.3-8) |  |  |  |
| featureSetDownlinkPerCC-v1620 SEQUENCE (SIZE (1.. maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC-v1620 |  | Not checked |  |  |  |  |  |
| featureSetsUplink-v1630 SEQUENCE (SIZE (1.. maxUplinkFeatureSets)) OF FeatureSetUplink-v1630 |  | Not checked |  |  |  |  |  |
| featureSetsUplink-v1640 SEQUENCE (SIZE (1.. maxUplinkFeatureSets)) OF FeatureSetUplink-v1640 |  | Not checked |  |  |  |  |  |
| featureSetsDownlink-v1700 SEQUENCE (SIZE (1.. maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1700 |  | Not checked |  |  |  |  |  |
| featureSetsDownlinkPerCC-v1700 SEQUENCE (SIZE (1.. maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC-v1700 |  | Not checked |  |  |  |  |  |
| featureSetsUplink-v1710 SEQUENCE (SIZE (1.. maxUplinkFeatureSets)) OF FeatureSetUplink-v1710 |  | Not checked |  |  |  |  |  |
| featureSetsUplinkPerCC-v1700 SEQUENCE (SIZE (1.. maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC-v1700 |  | Not checked |  |  |  |  |  |
| featureSetsDownlink-v1720 SEQUENCE (SIZE (1.. maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1720 |  | Not checked |  |  |  |  |  |
| featureSetsDownlinkPerCC-v1720 SEQUENCE (SIZE (1.. maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC-v1720 |  | Not checked |  |  |  |  |  |
| featureSetsUplink-v1720 SEQUENCE (SIZE (1.. maxUplinkFeatureSets)) OF FeatureSetUplink-v1720 |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| featureSetCombinations SEQUENCE (SIZE (1..maxFeatureSetCombinations)) OF FeatureSetCombination { |  |  |  |  |  |  |  |
| FeatureSetCombination SEQUENCE (SIZE (1..maxSimultaneousBands)) OF FeatureSetsPerBand { |  |  |  |  |  |  |  |
| FeatureSetsPerBand SEQUENCE (SIZE (1..maxFeatureSetsPerBand)) OF FeatureSet { |  |  |  |  |  |  |  |
| FeatureSetCHOICE { |  |  |  |  |  |  |  |
| eutra SEQUENCE { |  |  |  |  |  |  |  |
| downlinkSetEUTRA |  | Not checked |  | FeatureSetEUTRA-DownlinkId |  |  |  |
| uplinkSetEUTRA |  | Not checked |  | FeatureSetEUTRA-UplinkId |  |  |  |
| } |  |  |  |  |  |  |  |
| nr SEQUENCE { |  |  |  |  |  |  |  |
| downlinkSetNR |  | Not checked |  | FeatureSetDownlinkId |  |  |  |
| uplinkSetNR |  | Not checked |  | FeatureSetUpinkId |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| fdd-Add-UE-NR-Capabilities-v1530 SEQUENCE { |  |  |  |  |  |  |  |
| eutra-ParametersXDD-Diff SEQUENCE { |  |  |  |  |  |  |  |
| rsrqMeasWidebandEUTRA |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| tdd-Add-UE-NR-Capabilities-v1530 SEQUENCE { |  |  |  |  |  |  |  |
| eutra-ParametersXDD-Diff SEQUENCE { |  |  |  |  |  |  |  |
| rsrqMeasWidebandEUTRA |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| dummy |  | Not checked |  |  |  |  |  |
| interRAT-Parameters SEQUENCE { |  |  |  |  |  |  |  |
| eutra SEQUENCE { |  |  |  |  |  |  |  |
| supportedBandListEUTRA SEQUENCE (SIZE (1..maxBandsEUTRA)) OF FreqBandIndicatorEUTRA |  | Not checked |  |  |  |  |  |
| eutra-ParametersCommon SEQUENCE { |  |  |  |  |  |  |  |
| mfbi-EUTRA |  | Not checked |  |  |  |  |  |
| modifiedMRP-BehaviorEUTRA |  | Not checked |  |  |  |  |  |
| multiNS-Pmax-EUTRA |  | Not checked |  |  |  |  |  |
| rs-SINR-MeasEUTRA |  | Checked |  |  |  | pc_RS_SINR_MeasEUTRA |  |
| ne-DC |  | Not checked |  |  |  |  |  |
| nr-HO-ToEN-DC-r16 |  | Checked |  |  |  | pc_interRAT_NR_ToENDC |  |
| } |  |  |  |  |  |  |  |
| eutra-ParametersXDD-Diff SEQUENCE { |  |  |  |  |  |  |  |
| rsrqMeasWidebandEUTRA |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| inactiveState |  | Checked |  |  |  | pc_inactiveState |  |
| delayBudgetReporting |  | Not checked |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| sdap-Parameters SEQUENCE { |  |  |  |  |  |  |  |
| as-ReflectiveQoS |  | Checked |  |  |  | pc_as_ReflectiveQoS |  |
| } |  |  |  |  |  |  |  |
| overheatingInd |  | Not checked |  |  |  |  |  |
| ims-Parameters SEQUENCE { |  |  |  |  |  |  |  |
| ims-ParametersCommon SEQUENCE { |  |  |  |  |  |  |  |
| voiceOverEUTRA-5GC |  | Not checked |  |  |  |  |  |
| voiceOverSCG-BearerEUTRA-5GC |  | Not checked |  |  |  |  |  |
| voiceFallbackIndicationEPS-r16 |  | Checked |  |  |  | pc_voiceFallbackIndication |  |
| } |  |  |  |  |  |  |  |
| ims-ParametersFRX-Diff SEQUENCE { |  |  |  |  |  |  |  |
| voiceOverNR |  | Checked (NOTE 16) |  |  |  | pc_voiceOverNR |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| fr1-Add-UE-NR-Capabilities-v1540 SEQUENCE { |  |  |  |  |  |  |  |
| ims-ParametersFRX-Diff SEQUENCE { |  |  |  |  |  |  |  |
| voiceOverNR |  | Checked (NOTE 16) |  |  |  | pc_voiceOverNR |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| fr2-Add-UE-NR-Capabilities-v1540 SEQUENCE { |  |  |  |  |  |  |  |
| ims-ParametersFRX-Diff SEQUENCE { |  |  |  |  |  |  |  |
| voiceOverNR |  | Checked (NOTE 16) |  |  |  | pc_voiceOverNR |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| fr1-fr2-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |  |  |  |  |
| phy-ParametersFRX-Diff SEQUENCE { |  |  |  |  |  |  |  |
| dynamicSFI |  | Not checked |  |  |  |  |  |
| dummy1 |  | Not checked |  |  |  |  |  |
| twoFL-DMRS |  | Not checked |  |  |  |  |  |
| dummy2 |  | Not checked |  |  |  |  |  |
| dummy3 |  | Not checked |  |  |  |  |  |
| supportedDMRS-TypeDL |  | Not checked |  |  |  |  |  |
| supportedDMRS-TypeUL |  | Not checked |  |  |  |  |  |
| semiOpenLoopCSI |  | Not checked |  |  |  |  |  |
| csi-ReportWithoutPMI |  | Not checked |  |  |  |  |  |
| csi-ReportWithoutCQI |  | Not checked |  |  |  |  |  |
| onePortsPTRS |  | Not checked |  |  |  |  |  |
| twoPUCCH-F0-2-ConsecSymbols |  | Not checked |  |  |  |  |  |
| pucch-F2-WithFH |  | Not checked |  |  |  |  |  |
| pucch-F3-WithFH |  | Not checked |  |  |  |  |  |
| pucch-F4-WithFH |  | Not checked |  |  |  |  |  |
| freqHoppingPUCCH-F0-2 |  | Not checked |  |  |  |  |  |
| freqHoppingPUCCH-F1-3-4 |  | Not checked |  |  |  |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot |  | Not checked |  |  |  |  |  |
| uci-CodeBlockSegmentation |  | Not checked |  |  |  |  |  |
| onePUCCH-LongAndShortFormat |  | Not checked |  |  |  |  |  |
| twoPUCCH-AnyOthersInSlot |  | Not checked |  |  |  |  |  |
| intraSlotFreqHopping-PUSCH |  | Not checked |  |  |  |  |  |
| pusch-LBRM |  | Not checked |  |  |  |  |  |
| pdcch-BlindDetectionCA |  | Not checked |  |  |  |  |  |
| tpc-PUSCH-RNTI |  | Not checked |  |  |  |  |  |
| tpc-PUCCH-RNTI |  | Not checked |  |  |  |  |  |
| tpc-SRS-RNTI |  | Not checked |  |  |  |  |  |
| absoluteTPC-Command |  | Not checked |  |  |  |  |  |
| twoDifferentTPC-Loop-PUSCH |  | Not checked |  |  |  |  |  |
| twoDifferentTPC-Loop-PUCCH |  | Not checked |  |  |  |  |  |
| pusch-HalfPi-BPSK |  | Not checked |  |  |  |  |  |
| pucch-F3-4-HalfPi-BPSK |  | Not checked |  |  |  |  |  |
| almostContiguousCP-OFDM-UL |  | Not checked |  |  |  |  |  |
| sp-CSI-RS |  | Not checked |  |  |  |  |  |
| sp-CSI-IM |  | Not checked |  |  |  |  |  |
| tdd-MultiDL-UL-SwitchPerSlot |  | Not checked |  |  |  |  |  |
| multipleCORESET |  | Not checked |  |  |  |  |  |
| csi-RS-IM-ReceptionForFeedback |  | Not checked |  | CSI-RS-IM-ReceptionForFeedback |  |  |  |
| csi-RS-ProcFrameworkForSRS |  | Not checked |  | CSI-RS-ProcFrameworkForSRS |  |  |  |
| csi-ReportFramework |  | Not checked |  | CSI-ReportFramework |  |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot SEQUENCE { |  |  |  |  |  |  |  |
| sameSymbol |  | Not checked |  |  |  |  |  |
| diffSymbol |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| mux-SR-HARQ-ACK-PUCCH |  | Not checked |  |  |  |  |  |
| mux-MultipleGroupCtrlCH-Overlap |  | Not checked |  |  |  |  |  |
| dl-SchedulingOffset-PDSCH-TypeA |  | Checked (NOTE 18) |  |  |  | pc_dl_SchedulingOffset_PDSCH_TypeA |  |
| dl-SchedulingOffset-PDSCH-TypeB |  | Not checked |  |  |  |  |  |
| ul-SchedulingOffset |  | Not checked |  |  |  |  |  |
| dl-64QAM-MCS-TableAlt |  | Not checked |  |  |  |  |  |
| ul-64QAM-MCS-TableAlt |  | Not checked |  |  |  |  |  |
| cqi-TableAlt |  | Not checked |  |  |  |  |  |
| oneFL-DMRS-TwoAdditionalDMRS-UL |  | Not checked |  |  |  |  |  |
| twoFL-DMRS-TwoAdditionalDMRS-UL |  | Not checked |  |  |  |  |  |
| oneFL-DMRS-ThreeAdditionalDMRS-UL |  | Not checked |  |  |  |  |  |
| pdcch-BlindDetectionNRDC |  | Not checked |  |  |  |  |  |
| mux-HARQ-ACK-PUSCH-DiffSymbol |  | Not checked |  |  |  |  |  |
| type1-HARQ-ACK-Codebook-r16 |  | Not checked |  |  |  |  |  |
| enhancedPowerControl-r16 |  | Not checked |  |  |  |  |  |
| simultaneousTCI-ActMultipleCC-r16 |  | Not checked |  |  |  |  |  |
| simultaneousSpatialRelationMultipleCC-r16 |  | Not checked |  |  |  |  |  |
| cli-RSSI-FDM-DL-r16 |  | Not checked |  |  |  |  |  |
| cli-SRS-RSRP-FDM-DL-r16 |  | Not checked |  |  |  |  |  |
| maxLayersMIMO-Adaptation-r16 |  | Not checked |  |  |  |  |  |
| aggregationFactorSPS-DL-r16 |  | Not checked |  |  |  |  |  |
| maxTotalResourcesForOneFreqRange-r16 |  | Not checked |  |  |  |  |  |
| csi-ReportFrameworkExt-r16 |  | Not checked |  |  |  |  |  |
| twoTCI-Act-servingCellInCC-List-r16 |  | Not checked |  |  |  |  |  |
| cri-RI-CQI-WithoutNon-PMI-PortInd-r16 |  | Not checked |  |  |  |  |  |
| cqi-4-BitsSubbandTN-NonSharedSpectrumChAccess-r17 |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| measAndMobParametersFRX-Diff SEQUENCE { |  |  |  |  |  |  |  |
| ss-SINR-Meas |  | Checked (NOTE 10) |  |  |  | pc_ss_SINR_Meas |  |
| csi-RSRP-AndRSRQ-MeasWithSSB |  | Checked (NOTE 3) |  |  |  | pc_csi-RSRP-AndRSRQ-MeasWithSSB |  |
| csi-RSRP-AndRSRQ-MeasWithoutSSB |  | Checked (NOTE 11) |  |  |  | pc_csi_RSRP_AndRSRQ_MeasWithoutSSB |  |
| csi-SINR-Meas |  | Not checked |  |  |  |  |  |
| csi-RS-RLM |  | Not checked |  |  |  |  |  |
| handoverInterF |  | Checked (NOTE 14) |  |  |  | pc_ handoverInterF |  |
| handoverLTE-EPC |  | Not checked |  |  |  |  |  |
| handoverLTE-5GC |  | Not checked |  |  |  |  |  |
| maxNumberResource-CSI-RS-RLM |  | Not checked |  |  |  |  |  |
| simultaneousRxDataSSB-DiffNumerology |  | Not checked |  |  |  |  |  |
| nr-AutonomousGaps-r16 |  | Not checked |  |  |  |  |  |
| nr-AutonomousGaps-ENDC-r16 |  | Not checked |  |  |  |  |  |
| nr-AutonomousGaps-NEDC-r16 |  | Not checked |  |  |  |  |  |
| nr-AutonomousGaps-NRDC-r16 |  | Not checked |  |  |  |  |  |
| dummy |  | Not checked |  |  |  |  |  |
| cli-RSSI-Meas-r16 |  | Not checked |  |  |  |  |  |
| cli-SRS-RSRP-Meas-r16 |  | Not checked |  |  |  |  |  |
| interFrequencyMeas-NoGap-r16 |  | Not checked |  |  |  |  |  |
| simultaneousRxDataSSB-DiffNumerology-Inter-r16 |  | Not checked |  |  |  |  |  |
| idleInactiveNR-MeasReport-r16 |  | Not checked |  |  |  |  |  |
| idleInactiveNR-MeasBeamReport-r16 |  | Not checked |  |  |  |  |  |
| increasedNumberofCSIRSPerMO-r16 |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | UE-NR-Capability-v1550 |  |  |  |
| reducedCP-Latency |  | Checked |  |  |  | pc_reducedCP_Latency |  |
| nonCriticalExtension SEQUENCE { |  |  |  | UE-NR-Capability-v1560 |  |  |  |
| nrdc-Parameters |  | Not checked |  | NRDC-Parameters |  |  |  |
| receivedFilters |  | Not checked |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | UE-NR-Capability-v1570 |  |  |  |
| nrdc-Parameters-v1570 |  | Not checked |  | NRDC-Parameters-v1570 |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | UE-NR-Capability-v1610 |  |  |  |
| inDeviceCoexInd-r16 |  | Not checked |  |  |  |  |  |
| dl-DedicatedMessageSegmentation-r16 |  | Checked |  |  |  | pc_NR_dl_DedicatedMessageSegmentation |  |
| nrdc-Parameters-v1610 |  | Not checked |  |  |  |  |  |
| powSav-Parameters-r16 |  | Checked |  | PowSav-Parameters-r16 (Table 8.2.1.1.2.3.3-10) |  |  |  |
| fr1-Add-UE-NR-Capabilities-v1610 |  | Not checked |  |  |  |  |  |
| fr2-Add-UE-NR-Capabilities-v1610 |  | Not checked |  |  |  |  |  |
| bh-RLF-Indication-r16 |  | Not checked |  |  |  |  |  |
| directSN-AdditionFirstRRC-IAB-r16 |  | Not checked |  |  |  |  |  |
| bap-Parameters-r16 |  | Not checked |  |  |  |  |  |
| referenceTimeProvision-r16 |  | Not checked |  |  |  |  |  |
| sidelinkParameters-r16 |  | Not checked |  |  |  |  |  |
| highSpeedParameters-r16 |  | Not checked |  |  |  |  |  |
| mac-Parameters-v1610 |  | Not checked |  |  |  |  |  |
| mcgRLF-RecoveryViaSCG-r16 |  | Not checked |  |  |  |  |  |
| resumeWithStoredMCG-SCells-r16 |  | Not checked |  |  |  |  |  |
| resumeWithStoredSCG-r16 |  | Not checked |  |  |  |  |  |
| resumeWithSCG-Config-r16 |  | Not checked |  |  |  |  |  |
| ue-BasedPerfMeas-Parameters-r16 |  | Checked |  | UE-BasedPerfMeas-Parameters-r16 (Table 8.2.1.1.2.3.3-11) |  |  |  |
| son-Parameters-r16 { |  |  |  |  |  |  |  |
| rach-Report-r16 |  | Checked |  |  |  | pc_rachReport_r16 |  |
| } |  |  |  |  |  |  |  |
| onDemandSIB-Connected-r16 |  | Not checked |  |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  | UE-NR-Capability-v1640 |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| Note 1: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and intraAndInterF-MeasAndReport is supported in both modes, then support of intraAndInterF-MeasAndReport will be signalled in measAndMobParameters/measAndMobParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and intraAndInterF-MeasAndReport is only supported in one mode, then support of intraAndInterF-MeasAndReport will be signalled in one of fdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff as appropriate.) Note 2: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and eventA-MeasAndReport is supported in both modes, then support of eventA-MeasAndReport will be signalled in measAndMobParameters/measAndMobParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and eventA-MeasAndReport is only supported in one mode, then support of eventA-MeasAndReport will be signalled in one of fdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff as appropriate. Note 3: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and csi-RSRP-AndRSRQ-MeasWithSSB is supported in both frequency ranges, then support of csi-RSRP-AndRSRQ-MeasWithSSB will be signalled in measAndMobParameters/MeasAndMobParametersFRX-Diff. If the UE supports both frequency ranges (FR1 + FR2) and csi-RSRP-AndRSRQ-MeasWithSSB is only supported in one frequency range, then support of csi-RSRP-AndRSRQ-MeasWithSSB will be signalled in one of fr1-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff or fr2-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff as appropriate.) Note 4: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and skipUplinkTxDynamic is supported in both modes, then support of skipUplinkTxDynamic will be signalled in mac-Parameters/mac-ParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and skipUplinkTxDynamic is only supported in one mode, then skipUplinkTxDynamic will be signalled in one of fdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff as appropriate. Note 5: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and logicalChannelSR-DelayTimer is supported in both modes, then support of logicalChannelSR-DelayTimer will be signalled in mac-Parameters/mac-ParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and logicalChannelSR-DelayTimer is only supported in one mode, then support of logicalChannelSR-DelayTimer will be signalled in one of fdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff as appropriate. Note 6: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and longDRX_Cycle is supported in both modes, then support of longDRX_Cycle will be signalled in mac-Parameters/mac-ParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and longDRX_Cycle is only supported in one mode, then support of longDRX_Cycle will be signalled in one of fdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff as appropriate. Note 7: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and shortDRX_Cycle is supported in both modes, then support of shortDRX_Cycle will be signalled in mac-Parameters/mac-ParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and shortDRX_Cycle is only supported in one mode, then support of shortDRX_Cycle will be signalled in one of fdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff as appropriate. Note 8: If the UE supports FR2 and pc_pdsch_256QAM_FR2 is supported, then support of pdsch_256QAM_FR2 will be signaled in at least one entry of supportedBandListNR as appropriate. Note 9: If pc_pusch_256QAM_FR1 or pc_pusch_256QAM_FR2 is supported, then support of pusch-256QAM will be signaled in at least one entry of supportedBandListNR as appropriate. Note 10: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and ss-SINR-Meas is supported in both frequency ranges, then support of ss-SINR-Meas will be signaled in measAndMobParameters/MeasAndMobParametersFRX-Diff. If the UE supports both frequency ranges (FR1 + FR2) and ss-SINR-Meas is only supported in one frequency range, then support of ss-SINR-Meas will be signaled in one of fr1-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff or fr2-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff as appropriate.) Note 11: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and csi-RSRP-AndRSRQ-MeasWithoutSSB is supported in both frequency ranges, then support of csi-RSRP-AndRSRQ-MeasWithoutSSB will be signaled in measAndMobParameters/MeasAndMobParametersFRX-Diff. If the UE supports both frequency ranges (FR1 + FR2) and csi-RSRP-AndRSRQ-MeasWithoutSSB is only supported in one frequency range, then support of csi-RSRP-AndRSRQ-MeasWithoutSSB will be signaled in one of fr1-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff or fr2-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff as appropriate.) Note 12: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and multipleConfiguredGrants is supported in both modes, then support of multipleConfiguredGrants will be signaled in mac-Parameters/mac-ParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and multipleConfiguredGrants is only supported in one mode, then support of multipleConfiguredGrants will be signaled in one of fdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff as appropriate. Note 13: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and sftd-MeasNR-Neigh is supported in both modes, then support of sftd-MeasNR-Neigh will be signaled in measAndMobParameters/measAndMobParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and sftd-MeasNR-Neigh is only supported in one mode, then support of sftd-MeasNR-Neigh will be signaled in one of fdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff as appropriate.) Note 14: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and handoverInterF is supported in both frequency ranges, then support of handoverInterF will be signaled in measAndMobParameters/MeasAndMobParametersFRX-Diff. If the UE supports both frequency ranges (FR1 + FR2) and handoverInterF is only supported in one frequency range, then support of handoverInterF will be signaled in one of fr1-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff or fr2-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff as appropriate.) Note 15: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and handoverInterF is supported in both modes, then support of handoverInterF will be signaled in measAndMobParameters/measAndMobParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and handoverInterF is only supported in one mode, then support of handoverInterF will be signaled in one of fdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff as appropriate. Note 16: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and voiceOverNR is supported in both frequency ranges, then support of voiceOverNR will be signaled in ims-Parameters/ims-ParametersFRX-Diff. If the UE supports both frequency ranges (FR1 + FR2) and voiceOverNR is only supported in one frequency range, then support of voiceOverNR will be signaled in one of fr1-Add-UE-NR-Capabilities-v1540/ims-ParametersFRX-Diff or fr2-Add-UE-NR-Capabilities-v1540/ims-ParametersFRX-Diff as appropriate.) Note 17: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and dl-SchedulingOffset-PDSCH-TypeA is supported in both modes, then support of dl-SchedulingOffset-PDSCH-TypeA will be signaled in phy-Parameters/phy-ParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and dl-SchedulingOffset-PDSCH-TypeA is only supported in one mode, then support of dl-SchedulingOffset-PDSCH-TypeA will be signaled in one of fdd-Add-UE-NR-Capabilities/ phy-ParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/phy-ParametersXDD-Diff as appropriate.) Note 18: If the UE supports single frequency range (FR1 or FR2) the value of dl-SchedulingOffset-PDSCH-TypeA will be signaled in phy-ParametersFRX-Diff. If the UE supports both frequency ranges (FR1 and FR2) and dl-SchedulingOffset-PDSCH-TypeA is supported in both frequency ranges, then support of dl-SchedulingOffset-PDSCH-TypeA will be signaled in fr1-fr2-Add-UE-NR-Capabilities/phy-ParametersFRX-Diff and it will not be present in the common IE. If the UE supports both frequency ranges (FR1 + FR2) and dl-SchedulingOffset-PDSCH-TypeA is only supported in one frequency range, then support of dl-SchedulingOffset-PDSCH-TypeA will be signaled in one of fr1-Add-UE-NR-Capabilities/phy-ParametersFRX-Diff or fr2-Add-UE-NR-Capabilities/phy-ParametersFRX-Diff as appropriate. |  |  |  |  |  |  |  |

Table 8.2.1.1.2.3.3-5: UE-MRDC-Capability (Table 8.2.1.1.2.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.4-48 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE-MRDC-Capability ::= SEQUENCE { |  |  |  |
| measAndMobParametersMRDC SEQUENCE { |  |  |  |
| measAndMobParametersMRDC-Common SEQUENCE { |  |  |  |
| independentGapConfig | Checked |  | pc_independentGapConfig |
| } |  |  |  |
| measAndMobParametersMRDC-XDD-Diff SEQUENCE { |  |  |  |
| sftd-MeasPSCell | Checked (NOTE 5) |  | pc_SFTD_MeasPSCell_MRDC_FDD and/or pc_SFTD_MeasPSCell_MRDC_T DD |
| sftd-MeasNR-Cell | Checked (NOTE 6) |  | pc_SFTD_MeasNR_Cell_FDD and/or pc_SFTD_MeasNR_Cell_TDD |
| } |  |  |  |
| measAndMobParametersMRDC-FRX-Diff SEQUENCE { |  |  |  |
| simultaneousRxDataSSB-DiffNumerology | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| phy-ParametersMRDC-v1530 SEQUENCE { |  |  |  |
| naics-Capability-List SEQUENCE (SIZE (1..maxNrofNAICS-Entries)) OF NAICS-Capability-Entry { | Not checked |  |  |
| NAICS-Capability-Entry SEQUENCE { |  |  |  |
| numberOfNAICS-CapableCC | Not checked |  |  |
| numberOfAggregatedPRB | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| rf-ParametersMRDC SEQUENCE { |  |  |  |
| supportedBandCombinationList SEQUENCE { (SIZE (1..maxBandComb)) OF BandCombination { |  |  |  |
| BandCombination SEQUENCE { |  |  |  |
| bandList SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters { | Not checked |  |  |
| BandParameters CHOICE |  |  |  |
| eutra SEQUENCE { |  | EUTRA bands |  |
| bandEUTRA | Not checked | FreqBandIndicatorEUTRA |  |
| ca-BandwidthClassDL-EUTRA | Not checked | CA-BandwidthClassEUTRA |  |
| ca-BandwidthClassUL-EUTRA | Not checked | CA-BandwidthClassEUTRA |  |
| } |  |  |  |
| nr SEQUENCE { |  | NR bands |  |
| bandNR | Not checked | FreqBandIndicatorNR |  |
| ca-BandwidthClassDL-NR | Not checked | CA-BandwidthClassNR |  |
| ca-BandwidthClassUL-NR | Not checked | CA-BandwidthClassNR |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| featureSetCombination | Not checked | FeatureSetCombinationId |  |
| ca-ParametersEUTRA | Not checked | CA-ParametersEUTRA |  |
| ca-ParametersNR | Not checked | CA-ParametersNR |  |
| mrdc-Parameters SEQUENCE { |  |  |  |
| singleUL-Transmission | Not checked |  |  |
| dynamicPowerSharingENDC | Checked | Note 4 | pc_dynamicPowerSharing |
| tdm-Pattern | Not checked |  |  |
| ul-SharingEUTRA-NR | Not checked |  |  |
| ul-SwitchingTimeEUTRA-NR | Not checked |  |  |
| simultaneousRxTxInterBandENDC | Not checked |  |  |
| asyncIntraBandENDC | Not checked |  |  |
| dualPA-Architecture | Not checked |  |  |
| intraBandENDC-Support-v1540 | Not checked |  |  |
| ul-TimingAlignmentEUTRA-NR | Not checked |  |  |
| } |  |  |  |
| supportedBandwidthCombinationSet | Not checked |  |  |
| powerClass-v1530 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| appliedFreqBandListFilter | Not checked | FreqBandList |  |
| srs-SwitchingTimeRequested | Not checked |  |  |
| supportedBandCombinationList-v1540 SEQUENCE (SIZE (1..maxBandComb)) OF OF BandCombination-v1540 { |  | BandCombinationList-v1540 |  |
| BandCombination-v1540 SEQUENCE { |  |  |  |
| bandList-v1540 SEQUENCE (SIZE (1..maxSimultaneousBands)) OF OF BandParameters-v1540 { | Not checked | BandParameters-v1540 |  |
| BandParameters-v1540 SEQUENCE { |  |  |  |
| srs-CarrierSwitch CHOICE { |  |  |  |
| nr SEQUENCE { |  | NR bands |  |
| srs-SwitchingTimesListNR SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingTimeNR | Not checked |  |  |
| } |  |  |  |
| eutra SEQUENCE { |  | EUTRA bands |  |
| srs-SwitchingTimesListEUTRA SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingTimeEUTRA | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| srs-TxSwitch-v1540 SEQUENCE { |  |  |  |
| supportedSRS-TxPortSwitch | Not checked |  |  |
| txSwitchImpactToRx | Not checked |  |  |
| txSwitchWithAnotherBand | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ca-ParametersNR-v1540 SEQUENCE { |  |  |  |
| simultaneousSRS-AssocCSI-RS-AllCC | Not checked |  |  |
| csi-RS-IM-ReceptionForFeedbackPerBandComb SEQUENCE { |  |  |  |
| maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC | Not checked |  |  |
| totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC | Not checked |  |  |
| } |  |  |  |
| simultaneousCSI-ReportsAllCC | Not checked |  |  |
| dualPA-Architecture | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| generalParametersMRDC SEQUENCE { |  |  |  |
| splitSRB-WithOneUL-Path | Checked (NOTE 1) |  | pc_splitSRB_WithOneUL_Path |
| splitDRB-withUL-Both-MCG-SCG | Checked (NOTE 2) |  | pc_splitDRB_withUL_Both_MCG_SCG |
| srb3 | Checked (NOTE 1) |  | pc_srb3 |
| v2x-EUTRA | Not checked |  |  |
| } |  |  |  |
| fdd-Add-UE-MRDC-Capabilities SEQUENCE { |  |  |  |
| measAndMobParametersMRDC-XDD-Diff SEQUENCE { |  |  |  |
| sftd-MeasPSCell | Checked (NOTE 5) |  | pc_SFTD_MeasPSCell_MRDC_FDD and/or pc_SFTD_MeasPSCell_MRDC_T DD |
| sftd-MeasNR-Cell | Checked (NOTE 6) |  | pc_SFTD_MeasNR_Cell_FDD and/or pc_SFTD_MeasNR_Cell_TDD |
| } |  |  |  |
| generalParametersMRDC-XDD-Diff SEQUENCE { |  |  |  |
| splitSRB-WithOneUL-Path | Checked (NOTE 1) |  | pc_splitSRB_WithOneUL_Path |
| splitDRB-withUL-Both-MCG-SCG | Checked (NOTE 2) |  | pc_splitDRB_withUL_Both_MCG_SCG |
| srb3 | Checked (NOTE 3) |  | pc_srb3 |
| v2x-EUTRA-v1530 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| tdd-Add-UE-MRDC-Capabilities SEQUENCE { |  |  |  |
| measAndMobParametersMRDC-XDD-Diff SEQUENCE { |  |  |  |
| sftd-MeasPSCell | Checked (NOTE 5) |  | pc_SFTD_MeasPSCell_MRDC_FDD and/or pc_SFTD_MeasPSCell_MRDC_T DD |
| sftd-MeasNR-Cell | Checked (NOTE 6) |  | pc_SFTD_MeasNR_Cell_FDD and/or pc_SFTD_MeasNR_Cell_TDD |
| } |  |  |  |
| generalParametersMRDC-XDD-Diff SEQUENCE { |  |  |  |
| splitSRB-WithOneUL-Path | Checked (NOTE 1) |  | pc_splitSRB_WithOneUL_Path |
| splitDRB-withUL-Both-MCG-SCG | Checked (NOTE 2) |  | pc_splitDRB_withUL_Both_MCG_SCG |
| srb3 | Checked (NOTE 3) |  | pc_srb3 |
| v2x-EUTRA-v1530 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| fr1-Add-UE-MRDC-Capabilities SEQUENCE { |  |  |  |
| measAndMobParametersMRDC-FRX-Diff SEQUENCE { |  |  |  |
| simultaneousRxDataSSB-DiffNumerology | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| fr2-Add-UE-MRDC-Capabilities SEQUENCE { |  |  |  |
| measAndMobParametersMRDC-FRX-Diff SEQUENCE { |  |  |  |
| simultaneousRxDataSSB-DiffNumerology | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| featureSetCombinations SEQUENCE (SIZE (1.. maxFeatureSetCombinations)) OF FeatureSetCombination { | Not checked | FeatureSetCombination |  |
| FeatureSetCombination SEQUENCE (SIZE (1..maxSimultaneousBands)) OF FeatureSetsPerBand { |  |  |  |
| FeatureSetsPerBand SEQUENCE (SIZE (1..maxFeatureSetsPerBand)) OF FeatureSet { |  |  |  |
| FeatureSet CHOICE { |  |  |  |
| eutra SEQUENCE { |  | EUTRA bands |  |
| downlinkSetEUTRA | Not checked |  |  |
| uplinkSetEUTRA | Not checked |  |  |
| } |  |  |  |
| nr SEQUENCE { |  | NR bands |  |
| downlinkSetNR | Not checked |  |  |
| uplinkSetNR | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| pdcp-ParametersMRDC-v1530 SEQUENCE { |  |  |  |
| pdcp-DuplicationSplitSRB | Checked |  | pc_pdcp_DuplicationSplitSRB |
| pdcp-DuplicationSplitDRB | Checked |  | pc_pdcp_DuplicationSplitDRB |
| } |  |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension SEQUENCE {} | Not checked |  |  |
| } |  |  |  |
| Note 1: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and splitSRB-WithOneUL-Path is supported in both modes, then support of splitSRB-WithOneUL-Path will be signalled in generalParametersMRDC/generalParametersMRDC-XDD-Diff. If the UE is dual mode (FDD + TDD) and splitSRB-WithOneUL-Path is only supported in one mode, then support of splitSRB-WithOneUL-Path will be signalled in one of fdd-Add-UE-MRDC-Capabilities/generalParametersMRDC-XDD-Diff or tdd-Add-UE-MRDC-Capabilities/generalParametersMRDC-XDD-Diff as appropriate. Note 2: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and splitDRB-withUL-Both-MCG-SCG is supported in both modes, then support of splitDRB-withUL-Both-MCG-SCG will be signalled in generalParametersMRDC/generalParametersMRDC-XDD-Diff. If the UE is dual mode (FDD + TDD) and splitDRB-withUL-Both-MCG-SCG is only supported in one mode, then support of splitDRB-withUL-Both-MCG-SCG will be signalled in one of fdd-Add-UE-MRDC-Capabilities/generalParametersMRDC-XDD-Diff or tdd-Add-UE-MRDC-Capabilities/generalParametersMRDC-XDD-Diff as appropriate. Note 3: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and srb3 is supported in both modes, then support of srb3 will be signalled in generalParametersMRDC/generalParametersMRDC-XDD-Diff. If the UE is dual mode (FDD + TDD) and srb3 is only supported in one mode, then support of srb3 will be signalled in one of fdd-Add-UE-MRDC-Capabilities/generalParametersMRDC-XDD-Diff or tdd-Add-UE-MRDC-Capabilities/generalParametersMRDC-XDD-Diff as appropriate. Note 4: If pc_dynamicPowerSharing is supported, then support of dynamicPowerSharingENDC will be signalled in at least one entry of supportedBandCombinationList as appropriate. As per 38.306 cl 4.2.7.9 this is applicable to FR1 only. Note 5: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and sftd-MeasPSCell is supported in both modes, then support of sftd-MeasPSCell will be signalled in measAndMobParametersMRDC/measAndMobParametersMRDC-XDD-Diff If the UE is dual mode (FDD + TDD) and sftd-MeasPSCell is only supported in one mode, then support of sftd-MeasPSCell will be signalled in one of fdd-Add-UE-MRDC-Capabilities/measAndMobParametersMRDC-XDD-Diff or tdd-Add-UE-MRDC-Capabilities/ measAndMobParametersMRDC-XDD-Diff as appropriate. Note 6: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and sftd-MeasNR-Cell is supported in both modes, then support of sftd-MeasNR-Cell will be signalled in measAndMobParametersMRDC/measAndMobParametersMRDC-XDD-Diff If the UE is dual mode (FDD + TDD) and sftd-MeasNR-Cell is only supported in one mode, then support of sftd-MeasNR-Cell will be signalled in one of fdd-Add-UE-MRDC-Capabilities/measAndMobParametersMRDC-XDD-Diff or tdd-Add-UE-MRDC-Capabilities/ measAndMobParametersMRDC-XDD-Diff as appropriate. |  |  |  |

Table 8.2.1.1.2.3.3-6: ULDedicatedMessageSegment (steps 2a1 Table 8.2.1.1.2.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-32C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ULDedicatedMessageSegment-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ulDedicatedMessageSegment-r16 SEQUENCE { |  |  |  |
| segmentNumber-r16 | 0 for first segment, and incremented by 1 for each subsequent segment |  |  |
| rrc-MessageSegmentContainer-r16 | Not Checked | OCTET STRING including segmented UECapabilityInformation message |  |
| rrc-MessageSegmentType-r16 | notLastSegment for all segments except the last segment. lastSegment for the last segment |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.1.1.2.3.3-7: FeatureSetDownlink-v1610 (Table 8.2.1.1.2.3.3-4)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| FeatureSetDownlink-v1610 ::= SEQUENCE { |  |  |  |
| cbgPDSCH-ProcessingType1-DifferentTB-PerSlot-r16 | Not checked |  |  |
| cbgPDSCH-ProcessingType2-DifferentTB-PerSlot-r16 | Not checked |  |  |
| intraFreqDAPS-r16 | Checked |  | pc_intraFreqDAPS_r16 |
| intraBandFreqSeparationDL-v1620 | Not checked |  |  |
| intraBandFreqSeparationDL-Only-r16 | Not checked |  |  |
| pdcch-Monitoring-r16 | Not checked |  |  |
| pdcch-MonitoringMixed-r16 | Not checked |  |  |
| crossCarrierSchedulingProcessing-DiffSCS-r16 | Not checked |  |  |
| singleDCI-SDM-scheme-r16 | Not checked |  |  |
| } |  |  |  |

Table 8.2.1.1.2.3.3-8: FeatureSetUplink-v1610 (Table 8.2.1.1.2.3.3-4)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| FeatureSetUplink-v1610 ::= SEQUENCE { |  |  |  |
| pusch-RepetitionTypeB-r16 | Checked |  | pc_pusch_RepetitionTypeB_r16 |
| ul-CancellationSelfCarrier-r16 | Not checked |  |  |
| ul-CancellationCrossCarrier-r16 | Not checked |  |  |
| ul-FullPwrMode2-MaxSRS-ResInSet-r16 | Not checked |  |  |
| cbgPUSCH-ProcessingType1-DifferentTB-PerSlot-r16 | Not checked |  |  |
| cbgPUSCH-ProcessingType2-DifferentTB-PerSlot-r16 | Not checked |  |  |
| supportedSRS-PosResources-r16 | Not checked |  |  |
| intraFreqDAPS-UL-r16 | Not checked |  |  |
| intraBandFreqSeparationUL-v1620 | Not checked |  |  |
| multiPUCCH-r16 | Not checked |  |  |
| twoPUCCH-Type1-r16 | Not checked |  |  |
| twoPUCCH-Type2-r16 | Not checked |  |  |
| twoPUCCH-Type3-r16 | Not checked |  |  |
| twoPUCCH-Type4-r16 | Not checked |  |  |
| mux-SR-HARQ-ACK-r16 | Not checked |  |  |
| dummy1 | Not checked |  |  |
| dummy2 | Not checked |  |  |
| twoPUCCH-Type5-r16 | Not checked |  |  |
| twoPUCCH-Type6-r16 | Not checked |  |  |
| twoPUCCH-Type7-r16 | Not checked |  |  |
| twoPUCCH-Type8-r16 | Not checked |  |  |
| twoPUCCH-Type9-r16 | Not checked |  |  |
| twoPUCCH-Type10-r16 | Not checked |  |  |
| twoPUCCH-Type11-r16 | Not checked |  |  |
| ul-IntraUE-Mux-r16 | Not checked |  |  |
| ul-FullPwrMode-r16 | Not checked |  |  |
| crossCarrierSchedulingProcessing-DiffSCS-r16 | Not checked |  |  |
| ul-FullPwrMode1-r16 | Not checked |  |  |
| ul-FullPwrMode2-SRSConfig-diffNumSRSPorts-r16 | Not checked |  |  |
| ul-FullPwrMode2-TPMIGroup-r16 | Not checked |  |  |
| } |  |  |  |

Table 8.2.1.1.2.3.3-9: CA-ParametersNR-v1610 (Table 8.2.1.1.2.3.3-4)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| CA-ParametersNR-v1610 ::= SEQUENCE { |  |  |  |
| parallelTxMsgA-SRS-PUCCH-PUSCH-r16 | Not checked |  |  |
| msgA-SUL-r16 | Not checked |  |  |
| jointSearchSpaceSwitchAcrossCells-r16 | Not checked |  |  |
| half-DuplexTDD-CA-SameSCS-r16 | Not checked |  |  |
| scellDormancyWithinActiveTime-r16 | Not checked |  |  |
| scellDormancyOutsideActiveTime-r16 | Checked |  | pc_scellDormancyOutsideActiveTime_r16 |
| crossCarrierA-CSI-trigDiffSCS-r16 | Not checked |  |  |
| defaultQCL-CrossCarrierA-CSI-Trig-r16 | Not checked |  |  |
| interCA-NonAlignedFrame-r16 | Not checked |  |  |
| simul-SRS-Trans-BC-r16 | Not checked |  |  |
| interFreqDAPS-r16 | Checked |  | pc_interFreqDAPS_r16 |
| codebookParametersPerBC-r16 | Not checked |  |  |
| blindDetectFactor-r16 | Not checked |  |  |
| pdcch-MonitoringCA-r16 | Not checked |  |  |
| pdcch-BlindDetectionCA-Mixed-r16 | Not checked |  |  |
| pdcch-BlindDetectionMCG-UE-r16 | Not checked |  |  |
| pdcch-BlindDetectionSCG-UE-r16 | Not checked |  |  |
| pdcch-BlindDetectionMCG-UE-Mixed-r16 | Not checked |  |  |
| pdcch-BlindDetectionSCG-UE-Mixed-r16 | Not checked |  |  |
| crossCarrierSchedulingDL-DiffSCS-r16 | Not checked |  |  |
| crossCarrierSchedulingDefaultQCL-r16 | Not checked |  |  |
| crossCarrierSchedulingUL-DiffSCS-r16 | Not checked |  |  |
| simul-SRS-MIMO-Trans-BC-r16 | Not checked |  |  |
| codebookParametersAdditionPerBC-r16 | Not checked |  |  |
| codebookComboParametersAdditionPerBC-r16 | Not checked |  |  |
| } |  |  |  |

Table 8.2.1.1.2.3.3-10: PowSav-Parameters-r16 (Table 8.2.1.1.2.3.3-4)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| PowSav-Parameters-r16 ::= SEQUENCE { |  |  |  |
| powSav-ParametersCommon-r16 SEQUENCE { |  |  |  |
| drx-Preference-r16 | Not checked |  |  |
| maxCC-Preference-r16 | Not checked |  |  |
| releasePreference-r16 | Checked |  | pc_releasePreference_r16 |
| minSchedulingOffsetPreference-r16 | Not checked |  |  |
| } |  |  |  |
| powSav-ParametersFRX-Diff-r16 | Not checked |  |  |
| } |  |  |  |

Table 8.2.1.1.2.3.3-11: UE-BasedPerfMeas-Parameters-r16 (Table 8.2.1.1.2.3.3-4)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| UE-BasedPerfMeas-Parameters-r16 ::= SEQUENCE { |  |  |  |
| barometerMeasReport-r16 | Checked |  | pc_barometer_r16 |
| immMeasBT-r16 | Not checked |  |  |
| immMeasWLAN-r16 | Checked |  | pc_immMeasWLAN_r16 |
| loggedMeasBT-r16 | Not checked |  |  |
| loggedMeasurements-r16 | Checked |  | pc_loggedMeasurements_r16 |
| loggedMeasWLAN-r16 | Checked |  | pc_loggedMeasWLAN_r16 |
| orientationMeasReport-r16 | Checked |  | pc_orientation_r16 |
| speedMeasReport-r16 | Checked |  | pc_speed_r16 |
| gnss-Location-r16 | Checked |  | pc_GNSS_location_r16 |
| ulPDCP-Delay-r16 | Checked |  | pc_PDCP_Delay_r16 |
| } |  |  |  |