3GPP TSG-RAN5 Meeting #97	R5-226085r1
Toulouse, France, 14th – 18th November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1917 | rev | - | Current version: | 17.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | almost contiguous allocation correction in SEM test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When almost contiguous RB allocation (RB allocation Type 0 as defined in 38.214) is used, the size of the first and last resource block groups (RBG) depend on the parameter RBstart that, according to 38.213, depends on the parameter offsetToCarrier. The parameter offsetToCarrier is defined differently in 38.508-1 depending on frequency Low, Mid or High and depending on the duplex mode FDD or TDD. Current definition of the almost contiguous RB allocations are only valid if offsetToCarrier = 0 what in practice is only valid when low frequency is tested. RB allocations need to be defined for the other frequency ranges Mid and High, for both FDD and TDD. In addition, observed that RB allocations for 45MHz are missing. By the way, almost contiguous RB allocation definition in this test is a copy of definition made in test case 6.2.2, so to facilitate specs maintenance, unique definition should be maintained only in test case 6.2.2 and refer to it. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed almost contiguous RB allocation definition table an pointed to those made in test case 6.2.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain incorrect and complex to maintain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R5-226084 (CR 1916) makes the actual correction to almost contiguous RB allocation definition made in test case 6.2.2. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: -To faciliate specs maintenance, removed RB allocation tables and referred those from test case 6.2.2. |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

#### 6.5.2.2	Spectrum emission mask

Editor’s Note: The following aspects are either missing or not yet determined:

- PC1 requirements for NR operating bands other than Band n14 are not defined in RAN4 Rel-15 and Rel-16 specifications.

6.5.2.2.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth.

6.5.2.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

6.5.2.2.3	Minimum conformance requirements

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the assigned NR channel bandwidth. For frequencies offset greater than ΔfOOB, the spurious requirements in subclause 6.5.3 are applicable.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

The power of any UE emission shall not exceed the levels specified in Table 6.5.2.2.3-1 for the specified channel bandwidth.

Table 6.5.2.2.3-1: General NR spectrum emission mask

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- |
|  | 5 | 10, 15, 20, 25, 30, 40, 45 | 50, 60, 70, 80, 90, 100 |  |
| ± 0-1 | -13 | -13 |  | 1 % of channel BW |
| ± 0-1 |  |  | -24 | 30 kHz |
| ± 1-5 | -10 | -10 |  | 1 MHz |
| ± 5-6 | -13 |  |  |  |
| ± 6-10 | -25 |  |  |  |
| ± 5-BWChannel |  | -13 |  |  |
| ± BWChannel-(BWChannel+5) |  | -25 |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5.2.2

6.5.2.2.4	Test description

6.5.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 6.5.2.2.4.1-1, 6.5.2.2.4.1-2, 6.5.2.2.4.1-2a and Table 6.5.2.2.4.1-3. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Table 6.5.2.2.4.1-1: Test Configuration Table for power class 3 (contiguous allocation)

| Default Conditions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  | Low range, High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | Freq |  | ChBw |  | SCS |  | Downlink Configuration |  |  | Uplink Configuration |  |
|  |  |  | Default |  | Default |  | N/A for Spectrum Emission Mask test case |  |  | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 13 | Low |  |  |  |  |  |  |  |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Left |
| 23 | High |  |  |  |  |  |  |  |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Right |
| 33 | Default |  |  |  |  |  |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 44 | Low |  |  |  |  |  |  |  |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Left |
| 54 | High |  |  |  |  |  |  |  |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Right |
| 64 | Default |  |  |  |  |  |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 7 | Low |  |  |  |  |  |  |  |  | DFT-s-OFDM QPSK | Edger_1RB_Left |
| 8 | High |  |  |  |  |  |  |  |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 9 | Default |  |  |  |  |  |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 10 | Low |  |  |  |  |  |  |  |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  |  |  |  |  |  |  |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  |  |  |  |  |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 13 | Low |  |  |  |  |  |  |  |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  |  |  |  |  |  |  |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  |  |  |  |  |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 16 | Low |  |  |  |  |  |  |  |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  |  |  |  |  |  |  |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  |  |  |  |  |  |  |  | DFT-s-OFDM 256 QAM | Outer_Full |
| 19 | Low |  |  |  |  |  |  |  |  | CP-OFDM QPSK | Edge_1RB_Left |
| 20 | High |  |  |  |  |  |  |  |  | CP-OFDM QPSK | Edge_1RB_Right |
| 21 | Default |  |  |  |  |  |  |  |  | CP-OFDM QPSK | Outer_Full |
| 22 | Low |  |  |  |  |  |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 23 | High |  |  |  |  |  |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 24 | Default |  |  |  |  |  |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 25 | Low |  |  |  |  |  |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 26 | High |  |  |  |  |  |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 27 | Default |  |  |  |  |  |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| 28 | Low |  |  |  |  |  |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 29 | High |  |  |  |  |  |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 30 | Default |  |  |  |  |  |  |  |  | CP-OFDM 256 QAM | Outer_Full |
| 315,6 |  | Low |  |  |  |  |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 325,6 |  | High |  |  |  |  |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 335,6 |  | Default |  |  |  |  |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n40, n41, n77, n78 and n79. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n41, n77, n78 and n79. NOTE 5: For Power Class 3 testing, UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n77, n78 and n79. NOTE 6: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.5.2.2.4.1-2: Test Configuration Table for power class 2 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 2 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 3 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 4 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 5 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 6 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 7 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 8 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 10 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 16 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 17 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 18 | Default |  | CP-OFDM QPSK | Outer Full |
| 19 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 20 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 21 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 22 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 23 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 24 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 25 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 26 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 27 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: It is essential that all test points in this table also exist in table 6.2.2.4.1-2. |  |  |  |  |

Table 6.5.2.2.4.1-2a: Test Configuration Table for power class 1 for Band n14 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Spectrum Emission Mask test case | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 256 QAM | Outer Full |
| 333 | Low |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 343 | High |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 353 | Default |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. NOTE 4: It is essential that all test points in this table also exist in table 6.2.2.4.1-2. |  |  |  |  |

Table 6.5.2.2.4.1-3: Test Configuration Table for power class 2&3 (almost contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 | Default |  | CP-OFDM QPSK | Inner Full |
| 2 | Default |  | CP-OFDM QPSK | Outer Full |
| 3 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 4 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 5 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 6 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.2.2.4.1-4, 6.2.2.4.1-4a and 6.2.2.4.1-4b. NOTE 2: It is essential that all test points in this table also exist in table 6.2.2.4.1-3. NOTE 3: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5.2.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5.2.2.4.3.

6.5.2.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5.2.2.4.1-1, Table 6.5.2.2.4.1-2, Table 6.5.2.2.4.1-2a and Table 6.5.2.2.4.1-3. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously power control “up” commands to the UE until the UE transmits at PUMAX level. Allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2.2.5-1 to 6.2.2.5-9. The period of the measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots. For TDD, only slots consisting of only UL symbols are under test.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.5.2.2.5-1. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

NOTE 1: When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.5.2.2.4.1-1, table 6.5.2.2.4.1-2, and table 6.5.2.2.4.1-2a, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.5.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with the following exceptions:

Table 6.5.2.2.4.3-1: PUSCH-Config

| Derivation Path: TS 38.508-1 [5] subclause 4.6.3 Table 4.6.3-118 PUSCH-Config |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType0 |  | Almost contiguous allocation |
|  | resourceAllocationType1 |  | Contiguous allocation |
| } |  |  |  |

Table 6.5.2.2.4.3-2: DMRS-UplinkConfig (Test ID 28 – 30 in Table 6.5.2.2.4.1-1)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-51 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-UplinkConfig ::= SEQUENCE { |  |  |  |
| transformPrecodingEnabled SEQUENCE { |  |  |  |
| dmrs-UplinkTransformPrecoding-r16 SEQUENCE { |  |  |  |
| pi2BPSK-ScramblingID0 | Not present |  |  |
| pi2BPSK-ScramblingID1 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.5.2.2.5	Test requirement

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Tables 6.2.2.5-1 to 6.2.2.5-9 as appropriate, and the power of any UE emission shall fulfil requirements in Table 6.5.2.2.5-1.

Table 6.5.2.2.5-1: General NR spectrum emission mask

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- |
|  | 5 | 10, 15, 20, 25, 30, 40, 45 | 50, 60, 70, 80, 90, 100 |  |
| ± 0-1 | -13 + TT | -13 + TT |  | 1 % of channel BW |
| ± 0-1 |  |  | -24 + TT | 30 kHz |
| ± 1-5 | -10 + TT | -10 + TT |  | 1 MHz |
| ± 5-6 | -13 + TT |  |  |  |
| ± 6-10 | -25 + TT |  |  |  |
| ± 5-BWChannel |  | -13 + TT |  |  |
| ± BWChannel-(BWChannel+5) |  | -25 + TT |  |  |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel. Note 4: TT for each frequency and channel bandwidth is specified in Table 6.5.2.2.5-2. |  |  |  |  |

Table 6.5.2.2.5-2: Test Tolerance (Spectrum Emission Mask)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 100MHz | 1.5 dB | 1.8 dB | 1.8 dB |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

## <<< END OF CHANGES >>>