3GPP TSG-RAN5 Meeting #97	    R5-226092r1
Toulouse, France, 14th – 18th November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0815 | rev | - | Current version: | 17.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PC1 - MOP test case update in 38.521-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MU and TT analysis has progressed for PC1 devices. Test case needs to be updated accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For 6.2.1.1 and 6.2.1.2: -Editor’s note update and TT table update/addition. -Anex F update: MU and TT definition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain incomple. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.1, 6.2.1.2, F.1.2, F.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: -Corrected current version from 17.0.0 to 17.0.1. |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

### 6.2.1	UE maximum output power

#### 6.2.1.0	General

Note: Power class 1, 2, 3, and 4 are specified based on the assumption of certain UE types with specific device architectures. The UE types can be found in Table 6.2.1.0-1.

Table 6.2.1.0-1: Assumption of UE Types

| UE Power class | UE type |
| --- | --- |
| 1 | Fixed wireless access (FWA) UE |
| 2 | Vehicular UE |
| 3 | Handheld UE |
| 4 | High power non-handheld UE |

#### 6.2.1.1	UE maximum output power - EIRP and TRP

Editor’s note: The following aspects are either missing or not yet determined:

-	Measurement Uncertainties and Test Tolerances are FFS for power class 2 and 4.

-	The test case is incomplete for band n259.

6.2.1.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2.1.1.2	Test applicability

This test case applies to all types of release 15 NR UEs and release 16 NR and forward UEs not supporting either CSI-RS based or SSB-based enhanced beam correspondence.

6.2.1.1.3	Minimum conformance requirements

###### 6.2.1.1.3.1	UE maximum output power for power class 1

The following requirements define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The minimum output power values for EIRP are found in Table 6.2.1.1.3.1-1. The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1.1.3.1-1: UE minimum peak EIRP for power class 1

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 40.0 |
| n258 | 40.0 |
| n260 | 38.0 |
| n261 | 40.0 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance |  |

The maximum output power values for TRP and EIRP are found in Table 6.2.1.1.3.1-2 below. The maximum allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode and EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1.1.3.1-2: UE maximum output power limits for power class 1

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 35 | 55 |
| n258 | 35 | 55 |
| n260 | 35 | 55 |
| n261 | 35 | 55 |

The minimum EIRP at the 85th percentile of the distribution of radiated power measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 6.2.1.1.3.1-3 below. The requirement is verified with the test metric of EIRP (Link=Spherical coverage grid, Meas=Link angle).

Table 6.2.1.1.3.1-3: UE spherical coverage for power class 1

| Operating band | Min EIRP at 85%-tile CDF (dBm) |
| --- | --- |
| n257 | 32.0 |
| n258 | 32.0 |
| n260 | 30.0 |
| n261 | 32.0 |
| NOTE 1: Minimum EIRP at 85%-tile CDF is defined as the lower limit without tolerance NOTE 2: The requirements in this table are verified only under normal temperature conditions as defined in TS 38.508-1 [10] subclause 4.1.1. |  |

###### 6.2.1.1.3.2	UE maximum output power for power class 2

The following requirements define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The minimum output power values for EIRP are found in Table 6.2.1.1.3.2-1. The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1.1.3.2-1: UE minimum peak EIRP for power class 2

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 29 |
| n258 | 29 |
| n261 | 29 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance |  |

The maximum output power values for TRP and EIRP are found in Table 6.2.1.1.3.2-2 below. The maximum allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode and EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1.1.3.2-2: UE maximum output power limits for power class 2

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 23 | 43 |
| n258 | 23 | 43 |
| n261 | 23 | 43 |

The minimum EIRP at the 60th percentile of the distribution of radiated power measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 6.2.1.1.3.2-3 below. The requirement is verified with the test metric of EIRP (Link=Spherical coverage grid, Meas=Link angle).

Table 6.2.1.1.3.2-3: UE spherical coverage for power class 2

| Operating band | Min EIRP at 60%-tile CDF (dBm) |
| --- | --- |
| n257 | 18.0 |
| n258 | 18.0 |
| n261 | 18.0 |
| NOTE 1: Minimum EIRP at 60%-tile CDF is defined as the lower limit without tolerance NOTE 2: The requirements in this table are verified only under normal temperature conditions as defined in TS 38.508-1 [10] subclause 4.1.1. |  |

###### 6.2.1.1.3.3	UE maximum output power for power class 3

The following requirements define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The minimum output power values for EIRP are found in Table 6.2.1.1.3.3-1. The requirement is verified with the test metric of total component of EIRP (Link=TX beam peak direction, Meas=Link angle). The requirement for the UE which supports a single FR2 band is specified in Table 6.2.1.1.3.3-1. The requirement for the UE which supports multiple FR2 bands is specified in both Table 6.2.1.1.3.3-1 and Table 6.2.1.1.3.3-4 or Table 6.2.1.1.3.3-5.

Table 6.2.1.1.3.3-1: UE minimum peak EIRP for power class 3

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 22.4 |
| n258 | 22.4 |
| n259 | 18.7 |
| n260 | 20.6 |
| n261 | 22.4 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance NOTE 2: Void |  |

The maximum output power values for TRP and EIRP are found on the Table 6.2.1.1.3.3-2. The max allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode and the total component of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1.1.3.3-2: UE maximum output power limits for power class 3

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 23 | 43 |
| n258 | 23 | 43 |
| n259 | 23 | 43 |
| n260 | 23 | 43 |
| n261 | 23 | 43 |

The minimum EIRP at the 50th percentile of the distribution of radiated power measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 6.2.1.1.3.3-3 below. The requirement is verified with the test metric of the total component of EIRP, as defined in [5] (Link=Spherical coverage grid, Meas=Link angle). The requirement for the UE which supports a single FR2 band is specified in Table 6.2.1.1.3.3-3. The requirement for the UE which supports multiple FR2 bands is specified in both Table 6.2.1.1.3.3-3 and Table 6.2.1.1.3.3-4 or Table 6.2.1.1.3.3-5.

Table 6.2.1.1.3.3-3: UE spherical coverage for power class 3

| Operating band | Min EIRP at 50t%-tile CDF (dBm) |
| --- | --- |
| n257 | 11.5 |
| n258 | 11.5 |
| n259 | 5.8 |
| n260 | 8 |
| n261 | 11.5 |
| NOTE 1: Minimum EIRP at 50 %-tile CDF is defined as the lower limit without tolerance NOTE 2: Void NOTE 3: The requirements in this table are verified only under normal temperature conditions as defined in TS 38.508-1 [10] subclause 4.1.1. |  |

For the UEs that support multiple FR2 bands, minimum requirement for peak EIRP and EIRP spherical coverage in Tables 6.2.1.1.3.3-1 and 6.2.1.1.3.3-3 shall be decreased per band, respectively, by the peak EIRP relaxation parameter MBP,n and EIRP spherical coverage relaxation parameter MBS,n, as indicated in Table 6.2.1.1.3.3-4 to 6.2.1.1.3.3-5. For Rel-15 UE, each combination of supported bands ΔMBP,n and ΔMBS,n apply to each supported band n, such that the total relaxations, ∑MBP and ∑MBS, across all supported bands shall not exceed the total value indicated in Table 6.2.1.1.3.3-4.

Table 6.2.1.1.3.3-4: UE multi-band relaxation factors for power class 3 (Rel-15)

| Supported bands | ∑MBP (dB) | ∑MBS (dB) |
| --- | --- | --- |
| n257, n258 | ≤ 1.3 | ≤ 1.25 |
| n257, n260 | ≤ 1.03 | ≤ 0.753 |
| n258, n260 | ≤ 1.03 | ≤ 0.753 |
| n258, n261 | ≤ 1.0 | ≤ 1.25 |
| n260, n261 | 0.0 | ≤ 0.752 |
| n257, n261 | 0.0 | 0.0 |
| n257, n258, n260 | ≤ 1.73 | ≤ 1.753 |
| n257, n258, n261 | ≤ 1.7 | ≤ 1.75 |
| n257, n260, n261 | ≤ 0.53 | ≤ 1.253 |
| n258, n260, n261 | ≤ 1.53 | ≤ 1.253 |
| n257, n258, n260, n261 | ≤ 1.73 | ≤ 1.753 |
| NOTE 1: The requirements in this table are applicable to UEs which support only the indicated bands. NOTE 2: For supported bands n260 + n261, ΔMBS,n is not applied for band n260. NOTE 3: For band n260, maximum applicable MBS,n is 0.4 dB and MBP,n is 0.75 dB. NOTE 4: For all bands except n260, the maximum applicable MBP,n and MBS,n is 0.75 dB. |  |  |

Table 6.2.1.1.3.3-5: UE multi-band relaxation factors for power class 3 (Rel-16 and forward)

| Band | MBP,n (dB) | MBS,n (dB) |
| --- | --- | --- |
| n257 | 0.73 | 0.73 |
| n258 | 0.6 | 0.7 |
| n259 | 0.5 | 0.4 |
| n260 | 0.51 | 0.41 |
| n261 | 0.52,4 | 0.74 |
| Note 1: n260 peak and spherical relaxations are 0 dB for UE that exclusively supports n261+n260 Note 2: n261 peak relaxation is 0 dB for UE that exclusively supports n261+n260 Note 3: n257 peak and spherical relaxations are 0 dB for UE that exclusively supports n261+n257 Note 4: n261 peak and spherical relaxations are 0 dB for UE that exclusively supports n261+n257 |  |  |

###### 6.2.1.1.3.4	UE maximum output power for power class 4

The following requirements define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The minimum output power values for EIRP are found in Table 6.2.1.1.3.4-1. The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1.1.3.4-1: UE minimum peak EIRP for power class 4

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 34 |
| n258 | 34 |
| n260 | 31 |
| n261 | 34 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance |  |

The maximum output power values for TRP and EIRP are found in Table 6.2.1.1.3.4-2 below. The maximum allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode and EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1.1.3.4-2: UE maximum output power limits for power class 4

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 23 | 43 |
| n258 | 23 | 43 |
| n260 | 23 | 43 |
| n261 | 23 | 43 |

The minimum EIRP at the 20th percentile of the distribution of radiated power measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 6.2.1.1.3.4-3 below. The requirement is verified with the test metric of EIRP (Link=Spherical coverage grid, Meas=Link angle).

Table 6.2.1.1.3.4-3: UE spherical coverage for power class 4

| Operating band | Min EIRP at 20%-tile CDF (dBm) |
| --- | --- |
| n257 | 25 |
| n258 | 25 |
| n260 | 19 |
| n261 | 25 |
| NOTE 1: Minimum EIRP at 20%-tile CDF is defined as the lower limit without tolerance NOTE 2: The requirements in this table are verified only under normal temperature conditions as defined in TS 38.508-1 [10] subclause 4.1.1. |  |

The normative reference for this requirement is TS 38.101-2 [3] clause 6.2.1.

6.2.1.1.4	Test description

6.2.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.2.1.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.1.1.4.1-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid Range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest, 100 MHz, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120 kHz |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Default | - | Modulation |  | RB allocation (NOTE 1) |
| 1 | 50 |  |  | DFT-s-OFDM QPSK |  | Inner_Full for PC2, PC3 |
| 2 | 100 |  |  |  |  | and PC4 |
| 3 | 200 |  |  |  |  | Inner_Full_Region1 for |
| 4 | 400 |  |  |  |  | PC1 |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: Void |  |  |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement channels are set according to Table 6.2.1.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.2.1.1.4.3

6.2.1.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.1.1.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Messages to configure the appropriate uplink modulation in section 6.2.1.1.4.3.

2.	Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

3.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 msec starting from the first TPC command in this step to ensure that the UE transmits at its maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

4.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

5.	Measure UE EIRP in the Tx beam peak direction in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Tables 6.2.1.1.5-1 to 6.2.1.1.5-4. EIRP test procedure is defined in Annex K.1.3. The measuring duration is one active uplink subframe. EIRP is calculated considering both polarizations, theta and phi.

6.	Measure TRP of the transmitted signal for the assigned NR channel with a rectangular measurement filter with bandwidths according to Table 6.5.2.3.5-1. Total radiated power is measured according to TRP measurement procedure defined in Annex K.1.7 and measurement grid specified in Annex M.4. TRP is calculated considering both polarizations, theta and phi.

7.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.1.

6.2.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6 with TRANSFORM_PRECODER_ENABLED condition in Table 4.6.3-118 PUSCH-Config.

6.2.1.1.5	Test requirement

The EIRP derived in step 5 and TRP derived in step 6 shall not exceed the values specified in Table 6.2.1.1.5-1 to Table 6.2.1.1.5-4.

Table 6.2.1.1.5-1: UE maximum output test requirements for power class 1

| Operating band | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| n257 | 35+TT | 55 | 40.0-TT |
| n258 | 35+TT | 55 | 40.0-TT |
| n260 | 35+TT | 55 | 38.0-TT |
| n261 | 35+TT | 55 | 40.0-TT |

Table 6.2.1.1.5-1a: Test Tolerance (Max TRP for Power class 1)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | 2.78 dB, NTC 2.94 dB, ETC | 2.82 dB, NTC 2.98 dB, ETC |

Table 6.2.1.1.5-1b: Test Tolerance (Min peak EIRP for Power class 1)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | 3,12 dB, NTC 3.28 dB, ETC | 3.12 dB, NTC 3.28 dB, ETC |

Table 6.2.1.1.5-2: UE maximum output test requirements for power class 2

| Operating band | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| n257 | 23+TT | 43 | 29-TT |
| n258 | 23+TT | 43 | 29-TT |
| n260 |  |  |  |
| n261 | 23+TT | 43 | 29-TT |

Table 6.2.1.1.5-3: UE maximum output test requirements for power class 3 for single band UE

| Operating band | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| n257 | 23+TT | 43 | 22.4-TT |
| n258 | 23+TT | 43 | 22.4-TT |
| n260 | 23+TT | 43 | 20.6-TT |
| n261 | 23+TT | 43 | 22.4-TT |

Table 6.2.1.1.5-3a: UE maximum output test requirements for power class 3 for multi-band UE (Rel-15)

| ID | Supported FR2 bands set | Test requirement (dB) (Note 1) |  |  |  | Maximum sum of MBp, ∑MBP (dB) (Note 3) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | n257 | n258 | n260 | n261 |  |  |
| 1 | n257, n258 | 22.4-TT-MBp | 22.4-TT-MBp |  |  | 1.3 | Maximum 0.75 dB relaxation allowed for each band |
| 2 | n257, n260 | 22.4-TT-MBp |  | 20.6-TT-MBp |  | 1.0 | Maximum 0.75 dB relaxation allowed for each band |
| 3 | n258, n260 |  | 22.4-TT-MBp | 20.6-TT-MBp |  | 1.0 | Maximum 0.75 dB relaxation allowed for each band |
| 4 | n258, n261 |  | 22.4-TT-MBp |  | 22.4-TT-MBp | 1.0 | Maximum 0.75 dB relaxation allowed for each band |
| 5 | n260, n261 |  |  | 20.6-TT | 22.4-TT | 0.0 | No relaxation factor allowed |
| 6 | n257, n258, n260 | 22.4-TT-MBp | 22.4-TT-MBp | 20.6-TT-MBp |  | 1.7 | Maximum 0.75 dB relaxation allowed for each band |
| 7 | n257, n258, n261 | 22.4-TT-MBp | 22.4-TT-MBp |  | 22.4-TT-MBp | 1.7 | Maximum 0.75 dB relaxation allowed for each band |
| 8 | n257, n260, n261 | 22.4-TT-MBp |  | 20.6-TT-MBp | 22.4-TT-MBp | 0.5 | Maximum 0.75 dB relaxation allowed for each band |
| 9 | n258, n260, n261 |  | 22.4-TT-MBp | 20.6-TT-MBp | 22.4-TT-MBp | 1.5 | Maximum 0.75 dB relaxation allowed for each band |
| 10 | n257, n258, n260, n261 | 22.4-TT-MBp | 22.4-TT-MBp | 20.6-TT-MBp | 22.4-TT-MBp | 1.7 | Maximum 0.75 dB relaxation allowed for each band |
| 11 | n257, n261 | 22.4-TT |  |  | 22.4-TT | 0.0 | No relaxation factor allowed |
| Note 1: MBp is the Multi-band Relaxation factor declared by the UE for the tested band in table A.4.3.9-2 of TS38.508-2 [11]. This declaration shall fulfil the requirements in Table 6.2.1.1.3.3-4. Note 2: All UE supported bands needs to be tested to ensure the multi-band relaxation declaration is compliant Note 3: Max allowed sum of MBp over all supported FR2 bands as defined in clause 6.2.1.1.3.3. Note 4: For a Rel-15 UE supporting FR2 bands set not defined in Table 6.2.1.1.3.3-4, Table 6.2.1.1.5-3d applies. |  |  |  |  |  |  |  |

Table 6.2.1.1.5-3b: Test Tolerance (Max TRP for Power class 3)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | 2.65 dB, NTC 2.82 dB, ETC | 2.77 dB, NTC 2.94 dB, ETC |

Table 6.2.1.1.5-3c: Test Tolerance (Min peak EIRP for Power class 3)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | 2.87 dB, NTC 3.04 dB, ETC | 2.87 dB, NTC 3.04 dB, ETC |

Table 6.2.1.1.5-3d: UE maximum output test requirements for power class 3 (Rel-16 and forward)

| ID | FR2 bands/set | Test requirement (dB) (Note 1) |  |  |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | n257 | n258 | n259 | n260 | n261 |  |
| 1 | n257 | 22.4-TT-MBP,n |  |  |  |  |  |
| 2 | n258 |  | 22.4-TT-MBP,n |  |  |  |  |
| 3 | n259 |  |  | 18.7-TT-MBP,n |  |  |  |
| 4 | n260 |  |  |  | 20.6-TT-MBP,n |  |  |
| 5 | n261 |  |  |  |  | 22.4-TT-MBP,n |  |
| 6 | n257, n261 | 22.4-TT |  |  |  | 22.4-TT | MBP,n relaxation is 0 dB |
| 7 | n260, n261 |  |  |  | 20.6-TT | 22.4-TT | MBP,n relaxation is 0 dB |
| Note 1: MBP,n is the Multi-band Relaxation factor for the tested band. This shall fulfil the requirements in Table 6.2.1.1.3.3-5. |  |  |  |  |  |  |  |

Table 6.2.1.1.5-4: UE maximum output power test requirements for power class 4

| Operating band | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| n257 | 23+TT | 43 | 34-TT |
| n258 | 23+TT | 43 | 34-TT |
| n260 | 23+TT | 43 | 31-TT |
| n261 | 23+TT | 43 | 34-TT |

## <<< Skip unchanged sections >>>

#### 6.2.1.2	UE maximum output power - Spherical coverage

Editor’s note: The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS for power class  2 and 4.

- The test case is incomplete for band n259.

6.2.1.2.1	Test purpose

To verify that the spatial coverage of the UE in expected directions is acceptable.

6.2.1.2.2	Test applicability

This test case applies to all types of release 15 NR UEs    and release 16 and forward NR UEs not supporting either CSI-RS based or SSB-based enhanced beam correspondence.

6.2.1.2.3	Minimum conformance requirements

Minimum conformance requirements are defined in clause 6.2.1.1.3.

6.2.1.2.4	Test description

6.2.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.2.1.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.1.2.4.1-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid Range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120 kHz |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Default | - | Modulation |  | RB allocation (NOTE 1) |
| 1 | 50 |  |  | DFT-s-OFDM QPSK |  | Inner_Full for PC2, PC3 |
| 2 | 100 |  |  |  |  | and PC4 |
| 3 | 200 |  |  |  |  | Inner_Full_Region1 for |
| 4 | 400 |  |  |  |  | PC1 |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. |  |  |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement channels are set according to Table 6.2.1.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.2.1.2.4.3

6.2.1.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.1.2.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Messages to configure the appropriate uplink modulation in section 6.2.1.2.4.3.

2.	Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

3.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 msec to ensure that the UE transmits at its maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

4.	Through its beam correspondence procedure, DUT refines its TX beam toward that direction depending on DUT’s beam correspondence capability which shall match OEM declaration:

4a 	If the DUT’s beam correspondence capability beamCorrespondenceWithoutUL-BeamSweeping is supported, then DUT autonomously chooses the corresponding TX beam for PUSCH transmission using downlink reference signals to transmit in the direction of the incoming DL signal, which is based on beam correspondence without relying on UL beam sweeping;

4b 	If the DUT’s beam correspondence capability beamCorrespondenceWithoutUL-BeamSweeping is not present, then DUT chooses the TX beam for PUSCH transmission which is based on beam correspondence with relying on both DL measurements on downlink reference signals and network-assisted uplink beam sweeping:

4b.1) DUT uses downlink reference signals to select proper RX beam and uses autonomous beam correspondence to select the TX beam.

4b.2) SS configures M=8 SRS resources to DUT, with the field spatialRelationInfo omitted and the field usage set as ‘beamManagement’. In case DUT supports less than 8 SRS resources, SS configures the number of SRS resources according to the maximum number of SRS resources indicated by UE capability signalling. Additionally, for codebook based PUSCH transmission, SS configures a semi-persistent SRS resource set with the field usage as 'codebook'.

4b.3) Based on the TX beam autonomously selected by DUT, DUT chooses TX beams to transmit SRS-resources configured by SS.

4b.4) Based on measurement of the received beamManagement SRS, SS chooses the best SRS beam and, if needed, updates the spatial relation information between the semi-persistent codebook SRS resources and the SS selected beamManagement SRS resource in the activation MAC CE of the semi-persistent SRS resource. The SS indicates in the SRS Resource Indicator (SRI) field in the scheduling grant for PUSCH, if present, the SRS resource within the semi-persistent SRS resource set whose spatial relation is linked to the best detected SRS beam.

4b.5) DUT transmits PUSCH corresponding to the SRS resource indicated by the SRI.

5.	Measure UE EIRP value for each grid point according to the EIRP spherical coverage procedure defined in Annex K.1.5, and obtain a cumulative distribution function (CDF) of all EIRP dBm values. Alternatively, UE EIRP measurement for each grid point could be done according to Tx Fast spherical coverage procedure defined in Annex K.1.5.1. After a rotation, allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for UE to find the best beam to use. The measuring duration is one active uplink subframe. EIRP is calculated considering both polarizations, theta and phi.

6.	Identify the EIRP dBm value corresponding to %-tile (UE power class dependent) value in the applicable test requirement table in section 6.2.1.2.5.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.1.

6.2.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6 with TRANSFORM_PRECODER_ENABLED condition in Table 4.6.3-118 PUSCH-Config.

6.2.1.2.5	Test requirement

The defined %-tile EIRP in measurement distribution derived in step 5 shall exceed the values specified in Table 6.2.1.2.5-1 to Table 6.2.1.2.5-4.

Table 6.2.1.2.5-1: UE spherical coverage for power class 1

| Operating band | Min EIRP at 85%-tile CDF (dBm) |
| --- | --- |
| n257 | 32.0-TT |
| n258 | 32.0-TT |
| n260 | 30.0-TT |
| n261 | 32.0-TT |

Table 6.2.1.2.5-1b: Test Tolerance (UE spherical coverage for Power class 1)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | 2.69 dB | 2.69 dB |

Table 6.2.1.2.5-2: UE spherical coverage for power class 2

| Operating band | Min EIRP at 60%-tile CDF (dBm) |
| --- | --- |
| n257 | 18.0-TT |
| n258 | 18.0-TT |
| n260 |  |
| n261 | 18.0-TT |

Table 6.2.1.2.5-3: UE spherical coverage for power class 3 for single band UE or multiband UE declaring MBs = 0 in all FR2 bands

| Operating band | Min EIRP at 50t%-tile CDF (dBm) |
| --- | --- |
| n257 | 11.5-TT |
| n258 | 11.5-TT |
| n259 | 5.8-TT |
| n260 | 8-TT |
| n261 | 11.5-TT |

Table 6.2.1.2.5-3a: UE spherical coverage for power class 3 for multi band UE declaring MBs>0 in any FR2 band (Rel-15)

| ID | Supported FR2 bands set | Test requirement (dB) (Note 1) |  |  |  | Maximum sum of MBs, ∑MBs (dB) (Note 3) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | n257 | n258 | n260 | n261 |  |  |
| 1 | n257, n258 | 11.5-TT-MBs | 11.5-TT-MBs |  |  | 1.25 | Maximum 0.75 dB relaxation allowed for each band |
| 2 | n257, n260 | 11.5-TT-MBs |  | 8-TT-MBs |  | 0.75 | Maximum 0.4 dB relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| 3 | n258, n260 |  | 11.5-TT-MBs | 8-TT-MBs |  | 0.75 | Maximum 0.4 dB relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| 4 | n258, n261 |  | 11.5-TT-MBs |  | 11.5-TT-MBs | 1.25 | Maximum 0.75 dB relaxation allowed for each band |
| 5 | n260, n261 |  |  | 8-TT-MBs | 11.5-TT-MBs | 0.75 | No relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| 6 | n257, n258, n260 | 11.5-TT-MBs | 11.5-TT-MBs | 8-TT-MBs |  | 1.75 | Maximum 0.4 dB relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| 7 | n257, n258, n261 | 11.5-TT-MBs | 11.5-TT-MBs |  | 11.5-TT-MBs | 1.75 | Maximum 0.75 dB relaxation allowed for each band |
| 8 | n257, n260, n261 | 11.5-TT-MBs |  | 8-TT-MBs | 11.5-TT-MBs | 1.25 | Maximum 0.4 dB relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| 9 | n258, n260, n261 |  | 11.5-TT-MBs | 8-TT-MBs | 11.5-TT-MBs | 1.25 | Maximum 0.4 dB relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| 10 | n257, n258, n260, n261 | 11.5-TT-MBs | 11.5-TT-MBs | 8-TT-MBs | 11.5-TT-MBs | 1.75 | Maximum 0.4 dB relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| Note 1: MBs is the Multiband Relaxation factor declared by the UE for the tested band in table A.4.3.9-3 of TS38.508-2 [11]. This declaration shall fulfil the requirements in Table 6.2.1.1.3.3-4. Note 2: All UE supported bands needs to be tested to ensure the multiband relaxation declaration is compliant Note 3: Max allowed sum of MBs over all supported FR2 bands as defined in clause 6.2.1.1.3.3. Note 4: For a Rel-15 UE supporting FR2 bands set not defined in Table 6.2.1.1.3.3-4, Table 6.2.1.2.5-3c applies. |  |  |  |  |  |  |  |

Table 6.2.1.2.5-3b: Test Tolerance (UE spherical coverage for Power class 3)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | 2.58 dB | 2.58 dB |

Table 6.2.1.2.5-3c: UE spherical coverage for power class 3 (Rel-16 and forward)

| ID | FR2 bands/set | Test requirement (dB) (Note 1) |  |  |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | n257 | n258 | n259 | n260 | n261 |  |
| 1 | n257 | 11.5-TT-MBs,n |  |  |  |  |  |
| 2 | n258 |  | 11.5-TT-MBs,n |  |  |  |  |
| 3 | n259 |  |  | 5.8-TT-MBs,n |  |  |  |
| 4 | n260 |  |  |  | 8-TT-MBs,n |  |  |
| 5 | n261 |  |  |  |  | 11.5-TT-MBs,n |  |
| 6 | n257, n261 | 11.5-TT-MBs,n |  |  |  | 11.5-TT-MBs,n | MBs,n relaxation is 0 dB |
| 7 | n260, n261 |  |  |  | 8-TT-MBs,n | 11.5-TT-MBs,n | MBs,n relaxation is 0 dB for n260 |
| Note 1: MBs,n is the Multiband Relaxation factor for the tested band. This shall fulfil the requirements in Table 6.2.1.1.3.3-5. |  |  |  |  |  |  |  |

Table 6.2.1.2.5-4: UE spherical coverage for power class 4

| Operating band | Min EIRP at 20%-tile CDF (dBm) |
| --- | --- |
| n257 | 25 |
| n258 | 25 |
| n260 | 19 |
| n261 | 25 |

## <<< Skip unchanged sections >>>

## F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty (MTSU) for transmitter tests

| Sub clause | Maximum Test System Uncertainty | Derivation of MTSU |
| --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) | PC3 Minimum peak EIRP, Max EIRP Max Device size ≤ 30 cm ±4.89 dB (FR2a, NTC testing) ±5.09 dB (FR2b, NTC testing) ±5.17 dB (FR2a, ETC testing) ±5.37 dB (FR2b, ETC testing) PC1 Minimum peak EIRP, Max EIRP Max Device size ≤ 30 cm ±5.33 dB (FR2a, NTC testing) ±5.40 dB (FR2b, NTC testing) ±5.60 dB (FR2a, ETC testing) ±5.67 dB (FR2b, ETC testing) | MTSU = 1.00 x MU (from Table B.3-1 in TR 38.903) |
| 6.2.1.1 UE maximum output power (TRP) | PC3 Max TRP Max Device size ≤ 30 cm ±4.42 dB (FR2a, NTC testing) ±4.62 dB (FR2b, NTC testing) ±4.70 dB (FR2a, ETC testing) ±4.90 dB (FR2b, ETC testing) PC1 Max TRP Max Device size ≤ 30 cm ±4.64 dB (FR2a, NTC testing) ± 4.71 dB (FR2b, NTC testing) ± 4.90 dB (FR2a, ETC testing) ± 4.97 dB (FR2b, ETC testing) | MTSU = 1.00 x MU (from Table B.3-2 in TR 38.903) |
| 6.2.1.1_1 UE maximum output power – EIRP (Rel-16 and forward) | Same as 6.2.1.1 |  |
| 6.2.1.2 UE maximum output power (Spherical coverage) | PC3 Max Device size ≤ 30 cm ±4.60 dB (FR2a) ±5.20 dB (FR2b) PC1 Max Device size ≤ 30 cm ±4.69 dB (FR2a) ±4.84 dB (FR2b) | MTSU = 1.00 x MU (from Table B.3-3 in TR 38.903) |
| 6.2.1.2_1 UE maximum output power – Spherical coverage (Rel16 and forward) | Same as 6.2.1.2 |  |
| 6.2.2 UE maximum output power reduction | PC3 Max Device size ≤ 30 cm ±4.92 dB (FR2a), NTC testing ±5.10 dB (FR2b), NTC testing ±5.20 dB (FR2a, ETC testing) ±5.38 dB (FR2b, ETC testing) | MTSU = 1.00 x MU (from Table B.4-1 in TR 38.903) |
| 6.2.2_1 UE maximum output power reduction enhancements | Same as 6.2.2 |  |
| 6.2.3 UE maximum output power with additional requirements | Same as 6.2.2 |  |
| 6.2.4 Configured transmitted power | TBD |  |
| 6.2.4_1 Configured transmitted power with Power Boost | Same as 6.2.1.1 |  |

## <<< Skip unchanged sections >>>

## F.3.2	Measurement of transmitter

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Influence of noise is subtracted from MTSU before calculating the TT for lower limit Tx test cases.

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) | PC3 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 2.87 dB (FR2a, NTC) 2.87 dB (FR2b, NTC) 3.04 dB (FR2a, ETC) 3.04 dB (FR2b, ETC) PC1 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 3.12 dB (FR2a, NTC) 3.12 dB (FR2b, NTC) 3.28 dB (FR2a, ETC) 3.28 dB (FR2b, ETC) Max EIRP 0 dB | PC3 Minimum peak EIRP TT = 0.60 x (MTSUIFF - 0.1) (FR2a) TT = 0.60 x (MTSUIFF - 0.3) (FR2b) PC1 Minimum peak EIRP TT = 0.60 x (MTSUIFF – 0.13) (FR2a) TT = 0.60 x (MTSUIFF – 0.20) (FR2b) |
| 6.2.1.1 UE maximum output power (TRP) | PC3 Max TRP IFF (Max Device size ≤ 30 cm) 2.65 dB (FR2a, NTC) 2.77 dB (FR2b, NTC) 2.82 dB (FR2a, ETC) 2.94 dB (FR2b, ETC) PC1 Max TRP IFF (Max Device size ≤ 30 cm) 2.78 dB (FR2a, NTC) 2.82 dB (FR2b, NTC) 2.94 dB (FR2a, ETC) 2.98 dB (FR2b, ETC) | Max TRP TT = 0.60 x MTSUIFF |
| 6.2.1.1_1 UE maximum output power – EIRP (Rel-16 and forward) | Same as 6.2.1.1 |  |
| 6.2.1.2 UE maximum output power (Spherical coverage) | PC1 IFF (Max Device size ≤ 30 cm) 2.69 dB (FR2a) 2.69 dB (FR2b) PC2 TBD PC3 IFF (Max Device size ≤ 30 cm) 2.58 dB (FR2a) 2.58 dB (FR2b) PC4 TBD | PC3 TT = 0.60 x (MTSUIFF - 0.3) (FR2a) TT = 0.60 x (MTSUIFF - 0.9) (FR2b) PC1 TT = 0.60 x (MTSUIFF - 0.20) (FR2a) TT = 0.60 x (MTSUIFF - 0.35) (FR2b) |
| 6.2.1.2_1 UE maximum output power – Spherical coverage (Rel16 and forward) | Same as 6.2.1.2 |  |
| 6.2.2 UE maximum output power reduction | PC3 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 3.11 dB (FR2a, NTC) 3.11 dB (FR2b, NTC) 3.30 dB (FR2a, ETC) 3.30 dB (FR2b, ETC) | Minimum peak EIRP TT = 0.65 x (MTSUIFF - 0.13) (FR2a) TT = 0.65 x (MTSUIFF - 0.31) (FR2b) |
| 6.2.2_1 UE maximum output power reduction enhancements | Same as 6.2.2 |  |

## <<< END OF CHANGES >>>