3GPP TSG-RAN WG5 Meeting #97	R5-226349r1

Toulouse, France, Nov 14th - 18th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2604 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update IE ServingCellConfigCommonSIB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR1_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Rel-17 HST enh relative IEs and conditions have been missing in Table 4.6.3-169 and need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Rel-17 HST enh relative IEs and conditions have been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE conformance test for Rel-17 HST enh can not be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1:1. As per description in 38.306, a UE supporting R17 HST enh should support R16 HST enh first. Thus, adding R17 HST condition into IE highSpeedConfig-r16 in Table 4.6.3-169. 2.Changing the format of IE highSpeedConfig-v1700 to align with other IEs in Table 4.6.3-169. |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

4.6.3	Radio resource control information elements

<<<UNCHANGED PARTS SKIPPED >>>

–	ServingCellConfigCommonSIB

Table 4.6.3-169: ServingCellConfigCommonSIB

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB |  |  |
| uplinkConfigCommon | UplinkConfigCommonSIB |  |  |
| supplementaryUplink | Not present |  |  |
|  | UplinkConfigCommonSIB with condition SUL_SUL |  | SUL |
| n-TimingAdvanceOffset | Not present |  |  |
| ssb-PositionsInBurst SEQUENCE { |  |  |  |
| inOneGroup | ’0100 0000’B | When carrier frequency is smaller than or equal to 3 GHz, only the 4 leftmost bits are valid | SSB#1 |
|  | ’1000 0000’B |  | SSB#0 |
| groupPresence | Not present |  | FR1 |
|  | ’1000 0000’B |  | FR2 |
| } |  |  |  |
| ssb-PeriodicityServingCell | ms20 |  |  |
| tdd-UL-DL-ConfigurationCommon | TDD-UL-DL-ConfigCommon |  | TDD |
|  | Not present |  | FDD |
| ss-PBCH-BlockPower | 0 |  |  |
| channelAccessMode-r16 | Not present |  |  |
| channelAccessMode-r16 CHOICE { |  |  | SharedSpectrum |
| dynamic | NULL |  |  |
| } |  |  |  |
| discoveryBurstWindowLength-r16 | Not present |  |  |
| highSpeedConfig-r16 | Not present |  |  |
|  | HighSpeedConfig-r16 |  | R16 HST OR R17 HST |
| highSpeedConfig-v1700 | Not present |  |  |
|  | HighSpeedConfig-v1700 |  | R17 HST |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SUL | Supplementary uplink |
| SSB#N | Cell configured with SSB-Index set to N as defined in Table 4.4.2-2 |
| SharedSpectrum | Operation with shared spectrum channel access |
| R16 HST | For R16 HST test |
| R17 HST | For R17 HST test |

<< END OF CHANGES >>