3GPP TSG-RAN5 Meeting #97	R5-226371r1

# Toulouse, France, Nov 14th - 18th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2609 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of information elements for slice specific RACH configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_slice-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-3 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The information elements related to Rel-17 slice specific two-step RACH configuration is missing in TS 38.508-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added following new information elements for slice specific RACH configuration Table 4.6.3-84BA: NSAG-ID - Updated following information elements to add slice specific two-step RACH configuration - Table 4.6.3-14: BWP-UplinkCommon - Table 4.6.3-84B: NSAG-IdentityInfo - Table 4.6.3-128A: RACH-ConfigCommonTwoStepRA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Table 4.6.3-84BA: NSAG-ID has been updated to merge the contents of R5-226258 and Ericsson has been added as co-source company. Table 4.6.3-14: BWP-UplinkCommon has been updated to add Value/remark. Remove trackingAreaCode-r17 from Table 4.6.3-84B: NSAG-IdentityInfo. |  |  |  |  |  |  |  |  |

<Start of modified section>

4.6.3	Radio resource control information elements

< Unchanged sections omitted >

–	BWP-UplinkCommon

Table 4.6.3-14: BWP-UplinkCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| genericParameters | BWP |  |  |
| rach-ConfigCommon CHOICE { |  |  |  |
| setup | RACH-ConfigCommon |  |  |
| } |  |  |  |
| rach-ConfigCommon | Not present |  | SUL_SUL AND (RF OR RRM), RF AND SCell_Add |
| pusch-ConfigCommon CHOICE { |  |  |  |
| setup | PUSCH-ConfigCommon |  |  |
| } |  |  |  |
| pucch-ConfigCommon CHOICE { |  |  |  |
| setup | PUCCH-ConfigCommon |  |  |
| } |  |  |  |
| pucch-ConfigCommon | Not present |  | RF AND SCell_Add |
| enableRA-PrioritizationForSlicing-r17 | Not present |  |  |
|  | true |  | Slice_RACH |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SUL_SUL | On the SUL carrier when supplementary carrier is configured |
| SCell_Add | Add SCell |
| Slice_RACH | Slice specific RACH configuration |

< Unchanged sections omitted >

–	NSAG-ID

Table 4.6.3-84BA: NSAG-ID

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NSAG-ID-r17 | FFS |  |  |

–	NSAG-IdentityInfo

Table 4.6.3-84B: NSAG-IdentityInfo

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NSAG-IdentityInfo-r17 ::= SEQUENCE { |  |  |  |
| nsag-ID-r17 | NSAG-ID-r17 |  |  |
| } |  |  |  |

< Unchanged sections omitted >

–	RACH-ConfigCommonTwoStepRA

Table 4.6.3-128A: RACH-ConfigCommonTwoStepRA

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommonTwoStepRA-r16 ::= SEQUENCE { |  |  |  |
| rach-ConfigGenericTwoStepRA-r16 | RACH-ConfigGenericTwoStepRA |  |  |
| msgA-TotalNumberOfRA-Preambles-r16 | Not present |  |  |
| msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB-r16 CHOICE { | Not present |  |  |
| one | n8 |  | FR1 |
|  | n4 |  | FR2 |
| } |  |  |  |
| msgA-CB-PreamblesPerSSB-PerSharedRO-r16 | Not present |  |  |
| msgA-SSB-SharedRO-MaskIndex-r16 | Not present |  |  |
| groupB-ConfiguredTwoStepRA-r16 | Not present |  |  |
| msgA-PRACH-RootSequenceIndex-r16 CHOICE {} | Not present |  |  |
| msgA-TransMax-r16 | N8 |  |  |
| msgA-RSRP-Threshold-r16 | RSRP-Range |  |  |
| msgA-RSRP-ThresholdSSB-r16 | RSRP-Range |  |  |
| msgA-SubcarrierSpacing-r16 | Not present |  |  |
| msgA-RestrictedSetConfig-r16 | Not present |  |  |
| ra-PrioritizationForAccessIdentityTwoStep-r16 SEQUENCE { |  |  |  |
| ra-Prioritization-r16 | RA-Prioritization |  |  |
| ra-PrioritizationForAI-r16 | '10'B |  |  |
| } |  |  |  |
| ra-ContentionResolutionTimer-r16 | sf32 |  |  |
| ra-PrioritizationForSlicingTwoStep-r17 | Not present |  |  |
|  | RA-PrioritizationForSlicing-r17 |  | Slice_RACH |
| featureCombinationPreamblesList-r17 SEQUENCE (SIZE(1..maxFeatureCombPreamblesPerRACHResource-r17)) OF FeatureCombinationPreambles-r17{} | Not present |  |  |
|  | FeatureCombinationPreambles-r17 |  | Slice_RACH |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Slice_RACH | Slice specific RACH configuration |

<End of modified section>