3GPP TSG-RAN WG5 Meeting # 97	R5-226417r1

# Toulouse, France, November 14 - 18, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 0682 | rev |  | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update TP analysis for reference sensitivity frequency selections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Adjust selected frequencies for avoiding reference sensitivity exception. Correct DL frequency selection of UL homonic interference due to Note 3 for DC_2A_n77A and DC_66A_n77A |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update frequency selections in Table D.2.8-1: Test frequency selection per band pair for exceptions avoidable by test frequency setting (UL harmonics, Rx mixing, cross band isolation) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Selected frequencies for avoiding reference sensitivity exception is outside the availabe range of US band n77 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | D.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.521-3 CR 1453 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: Correct the work item code on title page |  |  |  |  |  |  |  |  |

##### <Start of Changes>

## D.2.8	Test Frequencies selections for EN-DC

In E-UTRA DL CA and UL CA reference sensitivity testing, Low range and High range are selected for intra-band CA testing. Mid-range is selected for inter band E-UTRA CA testing.

In standalone NR CA reference sensitivity testing, Low and High Range are tested as default for intra-band and Mid-range for inter-band. For CA combinations containing intra-band configuration in an inter-band CA configuration, low and high range are tested for intra-band CA and mid channel for inter-band without intra-band component.

The same principle shall apply for EN-DC testing.

For EN-DC configurations affected by exceptions, the test frequency cannot be freely chosen. One test frequency per exception requirement is sufficient to test the requirement. This is indicated in Table D.2.8-1 following the frequency relation formulas defined in clause D.2.3. Exceptions in TS 38.101-3 clause 7.3A.5 (2UL intermodulation) are not specified since the test frequency and channel bandwidth is already specified in TS 38.101-1.

Table D.2.8-1: Test frequency selection per band pair for exceptions avoidable by test frequency setting (UL harmonics, Rx mixing, cross band isolation)

| Band set (Note 2) | Frequency | Channel BW [MHz] | Exception type | Comments |
| --- | --- | --- | --- | --- |
| 1A / n3A | Mid/Mid | Highest/Highest | - | Non-Exception. |
|  | Low/High | 20 / 40 | Cross band isolation | Exception in TS 38.101-3 table 7.3B.2.3.4-1 |
| 2A/71A | DL1980 MHz/ UL665.5 MHz | 20/5 | UL Harmonic Interference | Exception Note 11 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | DL1980 MHz/ UL673 MHz | 20/10 | UL Harmonic Interference | Exception Note 12 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | UL1881 MHz/UL Low | 20/20 | Receiver Harmonic Mixing | Exception Note 4 in TS 38.101-3 Table 7.3B.2.3.2-1 |
| 2A/n77A | UL 1860 MHz / DL 3720 MHz | 20/100 | UL Harmonic Interference | Exception Note 2, 13 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | UL 1860 MHz / DL 3690 MHz | 20/20 | UL Harmonic Interference | Exception Note 3 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | DL Mid (1960) / UL 3920 | 20/100 | Receiver harmonic mixing HM2 | Receiver Harmonic Mixing Exception in TS38.101-3 Table 7.3B.2.3.2-1 |
|  | UL 1860 MHz / DL 3850 MHz | 20/100 |  | HD avoid |
|  | DL Mid / UL Mid | 20/100 |  | HM avoid |
| 2A/78A | DL 3740 MHz/ UL 1870 MHZ | Highest (20 MHz) / Highest (100 MHz) | UL Harmonic Interference HD2 | Exception Note 2, 13 of TS38.101-3 table 7.3B.2.3.1-1 |
|  | DL 3740 MHz/ UL 1885 MHz | 20 MHz) / Highest (20 MHz) | UL Harmonic Interference HD2 | Exception Note 3 of TS38.101-3 table 7.3B.2.3.1-1 |
| 3A / n1A | Mid/Mid | Highest/Highest | - | Non-Exception. |
|  | High/Low | 20 / 20 | Cross band isolation | Exception in TS 38.101-3 table 7.3B.2.3.4-1 |
| 3A / n41A |  |  |  | CMCC CR R5-205929 |
| 3A / n78A | Mid/High | Highest/Highest | - | Non-Exception. |
|  | Mid/3495 | Highest/Highest | UL Harmonic Interference | Exception Note 2 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | Mid/3525 | 20/20 | UL Harmonic Interference | Exception Note 3 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | Mid /3685 | 20/20 | Receiver Harmonic Mixing | Receiver Harmonic Mixing Exception in TS 38.101-3 Table 7.3B.2.3.2-1 |
| 5A/n77A | UL 829 MHz / DL 3316 MHz | 10/100 | UL Harmonic Interference | Exception Note 6, 7, 17 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | UL 829 MHz / DL 4145 MHz | 10/100 | UL Harmonic Interference | Exception Note 4, 5, 17 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | UL Mid/DL Mid | 10/100 |  | HD avoid |
| 7A / n78A | High/Low | 20/100 | Cross band isolation | Exception in TS 38.101-3 table 7.3B.2.3.4-1 |
| 8A / n77A | Mid (UL 897.5 MHz) / 3590 | Highest (10 MHz) / Highest (100 MHz) | UL Harmonic Interference HD4 | Exception Note 7 of TS38.101-3 table 7.3B.2.3.1-1 |
|  | Mid (UL 897.5 MHz) / 3520 | Highest (10 MHz) / Highest (100 MHz) |  | Non-Exception. Not overlapping interference since BWINT=40 MHz, FINT = (40+100)/2 = 70 |
| 11A / n79A | Mid (DL 1485.9 MHz) / 4457.7 | Highest (10 MHz) / Highest (100 MHz) | Receiver Harmonic Mixing HM3 | Receiver Harmonic Mixing Exception in TS38.101-3 Table 7.3B.2.3.2-1 |
|  | Mid (DL 1485.9 MHz) / 4512.7 MHz | Highest (10 MHz) / Highest (100 MHz) |  | Non-Exception. Not overlapping interference since BWINT=100 MHz, FINT = (100+10)/2 = 55 |
| 13A/n77A | UL 782 MHz / DL 3910 MHz | 10/100 | UL Harmonic Interference | Exception Note 4, 5 of TS 38.101-3 table 7.3B.2.3.1-1 |
|  | DL Mid (751) / UL 3755 | 10/100 | Receiver harmonic mixing | Receiver Harmonic Mixing Exception in TS38.101-3 Table 7.3B.2.3.2-1 |
|  | UL 782 MHz /DL Mid | 10/100 |  | HD avoid |
|  | DL Mid / UL 3850 | 10/100 |  | HM avoid |
| 19A / n79A | DL 884 MHz / Low (4420.02 MHz) | 10 MHz / 40 MHz | Receiver Harmonic Mixing HM3 | Receiver Harmonic Mixing Exception in TS38.101-3 Table 7.3B.2.3.2-1 |
|  | DL 884 MHz / Low (4445.02 MHz) | 10 MHz / 40 MHz |  | Non-Exception. Not overlapping interference since BWINT=40 MHz, FINT = (40+10)/2 = 25 |
| 20A / n78A | Mid/Mid | Highest/Highest | - | Non-Exception. |
|  | Mid/3388 | Highest/Highest | UL Harmonic Interference | Exception Note 7 of TS 38.101-3 table 7.3B.2.3.1-1 |
| 26A / n41A | High (UL 841.5 MHz) / 2524.5 MHz) | Highest (15 MHz) / 50 MHz | UL Harmonic Interference HD3 | Exception Note 9 of TS38.101-3 table 7.3B.2.3.1-1 |
|  | High (UL 841.5 MHz) / 2572 MHz | Highest (15 MHz) / 50 MHz |  | Non-Exception. Not overlapping interference since BWINT=45 MHz, FINT = (45+50)/2 = 47.5 |
| 26A / n77A 26A / n78A | High (UL 841.5 MHz) / 3366 MHz | Highest (15 MHz) / Highest (100 MHz) | UL Harmonic Interference HD4 | Exception Note 7 of TS38.101-3 table 7.3B.2.3.1-1 |
|  | High (UL 841.5 MHz) / 3446 MHz | Highest (15 MHz) / Highest (100 MHz) |  | Non-Exception. Not overlapping interference since BWINT=60 MHz, FINT = (60+100)/2 = 80 |
| 26A / n79A | High (DL 886.5 MHz) / 4432.5 MHz) | Highest (15 MHz) / 60 MHz | Receiver Harmonic Mixing HM5 | Receiver Harmonic Mixing Exception in TS38.101-3 Table 7.3B.2.3.2-1 |
|  | High (DL 886.5 MHz) / 4470.5 MHz) | Highest (15 MHz) / 60 MHz |  | Non-Exception. Not overlapping interference since BWINT=60 MHz, FINT = (60+15)/2 = 37.5 |
| 40A / n78A | Low/Mid | Highest/Highest | - | Non-Exception. |
|  | Mid/3525 | 20/20 | Receiver Harmonic Mixing | Receiver Harmonic Mixing Exception Note 8 in TS38.101-3 Table 7.3B.2.3.2-1 |
| 41A / n77A 41A / n78A | Low (DL 2593 MHz) / 3750 MHz) | Highest (20 MHz) / Highest (100 MHz) | Receiver Harmonic Mixing HM4 | Receiver Harmonic Mixing Exception in TS38.101-3 Table 7.3B.2.3.2-1 |
|  | Mid (DL 2593 MHz) / 3974.5 MHz) | Highest (20 MHz) / Highest (100 MHz) | Cross band isolation | Not overlapping HM interference since BWINT=240 MHz, FINT = (240+100)/2 = 170 |
|  | Mid / Mid | Highest / Highest | - | Non-Exception only possible with 1UL in band 41 |
| 48A/n66A | DL 3557,6 MHz/ UL 1778.8 MHZ | Highest (20 MHz) / Highest (400 MHz) | UL Harmonic Interference HD2 | Exception Note 2, 13 of TS38.101-3 table 7.3B.2.3.1-1 |
|  | DL 3581.0 MHz/ UL 1777.5 MHz | Highest (20 MHz) / Highest (400 MHz) | UL Harmonic Interference HD2 | Exception Note 3 of TS38.101-3 table 7.3B.2.3.1-1 |
| 66A/n77A | UL 1720 MHz / DL 3440 MHz | 20/100 | UL Harmonic Interference | Exception Note 2, 13 of TS38.101-3 table 7.3B.2.3.1-1 |
|  | UL 1720 MHz / DL 3420 MHz | 20/20 | UL Harmonic Interference | Exception Note 3 of TS38.101-3 table 7.3B.2.3.1-1 |
|  | UL Mid / DL Mid | 20/100 |  | HD avoid |
| 66A/n78A | DL 3510 MHz/ UL 1755 MHZ | Highest (20 MHz) / Highest (100 MHz) | UL Harmonic Interference HD2 | Exception Note 2, 13 of TS38.101-3 table 7.3B.2.3.1-1 |
|  | DL 3480 MHz/ UL 1755 MHz | Highest (20 MHz) / Highest (100 MHz) | UL Harmonic Interference HD2 | Exception Note 3 of TS38.101-3 table 7.3B.2.3.1-1 |
| NOTE 1: This selection is used in test case 7.3B.2.3 unless otherwise stated NOTE 2: Aggressor band in bold |  |  |  |  |  |

For EN-DC configurations affected by IMD exceptions where the exception is avoided, the test frequency and bandwidth are FFS.

##### <End of Changes>