3GPP TSG RAN WG5 Meeting #97	R5-226531r1

Toulouse, France, 14th – 18th November 2022

Source:		Rohde & Schwarz

Title:	Discussion on improvement of permitted methods

Agenda Item:		5.3.48.1

Document for:	Approval

# Introduction

In this paper we present our analysis on the improvement of existing permitted methods for FR2 to remove the test requirement relaxations that are currently present in the FR2 test specification TS 38.521-2 [3].

# Discussion

As shown in [1], a standard test system may look like this:

Radio Communication Tester

Remote Radio Head(s)

Signal Conditioning

OTA Chamber

Figure 1: High level system diagram for FR2 conformance testing

The intermediate step (i.e. signal conditioning) becomes the critical piece defining most of the dynamic range / SNR limitations driving the testability issues documented so far [reference to LS]. This “signal conditioning” may comprise elements like switches, combiners, power amplifiers / LNAs, etc., and thus they become an expensive and complex piece of equipment, having many drawbacks (RF noise, heat, etc.) in case discrete RF components are used and connected by cables.

As shown in [1], the natural evolution of such “signal conditioning” boxes is the usage of highly integrated circuits. With this approach and based on state-of-the-art components, the overall test system dynamic range / SNR could be improved and several relaxations could be either improved or removed completely.

RAN4 thus has identified the following improvements for existing permitted methods from TS 38.810 in TS 38.884 [2] as follows:

Table 5.1.6-1: Summary of potential improvement of permitted methods by Tx test case (24.25 – 43.5 GHz)

| Clause | Requirement | Testability issue | Test Metric | Regulatory related | TS 38.521-2 Test Requirements | Potential improvement |
| --- | --- | --- | --- | --- | --- | --- |
| 6.3.1 | Minimum output power | Low UL power | EIRP (Link=TX beam peak direction, Meas=Link angle). | No | No relaxation for PC1. For other power classes, relaxation varies from 0dB to 13.5dB depending on the operating band and channel bandwidth. | Improvements remove required relaxations from TC |
| 6.3.2 | Transmit OFF power | Low UL power | TRP (Link=TX beam peak direction, Meas=TRP grid) | Yes | Relaxations for n257: 21.4dB @ 50MHz, 24.4dB @ 100MHz, 27.4dB @ 200MHz and 30.4dB @ 400MHz. Relaxations for n258 and n261: [21.4]dB @ 50MHz, [24.4]dB @ 100MHz, [27.4]dB @ 200MHz and [30.4]dB @ 400MHz. Relaxations for n260: [24.1]dB @ 50MHz, [27.1]dB @ 100MHz, [30.1]dB @ 200MHz and [33.1]dB @ 400MHz. | ~ 10dB for FR2a and FR2b |
| 6.5.1 | Occupied bandwidth | Low UL power | OBW (Link=TX beam peak direction, Meas=Link angle) | Yes | No relaxations for FR2a and FR2b | N/A for FR2a and FR2b |
| 6.5.2.3 | Adjacent channel leakage ratio | Low UL power | TRP (Link=TX beam peak direction, Meas=TRP grid). | Yes | Relaxation for n257, n258 and n261: 0dB, except for 200MHz (1.5dB in two test IDs) and 400MHz (between 0 and 5.5dB) | Improvements remove required relaxations from TC TC coverage is extended for FR2b |
| 6.5.3.2 | Additional spurious emissions | Low UL power | TRP (Link=TX beam peak direction, Meas=TRP grid). | Yes | Between 0.3dB and 13dB relaxation depending on the combination of NR Band and Protected band. | TBD |

Table 5.1.6-2: Summary of potential improvement of permitted methods by Rx test case (24.25 – 43.5 GHz)

| Clause | Requirement | Testability issue | Test Metric | Regulatory related | TS 38.521-2 Test Requirements | Potential improvement |
| --- | --- | --- | --- | --- | --- | --- |
| 7.4 | Maximum input power | High DL power | EIS (Link=RX beam peak direction, Meas=Link angle). | No | 26dB relaxation for 24.25 ~ 29.5 GHz and 34 dB relaxation for 37 ~ 40 GHz with respect to minimun requirements. | ~ 12dB for FR2a ~16dB for FR2b |
| 7.5 | Adjacent channel selectivity (case 1) | High DL power | EIS (Link=RX beam peak direction, Meas=Link angle) | Yes | 50MHz: 1.8dB relaxation for power in transmission BW and interferer for band n260. 100MHz: 4.8dB relaxation for power in transmission BW and interferer for band n260. 200MHz and 400MHz are deemed not testable. | Similar improvements as for TC 7.4 Improvements remove required relaxations from TC |
| 7.5 | Adjacent channel selectivity (case 2) | High DL power | EIS (Link=RX beam peak direction, Meas=Link angle) | No | Decision not test ACS case 2. | Interferer need ~ 15-22dB relaxation |
| 7.6.2 | In-band blocking | High DL power | EIS (Link=RX beam peak direction, Meas=Link angle) | Yes | 50MHz: 1.8dB relaxation for power in transmission BW and interferer for band n260. 100MHz: 4.8dB relaxation for power in transmission BW and interferer for band n260. 200MHz and 400MHz are deemed not testable. | Similar improvements as for TC 7.4 Improvements remove required relaxations from TC |
| 7.9 | Receiver spurious emissions | Low UL power | TRP (Link=TX beam peak direction, Meas=TRP grid). | Yes | Relaxations for n257, n258, n260, and n261: 10.2dB between 6-20GHz, 17.2dB between 20-40GHz and 33.1dB between 40GHz and the 2nd harmonic. Relaxations for other bands are still TBD. | TBD |

In this contribution, we analyse and provide updates on the achievable improvements for the yellow highlighted in-band TCs, with a special focus on FR2a and FR2b since RAN5 has not yet discussed testability issues for frequencies larger than 40 GHz (i.e. band n259) yet.

The results presented in the following sections are based on simulations and measurements of a test system supporting single carrier, as well as CA configurations (inter- and intra-band) currently defined up to Rel-17. The analysis is valid for the SA TCs above as well as the corresponding EN-DC where an LTE carrier is needed, since the presence of the LTE anchor does not affect the FR2 performance.

## Minimum Output Power

For the minimum Output power TC TS 38.521-2 [3] currently has the following Test Requirements:

As can be seen from the table, an influence of noise value of 1.0 dB is currently required, which is used in the MTSU calculation for the TC. Based on this assumption, it is feasible to remove completely the relaxations mentioned in the table above and make this TC testable without any relaxations.

Observation 1: The relaxations for the Minimum Output Power TC can be removed up to 400MHz BW with the improvements to permitted methods.

## Off Power

For the Off Power TC TS 38.521-2 [3] currently defines the following test requirements:

Table 6.3.2.5-1: Transmit OFF power

| Operating band | Channel bandwidth / Transmit OFF power (dBm) / measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n2572 | -35+21.4 | -35+24.4 | -35+27.4 | -35+30.4 |
|  | 47.58 MHz | 95.16 MHz | 190.20 MHz | 380.28 MHz |
| n258, n261 | -35+[21.4] | -35+[24.4] | -35+[27.4] | -35+[30.4] |
|  | 47.58 MHz | 95.16 MHz | 190.20 MHz | 380.28 MHz |
| n260 | -35+[24.1] | -35+[27.1] | -35+[30.1] | -35+[33.1] |
|  | 47.58 MHz | 95.16 MHz | 190.20 MHz | 380.28 MHz |
| NOTE 1: Core requirement cannot be tested due to testability issue and test requirement includes relaxation to achieve impact from test system noise to measurement result = 1.0 dB (Minimum requirement + relaxation). NOTE 2: Relaxed n257 test requirement is testable for PC3 and PC1. |  |  |  |  |

Since for this TC a very low power needs to be measured similar to the Minimum Output Power TC, the Noise Floor of the test system will also be very similar, thus a reduction of relaxations is possible in a similar way as for the Minimum Output Power TC.

Improved Table 6.3.2.5-1: Transmit OFF power

| Operating band | Channel bandwidth / Transmit OFF power (dBm) / measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n2572 | -35+13 | -35+16.4 | -35+19.4 | -35+22 |
|  | 47.58 MHz | 95.16 MHz | 190.20 MHz | 380.28 MHz |
| n258, n261 | -35+[13] | -35+[16.4] | -35+[19.4] | -35+[22] |
|  | 47.58 MHz | 95.16 MHz | 190.20 MHz | 380.28 MHz |
| n260 | -35+[10.6] | -35+[13.6] | -35+[16.6] | -35+[19.6] |
|  | 47.58 MHz | 95.16 MHz | 190.20 MHz | 380.28 MHz |
| NOTE 1: Core requirement cannot be tested due to testability issue and test requirement includes relaxation to achieve impact from test system noise to measurement result = 1.0 dB (Minimum requirement + relaxation). NOTE 2: Relaxed n257 test requirement is testable for PC3 and PC1. |  |  |  |  |

Further improvements of the relaxations need to be investigated during the course of the work in RAN.5

Observation 2: The relaxations for the Off Power TC can be reduced by at least 8.4 dB in FR2a and 11.4 dB in FR2b with the improvements to permitted methods, assuming a similar noise floor as in the Minimum Output Power case.

## Occupied Bandwidth

Since no relaxations are defined in the specification for this TC, it should be removed list of TCs to be investigated for testability improvements.

Observation 3: No relaxations are required for the occupied bandwidth TC.

## Adjacent channel leakage ratio

ACLR currently requires relaxations due to the high MPR applied in some test points. From the table below, the required relaxations can be seen and it should be noted that according to the test configuration table, the highlighted test points are not tested in FR2b, since they were deemed not feasible there.

Table 6.5.2.3.5-1b: Relaxation due to testability limit (Adjacent channel leakage ratio)

|  |  | Channel bandwidth / NRACLR / Measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Test ID | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| NRACLR for band n257, n258, n261 | 1-6 | 0 | 0 | 0 | 0 |
|  | 7 | 0 | 0 | 0 | 2.5 |
|  | 8 | 0 | 0 | 0 | 2.5 |
|  | 9 | 0 | 0 | 0 | 2.5 |
|  | 10 | 0 | 0 | 1.5 | 5.5 |
|  | 11 | 0 | 0 | 1.5 | 5.5 |
|  | 12 | 0 | 0 | 1.5 | 5.5 |
|  | 13 | 0 | 0 | 0 | 3 |
|  | 14 | 0 | 0 | 0 | 3 |
|  | 15 | 0 | 0 | 0 | 3 |
| NOTE 1: Relaxation value is derived by Table 6.5.2.3.5-1c for FR2a. NOTE 2: Relaxation value is 0 for FR2b. |  |  |  |  |  |

According to TS 38.903 the following influence of Noise has been assumed in deriving the measurement uncertainty for the TC:

Table B.17.2-4: Influence of noise measurement (f=23.45GHz, 32.125GHz, 40.8GHz, Quiet Zone size ≤ 30 cm) for PC3 UEs

|  | FR2a | FR2b |
| --- | --- | --- |
| ChBW (50MHz) | 0.54 | 1.0 (NOTE 6) |
| ChBW (100MHz) | 1.0 | 1.0 (NOTE 5) |
| ChBW (200MHz) | 1.0 (NOTE 4) | 1.0 (NOTE 2) |
| ChBW (400MHz) | 1.0 (NOTE 1) | 1.0 (NOTE 3) |
| NOTE 1: This value is based on the relaxation of (MPR – 3.0) dB for MPR > 3.0dB. NOTE 2: Not applicable for MPR > 3.5dB NOTE 3: Not applicable for MPR > 2.0dB NOTE 4: This value is based on the relaxation of (MPR – 5.0) dB for MPR > 5.0dB. NOTE 5: Not applicable for MPR > 5.0dB NOTE 6: Not applicable for MPR >7. 5 dB |  |  |

Following our analysis, it is possible to achieve these influence of noise levels for all test points and MPR levels specified, for all channel bandwidths and both FR2a and FR2b without requiring any relaxations by means of evolved signal conditioning. Therefore, this TC can be removed from the list of TC with low PSD issues without impacting the existing MU for the TC.

Observation 4: The relaxations for the ACLR TC can be removed up to 400MHz BW with the improvements to permitted methods.

## Adjacent channel selectivity (case 1)

Based on the current version of TS 38.521-2 [3] the requirements for FR2b are relaxed by 1.8 and 4.8 dB for 50 and 100 MHz bandwidth respectively, which are the tested bandwidths in the TC.

The table above then results in the following level requirements for the interferer (without the relaxations above).

| Operating band | Interferer Level in dBm |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -53.32 | -50.32 | -47.32 | -44.32 |
| n258 | -53.32 | -50.32 | -47.32 | -44.32 |
| n260 | -51.72 | -47.72 | -45.72 | -42.72 |
| n261 | -53.32 | -50.32 | -47.32 | -44.32 |

From our analysis, these levels are feasible with improvements to existing methods, thus the relaxations and corresponding Notes in the test requirement table can be removed.

Observation 5: The relaxations for the ACS case 1 TC can be removed up to 400MHz BW with the improvements to permitted methods.

## Adjacent channel selectivity (case 2)

For the ACS case 2 requirements the same maximum interferer level is achievable as for the case 1, since the maximum achievable interferer level of the system does not depend as much on the TC requirements, rather on the hardware of the test system.

Thus, there is no change in the testability of ACS case 2. However, it should be noted that further improvement, by e.g. NF methods described in TR 38.884 [2], will not be enough to reach the required input power to make this case feasible.

In our analysis, the required interferer power for ACS case 2 cannot be achieved with the improved signal conditioning as the interferer level is set to -25dB, which roughly 20 dB higher than the achievable interferer level for Case 1. The estimated maximum improvement of relaxation with NF based methods as reported in TR 38.884 is 14dB, which is still not enough to overcome the 20dB difference.

Even more, the claimed 14dB improvement is defined for a very concrete theoretical implementation with a 20cm range length, applicable only PC3, NTC and QZ ≤ 30cm. Any practical implementation which will require additional flexibility to cover other PC, ETC or larger QZ, will require a larger range length and thus the improvement is effectively less than 14dB.

Observation 6: ACS case 2 is still not feasible with improved permitted methods.

Observation 7: The usage of NF methods is not enough to solve the testability issue for ACS case 2.

## In-band blocking

For in-band blocking, the same level parameters as for ACS case 1 apply as can be seen from the test requirement table below. Therefore, the relaxations and corresponding Notes in the test requirement table can be removed as well.

Observation 8: The relaxations for the In-band blocking TC can be removed up to 400MHz BW with the improvements to permitted methods.

## Conclusion

As shown in the discussion section above, several TCs do no longer required relaxations and thus should be removed from the list of TCs with high/low PSD issues.

It should be noted that these findings not only have been confirmed by us, but also by a second TE vendor in their contribution [4] in the last meeting, where they have confirmed the findings by RAN4.

Therefore, we propose the following:

Proposal 1: The following TCs shall be removed from the list of TCs with high/low PSD issues: 6.3.1, 6.5.1, 6.5.3.2, 7.5 (case1), 7.6.2.

# Proposals

In this contribution, the following observations and proposals were made:

Observation 1: The relaxations for the Minimum Output Power TC can be removed up to 400MHz BW with the improvements to permitted methods.

Observation 2: The relaxations for the Off Power TC can be reduced by at least 8.4 dB in FR2a and 11.4 dB in FR2b with the improvements to permitted methods, assuming a similar noise floor as in the Minimum Output Power case.

Observation 3: No relaxations are required for the occupied bandwidth TC.

Observation 4: The relaxations for the ACLR TC can be removed up to 400MHz BW with the improvements to permitted methods.

Observation 5: The relaxations for the ACS case 1 TC can be removed up to 400MHz BW with the improvements to permitted methods.

Observation 6: ACS case 2 is still not feasible with improved permitted methods.

Observation 7: The usage of NF methods is not enough to solve the testability issue for ACS case 2.

Observation 8: The relaxations for the In-band blocking TC can be removed up to 400MHz BW with the improvements to permitted methods.

Proposal 1: The following TCs shall be removed from the list of TCs with high/low PSD issues: 6.3.1, 6.5.1, 6.5.3.2, 7.5 (case1), 7.6.2.

# References

- R4-2016562, “Views on test methods for high DL power and low UL power TCs”, Rohde & Schwarz, RAN4#97-e, November 2020

- TS 38.884 v16.6.0

- TS 38.521-2 v17.0.0

- R5-225222, “On achievable improvements of permitted methods”, Keysight Technologies, RAN5#96-e, August 2022