3GPP TSG-RAN WG5 Meeting #97	R5-226582r1

# Toulouse, France, 14th – 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1458 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Ref sense for R15 combos |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Korea |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | DC_12A_n66A and DC_30A_n66A are still in old table format for ref sensitivity. DC combos marked as “Pending” in PRD21 shall be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test configuration for DC_12A_n66A to Table 7.3B.2.3.4.2.1-6 Deleted Table 7.3B.2.3.4.2.1-2_13 and renamed as Void. Added test configuration for DC_30A_n66A to Table 7.3B.2.3.4.2.1-6 Deleted Table 7.3B.2.3.4.2.1-4m and renamed as Void. Deleted Table 7.3B.2.3.4.2.1-2_28 for DC_28A_n51A as this combo is marked as “Pending” in PRD 21 and renamed the table as Void. Deleted DC_28A_n51A from Table 7.3B.2.3.4.2.1-5. Deleted DC_3A_n7A, and DC_3A_n8A from Table 7.3B.2.3.4.2.1-5 as they are already included in new format. Added test configuration for DC_66A_n5A to Table 7.3B.2.3.4.2.1-6. Deleted DC_66A_n5A from Table 7.3B.2.3.4.2.1-5. Deleted Table 7.3B.2.3.4.2.1-4k for DC_1A_n40A as this combo is marked as “Pending” in PRD 21 and renamed the table as Void. Deleted Table 7.3B.2.3.4.2.1-4l for DC_3A_n51A as this combo is marked as “Pending” in PRD 21 and renamed the table as Void. Deleted Table 7.3B.2.3.4.2.1-4n for DC_46A_n78A as this combo is marked as “Pending” in PRD 21 and renamed the table as Void. Deleted DC_3A_n20A, DC_20A_n77A, DC_28A_n50A and DC_7A_n20A from Table 7.3B.2.3.4.2.1-5 since these combos are marked as “Pending” in PRD21. Deleted tables Table 7.3B.2.3.4.2.1-4j for DC_1A_n40A as this combo is marked as “Pending” in PRD 21 and renamed the table as Void. Deleted Table 7.3B.2.3.4.2.1-3: Default test conditions for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1 since all test configurations are moved to Table 7.3B.2.3.4.2.1-6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will not be aligned with the core specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TP analysis covered in R5-226585, R5-226587 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1 change - Updated other comments on cover sheet |  |  |  |  |  |  |  |  |

<< Start of changes >>

#### 7.3B.2.3	Reference sensitivity for Inter-band EN-DC within FR1 (2 CCs)

7.3B.2.3.1	Test purpose

Same as in clause 7.3B.2.1.1.

7.3B.2.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting inter-band EN-DC within FR1 with 2 DL CCs.

7.3B.2.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3B.2.0.

LTE anchor agnostic approach is not applied.

7.3B.2.3.4	Test description

7.3B.2.3.4.1	Void

7.3B.2.3.4.2	Test description

7.3B.2.3.4.2.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths based on EN-DC operating bands specified in clause 5.5B, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration, and are shown in Table 7.3B.2.3.4.2.1-0 to Table 7.3B.2.3.4.2.1-6.

The details of the uplink and downlink reference measurement channels (RMCs) are specified in TS 36.521-1 [10] Annex A.2 and A.3 for E-UTRA RMC, and in TS 38.521-1 [8] Annex A.2 and A.3 for NR RMC respectively. The details of the OCNG patterns used are specified in TS 36.521-1 [10] Annex A.5 and in TS 38.521-1 [8] Annex A.5 for E-UTRA CG and NR CG, respectively. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG, respectively.

For configurations without any reference sensitivity exception, in a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. For configurations with reference sensitivity exception, in an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected.

Table 7.3B.2.3.4.2.1-0: Test configurations table for all EN-DC configurations for FR1 non-exception requirements

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  | Mid range for E-UTRA and Mid range for NR |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1 |  |  |  |  |  | Refer to "NR NRB "and "E-UTRA NRB " columns |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Lowest supported SCS |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  | SCG -NR |  |  |  |  |
|  | Band | Range | NRB |  | Band |  | Range | NRB |  |
|  | UL MOD | DL MOD | CH BW | DLalloc / UL alloc | UL MOD |  | DL MOD | UL/DL Ch BW | DLalloc / UL alloc |
| Test Settings for a DC_XA_nYA Configuration (Inter-band EN-DC) – Note 2 |  |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  | nY |  | default |  |  |
|  | N/A | N/A | 5 MHz | N/A | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | Highest NRB | All RBs / REFSENS_NR |
| NOTE 1: REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. NOTE 2: LTE anchor agnostic configuration NOTE 3: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. NOTE 4: The test configuration table only applies to EN-DC only capable devices and devices not supporting NR/5GC mode in the tested band. NOTE 5: Only one EN-DC combination per FR1 band is tested for each EN-DC configuration as defined in clause 5.5B. |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-1: Initial test conditions for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1 E-UTRA Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1 |  |  |  | Mid range for E-UTRA and NR, unless otherwise specified in Table 7.3B.2.3.4.2.1-2_1 to Table 7.3B.2.3.4.2.1-2_28 |  |  |  |
| NR Test Channel Bandwidths as specified in TS 38.508-1 [6] clause 4.3.1 E-UTRA Test Channel Bandwidths as specified in TS 36.508 [11] clause 4.3.1 |  |  |  | Highest NRB_agg , unless otherwise specified in Table 7.3B.2.3.4.2.1-2_1 to Table 7.3B.2.3.4.2.1-2_28 |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest otherwise specified |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |
| Downlink Configuration |  |  |  | Uplink Configuration |  |  |  |
| E-UTRA Cell |  | NR Cell |  | E-UTRA Cell |  | NR Cell |  |
| Modulation | RB allocation | Modulation | RB allocation | Modulation | RB allocation | Modulation | RB allocation |
| QPSK | Full RB | CP-OFDM QPSK | Full RB (NOTE 1) | QPSK | Full RB | DFT-s-OFDM QPSK | Full RB (NOTE 1) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8] unless otherwise specified in Table 7.3B.2.3.4.2.1-2_1 to Table 7.3B.2.3.4.2.1-2_28. |  |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_1: Void

Table 7.3B.2.3.4.2.1-2_2: Void

Table 7.3B.2.3.4.2.1-2_3: Void

Table 7.3B.2.3.4.2.1-2_4: Void

Table 7.3B.2.3.4.2.1-2_5: Void

Table 7.3B.2.3.4.2.1-2_6: Void

Table 7.3B.2.3.4.2.1-2_7: Void

Table 7.3B.2.3.4.2.1-2_8: Void

Table 7.3B.2.3.4.2.1-2_9: Void

Table 7.3B.2.3.4.2.1-2_10: Test configurations table for exceptions due to UL harmonic interference for EN-DC 8_n41 (Test frequencies are selected to fulfil Requirement of Note 8 and 9 in Table 7.3B.2.0.3.1-1)

| E-UTRA Band 8 |  |  |  | NR Band n41 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | NR FC (DL) (MHz) NDL | NR CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 885 MHz/ 21500 | 16@17 | 2655 MHz/ 531000 | Lowest | REFSENS (NOTE 2) |
| 2 | 10 | 885 MHz/ 21500 | 25@12 | 2655 MHz/ 531000 | Mid | REFSENS (NOTE 2) |
| 3 |  | 890 MHz/ 21550 | 16@17 | 2670 MHz/ 534000 | Lowest | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 8 and 9 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_11: Void

Table 7.3B.2.3.4.2.1-2_12: Void

Table 7.3B.2.3.4.2.1-2_13: Void

Table 7.3B.2.3.4.2.1-2_14: Test configurations table for exceptions due to UL harmonic interference for EN-DC 18_n77 (Requirement of Note 4 and 5)

| E-UTRA Band 18 |  |  |  | NR Band n77 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | NR FC (DL) (MHz) NDL | NR CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 820 MHz/ 23900 | 16@17 | 4100.01 MHz/ 673334 | Lowest | REFSENS (NOTE 2) |
| 2 |  | 820 MHz/ 23900 | 25@12 | 4100.01 MHz/ 673334 | Mid Highest | REFSENS (NOTE 2) |
| 3 | 10 | 822.5 MHz/ 23925 | 16@17 | 4112.52 MHz/ 674168 | Lowest | REFSENS (NOTE 2) |
| 4 |  | 822.5 MHz/ 23925 | 25@12 | 4112.52 MHz/ 674168 | Mid Highest | REFSENS (NOTE 2) |
| 5 |  | 825 MHz/ 23950 | 16@17 | 4125 MHz/ 675000 | Lowest | REFSENS (NOTE 2) |
| 6 |  | 825 MHz/ 23950 | 25@12 | 4125 MHz/ 675000 | Mid Highest | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 4 and 5 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_15: Test configurations table for exceptions due to UL harmonic interference for EN-DC 19_n77 (Requirement of Note 4 and 5)

| E-UTRA Band 19 |  |  |  | NR Band n77 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | NR FC (DL) (MHz) NDL | NR CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 835 MHz/ 24050 | 16@17 | 4175.01 MHz/ 678334 | Lowest | REFSENS (NOTE 2) |
| 2 | 10 | 835 MHz/ 24050 | 25@12 | 4175.01 MHz/ 678334 | 40 | REFSENS (NOTE 2) |
| 3 |  | 838.9 MHz/ 24089 | 16@17 | 4194.99 MHz/ 679666 | Lowest | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 4 and 5 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. NOTE 3: For UEs with limited UE NR channel bandwidth capability, if the channel BW is not supported by the UE, skip the test point. This shall apply only for Rel 15 UEs. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_16: Void

Table 7.3B.2.3.4.2.1-2_17: Void

Table 7.3B.2.3.4.2.1-2_18: Void

Table 7.3B.2.3.4.2.1-2_19: Void

Table 7.3B.2.3.4.2.1-2_20: Void

Table 7.3B.2.3.4.2.1-2_21: Void

Table 7.3B.2.3.4.2.1-2_22: Test configurations table for exceptions due to UL harmonic interference for EN-DC 1_n28

| NR Band n28 |  |  |  | E-UTRA Band 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | E-UTRA FC (DL) (MHz) NDL | E-UTRA CBW (MHz) | UL allocation (LCRB) |
| 1 | 10 | 715 MHz/ 143000 | 8@21 | 2145 MHz/ 350 | 5 | REFSENS (NOTE 2) |
| 2 | 10 | 715 MHz/ 143000 | 16@17 | 2145 MHz/ 350 | 10 | REFSENS (NOTE 2) |
| 3 | 10 | 715 MHz/ 143000 | 25@12 | 2145 MHz/ 350 | 15, 20 | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 8, 9 and 10 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.4.1-1 in TS 36.521-1 [10] which defines uplink RB configuration and start RB location for each channel BW. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_23: Test configurations table for exceptions due to UL harmonic interference for EN-DC 7_n71 (Requirement of Note 6 and 7)

| NR Band n71 |  |  |  | E-UTRA Band 7 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | FC (DL) (MHz) NDL | CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 668 MHz/ 133600 | 16@17 | 2672 MHz/ 3270 | 10 | REFSENS (NOTE 2) |
| 2 | 10 |  | 25@17 |  | 15 | REFSENS (NOTE 2) |
| 3 |  |  | 25@17 |  | 20 | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 6 and 7 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.4.1-1 in TS 36.521-1 [10] which defines uplink RB configuration and start RB location for each channel BW and E-UTRA band. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_24: Void

Table 7.3B.2.3.4.2.1-2_25: Test configurations table for exceptions due to UL harmonic interference for EN-DC 12_n78 (Test frequencies are selected to fulfil Requirement of Note 4 and 5 in Table 7.3B.2.0.3.1-1)

|  | E-UTRA Band 12 |  |  | NR Band 78 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) / NUL | UL allocation (LCRB) | NR FC (DL) (MHz) / NDL | NR CBW (MHz) | UL allocation (LCRB) |
|  |  |  | 10@10 |  | 10 |  |
| 1 | 10 | 711 MHz / 23130 | 15@22 | Mid | 15 | REFSENS (NOTE 2) |
|  |  |  | 20@15 |  | 20 |  |
|  |  |  | 25@12 |  | >=40 |  |
| NOTE 1: Test frequencies are selected to fulfil Note 4 and 5 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. NOTE 3: Only Highest NR channel bandwidths supported by the UE are tested. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_26: Void

Table 7.3B.2.3.4.2.1-2_27: Void

Table 7.3B.2.3.4.2.1-2_28: Void

The initial test configurations for E-UTRA band and NR band consist of environmental conditions, test frequencies, and channel bandwidths and RB allocations for exceptional test scenarios due to receiver harmonic mixing for EN-DC in NR FR1 are specified in Table 7.3B.2.3.4.2.1-6.

Table 7.3B.2.3.4.2.1-3: Void

Table 7.3B.2.3.4.2.1-3a: Void

Table 7.3B.2.3.4.2.1-3aa: Void

Table 7.3B.2.3.4.2.1-3b: Void

Table 7.3B.2.3.4.2.1-3c: Void

Table 7.3B.2.3.4.2.1-3d: Void

Table 7.3B.2.3.4.2.1-3e: Void

Table 7.3B.2.3.4.2.1-3f: Void

Table 7.3B.2.3.4.2.1-3g: Void

Table 7.3B.2.3.4.2.1-3h: Void

Table 7.3B.2.3.4.2.1-3i: Void

Table 7.3B.2.3.4.2.1-3j: Void

Table 7.3B.2.3.4.2.1-3k: Void

The initial test configurations for E-UTRA band and NR band consist of environmental conditions, test frequencies, and channel bandwidths and RB allocations for exceptional test scenarios due to cross band isolation for EN-DC in NR FR1 are specified in Table 7.3B.2.3.4.2.1-4, to Table 7.3B.2.3.4.2.1-4n.

Table 7.3B.2.3.4.2.1-4: Test configurations table for exceptions due to cross band isolation for EN-DC FR1

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1 E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  | Specified in Table 7.3B.2.3.4.2.1-4a to Table 7.3B.2.3.4.2.1-4n. |  |  |  |
| NR Test Channel Bandwidths as specified in TS 38.508-1 [6] clause 4.3.1 E-UTRA Test Channel Bandwidths as specified in TS 36.508 [11] clause 4.3.1 |  |  |  | Specified in Table 7.3B.2.3.4.2.1-4a to Table 7.3B.2.3.4.2.1-4n. |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 |  |  |  | 30 KHz |  |  |  |
| NR Test Parameters |  |  |  |  |  |  |  |
| Downlink Configuration |  |  |  | Uplink Configuration |  |  |  |
| NR Modulation | NR RB allocation | E-UTRA Modulation | E-UTRA RB allocation | NR Modulation | NR RB allocation | E-UTRA Modulation | E-UTRA RB allocation |
| CP-OFDM QPSK | Full RB (NOTE 1) | QPSK | Full RB | DFT-s-OFDM QPSK | Specified in Table 7.3B.2.3.4.2.1-4a to Table 7.3B.2.3.4.2.1-4n | QPSK | Table 7.3.4.1-1 in TS 36.521-1 [10] |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8]. |  |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-4a: Void

Table 7.3B.2.3.4.2.1-4b: Test configurations for exceptions due to cross band isolation for EN-DC 25_n41 (Victim band LTE)

| Aggressor UL |  |  | E-UTRA B25 Ch BW/Frequency range |  |  |
| --- | --- | --- | --- | --- | --- |
| CBW/RB allocation |  |  | 10MHz | 15 MHz | 20MHz |
|  |  |  | High range | High range | High range |
| NR | 100 MHz 160@0 | low range | X1- | X1- | X1 |
| n41 | 100 MHz 160@0 | mid range | - | - | X1 |
| NOTE 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: NR UL RB configuration shall set per Table 7.3.2.4.1-3 in TS 38.521-1 [8]. |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-4c: Void

Table 7.3B.2.3.4.2.1-4d: Test configurations for exceptions due to cross band isolation for EN-DC 38_n78 (Victim band LTE)

| Aggressor UL |  |  | E-UTRA B38 DL CBM/RB allocation |  |  |
| --- | --- | --- | --- | --- | --- |
| CBW/RB allocation |  |  | 10MHz | 15 MHz | 20MHz |
|  |  |  | Low, Mid High range | Low, Mid High range | Low, Mid High range |
| NR Band | 100 MHz 270@0 | Low range | X | X | X |
| n78 | 100 MHz 270@0 | High range | - | - | X |
| NOTE 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: NR UL RB configuration shall be further limited to that specified per Table 7.3.2.4.1-3 in TS 38.521-1 [8]. |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-4e: Void

Table 7.3B.2.3.4.2.1-4f: Void

Table 7.3B.2.3.4.2.1-4g: Void

Table 7.3B.2.3.4.2.1-4h: Void

Table 7.3B.2.3.4.2.1-4i: Test configurations table for exceptions due to cross band isolation for EN-DC 66_n41(Victim band LTE)

| Aggressor UL |  |  | E-UTRA B66 Ch BW/Frequency range |  |  |
| --- | --- | --- | --- | --- | --- |
| CBW/RB allocation |  |  | 10MHz | 15 MHz | 20MHz |
|  |  |  | High range | High range | High range |
| NR | 100 MHz 128@0 | low range | X1 | X1 | X1 |
| n41 | 100 MHz 128@0 | mid range | - | - | X1 |
| NOTE 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: NR UL RB configuration shall be further limited to that specified per Table 7.3.2.4.1-3 in TS 38.521-1 [8]. NOTE 3: Applicable only when harmonic mixing MSD for this combination is not applied. |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-4j: Void

Table 7.3B.2.3.4.2.1-4k: Void

Table 7.3B.2.3.4.2.1-4l: Void

Table 7.3B.2.3.4.2.1-4m: Void

Table 7.3B.2.3.4.2.1-4n: Void

The initial test configurations for E-UTRA band and NR band consist of environmental conditions, test frequencies, and channel bandwidths and RB allocations for exceptional test scenarios due to dual uplink operation for EN-DC in NR FR1 (two bands) are specified in Table 7.3B.2.3.4.2.1-5.

Table 7.3B.2.3.4.2.1-5: Test Configuration Table Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (two bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1 E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  | For test frequencies refer to "Range" columns. |  |  |  |  |  |
| NR Test Channel Bandwidths as specified in TS 38.508-1 [6] clause 4.3.1 E-UTRA Test Channel Bandwidths as specified in TS 36.508 [11] clause 4.3.1. |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | DC Configuration / NRB_agg |  |  |  |  |  | DL Allocation |  |  | UL Allocation (Note 2,3) |  |  |
|  | DC Configuration |  |  |  | E-UTRA Ch BW | NR Ch BW | CC MOD E-UTRA/NR | E-UTRA & NR RB allocation |  | CC MOD E-UTRA/NR | E-UTRA & NR allocations (LCRB @ RBstart) |  |
|  | E-UTRA |  | NR |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range | Band | Range |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA_nYA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid | Y | Mid | 5 | Mid | QPSK /CP-OFDM QPSK | All RBs |  | QPSK/ DFT-s-OFDM QPSK | REFSENS | REFSENS |
| Test Settings for DC_7A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | Note 5 | 66 | Note 5 | 10 | 5 | Note 7 | All RBs |  | Note 7 | 50@0 | 25@0 |
| Test Settings for DC_7A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | Low | 77 | High | 5 | 10 | Note 7 | All RBs |  | Note 7 | 25@0 | 50@0 |
| Test Settings for DC_8A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 | Low | 41 | High | 5 | 10 | Note 7 | All RBs |  | Note 7 | 25@0 | 50@0 |
| Test Settings for DC_12A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 12 | Note 5 | 78 | Note 5 | 5 | 10 | Note 7 | All RBs |  | All RBs | 25@0 | 50@0 |
| Test Settings for DC_66A_n25A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | Note 5 | 5 | Note 5 | 5 | 5 | Note 7 | All RBs |  | Note 7 | 25@0 | 25@0 |
| Note 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in clause 6.2.5A. In case Single UL is allowed and the UE only indicates support of "Single UL" the output power of the active UL shall be set at PCMAX_L,c or set to the maximum output power according to the UE power scaling capability. Note 2: Use DC Configuration – specific test points if present in the table, otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use the Default Test Settings test points. Note 3: X,Y correspond to the different bands in the DC Configuration. E.g. for DC_1A-n3A, X=1, Y=3. Note 4: REFSENS refers to the E_UTRA bands and NR band NRB 's single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2 of TS 36.521-1 [10] and Table 7.3.2.4.1-3 of TS 38.521-1 [8], respectively Note 5: Test frequency for each DC configuration shall follow Table 7.3B.2.0.3.5.1-1. If test configurations of each ID in a DC configuration are same, test frequency shall follow the order of Table 7.3B.2.0.3.5.1-1. Note 6: Not applicable if the UE only supports Bandwidth Combination Set 1. Note 7: Same as default. Note 8: RBSTART = 0 |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-6: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions (two bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |  |  |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |  |  |  |  |  |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Lowest supported SCS |  |  |  |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SCG -NR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  |  |  |  | Range |  |  |  |  |  | NRB |  |  |  |  |  |  |  |  |  |  | Band |  |  |  |  |  | Range |  |  |  | NRB |  |  |  |  |
|  | UL MOD |  |  |  |  | DL MOD |  |  |  |  |  | CH BW |  |  |  |  |  |  | DLalloc / UL alloc |  |  |  | UL MOD |  |  |  |  |  | DL MOD |  |  |  | UL/DL Ch BW | DLalloc / UL alloc |  |  |  |
| Test Settings for a DC_1A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12,11 | 1 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz | All RBs / 0 |  |  |  |
| 24,10 | 1 |  |  |  |  | UL 1950 / DL 2140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  | UL 1760 / DL 1855 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| 36 | 1 |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  | High |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz | All RBs / 0 |  |  |  |
| Test Settings for a DC_1A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 1 |  |  |  |  | UL 1965 / DL 2155 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n8 |  |  |  |  |  | UL 887.5 / DL 932.5 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_1A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 1 |  |  |  |  | UL 1950/ DL 2140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3900 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 23 | 1 |  |  |  |  | UL 1950/ DL 2140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3870 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz | All RBs / 0 |  |  |  |  |
| 34 | 1 |  |  |  |  | UL 1950/ DL 2140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 4090.005 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| 44 | 1 |  |  |  |  | UL 1950/ DL 2140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3709.005 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_1A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 1 |  |  |  |  | UL 1950 / DL 2140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | UL/DL 3709.995 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_2A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 2 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_LTE |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_3 |  |  |  |
| Test Settings for a DC_2A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  | 2 |  |  |  | UL 1855 / DL 1935 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | DL 2175 |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| 24 |  | 2 |  |  |  | UL 1883.3 /DL1963.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | DL 2150 |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_2A_n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 2 |  |  |  |  | DL 1980 MHZ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n71 |  |  |  |  |  | UL 665.5 MHz |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_1 |  |  |  |  |
| 23 | 2 |  |  |  |  | DL1980 MHZ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n71 |  |  |  |  |  | UL 673 MHz |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_1 |  |  |  |  |
| 35 | 2 |  |  |  |  | UL 1881 MHZ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n71 |  |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz | All RBs / 0 |  |  |  |  |
| Test Settings for a DC_2A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 2 |  |  |  |  | UL 1860 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3720 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 23 | 2 |  |  |  |  | UL 1860 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3700.005 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz | All RBs / 0 |  |  |  |  |
| 3 8 | 2 |  |  |  |  | UL 1860 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3400.005 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz | All RBs / 0 |  |  |  |  |
| 45 | 2 |  |  |  |  | DL Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3920.01 |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| 4a 9 | 2 |  |  |  |  | DL 1950 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3999.99 |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| 54 | 2 |  |  |  |  | UL 1855/DL 1935 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3790.005 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| 64 | 2 |  |  |  |  | UL 1900/DL 1980 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3720 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| 74 | 2 |  |  |  |  | UL 1885/DL 1965 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3810 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_2A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 2 |  |  |  |  | UL 1870 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | UL/DL 3740.01 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 23 | 2 |  |  |  |  | UL 1885 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | UL/DL 3739.995 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz | All RBs / 0 |  |  |  |  |
| 34 | 2 |  |  |  |  | UL 1855/ DL 1935 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | UL/DL 3790.005 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| 44 | 2 |  |  |  |  | UL 1885/ DL 1965 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | UL/DL 3690 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_3A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1,11 | 3 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n1 |  |  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |  |  |
| 26 | 3 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n1 |  |  |  |  |  | Low |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |  |  |
| 34 | 3 |  |  |  |  | UL 1760 / DL 1855 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n1 |  |  |  |  |  | UL 1950 / DL 2140 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_3A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 3 |  |  |  |  | UL 1771 / DL 1866 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  |  |  |  | UL 838 / DL 883 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| 24 | 3 |  |  |  |  | UL 1721 / DL 1816 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  |  |  |  | UL 838 / DL 883 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_3A_n7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 3 |  |  |  |  | UL 1730 / DL 1825 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n7 |  |  |  |  |  | UL 2535 / DL 2655 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_3A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 3 |  |  |  |  | UL 1755 / DL 1850 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n8 |  |  |  |  |  | UL 900 / DL 945 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| 24 | 3 |  |  |  |  | UL 1747.5 / DL 1842.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n8 |  |  |  |  |  | UL 897.5 / DL 942.5 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_3A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 3 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  |  | Low |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |
| 26 | 3 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  |  | Low |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |  |  |
| 34, 10 | 3 |  |  |  |  | UL 1740 / DL 1835 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  |  | UL/DL 2657.5 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for DC_3A_n77A and DC_3A_n78A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 3 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | UL/DL 3495 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |
| 23 | 3 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | UL/DL 3525 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz | All RBs / 0 |  |  |  |
| 35 | 3 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | UL/DL 3685.005 |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |
| 39 | 3 |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | High |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs/0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |
| 44 | 3 |  |  |  |  | UL 1740 / DL 1835 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | UL/DL 3574.995 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| 54 | 3 |  |  |  |  | UL 1765 / DL 1860 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | UL/DL 3435 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_5A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 5 |  |  |  |  | UL 838/ DL 883 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | UL 1721/ DL 2121 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | N/A |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_5A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 5 |  |  |  |  | UL 837.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3350.01 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 23 | 5 |  |  |  |  | UL 829 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 4145.01 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 38 | 5 |  |  |  |  | UL 840 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3600 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 44, 13 | 5 |  |  |  |  | UL 844/DL 889 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3421.005 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| 54, 13 | 5 |  |  |  |  | UL 826.5/DL 871.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 4177.5 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for DC_5A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 5 |  |  |  |  | UL 840 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | UL/DL 3360 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 28 | 5 |  |  |  |  | UL 840 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | UL/DL 3560.01 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 34 | 5 |  |  |  |  | UL 844/DL 889 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | UL/DL 3421.005 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_7A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 7 |  |  |  |  | DL 2655 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  | DL 1825 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | N/A |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_7A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 7 |  |  |  |  | DL 2667 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  |  |  |  | DL 879 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | N/A |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_7A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 7 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | Low |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |
| Test Settings for a DC_8A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 |  |  |  |  | DL 932.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n1 |  |  |  |  |  | DL 2155 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | N/A |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_8A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 |  |  |  |  | DL 945 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  | DL 1850 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | N/A |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| 24 | 8 |  |  |  |  | DL 942.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  | DL 1842.5 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | N/A |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_8A_n20A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 |  |  |  |  | UL 890.5 / DL 935.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n20 |  |  |  |  |  | UL 849.5 / DL 808.5 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| 24 | 8 |  |  |  |  | UL 892.5 / DL 937.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n20 |  |  |  |  |  | UL 847.5 / DL 806.5 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for DC_8A_n77A and DC_8A_n78A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 8 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3590.01 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |
| 2,8 | 8 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3520.005 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |
| 34 | 8 |  |  |  |  | UL 897.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3634.995 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for DC_8A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 8 |  |  |  |  | UL 900 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  | UL/DL 4500 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 2,8 | 8 |  |  |  |  | UL 900 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  | UL/DL 4299.99 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 34 | 8 |  |  |  |  | UL 897.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  | UL/DL 4532.505 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_11A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 11 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  | UL/DL 4457.7 |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |
| 2,9 | 11 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  | UL/DL 4512.69 |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |
| Test Settings for a DC_12A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 12 |  |  |  |  | UL 710 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | UL 1730 /DL 2130 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz | All RBs / 0 |  |  |  |
| 28 | 12 |  |  |  |  | UL 710 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | UL 1770 /DL 2170 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz | All RBs / 0 |  |  |  |
| Test Settings for a DC_13A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 13 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3909.99 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 28 | 13 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3600 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 35 | 13 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3909.99 |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| 49 | 13 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3834 |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| 54 | 13 |  |  |  |  | UL 784.5/DL 753.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3891.495 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_19A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 19 |  |  |  |  | DL 884 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  | Low |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |
| 2,9 | 19 |  |  |  |  | DL 884 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  | UL/DL 4445.025 |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |
| Test Settings for a DC_20A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 20 |  |  |  |  | UL 840 / DL 799 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  | UL 1775 / DL 1870 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| 24 | 20 |  |  |  |  | UL 847 / DL 806 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  | UL 1735 / DL 1830 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_20A_n7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 20 |  |  |  |  | UL 851 / DL 810 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n7 |  |  |  |  |  | UL 2512 / DL 2632 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_20A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 20 |  |  |  |  | UL 849.5 / DL 808.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n8 |  |  |  |  |  | UL 892.5 / DL 937.5 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_20A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1,8 | 20 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |
| 23 | 20 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | UL/DL 3387.99 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |
| 34 | 20 |  |  |  |  | UL 850 / DL 809 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | UL/DL 3358.995 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_21A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 21 |  |  |  |  | DL 1506 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  | UL/DL 4518 |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| 29 | 21 |  |  |  |  | DL 1506 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  | UL/DL 4800 |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| 34 | 21 |  |  |  |  | UL 1457.5/ DL 1505.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  | UL/DL 4420.5 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_26A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 26 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  | UL/DL 2524.5 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  | 15 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  |  |  | N/A |  |  | CP-OFDM QPSK |  |  |  | 50 MHz |  |  |  |  | All RBs / 0 |  |
| 2,8 | 26 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  | UL/DL 2572 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  | 15 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  |  |  | N/A |  |  | QPSK |  |  |  | 50 MHz |  |  |  |  | All RBs / 0 |  |
| 34 | 26 |  |  |  |  | UL 839 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  | UL/DL 2562 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  |  |  | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  | 10 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| 45 | 26 |  |  |  |  | DL 886.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  | UL/DL 2660 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  | 15 MHz |  |  |  |  |  | All RBs / REFSENS_LTE |  |  |  |  |  |  | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |
| Test Settings for a DC_26A_n77A/DC_26A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 26 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | UL/DL 3366 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  | 15 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  |  |  | N/A |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / 0 |  |
| 2,8 | 26 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | UL/DL 3446.01 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  | 15 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  |  |  | N/A |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / 0 |  |
| 34 | 26 |  |  |  |  | UL 836.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | UL/DL 3391.005 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  |  |  | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  | 10 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_26A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 26 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4432.5 |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  | 15 MHz |  |  |  |  |  | All RBs / 0 |  |  |  |  |  |  | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  | 60 MHz |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |
| 22,9 | 26 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4470.51 |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  | 15 MHz |  |  |  |  |  | All RBs / 0 |  |  |  |  |  |  | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  | 60 MHz |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |
| Test Settings for a DC_28A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 28 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  | Low |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  | 20MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  |  |  |  | N/A |  |  | CP-OFDM QPSK |  |  |  | 20 MHz |  |  |  |  | All RBs / 0 |  |
| Test Settings for a DC_28A_n77A/DC_28A_n78A Configuration (Test ID 4 only apply to DC_28A_n77A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | 28 |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  |  | UL/DL 3540 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  |  | QPSK |  |  |  | 10 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / 0 |  |  |  |
| 28 |  |  | 28 |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  |  | QPSK |  |  |  | 10 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / 0 |  |  |  |
| 34 |  |  | 28 |  |  | UL 705.5 / DL 760.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  |  | UL/DL 3582.495 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  |  | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |
| 45 |  |  | 28 |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3815.01 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | N/A |  |  | QPSK |  |  |  | 10 MHz |  |  |  |  |  | All RBs / 0 |  |  |  |  |  | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |  |  |
| Test Settings for a DC_30A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 |  |  | 30 |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | N/A |  |  | QPSK |  |  |  | 10 MHz |  |  |  |  |  | All RBs / 0 |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  |
| Test Settings for a DC_40A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 40 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3525 |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  | 20 MHz |  |  |  |  |  | All RBs / 0 |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  | 20 MHz |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |
| 2,9 | 40 |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  | Mid |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  | 20 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |
| Test Settings for a DC_41A_n77A/DC_41A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 41 |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | UL/DL 3750 |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  | 20 MHz |  |  |  |  |  | All RBs / 0 |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |
| 26 | 41 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | Low |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  | 20 MHz |  |  |  |  |  | All RBs / 0 |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |
| 36 | 41 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | Low |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  | 20MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  |  |  |  | N/A |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / 0 |  |
| Test Settings for a DC_48A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  |  | 48 |  |  |  |  | UL/DL 3555 |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  |  | UL 1760 MHZ |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | N/A |  |  |  |  | QPSK |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |
| 23 |  |  |  | 48 |  |  |  |  | UL/DL 3565 |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  |  | UL 1770 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | N/A |  |  |  |  | QPSK |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |
| 38 |  |  |  | 48 |  |  |  |  | UL/DL 3600 |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  |  | UL1760 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | N/A |  |  |  |  | QPSK |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |
| 44 |  |  |  | 48 |  |  |  |  | UL/DL 3630 |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  |  | UL 1715 / DL 2115 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | QPSK |  |  |  |  | QPSK |  |  |  | 20 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_66A_n2A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14, 10 |  |  |  | 66 |  |  |  |  | UL1775/ DL2175 |  |  |  |  |  |  |  |  |  |  | n2 |  |  |  |  |  |  | UL 1855 / DL 1935 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | QPSK |  |  |  |  | QPSK |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |
| 24, 10 |  |  |  | 66 |  |  |  |  | UL 1750/ DL 2150 |  |  |  |  |  |  |  |  |  |  | n2 |  |  |  |  |  |  | UL 1883.3 / DL 1963.3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | QPSK |  |  |  |  | QPSK |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_66A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  |  | 66 |  |  |  |  | UL 1721/ DL 2121 |  |  |  |  |  |  |  |  |  |  | n5 |  |  |  |  |  |  | UL 838/ DL 883 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | QPSK |  |  |  |  | QPSK |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_66A_n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  |  | 66 |  |  | UL 1750/ DL 2150 |  |  |  |  |  |  |  |  |  |  |  |  | n71 |  |  |  |  |  |  | UL 675/ DL 629 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_66A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  |  |  | 66 |  |  | UL 1720 |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3440.01 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  | 20 |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / 0 |  |
| 23 |  |  |  |  | 66 |  |  | UL 1720 |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3420 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  | 20 |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz |  |  |  |  | All RBs / 0 |  |
| 38 |  |  |  |  | 66 |  |  | UL 1720 |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3600 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  | 20 |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / 0 |  |
| 44 |  |  |  |  | 66 |  |  | UL 1775/ DL 2175 |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3949.995 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| 54 |  |  |  |  | 66 |  |  | UL 1760 / DL 2160 |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3720 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_66A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  |  |  | 66 |  |  | UL 1755 |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | UL/DL 3510 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / 0 |  |
| 23 |  |  |  |  | 66 |  |  | UL 1755 |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | UL/DL 3480 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz |  |  |  |  | All RBs / 0 |  |
| 34 |  |  |  |  | 66 |  |  | UL 1730/ DL2130 |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | UL/DL 3660 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| NOTE 1: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. REFSENS_ENDC_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3B.2.0.3.1-2. REFSENS_ENDC_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3B.2.0.3.2-2. REFSENS_ENDC_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation according to table 7.3B.2.0.3.4-2. REFSENS_ENDC_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to dual uplink operation for ENDC according to table 7.3B.2.0.3.5.1-1 for PC3 and table 7.3B.2.0.3.5.1-1a for PC2. NOTE 2: Void NOTE 3: Test ID with UL harmonic exception NOTE 4: Test ID with 2UL intermodulation exception NOTE 5: Test ID with UL receiver harmonic mixing NOTE 6: Test ID with UL cross band isolation NOTE 7: Void NOTE 8: Test ID with UL harmonic exception avoided NOTE 9: Test ID with UL receiving harmonic mixing exception avoided NOTE 10: Only applicable to UEs not supporting UE capability singleUL-Transmission. NOTE 11: Test ID with Cross band isolation exception avoided, which is only applicable to DC_1_n3, DC_3_n1 and DC_3_n84. NOTE 12: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 14: If the NR frequency does not match to a valid NR-ARFCN, apply the closest NR frequency with a valid NR-ARFCN. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

4.	NR downlink signals are initially set up according to Annex C.0, C.1, C.2, C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1 of TS 38.521-1 [8].

5.	E-UTRA downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0 of TS 36.521-1 [10].

6.	The DL and UL Reference Measurement channels are set according to Tables 7.3B.2.3.4.2.1-0 to 7.3B.2.3.4.2.1-6 for E-UTRA CG and NR CG.

7.	NR propagation conditions are set according to Annex B.0 of TS 38.521-1 [8]. E-UTRA propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

8.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause7.3B.2.3.4.2.3.

9.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6. Step 9 only applicable to the test configuration in Table 7.3B.2.3.4.2.1-0.

7.3B.2.3.4.2.2	Test procedure

For test points in Table 7.3B.2.3.4.2.1-0:

1.	NR SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3B.2.3.4.2.1-0 on the NR CC. The NR SS sends downlink MAC padding bits on the DL RMC.

2.	NR SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.3B.2.3.4.2.1-0 Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the appropriate REFSENS value defined in TS 38.521-1 [8] Table 7.3.2.5-1 for NR band. Send continuously uplink power control "up" commands in the uplink scheduling information to both carriers to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

4.	Measure the average throughput of the NR carrier for a duration sufficient to achieve statistical significance according to Annex H.2 of TS 38.521-1 [8] for NR band.

For test points in Table 7.3B.2.3.4.2.1-6:

1.	SS transmits PDSCH via PDCCH DCI format 1A and PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3B.2.3.4.2.1-6 on the E-UTRA CC and NR CC. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.3B.2.3.4.2.1-6 on the E-UTRA CC and NR CC. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level for the E-UTRA CC and NR CC to the appropriate REFSENS value defined in Tables 7.3B.2.3.5-1 to 7.3B.2.3.5-4. Send continuously uplink power control "up" commands in the uplink scheduling information to both carriers to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

4.	Measure the average throughput of both NR and E-UTRA for a duration sufficient to achieve statistical significance according to Annex H.2 of TS 38.521-1 [8] for NR band, and Annex G.2 of TS 36.521-1 [10] for EUTRA band.

7.3B.2.3.4.2.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED for NR band.

Message contents exceptions for E-UTRA band are according to TS 36.521-1 [10] clause 7.3.4.3 for each network signalling value. Message contents exceptions for NR band are according to TS 38.521-1 [8] clause 7.3.2.4.3 for each network signalling value.

For test points with NOTE 4 in Table 7.3B.2.3.4.2.1-6, the following message exception applies:

Table 7.3B.2.3.4.2.3-1: RRCConnectionReconfiguration: nr-Config-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

Table 7.3B.2.3.4.2.3-2: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6] Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

7.3B.2.3.5	Test requirement

For inter-band EN-DC configurations affected by reference sensitivity exceptions, when test points without NOTE 8, 9 and 11 in Table 7.3B.2.3.4.2.1-6 are tested, the throughput of each CG shall be ≥ 95% of the maximum throughput for the reference receive power level specified in Table 7.3B.2.3.5-1, Table 7.3B.2.3.5-2, Table 7.3B.2.3.5-3, Table 7.3B.2.3.5-3a, Table 7.3B.2.3.5-4 and Table 7.3B.2.3.5-5 for MSDs due to uplink harmonic, harmonic mixing, cross band isolation for PC3 EN-DC, cross band isolation for PC2 EN-DC, dual uplinks for PC3 EN-DC and dual uplink operation for PC2 EN-DC, respectively. For a given EN-DC combo, if more than one category of MSD applies, UE shall pass all requirement.

For test points with NOTE 8, 9 or 11 in Table 7.3B.2.3.4.2.1-6, reference sensitivity test requirements are specified in Table 7.3.2.5-1 in TS 38.521-1 [8] for the NR CC, and Table 7.3.5-1 in TS 36.521-1 [10] for E-UTRA CC.

Reference sensitivity test requirements for test points in Table 7.3B.2.3.4.2.1-0 are specified in Table 7.3.2.5-1 in TS 38.521-1 [8].

For the UE which supports inter-band EN-DC, the minimum requirement for reference sensitivity in Table 7.3.2.5-1 of TS 38.521-1 [8] for NR band and Table 7.3.5-1 of TS 36.521-1 [10] for EUTRA band, shall be increased by the amount given in ΔRIB,c defined in clause 7.3B.3.3 for the applicable two, three, four and five bands operation.

Table 7.3B.2.3.5-1: Reference sensitivity due to UL harmonic for EN-DC in NR FR1

| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 3 | n772 | 15 |  | -71.4 +TT | -71.4 +TT | -71.3 +TT |  |  | -71.2 +TT | -71.3 +TT |  |  |  |  |
|  |  | 30 |  | -71.7 +TT | -71.5 +TT | -71.5 +TT |  |  | -71.3 +TT | -71.4 +TT | -71.4 +TT | -71.3 +TT | -71.3 +TT | -71.3 +TT |
|  |  | 60 |  | -72.1 +TT | -71.8 +TT | -71.7 +TT |  |  | -71.5 +TT | -71.5 +TT | -71.5 +TT | -71.4 +TT | -71.4 +TT | -71.4 +TT |
|  | n773 | 15 |  | -94.2 +TT | -92.7 +TT | -91.9 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -94.5 +TT | -92.8 +TT | -92.1 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -94.9 +TT | -93.1 +TT | -92.3 +TT |  |  |  |  |  |  |  |  |
| 2 | n772, 13 | 15 |  | -71.4 +TT | -71.4 +TT | -71.3 +TT |  |  | -71.2 +TT | -71.3 +TT |  |  |  |  |
|  |  | 30 |  | -71.7 +TT | -71.5 +TT | -71.5 +TT |  |  | -71.3 +TT | -71.4 +TT | -71.4 +TT | -71.3 +TT | -71.3 +TT | -71.3 +TT |
|  |  | 60 |  | -72.1 +TT | -71.8 +TT | -71.7 +TT |  |  | -71.5 +TT | -71.5 +TT | -71.5 +TT | -71.4 +TT | -71.4 +TT | -71.4 +TT |
|  | n773 | 15 |  | -94.2 +TT | -92.7 +TT | -91.9 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -94.5 +TT | -92.8 +TT | -92.1 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -94.9 +TT | -93.1 +TT | -92.3 +TT |  |  |  |  |  |  |  |  |
| 2 | n782 | 15 |  | -71.9+TT | -71.9 +TT | -71.8 +TT |  |  | -71.7 +TT | -71.8 +TT |  |  |  |  |
|  |  | 30 |  | -72.2 +TT | -72.0 +TT | -72.0 +TT |  |  | -71.8 +TT | -71.9 +TT | -71.9 +TT | -71.8 +TT | -71.8 +TT | -71.8 +TT |
|  |  | 60 |  | -72.6 +TT | -72.3 +TT | -72.2 +TT |  |  | -72.0 +TT | -72.0 +TT | -72.0 +TT | -71.9 +TT | -71.9 +TT | -71.9 +TT |
| 2 | n783 | 15 |  | -94.7 +TT | -93.2 +TT | -92.4 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -94.9 +TT | -93.3 +TT | -92.6 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | --95.4 +TT | -93.6 +TT | -92.8 +TT |  |  |  |  |  |  |  |  |
| 3 | n782 | 15 |  | -71.9 +TT | -71.9 +TT | -71.8 +TT |  |  | -71.7 +TT | -71.8 +TT |  |  |  |  |
|  |  |  |  | -74.1 +TT15 | -74.1 +TT15 | -74.0 +TT15 |  |  | -73.9 +TT15 | -74.0 +TT15 |  |  |  |  |
|  |  | 30 |  | -72.2 +TT | -72.0 +TT | -72.0 +TT |  |  | -71.8 +TT | -71.9 +TT | -71.9 +TT | -71.8 +TT | -71.8 +TT | -71.8 +TT |
|  |  |  |  | -74.4 +TT15 | -74.2 +TT15 | -74.2 +TT15 |  |  | -74.0 +TT15 | -74.1 +TT15 | --74.1 +TT15 | -74.0 +TT15 | -74.0 +TT15 | -74.0 +TT15 |
|  |  | 60 |  | -72.6 +TT | -72.3 +TT | -72.2 +TT |  |  | -72.0 +TT | -72.0 +TT | -72.0 +TT | -71.9 +TT | -71.9 +TT | -71.9 +TT |
|  |  |  |  | -74.8 +TT15 | -74.5 +TT15 | -74.4 +TT15 |  |  | -74.2 +TT15 | -74.2 +TT15 | -74.2 +TT15 | -74.1 +TT15 | -74.1 +TT15 | -74.1 +TT15 |
|  | n783 | 15 |  | -94.7 +TT | -93.2 +TT | -92.4 +TT |  |  |  |  |  |  |  |  |
|  |  |  |  | -96.9 +TT15 | -95.4 +TT15 | -94.6 +TT15 |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -95.0 +TT | -93.3 +TT | -92.6 +TT |  |  |  |  |  |  |  |  |
|  |  |  |  | -97.2 +TT15 | -95.5 +TT15 | -94.8 +TT15 |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -95.4 +TT | -93.6 +TT | -92.8 +TT |  |  |  |  |  |  |  |  |
|  |  |  |  | -97.6 +TT15 | -95.8 +TT15 | -95.0 +TT15 |  |  |  |  |  |  |  |  |
| 5 | n78 | 15 |  | -84.2 +TT | -84.4 +TT |  |  |  | -84.2 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -84.3 +TT | -84.5 +TT | -84.4 +TT |  |  | -84.3 +TT | -84.5 +TT | -84.4 +TT | -84.3 +TT | -84.0 +TT | -84.2 +TT |
|  |  | 60 |  | -84.5 +TT | -84.6 +TT | -84.5 +TT |  |  | -84.5 +TT | -84.6 +TT | -84.5 +TT | -84.4 +TT | -84.1 +TT | -84.3 +TT |
| 8 | n776,7 n786,7 | 15 |  | -84.5 +TT | -84.4 +TT | -84.2 +TT |  |  | -84.0 +TT | -83.9 +TT |  |  |  |  |
|  |  | 30 |  | -84.8 +TT | -84.5 +TT | -84.4 +TT |  |  | -84.1 +TT | -84.0 +TT | -83.9 +TT | -83.8 +TT | -83.5 +TT | -83.7 +TT |
|  |  | 60 |  | -85.2 +TT | -84.8 +TT | -84.6 +TT |  |  | -84.3 +TT | -84.1 +TT | -84.0 +TT | -83.9 +TT | -83.6 +TT | -83.8 +TT |
| 8 | n41 | 15 |  | -81.8 +TT | -81.7 +TT | -81.7 +TT |  |  | -81.6 +TT | -81.5 +TT |  |  |  |  |
|  |  | 30 |  | -82.1 +TT | -81.8 +TT | -81.9 +TT |  |  | -81.7 +TT | -81.6 +TT | -81.4 +TT | -81.3 +TT | -81.2 +TT | -81.2 +TT |
|  |  | 60 |  | -82.5 +TT | -82.1 +TT | -81.1 +TT |  |  | -81.9 +TT | -81.7 +TT | -81.6 +TT | -81.3 +TT | -81.2 +TT | -81.2 +TT |
| 8 | n794,5 | 15 |  |  |  |  |  |  | -82.8 +TT | -82.4 +TT |  |  |  |  |
|  |  | 30 |  |  |  |  |  |  | -82.9 +TT | -82.5 +TT | -82.3 +TT | -81.7 +TT |  | -81.2 +TT |
|  |  | 60 |  |  |  |  |  |  | -83.1 +TT | -82.6 +TT | -82.4 +TT | -81.8 +TT |  | -81.3 +TT |
| 12 | n66 | 15 | -89.5 +TT | -88.8 +TT | -88.3 +TT | -87.8 +TT |  |  | -87.7 +TT |  |  |  |  |  |
|  |  | 30 |  | -89.1 +TT | -88.4 +TT | -88.0 +TT |  |  | -87.8 +TT |  |  |  |  |  |
|  |  | 60 |  | -89.5 +TT | -88.7 +TT | -88.2 +TT |  |  | -87.9 +TT |  |  |  |  |  |
| 12 | n78 | 15 |  | -85.4 +TT | -85.1 +TT | -84.9 +TT |  |  | -84.9 +TT | -84.9 +TT |  |  |  |  |
|  |  | 30 |  | -85.7 +TT | -85.2 +TT | -85.1 +TT |  |  | -85.0 +TT | -85.0 +TT | -84.9 +TT | -84.9 +TT | -84.9 +TT | -84.9 +TT |
|  |  | 60 |  | -86.1 +TT | -85.5 +TT | -85.3 +TT |  |  | -85.2 +TT | -85.1 +TT | -85.0 +TT | -85.0 +TT | -85.0 +TT | -85.0 +TT |
| 13 | n774, 5 | 15 |  | -84.9 +TT | -84.6 +TT | -84.4 +TT |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -85.2 +TT | -84.7 +TT | -84.6 +TT |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT |
|  |  | 60 |  | -85.6 +TT | -85 +TT | -84.8 +TT |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT |
| 18， 19 | n774,5 | 15 |  | -84.9 +TT | -84.6 +TT | -84.4 +TT |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -85.2 +TT | -84.7 +TT | -84.6 +TT |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT |
|  |  | 60 |  | -85.6 +TT | -85.0 +TT | -84.8 +TT |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT |
| 28 | n51 | 15 | -72.2 +TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 | n774,5 n784,5 | 15 |  | -84.9 +TT | -84.6 +TT | -84.4 +TT |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -85.2 +TT | -84.7 +TT | -84.6 +TT |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT |
|  |  | 60 |  | -85.6 +TT | -85.0 +TT | -84.8 +TT |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT |
| 20 | n776,7 n786,7 | 15 |  | -84.5 +TT | -84.4 +TT | -84.2 +TT |  |  | -83.1 +TT |  |  |  |  |  |
|  |  | 30 |  | -84.8 +TT | -84.5 +TT | -84.4 +TT |  |  | -83.2 +TT |  |  |  |  |  |
|  |  | 60 |  | -85.2 +TT | -84.8 +TT | -84.6 +TT |  |  | -83.4 +TT |  |  |  |  |  |
| 26 | n41 | 15 |  | -84.5 +TT | -84.6 +TT | -84.4 +TT |  |  | -83.6 +TT | -83.3 +TT | 3.9 +TT | 3.1 +TT | 2.7 +TT |  |
|  |  | 30 |  | -84.8 +TT | -84.7 +TT | -84.6 +TT |  |  | -83.7 +TT | -83.4 +TT | -83.0 +TT | -82.5 +TT | -82.4 +TT |  |
|  |  | 60 |  | -85.2 +TT | -85.0 +TT | -84.8 +TT |  |  | -83.9 +TT | -83.5 +TT | -83.2 +TT | -82.5 +TT | -82.4 +TT |  |
| 26 | n776,7 n786,7 | 15 |  | -85.0 +TT | -84.9 +TT | -84.7 +TT |  |  | -83.6 +TT | -84.6 +TT |  |  |  |  |
|  |  | 30 |  | -85.3 +TT | -85.0+TT | -84.9 +TT |  |  | -83.7 +TT | -84.7 +TT | -84.7 +TT | -84.6 +TT | -84.6 +TT | -84.6 +TT |
|  |  | 60 |  | -85.7 +TT | --85.3 +TT | --85.1 +TT |  |  | -83.9 +TT | -84.8 +TT | -84.8 +TT | -84.7 +TT | -84.7 +TT | -84.7 +TT |
| 26 | n774,5 | 15 |  | -84.9 +TT | -84.6 +TT | -84.4 +TT |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -85.2 +TT | -84.7 +TT | -84.6 +TT |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -85.6 +TT | -84.4 +TT |
|  |  | 60 |  | -85.6 +TT | -85.0 +TT | -84.8 +TT |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -85.7 +TT | -84.5 +TT |
| n28 | 18,9,10 | N/A | -89.1 | -88.7 | -88.3 | -88.0 |  |  |  |  |  |  |  |  |
| n71 | 211 | N/A | -92.7 | -93.3 | -91.8 | -90.7 |  |  |  |  |  |  |  |  |
| n71 | 212 | N/A | -95.6 | -93.3 | -91.8 | -90.7 |  |  |  |  |  |  |  |  |
| n71 | 7 | N/A | -82.7 | -82.6 | -82.4 | -82.3 |  |  |  |  |  |  |  |  |
| 66 | n772, 13 | 15 |  | -71.4 +TT | -71.4 +TT | -71.3 +TT |  |  | -71.2 +TT | -71.3 +TT |  |  |  |  |
|  |  | 30 |  | -71.7 +TT | -71.5 +TT | -71.5 +TT |  |  | -71.3 +TT | -71.4 +TT | -71.4 +TT | -71.3 +TT | -71.3 +TT | -71.3 +TT |
|  |  | 60 |  | -72.1 +TT | -71.8 +TT | -71.7 +TT |  |  | -71.5 +TT | -71.5 +TT | -71.5 +TT | -71.4 +TT | -71.4 +TT | -71.4 +TT |
|  | n773 | 15 |  | -94.2 +TT | -92.7 +TT | -91.9 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -94.5 +TT | -92.8 +TT | -92.1 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -94.9 +TT | -93.1 +TT | -92.3 +TT |  |  |  |  |  |  |  |  |
| 66 | n782 | 15 |  | -71.9 +TT | -71.9 +TT | -71.8 +TT |  |  | -71.7 +TT | -71.8 +TT |  |  |  |  |
|  |  | 30 |  | -72.2+TT | -72.0 +TT | -72.0 +TT |  |  | -71.8 +TT | -71.9 +TT | -71.9 +TT | -71.8 +TT | -71.8 +TT | -71.8 +TT |
|  |  | 60 |  | -72.6 +TT | -72.3 +TT | -72.2 +TT |  |  | -72.0 +TT | -72.0 +TT | -72.0 +TT | -71.9 +TT | -71.9 +TT | -71.9 +TT |
|  | n783 | 15 |  | -94.7 +TT | -93.2 +TT | -92.4 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -95.0 +TT | -93.3 +TT | -92.6 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -95.4 +TT | -93.6 +TT | -92.8 +TT |  |  |  |  |  |  |  |  |
| n66 | 48,2, 13 | N/A | -70.7 | -70.6 | -70.8 | -70.8 |  |  |  |  |  |  |  |  |
| n66 | 48,3 | N/A | -96.1 | -93.6 | -92.3 | -91.6 |  |  |  |  |  |  |  |  |
| NOTE 1: Void. NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , where andare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 5: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 7: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 9 The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 10: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. NOTE 11: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 12: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 13: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the EN-DC band combination: ∆FHD = 10 MHz for DC_1_n77, DC_2_n48, DC_2_n77, DC_42_n3, DC_48_n25, DC_48_n66, DC_66_n48, DC_66_n77, DC_3_n77, DC_3_n78, DC_11_n28 and DC_28_n50, DC_28_n51, DC_66_n78, DC_25_n77, DC_25_n78. NOTE 14: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 15: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. NOTE 16: Void NOTE 17: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.5-1a: Test Tolerance (TT) for RX sensitivity level

| f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0 GHz |
| --- | --- |
| 0.7 dB | 1.0 dB |

Reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1, are specified in Table 7.3B.2.3.5-2 with uplink configuration specified in Table 7.3B.2.3.4.2.1-3a to Table 7.3B.2.3.4.2.1-3k for each specific EN-DC combination scenario.

Table 7.3B.2.3.5-2: Reference sensitivity due to receiver harmonic mixing for EN-DC in NR FR1

| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | n714 | 15 | -70.4 +TT | -70.4 +TT | -70.4 +TT | -70.4 +TT |  |  |  |  |  |  |  |
|  |  | 30 |  | -70.7 +TT | -70.7 +TT | -71.8 +TT |  |  |  |  |  |  |  |
|  |  | 60 |  | -72.4 +TT | -72.7 +TT | -77.0 +TT |  |  |  |  |  |  |  |
| n41 | 264 | N/A | -72.5 | -69.5 | -69.5 | N/A |  |  |  |  |  |  |  |
| n77 | 2 | N/A | -91.2 | -89.3 | -88.5 | -87.6 |  |  |  |  |  |  |  |
|  |  |  | -93.910 | -9210 | -91.210 | -90.310 |  |  |  |  |  |  |  |
| n77 | 13 | N/A |  | -65.3 | -65.3 |  |  |  |  |  |  |  |  |
| n77 | 418 | N/A | -86.9 | -83.9 | -82.1 | -80.9 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| n77 | 282 | N/A | -69.8 | -69.8 | -69.8 | -68.3 |  |  |  |  |  |  |  |
| n78 | 3 | N/A | -90.6 | -89.3 | -88.5 | -87.6 |  |  |  |  |  |  |  |
|  |  |  | -93.310 | -92.010 | -91.210 | -90.310 |  |  |  |  |  |  |  |
| n78 | 40 | N/A | -86.9 | -83.9 | -82.1 | -80.9 |  |  |  |  |  |  |  |
| n78 | 418 | N/A | -86.9 | -83.9 | -82.1 | -80.9 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| n79 | 192 | N/A | -69.8 | -69.8 | -69.8 |  |  |  |  |  |  |  |  |
| n79 | 262 | N/A | -69.8 | -69.8 | -69.8 | N/A | N/A | N/A | N/A | N/A | N/A |  | N/A |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 3: Void. NOTE 4: The requirements should be verified for DL EARFCN or NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band andfULHB the UL carrier frequency in the higher band, both in MHz. NOTE 5: Void. NOTE 6: Void. NOTE 7: The requirements should be verified for DL EARFCN of the victim (higher) band (superscript HB) such that fDLHB=50×fULLB0.03 with fDLHB the DL carrier frequency in the higher band and fULLB the UL carrier frequency in the lower band, both in MHz. NOTE 8: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band andfULHBfULHBfDLLB=fULHB/0.150.1 the UL carrier frequency in the higher band, both in MHz. NOTE 9: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 10: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1, are specified in Table 7.3B.2.3.5-3 with uplink configuration specified in Table 7.3B.2.3.4.2.1-4a to Table 7.3B.2.3.4.2.1-4n.

Table 7.3B.2.3.5-3: Reference sensitivity exceptions due to cross band isolation for PC3 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| n1 | 3 | N/A | -93.3 | -91 | -89.5 | -88.5 |  |  |  |  |  |  |  |  |  |
| 1 | n3 | 15 | -94 +TT | -91.6 +TT | -90.1 +TT | -89.1 +TT | -88.1 +TT | -87.4 +TT | -86.2 +TT |  |  |  |  |  |  |
| 3 | n41 | 30 |  | -94.4 +TT | -92.4 +TT | -91.3 +TT |  | -89.3 +TT | -88.0 +TT | -87 +TT | -86.2 +TT |  | -84.9 +TT | -84.4 +TT | -84.0 +TT |
|  |  |  |  | -97.1 +TT4 | -95.1 +TT4 | -94 +TT4 |  | -92.0 +TT4 | -90.7 +TT4 | -89.7 +TT4 | -88.9 +TT4 |  | -87.6 +TT4 | -87.1 +TT4 | -86.7 +TT4 |
| n5 | 28 | 15 | -93.3 | -91.8 | -90.8 | -90 |  |  |  |  |  |  |  |  |  |
| n40 | 1 | N/A | -91.0 | -88.0 | -86.2 | -85.0 |  |  |  |  |  |  |  |  |  |
| n41 | 2 | N/A | -96.7 | -93.7 | -91.9 | -90.7 |  |  |  |  |  |  |  |  |  |
| n41 | 3 | N/A | 3 | n41 | 30 | 3 |  |  |  |  |  |  |  |  |  |
| n41 | 25 | N/A | -95.2 | -92.2 | -90.4 | -89.2 |  |  |  |  |  |  |  |  |  |
| n77 | 411 | N/A | -92.8 | -89.8 | -88.0 | -86.8 |  |  |  |  |  |  |  |  |  |
| n41 | 66 | N/A | -95.3 | -92.3 | -90.5 | -89.3 |  |  |  |  |  |  |  |  |  |
| 41 | n773 | 30 |  | -87.3 +TT | -85.3 +TT | -84.1 +TT |  |  | -82.9 +TT | -82.9 +TT | -82.9 +TT |  | -82.1 +TT | -81.7 +TT | -81.3 +TT |
| 3 | n51 | 15 | -93.6 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
| 30 | n66 | 15 | -91.2 +TT | -88.0 +TT | -86.2 +TT | -85.0 +TT |  |  |  | -81.8 +TT |  |  |  |  |  |
| n78 | 71 | N/A | -93.5 | -90.5 | -88.7 | -87.5 |  |  |  |  |  |  |  |  |  |
| n78 | 38 | N/A | -96.7 | -93.7 | -91.9 | -90.7 |  |  |  |  |  |  |  |  |  |
| n78 | 411 | N/A | -92.8 | -89.8 | -88.0 | -86.8 |  |  |  |  |  |  |  |  |  |
| 41 | n78 | 30 |  | -87.8 +TT | -85.8 +TT | -84.6 +TT |  |  | -83.4 +TT | -83.4 +TT | -83.4 +TT |  | -82.6 +TT | -82.2 +TT | -81.8 +TT |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 3: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.5-3a: Reference sensitivity exceptions due to cross band isolation for PC2 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| 3 | n41 | 30 |  | -94.4 +TT | -92.4 +TT | -91.3 +TT |  | -89.3 +TT | -88.0 +TT | -87.0 +TT | -86.2 +TT |  | -84.9 +TT | -84.4 +TT | -84.0 +TT |
|  |  |  |  | -97.1 +TT3 | -95.1 +TT3 | -94 +TT3 |  | -92.0 +TT3 | -90.7 +TT3 | -89.7 +TT3 | -88.9 +TT3 |  | -87.6 +TT3 | -87.1 +TT3 | -86.7 +TT3 |
| n41 | 3 | N/A | -94.0 | -91.0 | -89.2 | -88.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | -96.73 | -93.73 | -91.93 | -90.73 |  |  |  |  |  |  |  |  |  |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 3: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1, are specified in Table 7.3B.2.3.5-4 with uplink configuration specified in Table 7.3B.2.3.4.2.1-6.

Table 7.3B.2.3.5-4: Reference sensitivity exceptions due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 40 MHz (dBm) | IMD order | Duplex mode |
| DC_1A_n3A | 1 | N/A | [-76.3] | - | - | - | - | IMD3 | FDD |
|  | n3 | 15 | REFSENS |  | - | - | - | N/A | TDD |
| DC_1A_n8A | 1 | N/A | -93.3 | - | - | - | - | IMD4 | FDD |
|  | n8 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_1A_n77A | 1 | N/A | -69.5 | - | - | - | - | IMD23 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_1A_n77A | 1 | N/A | -91.3 | - | - | - | - | IMD4- | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_1A_n78A | 1 | N/A | -91.3 | - | - | - | - | IMD4 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_2A_n66A | 2 | N/A | -77.3 | - | - | - | - | IMD3 |  |
|  | n66 | 15 | REFSENS | - | - | - | - | N/A |  |
| DC_2A_n66A | 2 | N/A | REFSENS | - | - | - | - | N/A |  |
|  | n66 | 15 | -95.5 +TT | - | - | - | - | IMD5 |  |
| DC_2A_n77A | 2 | N/A | -71.3 | - | - | - | - | IMD2 | FDD |
| DC_2A-2A_n77A | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 2 | N/A | -89.3 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 2 | N/A | -92.3 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_2A_n78A | 2 | N/A | -71.37 | - | - | - | - | IMD23 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | - | TDD |
| DC_2A_n78A | 2 | N/A | -89.37 | - | - | - | - | N/A | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | IMD43 | TDD |
| DC_3A_n1A | 3 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n1 | 15 | -74.0+TT | - | - | - | - | IMD3 | FDD |
| DC_3A_n5A | 3 | N/A |  | -89.3 | - | - | - | IMD4 | FDD |
|  | n5 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 3 | N/A |  | REFSENS | - | - | - | N/A | FDD |
|  | n5 | 15 | -74.0+TT | - | - | - | - | IMD23 | FDD |
| DC_3A_n7A | 3 | N/A | REFSENS | - | - | - | - | - |  |
|  | n7 | 15 | - | -84.6 +TT5 | - | - | - | - |  |
| DC_3A_n8A | 3 | N/A | - | REFSENS | - | - | - | N/A | FDD |
|  | n8 | 15 | -88.3 | - | - | - | - | IMD43 | FDD |
|  | 3 | N/A | - | -86.9 | - | - | - | IMD5 | FDD |
|  | n8 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| CA_3A-n20A | 3 | N/A | -92.3 | - | - | - | - | IMD4 | FDD |
|  | n20 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 3 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n20 | 15 | -88.0+TT | - | - | - | - | IMD4 | FDD |
| DC_3A_n41A | 3 | N/A | -88.1 | - | - | - | - | IMD4 | FDD |
|  |  |  | -90.87 | - | - | - | - |  |  |
| DC_3A_n77A, DC_3A_n77(2A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) | 3 | N/A | -70.3 | - | - | - | - | IMD23 | FDD |
|  |  |  | -70.67 | - | - | - | - |  |  |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n77A, DC_3A_n77(2A), DC_3C_n77A, DC_3C_n77(2A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) | 3 | N/A | -88.3 | - | - | - | - | IMD43 | FDD |
|  |  |  | -88.37 | - | - | - | - |  |  |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_5A_n66A | 5 | N/A | -67.3 | - | - | - | - | IMD23 | FDD |
|  | n66 |  | REFSENS | - | - | - | - | N/A | FDD |
| DC_5A_n77A8 | 5 | N/A | -89 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 5 | N/A | -91.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_5A_n78A | 5 | N/A | -89.0 | - | - | - | - | IMD4 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_7A_n3A | 7 | N/A | - | -81.3 | - | - | - | IMD4 | FDD |
|  | n3 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_7A_n5A | 7 | N/A |  | REFSENS | - | - | - | N/A | FDD |
|  | n5 | 15 | -86.0+TT | - | - | - | - | IMD33 | FDD |
| DC_7A_n66A | 7 | N/A | - | -79.3 | - | - | - | IMD4 | FDD |
|  | n66 | 15 | REFSENS | - |  | - | - | N/A | TDD |
| DC_7A_n77A | 7 | N/A | -90.2 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_8A_n1A | 8 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n1 | 15 | -94.0 + TT | - | - | -- | - | IMD4 | FDD |
| DC_8A_n3A | 8 | N/A | -88.3 | - | - | - | - | IMD43 | FDD |
|  | n3 | 15 | - | REFSENS | - | - | - | N/A | FDD |
|  | 8 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n3 | 15 | - | -87.4+ TT | - | - | - | IMD5 | FDD |
| DC_8A_n20A | n20 | 15 | -71.3 | - | - | - | - | IMD3 | FDD |
|  | 8 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n20 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 8 | N/A | -71.3 | - | - | - | - | IMD3 | FDD |
| DC_8A_n41A | 8 | N/A | -84.2 | - | - | - | - | IMD33 | FDD |
|  | n41 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_8A_n77A DC_8A_n78A DC_8A-SUL_n78A-n81A | 8 | N/A | -88.0 | - | - | - | - | IMD4 | FDD |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | H4 | TDD |
| DC_8A_n79A DC_8A-SUL_n79A-n81A | 8 | N/A | -91.5 | - | - | - | - | IMD5 | FDD |
|  | n79 | 15 | - | - | - | - | REFSENS | N/A | TDD |
| DC_12_n78 | 12 | N/A | -90.8 | - | - | - | - | IMD5 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_13A_n77A | 13 | N/A | -90.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_20A_n3A | 20 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n3 | 15 | -93.0 +TT | - | - | - | - | IMD4 | FDD |
|  | 20 | N/A | -87.3 | - | - | - | - | IMD4 | FDD |
|  | n3 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_20_n7 | 20 | N/A | -84.3 | - | - | - | - | IMD3 | FDD |
|  | n7 | 15 | - | REFSENS | - | - | - | N/A | FDD |
| DC_20_n8 | 20 | N/A | -71.3 | - | - | - | - | IMD3 | FDD |
|  | n8 | 15 | -71.3 | - | - | - | - | IMD3 | FDD |
| DC_20A_n77A | 20 | N/A | -85.3 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 20 | N/A | -89.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_20A_n78A, DC_20A-SUL_n78A-n82A | 20 | N/A | -74.6 | - | - | - | - | IMD44 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_21A_n79A | 21 | N/A | -80.9 | - | - | - | - | IMD3 | FDD |
|  | n79 | 15 | - | - | - | - | REFSENS | N/A | TDD |
| DC_28_n50 | 28 | N/A | - | -79.5 | - | - | - | IMD 2 | FDD |
|  | n50 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 28 | N/A | - | -88.8 | - | - | - | IMD 4 | FDD |
|  | n50 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 28 | N/A | - | -94.3 | - | - | - | IMD 5 | FDD |
|  | n50 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| CA_28A_n77A, CA_28A_n78A, DC_28A-SUL_n78A-n83A | 28 | N/A | -92.3 | - | - | - | - | IMD5 | FDD |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_48A_n66A | 48 | N/A | - | REFSENS | - | - | - | N/A | TDD |
|  | n66 | 15 | -95.5 +TT | - | - | - | - | IMD5 | FDD |
| DC_66A_n2A | 66 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n2 | 15 | -78.0+TT | - | - | - | - | IMD3 | FDD |
|  | 66 | N/A | -94.8 | - | - | - | - | IMD5 | FDD |
|  | n2 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n5A | 66 | N/A | -68.8 | - | - | - | - | IMD23 | FDD |
|  | n5 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n25A | 66 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n25 | 15 | -76.5+TT | - | - | - | - | IMD3 | FDD |
|  | 66 | N/A | -75.8 | - | - | - | - | IMD3 | FDD |
|  | n25 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 66 | N/A | -94.8 | - | - | - | - | IMD5 | FDD |
|  | n25 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n71A | 66 | N/A | -93.8 | - | - | - | - | IMD4 | FDD |
|  | n71 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n77A | 66 | N/A | -67.8 | - | - | - | - | IMD2 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 66 | N/A | -93.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_66A_n78A | 66 | N/A | -93.8 | - | - | - | - | IMD5 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| NOTE 1: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1 for 2 antenna port NR band and Table 7.3.2.5-2 for 4 antenna port NR band. NOTE 5: Void NOTE 6: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 7: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.5-5: Reference sensitivity exceptions due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band |  | SCS (kHz) |  | 5 MHz (dBm) |  | 10 MHz (dBm) | 15 MHz (dBm)) | 20 MHz (dBm) | 40 MHz (dBm) | IMD order) | Duplex mode |
| DC_1A_n78A | 1 |  | N/A |  | REFSENS-17.8 |  | - | - | - | - | IMD4 |  |
|  | n78 |  | 15 |  | - |  | REFSENS | - | - | - | N/A | TDD |
| DC_2A_n77A |  | 2 |  | N/A |  | -65.2 | - | - | - | - | IMD2 | FDD |
| DC_2A-2A_n77A |  |  |  |  |  | -65.157 |  |  |  |  |  |  |
| DC_2A_n77C |  | n77 |  | 15 |  | - | REFSENS | - | - | - | N/A | TDD |
| DC_2A-2A_n77C |  | 2 |  | N/A |  | -78.2 | - | - | - | - | IMD49 | FDD |
|  |  |  |  |  |  | -78.157 |  |  |  |  |  |  |
|  |  | n77 |  | 15 |  | - | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n41A | 3 |  | N/A |  | -77.9 |  | - | - | - | - | IMD4 | FDD |
|  |  |  |  |  | -80.67 |  | - | - | - | - |  |  |
|  | n41 |  | 15 |  | - |  | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n78A | 3 |  | N/A |  | -64.4 |  | - | - | - | - | IMD2 | FDD |
|  |  |  |  |  | -67.17 |  | - | - | - | - |  |  |
|  | n78 |  | 15 |  | - |  | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n78A | 3 |  | N/A |  | -77.8 |  | - | - | - | - | IMD4 | FDD |
|  |  |  |  |  | -80.57 |  | - | - | - | - |  |  |
|  | n78 |  | 15 |  | - |  | REFSENS | - | - | - | N/A | TDD |
| DC_5A_n77A8 |  | 5 |  | N/A |  | -78.7 | - | - | - | - | IMD49 | FDD |
| DC_5A_n77C8 |  | n77 |  | 15 |  | - | REFSENS | - | - | - | N/A | TDD |
| DC_13A_n77A |  | 13 |  | N/A |  | -80.93 | - | - | - | - | IMD5 | FDD |
| DC_13A_n77C |  | n77 |  | 15 |  | - | REFSENS | - | - | - | N/A | TDD |
| DC_66A_n77A DC_66A-66A_n77A DC_66A-66A-66A_n77A DC_66A_n77C DC_66A-66A_n77C DC_66A-66A-66A_n77C |  | 66 |  | N/A |  | -64.47 | - | - | - | - | IMD2 | FDD |
|  |  | n77 |  | 15 |  | - | REFSENS | - | - | - | N/A | TDD |
|  |  | 66 |  | N/A |  | -87.53 | - | - | - | - | IMD5 | FDD |
|  |  | n77 |  | 15 |  | - | REFSENS | - | - | - | N/A | TDD |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in clause 6.2.5A. In case Single UL is allowed and the UE only indicates support of "Single UL" the output power of the active UL shall be set at PCMAX_L,c or set to the maximum output power according to the UE power scaling capability. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1 for 2 antenna port NR band and Table 7.3.2.5-2 for 4 antenna port NR band. NOTE 5: For UEs only indicating support of Single UL, this requirement is verified with non-simultaneous uplink transmissions on the E-UTRA and NR CGs. NOTE 6: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 7: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 9: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections skipped>>

<< End of changes >>