3GPP TSG-RAN5 Meeting #97 	R5-227074r1

# Toulouse,  France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0842 | rev | - | Current version: | 17.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Definition of PC1 MU and TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For FR2 PC1 test cases, MTSU/TT is not defined or [ ] is remaining. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the following test cases, [ ] was removed from PC1 MTSU/TT value and editor’s note was corrected. - MOP (EIRP, TRP) - OFF power - Reference sensitivity - ACS - In-band blocking In the following test cases, PC1 MTSU/TT value was defined and editor’s note was corrected. - OBW - SEM - ACLR - General Tx Spurious - Spurious co-existence - Rx Spurious Editorial correction to frequency range of spurious test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FR2 PC1 test cases cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.1, 6.3.2, 6.5.1, 6.5.3, 7.3.2, 7.5, 7.6.2, 7.9, F.1, F.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depending on the outcome of MU discussion R5-227073 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: - Corrected current version 17.0.0 -> 17.0.1 - Changes for MOP (EIRP, TRP) (6.2.1.1), OFF power (6.3.2), OBW (6.5.1), SEM (6.5.2.1), ACLR (6.5.2.3), Reference sensitivity (7.3.2), ACS (7.5), In-band blocking (7.6.2) were reverted to solve overlapping with R5-226089, R5-226090, R5-226092, R5-226094, R5-226095, R5-226096. Reverted sections and rows were deleted from this paper. - MTSU values in Spurious test cases were corrected. |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

6.5.3.1	Transmitter Spurious emissions

Editor’s Note: This clause is complete for Band n257, n258, n260 and n261 and PC3, and Band n257 and PC1. The following aspects of the clause are for future consideration:

-	TRP Measurement uncertainty is TBD for above 80 GHz.

-	Connection diagram between SS and UE in TS 38.508-1 [10] Annex A is FFS.

-	Test procedure only includes the testing of smartphone and  is FFS for laptop and FWA.

<<Unchanged sections skipped>>

6.5.3.2	Spurious emission band UE co-existence

Editor’s note: This clause is complete for Band n257, n258, n260 and n261 and PC3, and Band n257 and PC1. The following aspects of the clause are for future consideration:

- TRP Measurement uncertainty is TBD for PC2 and PC4.

-	Connection diagram between SS and UE in TS 38.508-1 [10] Annex A is FFS.

- Test procedure only includes the testing of smartphone and  is FFS for laptop and FWA.

<<Unchanged sections skipped>>

7.9	Spurious emissions

Editor's note:	Following aspects are either missing or not yet determined:

- The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n260 and n261.

- Measurement Uncertainties and Test Tolerances are FFS for power class 2, and 4.

- Connection diagram between SS and UE in TS 38.508-1 [10] Annex A is FFS.

- Test procedure only includes the testing of smartphone and  is FFS for laptop and FWA.

<<Unchanged sections skipped>>

F.1	Acceptable uncertainty of Test System (normative)

The maximum acceptable uncertainty of the Test System is specified below for each test, where appropriate. The Test System shall enable the stimulus signals in the test case to be adjusted to within the specified range, and the equipment under test to be measured with an uncertainty not exceeding the specified values. Care should be taken to ensure that each conformance test implementation including the OTA chamber aspects meets the specified measurement uncertainty for each test case by requiring the test laboratory to maintain a detailed measurement uncertainty test report showing compliance to all the measurement uncertainty requirements. The detailed measurement uncertainty report would contain the justification for each measurement uncertainty component and its value and distribution. The derivation of these values is based on the minimum conformance requirements plus relaxation, i.e., test tolerance is not to be considered. All ranges and uncertainties are absolute values, and are valid for a confidence level of 95 %, unless otherwise stated.

A confidence level of 95 % is the measurement uncertainty tolerance interval for a specific measurement that contains 95 % of the performance of a population of test equipment.

The downlink signal uncertainties apply at the defined quiet zone with the UE properly positioned in the quiet zone. The uplink signal uncertainties apply at the measurement equipment with the UE positioned properly in the quiet zone.

F.1.1	Measurement of test environments

Editor’s note: Various measurement accuracies for UE test environments, e.g., pressure, relative humidity, DC&AC voltage, vibration, and vibration frequency, are FFS:

The measurement accuracy of the UE test environments defined in TS 38.508-1 [5] subclause 4.1, Test environments shall be

-	Temperature			±4 degrees.

The above values shall apply unless the test environment is otherwise controlled and the specification for the control of the test environment specifies the uncertainty for the parameter.

F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty (MTSU) for transmitter tests

| Sub clause | Maximum Test System Uncertainty | Derivation of MTSU |
| --- | --- | --- |
| <<Unchanged rows omitted>> |  |  |  |
| 6.5.3.1 Transmitter Spurious emissions | Max Device size ≤ 30 cm Maximum in-band BW ≤ 400MHz PC3: ±5.14 dB (6GHz ≤ f < 12.75GHz) ±5.11 dB (12.75GHz ≤ f < 23.45GHz) ±5.41 dB (23.45GHz ≤ f < 40.8GHz) ±7.42 dB (40.8GHz ≤ f < 66GHz) ±7.72 dB (66GHz ≤ f ≤ 80GHz) PC1: ±5.84 dB (6GHz ≤ f < 12.75GHz) ±6.38 dB (12.75GHz ≤ f < 23.45GHz) ±6.52 dB (23.45GHz ≤ f < 40.8GHz) ±8.42 dB (40.8GHz ≤ f < 66GHz) ±TBD dB (66GHz ≤ f ≤ 80GHz) | MTSU = 1.00 x MU (from Table B.18-1 in TR 38.903) |
| 6.5.3.2 Spurious emission band UE co-existence | Max Device size ≤ 30 cm Maximum in-band BW ≤ 400MHz PC3: Protected band n260, n261, n257: ±6.00 dB Protected frequency 23.6 GHz ≤ f ≤ 24.0 GHz:±6.00 dB Protected frequency 57 GHz ≤ f ≤ 66GHz: ±8.01 dB Protected frequency 36 GHz ≤ f ≤ 37GHz: ±6.00 dB PC1: Protected frequency 23.6 GHz ≤ f ≤ 24.0 GHz:±7.68 dB | MTSU = 1.00 x MU (from Table B.18-1a in TR 38.903) |
| <<Unchanged rows omitted>> |  |  |  |
| NOTE 1: FR2a: 23.45GHz ≤ f ≤ 32.125GHz FR2b: 32.125GHz ≤ f ≤ 40.8GHz FR2c: 40.8GHz ≤ f ≤ 44.3GHz |  |  |

Table F.1.2-2: EVM Measurement Uncertainty (MU) for PUSCH, PC3, FR2a (23.45GHz <= f <= 32.125GHz)

| Test ID | Modulation | RB alloc. | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 2.78% | 3.85% | 5.44% | 7.69% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 3.10% | 4.16% | 5.88% | 8.99% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 2.78% | 3.85% | 5.44% | 7.69% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 3.10% | 4.16% | 5.88% | 8.99% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 3.31% | 4.50% | 6.36% | 11.21% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 3.60% | 4.73% | 6.68% | 11.21% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 4.26% | 5.96% | 8.41% | 15.84% |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 5.01% | 7.08% | 9.99% | 15.84% |
| 9 | CP-OFDM QPSK | Inner_Full | 3.60% | 4.73% | 6.68% | 11.89% |
| 10 | CP-OFDM QPSK | Outer_Full | 3.71% | 4.99% | 7.07% | 11.89% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 4.26% | 5.96% | 8.41% | 15.84% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 4.26% | 5.96% | 8.41% | 15.84% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 6.31% | 8.91% | 12.59% | 21.13% |
| 14 | CP-OFDM 64 QAM | Outer_Full | 6.31% | 8.91% | 12.59% | 21.13% |

Table F.1.2-3: EVM Measurement Uncertainty (MU) for PUSCH, PC3, FR2b (32.125GHz < f <= 40.8GHz)

| Test ID | Modulation | RB alloc. | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 3.56% | 4.83% | 6.91% | 9.65% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 4.15% | 5.69% | 8.11% | 12.50% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 3.56% | 4.83% | 6.91% | 9.65% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 4.15% | 5.69% | 8.11% | 12.50% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 4.54% | 6.26% | 8.91% | 18.06% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 5.09% | 7.19% | 10.15% | 18.06% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 6.78% | 9.58% | 13.54% | 25.50% |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 8.06% | 11.38% | 16.09% | 25.50% |
| 9 | CP-OFDM QPSK | Inner_Full | 5.09% | 7.19% | 10.15% | 19.13% |
| 10 | CP-OFDM QPSK | Outer_Full | 5.39% | 7.61% | 10.75% | 19.13% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 6.78% | 9.58% | 13.54% | 25.50% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 6.78% | 9.58% | 13.54% | 25.50% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 10.14% | 14.33% | 20.25% | 34.01% |
| 14 | CP-OFDM 64 QAM | Outer_Full | 10.14% | 14.33% | 20.25% | 34.01% |

F.1.3	Measurement of receiver

Table F.1.3-1: Maximum Test System Uncertainty (MTSU) for receiver tests

| Sub clause | Maximum Test System Uncertainty | Derivation of MTSU |
| --- | --- | --- |
| <<Unchanged rows omitted>> |  |  |  |
| 7.9 Spurious emissions | Max Device size ≤ 30 cm Maximum in-band BW ≤ 400MHz PC3: For Band n257, n258, n260, n261: ±5.50dB (6GHz ≤ f < 12.75GHz) ±5.46dB (12.75GHz ≤ f < 23.45GHz) ±6.11dB (23.45GHz ≤ f < 40.8GHz) ±7.65dB (40.8GHz ≤ f < 66GHz) ±7.95 dB (66GHz ≤ f ≤ 80GHz) PC1: For Band n257, n258, n260, n261: ±6.18dB (6GHz ≤ f < 12.75GHz) ±6.15dB (12.75GHz ≤ f < 23.45GHz) ±6.64dB (23.45GHz ≤ f < 40.8GHz) ±8.08dB (40.8GHz ≤ f < 66GHz) ±TBD dB (66GHz ≤ f ≤ 80GHz) | MTSU = 1.00 x MU (from Table B.25-1 in TR 38.903) |
| NOTE 1: FR2a, FR2b and FR2c are specified in Table F.1.2-1. |  |  |

<<Unchanged sections skipped>>

F.3	Test Tolerance and Derivation of Test Requirements (informative)

TBD

F.3.1	Measurement of test environments

TBD

F.3.2	Measurement of transmitter

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Influence of noise is subtracted from MTSU before calculating the TT for lower limit Tx test cases.

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| <<Unchanged rows omitted>> |  |  |  |
| 6.5.3.1 Transmitter Spurious emissions | 0 dB | Minimum requirement + TT |
| 6.5.3.2 Spurious emission band UE co-existence | 0 dB | Minimum requirement + TT |
| <<Unchanged rows omitted>> |  |  |  |
| NOTE 1: FR2a: 23.45GHz ≤ f ≤ 32.125GHz FR2b: 32.125GHz ≤ f ≤ 40.8GHz FR2c: 40.8GHz ≤ f ≤ 44.3GHz |  |  |

F.3.3	Measurement of receiver

Table F.3.3-1: Derivation of Test Requirements (Receiver tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| <<Unchanged rows omitted>> |  |  |  |
| 7.9 Spurious emissions | 0 dB | Minimum requirement + TT T-put limit unchanged |
| NOTE 1: FR2a, FR2b and FR2c are specified in Table F.3.2-1. |  |  |

<<End of change>>