3GPP TSG-RAN5 Meeting #97	R5-227159r3

# Toulouse, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3393 | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of RedCap TC 8.1.3.1.24-NCD-SSB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RedCap TC 8.1.3.1.24 is not completed |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of RedCap TC 8.1.3.1.24 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RedCap TC 8.1.3.1.24 can’t be executed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.1.24(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | 1st revision: (1) In Table 8.1.3.1.24.3.2-5: Main behaviour step 9 should refer to TP 2 (2) Add specific pucch-configcommon message content for RedCap initialUplinkBWP to set pucch-ResourceCommonRedCap-r17=0 and set pucch-ResourceCommon to not present. (3)Config a common coreset for RedCap initialDownlinkBWP because the RedCap initialDownlinkBWP does not include Coreset#0 and CD-SSB. And set searchspaceSIB1/ searchspaceothersysteminformation /pagingsearchspace/ controlResourcesetZero/searchspaceZero in the RedCap BWP to not present. searchSpaceSIB1 ID of the search space for SIB1 message. In the initial DL BWP of the UE′s PCell, the network sets this field to 0. If the field is absent, the UE does not receive SIB1 in this BWP. (see TS 38.213 [13], clause 10). This field is absent for the RedCap-specific initial DL BWP, if it does not include CD-SSB and the entire CORESET#0. searchSpaceOtherSystemInformation ID of the Search space for other system information, i.e., SIB2 and beyond (see TS 38.213 [13], clause 10.1) If the field is absent, the UE does not receive other system information in this BWP. This field is absent for the RedCap-specific initial DL BWP, if it does not include CD-SSB and the entire CORESET#0. pagingSearchSpace ID of the search space for paging (see TS 38.213 [13], clause 10.1). If the field is absent, the UE does not receive paging in this BWP (see TS 38.213 [13], clause 10). This field is absent for the RedCap-specific initial downlink BWP, if it does not include CD-SSB and the entire CORESET#0. In that case, a RedCap UE shall monitor paging in the initial DL BWP that includes CORESET#0 commonControlResourceSet An additional common control resource set which may be configured and used for any common or UE-specific search space. If the network configures this field, it uses a ControlResourceSetId other than 0 for this ControlResourceSet. The network configures the commonControlResourceSet in SIB1 so that it is contained in the bandwidth of CORESET#0. If the RedCap-specific initial downlink BWP does not contain the entire CORESET#0, the network configures the commonControlResourceSet in SIB1 for RedCap so that it is not contained in the bandwidth of CORESET#0. (4)rearrange the number of the message content table. 2nd revision: - Delete QuantityConfig message content and directly use the table in the 508. - Configure the servingCellMO-r17 together with measconfig. 3rd revision: - Delete the servingCellMO-r17 in RRC Setup |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

8.1.3.1.24	Measurement configuration control and reporting / Intra NR measurements / NCD-SSB / RedCap

8.1.3.1.24.1	Test Purpose (TP)

(1)

with { UE supporting RedCap and in NR RRC_CONNECTED state and initialDownlinkBWP-RedCap does not contain the entire CORESET#0 and measurement object for serving cell measurement using NCD-SSB and measurement configured for event A1 with event based periodical reporting }

ensure that {

when { Serving cell becomes better than absolute threshold plus hysteresis and entering condition for event A1 is met  }

then { UE sends MeasurementReport message at regular intervals while entering condition for event A1 is satisfied }

}

(2)

with { UE in NR RRC_CONNECTED and periodical measurement reporting triggered by event A1 ongoing }

ensure that {

when { Serving cell becomes worse than absolute threshold minus hysteresis }

then { UE stops sending MeasurementReport message }

}

8.1.3.1.24.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.300, clauses 16.13.5, and TS 38.331, clause 6.3.2. Unless otherwise stated these are Rel-17 requirements.

[TS 38.300, clause 16.13.5]

A RedCap UE may be configured with multiple NCD-SSBs provided that each BWP is configured with at most one SSB. NCD-SSB may be configured for a RedCap UE in RRC_CONNECTED to perform RLM, BFD, and RRM measurements when the active BWP does not contain CD-SSB.

[TS 38.331, clause 6.3.2]

DownlinkConfigCommonSIB field descriptions

initialDownlinkBWP-RedCap If present, RedCap UEs use this DL BWP instead of initialDownlinkBWP. If the locationAndBandwidth of this BWP contains the entire CORESET#0, the UE applies the locationAndBandwidth upon reception of this field (e.g. to determine the frequency position of signals described in relation to this locationAndBandwidth) but it keeps CORESET#0 until after reception of RRCSetup/RRCResume/RRCReestablishment. Otherwise, i.e., if the locationAndBandwidth of this BWP does not contain the entire CORESET#0, the UE uses this BWP for receiving DL messages during initial access (Msg2, MsgB, Msg4) and after initial access. If absent, RedCap UEs use initialDownlinkBWP provided that it does not exceed the RedCap UE maximum bandwidth (see also clause 5.2.2.4.2).

BWP-DownlinkDedicated field descriptions

nonCellDefiningSSB If configured, the RedCap UE operating in this BWP uses this SSB for the purposes for which it would otherwise have used the cell-defining SSB of the serving cell (e.g. obtaining sync, measurements, RLM). Furthermore, other parts of the BWP configuration that refer to an SSB (e.g. the "SSB" configured in the QCL-Info IE; the "ssb-Index" configured in the RadioLinkMonitoringRS; CFRA-SSB-Resource; PRACH-ResourceDedicatedBFR) refer implicitily to this NCD-SSB. The NCD-SSB has the same values for the properties (e.g., ssb-PositionsInBurst, PCI, ssb-periodicity, ssb-PBCH-BlockPower) of the corresponding CD-SSB apart from the values of the properties configured in the NonCellDefiningSSB-r17 IE.

servingCellMO measObjectId of the MeasObjectNR in MeasConfig which is associated to the serving cell. For this MeasObjectNR, the following relationship applies between this MeasObjectNR and nonCellDefiningSSB in BWP-DownlinkDedicated of the associated downlink BWP: if ssbFrequency is configured, its value is the same as the absoluteFrequencySSB in the nonCellDefiningSSB. If the field is present in a downlink BWP and the BWP is activated, the RedCap UE uses this measurement object for serving cell measurements, otherwise, the RedCap UE uses the servingCellMO in ServingCellConfig IE.

8.1.3.1.24.3	Test description

8.1.3.1.24.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is configured to use Test frequency NRf1 as specified in TS 38.508-1 [4] clause 4.3.1 using the common highest mandatory UL and DL channel bandwidth and using the default subcarrier spacing specified in TS 38.508-1 [4] clause 6.2.3.1.

UE:

-	None

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

8.1.3.1.24.3.2	Test procedure sequence

Table 8.1.3.1.24.3.2-1/2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" and "T2", are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.3.1.24.3.2-1: Time instances of cell power level and parameter changes in FR1

|  | Parameter | Unit | NR Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -90 | Power level is such that entry condition for event A1 is not satisfied. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -78 | Power level is such that entry condition for event A1 is satisfied Ms – Hys > Thresh |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -94 | Power level is such that exit condition for event A1 is satisfied Ms + Hys < Thresh |

Table 8.1.3.1.24.3.2-2: Time instances of cell power level and parameter changes in FR2

|  | Parameter | Unit | NR Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | FFS | Power level is such that entry condition for event A1 is not satisfied. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | FFS | Power level is such that entry condition for event A1 is satisfied Ms – Hys > Thresh |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | FFS | Power level is such that exit condition for event A1 is satisfied Ms + Hys < Thresh |

Table 8.1.3.1.24.3.2-3/4 illustrates the specific parameters for NCD-SSB and initialDownlinkBWP-RedCap.

Table 8.1.3.1.24.3.2-3: Specific Parameters in FR1

| NR Band | SCS [kHz] Note1 | CBW [MHz] Note2 | SCS for NCD-SSB[kHz] | absoluteFrequencySSB for NCD-SSB [ARFCN] | Rbstart of initialDownlinkBWP-RedCap | LRB of initialDownlinkBWP-RedCap | locationAndBandwidth of initialDownlinkBWP-RedCap |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | 15 | 50 | 15 | 428332 | 162 | 106 | 29037 |
| n2 | 15 | 40 | 15 | 390360 | 108 | 106 | 28983 |
| n3 | 15 | 50 | 15 | 367332 | 162 | 106 | 29037 |
| n25 | 15 | 40 | 15 | 390360 | 108 | 106 | 28983 |
| n28 | 15 | 30 | 15 | 154024 | 54 | 106 | 40970 |
| n66 | 15 | 40 | 15 | 426360 | 108 | 106 | 28983 |
| n38 | 30 | 40 | 30 | 518792 | 54 | 51 | 13804 |
| n39 | 30 | 40 | 30 | 380792 | 54 | 51 | 13804 |
| n40 | 30 | 80 | 30 | 472572 | 162 | 51 | 13912 |
| n48 | 30 | 40 | 30 | 638264 | 54 | 51 | 13804 |
| n50 | 30 | 80 | 30 | 298972 | 162 | 51 | 13912 |
| n41 | 30 | 100 | 30 | 516078 | 222 | 51 | 13972 |
| n77 | 30 | 100 | 30 | 625626 | 222 | 51 | 13972 |
| n78 | 30 | 100 | 30 | 625626 | 222 | 51 | 13972 |
| n79 | 30 | 100 | 30 | 698960 | 222 | 51 | 13972 |
| Note 1: Subcarrier spacing is configured according to TS 38.508-1 [4] clause 6.2.3.1. Note 2: Channel bandwidth is configured according to highest mandatory UL and DL channel bandwidth in TS 38.508-1 [4] clause 4.3.1. |  |  |  |  |  |  |  |

Table 8.1.3.1.24.3.2-4: Specific Parameters in FR2

| NR Band | SCS [kHz] Note1 | CBW [MHz] Note2 | SCS for NCD-SSB[kHz] | absoluteFrequencySSB for NCD-SSB [ARFCN] | Rbstart of initialDownlinkBWP-RedCap | LRB of initialDownlinkBWP-RedCap | locationAndBandwidth of initialDownlinkBWP-RedCap |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n257 | 120 | 400 | 120 | 2059323 | 198 | 66 | 18073 |
| n258 | 120 | 400 | 120 | 2021823 | 198 | 66 | 18073 |
| n259 | 120 | 400 | 120 | 2275991 | 198 | 66 | 18073 |
| n260 | 120 | 400 | 120 | 2234323 | 198 | 66 | 18073 |
| n261 | 120 | 400 | 120 | 2075991 | 198 | 66 | 18073 |
| Note 1: Subcarrier spacing is configured according to TS 38.508-1 [4] clause 6.2.3.1. Note 2: Channel bandwidth is configured according to highest mandatory UL and DL channel bandwidth in TS 38.508-1 [4] clause 4.3.1. |  |  |  |  |  |  |  |

Table 8.1.3.1.24.3.2-5: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCReconfiguration message including measConfig to setup intra NR measurement and reporting for event A1. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigrationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | SS re-adjusts the SS/PBCH EPRE level according to row "T1" in Table 8.1.3.1.24.3.2-1/2. | - | - | - | - |
| 4 | Check: Does the UE transmit a MeasurementReport message to report event A1 with the measured RSRP and RSRQ value for NR Cell 1? | --> | NR RRC: MeasurementReport | 1 | P |
| 5 | EXCEPTION: Step 4 below is repeated until 3 MeasurementReport messages are received from the UE and Interval between two MeasurementReport is same as the IE reportInterval configured in measConfig. | - | - | - | - |
| 6 | Check: Does the UE transmit a MeasurementReport message at regular intervals, with the measured RSRP and RSRQ value for NR Cell 1? | --> | NR RRC: MeasurementReport | 1 | P |
| 7 | SS re-adjusts the SS/PBCH EPRE level according to row "T2" in Table 8.1.3.1.24.3.2-1/2. | - | - | - | - |
| 8 | Wait and ignore MeasurementReport messages for 15 s to allow change of power levels for NR Cell 1 and UE measurement. | - | - | - | - |
| 9 | Check: Does the UE attempt to transmit MeasurementReport message within the next 10s? | - | - | 2 | F |

8.1.3.1.24.3.3	Specific message contents

Table 8.1.3.1.24.3.3-1: SIB1 (preamble and all step, Table 8.1.3.1.24.3.2-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| servingCellConfigCommon |  | ServingCellConfigCommonSIB | Table 8.1.3.1.24.3.3-2 |  |
| } |  |  |  |  |

Table 8.1.3.1.24.3.3-2: ServingCellConfigCommonSIB (Table 8.1.3.1.24.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB | Table 8.1.3.1.24.3.3-3 |  |
| uplinkConfigCommon-v1700 SEQUENCE { |  |  |  |
| initialUplinkBWP-RedCap-r17 | BWP-UplinkCommon-RedCap | Table 8.1.3.1.24.3.3-6 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.1.24.3.3-3: DownlinkConfigCommonSIB (Table 8.1.3.1.24.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-53 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommonSIB ::= SEQUENCE { |  |  |  |
| pei-Config-r17 | Not present |  |  |
| initialDownlinkBWP-RedCap-r17 | BWP-DownlinkCommon-RedCap | Table 8.1.3.1.24.3.3-4 |  |
| } |  |  |  |

Table 8.1.3.1.24.3.3-4: BWP-DownlinkCommon-RedCap (Table 8.1.3.1.24.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-10 with condition InitialBWP_SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkCommon ::= SEQUENCE { |  |  |  |
| genericParameters | BWP-RedCap |  |  |
| pdcch-ConfigCommon CHOICE { |  |  |  |
| setup | PDCCH-ConfigCommon-RedCap | Table 8.1.3.1.24.3.3-5 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.1.24.3.3-5: PDCCH-ConfigCommon-RedCap (Table 8.1.3.1.24.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-96 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| controlResourceSetZero | Not present |  |  |
| commonControlResourceSet | ControlResourceSet-RedCap | Table 8.1.3.1.24.3.3-6 |  |
| searchSpaceZero | Not present |  |  |
| commonSearchSpaceList SEQUENCE (SIZE (1..4)) OF SearchSpace { | 1 entry |  |  |
| SearchSpace[1] | SearchSpace with condition CSS | entry 1 Table 8.1.3.1.24.3.3-7 |  |
| } |  |  |  |
| searchSpaceSIB1 | Not present |  |  |
| searchSpaceOtherSystemInformation | Not present |  |  |
| pagingSearchSpace | Not present |  |  |
| } |  |  |  |

Table 8.1.3.1.24.3.3-6: ControlResourceSet (Table 8.1.3.1.24.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| } |  |  |  |

Table 8.1.3.1.24.3.3-7: SearchSpace (Table 8.1.3.1.24.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| } |  |  |  |

Table 8.1.3.1.24.3.3-8: BWP-RedCap (Table 8.1.3.1.24.3.3-4 and Table 8.1.3.1.24.3.3-9)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP ::= SEQUENCE { |  |  |  |
| locationAndBandwidth | Set to value of locationAndBandwidth in Table 8.1.3.1.24.3.2-3 |  | FR1 |
|  | Set to value of locationAndBandwidth in Table 8.1.3.1.24.3.2-4 |  | FR2 |
| } |  |  |  |

Table 8.1.3.1.24.3.3-9: BWP-UplinkCommon-RedCap (Table 8.1.3.1.24.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| genericParameters | BWP-RedCap | Table 8.1.3.1.24.3.3-8 |  |
| pucch-ConfigCommon CHOICE { |  |  |  |
| setup | PUCCH-ConfigCommon-RedCap | Table 8.1.3.1.24.3.3-10 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.1.24.3.3-10: PUCCH-ConfigCommon-RedCap (Table 8.1.3.1.24.3.3-9)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-113 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| pucch-ResourceCommon | Not present |  |  |
| pucch-ResourceCommonRedCap-r17 | 0 |  |  |
| } |  |  |  |

Table 8.1.3.1.24.3.3-11: RRCSetup (preamble, Table 8.1.3.1.24.3.2-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSetup ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcSetup SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig-RedCap | Table 8.1.3.1.24.3.3-12 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.3.1.24.3.3-12: CellGroupConfig-RedCap (Table 8.1.3.1.24.3.3-11)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig-RedCap | Table 8.1.3.1.24.3.3-13 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.1.24.3.3-13: ServingCellConfig-RedCap (Table 8.1.3.1.24.3.3-12)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated-RedCap | Table 8.1.3.1.24.3.3-14 |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated-RedCap | Table 8.1.3.1.24.3.3-15 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.1.24.3.3-14: BWP-DownlinkDedicated-RedCap (Table 8.1.3.1.24.3.3-13)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| nonCellDefiningSSB-r17 | NonCellDefiningSSB | Table 8.1.3.1.24.3.3-16 |  |
| } |  |  |  |

Table 8.1.3.1.24.3.3-15: BWP-UplinkDedicated-RedCap (Table 8.1.3.1.24.3.3-11)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pucch-Config CHOICE { |  |  |  |
| setup | PUCCH-Config | Table 8.1.3.1.24.3.3-17 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.1.24.3.3-16: NonCellDefiningSSB (Table 8.1.3.1.24.3.3-14)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-84AAA |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| NonCellDefiningSSB-r17 ::= SEQUENCE { |  |  |  |  |
| absoluteFrequencySSB-r17 |  | Set to value of absoluteFrequencySSB in Table 8.1.3.1.24.3.2-3/4 |  |  |
| ssb-Periodicity-r17 |  | ms40 |  |  |
| ssb-TimeOffset-r17 |  | ms10 |  |  |
| } |  |  |  |  |

Table 8.1.3.1.24.3.3-17: PUCCH-Config (Table 8.1.3.1.24.3.3-15)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-112 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-Config ::= SEQUENCE { |  |  |  |
| resourceToAddModList SEQUENCE (SIZE (1..maxNrofPUCCH-Resources)) OF PUCCH-Resource { | 16 entries |  |  |
| PUCCH-Resource[n] SEQUENCE { |  | entry 1- 16 n=1,2,…16 |  |
| secondHopPRB | PRB-Id with condition secondHopPRB | Table 8.1.3.1.24.3.3-18 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.1.24.3.3-18: PRB-Id (Table 8.1.3.1.24.3.3-17)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-109 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PRB-Id | 0 |  |  |
|  | Set to value of the L_RBs - nrofPRBs where L_RBs is found in Table 8.1.3.1.24.3.2-3 or Table 8.1.3.1.24.3.2-4 for the bandwidth and SCS configured and nrofPRBs is defined for the corresponding PUCCH-Resource (1 otherwise). |  | secondHopPRB |

Table 8.1.3.1.24.3.3-19: RRCReconfiguration (step 1 Table 8.1.3.1.24.3.2-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition NR_MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| measConfig | Table 8.1.3.1.24.3.3-20 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig-RedCap | Table 8.1.3.1.24.3.3-24 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.1.24.3.3-20: MeasConfig (Table 8.1.3.1.24.3.3-19)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1.. maxNrofObjectId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR(57) | Thres=57(-100dBm ≤ SS-RSRP<-99dBm) Table 8.1.3.1.24.3.3-21 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig | ReportConfigNR-EventA1 | Table 8.1.3.1.24.3.3-22 |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1.. maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.1.24.3.3-21: MeasObjectNR (Table 8.1.3.1.24.3.3-20)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR ::= SEQUENCE { |  |  |  |
| ssbFrequency | Set to value of absoluteFrequencySSB in Table 8.1.3.1.24.3.2-3/4 | Downlink ARFCN of NR cell 1 NCD-SSB |  |
| smtc1 SEQUENCE { |  |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sf40 | 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.1.24.3.3-22: ReportConfigNR-EventA1 (Table 8.1.3.1.24.3.3-20)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-142 with condition EVENT_A1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA1 SEQUENCE { |  |  | EVENT_A1 |
| a1-Threshold CHOICE { |  |  |  |
| rsrp | 71 | -86dBm ≤ SS-RSRP<-85dBm | FR1 |
| rsrp | FFS |  | FR2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | Infinity |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.1.24.3.3-23: MeasurementReport (steps 4, 6, Table 8.1.3.1.24.3.2-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-5A |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| measurementReport SEQUENCE { |  |  |  |  |
| measResults SEQUENCE { |  |  |  |  |
| measId |  | 1 |  |  |
| measResultServingMOList SEQUENCE { |  | 1 entry |  |  |
| servCellId |  | ServCellIndex of NR Cell 1 |  |  |
| measResultServingCell SEQUENCE { |  |  |  |  |
| physCellId |  | Physical CellID of the NR Cell 1 |  |  |
| measResult SEQUENCE { |  |  |  |  |
| cellResults SEQUENCE { |  |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |  |
| rsrp |  | (0..127) |  |  |
| rsrq |  | (0..127) |  |  |
| sinr |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.3.1.24.3.3-24: CellGroupConfig-RedCap (Table 8.1.3.1.24.3.3-19)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig-RedCap | Table 8.1.3.1.24.3.3-13 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.1.24.3.3-25: ServingCellConfig-RedCap (Table 8.1.3.1.24.3.3-24)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated-RedCap | Table 8.1.3.1.24.3.3-26 |  |
| uplinkConfig | Not Present |  |  |
| } |  |  |  |

Table 8.1.3.1.24.3.3-26: BWP-DownlinkDedicated-RedCap (Table 8.1.3.1.24.3.3-25)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| nonCellDefiningSSB-r17 | NonCellDefiningSSB | Table 8.1.3.1.24.3.3-16 |  |
| servingCellMO-r17 | 1 |  |  |
| } |  |  |  |

<End of modified section 1>