3GPP TSG-RAN5 Meeting #97	R5-227185r2

# Toulouse, November 14 - 18 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2104 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of CADC enhancement test cases to Annex E and F including Test Tolerance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RRM CA/DC enhancement test cases are missing in the cell mapping tables in Annex E and MU/TT Tables in Annex F |  |  |  |  |  |  |  |  |
| Summary of change: |  | RRM CA/DC enhancement test cases has been added to Annex E and Annex F. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM CA/DC enhancement test cases will remain incomplete in TS 38.533 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 Dependant(R4-2219847) R1: RAN4 Tdoc# added. SS-RSRQ reporting values for config 3 corrected. R2: Following the revision of the corresponding TT analysis in R5-227177, the TT is applied to Thresh_X_HighP instead of the Cell 2 Es/Noc. Affected paramters has been updated. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

///Start of changes

# E.4	Cell configuration mapping for SA FR1 test cases in Chapter 6

Table E.4-1 defines the cell configuration mapping for NR/5GC FR1 test cases in chapter 6 of this test specification.

Table E.4-1: Cell configuration mapping for SA FR1 RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 6.1.1.1 | NR SA FR1 cell re-selection | NR Cell 1 | NR Cell 11 |  |  |  |
| Unchanged rows omitted |  |  |  |  |  |  |
| 6.6.8.3 | NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX | NR Cell 1 |  |  |  |  |
| 6.6.9.1 | NR SA FR1 Idle mode CA/DC measurement for FR1 | NR Cell 1 | NR Cell 6 |  |  | intra-band |
|  |  |  | NR Cell 10 |  |  | inter-band |
| 6.7.1.3.1 | NR SA FR1-FR2 SS-RSRP absolute measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |
| Unchanged rows omitted |  |  |  |  |  |  |

Table E.4-2 defines the cell configuration mapping for NR/5GC FR1 – E-UTRA Inter-RAT test cases (serving cell in NR) in chapter 6 of this test specification.

Table E.4-2: Cell configuration mapping for SA FR1 – E-UTRA Inter-RAT RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 LTE Cell2 | 38.533 LTE Cell3 | 38.533 LTE Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 6.1.2.1 | NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA | NR Cell 1 | LTE Cell 1 |  |  |  |
| Unchanged rows omitted |  |  |  |  |  |  |
| 6.6.3.3 | NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16 | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.6.15.1 | NR SA FR1 Idle Mode measurements of inter-RAT CA candidate cells for early reporting | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.7.5.1 | NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.7.6.1 | NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.7.7.1 | NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy | NR Cell 1 | LTE Cell 1 |  |  |  |

Table E.4-3: Cell configuration mapping for SA FR1 – UTRA FDD Inter-RAT RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 UTRA Cell2 | 38.533 UTRA Cell3 | 38.533 UTRA Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 6.3.1.6 | NR SA FR1 – UTRA cell re-selection to higher priority UTRA | NR Cell 1 | UTRA Cell 1 |  |  |  |
| 6.6.5.1 | NR SA FR1 – UTRAN event-triggered reporting in non-DRX | NR Cell 1 | UTRA Cell 1 |  |  |  |

///Unchanged sections omitted

# E.6	Cell configuration mapping for E-UTRAN – SA test cases in Chapter 8

Table E.6-1 defines the cell configuration mapping for E-UTRAN - NR/5GC FR1 Inter-RAT test cases (serving cell in LTE) in chapter 8 of this test specification.

Table E.6-1: Cell configuration mapping for E-UTRA – SA FR1 Inter-RAT RRM testing

| TC | Description | 38.533 LTE Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 8.2.1.1 | E-UTRA – NR FR1 cell re-selection to higher priority NR target cell | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.2.1.2 | E-UTRA – NR FR1 Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16 | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.2.2.1 | E-UTRA – NR FR1 Early Measurement Reporting | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.3.1.1 | E-UTRA – NR FR1 handover with unknown target cell | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.1.1 | E-UTRA – NR FR1 SFTD measurement delay in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.1.2 | E-UTRA – NR FR1 SFTD measurement delay in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.1 | E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.2 | E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.3 | E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.4 | E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.9 | E-UTRA – NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16 | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.5.1.1 | E-UTRA – NR FR1 SFTD measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.5.2.1.1.1 | E-UTRA SS-RSRP absolute measurement accuracy of a NR FR1 neighbour cell | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.5.2.2.1 | Void | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.5.2.3.1 | E-UTRA SS-RSRP absolute measurement accuracy of a NR FR2 neighbour cell | LTE Cell 1 | NR Cell 1 |  |  |  |

Table E.6-2 defines the cell configuration mapping for E-UTRAN - NR/5GC FR2 Inter-RAT test cases (serving cell in LTE) in chapter 8 of this test specification.

Table E.6-2: Cell configuration mapping for E-UTRA – SA FR2 Inter-RAT RRM testing

| TC | Description | 38.533 LTE Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 8.2.2.2 | E-UTRA – NR FR2 Early Measurement Reporting | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.5 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.6 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.7 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.8 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.5.2.1.2 | E-UTRA SS-RSRP absolute measurement accuracy of a NR FR2 neighbour cell | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.5.2.2.2 | SS-RSRQ | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.5.2.3.2 | E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR1 neighbour cell | LTE Cell 1 | NR Cell 1 |  |  |  |

///Unchanged sections omitted

# F.1	Measurement uncertainties and test tolerances for FR1 and FR2

## F.1.1	Acceptable uncertainty of test system (normative)

///Unchanged sections omitted

Table F.1.1.2-4 Maximum test system uncertainty for RRM requirements for SA FR1 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.1.1.1 NR SA FR1 cell re-selection | Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| Unchanged rows omitted |  |  |
| 6.6.8.3 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB | Noc is the AWGN on Cell 1 frequency Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-SINR #RB0 to RB275 |  |
| 6.6.9.1 NR SA FR1 Idle mode CA/DC measurement for FR1 | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.6.15.1 NR SA FR1 Idle Mode measurements of inter-RAT CA candidate cells for early reporting | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on LTE freq2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.7.1.1.1 NR SA FR1 SS-RSRP absolute measurement accuracy | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB meas PRB Ês1 / Noc ±0.3 dB meas PRB Ês2 / Noc ±0.3 dB | Noc is the AWGN on NR freq1 Ês1 / Noc is the cell 1 SNR Ês2 / Noc is the cell 2 SNR meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| Unchanged rows omitted |  |  |  |

///Unchanged sections omitted

Table F.1.1.2-7: Maximum test system uncertainty for RRM requirements for E-UTRA – NR inter-RAT test cases with E-UTRA serving cell

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 8.2.1.1 E-UTRA – NR FR1 cell re-selection to higher priority NR target cell | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB on cell 2 are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.2.1.2 E-UTRA – NR FR1 Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16 | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB on cell 2 are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.2.2.1 E-UTRA – NR FR1 Early Measurement Reporting | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB on cell 2 are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.2.2.2 E-UTRA – NR FR2 Early Measurement Reporting | Average Noc1 ±5.65 dB Average Noc2 ±5.65 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±5.65 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.3.1.1 E-UTRA – NR FR1 handover with known target cell | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB on cell 2 are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.4.1.1 E-UTRA – NR FR1 SFTD measurement delay in non-DRX | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB on cell 2 are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.4.1.2 E-UTRA – NR FR1 SFTD measurement delay in DRX | Same as 8.4.1.1 | Same as 8.4.1.1 |
| 8.4.2.1 E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in non-DRX | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.5 dB | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB on cell 2 are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.4.2.2 E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in DRX | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 8.4.2.3 E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in non-DRX | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 8.4.2.4 E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 8.4.2.9 E-UTRA – NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16 | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB |
| 8.5.1.1 E-UTRA – NR FR1 SFTD measurement accuracy | Average Ês1 / Noc1 ±0.8 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Ês1 / Noc1 ±0.8 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB on cell 2 are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.5.2.1.1.1 E-UTRA SS-RSRP absolute measurement accuracy of a NR FR1 neighbour cell | Average Noc ±1.5 dB Noc over meas PRBs #RBJ-J+19 ±1.5 dB Average Ês / Noc ±0.3 dB meas PRB Ês1 / Noc1 ±0.8 dB | Noc is the AWGN on the NR freq Ês / Noc is the cell SNR meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.5.2.1.2 E-UTRA SS-RSRP absolute measurement accuracy of a NR FR2 neighbour cell | Test 1 Average Noc ±5.65 dB Noc over meas PRBs ±5.65 dB Average Ês / Noc ±0.3 dB Ês / Noc over meas PRBs ±0.3 dB Test 2 Average Es ±5.65 dB Es over meas PRBs ±5.65 dB | Test 1 Noc is the AWGN on the NR freq Ês / Noc is the cell SNR meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.5.2.2.1 E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR1 neighbour cell | Same as 8.5.2.1.1.1 | Same as 8.5.2.1.1.1 |
| 8.5.2.2.2 E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR2 neighbour cellA | Same as 8.5.2.1.2 | Same as 8.5.2.1.2 |
| 8.5.2.3.1 E-UTRA SS-SINR absolute measurement accuracy of a NR FR1 neighbour cell | Same as 8.5.2.1.1.1 | Same as 8.5.2.1.1.1 |
| 8.5.2.3.2 E-UTRA SS-SINR absolute measurement accuracy of a NR FR2 neighbour cell | Same as 8.5.2.1.2 | Same as 8.5.2.1.2 |

///Unchanged sections omitted

## F.1.3	Test Tolerance and Derivation of Test Requirements (informative)

///Unchanged sections omitted

Table F.1.3.2-2: Derivation of test requirements for NR SA FR1 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 6.1.1.1 NR SA FR1 cell re-selection | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.45dB During T3: 0dB 0.45dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16.45dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16.45dB Ês2 / Noc: +13dB |
| Unchanged rows omitted |  |  |  |
| 6.6.8.3 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-SINR values ±4.5dB Reported Differential CSI-SINR values ±3.5dB | 0dB 0dB 0dB Via mapping | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB For configuration 1,2,3,4,5,6 CSI-RS#1: SINR_43 to SINR_62 CSI-RS#0: DIFFSINR_0 to DIFFSINR_6 |
| 6.6.9.1 NR SA FR1 Idle mode CA/DC measurement for FR1 | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB Reported SS-RSRP and SS-RSRQ | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB Via mapping | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB Configuration 1 and 2: Cell 1: RSRP_54 to RSRP_77 RSRQ_55 to RSRQ_72 Cell 2: RSRP_47 to RSRP_70 RSRQ_51 to RSRQ_67 Configuration 3: Cell 1: RSRP_57 to RSRP_80 RSRQ_55 to RSRQ_72 Cell 2: RSRP_50 to RSRP_73 RSRQ_51 to RSRQ_67 |
| 6.6.15.1 NR SA FR1 Idle Mode measurements of inter-RAT CA candidate cells for early reporting | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: -3dB During T4: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +8dB During T5: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB Thresh_X_HighP 48dB Reported SS-RSRP and SS-RSRQ | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB During T4: 0dB 0dB 0dB 0dB During T5: 0dB 0dB 0dB 0dB +4dB Via Mapping | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: -3dB During T4: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +8dB During T5: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5dB Ês2 / Noc2: +5dB 52dB Configuration 1 and 2: Cell 1: RSRP_52 to RSRP_75 RSRQ_54 to RSRQ_71 Cell 2: RSRP_39 to RSRP_62 RSRQ_6 to RSRQ_27 Configuration 3: Cell 1: RSRP_55 to RSRP_78 RSRQ_54 to RSRQ_71 Cell 2: RSRP_39 to RSRP_62 RSRQ_6 to RSRQ_27 |
| 6.7.1.1.1 NR SA FR1 SS-RSRP absolute measurement accuracy | TEST CONFIGURATION 1, 2, |  |  |
|  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported RSRP values: ±6dB | Test 1: -1.5dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping | Test 1: Noc: -107.5dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_44 to RSRP_56 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_60 to RSRP_79 Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -0.8dB RSRP_34 to RSRP_46 RSRP_34 to RSRP_46 RSRP_35 to RSRP_47 RSRP_35 to RSRP_47 RSRP_36 to RSRP_48 RSRP_37 to RSRP_49 RSRP_37 to RSRP_49 depending on operating band |
| Unchanged rows omitted |  |  |  |

///Unchanged sections omitted

Table F.1.3.2-5: Derivation of test requirements for E-UTRA – NR inter-RAT test cases with E-UTRA serving cell RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 8.2.1.1 E-UTRA – NR FR1 cell re-selection to higher priority NR target cell | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB | During T1: 0dB -1.90dB 0dB 0.35dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 1.55dB 1.55dB | During T1: Noc1: -98dBm/15kHz Noc2: -99.90dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -3.65dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.55dB Ês2 / Noc2: +13.55dB |
| 8.2.1.2 E-UTRA – NR FR1 Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16 | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: 14dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: +14dB | During T1: -1.90dB -1.90dB 0.35dB 3.45dB During T2: -1.90dB -1.90dB 3.45dB 0dB | During T1: Noc1: -99.9dBm/15kHz Noc2: -99.9dBm/15kHz Ês1 / Noc1: -3.65dB Ês2 / Noc2: 17.45dB During T2: Noc1: -99.9dBm/15kHz Noc2: -99.9dBm/15kHz Ês1 / Noc1: +15.45dB Ês2 / Noc2: +14dB |
| 8.2.2.1 E-UTRA – NR FR1 Early Measurement Reporting | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB Reported SS-RSRP and SS-RSRQ for NR Cell 2 | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB +0.3dB During T3: 0dB 0dB 0dB +0.3dB Via mapping | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -3.7dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -3.7dB Configuration 1 and 2: RSRP_44 to RSRP_66 RSRQ_44 to RSRQ_65 Configuration 3: RSRP_47 to RSRP_69 RSRQ_44 to RSRQ_65 |
| 8.2.2.2 E-UTRA – NR FR2 Early Measurement Reporting | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB Signalled Thresh_X_HighP: 48dB Reported SS-RSRP and SS-RSRQ for NR Cell 2 | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB +2.6dB During T3: 0dB 0dB 0dB +2.6dB +14dB Via mapping | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz 1: +14dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -1.4dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -1.4dB 62dB RSRP_27 to RSRP_87 RSRQ_46 to RSRQ_66 |
| 8.3.1.1 E-UTRA – NR FR1 handover with known target cell | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB -1.55dB +1.55dB During T3: 0dB 0dB -1.55dB +1.55dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5.45dB Ês2 / Noc2: +1.55dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5.45dB Ês2 / Noc2: +1.55dB |
| 8.4.1.1 E-UTRA – NR FR1 SFTD measurement delay in non-DRX | During T1: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +4dB | During T1: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +4dB |
| 8.4.1.2 E-UTRA – NR FR1 SFTD measurement delay in DRX | Same as 8.4.1.1 | Same as 8.4.1.1 | Same as 8.4.1.1 |
| 8.4.2.1 E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in non-DRX | During T1: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: -infinity During T2: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +7dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB -1.65dB +1.65dB | During T1: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.35dB Ês2 / Noc2: +8.65dB |
| 8.4.2.2 E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in DRX | Same as 8.4.2.1 | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 8.4.2.3 E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in non-DRX | Same as 8.4.2.1 | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 8.4.2.4 E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX | Same as 8.4.2.1 | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 8.4.2.9 E-UTRA – NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16 | During T1: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: -infinity During T2: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +7dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: -infinity During T2: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +7dB |
| 8.5.1.1 E-UTRA – NR FR1 SFTD measurement accuracy | Test 1 Ês1 / Noc1: -3dB Ês2 / Noc2: -3dB | Test 1: +0.8dB +0.3dB | Test 1: Ês1 / Noc1: -2.2dB Ês2 / Noc2: -2.7dB |
| 8.5.2.1.1.1 E-UTRA SS-RSRP absolute measurement accuracy of a NR FR1 neighbour cell |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Reported RSRP values: ±8dB (±3.0dB additionally for extreme conditions) Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: -4.0dB Reported RSRP values: ±4.5dB (±4.5dB additionally for extreme conditions) | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Ês1 / Noc1: +10.0dB RSRP_62 to RSRP_82 Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3.2dB RSRP_30 to RSRP_43 RSRP_31 to RSRP_43 RSRP_31 to RSRP_44 RSRP_32 to RSRP_44 RSRP_32 to RSRP_45 RSRP_33 to RSRP_45 RSRP_33 to RSRP_46 RSRP_34 to RSRP_46 depending on operating band |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Reported RSRP values: ±8dB (±3.0dB additionally for extreme conditions) Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: -4.0dB Reported RSRP values: ±4.5dB (±4.5dB additionally for extreme conditions) | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -96.00dBm/15kHz Ês1 / Noc1: +10.0dB RSRP_64 to RSRP_83 Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3.2dB RSRP_33 to RSRP_46 RSRP_34 to RSRP_46 RSRP_34 to RSRP_47 RSRP_35 to RSRP_47 RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 RSRP_36 to RSRP_49 RSRP_37 to RSRP_49 depending on operating band |
| 8.5.2.1.2 E-UTRA SS-RSRP absolute measurement accuracy of a NR FR2 neighbour cell | Test 1: Noc: -105dBm/15kHz Ês / Noc 11dB Reported RSRP values: ±8dB (±3dB additionally for extreme conditions) Test 2: Ês FR2a PC3: -109.1dBm/SCS Reported RSRP values: ±8dB Ês FR2b PC3: -106.5dBm/SCS Reported RSRP values: ±8dB Ês FR2c PC3: -105.5dBm/SCS Reported RSRP values: ±8dB (±3dB additionally for extreme conditions in all frequency ranges) | Test 1: -0.1dB 0dB Via mapping Test 2: +5.65dB Via mapping +5.65dB Via mapping +5.65dB Via mapping | Test 1: Noc: -105.1dBm/15kHz Ês / Noc: +11.0dB RSRP_48 to RSRP_105 Test 2: Ês: -103.45dBm/SCS RSRP_29 to RSRP_87 Ês: -100.85dBm/SCS RSRP_32 to RSRP_89 Ês: -99.85dBm/SCS RSRP_33 to RSRP_90 |
| 8.5.2.2.1 E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR1 neighbour cell |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -80.18 dBm/15kHz Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 2: Noc1: -106dBm/15kHz Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) | Test 1: -1.5dB 0dB Via mapping Test 2: 0dB 0dB Via mapping Test 3: 0dB 0dB Via mapping | Test 1: Noc1: -81.68dBm/15kHz Ês1 / Noc1: -1.75dB RSRQ_51 to RSRQ_63 Test 2: Noc1: -106dBm/15kHz Ês1 / Noc1: -1.75dB RSRQ_51 to RSRQ_63 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: -1.75dB RSRQ_51 to RSRQ_63 |
| Test Configuration 3,6 | Test 1: Noc1: -86.27 dBm/15kHz Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 2: Noc1: -113dBm/15kHz Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) | Test 1: -1.53dB 0dB Via mapping Test 2: 0dB 0dB Via mapping Test 3: 0dB 0dB Via mapping | Test 1: Noc1: -87.80dBm/15kHz Ês1 / Noc1: -1.75dB RSRQ_51 to RSRQ_63 Test 2: Noc1: -113dBm/15kHz Ês1 / Noc1: -1.75dB RSRQ_51 to RSRQ_63 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: -1.75dB RSRQ_51 to RSRQ_63 |
| 8.5.2.2.2 E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR2 neighbour cell | Test 1: Noc1: -105dBm/15kHz Ês1 / Noc1: -0.5dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 2: Noc1 FR2a PC3: -115.1dBm/15kHz Noc1 FR2b PC3: -112.5dBm/15kHz Noc1 FR2c PC3: -111.5dBm/15kHz Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions in all frequency ranges) | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 5.3dB Via mapping | Test 1: Noc1: -105dBm/15kHz Ês1 / Noc1: -0.5dB RSRQ_23 to RSRQ_95 Test 2: Noc1: -115.1dBm/15kHz Noc1: -112.5dBm/15kHz Noc1: -111.5dBm/SCS Ês1 / Noc1: 3.55dB RSRQ_26 to RSRQ_98 |
| 8.5.2.3.1 E-UTRA SS-SINR absolute measurement accuracy of a NR FR1 neighbour cell | Test 1: Noc1: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Reported SINR values: ±3dB Test 2: Noc1: -108.5dBm/15kHz Ês1 / Noc1: 20dB Reported SINR values: ±3dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -4dB Reported SINR values: ±3.5dB | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0dB Via mapping Test 3: 0dB 0.8dB Via mapping | Test 1: Noc1: -88 dBm/15kHz Ês1 / Noc1: -1.75dB SINR_35 to SINR_51 Test 2: Noc1: -108.5dBm/15kHz Ês1 / Noc1: 20dB SINR_79 to SINR_94 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3.2dB SINR_32 to SINR_49 |
| 8.5.2.3.2 E-UTRA SS-SINR absolute measurement accuracy of a NR FR2 neighbour cell | Test 1: Noc1: -105dBm/15kHz Ês1 / Noc1: -0.5dB Reported SINR values: ±3dB (±1dB additionally for extreme conditions) Test 2: Noc1: -105dBm/15kHz Ês1 / Noc1: 11dB Reported SINR values: ±3dB (±1dB additionally for extreme conditions) Test 3: Noc1 FR2a PC3: -115.1dBm/15kHz Noc1 FR2b PC3: -112.5dBm/15kHz Noc1 FR2c PC3: -111.5dBm/15kHz Ês1 / Noc1: -1dB Reported SINR values: ±3dB (±1dB additionally for extreme conditions in all frequency ranges) | Test 1: 0dB 0dB Via mapping Test 2: -0.1dB 0dB Via mapping Test 3: 0dB 0dB 0dB 4.6dB Via mapping | Test 1: Noc1: -105dBm/15kHz Ês1 / Noc1: -0.5dB SINR_9 to SINR_82 Test 2: Noc1: -105.1dBm/15kHz Ês1 / Noc1: 11dB SINR_32 to SINR_105 Test 3: Noc1: -115.1dBm/15kHz Noc1: -112.5dBm/15kHz Noc1: -111.5dBm/SCS Ês1 / Noc1: 3.6dB SINR_17 to SINR_90 |

///End of changes