3GPP TSG-RAN5 Meeting #97	R5-227232r1

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2043 | rev | - | Current version: | 17.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of 7.3.2 Reference sensitivity power level for redcap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The note1 in Table 7.3.2.3-1a and 7.3.2.3-1b for redcap UE is not aligned with note1 in TS38.101-1 minimum conformance requirement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated Note1 in table 7.3.2.3-1a, 7.3.2.3-1b Updated Note1 in table 7.3.2.5-1a and 7.3.2.5-1b |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The minimum conformance requirement is not aligned with core spec |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Updated 7.2 to align with core spec TS 38.101-1 Updated Table 7.3.2.5-2a, Table 7.3.2.5-2b to align with core spec TS 38.101-1 Removed n85 from Tables 7.3I.2.3-2, 7.3I.2.3-3 and 7.3I.2.3-4 since it hasn’t been introduced into RAN5 yet. Updated 7.3I.2 minimum conformance requirement to align with core spec Updated Table 7.3I.2.4.1-1 Updated table format of Table 7.3I.2.5-2 Removed n85 from Tables 7.3I.2.5-5 and Table 7.3I.2.5-6 since it hasn’t been introduced into RAN5 yet. |  |  |  |  |  |  |  |  |

<<< Start OF CHANGES >>>

7.2	Diversity characteristics

The UE is required to be equipped with a minimum of two Rx antenna ports in all operating bands except for the bands n7, n38, n41, n77, n78, n79 where the UE is required to be equipped with a minimum of four Rx antenna ports. An exception is allowed for two Rx vehicular UE to be equipped with a minimum of two Rx antenna ports in bands n7, n38, n41, n77, n78, n79. This requirement applies when the band is used as a standalone band or as part of a band combination.

For the single carrier REFSENS requirements in clause 7, the UE shall be verified with two Rx antenna ports in all supported frequency bands, additional requirements for four Rx ports shall be verified in operating bands where the UE is equipped with four Rx antenna ports.

For Rx requirements other than single carrier REFSENS in Clause 7, the UE shall be verified with four Rx antenna ports and skip two Rx antenna ports requirements in operating bands where the UE is equipped with four Rx antenna ports, otherwise, the UE shall be verified with two Rx antenna ports.

The above rules apply for all subclasses with the exception of clause 7.9.

A Redcap UE is required to be equipped with a minimum of single Rx antenna port and maximum of two Rx antenna ports. Clause 7 requirements for four Rx antenna ports do not apply to a RedCap UE

<<< Unchanged Part Skipped >>>

7.3.2	Reference sensitivity power level

7.3.2.1	Test purpose

The test purpose is to verify the ability of the UE to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise.

7.3.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

7.3.2.3	Minimum conformance requirements

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.2.3.2, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2.3-1a, 7.3.2.3-1b, Table 7.3.2-1c, Table 7.3.2-1d and Table 7.3.2.3-2.

Table 7.3.2.3-1a: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n1 | 15 | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 | -91.9 |  | -90.6 | -90.1 | -89.6 |
|  | 30 |  | -97.1 | -95.1 | -94.0 | -92.8 | -92.0 |  | -90.7 | -90.2 | -89.7 |
|  | 60 |  | -97.5 | -95.4 | -94.2 | -93.0 | -92.1 |  | -90.9 | -90.3 | -89.7 |
| n2 | 15 | -98 | -94.8 | -93 | -91.8 | -90.7 | -84.1 |  | -81.5 |  |  |
|  | 30 |  | -95.1 | -93.1 | -92 | -90.8 | -84.2 |  | -81.6 |  |  |
|  | 60 |  | -95.5 | -93.4 | -92.2 | -90.9 | -84.3 |  | -81.7 |  |  |
| n3 | 15 | -97.0 | -93.8 | -92.0 | -90.8 | -89.7 | -88.9 | -86.2 | -82.3 | -81.3 | -79.7 |
|  | 30 |  | -94.1 | -92.1 | -91.0 | -89.8 | -89.0 | -86.3 | -82.4 | -81.4 | -79.8 |
|  | 60 |  | -94.5 | -92.4 | -91.2 | -90.0 | -89.1 | -86.4 | -82.6 | -81.5 | -79.9 |
| n5 | 15 | -98.0 | -94.8 | -93.0 | -86.8 | -84.8 |  |  |  |  |  |
|  | 30 |  | -95.1 | -93.1 | -88.6 | -84.9 |  |  |  |  |  |
| n71 | 15 | -98.0 | -94.8 | -93.0 | -91.8 |  |  |  |  |  |  |
|  | 30 |  | -95.1 | -93.1 | -92.0 |  |  |  |  |  |  |
|  | 60 |  | -95.5 | -93.4 | -92.2 |  |  |  |  |  |  |
| n8 | 15 | -97.0 | -93.8 | -91.4 | -85.8 |  |  | -78.4 |  |  |  |
|  | 30 |  | -94.1 | -91.7 | -87.2 |  |  | -78.5 |  |  |  |
| n12 | 15 | -97.0 | -93.8 | -84.0 |  |  |  |  |  |  |  |
|  | 30 |  | -94.1 | -84.1 |  |  |  |  |  |  |  |
| n14 | 15 | -97.0 | -93.8 |  |  |  |  |  |  |  |  |
|  | 30 |  | -94.1 |  |  |  |  |  |  |  |  |
| n20 | 15 | -97.0 | -93.8 | -91.0 | -89.8 |  |  |  |  |  |  |
|  | 30 |  | -94.1 | -91.1 | -90.0 |  |  |  |  |  |  |
| n24 | 15 | -100.0 | -96.8 |  |  |  |  |  |  |  |  |
|  | 30 |  | -97.1 |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 |  |  |  |  |  |  |  |  |
| n25 | 15 | -96.5 | -93.3 | -91.5 | -90.3 | -89.3 | -82.2 |  | -79.5 |  |  |
|  | 30 |  | -93.6 | -91.6 | -90.5 | -89.4 | -82.3 |  | -79.6 |  |  |
|  | 60 |  | -94.0 | -91.9 | -90.7 | -89.6 | -82.4 |  | -79.7 |  |  |
| n26 | 15 | -97.56 | -94.56 | -92.76 | -87.6 |  |  |  |  |  |  |
|  | 30 |  | -94.86 | -92.76 | -87.7 |  |  |  |  |  |  |
| n28 | 15 | -98.5 | -95.5 | -93.5 | -90.8 |  | -78.5 |  |  |  |  |
|  | 30 |  | -95.6 | -93.6 | -91.0 |  | -78.6 |  |  |  |  |
| n30 | 15 | -99.0 | -95.8 |  |  |  |  |  |  |  |  |
|  | 30 |  | -96.1 |  |  |  |  |  |  |  |  |
| n65 | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  | -89.2 |
|  | 30 |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  | -89.3 |
|  | 60 |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  | -89.4 |
| n66 | 15 | -99.5 | -96.3 | -94.5 | -93.3 | -92.2 | -91.4 |  | -90.1 | -89.6 |  |
|  | 30 |  | -96.6 | -94.6 | -93.5 | -92.3 | -91.5 |  | -90.2 | -89.7 |  |
|  | 60 |  | -97.0 | -94.9 | -93.7 | -92.5 | -91.6 |  | -90.4 | -89.8 |  |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 |  |  |  |  |  |
| n70 | 30 |  | -97.1 | -95.1 | -94.0 | -92.8 |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 | -93.0 |  |  |  |  |  |
| n71 | 15 | -97.2 | -94.0 | -91.6 | -86.0 |  |  |  |  |  |  |
|  | 30 |  | -94.3 | -91.9 | -87.4 |  |  |  |  |  |  |
| n74 | 15 | -99.53 | -96.33 | -94.53 | -89.33 |  |  |  |  |  |  |
|  | 30 |  | -96.63 | -94.63 | -89.53 |  |  |  |  |  |  |
|  | 60 |  | -97.03 | -94.93 | -89.63 |  |  |  |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 3: The requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9 - 1510.9 MHz. NOTE 4: Void NOTE 5: Void NOTE 6: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. NOTE 7: Void. |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2.3-1b: Two antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n34 | 15 | 5, 10, 15 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15 | -97.5 + 10log10(NRB/11) |  |
| n381 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11) |  |
| n39 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11) |  |
| n40 | 15 | 5, 10, 15, 20, 25, 30, 40, 50 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | -97.5 + 10log10(NRB/11) |  |
| n411 | 15 | 10, 15, 20, 30, 40, 50 | -94.8 + 10log10(NRB/50) | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -95.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -95.5 + 10log10(NRB/11) |  |
| n481 | 15 | 5, 10, 15, 20, 30, 40, 505 | -99 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 30, 40, 505, 605, 705, 805, 905, 1005 | -96.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 30, 40, 505, 605, 705, 805, 905, 1005 | -96.5 + 10log10(NRB/11) |  |
| n50 | 15 | 5, 10, 15, 20, 30, 40, 50 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 80 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 80 | -97.5 + 10log10(NRB/11) |  |
| n51 | 15 | 5 | -100 | TDD |
| n53 | 15 | 5, 10 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10 | -97.1 |  |
|  | 60 | 10 | -97.5 |  |
| n757 | 15 | 5,10,15,20 | -100 + 10log10(NRB/25) | SDL |
|  | 30 | 10,15,20 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10,15,20 | -97.5 + 10log10(NRB/11) |  |
| n767 | 15 | 5 | -100 | SDL |
| n771,4 | 15 | 10, 15, 20, 40, 50 | -95.3 + 10log10(NRB/50) | TDD |
|  | 30 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | -95.6 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | -96.0 + 10log10(NRB/11) |  |
| en781 | 15 | 10, 15, 20, 40, 50 | -95.8 + 10log10(NRB/50) | TDD |
|  | 30 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11) |  |
| n791 | 15 | 40, 50 | -89.6 + 10log10(NRB/216) | TDD |
|  | 30 | 40, 50, 60, 80, 100 | -89.7 + 10log10(NRB/106) |  |
|  | 60 | 40, 50, 60, 80, 100 | -89.9 + 10log10(NRB/51) |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 6: Void NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. |  |  |  |  |

<<< Unchanged Part Skipped >>>

7.3.2.5	Test requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A3.2 with reference receive power level specified in Tables 7.3.2.5-1a and Tables 7.3.2.5-1b for 2 Rx antenna port, Tables 7.3.2.5-2a and Tables 7.3.2.5-2b for 4 Rx antenna port, Table 7.3.2.5-2c and Table 7.3.2.5-2d for PC2 UE on FDD bands, and parameters specified Tables 7.3.2.4.1-1, Tables 7.3.2.4.1-2 and Tables 7.3.2.4.1-3.

Table 7.3.2.5-1a: Two antenna port Reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| n1 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.8 +TT | -92.7 +TT | -91.9 +TT |  | -90.6 +TT | -90.1 +TT | -89.6 +TT | FDD |
|  | 30 |  | -97.1 +TT | -95.1 +TT | -94.0 +TT | -92.8 +TT | -92.0 +TT |  | -90.7 +TT | -90.2 +TT | -89.7 +TT |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT | -93.0 +TT | -92.1 +TT |  | -90.9 +TT | -90.3 +TT | -89.7 +TT |  |
| n2 | 15 | -98.0 +TT | -94.8 +TT | -93.0 +TT | -91.8 +TT | -90.7 +TT | -84.1 +TT |  | -81.5 +TT |  |  | FDD |
|  | 30 |  | -95.1 +TT | -93.1 +TT | -92.0 +TT | -90.8 +TT | -84.2 +TT |  | -81.6 +TT |  |  |  |
|  | 60 |  | -95.5 +TT | -93.4 +TT | -92.2 +TT | -90.9 +TT | -84.3 +TT |  | -81.7 +TT |  |  |  |
| n3 | 15 | -97.0 +TT | -93.8 +TT | -92.0 +TT | -90.8 +TT | -89.7 +TT | -88.9 +TT | -86.2+TT | -87.6 +TT | -81.3+TT | -79.7 +TT | FDD |
|  | 30 |  | -94.1 +TT | -92.1 +TT | -91.0 +TT | -89.8 +TT | -89.0 +TT | -86.3+TT | -87.7 +TT | -81.4+TT | -79.8 +TT |  |
|  | 60 |  | -94.5 +TT | -92.4 +TT | -91.2 +TT | -90.0 +TT | -89.1 +TT | -86.4+TT | -87.9 +TT | -81.5+TT | -79.9 +TT |  |
| n5 | 15 | -98.0 +TT | -94.8 +TT | -93.0 +TT | -86.8 +TT | -84.8 +TT |  |  |  |  |  | FDD |
|  | 30 |  | -95.1 +TT | -93.1 +TT | -88.6 +TT | -84.9 +TT |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 15 | -98.0 +TT | -94.8 +TT | -93.0 +TT | -91.8 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  | -95.1 +TT | -93.1 +TT | -92.0 +TT |  |  |  |  |  |  |  |
|  | 60 |  | -95.5 +TT | -93.4 +TT | -92.2 +TT |  |  |  |  |  |  |  |
| n8 | 15 | -97.0 +TT | -93.8 +TT | -91.4 +TT | -85.8 +TT |  |  | -78.4+TT |  |  |  | FDD |
|  | 30 |  | -94.1 +TT | -91.7 +TT | -87.2 +TT |  |  | -78.5+TT |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n12 | 15 | -97.0 +TT | -93.8 +TT | -84.0 +TT |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -94.1 +TT | -84.1 +TT |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n14 | 15 | -97.0 +TT | -93.8 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -94.1 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n20 | 15 | -97.0 +TT | -93.8 +TT | -91.0 +TT | -89.8 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  | -94.1 +TT | -91.1 +TT | -90.0 +TT |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n24 | 15 | -100.0 +TT | -96.8 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -97.1 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 +TT |  |  |  |  |  |  |  |  |  |
| n25 | 15 | -96.5 +TT | -93.3 +TT | -91.5 +TT | -90.3 +TT | -89.3 +TT | -82.2 +TT |  | -79.5 +TT |  |  | FDD |
|  | 30 |  | -93.6 +TT | -91.6 +TT | -90.5 +TT | -89.4 +TT | -82.3 +TT |  | -79.6 +TT |  |  |  |
|  | 60 |  | -94.0 +TT | -91.9 +TT | -90.7 +TT | -89.6 +TT | -82.4 +TT |  | -79.7 +TT |  |  |  |
| n26 | 15 | -97.5 +TT | -94.5 +TT | -92.7 +TT | -87.6 +TT |  |  |  |  |  |  |  |
|  | 30 |  | -94.8 +TT | -92.7 +TT | -87.7 +TT |  |  |  |  |  |  |  |
| n28 | 15 | -98.5 +TT | -95.5 +TT | -93.5 +TT | -90.8 +TT |  | -78.5 +TT |  |  |  |  | FDD |
|  | 30 |  | -95.6 +TT | -93.6 +TT | -91.0 +TT |  | -78.6 +TT |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n30 | 15 | -99.0 +TT | -95.8 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -96.1 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n65 | 15 | -99.5+TT | -96.3+TT | -94.5+TT | -93.3+TT |  |  |  |  |  |  | FDD |
|  | 30 |  | -96.6+TT | -94.6+TT | -93.5+TT |  |  |  |  |  |  |  |
|  | 60 |  | -97.0+TT | -94.9+TT | -93.7+TT |  |  |  |  |  |  |  |
| n66 | 15 | -99.5 +TT | -96.3 +TT | -94.5 +TT | -93.3 +TT | -92.2 +TT | -91.4 +TT |  | -90.1 +TT |  |  | FDD |
|  | 30 |  | -96.6 +TT | -94.6 +TT | -93.5 +TT | -92.3 +TT | -91.5 +TT |  | -90.2 +TT |  |  |  |
|  | 60 |  | -97.0 +TT | -94.9 +TT | -93.7 +TT | -92.5 +TT | -91.6 +TT |  | -90.4 +TT |  |  |  |
| n70 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.8 +TT | -92.7 +TT |  |  |  |  |  | FDD |
|  | 30 |  | -97.1 +TT | -95.1 +TT | -94.0 +TT | -92.8 +TT |  |  |  |  |  |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT | -93.0 +TT |  |  |  |  |  |  |
| n71 | 15 | -97.2 +TT | -94.0 +TT | -91.6 +TT | -86.0 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  | -94.3 +TT | -91.9 +TT | -87.4 +TT |  |  |  |  |  |  |  |
|  | 60 | - |  |  |  |  |  |  |  |  |  |  |
| n74 | 15 | -99.53 +TT | -96.33 +TT | -94.53 +TT | -93.33 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  | -96.63 +TT | -94.63 +TT | -93.53 +TT |  |  |  |  |  |  |  |
|  | 60 |  | -97.03 +TT | -94.93 +TT | -93.73 +TT |  |  |  |  |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 3: 3 indicates that the requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9-1510.9 MHz. NOTE 4: Void NOTE 5: Void NOTE 6: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2.5-1b: Two antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n34 | 15 | 5, 10, 15 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15 | -97.5 + 10log10(NRB/11)+TT |  |
| n381 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11)+TT |  |
| n39 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11)+TT |  |
| n40 | 15 | 5, 10, 15, 20, 25, 30, 40, 50 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | -97.5 + 10log10(NRB/11)+TT |  |
| n411 | 15 | 10, 15, 20, 30, 40, 50 | -94.8 + 10log10(NRB/50)+TT | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -95.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -95.5 + 10log10(NRB/11)+TT |  |
| n481 | 15 | 5, 10, 15, 20, 30, 40, 505 | -99 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, 30, 40, 505, 605, 705, 805, 905, 1005 | -96.1 + 10log10(NRB/24) +TT |  |
|  | 60 | 10, 15, 20, 30, 40, 505, 605, 705, 805, 905, 1005 | -96.5 + 10log10(NRB/11)+TT |  |
| n50 | 15 | 5, 10, 15, 20, 30, 40, 50 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 80 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 80 | -97.5 + 10log10(NRB/11)+TT |  |
| n51 | 15 | 5 | -100+TT | TDD |
| n53 | 15 | 5, 10 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10 | -97.1+TT |  |
|  | 60 | 10 | -97.5+TT |  |
| n771,4 | 15 | 10, 15, 20, 40, 50 | -95.3 + 10log10(NRB/50)+TT | TDD |
|  | 30 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | -95.6 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | -96.0 + 10log10(NRB/11)+TT |  |
| n781 | 15 | 10, 15, 20, 40, 50 | -95.8 + 10log10(NRB/50) +TT | TDD |
|  | 30 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24) +TT |  |
|  | 60 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11)+TT |  |
| n791 | 15 | 40, 50 | -89.6 + 10log10(NRB/216)+TT | TDD |
|  | 30 | 40, 50, 60, 80, 100 | -89.7 + 10log10(NRB/106) +TT |  |
|  | 60 | 40, 50, 60, 80, 100 | -89.9 + 10log10(NRB/51) +TT |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 6: Void NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. |  |  |  |  |

Table 7.3.2.5-2a: Four antenna port Reference sensitivity QPSK PREFSENS FDD bands

| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | 15 | -102.7 +TT | -99.5 +TT | -97.7 +TT | -96.5 +TT | -95.4 +TT | -94.6 +TT |  | -93.3 +TT | -92.8 +TT | -92.3 +TT | FDD |
|  | 30 |  | -99.8 +TT | -97.8 +TT | -96.7 +TT | -95.5 +TT | -94.7 +TT |  | -93.4 +TT | -92.9 +TT | -92.4 +TT |  |
|  | 60 |  | -100.2 +TT | -98.1 +TT | -96.9 +TT | -95.7 +TT | -94.8 +TT |  | -93.6 +TT | -93 +TT | -92.4 +TT |  |
| n2 | 15 | -100.7 +TT | -97.5 +TT | -95.7 +TT | -94.5 +TT | -93.4 +TT | -86.8 +TT |  | -84.2 +TT |  |  | FDD |
|  | 30 |  | -97.8 +TT | -95.8 +TT | -94.7 +TT | -93.5 +TT | -86.9 +TT |  | -83.3 +TT |  |  |  |
|  | 60 |  | -98.2 +TT | -96.1 +TT | -94.9 +TT | -93.6 +TT | -87.0 +TT |  | -84.4 +TT |  |  |  |
| n3 | 15 | -99.7 +TT | -96.5 +TT | -94.7 +TT | -93.5 +TT | -92.4 +TT | -91.6 +TT | -88.9+TT | -90.3 +TT | -84.0+TT | -82.4 +TT | FDD |
|  | 30 |  | -96.8 +TT | -94.8 +TT | -93.7 +TT | -92.5 +TT | -91.7 +TT | -90.0+TT | -90.4 +TT | -84.1+TT | -82.5 +TT |  |
|  | 60 |  | -97.2 +TT | -95.1 +TT | -93.9 +TT | -92.7 +TT | -91.8 +TT | -90.1+TT | -90.6 +TT | -84.2+TT | -82.6 +TT |  |
| n71 | 15 | -100.7 +TT | -97.5 +TT | -95.7 +TT | -94.5 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  | -97.8 +TT | -95.8 +TT | -94.7 +TT |  |  |  |  |  |  |  |
|  | 60 |  | -98.2 +TT | -97.1 +TT | -94.9 +TT |  |  |  |  |  |  |  |
| n8 | 15 | -99.7 +TT | -96.5 +TT | -94.1 +TT | -88.5 +TT |  |  |  |  |  |  | FDD |
|  | 30 |  | -96.8 +TT | -94.4 +TT | -89.9 +TT |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n30 | 15 | -101.7 +TT | -98.5 +TT |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -98.8 +TT |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |
| n66 | 15 | -102.2 +TT | -99.0 +TT | -97.2 +TT | -96.0 +TT | -94.9 +TT | -94.1 +TT |  | -92.8 +TT |  |  | FDD |
|  | 30 |  | -99.3 +TT | -97.3 +TT | -96.2 +TT | -95.0 +TT | -94.2 +TT |  | -92.9 +TT |  |  |  |
|  | 60 |  | -99.7 +TT | -97.6 +TT | -96.4 +TT | -95.2 +TT | -94.3 +TT |  | -93.1 +TT |  |  |  |
| n70 | 15 | -102.7 +TT | -99.5 +TT | -97.7 +TT | -96.5 +TT | -95.4 +TT |  |  |  |  |  | FDD |
|  | 30 |  | -99.8 +TT | -97.8 +TT | -96.7 +TT | -95.5 +TT |  |  |  |  |  |  |
|  | 60 |  | -100.2 +TT | -98.1 +TT | -96.9 +TT | -95.7 +TT |  |  |  |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for above listed operating band except for two Rx vehicular UE. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2 The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 3: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2.5-2b: Four antenna port Reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n34 | 15 | 5, 10, 15 | -100 + 10log10(NRB/25)-2.7 +TT | TDD |
|  | 30 | 10, 15 | -97.1 + 10log10(NRB/24)+-2.7 TT |  |
|  | 60 | 10, 15 | -97.5 + 10log10(NRB/11)-2.7 +TT |  |
| n381 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25)-2.7 +TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24)-2.7 +TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11)-2.7 +TT |  |
| n39 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25)-2.7 +TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24)-2.7 +TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11)-2.7 +TT |  |
| n40 | 15 | 5, 10, 15, 20, 25, 30, 40, 50 | -100 + 10log10(NRB/25) -2.7 +TT | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | -97.1 + 10log10(NRB/24)-2.7 +TT |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | -97.5 + 10log10(NRB/11)-2.7 +TT |  |
| n411 | 15 | 10, 15, 20, 30, 40, 50 | -94.8 + 10log10(NRB/50)-2.7 +TT | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -95.1 + 10log10(NRB/24)-2.7 +TT |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -95.5 + 10log10(NRB/11)-2.7 +TT |  |
| n481 | 15 | 5, 10, 15, 20, 30, 40, 505 | -99 + 10log10(NRB/25)-2.2 +TT | TDD |
|  | 30 | 10, 15, 20, 40, 505, 605, 705, 805, 905, 1005 | -96.1 + 10log10(NRB/24)-2.2 +TT |  |
|  | 60 | 10, 15, 20, 40, 505, 605, 705, 805, 905, 1005 | -96.5 + 10log10(NRB/11)-2.2+TT |  |
| n771,4 | 15 | 10, 15, 20, 40, 50 | -95.3 + 10log10(NRB/50)-2.2 +TT | TDD |
|  | 30 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | -95.6 + 10log10(NRB/24)-2.2 +TT |  |
|  | 60 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | -96.0 + 10log10(NRB/11)-2.2 +TT |  |
| n781 | 15 | 10, 15, 20, 40, 50 | -95.8 + 10log10(NRB/50)-2.2 +TT | TDD |
|  | 30 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24)-2.2 +TT |  |
|  | 60 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11)-2.2 +TT |  |
| n791 | 15 | 40, 50 | -89.6 + 10log10(NRB/216)-2.2 +TT | TDD |
|  | 30 | 40, 50, 60, 80, 100 | -89.7 + 10log10(NRB/106)-2.2 +TT |  |
|  | 60 | 40, 50, 60, 80, 100 | -89.9 + 10log10(NRB/51)-2.2 +TT |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 6: Void NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. |  |  |  |  |

<<< Unchanged Part Skipped >>>

7.3I	Reference sensitivity for RedCap

7.3I.1	General

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

7.3I.2	Reference sensitivity power level

7.3I.2.1	Test purpose

The test purpose is to verify the ability of the UE to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise.

7.3I.2.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support NR RedCap.

7.3I.2.3	Minimum conformance requirements

For a RedCap UE equipped with 2 Rx antenna ports, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2.3-1a and Table 7.3.2.3-1b for the applicable operating bands. The reference sensitivity (REFSENS) requirement specified for a RedCap UE equipped with 2 Rx antenna ports shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.3-3 and, for FDD bands, with the Tx-Rx separation as defined in clause 5.4.4 for the applicable band and UE channel bandwidth.

For a RedCap UE equipped with 1 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2.3-1a and in Table 7.3.2.3-1b shall be modified by the amount given in ΔR1R in Table 7.3I.2.3-1 for the applicable operating bands. The reference sensitivity (REFSENS) requirement specified for a RedCap UE equipped with 1 Rx antenna ports shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.3-3 and, for FDD bands, with the Tx-Rx separation as defined in clause 5.4.4 for the applicable band and UE channel bandwidth.

Table 7.3I.2.3-1: Single antenna port reference sensitivity allowance ΔR1R

| Operating band | Channel bandwidth (MHz) | ΔR1R (dB) |
| --- | --- | --- |
| TDD band | 5, 10, 15, 20 | 2.5 |
| FDD band | 5 | 2.5 |
| FDD band | 10, 15, 20 | 3 |

For a RedCap UE equipped with 2 Rx antenna ports operating in HD-FDD mode, reference sensitivity for 2Rx antenna ports in Table 7.3I.2.3-2 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3I.2.3-4.

Table 7.3I.2.3-2: HD-FDD RedCap UE with 2 Rx antenna port reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n1 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n2 | 15 | -98.8 | -95.6 | -93.8 | -92.5 |
|  | 30 |  | -96.0 | -94.0 | -92.7 |
|  | 60 |  | -96.3 | -94.2 | -93.0 |
| n3 | 15 | -97.8 | -94.6 | -92.8 | -91.5 |
|  | 30 |  | -95.0 | -93.0 | -91.7 |
|  | 60 |  | -95.3 | -93.2 | -92.0 |
| n5 | 15 | -98.8 | -95.6 | -93.8 | -92.5 |
|  | 30 |  | -96.0 | -94.0 | -92.7 |
| n7 | 15 | -98.8 | -95.6 | -93.8 | -92.5 |
|  | 30 |  | -96.0 | -94.0 | -92.7 |
|  | 60 |  | -96.3 | -94.2 | -93.0 |
| n8 | 15 | -97.8 | -94.6 | -92.8 | -91.5 |
|  | 30 |  | -95.0 | -93.0 | -91.7 |
| n12 | 15 | -97.8 | -94.6 | -92.8 |  |
|  | 30 |  | -95.0 | -93.0 |  |
| n13 | 15 | -97.8 | -94.6 |  |  |
|  | 30 |  | -95.0 |  |  |
| n14 | 15 | -97.8 | -94.6 |  |  |
|  | 30 |  | -95.0 |  |  |
| n18 | 15 | -100.0 | -96.8 | -95.0 |  |
|  | 30 |  | -97.2 | -95.2 |  |
| n20 | 15 | -97.8 | -94.6 | -92.8 | -91.5 |
|  | 30 |  | -95.0 | -93.0 | -91.7 |
| n24 | 15 | -100.0 | -96.8 |  |  |
|  | 30 |  | -97.2 |  |  |
|  | 60 |  | -97.5 |  |  |
| n25 | 15 | -97.3 | -94.1 | -92.3 | -91.0 |
|  | 30 |  | -94.5 | -92.5 | -91.2 |
|  | 60 |  | -94.8 | -92.7 | -91.5 |
| n26 | 15 | -98.3 | -95.1 | -93.3 | -92.0 |
|  | 30 |  | -95.5 | -93.5 | -92.2 |
| n28 | 15 | -99.3 | -96.1 | -94.3 | -93.0 |
|  | 30 |  | -96.5 | -94.5 | -93.2 |
| n30 | 15 | -99.5 | -96.3 |  |  |
|  | 30 |  | -96.7 |  |  |
| n65 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n66 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n70 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n71 | 15 | -98.0 | -94.8 | -93.0 | -91.7 |
|  | 30 |  | -95.2 | -93.2 | -91.9 |
| n74 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |

For a RedCap UE equipped with 1 Rx antenna ports and operating in HD-FDD mode, reference sensitivity for 1Rx antenna ports in Table 7.3I.2.3-3 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3I.2.3-4.

Table 7.3I.2.3-3: HD-FDD RedCap UE with 1 Rx antenna port reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n1 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n2 | 15 | -96.3 | -93.1 | -91.3 | -90.0 |
|  | 30 |  | -93.5 | -91.5 | -90.2 |
|  | 60 |  | -93.8 | -91.7 | -90.5 |
| n3 | 15 | -95.3 | -92.1 | -90.3 | -89.0 |
|  | 30 |  | -92.5 | -90.5 | -89.2 |
|  | 60 |  | -92.8 | -90.7 | -89.5 |
| n5 | 15 | -96.3 | -93.1 | -91.3 | -90.0 |
|  | 30 |  | -93.5 | -91.5 | -90.2 |
| n7 | 15 | -96.3 | -93.1 | -91.3 | -90.0 |
|  | 30 |  | -93.5 | -91.5 | -90.2 |
|  | 60 |  | -93.8 | -91.7 | -90.5 |
| n8 | 15 | -95.3 | -92.1 | -90.3 | -89.0 |
|  | 30 |  | -92.5 | -90.5 | -89.2 |
| n12 | 15 | -95.3 | -92.1 | -90.3 |  |
|  | 30 |  | -92.5 | -90.5 |  |
| n13 | 15 | -95.3 | -92.1 |  |  |
|  | 30 |  | -92.5 |  |  |
| n14 | 15 | -95.3 | -92.1 |  |  |
|  | 30 |  | -92.5 |  |  |
| n18 | 15 | -97.5 | -94.3 | -92.5 |  |
|  | 30 |  | -94.7 | -92.7 |  |
| n20 | 15 | -95.3 | -92.1 | -90.3 | -89.0 |
|  | 30 |  | -92.5 | -90.5 | -89.2 |
| n24 | 15 | -97.5 | -94.3 |  |  |
|  | 30 |  | -94.7 |  |  |
|  | 60 |  | -95.0 |  |  |
| n25 | 15 | -94.8 | -91.6 | -89.8 | -88.5 |
|  | 30 |  | -92.0 | -90.0 | -88.7 |
|  | 60 |  | -92.3 | -90.2 | -89.0 |
| n26 | 15 | -95.8 | -92.6 | -90.8 | -89.5 |
|  | 30 |  | -93.0 | -91.0 | -89.7 |
| n28 | 15 | -96.8 | -93.6 | -91.8 | -90.5 |
|  | 30 |  | -94.0 | -92.0 | -90.7 |
| n30 | 15 | -97.0 | -93.8 |  |  |
|  | 30 |  | -94.2 |  |  |
| n65 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n66 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n70 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n71 | 15 | -95.5 | -92.3 | -90.5 | -89.2 |
|  | 30 |  | -92.7 | -90.7 | -89.4 |
| n74 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |

Table 7.3I.2.3-4: Uplink configuration for HD-FDD reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| n1 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n2 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n3 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n5 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n7 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n8 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n12 | 15 | 25 | 50 | 75 |  |
|  | 30 |  | 24 | 36 |  |
| n13 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
| n14 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
| n18 | 15 | 25 | 50 | 75 |  |
|  | 30 |  | 24 | 36 |  |
| n20 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n24 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
|  | 60 |  | 10 |  |  |
| n25 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n26 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n28 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n30 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
| n65 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n66 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n70 | 15 | 25 | 50 | 75 | NOTE1 |
|  | 30 |  | 24 | 36 | NOTE1 |
|  | 60 |  | 10 | 18 | NOTE1 |
| n71 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n74 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| NOTE 1: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest TX-RX separation (Table 5.4.4-1) shall be used unless otherwise specified. |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 7.3I.2.

7.3I.2.4	Test description

7.3I.2.4.1	Initial conditions

For RedCap UE with 1Rx or 2 Rx antenna ports, same initial conditions as in 7.3.2.4.1 with following exception:

-	The test channel bandwidth are specified in TS 38.508-1 [5] subclause 4.3.1 for RedCap.

For HD-FDD RedCap UE with 1 Rx or 2 Rx antenna ports, same initial conditions as in 7.3.2.4.1 with following exception:

-	The test channel bandwidth are specified in TS 38.508-1 [5] subclause 4.3.1 for RedCap.

-	The RB allocation for uplink configuration in Table 7.3.2.4.1-1 refers to Table 7.3I.2.4.1-1 for each SCS, channel BW and NR band.

Table 7.3I.2.4.1-1: Uplink configuration for reference sensitivity of HD-FDD RedCap UE, LCRB @ RBstart format

| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- |
| n1 | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  | 30 |  | 24@0 | 36@2 | 50@1 |  |
|  | 60 |  | 10@1 | 18@0 | 24@0 |  |
| n2 | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  | 30 |  | 24@0 | 36@2 | 50@1 |  |
|  | 60 |  | 10@1 | 18@0 | 24@0 |  |
| n3 | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  | 30 |  | 24@0 | 36@2 | 50@1 |  |
|  | 60 |  | 10@1 | 18@0 | 24@0 |  |
| n5 | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  | 30 |  | 24@0 | 36@2 | 50@1 |  |
| n7 | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  | 30 |  | 24@0 | 36@2 | 50@1 |  |
|  | 60 |  | 10@1 | 18@0 | 24@0 |  |
| n8 | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  | 30 |  | 24@0 | 36@2 | 50@1 |  |
| n12 | 15 | 25@0 | 50@2 | 75@4 |  | HD-FDD |
|  | 30 |  | 24@0 | 36@2 |  |  |
| n14 | 15 | 25@0 | 50@0 |  |  | HD-FDD |
|  | 30 |  | 24@0 |  |  |  |
| n20 | 15 | 25@0 | 50@0 | 75@0 | 100@0 | HD-FDD |
|  | 30 |  | 24@0 | 36@0 | 50@0 |  |
| n24 | 15 | 25@0 | 50@0 |  |  | HD-FDD |
|  | 30 |  | 24@0 |  |  |  |
|  | 60 |  | 10@0 |  |  |  |
| n25 | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  | 30 |  | 24@0 | 36@2 | 50@1 |  |
|  | 60 |  | 10@1 | 18@0 | 24@0 |  |
| n26 | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  | 30 |  | 24@0 | 36@2 | 50@1 |  |
| n28 | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  | 30 |  | 24@0 | 36@2 | 50@1 |  |
| n30 | 15 | 25@0 | 50@2 |  |  | HD-FDD |
|  | 30 |  | 24@0 |  |  |  |
| n65 | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  | 30 |  | 24@0 | 36@2 | 50@1 |  |
|  | 60 |  | 10@1 | 18@0 | 24@0 |  |
| n66 | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  | 30 |  | 24@0 | 36@2 | 50@1 |  |
|  | 60 |  | 10@1 | 18@0 | 24@0 |  |
| n70 | 15 | 25@0 | 50@2 | 75@4 | NOTE 1 | HD-FDD |
|  | 30 |  | 24@0 | 36@2 | NOTE 1 |  |
|  | 60 |  | 10@1 | 18@0 | NOTE 1 |  |
| n71 | 15 | 25@0 | 50@0 | 75@0 | 100@0 | HD-FDD |
|  | 30 |  | 24@0 | 36@0 | 50@0 |  |
| n74 | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  | 30 |  | 24@0 | 36@2 | 50@1 |  |
|  | 60 |  | 10@1 | 18@0 | 24@0 |  |
| NOTE 1: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest TX-RX separation (Table 5.4.4-1) shall be used unless otherwise specified. |  |  |  |  |  |  |

7.3I.2.4.2	Test procedure

Same test procedure as steps 1~4 of clause 7.3.2.4.2 with the following exceptions of step 3.

-	Set the Downlink signal level to the appropriate REFSENS value defined in Tables 7.3I.2.5-1, 7.3I.2.5-2 and 7.3I.2.5-5 if 2Rx antennas connected or Tables 7.3I.2.5-3, 7.3I.2.5-4 and 7.3I.2.5-6 if 1Rx antennas connected.

7.3I.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] clause 4.6 ensuring Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED for NR band.

7.3I.2.5	Test requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A3.2 with reference receive power level specified in Table 7.3I.2.5-1 and Table 7.3I.2.5-2 for RedCap UE with 2 Rx antenna port, Tables 7.3I.2.5-3 and Table 7.3I.2.5-4 for RedCap UE with single antenna port, Table 7.3I.2.5-5 for HD-FDD RedCap UE with 2 Rx antenna port, Table 7.3I.2.5-6 for HD-FDD RedCap UE single antenna port, and parameters specified Table 7.3.2.4.1-1, Table 7.3.2.4.1-2 and Table 7.3I.2.4.1-1.

Table 7.3I.2.5-1: Two antenna port Reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth / REFSENS / Duplex Mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| n1 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.8 +TT | FDD |
|  | 30 |  | -97.1 +TT | -95.1 +TT | -94.0 +TT |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT |  |
| n2 | 15 | -98.0 +TT | -94.8 +TT | -93.0 +TT | -91.8 +TT | FDD |
|  | 30 |  | -95.1 +TT | -93.1 +TT | -92.0 +TT |  |
|  | 60 |  | -95.5 +TT | -93.4 +TT | -92.2 +TT |  |
| n3 | 15 | -97.0 +TT | -93.8 +TT | -92.0 +TT | -90.8 +TT | FDD |
|  | 30 |  | -94.1 +TT | -92.1 +TT | -91.0 +TT |  |
|  | 60 |  | -94.5 +TT | -92.4 +TT | -91.2 +TT |  |
| n5 | 15 | -98.0 +TT | -94.8 +TT | -93.0 +TT | -90.8 +TT | FDD |
|  | 30 |  | -95.1 +TT | -93.1 +TT | -91.0 +TT |  |
| n7 | 15 | -98.0 +TT | -94.8 +TT | -93.0 +TT | -91.8 +TT | FDD |
|  | 30 |  | -95.1 +TT | -93.1 +TT | -92.0 +TT |  |
|  | 60 |  | -95.5 +TT | -93.4 +TT | -92.2 +TT |  |
| n8 | 15 | -97.0 +TT | -93.8 +TT | -91.4 +TT | -85.5 +TT | FDD |
|  | 30 |  | -94.1 +TT | -91.7 +TT | -87.2 +TT |  |
| n12 | 15 | -97.0 +TT | -93.8 +TT | -84.0 +TT |  | FDD |
|  | 30 |  | -94.1 +TT | -84.1 +TT |  |  |
| n14 | 15 | -97.0 +TT | -93.8 +TT |  |  | FDD |
|  | 30 |  | -94.1 +TT |  |  |  |
| n20 | 15 | -97.0 +TT | -93.8 +TT | -91.0 +TT | -89.8 +TT | FDD |
|  | 30 |  | -94.1 +TT | -91.1 +TT | -90.0 +TT |  |
| n24 | 15 | -100.0 +TT | -96.8 +TT |  |  | FDD |
|  | 30 |  | -97.1 +TT |  |  |  |
|  | 60 |  | -97.5 +TT |  |  |  |
| n25 | 15 | -96.5 +TT | -93.3 +TT | -91.5 +TT | -90.3 +TT | FDD |
|  | 30 |  | -93.6 +TT | -91.6 +TT | -90.5 +TT |  |
|  | 60 |  | -94.0 +TT | -91.9 +TT | -90.7 +TT |  |
| n26 | 15 | -97.5 +TT | -94.5 +TT | -92.7 +TT | -87.6 +TT | FDD |
|  | 30 |  | -94.8 +TT | -92.7 +TT | -87.7 +TT |  |
| n28 | 15 | -98.5 +TT | -95.5 +TT | -93.5 +TT | -90.8 +TT | FDD |
|  | 30 |  | -95.6 +TT | -93.6 +TT | -91.0 +TT |  |
| n30 | 15 | -99.0 +TT | -95.8 +TT |  |  | FDD |
|  | 30 |  | -96.1 +TT |  |  |  |
| n65 | 15 | -99.5+TT | -96.3+TT | -94.5+TT | -93.3+TT | FDD |
|  | 30 |  | -96.6+TT | -94.6+TT | -93.5+TT |  |
|  | 60 |  | -97.0+TT | -94.9+TT | -93.7+TT |  |
| n66 | 15 | -99.5 +TT | -96.3 +TT | -94.5 +TT | -93.3 +TT | FDD |
|  | 30 |  | -96.6 +TT | -94.6 +TT | -93.5 +TT |  |
|  | 60 |  | -97.0 +TT | -94.9 +TT | -93.7 +TT |  |
| n70 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.8 +TT | FDD |
|  | 30 |  | -97.1 +TT | -95.1 +TT | -94.0 +TT |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT |  |
| n71 | 15 | -97.2 +TT | -94.0 +TT | -91.6 +TT | -86.0 +TT | FDD |
|  | 30 |  | -94.3 +TT | -91.9 +TT | -87.4 +TT |  |
| n74 | 15 | -99.53 +TT | -96.33 +TT | -94.53 +TT | -93.33 +TT | FDD |
|  | 30 |  | -96.63 +TT | -94.63 +TT | -93.53 +TT |  |
|  | 60 |  | -97.03 +TT | -94.93 +TT | -93.73 +TT |  |
| NOTE 1: Void NOTE 2: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 3: 3 indicates that the requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9-1510.9 MHz. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 7.3I.2.5-7. |  |  |  |  |  |  |

Table 7.3I.2.5-2:  Two antenna port reference sensitivity QPSK PREFSENS for TDD bands

| Operating band / SCS / Channel bandwidth / REFSENS / Duplex Mode |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n34 | 15 | 5, 10, 15 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15 | -97.5 + 10log10(NRB/11)+TT |  |
| n381 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20 | -97.5 + 10log10(NRB/11)+TT |  |
| n39 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, | -97.5 + 10log10(NRB/11)+TT |  |
| n40 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20 | -97.5 + 10log10(NRB/11)+TT |  |
| n411 | 15 | 10, 15, 20 | -94.8 + 10log10(NRB/50)+TT | TDD |
|  | 30 | 10, 15, 20 | -95.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20 | -95.5 + 10log10(NRB/11)+TT |  |
| n481 | 15 | 5, 10, 15, 20, | -99 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, | -96.1 + 10log10(NRB/24) +TT |  |
|  | 60 | 10, 15, 20, | -96.5 + 10log10(NRB/11)+TT |  |
| n50 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20 | -97.5 + 10log10(NRB/11)+TT |  |
| n51 | 15 | 5 | -100+TT | TDD |
| n53 | 15 | 5, 10 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10 | -97.1+TT |  |
|  | 60 | 10 | -97.5+TT |  |
| n771,4 | 15 | 10, 15, 20, | -95.3 + 10log10(NRB/50)+TT | TDD |
|  | 30 | 10, 15, 20 | -95.6 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20 | -96.0 + 10log10(NRB/11)+TT |  |
| n781 | 15 | 10, 15, 20 | -95.8 + 10log10(NRB/50) +TT | TDD |
|  | 30 | 10, 15, 20 | -96.1 + 10log10(NRB/24) +TT |  |
|  | 60 | 10, 15, 20 | -96.5 + 10log10(NRB/11)+TT |  |
| n791 | 15 | 10, 20, | -89.6 + 10log10(NRB/216)+TT | TDD |
|  | 30 | 10, 20, | -89.7 + 10log10(NRB/106) +TT |  |
|  | 60 | 10, 20 | -89.9 + 10log10(NRB/51) +TT |  |
| NOTE 1: Void. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: TT for each frequency and channel bandwidth is specified in Table 7.3I.2.5-7. |  |  |  |  |

Table 7.3I.2.5-3: Single antenna port Reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| n1 | 15 | -100.0+2.5 +TT | -96.8 +3 +TT | -95.0 +3 +TT | -93.8 +3 +TT | FDD |
|  | 30 |  | -97.1 +3 +TT | -95.1 +3 +TT | -94.0 +3 +TT |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT |  |
| n2 | 15 | -98.0 +2.5+TT | -94.8 +3 +TT | -93.0 +3 +TT | -91.8 +3 +TT | FDD |
|  | 30 |  | -95.1 +3 +TT | -93.1 +3 +TT | -92.0 +3 +TT |  |
|  | 60 |  | -95.5 +3 +TT | -93.4 +3 +TT | -92.2 +3 +TT |  |
| n3 | 15 | -97.0+2.5 +TT | -93.8 +3 +TT | -92.0 +3 +TT | -90.8 +3 +TT | FDD |
|  | 30 |  | -94.1 +3 +TT | -92.1 +3 +TT | -91.0 +3 +TT |  |
|  | 60 |  | -94.5 +3 +TT | -92.4 +3 +TT | -91.2 +3 +TT |  |
| n5 | 15 | -98.0+2.5 +TT | -94.8 +3 +TT | -93.0 +3 +TT | -90.8 +3 +TT | FDD |
|  | 30 |  | -95.1 +3 +TT | -93.1 +3 +TT | -91.0 +3 +TT |  |
| n7 | 15 | -98.0+2.5 +TT | -94.8 +3 +TT | -93.0 +3 +TT | -91.8 +3 +TT | FDD |
|  | 30 |  | -95.1 +3 +TT | -93.1 +3 +TT | -92.0 +3 +TT |  |
|  | 60 |  | -95.5 +3 +TT | -93.4 +3 +TT | -92.2 +3 +TT |  |
| n8 | 15 | -97.0+2.5 +TT | -93.8 +3 +TT | -91.4 +3 +TT | -85.8 +3 +TT | FDD |
|  | 30 |  | -94.1 +3 +TT | -91.7 +3 +TT | -87.2 +3 +TT |  |
| n12 | 15 | -97.0 +2.5+TT | -93.8 +3 +TT | -84.0 +3 +TT |  | FDD |
|  | 30 |  | -94.1 +3 +TT | -84.1 +3 +TT |  |  |
| n14 | 15 | -97.0+2.5 +TT | -93.8 +3 +TT |  |  | FDD |
|  | 30 |  | -94.1 +3 +TT |  |  |  |
| n20 | 15 | -97.0 +2.5+TT | -93.8 +3 +TT | -91.0 +3 +TT | -89.8 +3 +TT | FDD |
|  | 30 |  | -94.1 +3 +TT | -91.1 +3 +TT | -90.0 +3 +TT |  |
| n24 | 15 | -100.0 +2.5+TT | -96.8 +3 +TT |  |  | FDD |
|  | 30 |  | -97.1 +3 +TT |  |  |  |
|  | 60 |  | -97.5 +3 +TT |  |  |  |
| n25 | 15 | -96.5 +2.5+TT | -93.3 +3 +TT | -91.5 +3 +TT | -90.3 +3 +TT | FDD |
|  | 30 |  | -93.6 +3 +TT | -91.6 +3 +TT | -90.5 +3 +TT |  |
|  | 60 |  | -94.0 +3 +TT | -91.9 +3 +TT | -90.7 +3 +TT |  |
| n26 | 15 | -97.5+2.5 +TT | -94.5 +3 +TT | -92.7 +3 +TT | -87.6 +3 +TT | FDD |
|  | 30 |  | -94.8 +3 +TT | -92.7 +3 +TT | -87.7 +3 +TT |  |
| n28 | 15 | -98.5 +2.5+TT | -95.5 +3 +TT | -93.5 +3 +TT | -90.8 +3 +TT | FDD |
|  | 30 |  | -95.6 +3 +TT | -93.6 +3 +TT | -91.0 +3 +TT |  |
| n30 | 15 | -99.0 +2.5+TT | -95.8 +3 +TT |  |  | FDD |
|  | 30 |  | -96.1 +3 +TT |  |  |  |
| n65 | 15 | -99.5 +2.5+TT | -96.3 +3+TT | -94.5 +3+TT | -93.3 +3+TT | FDD |
|  | 30 |  | -96.6 +3 +TT | -94.6 +3 +TT | -93.5 +3 +TT |  |
|  | 60 |  | -97.0 +3+TT | -94.9 +3+TT | -93.7 +3+TT |  |
| n66 | 15 | -99.5 +2.5+TT | -96.3 +3+TT | -94.5 +3 +TT | -93.3 +3 +TT | FDD |
|  | 30 |  | -96.6 +3 +TT | -94.6 +3 +TT | -93.5 +3+TT |  |
|  | 60 |  | -97.0 +3 +TT | -94.9 +3 +TT | -93.7 +3 +TT |  |
| n70 | 15 | -100.0 +2.5 +TT | -96.8 +3 +TT | -95.0 +3+TT | -93.8 +3 +TT | FDD |
|  | 30 |  | -97.1 +3 +TT | -95.1 +3 +TT | -94.0 +3 +TT |  |
|  | 60 |  | -97.5 +3 +TT | -95.4 +3 +TT | -94.2 +3 +TT |  |
| n71 | 15 | -97.2+2.5 +TT | -94.0 +3 +TT | -91.6 +3 +TT | -86.0 +3 +TT | FDD |
|  | 30 |  | -94.3 +3 +TT | -91.9 +3 +TT | -87.4 +3 +TT |  |
| n74 | 15 | -99.53 +2.5 +TT | -96.33 +3 +TT | -94.53 +3 +TT | -93.33 +3+TT | FDD |
|  | 30 |  | -96.63 +3 +TT | -94.63 +3 +TT | -93.53 +3 +TT |  |
|  | 60 |  | -97.03 +3 +TT | -94.93 +3 +TT | -93.73 +3+TT |  |
| NOTE 1: Void NOTE 2: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 3: 3 indicates that the requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9-1510.9 MHz. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 7.3I.2.5-7. |  |  |  |  |  |  |

Table 7.3I.2.5-4: Single antenna port reference sensitivity QPSK PREFSENS for TDD bands

| Operating band / SCS / Channel bandwidth / REFSENS/Duplex Mode |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n34 | 15 | 5, 10, 15 | -100 + 10log10(NRB/25) +2.5+TT | TDD |
|  | 30 | 10, 15 | -97.1 + 10log10(NRB/24) +2.5+TT |  |
|  | 60 | 10, 15 | -97.5 + 10log10(NRB/11) +2.5+TT |  |
| n381 | 15 | 5, 10, 15, 20, | -100 + 10log10(NRB/25) +2.5+TT | TDD |
|  | 30 | 10, 15, 20, | -97.1 + 10log10(NRB/24) +2.5+TT |  |
|  | 60 | 10, 15, 20 | -97.5 + 10log10(NRB/11) +2.5+TT |  |
| n39 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25) +2.5+TT | TDD |
|  | 30 | 10, 15, 20, | -97.1 + 10log10(NRB/24) +2.5+TT |  |
|  | 60 | 10, 15, 20, | -97.5 + 10log10(NRB/11) +2.5+TT |  |
| n40 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25) +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 + 10log10(NRB/24) +2.5+TT |  |
|  | 60 | 10, 15, 20 | -97.5 + 10log10(NRB/11) +2.5+TT |  |
| n411 | 15 | 10, 15, 20 | -94.8 + 10log10(NRB/50) +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -95.1 + 10log10(NRB/24) +2.5+TT |  |
|  | 60 | 10, 15, 20 | -95.5 + 10log10(NRB/11) +2.5+TT |  |
| n481 | 15 | 5, 10, 15, 20, | -99 + 10log10(NRB/25) +2.5+TT | TDD |
|  | 30 | 10, 15, 20, | -96.1 + 10log10(NRB/24) +2.5+TT |  |
|  | 60 | 10, 15, 20, | -96.5 + 10log10(NRB/11) +2.5+TT |  |
| n50 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25) +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 + 10log10(NRB/24) +2.5+TT |  |
|  | 60 | 10, 15, 20 | -97.5 + 10log10(NRB/11) +2.5+TT |  |
| n51 | 15 | 5 | -100 +2.5+TT | TDD |
| n53 | 15 | 5, 10 | -100 + 10log10(NRB/25) +2.5+TT | TDD |
|  | 30 | 10 | -97.1 +2.5+TT |  |
|  | 60 | 10 | -97.5 +2.5+TT |  |
| n771,4 | 15 | 10, 15, 20, | -95.3 + 10log10(NRB/50) +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -95.6 + 10log10(NRB/24) +2.5+TT |  |
|  | 60 | 10, 15, 20 | -96.0 + 10log10(NRB/11) +2.5+TT |  |
| n781 | 15 | 10, 15, 20 | -95.8 + 10log10(NRB/50) +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -96.1 + 10log10(NRB/24) +2.5+TT |  |
|  | 60 | 10, 15, 20 | -96.5 + 10log10(NRB/11) +2.5+TT |  |
| n791 | 15 | 10, 20, | -89.6 + 10log10(NRB/216) +2.5+TT | TDD |
|  | 30 | 10, 20, | -89.7 + 10log10(NRB/106) +2.5+TT |  |
|  | 60 | 10, 20 | -89.9 + 10log10(NRB/51) +2.5+TT |  |
| NOTE 1: Void. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: TT for each frequency and channel bandwidth is specified in Table 7.3I.2.5-7. |  |  |  |  |

Table 7.3I.2.5-5: Two antenna port reference sensitivity QPSK PREFSENS for HD-FDD operation

| Operating band / SCS / Channel bandwidth/ REFSENS / Duplex Mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| n1 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.7 +TT | HD-FDD |
|  | 30 |  | -97.2 +TT | -95.2 +TT | -93.9 +TT |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT |  |
| n2 | 15 | -98.8 +TT | -95.6 +TT | -93.8 +TT | -92.5 +TT | HD-FDD |
|  | 30 |  | -96.0 +TT | -94.0 +TT | -92.7 +TT |  |
|  | 60 |  | -96.3 +TT | -94.2 +TT | -93.0 +TT |  |
| n3 | 15 | -97.8 +TT | -94.6 +TT | -92.8 +TT | -91.5 +TT | HD-FDD |
|  | 30 |  | -95.0 +TT | -93.0 +TT | -91.7 +TT |  |
|  | 60 |  | -95.3 +TT | -93.2 +TT | -92.0 +TT |  |
| n5 | 15 | -98.8 +TT | -95.6 +TT | -93.8 +TT | -92.5 +TT | HD-FDD |
|  | 30 |  | -96.0 +TT | -94.0 +TT | -92.7 +TT |  |
| n7 | 15 | -98.8 +TT | -95.6 +TT | -93.8 +TT | -92.5 +TT | HD-FDD |
|  | 30 |  | -96.0 +TT | -94.0 +TT | -92.7 +TT |  |
|  | 60 |  | -96.3 +TT | -94.2 +TT | -93.0 +TT |  |
| n8 | 15 | -97.8 +TT | -94.6 +TT | -92.8 +TT | -91.5 +TT | HD-FDD |
|  | 30 |  | -95.0 +TT | -93.0 +TT | -91.7 +TT |  |
| n12 | 15 | -97.8 +TT | -94.6 +TT | -92.8 +TT |  | HD-FDD |
|  | 30 |  | -95.0 +TT | -93.0 +TT |  |  |
| n13 | 15 | -97.8 +TT | -94.6 +TT |  |  | HD-FDD |
|  | 30 |  | -95.0 +TT |  |  |  |
| n14 | 15 | -97.8 +TT | -94.6 +TT |  |  | HD-FDD |
|  | 30 |  | -95.0 +TT |  |  |  |
| n18 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT |  | HD-FDD |
|  | 30 |  | -97.2 +TT | -95.2 +TT |  |  |
| n20 | 15 | -97.8 +TT | -94.6 +TT | -92.8 +TT | -91.5 +TT | HD-FDD |
|  | 30 |  | -95.0 +TT | -93.0 +TT | -91.7 +TT |  |
| n24 | 15 | -100.0 +TT | -96.8 +TT |  |  | HD-FDD |
|  | 30 |  | -97.2 +TT |  |  |  |
|  | 60 |  | -97.5 +TT |  |  |  |
| n25 | 15 | -97.3 +TT | -94.1 +TT | -92.3 +TT | -91.0 +TT | HD-FDD |
|  | 30 |  | -94.5 +TT | -92.5 +TT | -91.2 +TT |  |
|  | 60 |  | -94.8 +TT | -92.7 +TT | -91.5 +TT |  |
| n26 | 15 | -98.3 +TT | -95.1 +TT | -93.3 +TT | -92.0 +TT | HD-FDD |
|  | 30 |  | -95.5 +TT | -93.5 +TT | -92.2 +TT |  |
| n28 | 15 | -99.3 +TT | -96.1 +TT | -94.3 +TT | -93.0 +TT | HD-FDD |
|  | 30 |  | -96.5 +TT | -94.5 +TT | -93.2 +TT |  |
| n30 | 15 | -99.5 +TT | -96.3 +TT |  |  | HD-FDD |
|  | 30 |  | -96.7 +TT |  |  |  |
| n65 | 15 | -100.0 +TT | -96.8+TT | -95.0 +TT | -93.7 +TT | HD-FDD |
|  | 30 |  | -97.2 +TT | -95.2 +TT | -93.9 +TT |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT |  |
| n66 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.7 +TT | HD-FDD |
|  | 30 |  | -97.2 +TT | -95.2 +TT | -93.9 +TT |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT |  |
| n70 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.7 +TT | HD-FDD |
|  | 30 |  | -97.2 +TT | -95.2 +TT | -93.9 +TT |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT |  |
| n71 | 15 | -98.0 +TT | -94.8 | -93.0 +TT | -91.7 +TT | HD-FDD |
|  | 30 |  | -95.2 +TT | -93.2 +TT | -91.9 +TT |  |
| n74 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.7 +TT | HD-FDD |
|  | 30 |  | -97.2 +TT | -95.2 +TT | -93.9 +TT |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 7.3I.2.5-7. |  |  |  |  |  |  |

Table 7.3I.2.5-6: Single antenna port Reference sensitivity QPSK PREFSENS for HD-FDD operation

| Operating band / SCS / Channel bandwidth/ REFSENS / Duplex Mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| n1 | 15 | -97.5 +TT | -94.3 +TT | -92.5 +TT | -91.2 +TT | HD-FDD |
|  | 30 |  | -94.7 +TT | -92.7 +TT | -91.4 +TT |  |
|  | 60 |  | -95.0 +TT | -92.9 +TT | -91.7 +TT |  |
| n2 | 15 | -96.3 +TT | -93.1 +TT | -91.3 +TT | -90.0 +TT | HD-FDD |
|  | 30 |  | -93.5 +TT | -91.5 | -90.2 |  |
|  | 60 |  | -93.8 +TT | -91.7 +TT | -90.5 +TT |  |
| n3 | 15 | -95.3 +TT | -92.1 +TT | -90.3 +TT | -89.0 +TT | HD-FDD |
|  | 30 |  | -92.5 +TT | -90.5 +TT | -89.2 +TT |  |
|  | 60 |  | -92.8 +TT | -90.7 +TT | -89.5 +TT |  |
| n5 | 15 | -96.3 +TT | -93.1 +TT | -91.3 +TT | -90.0 +TT | HD-FDD |
|  | 30 |  | -93.5 +TT | -91.5 +TT | -90.2 +TT |  |
| n7 | 15 | -96.3 +TT | -93.1 +TT | -91.3 +TT | -90.0 +TT | HD-FDD |
|  | 30 |  | -93.5 +TT | -91.5 +TT | -90.2 +TT |  |
|  | 60 |  | -93.8 +TT | -91.7 +TT | -90.5 +TT |  |
| n8 | 15 | -95.3 +TT | -92.1 +TT | -90.3 +TT | -89.0 +TT | HD-FDD |
|  | 30 |  | -92.5 +TT | -90.5 +TT | -89.2 +TT |  |
| n12 | 15 | -95.3 +TT | -92.1 +TT | -90.3 +TT |  | HD-FDD |
|  | 30 |  | -92.5 +TT | -90.5 +TT |  |  |
| n13 | 15 | -95.3 +TT | -92.1 +TT |  |  | HD-FDD |
|  | 30 |  | -92.5 +TT |  |  |  |
| n14 | 15 | -95.3 +TT | -92.1 +TT |  |  | HD-FDD |
|  | 30 |  | -92.5 +TT |  |  |  |
| n18 | 15 | -97.5 +TT | -94.3 +TT | -92.5 +TT |  | HD-FDD |
|  | 30 |  | -94.7 +TT | -92.7 +TT |  |  |
| n20 | 15 | -95.3 +TT | -92.1 +TT | -90.3 +TT | -89.0 +TT | HD-FDD |
|  | 30 |  | -92.5 +TT | -90.5 +TT | -89.2 +TT |  |
| n24 | 15 | -97.5 +TT | -94.3 +TT |  |  | HD-FDD |
|  | 30 |  | -94.7 +TT |  |  |  |
|  | 60 |  | -95.0 +TT |  |  |  |
| n25 | 15 | -94.8 +TT | -91.6 +TT | -89.8 +TT | -88.5 +TT | HD-FDD |
|  | 30 |  | -92.0 +TT | -90.0 +TT | -88.7 +TT |  |
|  | 60 |  | -92.3 +TT | -90.2 +TT | -89.0 +TT |  |
| n26 | 15 | -95.8 +TT | -92.6 +TT | -90.8 +TT | -89.5 +TT | HD-FDD |
|  | 30 |  | -93.0 +TT | -91.0 +TT | -89.7 +TT |  |
| n28 | 15 | -96.8 +TT | -93.6 +TT | -91.8 +TT | -90.5 +TT | HD-FDD |
|  | 30 |  | -94.0 +TT | -92.0 +TT | -90.7 +TT |  |
| n30 | 15 | -97.0 +TT | -93.8 +TT |  |  | HD-FDD |
|  | 30 |  | -94.2 +TT |  |  |  |
| n65 | 15 | -97.5 +TT | -94.3 +TT | -92.5 +TT | -91.2 +TT | HD-FDD |
|  | 30 |  | -94.7 +TT | -92.7 +TT | -91.4 +TT |  |
|  | 60 |  | -95.0 +TT | -92.9 +TT | -91.7 +TT |  |
| n66 | 15 | -97.5 +TT | -94.3 +TT | -92.5 +TT | -91.2 +TT | HD-FDD |
|  | 30 |  | -94.7 +TT | -92.7 +TT | -91.4 +TT |  |
|  | 60 |  | -95.0 +TT | -92.9 +TT | -91.7 +TT |  |
| n70 | 15 | -97.5 +TT | -94.3 +TT | -92.5 +TT | -91.2 +TT | HD-FDD |
|  | 30 |  | -94.7 +TT | -92.7 +TT | -91.4 +TT |  |
|  | 60 |  | -95.0 +TT | -92.9 +TT | -91.7 +TT |  |
| n71 | 15 | -95.5 +TT | -92.3 +TT | -90.5 +TT | -89.2 +TT | HD-FDD |
|  | 30 |  | -92.7 +TT | -90.7 +TT | -89.4 +TT |  |
| n74 | 15 | -97.5 +TT | -94.3 +TT | -92.5 +TT | -91.2 +TT | HD-FDD |
|  | 30 |  | -94.7 +TT | -92.7 +TT | -91.4 +TT |  |
|  | 60 |  | -95.0 +TT | -92.9 +TT | -91.7 |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 7.3I.2.5-7. |  |  |  |  |  |  |

Table 7.3I.2.5-7: Test Tolerance (TT) for RX sensitivity level for RedCap UE

| f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0 GHz |
| --- | --- |
| 0.7 dB | 1.0 dB |

<<< End OF CHANGES >>>