3GPP TSG-RAN5 Meeting #97	R5-227252r1

# Toulouse, France, 14th – 18th November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5428 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of EARFCN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, MediaTek Beijing Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The EARFCN doesn’t match with test frequency for 5 MHz bandwidth in several tables. |  |  |  |  |  |  |  |  |
| Summary of change: |  | EARFCN has been corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test frequencies will remain ambiguous which can result in inconsistent testing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4.4.1, 6.2.4_2.4.1, 6.2.4_4.4.1, 6.2.4B.4.1, 6.6.2.2_1.4.1, 6.6.2.2B.4.1, 6.6.2.2_s.4.1, 6.6.3.3.4.1, 6.6.3.3_1.4.1, 6.6.3.3_2.4.1, 6.6.3B.3.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: added MediaTek Beijing Inc. as co-sourcing company |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

##### 6.2.4.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.4.4.1-1 through table 6.2.4.4.1-26. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.4.4.1-1: Test Configuration Table (network signalled value "NS_03")

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for NS_03 A-MPR |  |  |  |  |  |  |  |
|  |  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Configura-tion ID | Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  |  | FDD | TDD |  | FDD | TDD |
| 1 | 1.4MHz | N/A for A-MPR testing. |  |  | QPSK | 6 | 6 |
| 2 | 1.4MHz |  |  |  | QPSK | 5 | 5 |
| 3 | 1.4MHz |  |  |  | 16QAM (Note 6) | 5 | 5 |
| 4 | 3MHz |  |  |  | QPSK | 15 | 15 |
| 5 | 3MHz |  |  |  | QPSK | 4 | 4 |
| 6 | 3MHz |  |  |  | 16QAM (Note 6) | 15 | 15 |
| 7 | 3MHz |  |  |  | 16QAM (Note 6) | 4 | 4 |
| 8 | 5MHz |  |  |  | QPSK | 25 | 25 |
| 9 | 5MHz |  |  |  | QPSK | 8 | 8 |
| 10 | 5MHz |  |  |  | QPSK | 6 | 6 |
| 11 | 5MHz |  |  |  | 16QAM (Note 6) | 25 | 25 |
| 12 | 5MHz |  |  |  | 16QAM (Note 6) | 8 | 8 |
| 13 | 10MHz |  |  |  | QPSK | 50 | 50 |
| 14 | 10MHz |  |  |  | QPSK | 12 | 12 |
| 15 | 10MHz |  |  |  | QPSK | 6 | 6 |
| 16 | 10MHz |  |  |  | 16QAM (Note 6) | 50 (Note 4) | 50 (Note 4) |
| 17 | 10MHz |  |  |  | 16QAM (Note 6) | 12 | 12 |
| 18 | 15MHz |  |  |  | QPSK | 75 | 75 |
| 19 | 15MHz |  |  |  | QPSK | 16 | 16 |
| 20 | 15MHz |  |  |  | QPSK | 8 | 8 |
| 21 | 15MHz |  |  |  | 16QAM (Note 6) | 75 (Note 4) | 75 (Note 4) |
| 22 | 15MHz |  |  |  | 16QAM (Note 6) | 16 | 16 |
| 23 | 20MHz |  |  |  | QPSK | 100 | 100 |
| 24 | 20MHz |  |  |  | QPSK | 18 | 18 |
| 25 | 20MHz |  |  |  | QPSK | 10 | 10 |
| 26 | 20MHz |  |  |  | 16QAM (Note 6) | 100 (Note 4) | 100 (Note 4) |
| 27 | 20MHz |  |  |  | 16QAM (Note 6) | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 3: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 4: Applies only for UE-Categories ≥2. Note 5: For band 23, above table only applies to mid and high range test frequencies. Low range test frequencies will be covered by NS_11 test configuration table. Note 6: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |  |

Table 6.2.4.4.1-2: Test Configuration Table (network signalled value "NS_04")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range Additional test frequencies for Rel-12 to Rel-13 and bit 2 of modifiedMPRbehavior = 0: For 5 MHz bandwidth: 2499.6 MHz, NUL= 39686 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 Additional test frequencies for Rel-12 and later and bit 2 of modifiedMPRbehavior = 1: For 5 MHz bandwidth: 2500.6 MHz, NUL= 39696 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10 MHz, 15 MHz, 20MHz |  |  |
| Test Parameters for NS_04 A-MPR |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configura-tion ID | Ch BW | Mod'n | RB allocation TDD | Mod'n | RB allocation TDD | RBstart TDD |
| 1 (Note 6) | 5MHz | N/A for A-MPR testing |  | QPSK | 25 | 0 |
| 2 (Note 6) | 5MHz |  |  | QPSK | 8 | Note 3 |
| 3(Note 6) | 5MHz |  |  | QPSK | 6 | Note 3 |
| 4 (Note 6) | 5MHz |  |  | 16QAM (Note 5) | 25 | 0 |
| 5 (Note 6) | 5MHz |  |  | 16QAM (Note 5) | 8 | Note 3 |
| 6 (Note 6) | 10MHz |  |  | QPSK | 1 | 0 |
| 7 (Note 6) | 10MHz |  |  | QPSK | 12 | 0 |
| 8 (Note 6) | 10MHz |  |  | QPSK | 50 | 0 |
| 9 Note 6) | 10MHz |  |  | 16QAM (Note 5) | 50 (Note 4) | 0 |
| 10 (Note 6) | 10MHz |  |  | QPSK | 24 | 13 |
| 11 (Note 6) | 10MHz |  |  | 16QAM (Note 5) | 24 | 13 |
| 12 (Note 6) | 10MHz |  |  | QPSK | 36 | 13 |
| 13 (Note 6) | 10MHz |  |  | QPSK | 12 | 37 |
| 14 (Note 6) | 10MHz |  |  | QPSK | 1 | 49 |
| 15 (Note 6) | 15MHz |  |  | QPSK | 1 | 0 |
| 16 (Note 6) | 15MHz |  |  | QPSK | 16 | 0 |
| 17 (Note 6) | 15MHz |  |  | QPSK | 75 | 0 |
| 18 (Note 6) | 15MHz |  |  | 16QAM (Note 5) | 75 (Note 4) | 0 |
| 19 (Note 6) | 15MHz |  |  | QPSK | 36 | 19 |
| 20 (Note 6) | 15MHz |  |  | 16QAM (Note 5) | 36 (Note 4) | 19 |
| 21 (Note 6) | 15MHz |  |  | QPSK | 50 | 19 |
| 22 (Note 6) | 15MHz |  |  | QPSK | 18 | 56 |
| 23 (Note 6) | 15MHz |  |  | QPSK | 1 | 74 |
| 24 (Note 6) | 20MHz |  |  | QPSK | 1 | 0 |
| 25 (Note 6) | 20MHz |  |  | QPSK | 18 | 0 |
| 26 (Note 6) | 20MHz |  |  | QPSK | 100 | 0 |
| 27 (Note 6) | 20MHz |  |  | 16QAM (Note 5) | 100 (Note 4) | 0 |
| 28 (Note 6) | 20MHz |  |  | QPSK | 50 | 25 |
| 29 (Note 6) | 20MHz |  |  | 16QAM (Note 5) | 50 (Note 4) | 25 |
| 30 (Note 6) | 20MHz |  |  | QPSK | 75 | 25 |
| 31 (Note 6) | 20MHz |  |  | QPSK | 25 | 75 |
| 32 (Note 6) | 20MHz |  |  | QPSK | 1 | 99 |
| 33 (Note 8) | 5MHz |  |  | QPSK | 1 | 0 |
| 34 (Note 8) | 5MHz |  |  | QPSK | 16 | 9 |
| 35 (Note 9) | 5MHz |  |  | QPSK | 25 | 0 |
| 36 (Note 8) | 5MHz |  |  | 16QAM (Note 5) | 16 | 9 |
| 37 (Note 9) | 5MHz |  |  | 16QAM (Note 5) | 25 | 0 |
| 38 (Note 8) | 10MHz |  |  | QPSK | 1 | 0 |
| 39 (Note 8) | 10MHz |  |  | QPSK | 1 | 49 |
| 40 (Note 8) | 10MHz |  |  | QPSK | 24 | 0 |
| 41 (Note 8) | 10MHz |  |  | QPSK | 40 | 10 |
| 42 (Note 7) | 10MHz |  |  | QPSK | 50 | 0 |
| 43 (Note 7) | 10MHz |  |  | 16QAM (Note 5) | 50 (Note 4) | 0 |
| 44 (Note 8) | 15MHz |  |  | QPSK | 1 | 0 |
| 45 (Note 8) | 15MHz |  |  | QPSK | 1 | 74 |
| 46 (Note 8) | 15MHz |  |  | QPSK | 32 | 0 |
| 47 (Note 8) | 15MHz |  |  | QPSK | 60 | 15 |
| 48 (Note 7) | 15MHz |  |  | QPSK | 75 | 0 |
| 49 (Note 7) | 15MHz |  |  | 16QAM (Note 5) | 75 (Note 4) | 0 |
| 50 (Note 8) | 20MHz |  |  | QPSK | 1 | 0 |
| 51 (Note 8) | 20MHz |  |  | QPSK | 1 | 99 |
| 52 (Note 8) | 20MHz |  |  | QPSK | 36 | 0 |
| 53 (Note 8) | 20MHz |  |  | QPSK | 75 | 25 |
| 54 (Note 7) | 20MHz |  |  | QPSK | 100 | 0 |
| 55 (Note 7) | 20MHz |  |  | 16QAM (Note 5) | 100 (Note 4) | 0 |
| Note 1: Test Channel Bandwidths are checked separately for E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The configuration ID will be used to map the applicable Test Configuration to be corresponding Test Requirement in subclause 6.2.4 as not all combinations are necessarily required based on the applicability of the UE. Note 3: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 4: Applies only for UE-Categories ≥2. Note 5: The 16QAM configurations shall not apply to UEs which support UL 64QAM. Note 6: Applies only for Rel-11 and earlier. Note 7: Applies only for Rel-12 and later. Note 8: Applies only for Rel-12 and later and Low Range. Note 9: Applies only for Rel-12 and later and Mid, High Range and additional test frequencies. |  |  |  |  |  |  |

< Unchanged sections omitted >

##### 6.2.4_2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.4_2.4.1-1 through table 6.2.4_2.4.1-27. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.4_2.4.1-1: Test Configuration Table (network signalled value "NS_03")

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for NS_03 A-MPR |  |  |  |  |  |  |  |
|  |  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Configura-tion ID | Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  |  | FDD | TDD |  | FDD | TDD |
| 1 | 1.4MHz | N/A for A-MPR testing |  |  | 64QAM | 5 | 5 |
| 2 | 3MHz |  |  |  | 64QAM (Note 5) | 15 | 15 |
| 3 | 3MHz |  |  |  | 64QAM | 4 | 4 |
| 4 | 5MHz |  |  |  | 64QAM (Note 5) | 25 | 25 |
| 5 | 5MHz |  |  |  | 64QAM | 8 | 8 |
| 6 | 10MHz |  |  |  | 64QAM (Note 5) | 50 | 50 |
| 7 | 10MHz |  |  |  | 64QAM | 12 | 12 |
| 8 | 15MHz |  |  |  | 64QAM (Note 5) | 75 | 75 |
| 9 | 15MHz |  |  |  | 64QAM | 16 | 16 |
| 10 | 20MHz |  |  |  | 64QAM (Note 5) | 100 | 100 |
| 11 | 20MHz |  |  |  | 64QAM | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4_2.5 as not all combinations are necessarily required based on the applicability of the UE. Note 3: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 4: For band 23, above table only applies to mid and high range test frequencies. Low range test frequencies will be covered by NS_11 test configuration table. Note 5: The 64QAM configurations shall not apply to UEs for which TC 6.2.4_4 is executed with network signalled value NS_03. |  |  |  |  |  |  |  |

Table 6.2.4_2.4.1-2: Test Configuration Table (network signalled value "NS_04")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low Range, Mid Range, High Range Additional test frequencies for Rel-12 to Rel-13 and bit 2 of modifiedMPRbehavior = 0: For 5 MHz bandwidth: 2499.6 MHz, NUL= 39686 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 Additional test frequencies for Rel-12 and later and bit 2 of modifiedMPRbehavior = 1: For 5 MHz bandwidth: 2500.6 MHz, NUL= 39696 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |
| Test Parameters for NS_04 A-MPR |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 (Note 1) | 5MHz | N/A for A-MPR testing |  | 64QAM | 16 | 9 |
| 2 (Note 2) | 5MHz |  |  | 64QAM | 25 | 0 |
| 3 | 10MHz |  |  | 64QAM | 50 | 0 |
| 4 | 15MHz |  |  | 64QAM | 75 | 0 |
| 5 | 20MHz |  |  | 64QAM | 100 | 0 |
| Note 1: Only applicable for Low Range. Note 2: Only applicable for Mid, High Range and additional test frequencies. Note 3: The 64QAM configurations shall not apply to UEs for which TC 6.2.4_4 is executed with network signalled value NS_04. |  |  |  |  |  |  |

< Unchanged sections omitted >

##### 6.2.4_4.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.4_4.4.1-1 through table 6.2.4_4.4.1-27. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.4_4.4.1-1: Test Configuration Table (network signalled value "NS_03")

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for NS_03 A-MPR |  |  |  |  |  |  |  |
|  |  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Configura-tion ID | Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  |  | FDD | TDD |  | FDD | TDD |
| 1 | 3MHz | N/A for A-MPR testing |  |  | 256QAM | 15 | 15 |
| 2 | 5MHz |  |  |  | 256QAM | 25 | 25 |
| 3 | 10MHz |  |  |  | 256QAM | 50 | 50 |
| 4 | 15MHz |  |  |  | 256QAM | 75 | 75 |
| 5 | 20MHz |  |  |  | 256QAM | 100 | 100 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4_4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 3: For band 23, above table only applies to mid and high range test frequencies. Low range test frequencies will be covered by NS_11 test configuration table. |  |  |  |  |  |  |  |

Table 6.2.4_4.4.1-2: Test Configuration Table (network signalled value "NS_04")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low Range, Mid Range, High Range Additional test frequencies for Rel-12 to Rel-13 and bit 2 of modifiedMPRbehavior = 0: For 5 MHz bandwidth: 2499.6 MHz, NUL= 39686 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 Additional test frequencies for Rel-12 and later and bit 2 of modifiedMPRbehavior = 1: For 5 MHz bandwidth: 2500.6 MHz, NUL= 39696 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |
| Test Parameters for NS_04 A-MPR |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 (Note 2) | 5MHz | N/A for A-MPR testing |  | 256QAM | 16 | 9 |
| 2 (Note 3) | 5MHz |  |  | 256QAM | 25 | 0 |
| 3 | 10MHz |  |  | 256QAM | 50 | 0 |
| 4 | 15MHz |  |  | 256QAM | 75 | 0 |
| 5 | 20MHz |  |  | 256QAM | 100 | 0 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4_4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 1: Only applicable for Low Range. Note 2: Only applicable for Mid, High Range and additional test frequencies. |  |  |  |  |  |  |

< Unchanged sections omitted >

##### 6.2.4B.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.4B.4.1-1 through table 6.2.4B.4.1-23. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.4B.4.1-1: Test Configuration Table (network signalled value "NS_03")

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] clause 4.1) |  |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS 36.508 [7] clause 4.3.1) |  |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] clause 4.3.1) |  |  |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for NS_03 A-MPR |  |  |  |  |  |  |  |
|  |  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Configuration ID | Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  |  | FDD | TDD |  | FDD | TDD |
| 1 | 1.4MHz | N/A for A-MPR testing. |  |  | QPSK | 6 | 6 |
| 2 | 1.4MHz |  |  |  | QPSK | 5 | 5 |
| 3 | 1.4MHz |  |  |  | 16QAM | 5 | 5 |
| 4 | 3MHz |  |  |  | QPSK | 15 | 15 |
| 5 | 3MHz |  |  |  | QPSK | 4 | 4 |
| 6 | 3MHz |  |  |  | 16QAM | 15 | 15 |
| 7 | 3MHz |  |  |  | 16QAM | 4 | 4 |
| 8 | 5MHz |  |  |  | QPSK | 25 | 25 |
| 9 | 5MHz |  |  |  | QPSK | 8 | 8 |
| 10 | 5MHz |  |  |  | QPSK | 6 | 6 |
| 11 | 5MHz |  |  |  | 16QAM | 25 | 25 |
| 12 | 5MHz |  |  |  | 16QAM | 8 | 8 |
| 13 | 10MHz |  |  |  | QPSK | 50 | 50 |
| 14 | 10MHz |  |  |  | QPSK | 12 | 12 |
| 15 | 10MHz |  |  |  | QPSK | 6 | 6 |
| 16 | 10MHz |  |  |  | 16QAM | 50 | 50 |
| 17 | 10MHz |  |  |  | 16QAM | 12 | 12 |
| 18 | 15MHz |  |  |  | QPSK | 75 | 75 |
| 19 | 15MHz |  |  |  | QPSK | 16 | 16 |
| 20 | 15MHz |  |  |  | QPSK | 8 | 8 |
| 21 | 15MHz |  |  |  | 16QAM | 75 | 75 |
| 22 | 15MHz |  |  |  | 16QAM | 16 | 16 |
| 23 | 20MHz |  |  |  | QPSK | 100 | 100 |
| 24 | 20MHz |  |  |  | QPSK | 18 | 18 |
| 25 | 20MHz |  |  |  | QPSK | 10 | 10 |
| 26 | 20MHz |  |  |  | 16QAM | 100 | 100 |
| 27 | 20MHz |  |  |  | 16QAM | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in clause 6.2.4B.5 as not all combinations are necessarily required based on the applicability of the UE. Note 3: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 4: For band 23, above table only applies to mid and high range test frequencies. Low range test frequencies will be covered by NS_11 test configuration table. |  |  |  |  |  |  |  |

Table 6.2.4B.4.1-2: Test Configuration Table (network signalled value "NS_04")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range Additional test frequencies for Rel-12 to Rel-13 and bit 2 of modifiedMPRbehavior = 0: For 5 MHz bandwidth: 2499.6 MHz, NUL= 39686 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 Additional test frequencies for Rel-12 and later and bit 2 of modifiedMPRbehavior = 1: For 5 MHz bandwidth: 2500.6 MHz, NUL= 39696 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10 MHz, 15 MHz, 20MHz |  |  |
| Test Parameters for NS_04 A-MPR |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configura-tion ID | Ch BW | Mod'n | RB allocation TDD | Mod'n | RB allocation TDD | RBstart TDD |
| 1 (Note 6) | 5MHz | N/A for A-MPR testing |  | QPSK | 25 | 0 |
| 2 (Note 6) | 5MHz |  |  | QPSK | 8 | Note 3 |
| 3(Note 6) | 5MHz |  |  | QPSK | 6 | Note 3 |
| 4 (Note 6) | 5MHz |  |  | 16QAM (Note 5) | 25 | 0 |
| 5 (Note 6) | 5MHz |  |  | 16QAM (Note 5) | 8 | Note 3 |
| 6 (Note 6) | 10MHz |  |  | QPSK | 1 | 0 |
| 7 (Note 6) | 10MHz |  |  | QPSK | 12 | 0 |
| 8 (Note 6) | 10MHz |  |  | QPSK | 50 | 0 |
| 9 Note 6) | 10MHz |  |  | 16QAM (Note 5) | 50 (Note 4) | 0 |
| 10 (Note 6) | 10MHz |  |  | QPSK | 24 | 13 |
| 11 (Note 6) | 10MHz |  |  | 16QAM (Note 5) | 24 | 13 |
| 12 (Note 6) | 10MHz |  |  | QPSK | 36 | 13 |
| 13 (Note 6) | 10MHz |  |  | QPSK | 12 | 37 |
| 14 (Note 6) | 10MHz |  |  | QPSK | 1 | 49 |
| 15 (Note 6) | 15MHz |  |  | QPSK | 1 | 0 |
| 16 (Note 6) | 15MHz |  |  | QPSK | 16 | 0 |
| 17 (Note 6) | 15MHz |  |  | QPSK | 75 | 0 |
| 18 (Note 6) | 15MHz |  |  | 16QAM (Note 5) | 75 (Note 4) | 0 |
| 19 (Note 6) | 15MHz |  |  | QPSK | 36 | 19 |
| 20 (Note 6) | 15MHz |  |  | 16QAM (Note 5) | 36 (Note 4) | 19 |
| 21 (Note 6) | 15MHz |  |  | QPSK | 50 | 19 |
| 22 (Note 6) | 15MHz |  |  | QPSK | 18 | 56 |
| 23 (Note 6) | 15MHz |  |  | QPSK | 1 | 74 |
| 24 (Note 6) | 20MHz |  |  | QPSK | 1 | 0 |
| 25 (Note 6) | 20MHz |  |  | QPSK | 18 | 0 |
| 26 (Note 6) | 20MHz |  |  | QPSK | 100 | 0 |
| 27 (Note 6) | 20MHz |  |  | 16QAM (Note 5) | 100 (Note 4) | 0 |
| 28 (Note 6) | 20MHz |  |  | QPSK | 50 | 25 |
| 29 (Note 6) | 20MHz |  |  | 16QAM (Note 5) | 50 (Note 4) | 25 |
| 30 (Note 6) | 20MHz |  |  | QPSK | 75 | 25 |
| 31 (Note 6) | 20MHz |  |  | QPSK | 25 | 75 |
| 32 (Note 6) | 20MHz |  |  | QPSK | 1 | 99 |
| 33 (Note 8) | 5MHz |  |  | QPSK | 1 | 0 |
| 34 (Note 8) | 5MHz |  |  | QPSK | 16 | 9 |
| 35 (Note 9) | 5MHz |  |  | QPSK | 25 | 0 |
| 36 (Note 8) | 5MHz |  |  | 16QAM (Note 5) | 16 | 9 |
| 37 (Note 9) | 5MHz |  |  | 16QAM (Note 5) | 25 | 0 |
| 38 (Note 8) | 10MHz |  |  | QPSK | 1 | 0 |
| 39 (Note 8) | 10MHz |  |  | QPSK | 1 | 49 |
| 40 (Note 8) | 10MHz |  |  | QPSK | 24 | 0 |
| 41 (Note 8) | 10MHz |  |  | QPSK | 40 | 10 |
| 42 (Note 7) | 10MHz |  |  | QPSK | 50 | 0 |
| 43 (Note 7) | 10MHz |  |  | 16QAM (Note 5) | 50 | 0 |
| 44 (Note 8) | 15MHz |  |  | QPSK | 1 | 0 |
| 45 (Note 8) | 15MHz |  |  | QPSK | 1 | 74 |
| 46 (Note 8) | 15MHz |  |  | QPSK | 32 | 0 |
| 47 (Note 8) | 15MHz |  |  | QPSK | 60 | 15 |
| 48 (Note 7) | 15MHz |  |  | QPSK | 75 | 0 |
| 49 (Note 7) | 15MHz |  |  | 16QAM (Note 5) | 75 | 0 |
| 50 (Note 8) | 20MHz |  |  | QPSK | 1 | 0 |
| 51 (Note 8) | 20MHz |  |  | QPSK | 1 | 99 |
| 52 (Note 8) | 20MHz |  |  | QPSK | 36 | 0 |
| 53 (Note 8) | 20MHz |  |  | QPSK | 75 | 25 |
| 54 (Note 7) | 20MHz |  |  | QPSK | 100 | 0 |
| 55 (Note 7) | 20MHz |  |  | 16QAM (Note 5) | 100 | 0 |
| Note 1: Test Channel Bandwidths are checked separately for E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The configuration ID will be used to map the applicable Test Configuration to be corresponding Test Requirement in subclause 6.2.4 as not all combinations are necessarily required based on the applicability of the UE. Note 3: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 4: Applies only for UE-Categories ≥2. Note 5: Void. Note 6: Applies only for Rel-11 and earlier. Note 7: Applies only for Rel-12 and later. Note 8: Applies only for Rel-12 and later and Low Range. Note 9: Applies only for Rel-12 and later and Mid, High Range and additional test frequencies. |  |  |  |  |  |  |

< Unchanged sections omitted >

6.6.2.2_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Tables 6.6.2.2_1.4.1-1 through 6.6.2.2_1.4.1-9. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 respectively. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.2_1.4.1-1: Test Configuration Table (network signalled value "NS_03")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 1.4MHz | N/A for Additional Spectrum Emission Mask testing. |  |  | 64QAM | 5 | 5 |
| 3MHz |  |  |  | 64QAM | 15 (Note 3) | 15 |
| 3MHz |  |  |  | 64QAM | 4 | 4 |
| 5MHz |  |  |  | 64QAM | 25 (Note 3) | 25 |
| 5MHz |  |  |  | 64QAM | 8 | 8 |
| 10MHz |  |  |  | 64QAM | 50 (Note 3) | 50 |
| 10MHz |  |  |  | 64QAM | 12 | 12 |
| 15MHz |  |  |  | 64QAM | 75 (Note 3) | 75 |
| 15MHz |  |  |  | 64QAM | 16 | 16 |
| 20MHz |  |  |  | 64QAM | 100 (Note 3) | 100 |
| 20MHz |  |  |  | 64QAM | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 3: The 64QAM configurations shall not apply to UEs for which TC 6.6.2.2_2 is executed with network signalled value NS_03. |  |  |  |  |  |  |

Table 6.6.2.2_1.4.1-2: Test Configuration Table (network signalled value "NS_04")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low Range, Mid Range, High Range Additional test frequencies for Rel-12 to Rel-13 and bit 2 of modifiedMPRbehavior = 0: For 5 MHz bandwidth: 2499.6 MHz, NUL= 39686 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 Additional test frequencies for Rel-12 and later and bit 2 of modifiedMPRbehavior = 1: For 5 MHz bandwidth: 2500.6 MHz, NUL= 39696 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |
| Test Parameters for NS_04 A-MPR |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 (Note 1) | 5MHz | N/A for A-MPR testing |  | 64QAM | 16 | 9 |
| 2 (Note 2) | 5MHz |  |  | 64QAM | 25 | 0 |
| 3 | 10MHz |  |  | 64QAM | 50 | 0 |
| 4 | 15MHz |  |  | 64QAM | 75 | 0 |
| 5 | 20MHz |  |  | 64QAM | 100 | 0 |
| Note 1: Only applicable for Low Range. Note 2: Only applicable for Mid, High Range and additional test frequencies. Note 3: The 64QAM configurations shall not apply to UEs for which TC 6.6.2.2_2 is executed with network signalled value NS_04. |  |  |  |  |  |  |

< Unchanged sections omitted >

6.6.2.2B.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Tables 6.6.2.2B.4.1-1 to 6.6.2.2B.4.1-7.The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2 respectively. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.2B.4.1-1: Test Configuration Table (network signalled value "NS_03", “NS_11”, and “NS_20”)

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] clause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS 36.508 [7] clause 4.3.1) |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] clause 4.3.1) |  |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 1.4MHz | N/A for Additional Spectrum Emission Mask testing. |  |  | QPSK | 6 | 6 |
| 1.4MHz |  |  |  | QPSK | 5 | 5 |
| 1.4MHz |  |  |  | 16QAM | 5 | 5 |
| 3MHz |  |  |  | QPSK | 15 | 15 |
| 3MHz |  |  |  | QPSK | 4 | 4 |
| 3MHz |  |  |  | 16QAM | 15 | 15 |
| 3MHz |  |  |  | 16QAM | 4 | 4 |
| 5MHz |  |  |  | QPSK | 25 | 25 |
| 5MHz |  |  |  | QPSK | 8 | 8 |
| 5MHz |  |  |  | QPSK | 6 | 6 |
| 5MHz |  |  |  | 16QAM | 25 | 25 |
| 5MHz |  |  |  | 16QAM | 8 | 8 |
| 10MHz |  |  |  | QPSK | 50 | 50 |
| 10MHz |  |  |  | QPSK | 12 | 12 |
| 10MHz |  |  |  | QPSK | 6 | 6 |
| 10MHz |  |  |  | 16QAM | 50 (Note 3) | 50 (Note 3) |
| 10MHz |  |  |  | 16QAM | 12 | 12 |
| 15MHz |  |  |  | QPSK | 75 | 75 |
| 15MHz |  |  |  | QPSK | 16 | 16 |
| 15MHz |  |  |  | QPSK | 8 | 8 |
| 15MHz |  |  |  | 16QAM | 75 (Note 3) | 75 (Note 3) |
| 15MHz |  |  |  | 16QAM | 16 | 16 |
| 20MHz |  |  |  | QPSK | 100 | 100 |
| 20MHz |  |  |  | QPSK | 18 | 18 |
| 20MHz |  |  |  | QPSK | 10 | 10 |
| 20MHz |  |  |  | 16QAM | 100 (Note 3) | 100 (Note 3) |
| 20MHz |  |  |  | 16QAM | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 3: Applies only for UE-Categories [FFS]. |  |  |  |  |  |  |

Table 6.6.2.2B.4.1-2: Test Configuration Table (network signalled value "NS_04")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range Additional test frequencies for Rel-12 to Rel-13 and bit 2 of modifiedMPRbehavior = 0: For 5 MHz bandwidth: 2499.6 MHz, NUL= 39686 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 Additional test frequencies for Rel-12 and later and bit 2 of modifiedMPRbehavior = 1: For 5 MHz bandwidth: 2500.6 MHz, NUL= 39696 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10 MHz, 15 MHz, 20MHz |  |  |
| Test Parameters for NS_04 A-MPR |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configura-tion ID | Ch BW | Mod'n | RB allocation TDD | Mod'n | RB allocation TDD | RBstart TDD |
| 1 (Note 6) | 5MHz | N/A for A-MPR testing |  | QPSK | 25 | 0 |
| 2 (Note 6) | 5MHz |  |  | QPSK | 8 | Note 3 |
| 3(Note 6) | 5MHz |  |  | QPSK | 6 | Note 3 |
| 4 (Note 6) | 5MHz |  |  | 16QAM (Note 5) | 25 | 0 |
| 5 (Note 6) | 5MHz |  |  | 16QAM (Note 5) | 8 | Note 3 |
| 6 (Note 6) | 10MHz |  |  | QPSK | 1 | 0 |
| 7 (Note 6) | 10MHz |  |  | QPSK | 12 | 0 |
| 8 (Note 6) | 10MHz |  |  | QPSK | 50 | 0 |
| 9 Note 6) | 10MHz |  |  | 16QAM (Note 5) | 50 (Note 4) | 0 |
| 10 (Note 6) | 10MHz |  |  | QPSK | 24 | 13 |
| 11 (Note 6) | 10MHz |  |  | 16QAM (Note 5) | 24 | 13 |
| 12 (Note 6) | 10MHz |  |  | QPSK | 36 | 13 |
| 13 (Note 6) | 10MHz |  |  | QPSK | 12 | 37 |
| 14 (Note 6) | 10MHz |  |  | QPSK | 1 | 49 |
| 15 (Note 6) | 15MHz |  |  | QPSK | 1 | 0 |
| 16 (Note 6) | 15MHz |  |  | QPSK | 16 | 0 |
| 17 (Note 6) | 15MHz |  |  | QPSK | 75 | 0 |
| 18 (Note 6) | 15MHz |  |  | 16QAM (Note 5) | 75 (Note 4) | 0 |
| 19 (Note 6) | 15MHz |  |  | QPSK | 36 | 19 |
| 20 (Note 6) | 15MHz |  |  | 16QAM (Note 5) | 36 (Note 4) | 19 |
| 21 (Note 6) | 15MHz |  |  | QPSK | 50 | 19 |
| 22 (Note 6) | 15MHz |  |  | QPSK | 18 | 56 |
| 23 (Note 6) | 15MHz |  |  | QPSK | 1 | 74 |
| 24 (Note 6) | 20MHz |  |  | QPSK | 1 | 0 |
| 25 (Note 6) | 20MHz |  |  | QPSK | 18 | 0 |
| 26 (Note 6) | 20MHz |  |  | QPSK | 100 | 0 |
| 27 (Note 6) | 20MHz |  |  | 16QAM (Note 5) | 100 (Note 4) | 0 |
| 28 (Note 6) | 20MHz |  |  | QPSK | 50 | 25 |
| 29 (Note 6) | 20MHz |  |  | 16QAM (Note 5) | 50 (Note 4) | 25 |
| 30 (Note 6) | 20MHz |  |  | QPSK | 75 | 25 |
| 31 (Note 6) | 20MHz |  |  | QPSK | 25 | 75 |
| 32 (Note 6) | 20MHz |  |  | QPSK | 1 | 99 |
| 33 (Note 8) | 5MHz |  |  | QPSK | 1 | 0 |
| 34 (Note 8) | 5MHz |  |  | QPSK | 16 | 9 |
| 35 (Note 9) | 5MHz |  |  | QPSK | 25 | 0 |
| 36 (Note 8) | 5MHz |  |  | 16QAM (Note 5) | 16 | 9 |
| 37 (Note 9) | 5MHz |  |  | 16QAM (Note 5) | 25 | 0 |
| 38 (Note 8) | 10MHz |  |  | QPSK | 1 | 0 |
| 39 (Note 8) | 10MHz |  |  | QPSK | 1 | 49 |
| 40 (Note 8) | 10MHz |  |  | QPSK | 24 | 0 |
| 41 (Note 8) | 10MHz |  |  | QPSK | 40 | 10 |
| 42 (Note 7) | 10MHz |  |  | QPSK | 50 | 0 |
| 43 (Note 7) | 10MHz |  |  | 16QAM (Note 5) | 50 | 0 |
| 44 (Note 8) | 15MHz |  |  | QPSK | 1 | 0 |
| 45 (Note 8) | 15MHz |  |  | QPSK | 1 | 74 |
| 46 (Note 8) | 15MHz |  |  | QPSK | 32 | 0 |
| 47 (Note 8) | 15MHz |  |  | QPSK | 60 | 15 |
| 48 (Note 7) | 15MHz |  |  | QPSK | 75 | 0 |
| 49 (Note 7) | 15MHz |  |  | 16QAM (Note 5) | 75 | 0 |
| 50 (Note 8) | 20MHz |  |  | QPSK | 1 | 0 |
| 51 (Note 8) | 20MHz |  |  | QPSK | 1 | 99 |
| 52 (Note 8) | 20MHz |  |  | QPSK | 36 | 0 |
| 53 (Note 8) | 20MHz |  |  | QPSK | 75 | 25 |
| 54 (Note 7) | 20MHz |  |  | QPSK | 100 | 0 |
| 55 (Note 7) | 20MHz |  |  | 16QAM (Note 5) | 100 | 0 |
| Note 1: Test Channel Bandwidths are checked separately for E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The configuration ID will be used to map the applicable Test Configuration to be corresponding Test Requirement in subclause 6.2.4 as not all combinations are necessarily required based on the applicability of the UE. Note 3: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 4: Applies only for UE-Categories ≥2. Note 5: Void. Note 6: Applies only for Rel-11 and earlier. Note 7: Applies only for Rel-12 and later. Note 8: Applies only for Rel-12 and later and Low Range. Note 9: Applies only for Rel-12 and later and Mid, High Range and additional test frequencies. |  |  |  |  |  |  |

< Unchanged sections omitted >

6.6.2.2_s.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Tables 6.6.2.2_s.4.1-1 through 6.6.2.2_s.4.1-9. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2 respectively. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.2.2_s.4.1-1: Test Configuration Table (network signalled value "NS_03")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 1.4MHz | N/A for Additional Spectrum Emission Mask testing. |  |  | QPSK | 6 | 6 |
| 1.4MHz |  |  |  | QPSK | 5 | 5 |
| 1.4MHz |  |  |  | 16QAM (Note 4) | 5 | 5 |
| 3MHz |  |  |  | QPSK | 15 | 15 |
| 3MHz |  |  |  | QPSK | 4 | 4 |
| 3MHz |  |  |  | 16QAM (Note 4) | 15 | 15 |
| 3MHz |  |  |  | 16QAM (Note 4) | 4 | 4 |
| 5MHz |  |  |  | QPSK | 25 | 25 |
| 5MHz |  |  |  | QPSK | 8 | 8 |
| 5MHz |  |  |  | QPSK | 6 | 6 |
| 5MHz |  |  |  | 16QAM (Note 4) | 25 | 25 |
| 5MHz |  |  |  | 16QAM (Note 4) | 8 | 8 |
| 10MHz |  |  |  | QPSK | 50 | 50 |
| 10MHz |  |  |  | QPSK | 12 | 12 |
| 10MHz |  |  |  | QPSK | 6 | 6 |
| 10MHz |  |  |  | 16QAM (Note 4) | 50 (Note 3) | 50 (Note 3) |
| 10MHz |  |  |  | 16QAM (Note 4) | 12 | 12 |
| 15MHz |  |  |  | QPSK | 75 | 75 |
| 15MHz |  |  |  | QPSK | 16 | 16 |
| 15MHz |  |  |  | QPSK | 8 | 8 |
| 15MHz |  |  |  | 16QAM (Note 4) | 75 (Note 3) | 75 (Note 3) |
| 15MHz |  |  |  | 16QAM (Note 4) | 16 | 16 |
| 20MHz |  |  |  | QPSK | 100 | 100 |
| 20MHz |  |  |  | QPSK | 18 | 18 |
| 20MHz |  |  |  | QPSK | 10 | 10 |
| 20MHz |  |  |  | 16QAM (Note 4) | 100 (Note 3) | 100 (Note 3) |
| 20MHz |  |  |  | 16QAM (Note 4) | 18 | 18 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 3: Applies only for UE-Categories ≥2. Note 4: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |

Table 6.6.2.2_s.4.1-2: Test Configuration Table (network signalled value "NS_04")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range Additional test frequencies for Rel-12 to Rel-13 and bit 2 of modifiedMPRbehavior = 0: For 5 MHz bandwidth: 2499.6 MHz, NUL= 39686 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 Additional test frequencies for Rel-12 and later and bit 2 of modifiedMPRbehavior = 1: For 5 MHz bandwidth: 2500.6 MHz, NUL= 39696 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10 MHz, 15 MHz, 20MHz |  |  |
| Test Parameters for NS_04 A-MPR |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configura-tion ID | Ch BW | Mod'n | RB allocation TDD | Mod'n | RB allocation TDD | RBstart TDD |
| 1 (Note 6) | 5MHz | N/A for A-MPR testing |  | QPSK | 25 | 0 |
| 2 (Note 6) | 5MHz |  |  | QPSK | 8 | Note 3 |
| 3(Note 6) | 5MHz |  |  | QPSK | 6 | Note 3 |
| 4 (Note 6) | 5MHz |  |  | 16QAM (Note 5) | 25 | 0 |
| 5 (Note 6) | 5MHz |  |  | 16QAM (Note 5) | 8 | Note 3 |
| 6 (Note 6) | 10MHz |  |  | QPSK | 1 | 0 |
| 7 (Note 6) | 10MHz |  |  | QPSK | 12 | 0 |
| 8 (Note 6) | 10MHz |  |  | QPSK | 50 | 0 |
| 9 Note 6) | 10MHz |  |  | 16QAM (Note 5) | 50 (Note 4) | 0 |
| 10 (Note 6) | 10MHz |  |  | QPSK | 24 | 13 |
| 11 (Note 6) | 10MHz |  |  | 16QAM (Note 5) | 24 | 13 |
| 12 (Note 6) | 10MHz |  |  | QPSK | 36 | 13 |
| 13 (Note 6) | 10MHz |  |  | QPSK | 12 | 37 |
| 14 (Note 6) | 10MHz |  |  | QPSK | 1 | 49 |
| 15 (Note 6) | 15MHz |  |  | QPSK | 1 | 0 |
| 16 (Note 6) | 15MHz |  |  | QPSK | 16 | 0 |
| 17 (Note 6) | 15MHz |  |  | QPSK | 75 | 0 |
| 18 (Note 6) | 15MHz |  |  | 16QAM (Note 5) | 75 (Note 4) | 0 |
| 19 (Note 6) | 15MHz |  |  | QPSK | 36 | 19 |
| 20 (Note 6) | 15MHz |  |  | 16QAM (Note 5) | 36 (Note 4) | 19 |
| 21 (Note 6) | 15MHz |  |  | QPSK | 50 | 19 |
| 22 (Note 6) | 15MHz |  |  | QPSK | 18 | 56 |
| 23 (Note 6) | 15MHz |  |  | QPSK | 1 | 74 |
| 24 (Note 6) | 20MHz |  |  | QPSK | 1 | 0 |
| 25 (Note 6) | 20MHz |  |  | QPSK | 18 | 0 |
| 26 (Note 6) | 20MHz |  |  | QPSK | 100 | 0 |
| 27 (Note 6) | 20MHz |  |  | 16QAM (Note 5) | 100 (Note 4) | 0 |
| 28 (Note 6) | 20MHz |  |  | QPSK | 50 | 25 |
| 29 (Note 6) | 20MHz |  |  | 16QAM (Note 5) | 50 (Note 4) | 25 |
| 30 (Note 6) | 20MHz |  |  | QPSK | 75 | 25 |
| 31 (Note 6) | 20MHz |  |  | QPSK | 25 | 75 |
| 32 (Note 6) | 20MHz |  |  | QPSK | 1 | 99 |
| 33 (Note 8) | 5MHz |  |  | QPSK | 1 | 0 |
| 34 (Note 8) | 5MHz |  |  | QPSK | 16 | 9 |
| 35 (Note 9) | 5MHz |  |  | QPSK | 25 | 0 |
| 36 (Note 8) | 5MHz |  |  | 16QAM (Note 5) | 16 | 9 |
| 37 (Note 9) | 5MHz |  |  | 16QAM (Note 5) | 25 | 0 |
| 38 (Note 8) | 10MHz |  |  | QPSK | 1 | 0 |
| 39 (Note 8) | 10MHz |  |  | QPSK | 1 | 49 |
| 40 (Note 8) | 10MHz |  |  | QPSK | 24 | 0 |
| 41 (Note 8) | 10MHz |  |  | QPSK | 40 | 10 |
| 42 (Note 7) | 10MHz |  |  | QPSK | 50 | 0 |
| 43 (Note 7) | 10MHz |  |  | 16QAM (Note 5) | 50 | 0 |
| 44 (Note 8) | 15MHz |  |  | QPSK | 1 | 0 |
| 45 (Note 8) | 15MHz |  |  | QPSK | 1 | 74 |
| 46 (Note 8) | 15MHz |  |  | QPSK | 32 | 0 |
| 47 (Note 8) | 15MHz |  |  | QPSK | 60 | 15 |
| 48 (Note 7) | 15MHz |  |  | QPSK | 75 | 0 |
| 49 (Note 7) | 15MHz |  |  | 16QAM (Note 5) | 75 | 0 |
| 50 (Note 8) | 20MHz |  |  | QPSK | 1 | 0 |
| 51 (Note 8) | 20MHz |  |  | QPSK | 1 | 99 |
| 52 (Note 8) | 20MHz |  |  | QPSK | 36 | 0 |
| 53 (Note 8) | 20MHz |  |  | QPSK | 75 | 25 |
| 54 (Note 7) | 20MHz |  |  | QPSK | 100 | 0 |
| 55 (Note 7) | 20MHz |  |  | 16QAM (Note 5) | 100 | 0 |
| Note 1: Test Channel Bandwidths are checked separately for E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The configuration ID will be used to map the applicable Test Configuration to be corresponding Test Requirement in subclause 6.2.4 as not all combinations are necessarily required based on the applicability of the UE. Note 3: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 4: Applies only for UE-Categories ≥2. Note 5: The 16QAM configurations shall not apply to UEs which support UL 64QAM. Note 6: Applies only for Rel-11 and earlier. Note 7: Applies only for Rel-12 and later. Note 8: Applies only for Rel-12 and later and Low Range. Note 9: Applies only for Rel-12 and later and Mid, High Range and additional test frequencies. |  |  |  |  |  |  |

< Unchanged sections omitted >

6.6.3.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 6.6.3.3.4.1-1 through Table 6.6.3.3.4.1-24. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.3.3.4.1-1: Test Configuration Table (network signalled value "NS_04") (Power Class 2)

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low Range Additional test frequencies: For 5 MHz bandwidth: 2500.6 MHz, NUL= 39696 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |
| Test Parameters for NS_04 A-MPR |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | Ch BW | Mod'n | RB allocation TDD | Mod'n | RB allocation TDD | RBstart TDD |
| 1 (Note 1) | 5MHz | N/A for A-MPR testing |  | QPSK | 1 | 0 |
| 2 (Note 1) | 5MHz |  |  | QPSK | 16 | 9 |
| 3 (Note 2) | 5MHz |  |  | QPSK | 25 | 0 |
| 4 (Note 1) | 5MHz |  |  | 16QAM | 16 | 9 |
| 5 (Note 2) | 5MHz |  |  | 16QAM | 25 | 0 |
| 6 (Note 1) | 10MHz |  |  | QPSK | 1 | 0 |
| 7 (Note 1) | 10MHz |  |  | QPSK | 1 | 49 |
| 8 (Note 1) | 10MHz |  |  | QPSK | 24 | 0 |
| 9 (Note 1) | 10MHz |  |  | QPSK | 40 | 10 |
| 10 | 10MHz |  |  | QPSK | 50 | 0 |
| 11 | 10MHz |  |  | 16QAM | 50 (Note 3) | 0 |
| 12 (Note 1) | 15MHz |  |  | QPSK | 1 | 0 |
| 13 (Note 1) | 15MHz |  |  | QPSK | 1 | 74 |
| 14 (Note 1) | 15MHz |  |  | QPSK | 32 | 0 |
| 15 (Note 1) | 15MHz |  |  | QPSK | 60 | 15 |
| 16 | 15MHz |  |  | QPSK | 75 | 0 |
| 17 | 15MHz |  |  | 16QAM | 75 (Note 3) | 0 |
| 18 (Note 1) | 20MHz |  |  | QPSK | 1 | 0 |
| 19 (Note 1) | 20MHz |  |  | QPSK | 1 | 99 |
| 20 (Note 1) | 20MHz |  |  | QPSK | 36 | 0 |
| 21 (Note 1) | 20MHz |  |  | QPSK | 75 | 25 |
| 22 | 20MHz |  |  | QPSK | 100 | 0 |
| 23 | 20MHz |  |  | 16QAM | 100 (Note 3) | 0 |
| Note 1: Only applicable for Low Range. Note 2: Only applicable for additional test frequencies. Note 3: Applies only for UE-Categories ≥2. |  |  |  |  |  |  |

Table 6.6.3.3.4.1-2: Test Configuration Table (network signalled value "NS_04") (Power Class 3)

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range Additional test frequencies for Rel-12 to Rel-13 and bit 2 of modifiedMPRbehavior = 0: For 5 MHz bandwidth: 2499.6 MHz, NUL= 39686 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 Additional test frequencies for Rel-12 and later and bit 2 of modifiedMPRbehavior = 1: For 5 MHz bandwidth: 2500.6 MHz, NUL= 39696 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10 MHz, 15 MHz, 20MHz |  |  |
| Test Parameters for NS_04 A-MPR |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configura-tion ID | Ch BW | Mod'n | RB allocation TDD | Mod'n | RB allocation TDD | RBstart TDD |
| 1 (Note 6) | 5MHz | N/A for A-MPR testing |  | QPSK | 25 | 0 |
| 2 (Note 6) | 5MHz |  |  | QPSK | 8 | Note 3 |
| 3(Note 6) | 5MHz |  |  | QPSK | 6 | Note 3 |
| 4 (Note 6) | 5MHz |  |  | 16QAM (Note 5) | 25 | 0 |
| 5 (Note 6) | 5MHz |  |  | 16QAM (Note 5) | 8 | Note 3 |
| 6 (Note 6) | 10MHz |  |  | QPSK | 1 | 0 |
| 7 (Note 6) | 10MHz |  |  | QPSK | 12 | 0 |
| 8 (Note 6) | 10MHz |  |  | QPSK | 50 | 0 |
| 9 Note 6) | 10MHz |  |  | 16QAM (Note 5) | 50 (Note 4) | 0 |
| 10 (Note 6) | 10MHz |  |  | QPSK | 24 | 13 |
| 11 (Note 6) | 10MHz |  |  | 16QAM (Note 5) | 24 | 13 |
| 12 (Note 6) | 10MHz |  |  | QPSK | 36 | 13 |
| 13 (Note 6) | 10MHz |  |  | QPSK | 12 | 37 |
| 14 (Note 6) | 10MHz |  |  | QPSK | 1 | 49 |
| 15 (Note 6) | 15MHz |  |  | QPSK | 1 | 0 |
| 16 (Note 6) | 15MHz |  |  | QPSK | 16 | 0 |
| 17 (Note 6) | 15MHz |  |  | QPSK | 75 | 0 |
| 18 (Note 6) | 15MHz |  |  | 16QAM (Note 5) | 75 (Note 4) | 0 |
| 19 (Note 6) | 15MHz |  |  | QPSK | 36 | 19 |
| 20 (Note 6) | 15MHz |  |  | 16QAM (Note 5) | 36 (Note 4) | 19 |
| 21 (Note 6) | 15MHz |  |  | QPSK | 50 | 19 |
| 22 (Note 6) | 15MHz |  |  | QPSK | 18 | 56 |
| 23 (Note 6) | 15MHz |  |  | QPSK | 1 | 74 |
| 24 (Note 6) | 20MHz |  |  | QPSK | 1 | 0 |
| 25 (Note 6) | 20MHz |  |  | QPSK | 18 | 0 |
| 26 (Note 6) | 20MHz |  |  | QPSK | 100 | 0 |
| 27 (Note 6) | 20MHz |  |  | 16QAM (Note 5) | 100 (Note 4) | 0 |
| 28 (Note 6) | 20MHz |  |  | QPSK | 50 | 25 |
| 29 (Note 6) | 20MHz |  |  | 16QAM (Note 5) | 50 (Note 4) | 25 |
| 30 (Note 6) | 20MHz |  |  | QPSK | 75 | 25 |
| 31 (Note 6) | 20MHz |  |  | QPSK | 25 | 75 |
| 32 (Note 6) | 20MHz |  |  | QPSK | 1 | 99 |
| 33 (Note 8) | 5MHz |  |  | QPSK | 1 | 0 |
| 34 (Note 8) | 5MHz |  |  | QPSK | 16 | 9 |
| 35 (Note 9) | 5MHz |  |  | QPSK | 25 | 0 |
| 36 (Note 8) | 5MHz |  |  | 16QAM (Note 5) | 16 | 9 |
| 37 (Note 9) | 5MHz |  |  | 16QAM (Note 5) | 25 | 0 |
| 38 (Note 8) | 10MHz |  |  | QPSK | 1 | 0 |
| 39 (Note 8) | 10MHz |  |  | QPSK | 1 | 49 |
| 40 (Note 8) | 10MHz |  |  | QPSK | 24 | 0 |
| 41 (Note 8) | 10MHz |  |  | QPSK | 40 | 10 |
| 42 (Note 7) | 10MHz |  |  | QPSK | 50 | 0 |
| 43 (Note 7) | 10MHz |  |  | 16QAM (Note 5) | 50 (Note 4) | 0 |
| 44 (Note 8) | 15MHz |  |  | QPSK | 1 | 0 |
| 45 (Note 8) | 15MHz |  |  | QPSK | 1 | 74 |
| 46 (Note 8) | 15MHz |  |  | QPSK | 32 | 0 |
| 47 (Note 8) | 15MHz |  |  | QPSK | 60 | 15 |
| 48 (Note 7) | 15MHz |  |  | QPSK | 75 | 0 |
| 49 (Note 7) | 15MHz |  |  | 16QAM (Note 5) | 75 (Note 4) | 0 |
| 50 (Note 8) | 20MHz |  |  | QPSK | 1 | 0 |
| 51 (Note 8) | 20MHz |  |  | QPSK | 1 | 99 |
| 52 (Note 8) | 20MHz |  |  | QPSK | 36 | 0 |
| 53 (Note 8) | 20MHz |  |  | QPSK | 75 | 25 |
| 54 (Note 7) | 20MHz |  |  | QPSK | 100 | 0 |
| 55 (Note 7) | 20MHz |  |  | 16QAM (Note 5) | 100 (Note 4) | 0 |
| Note 1: Test Channel Bandwidths are checked separately for E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The configuration ID will be used to map the applicable Test Configuration to be corresponding Test Requirement in subclause 6.2.4 as not all combinations are necessarily required based on the applicability of the UE. Note 3: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 4: Applies only for UE-Categories ≥2. Note 5: The 16QAM configurations shall not apply to UEs which support UL 64QAM. Note 6: Applies only for Rel-11 and earlier. Note 7: Applies only for Rel-12 and later. Note 8: Applies only for Rel-12 and later and Low Range. Note 9: Applies only for Rel-12 and later and Mid, High Range and additional test frequencies. |  |  |  |  |  |  |

< Unchanged sections omitted >

6.6.3.3_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 6.6.3.3_1.4.1-1 through Table 6.6.3.3_1.4.1-21. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.3.3_1.4.1-1: Test Configuration Table (network signalled value "NS_04")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low Range, Mid Range, High Range Additional test frequencies for Rel-12 to Rel-13 and bit 2 of modifiedMPRbehavior = 0: For 5 MHz bandwidth: 2499.6 MHz, NUL= 39686 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 Additional test frequencies for Rel-12 and later and bit 2 of modifiedMPRbehavior = 1: For 5 MHz bandwidth: 2500.6 MHz, NUL= 39696 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |
| Test Parameters for NS_04 A-MPR |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 (Note 1) | 5MHz | N/A for A-MPR testing |  | 64QAM | 16 | 9 |
| 2 (Note 2) | 5MHz |  |  | 64QAM | 25 | 0 |
| 3 | 10MHz |  |  | 64QAM | 50 | 0 |
| 4 | 15MHz |  |  | 64QAM | 75 | 0 |
| 5 | 20MHz |  |  | 64QAM | 100 | 0 |
| Note 1: Only applicable for Low Range. Note 2: Only applicable for Mid, High Range and additional test frequencies. Note 3: The 64QAM configurations shall not apply to UEs for which TC 6.6.3.3_2 is executed with network signalled value NS_04. |  |  |  |  |  |  |

< Unchanged sections omitted >

6.6.3.3_2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 6.6.3.3_2.4.1-1 through Table 6.6.3.3_2.4.1-22. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.3.3_2.4.1-1: Test Configuration Table (network signalled value "NS_04")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low Range, Mid Range, High Range Additional test frequencies for Rel-12 to Rel-13 and bit 2 of modifiedMPRbehavior = 0: For 5 MHz bandwidth: 2499.6 MHz, NUL= 39686 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 Additional test frequencies for Rel-12 and later and bit 2 of modifiedMPRbehavior = 1: For 5 MHz bandwidth: 2500.6 MHz, NUL= 39696 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 (Note 1) | 5MHz | N/A for Additional Spurious Emissions testing |  | 256QAM | 16 | 9 |
| 2 (Note 2) | 5MHz |  |  | 256QAM | 25 | 0 |
| 3 | 10MHz |  |  | 256QAM | 50 | 0 |
| 4 | 15MHz |  |  | 256QAM | 75 | 0 |
| 5 | 20MHz |  |  | 256QAM | 100 | 0 |
| Note 1: Only applicable for Low Range. Note 2: Only applicable for Mid, High Range and additional test frequencies. |  |  |  |  |  |  |

< Unchanged sections omitted >

6.6.3B.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Tables 6.6.3B.3.4.1-1 to 6.6.3B.3.4.1-19. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.3B.3.4.1-1: Test Configuration Table (network signalled value "NS_04") (Power Class 3)

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | NC |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range Additional test frequencies for Rel-12 to Rel-13 and bit 2 of modifiedMPRbehavior = 0: For 5 MHz bandwidth: 2499.6 MHz, NUL= 39686 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 Additional test frequencies for Rel-12 and later and bit 2 of modifiedMPRbehavior = 1: For 5 MHz bandwidth: 2500.6 MHz, NUL= 39696 For 10 MHz bandwidth: 2504.1 MHz, NUL= 39731 For 15 MHz bandwidth: 2510.9 MHz, NUL= 39799 For 20 MHz bandwidth: 2517.6 MHz, NUL= 39866 |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10 MHz, 15 MHz, 20MHz |  |  |
| Test Parameters for NS_04 A-MPR |  |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configura-tion ID | Ch BW | Mod'n | RB allocation TDD | Mod'n | RB allocation TDD | RBstart TDD |
| 1 (Note 6) | 5MHz | N/A for A-MPR testing |  | QPSK | 25 | 0 |
| 2 (Note 6) | 5MHz |  |  | QPSK | 8 | Note 3 |
| 3(Note 6) | 5MHz |  |  | QPSK | 6 | Note 3 |
| 4 (Note 6) | 5MHz |  |  | 16QAM (Note 5) | 25 | 0 |
| 5 (Note 6) | 5MHz |  |  | 16QAM (Note 5) | 8 | Note 3 |
| 6 (Note 6) | 10MHz |  |  | QPSK | 1 | 0 |
| 7 (Note 6) | 10MHz |  |  | QPSK | 12 | 0 |
| 8 (Note 6) | 10MHz |  |  | QPSK | 50 | 0 |
| 9 Note 6) | 10MHz |  |  | 16QAM (Note 5) | 50 (Note 4) | 0 |
| 10 (Note 6) | 10MHz |  |  | QPSK | 24 | 13 |
| 11 (Note 6) | 10MHz |  |  | 16QAM (Note 5) | 24 | 13 |
| 12 (Note 6) | 10MHz |  |  | QPSK | 36 | 13 |
| 13 (Note 6) | 10MHz |  |  | QPSK | 12 | 37 |
| 14 (Note 6) | 10MHz |  |  | QPSK | 1 | 49 |
| 15 (Note 6) | 15MHz |  |  | QPSK | 1 | 0 |
| 16 (Note 6) | 15MHz |  |  | QPSK | 16 | 0 |
| 17 (Note 6) | 15MHz |  |  | QPSK | 75 | 0 |
| 18 (Note 6) | 15MHz |  |  | 16QAM (Note 5) | 75 (Note 4) | 0 |
| 19 (Note 6) | 15MHz |  |  | QPSK | 36 | 19 |
| 20 (Note 6) | 15MHz |  |  | 16QAM (Note 5) | 36 (Note 4) | 19 |
| 21 (Note 6) | 15MHz |  |  | QPSK | 50 | 19 |
| 22 (Note 6) | 15MHz |  |  | QPSK | 18 | 56 |
| 23 (Note 6) | 15MHz |  |  | QPSK | 1 | 74 |
| 24 (Note 6) | 20MHz |  |  | QPSK | 1 | 0 |
| 25 (Note 6) | 20MHz |  |  | QPSK | 18 | 0 |
| 26 (Note 6) | 20MHz |  |  | QPSK | 100 | 0 |
| 27 (Note 6) | 20MHz |  |  | 16QAM (Note 5) | 100 (Note 4) | 0 |
| 28 (Note 6) | 20MHz |  |  | QPSK | 50 | 25 |
| 29 (Note 6) | 20MHz |  |  | 16QAM (Note 5) | 50 (Note 4) | 25 |
| 30 (Note 6) | 20MHz |  |  | QPSK | 75 | 25 |
| 31 (Note 6) | 20MHz |  |  | QPSK | 25 | 75 |
| 32 (Note 6) | 20MHz |  |  | QPSK | 1 | 99 |
| 33 (Note 8) | 5MHz |  |  | QPSK | 1 | 0 |
| 34 (Note 8) | 5MHz |  |  | QPSK | 16 | 9 |
| 35 (Note 9) | 5MHz |  |  | QPSK | 25 | 0 |
| 36 (Note 8) | 5MHz |  |  | 16QAM (Note 5) | 16 | 9 |
| 37 (Note 9) | 5MHz |  |  | 16QAM (Note 5) | 25 | 0 |
| 38 (Note 8) | 10MHz |  |  | QPSK | 1 | 0 |
| 39 (Note 8) | 10MHz |  |  | QPSK | 1 | 49 |
| 40 (Note 8) | 10MHz |  |  | QPSK | 24 | 0 |
| 41 (Note 8) | 10MHz |  |  | QPSK | 40 | 10 |
| 42 (Note 7) | 10MHz |  |  | QPSK | 50 | 0 |
| 43 (Note 7) | 10MHz |  |  | 16QAM (Note 5) | 50 | 0 |
| 44 (Note 8) | 15MHz |  |  | QPSK | 1 | 0 |
| 45 (Note 8) | 15MHz |  |  | QPSK | 1 | 74 |
| 46 (Note 8) | 15MHz |  |  | QPSK | 32 | 0 |
| 47 (Note 8) | 15MHz |  |  | QPSK | 60 | 15 |
| 48 (Note 7) | 15MHz |  |  | QPSK | 75 | 0 |
| 49 (Note 7) | 15MHz |  |  | 16QAM (Note 5) | 75 | 0 |
| 50 (Note 8) | 20MHz |  |  | QPSK | 1 | 0 |
| 51 (Note 8) | 20MHz |  |  | QPSK | 1 | 99 |
| 52 (Note 8) | 20MHz |  |  | QPSK | 36 | 0 |
| 53 (Note 8) | 20MHz |  |  | QPSK | 75 | 25 |
| 54 (Note 7) | 20MHz |  |  | QPSK | 100 | 0 |
| 55 (Note 7) | 20MHz |  |  | 16QAM (Note 5) | 100 | 0 |
| Note 1: Test Channel Bandwidths are checked separately for E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The configuration ID will be used to map the applicable Test Configuration to be corresponding Test Requirement in subclause 6.2.4 as not all combinations are necessarily required based on the applicability of the UE. Note 3: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 4: Applies only for UE-Categories ≥2. Note 5: Void. Note 6: Applies only for Rel-11 and earlier. Note 7: Applies only for Rel-12 and later. Note 8: Applies only for Rel-12 and later and Low Range. Note 9: Applies only for Rel-12 and later and Mid, High Range and additional test frequencies. |  |  |  |  |  |  |

< End of changes >