3GPP TSG-RAN5 Meeting #97	R5-227304r1

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.509 | CR | 0076 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding new test messages for RLM/BFD measurement relaxation testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | UE power saving test cases for RLM/BFD measurement relaxation need to be added according to the WP. However these test cases can not be executed without a reliable way to trigger deterministic UE behaviour. Both TS 38.331 and TS 38.133 leaves it to the UE implementation when and whether to apply or not RLM/BFD relaxation once the criteria(s) are met. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding test function to trigger deterministic UE behaviour. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RLM/BFD measurement relaxation can not be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X[New], 6.X[New] |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-227304->R5-227304r1 -corrected typo |  |  |  |  |  |  |  |  |

## 5.X	Measurement relaxation test function

### 5.X.1	General

The SS uses the measurement relaxation test function to trigger a change of relaxation state for RLM or BFD measurements - if this is allowed as per related core requirements - in the UE that will cause the indication of UE Assistance Information (UAI) by the UE, see figure 5.X.1-1.

Figure 5.X.1-1: Measurement relaxation test function

### 5.X.2	Initiation

The SS requests the UE to change its relaxation state for RLM or BFD measurements - if this is allowed as per related core requirements - by transmitting ACTIVATE MEASUREMENT RELAXATION REQUEST message and the UE confirms by responding with a ACTIVATE MEASUREMENT RELAXATION RESPONSE message.

### 5.X.3	Reception of ACTIVATE MEASUREMENT RELAXATION REQUEST message by UE

When UE receives ACTIVATE MEASUREMENT RELAXATION REQUEST message then the UE shall:

1> if the UE is operating in RRC_CONNECTED state:

2>	if Requested Relaxation State = RLM relaxation

3>	apply RLM measurement relaxation - if this is allowed as per related core requirements

2>	if Requested Relaxation State = BFD relaxation

3>	apply BFD measurement relaxation - if this is allowed as per related core requirements

2>	transmit ACTIVATE MEASUREMENT RELAXATION RESPONSE message.

1>	else:

2>	the UE behaviour is unspecified.

## [SECTION SKIPPED]

## 6.X	ACTIVATE MEASUREMENT RELAXATION REQUEST

### 6.X.1	ACTIVATE MEASUREMENT RELAXATION REQUEST

This message is only sent in the direction SS to UE.

| Information Element | Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Protocol discriminator | TS 24.007 [5], sub clause 11.2.3.1.1 | M | V | ½ |
| Skip indicator | TS 24.007 [5], sub clause 11.2.3.1.2 | M | V | ½ |
| Message type |  | M | V | 1 |
| Requested Relaxation State |  | O | TV | 1 |

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | octet 1 |

where Requested Relaxation State is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  |  |  |  |  |  | E0 | octet 2 |

E0=0 : RLM relaxation.

E0=1 : BFD relaxation.

### 6.X.2	ACTIVATE MEASUREMENT RELAXATION RESPONSE

This message is only sent in the direction UE to SS.

| Information Element | Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Protocol discriminator | TS 24.007 [5], sub clause 11.2.3.1.1 | M | V | ½ |
| Skip indicator | TS 24.007 [5], sub clause 11.2.3.1.2 | M | V | ½ |
| Message type |  | M | V | 1 |

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | octet 1 |