3GPP TSG-RAN5 Meeting #97	R5-227307r1

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2679 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test procedure for registration of a MUSIM UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_MUSIM_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Most of the test cases in MUSIM WP needs UE to perform registration on both the SIM cards and subsequent test procedure starts. Currently TTCN may not be able to handle simulataneous registrations from both the SIM cards, so there is a need to come up with deterministic way for registration with both the SIMs in a MUSIM UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test procedure for registration of a MUSIM UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Initial registration procedure needs to be unnecessarily repeated for MUSIM test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact Secretary to resolve SIM configurations xx and yy in 4.9.36.2.1 from TS 38.508-1 CR 2678. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-227307r1: 1. Hardcoded generic procedure from 4.9.zz to 4.9.36 |  |  |  |  |  |  |  |  |

<Start of modified section>

### 4.9.36	Test procedure for registration of a MUSIM UE

4.9.36.1	Scope

The purpose of this procedure is to perform initial registration of a MUSIM UE.

4.9.36.2	Procedure description

4.9.36.2.1	Initial conditions

System Simulator:

-	NGC Cell A (home PLMN1) and cell G (home PLMN2) are configured according to table 6.3.2.2-1 and table 6.3.2.2-3. Any change in the cells and their configurations shall be explicitly specified in the TC which calls the procedure in its entirety or refers to parts of it

User Equipment:

-	The UE is a MUSIM UE.

-	The UE is equipped with two USIMs with configuration as defined in Tables 6.4.1-xx and 6.4.1-yy.

4.9.36.2.2	Procedure sequence

Table 4.9.36.2.2-1: Test procedure sequence for registration of a MUSIM UE

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures: - NGC Cell A as the "Serving cell". - NGC Cell G as a "Non-Suitable Off cell". | - | - | - | - |
| 2 | UE is switched on. | - | - | - | - |
| - | The following messages are to be observed on NGC Cell A unless explicitly stated otherwise. | - | - | - | - |
| 3 | Check: Does USIM configuration 6.4.1-xx of the UE transmit REGISTRATION REQUEST message including UE network capability with at least one MUSIM feature is set to “support”? | --> | REGISTRATION REQUEST | - | P |
| 4-19 | Steps 5 to 20a1 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |
| 20 | The SS configures: - NGC Cell G as a "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on NGC Cell G unless explicitly stated otherwise. | - | - | - | - |
| 21 | Check: Does USIM configuration 6.4.1-yy of the UE transmit REGISTRATION REQUEST message including UE network capability with at least one MUSIM feature is set to “support”? | --> | REGISTRATION REQUEST | - | P |
| 22-37 | Steps 5 to 20a1 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed. | - | - | - | - |

4.9.36.3	Specific message contents

Table 4.9.36.3-1: Message REGISTRATION REQUEST (steps 3 and 21, Table 4.9.36.2.2-1)

| Derivation path: Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| 5GMM capability |  |  | pc_EPC_MUSIM |
| All octets with the exception of octet 6, bit 5 to bit 8 | Any allowed value |  |  |
| N1 NAS signalling connection release (NCR) (octet 6, bit 5) | ‘1’B | N1 NAS signalling connection release supported | pc_5GC_MUSIM_NCR |
| Paging indication for voice services (PIV) (octet 6, bit 6) | ‘1’B | paging indication for voice services is supported | pc_5GC_MUSIM_PIV |
| Reject paging request (RPR) (octet 6, bit 7) | ‘1’B | reject paging request supported | pc_5GC_MUSIM_RPR |
| Paging restriction (PR) (octet 6, bit 8) | ‘1’B | paging restriction is supported | pc_5GC_MUSIM_PR |

Table 4.9.36.3-2: Message REGISTRATION ACCEPT (steps 13 and 31, Table 4.9.36.2.2-1)

| Derivation path: Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| 5GMM capability |  |  | pc_EPC_MUSIM |
| All octets with the exception of octet 5, bit 4 to bit 7 | Any allowed value |  |  |
| N1 NAS signalling connection release (NCR) (octet 5, bit 4) | ‘1’B | N1 NAS signalling connection release supported | pc_5GC_MUSIM_NCR |
| Paging indication for voice services (PIV) (octet 5, bit 5) | ‘1’B | paging indication for voice services is supported | pc_5GC_MUSIM_PIV |
| Reject paging request (RPR) (octet 5, bit 6) | ‘1’B | reject paging request supported | pc_5GC_MUSIM_RPR |
| Paging restriction (PR) (octet 5, bit 7) | ‘1’B | paging restriction is supported | pc_5GC_MUSIM_PR |

<End of modifed section>