3GPP TSG-WG5 Meeting #97	R5-227317r1

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0850 | rev | - | Current version: | 17.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on EIS spherical coverage for Power Classes 1, 2,3 and 4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Hungary kft. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Update on NR channels for EIS spherical coverage on power classes 1, 2, 3 and 4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Channels, n262 and n263 were added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of requirements harmnizarion between RAN4 and RAN5 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | NR band n259 was removed to avoid overlaping with R5-225978 and R5-225979 |  |  |  |  |  |  |  |  |

## <Start of Change>

7.3.4.3	Minimum conformance requirements

The reference sensitivity power level REFSENS at a single grid point of the spherical grid is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

The reference measurement channels and throughput criterion shall be as specified in section 7.3.2.3.

For power class 1, the maximum EIS at the 85th percentile of the CCDF of EIS measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 7.3.4.3-1 below. The requirement is verified with the test metric of EIS (Link=Spherical coverage grid, Meas=Link angle).

Table 7.3.4.3-1: EIS spherical coverage for power class 1

| Operating band | EIS at 85th%ile CCDF (dBm) / Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| n257 | -89.5 | -86.5 | -83.5 | -80.5 | N/A | N/A | N/A |
| n258 | -89.5 | -86.5 | -83.5 | -80.5 | N/A | N/A | N/A |
| n260 | -86.5 | -83.5 | -80.5 | -77.5 | N/A | N/A | N/A |
| n261 | -89.5 | -86.5 | -83.5 | -80.5 | N/A | N/A | N/A |
| n262 | -84.3 | -81.3 | -78.3 | -75.3 | N/A | N/A | N/A |
| n263 | N/A | -73.5 | N/A | -67.5 | -64.5 | -61.5 | -60.5 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4. NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. |  |  |  |  |  |  |  |

For power class 2, the maximum EIS at the 60th percentile of the CCDF of EIS measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 7.3.4.3-2 below. The requirement is verified with the test metric of EIS (Link=Spherical coverage grid, Meas=Link angle).

Table 7.3.4.3-2: EIS spherical coverage for power class 2

| Operating band | EIS at 60th%ile CCDF (dBm) / Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| n257 | -81 | -78 | -75 | -72 | N.A | N.A | N.A |
| n258 | -81 | -78 | -75 | -72 | N.A | N.A | N.A |
| n261 | -81 | -78 | -75 | -72 | N.A | N.A | N.A |
| n262 | -74.9 | -71.9 | -68.9 | -65.9 | N.A | N.A | N.A |
| n263 | N.A | -71.2 | N.A | -65.2 | -62.2 | -59.2 | -58.2 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4. NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. |  |  |  |  |  |  |  |

For power class 3, the maximum EIS at the 50th percentile of the CCDF of EIS measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 7.3.4.3-3 below. The requirement is verified with the test metric of EIS (Link=Spherical coverage grid, Meas=Link angle).

For power class 3, the UEs that support operation in multiple FR2 bands, the minimum requirement for EIS spherical coverage in Table 7.3.4.3-3 shall be increased per band, respectively, by the reference sensitivity relaxation parameter ∑MBS and ∆MBS,n as specified in Table 7.3.2.3.3-1a and 7.3.2.3.3-1b..

Table 7.3.4.3-3: EIS spherical coverage for power class 3

| Operating band | EIS at 50th%ile CCDF (dBm) / Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| n257 | -77.4 | -74.4 | -71.4 | -68.4 | N.A | N.A | N.A |
| n258 | -77.4 | -74.4 | -71.4 | -68.4 | N.A | N.A | N.A |
| n259 | -71.9 | -68.9 | -65.9 | -62.9 | N.A | N.A | N.A |
| n260 | -73.1 | -70.1 | -67.1 | -64.1 | N.A | N.A | N.A |
| n261 | -77.4 | -74.4 | -71.4 | -68.4 | N.A | N.A | N.A |
| n262 | -69.7 | -66.7 | -63.7 | -60.7 | N.A | N.A | N.A |
| n263 | N.A | -66.2 | N.A | -60.2 | -57.2 | -54.2 | -53.2 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. |  |  |  |  |  |  |  |

For power class 4, the maximum EIS at the 20th percentile of the CCDF of EIS measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 7.3.4.3-4 below. The requirement is verified with the test metric of EIS (Link=Spherical coverage grid, Meas=Link angle).

Table 7.3.4.3-4: EIS spherical coverage for power class 4

| Operating band | EIS at 20th%ile CCDF (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -88.0 | -85.0 | -82.0 | -79.0 |
| n258 | -88.0 | -85.0 | -82.0 | -79.0 |
| n260 | -83.0 | -80.0 | -77.0 | -74.0 |
| n261 | -88.0 | -85.0 | -82.0 | -79.0 |
| n262 | -78.9 | -75.9 | -72.9 | -69.9 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. |  |  |  |  |

The REFSENS requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in Table 7.3.4.3-5.

Table 7.3.4.3-5: Uplink configuration for reference sensitivity

| NR Band / Channel bandwidth / NRB / SCS / Duplex mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NR Band | 50 MHz | 100 MHz | 200 MHz | 400 MHz | SCS | Duplex Mode |
| n257 | 32 | 64 | 128 | 256 | 120 kHz | TDD |
| n258 | 32 | 64 | 128 | 256 | 120 kHz | TDD |
| n260 | 32 | 64 | 128 | 256 | 120 kHz | TDD |
| n261 | 32 | 64 | 128 | 256 | 120 kHz | TDD |

Unless given by Table 7.3.4.3-6, the minimum requirements specified in Table 7.3.4.3-1, Table 7.3.4.3-2, Table 7.3.4.3-3 and Table 7.3.4.3-4 shall be verified with the network signalling value NS_200 configured.

Table 7.3.4.3-6: Network Signalling value for reference sensitivity

| NR Band | Network Signalling value |
| --- | --- |
| n258 | NS_201 |

For the UE which supports inter-band carrier aggregation, the minimum requirement for reference sensitivity in Table 7.3.4.3-1, Table 7.3.4.3-2, Table 7.3.4.3-3 and Table 7.3.4.3-4 shall be increased by the amount given in ΔRIB,P,n defined in subclause 7.3A.2.0.3 for the applicable operating bands.

The normative reference for this requirement is TS 38.101-2 [3] clause 7.3.4.

## <End of Change>