3GPP TSG-RAN5 Meeting #97	R5-227983r2

# Toulouse, France, 14th – 18th Nov 2022

Title:							MU discussion on FR2 PC1

Source:			Anritsu, NTT DOCOMO INC.

Agenda Item:			5.4.17

Document for:			Endorsement

# 1.	Introduction

In RAN5#95, the time plan for FR2 PC1 MU work was created for Japanese regulation [1]. In RAN5#96, the time plan was updated [2], and MU and relaxation value were defined with [ ] for MOP (EIRP, TRP), REFSENS, OFF power, Frequency error, In-band blocking, ACS [3][4][5]. According to the time plan, this paper provides MU discussion on FR2 PC1 for remaining P1 test cases. Unless otherwise commented, observations and proposals in this paper is for n257, CBW  400 MHz, IFF, max device size  30 cm.

- Priority 1 (P1) test cases

  - MOP (EIRP, TRP), REFSENS, OFF power, Frequency error, OBW, ACLR, SEM, General Spurious, Spurious co-existence, Inband blocking, ACS, Rx Spurious

- Priority 2 (P2) test cases

  - All other test cases

Proposal 1	: Update following time plan for FR2 Power Class 1 work.

| Meeting | RAN5 time plan |
| --- | --- |
| RAN5#96 (Aug-2022) | To define the upper limits of MU and Relaxation for MOP(EIRP, TRP) and REFSENS. To discuss the upper limits of MU and Relaxation for other P1 test cases based on [2]. |
| RAN5#97 (Nov-2022) | To define the upper limits of MU and Relaxation for other P1 test cases. |
| RAN5#98 (Feb-2023) | To define the upper limits of MU and Relaxation for P2 test cases. |

Following shows the current agreement of MU and relaxation for P1 test cases.

Table 1 Summary of current agreement of MU and relaxation for P1 test cases

| Test case | Relaxation [dB] | MTSU [dB] |
| --- | --- | --- |
| MOP (EIRP) | 0 | [5.33] |
| MOP (TRP) | 0 | [4.64] |
| REFSENS | 0 | [5.58] |
| OFF power | Same as PC3 | [5.67] |
| Frequency error | 0 | 0.01 ppm |
| OBW | TBD | TBD |
| ACLR | TBD | TBD |
| SEM | TBD | TBD |
| General Tx Spurious | TBD | TBD |
| Spurious co-existence | TBD | TBD |
| In-band blocking | 0 | [8.31] |
| ACS (case 1) | 0 | [8.31] |
| ACS (case 2) | Not testable | Not testable |
| Rx Spurious | TBD | TBD |

# 2.	Discussion

## 2.1.	Testability and Relaxation

Based on the difference between the requirements of PC1 and PC3, testability and the relaxation value required for each remaining P1 test case were analyzed as follows.

OBW: According to 6.5.1 [6], the requirement of OBW does not depend on power class. As same as PC3, no relaxation is required for PC1.

ACLR: According to 6.5.2.3 [6], the requirement of  ACLR does not depend on power class, but PC1 has absolute level different from PC3 due to the difference of MOP and MPR. As shown in the calculation for influence of noise (ACLR) in section 2.2, SNR is 9.2 dB in the worst case. Therefore, no testability issue exists and no relaxation is required for PC1.

SEM: According to 6.5.2.1 [6], the requirement of SEM does not depend on power class, but PC1 has Max EIRP higher than PC3. When measuring spurious emission on a grid with a carrier level above that assumed in PC3, there is a risk of degradation of measurement accuracy by amplifier compression due to the high input power to the TE with high sensitivity setting. If TE adjusts its attenuator to avoid amplifier compression, SNR will be -7.9 dB at the worst case assuming the carrier level of 55 dBm. PC1 has a testability issue (dynamic range).

General Tx Spurious: According to 6.5.3.1 [6], the requirement of General Tx Spurious does not depend on power class, but due to the same reason as SEM, SNR will be -5.0 dB at the worst case assuming the carrier level of 55 dBm. PC1 has a testability issue (dynamic range). Note that the testability issue does not exist in the frequency range 6 GHz to 12.75 GHz because it is away from the carrier frequency.

Spurious co-existence: According to 6.5.3.2 [6], the requirement of Spurious co-existence does not depend on power class, but due to the same reason as SEM and General Tx Spurious, SNR will be -9.0 dB at the worst case assuming the carrier level of 55 dBm and the same relaxation value as PC3. PC1 has a testability issue (dynamic range).

Rx Spurious: According to 7.9 [6], Rx Spurious does not depend on power class. PC1 has the same testability issue (low UL power) as PC3, and PC1 requires the same relaxation value as PC3. The concrete relaxation values are 10.2 dB for 6 GHz  f < 20 GHz, 17.2 dB for 20 GHz  f < 40 GHz, 33.1 dB for 40 GHz  f < second harmonic.

Observation 1: PC1 has no testability issue and requires no relaxation for OBW, ACLR, General Tx Spurious (6 GHz  f < 12.75 GHz).

Observation 2: PC1 has the same testability issue as PC3 and requires the same relaxation value as PC3 for Rx Spurious.

Observation 3: PC1 has testability issues for SEM, General Tx Spurious (12.75 GHz  f < 66 GHz), and Spurious co-existence.

The above observations are summarized as below.

Table 2 Summary of observation on testability for remaining P1 test cases

| Test case | Testability | Required relaxation |
| --- | --- | --- |
| OBW | No testability issue | - |
| ACLR | No testability issue | - |
| SEM | Testability issue due to TE dynamic range | Not allowed |
| General Tx Spurious |  |  |
| 6 GHz f < 12.75 GHz | No testability issue | - |
| 12.75 GHz f < 66 GHz | Testability issue due to TE dynamic range | Not allowed |
| Spurious co-existence | Testability issue due to TE dynamic range | Not allowed |
| Rx Spurious | Same testability issue as PC3 | Same relaxation value as PC3 |

Observation 3 shows that relaxation is required for SEM and Spurious. However, relaxation is not allowed due to regulatory requirements and it was endorsed to discuss ways to avoid additional relaxation for these test cases [2].

Proposal 2	: Discuss ways to avoid testability issues other than additional relaxation for SEM, General Spurious and Spurious co-existence.

In order to solve the testability issue for SEM and Spurious without additional relaxation, we focused on two facts:

- The above testability issue causes at high EIRP grids, where “high EIRP grid” is defined as a grid with EIRP larger than 43 dBm in this paper. However, it is impossible that all grids are high EIRP grid, and testability issue does not exist at other grids.

- SEM and Spurious are tested by TRP value which is the average of measurement results of all grids.

Based on the above two facts, we can estimate SEM and Spurious as the average of some high EIRP grids with testability issue and some other grids without testability issue. As a result, testability issue will be improved.

Here, antenna pattern assumption is an important foctor for testability, because the amount of testability improvement depends on the percentage of high EIRP grids. In [4], we assume that high EIRP grids are less than 16 % of all grids based on the core requirement of the maximum TRP = 35 dBm. Note that 16 % is the most conservative assumption that EIRP equals just 43 dBm at 16 % grids and EIRP is completely 0 at the other grids. A new action point AP#96.32 was added for more realistic antenna pattern [4].

Proposal 6b: Request an action point for UE vendors to study the maximum percentage of grids with EIRP > 43 dBm acceptable for antenna pattern assumptions in SEM, General Tx Spurious, and Spurious co-existence.

Based on the answer to AP#96.32 [7], we use 6 % for the maximum percentage of high EIRP grids in this paper.

Observation 4: In PC1, grids with EIRP > 43 dBm are less than 6 % of the all grids.

For example, in General Tx Spurious, high EIRP grids are 6 % of all grids and tested with SNR = -5 dB, and other 94 % grids are tested with SNR = 10 dB. Therefore, the total SNR will be 5.5 dB by calculating a weighted average of them. Similarly, the total SNR of SEM and Spurious co-existence will be 2.9 dB and 1.5 dB by assuming Observation 4.

Observation 5: Assuming Observation 4 in PC1, the total SNR of SEM, General Tx Spurious (12.75 GHz  f < 66 GHz), and Spurious co-existence will be 2.9 dB, 5.5 dB, and 1.5 dB, respectively.

Although the above total SNR is less than 6 dB, considering regulatory requirement, we propose to test SEM, General Tx Spurious, and Spurious co-existence with the above SNR value without additional relaxation.

Based on the above observations, following shows our proposals for PC1, n257, CBW  400 MHz, IFF, max device size  30 cm.

Proposal 1: For PC1 OBW, do not relax the test requirement.

Proposal 2: For PC1 ACLR, do not relax the test requirement.

Proposal 3: For PC1 SEM, do not relax the test requirement assuming Observation 4 and Observation 5.

Proposal 4: For PC1 General Tx Spurious, do not relax the test requirement assuming Observation 4 and Observation 5.

Proposal 5: For PC1 Spurious co-existence, adopt the same relaxation value as PC3 based on the facts of Observation 4 and Observation 5.

Proposal 6: For PC1 Rx Spurious, adopt the same relaxation value as PC3.

## 2.2.	MU element

The following table shows the current agreement on each MU element for PC1 P1 test cases: MOP (EIRP, TRP), REFSENS, OFF power, Frequency error, In-band blocking, ACS (case 1). Unless otherwise referenced, these agreements are clarified in [8]. Red texts show that further analysis and agreement are needed for the remaining P1 test cases.

Table 3 Agreement on each MU element for PC1 before RAN5#97

| MU element | Agreement |
| --- | --- |
| Positioning misalignment | 0.02 dB |
| Measure distance uncertainty | Same value as PC3 |
| Quality of Quiet Zone | Same value as PC3 |
| Mismatch | Same value as PC3 for Default, Rx Spurious Same value as PC3 with [ ] for ACLR TBD for SEM, Tx Spurious |
| Standing wave between DUT and measurement antenna | Same value as PC3 |
| Uncertainty of the RF power measurement equipment | Same value as PC3 with [ ] for MOP, MPR, ACLR Same value as PC3 for OFF power, Rx Spurious TBD for SEM, Tx Spurious |
| Phase Curvature | Same value as PC3 |
| Amplifier Uncertainties | Same value as PC3 with [ ] for Default TBD for SEM, Tx Spurious |
| Random uncertainty | Same value as PC3 |
| Influence of XPD | Same value as PC3 |
| Insertion Loss Variation | Same value as PC3 |
| RF leakage (from measurement antenna to receiver/transmitter) | Same value as PC3 |
| Misalignment of positioning system | Same value as PC3 |
| Uncertainty of the Network Analyzer | 1.50 dB for Default (6 GHz to 40.8 GHz) 1.70 dB for Spurious (40.8 GHz to 80 GHz) |
| Uncertainty of the absolute gain of the calibration antenna | Same value as PC3 |
| Positioning and pointing misalignment between the reference antenna and the measurement antenna | Same value as PC3 |
| gNB emulator uncertainty | Same value as PC3 |
| Phase centre offset of calibration | Same value as PC3 |
| Quality of the Quiet Zone for Calibration Process | Same value as PC3 |
| Standing wave between reference calibration antenna and measurement antenna | Same value as PC3 |
| Influence of the calibration antenna feed cable (Flexing cables, adapters, attenuators, connector repeatability) | Same value as PC3 |
| Influence of TRP measurement grid | Same value as PC3 for Default TBD for Spurious |
| Influence of beam peak search grid | Same value as PC3 |
| Systematic error due to TRP calculation/quadrature | Same value as PC3 |
| Multiple measurement antenna uncertainty | Same value as PC3 |
| DUT repositioning | Same value as PC3 for TRP 0.35 for EIRP, EIS |
| Influence of noise | [0.13] dB for MOP (EIRP, TRP) Same value as PC3 for OFF power (Proposal 7 of [4]) TBD for ACLR, SEM, Spurious |
| Systematic error related to beam peak search | 0.7 dB |
| Influence of spherical coverage grid | 0.13 dB |
| Systematic error related to EIS spherical coverage | Same value as PC3 |
| Misalignment of DUT due to change of DUT orientation | TBD |
| Additional Impact of Interferer ACLR | Same value as PC3 |
| Modulated Interferer uncertainty | Same value as PC3 |
| Uncertainty of the RF relative power measurement equipment | TBD (not for P1 test cases) |

MU values were analyzed as follows for MU elements without agreement.

Mismatch, Uncertainty of the RF power measurement equipment, Amplifier Uncertainties: Because the difference between PC1 and PC3 is no enough to change the value of these MU elements, the same value as PC3 can be used for PC1.

Influence of TRP measurement grid, Systematic error due to TRP calculation/quadrature (Spurious): In order to define MU value of Influence of TRP measurement grid and Systematic error due to TRP calculation/quadrature, the standard deviation and the mean error for each grid assumptions were simulated as below. In the simulation, we assumed 1 antenna array, 12x12 elements in an array, horizontal/vertical HPW = 90°/90°, and element gain = 5.0 dBi as same as [9]. Considering a trade-off between measurement time and uncertainty, we propose to define the number of grid points as 750 pt for constant density and keep grid points as the same number as PC3 for constant step size (7.5° step, 1106 pt), and we propose to adopt 1.1 dB for MU value of Influence of TRP measurement grid and 0.13 dB for MU value of Systematic error due to TRP calculation/quadrature.

Table 4 Standard deviation and mean error for constant density measurement grids for 12x12 antenna array

| Number of grid points | Standard deviation [dB] | Mean error [dB] |
| --- | --- | --- |
| 500 | 1.593 | 0.250 |
| 750 | 1.110 | 0.134 |
| 1000 | 0.760 | 0.081 |
| 1250 | 0.504 | 0.037 |
| 1500 | 0.302 | 0.004 |
| 2000 | 0.132 | 0.001 |

Table 5 Standard deviation and mean error for constant step size measurement grids for 12x12 antenna array

| Step size Δθ = Δφ | Number of latitudes | Number of longitudes | Number of grid points | Standard deviation [dB] | Mean error [dB] |
| --- | --- | --- | --- | --- | --- |
| 7.5° | 25 | 48 | 1106 | 1.049 | 0.100 |
| 6.0° | 31 | 60 | 1742 | 0.572 | 0.033 |
| 5.0° | 37 | 72 | 2522 | 0.220 | 0.008 |

Influence of noise (ACLR): Based on the minimum peak EIRP = 40.0 dBm, the maximum MPR = 14.4 dB, T(MPR) = 7 dB for 14.4 dB MPR, the core requirement for ACLR = 17 dB, the lower limits of carrier level and adjacent channel level in PC1 ACLR test case are 18.6 dBm and 1.6 dBm. Considering that TE noise floor is -7.6 dBm/400 MHz [10], SNR and ΔSNR are 9.2 dB and 0.50 dB in the worst case. Note that TP analysis is under discussion.

Influence of noise (SEM): Observation 5 shows that SNR = 2.9 dB and ΔSNR = 1.81 dB.

Influence of noise (General Tx Spurious): For the frequency range from 6 GHz to 12.75 GHz, the test level and SNR are the same as PC3, so the same value as PC3 can be used for PC1. For the frequency ranges from 12.75 GHz to 66 GHz, Observation 5 shows that SNR = 5.5 dB and ΔSNR = 1.08 dB.

Influence of noise (Spurious co-existence): Observation 5 shows that SNR = 1.5 dB and ΔSNR = 2.34 dB.

Influence of noise (Rx Spurious): The test level and SNR are the same as PC3, so the same value as PC3 can be used for PC1.

Misalignment of DUT due to change of DUT orientation (Spurious): The MU value for PC3 was defined by a result of numerical simulation [11]. Annex shows the results of a similar simulation for PC1 assuming constant density (750 pt) and constant step size (7.5° step, 1106 pt). The standard deviation is increased from 1.110 dB without rotation to 1.120 dB with +/- 3 degrees rotational error, so we propose to add 0.1 dB as Misalignment of DUT due to change of DUT orientation.

The following table shows the summary of MU analysis, replacing red texts in Table 3. In addition to the above analysis, following points were updated according to the outcome of MU discussion.

- Influence of noise:

The worst value among TEVs provided in [12], [13], and this paper was used for Influence of noise.

- Influence of TRP measurement grid (Spurious):

According to the Proposal 1 in [14], it was agreed to adopt 1600 grid points for constant density and 2522 unique grid points (5° step) for constant step size. As a result, the value of Influence of TRP measurement grid is 0.25 dB.

- Systematic error due to TRP calculation/quadrature (Spurious):

As a result of the above agreement on the number of TRP measurement grid, the value of Systematic error due to TRP calculation/quadrature is 0 dB.

Table 6 Summary of MU analysis and the current agreement for PC1

| MU element | Agreement |
| --- | --- |
| Positioning misalignment | 0.02 dB |
| Measure distance uncertainty | Same value as PC3 |
| Quality of Quiet Zone | Same value as PC3 |
| Mismatch | Same value as PC3 for Default, Rx Spurious Same value as PC3 with [ ] for ACLR Same value as PC3 for SEM, Tx Spurious |
| Standing wave between DUT and measurement antenna | Same value as PC3 |
| Uncertainty of the RF power measurement equipment | Same value as PC3 with [ ] for MOP, MPR, ACLR Same value as PC3 for OFF power, Rx Spurious Same value as PC3 for SEM, Tx Spurious |
| Phase Curvature | Same value as PC3 |
| Amplifier Uncertainties | Same value as PC3 with [ ] for Default Same value as PC3 for SEM, Tx Spurious |
| Random uncertainty | Same value as PC3 |
| Influence of XPD | Same value as PC3 |
| Insertion Loss Variation | Same value as PC3 |
| RF leakage (from measurement antenna to receiver/transmitter) | Same value as PC3 |
| Misalignment of positioning system | Same value as PC3 |
| Uncertainty of the Network Analyzer | 1.50 dB for Default (6 GHz to 40.8 GHz) 1.70 dB for Spurious (40.8 GHz to 80 GHz) |
| Uncertainty of the absolute gain of the calibration antenna | Same value as PC3 |
| Positioning and pointing misalignment between the reference antenna and the measurement antenna | Same value as PC3 |
| gNB emulator uncertainty | Same value as PC3 |
| Phase centre offset of calibration | Same value as PC3 |
| Quality of the Quiet Zone for Calibration Process | Same value as PC3 |
| Standing wave between reference calibration antenna and measurement antenna | Same value as PC3 |
| Influence of the calibration antenna feed cable (Flexing cables, adapters, attenuators, connector repeatability) | Same value as PC3 |
| Influence of TRP measurement grid | Same value as PC3 for Default 0.25 dB for Spurious |
| Influence of beam peak search grid | Same value as PC3 |
| Systematic error due to TRP calculation/quadrature | Same value as PC3 for Default 0 dB for Spurious |
| Multiple measurement antenna uncertainty | Same value as PC3 |
| DUT repositioning | Same value as PC3 for TRP 0.35 for EIRP, EIS |
| Influence of noise | [0.13] dB for MOP (EIRP, TRP) Same value as PC3 for OFF power (Proposal 7 of [4]) [0.95] dB for ACLR 1.81 dB for SEM Same value as PC3 for General Tx Spurious (6 – 12.75 GHz) 1.08 dB for General Tx Spurious (12.75 – 66 GHz) 2.34 dB for Spurious co-existence (23.6 – 24.0 GHz) Same value as PC3 for Rx Spurious |
| Systematic error related to beam peak search | 0.7 dB |
| Influence of spherical coverage grid | 0.13 dB |
| Systematic error related to EIS spherical coverage | Same value as PC3 |
| Misalignment of DUT due to change of DUT orientation | 0.1 dB |
| Additional Impact of Interferer ACLR | Same value as PC3 |
| Modulated Interferer uncertainty | Same value as PC3 |
| Uncertainty of the RF relative power measurement equipment | TBD (not for P1 test cases) |

Proposal 7: For PC1, n257, CBW  400 MHz, IFF, max device size  30 cm, adopt Table 6 for total MU calculation of ACLR, SEM, General Tx Spurious, Spurious co-existence, Rx Spurious.

## 2.3.	Total MU

Total MU in each test case is calculated as below by using the MU element values proposed in Table 6. The detail of calculation process other than OBW is shown in MU assessment sheets in [15] [16] [17]. OBW MU is based on our internal analysis.

Table 7 Summary of total MU in each test case

| Test case | Total MU for PC1 [dB] | Total MU for PC3 [dB] | Difference (PC1 – PC3) [dB] |
| --- | --- | --- | --- |
| OBW |  |  |  |
| BW = 50 MHz, 100 MHz | 0.4 % | 0.4 % | - |
| BW = 200 MHz, 400 MHz | 1.2% | 1.2% | - |
| ACLR |  |  |  |
| CBW = 50 MHz | [6.04] | 5.63 | 0.41 |
| 50 MHz < CBW 400 MHz | [6.04] | 6.09 | -0.05 |
| SEM | 6.32 | 4.94 | 1.38 |
| General Tx Spurious |  |  |  |
| 6 GHz f < 12.75 GHz | [5.28] | 5.14 | 0.14 |
| 12.75 GHz f < 23.45 GHz | [5.91] | 5.11 | 0.80 |
| 23.45 GHz f < 40.8 GHz | [6.07] | 5.41 | 0.66 |
| 40.8 GHz f < 66.0 GHz | [8.09] | 7.42 | 0.67 |
| Spurious co-existence |  |  |  |
| Protected frequency: 23.6 GHz f 24.0 GHz | [7.32] | 6.00 | 1.32 |
| Rx Spurious |  |  |  |
| 6 GHz f < 12.75 GHz | [5.63] | 5.50 | 0.13 |
| 12.75 GHz f < 23.45 GHz | [5.59] | 5.46 | 0.13 |
| 23.45 GHz f < 40.8 GHz | [6.10] | 6.11 | -0.01 |
| 40.8 GHz f < 66.0 GHz | [7.64] | 7.65 | -0.01 |

Proposal 8: For PC1, n257, CBW  400 MHz, IFF, max device size  30 cm, adopt Table 7 for the upper limit of total MU for ACLR, SEM, General Tx Spurious, Spurious co-existence, Rx Spurious.

Proposal 9: Remove [ ] from MTSU and each MU elements of PC1 MOP (EIRP, TRP), REFSENS, OFF power, In-band blocking, ACS (case 1).

# 3.	Conclusion

Following show the summary of observations.

Observation 1: PC1 has no testability issue and requires no relaxation for OBW, ACLR, General Tx Spurious (6 GHz  f < 12.75 GHz).

Observation 2: PC1 has the same testability issue as PC3 and requires the same relaxation value as PC3 for Rx Spurious.

Observation 3: PC1 has testability issues for SEM, General Tx Spurious (12.75 GHz  f < 66 GHz), and Spurious co-existence.

Observation 4: In PC1, grids with EIRP > 43 dBm are less than 6 % of the all grids.

Observation 5: Assuming Observation 4 in PC1, the total SNR of SEM, General Tx Spurious (12.75 GHz  f < 66 GHz), and Spurious co-existence will be 2.9 dB, 5.5 dB, and 1.5 dB, respectively.

RAN5 is asked to endorse following proposals.

For testability and relaxation:

Proposal 1: For PC1 OBW, do not relax the test requirement.

Proposal 2: For PC1 ACLR, do not relax the test requirement.

Proposal 3: For PC1 SEM, do not relax the test requirement assuming Observation 4 and Observation 5.

Proposal 4: For PC1 General Tx Spurious, do not relax the test requirement assuming Observation 4 and Observation 5.

Proposal 5: For PC1 Spurious co-existence, adopt the same relaxation value as PC3 based on the facts of Observation 4 and Observation 5.

Proposal 6: For PC1 Rx Spurious, adopt the same relaxation value as PC3.

For measurement uncertainty:

Proposal 7: For PC1, n257, CBW  400 MHz, IFF, max device size  30 cm, adopt Table 6 for total MU calculation of ACLR, SEM, General Tx Spurious, Spurious co-existence, Rx Spurious.

Proposal 8: For PC1, n257, CBW  400 MHz, IFF, max device size  30 cm, adopt Table 7 for the upper limit of total MU for ACLR, SEM, General Tx Spurious, Spurious co-existence, Rx Spurious.

Proposal 9: Remove [ ] from MTSU and each MU elements of PC1 MOP (EIRP, TRP), REFSENS, OFF power, In-band blocking, ACS (case 1).

# 4.	References

[1] R5-223621, “Handling of FR2 Power Class 1 in RAN5”, NTT DOCOMO, INC., RAN5#95-e

[2] R5-225641, “Handling of FR2 PC1 in RAN5”, NTT DOCOMO, INC., Anritsu, Keysight TechnologiesUK Ltd, ROHDE&SCHWARZ, RAN5#96-e

[3] R5-225221, “On PC1 measurement uncertainties for other test cases”, Keysight Technologies, RAN5#96-e

[4] R5-225639, “MU discussion on FR2 PC1”, Anritsu, NTT DOCOMO INC., RAN5#96-e

[5] R5-225643, “On the MU for FR2 PC1 TRx test cases”, Rohde & Schwarz, RAN5#96-e

[6] TS 38.521-2, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”, V17.0.0 (2022-09)

[7] R5-226243, “On AP96.32 Number of grid points exceeding 43dBm”, Keysight Technologies UK Ltd, RAN5#97

[8] TR 38.903, “Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases”, V16.13.0 (2022-09)

[9] R5-211189, “PC1 MUs based on the revised antenna array assumptions”, Keysight Technologies, RAN5#90-e

[10] R5-204741, “On FR2 EIRP OFF Power relaxation and measurement uncertainty”, Keysight Technologies, Samsung, RAN5#88-e

[11] R5-195915, “DUT turnover MU for spurious region”, Anritsu, RAN5#84

[12] R5-227984, “Discussion on FR2 PC1 MU”, Keysight Technologies, RAN5#97

[13] R5-227987, “On the MU for FR2 PC1 TRx test cases”, Rohde & Schwarz, RAN5#97

[14] R5-226244, “On TRP MU for Spurious Emissions”, Keysight Technologies, RAN5#97

[15] R5-227967, “PC1 MU - definition for ACLR in 38.903”, Keysight Technologies, RAN5#97

[16] R5-227972, “PC1 MU - definition for SEM in 38.903”, Keysight Technologies, RAN5#97

[17] R5-227989, “Definition of PC1 MU”, Anritsu, RAN5#97

# Annex Simulation results for Misalignment of DUT due to change of DUT orientation

Figure 1 TRP MU impact due to non-ideal DUT orientation (Constant density 750pt, xrot=yrot=zrot=A)

Figure 2 TRP MU impact due to non-ideal DUT orientation (Constant density 750pt, zrot=A, xrot=yrot=0)

Figure 3 TRP MU impact due to non-ideal DUT orientation (Constant step size 7.5 deg, xrot=yrot=zrot=A)

Figure 4 TRP MU impact due to non-ideal DUT orientation (Constant step size 7.5 deg, zrot=A, xrot=yrot=0)