3GPP TSG-RAN5 Meeting #98	R5-231933

# Athens, Greece, 27th February - 3rd March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4713 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NTN-IoT: Initial Test Model for NB-IoT NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Specification of an NB-IoT NTN Test Model is required to enable UE conformance testing for Rel-17 NB-IoT support for Non-Terrestrial Networks (NTN). |  |  |  |  |  |  |  |  |
| Summary of change: |  | An Initial Test Model for NB-IoT NTN has been introduced, together with the associated design considerations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NB-IoT NTN test cases cannot be implemented in TTCN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4A.3(new), 7A.14(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the outcome of 1 revision of R5-230105. |  |  |  |  |  |  |  |  |

### 4A.2.2	RRC / NAS test model

Figure 4A.2.2-1: Test model for NB-IoT RRC/NAS testing

The UE is configured in normal mode. On UE side NAS security (ciphering/integrity) is enabled and ROHC is optionally configured.  For UP, PDCP and AS security (ciphering/integrity) are enabled.

On the SS Side L1, MAC and RLC are configured in normal mode. They shall perform all of their functions. For SRB0 the DL and UL port is above RLC. For SRB1/SRB1bis the port is above/below the RRC and NAS emulator, which is implemented as a parallel test component. NAS security (integrity/ciphering) is enabled.  For UP, PDCP is configured in normal mode and the DRB port is above PDCP. AS security (ciphering/integrity) is enabled.

The NAS emulator for SRB1/SRB1bis shall provide the ciphering and integrity functionality for the NAS messages. In the UL direction, the SS shall report RRC messages, still containing (where appropriate) the secure and encoded NAS message, to the RRC port. In DL, RRC and NAS messages with same timing information shall be embedded in one PDU after integrity and ciphering for NAS messages.

The UL Scheduling Grant and DL Scheduling Assignments are configured from TTCN over the system control port.

## 4A.3 NTN test models

The NB-IoT test models specified in clause 4A.2 are re-used for NTN testing, with additional aspects specified hereafter.

The UE under test shall be pre-configured with a geographical position that will not change throughout the execution of the test case.

The system simulator (SS) shall implement one or several simulated NB-IoT NTN cell(s). An NB-IoT NTN cell is defined in clause 7A.14.

[End of change]

[Start of change]

## 7A.13	RRC Connection Suspend and Resume

General sequence of RRC Connection Suspend and Resume for NBIOT:

1.	RRC Connection Release Sequence according to clause 7A.7.

2.	Reestablishment of the AS security at the SS (for the cell at which subsequent RRC Connection Resume is expected).

3.	SS configuration with DL DCCH message to be contained in Msg4 of the RACH procedure according to clause 7A.8.2.

4.	Regular RACH procedure but with Msg4 containing pre-configured DL DCCH message:

-	The final encoding of DL DCCH message (AS security, PDCP header, RLC header) shall not be done before start of the RRC connection resume procedure.

## 7A.14 Non-Terrestrial Network (NTN)

## 7A.14.1 NTN cell configuration

An NB-IoT cell to be used in a Non-Terrestrial Network is an NB-IoT cell that is broadcasting a system block combination 8 or 9 as defined in clause 8.1.4.3.1.1 of TS 36.508 [3], i.e. it shall at least broadcast SystemInformationBlockType31-NB. In addition to that, the NB-IoT NTN cell shall be configured as "barred" in cellAccessRelatedInfo-r13 IE and as "notBarred" in cellAccessRelatedInfo-NTN-r17 IE of SystemInformationBlockType1-NB.

TTCN implementation is responsible for maintaining the system information up-to-date and to ensure that the updated system information is being broadcast in the NB-IoT NTN cell before the timer T317 expires at UE.

[End of change]