3GPP TSG-RAN5 Meeting #9	R5-232770

# Incheon, South Korea, 22nd – 26th May, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2784 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to default 5GMM messages |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Information Elements of the REGISTRATION REQUEST 5GMM message have been updated in the core specifications. The default message definitions have to be updated accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The definition of the REGISTRATION REQUEST 5GMM message has been updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will not be correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### –	Registration request

Table 4.7.1-6: REGISTRATION REQUEST

| Derivation Path: 24.501 clause 8.2.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Registration request message identity |  | ‘0100 0001’B |  |  |
| 5GS registration type |  |  |  |  |
| 5GS registration type value |  | ‘001’B | Initial registration |  |
|  |  | ‘010’B |  | MOBILITY |
|  |  | ‘011’B |  | PERIODIC |
|  |  | ‘100’B |  | EMERGENCY |
| FOR |  | Present but contents not checked |  |  |
| FOR |  | '1'B | Follow-on request pending | EMERGENCY |
| ngKSI |  | Present but contents not checked |  |  |
| 5GS mobile identity |  | Present but contents not checked |  |  |
| Non-current native NAS key set identifier |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| 5GMM capability |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| UE security capability |  | If present: contents not checked |  |  |
| Requested NSSAI |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Last visited registered TAI |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| S1 UE network capability |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Uplink data status |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| PDU session status |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| MICO indication |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| UE status |  | If present: contents not checked |  |  |
| Additional GUTI |  | If present: contents not checked |  |  |
| Allowed PDU session status |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| UE's usage setting |  | If present: contents not checked |  | NOT pc_IMS AND NON_CLEARTEXT_IE |
| UE's usage setting |  | Present but contents not checked |  | NON_CLEARTEXT_IE |
| Requested DRX parameters |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| EPS NAS message container |  | If present: contents not checked |  |  |
| LADN indication |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Payload container type |  | If present: ‘0101’B | UE policy container | NON_CLEARTEXT_IE |
| Payload container |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Network slicing indication |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| 5GS update type |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Mobile station classmark 2 |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Supported codecs |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| NAS message container |  | The complete, ciphered, REGISTRATION REQUEST message including all IEs. |  | CIPHERED_MESSAGE |
| EPS bearer context status |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Requested extended DRX parameters |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| T3324 value |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| UE radio capability ID |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Requested mapped NSSAI |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Additional information requested |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Requested WUS assistance information |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| N5GC indication |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Requested NB-N1 mode DRX parameters |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| UE request type |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Paging restriction |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Service-level-AA container |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| NID |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| MS determined PLMN with disaster condition |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Requested PEIPS assistance information |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Requested T3512 value |  | If present: contents not checked |  | NON_CLEARTEXT_IE |

| Condition | Explanation |
| --- | --- |
| INITIAL | Initial registration |
| MOBILITY | Mobility registration updating |
| PERIODIC | Periodic registration updating |
| EMERGENCY | Emergency registration |
| NON_CLEARTEXT_IE | An information element that is not allowed to be sent in cleartext and shall only be included in the complete REGISTRATION REQUEST message in the NAS message container IE. |
| CIPHERED_MESSAGE | If any of the IEs marked with the condition NON_CLEARTEXT_IE is present, and the UE has a valid 5G NAS security context, this condition applies. |

NOTE:	This message is sent interity protected when a valid security context exists otherwise sent without integrity protection, including only cleartext IEs.