3GPP TSG-RAN5 Meeting #99	R5-233476

# Incheon, South Korea, 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-2 | CR | 0353 | rev | 1 | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of applicability of new Idle mode TC to test the intraFreqReselection in MIB message is set to not allowed |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm CDMA Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add applicability clauses for new Idle mode TC 6.1.2.3a Intra frequency reselection not allowed |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding applicability for new Idle mode TC 6.1.2.3a Intra frequency reselection not allowed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Idle mode TC for Intra frequency reselection not allowed will have no applicability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.3a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-233476 is the final version of R5-232689 with below change: 1. Removed 5GS WIC from cover page. |  |  |  |  |  |  |  |  |

## <<Star of changes>>

## 4.1	Protocol conformance test cases applicability

Table 4.1-1a: Applicability of Protocol conformance Idle mode test cases, ref. TS 38.523-1 [2]

| Clause | TC Title | Release | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |
| 6 | Idle mode operations |  |  |  |
| 6.1 | NR idle mode operations |  |  |  |
| 6.1.1 | NG-RAN Only PLMN Selection |  |  |  |
| 6.1.1.1 | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.1.2 | PLMN selection of "Other PLMN/access technology combinations" / Automatic mode | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.1.3 | Cell reselection of ePLMN in manual mode | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.1.4 | PLMN selection in shared network environment / Automatic mode | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.1.4a | PLMN selection in shared network environment / Automatic mode / Cells broadcasting multiple PLMN IDs with unique TAC's, RAN areas, and cell identities | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.1.5 | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / User reselection | Rel-15 | C36 | UEs supporting 5G Core and user initiated PLMN reselection in automatic mode on NR |
| 6.1.1.6 | PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer | Rel-15 | C34 | UEs supporting 5G Core and MinimumPeriodicSearchTimer |
| 6.1.1.7 | PLMN selection of RPLMN or (E)HPLMN; Automatic mode | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.1.8 | PLMN selection of RPLMN or (E)HPLMN; Manual mode | Rel-15 | C91 | UEs supporting 5G Core and ManualModeNetworkSelectionException |
| 6.1.2 | NG-RAN Only Cell Selection |  |  |  |
| 6.1.2.1 | Cell selection / Qrxlevmin & Cell reselection (Intra NR) | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.2 | Cell selection / Qqualmin / Intra NR / Serving cell becomes non-suitable (Srxlev > 0, Squal < 0) | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.3 | Cell selection / Intra NR / Serving cell becomes non-suitable (S<0, MIB Indicated barred) | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.3a | Intra frequency reselection not allowed | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.4 | Cell reselection for interband operation | Rel-15 | C37 | UEs supporting 5G Core and more than 1 FDD or TDD NR band |
| 6.1.2.5 | Cell reselection for interband operation using Pcompensation / Between FDD and TDD | Rel-15 | C38 | UEs supporting 5G Core and NR FDD and NR TDD |
| 6.1.2.7 | Cell reselection / Equivalent PLMN | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.8 | Cell reselection / Equivalent PLMN / Single Frequency operation | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.9 | Cell reselection using Qhyst, Qoffset and Treselection | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.11 | Area Specific SIBs using systemInformationAreaID | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.12 | Cell reselection using cell status and cell reservations / cellReservedForOtherUse | Rel-15 | C21 | UEs supporting 5G Core. |
| 6.1.2.13 | Cell reselection using cell status and cell reservations / Access Identity 0, 1, 2 and 12 to 14 - cellReservedForOperatorUse | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.14 | Cell reselection using cell status and cell reservations / Access Identity 11 or 15 - cellReservedForOperatorUse | Rel-15 | C21 | UEs supporting 5G Core. |
| 6.1.2.15 | Cell reselection in shared network environment | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.15a | Cell reselection in shared network environment / Cells broadcasting multiple PLMN IDs with unique TAC's, RAN areas, and cell identities | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.16 | Inter-frequency cell reselection (equal priority) | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.17 | Cell reselection / Cell-specific reselection parameters provided by the network in a neighbouring cell list | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.18 | Cell reselection, Sintrasearch, Snonintrasearch | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.19 | Speed dependent cell reselection | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.20 | Inter-frequency cell reselection according to cell reselection priority provided by SIBs | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.21 | Cell reselection, SIntraSearchQ and SnonIntraSearchQ | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.22 | Inter-frequency cell reselection based on common priority information with parameters ThreshX, HighQ, ThreshX, LowQ and ThreshServing, LowQ | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.23 | Cell reselection / MFBI | Rel-15 | C21 | UEs supporting 5G Core |
| 6.1.2.24 | Slice-based cell reselection / Re-seletion priorities provided by SIB16 | Rel-17 | C240 | UEs supporting 5G Core and slice based cell reselection |
| 6.1.2.26 | Cell Selection / RedCap | Rel-17 | C212 | UEs supporting 5G Core and RedCap |
| 6.1.2.27 | Cell reselection / inter-frequency / RedCap | Rel-17 | C212 | UEs supporting 5G Core and RedCap |

## <<End of changes>>