3GPP TSG RAN WG5 IoT Adhoc#2	R5-167123

# Singapore, 10th – 14th October 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR |  | rev |  | Current version: | 12.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corp. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1-UEConTest |  |  |  |  |  | Date: |  |  | 2016-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The “E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX” TC is not included in 36.521-3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added “E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX” into a new Section 8.1.26 in TS36.521-3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The “E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX” TC will remain missing in TS36.521-3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.28 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

### 8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX

Editor’s notes: This test case is incomplete. The following items are missing or incomplete:

- Test tolerances and test requirement values

#### 8.1.28.1	Test purpose

To verify Cat-M1 UE’s ability to make a correct reporting of an event in DRX under fading propagation conditions in asynchronous cells within the E-UTRA HD-FDD intra-frequency cell search requirements in DRX for Cat-M1 UE.

#### 8.1.28.2	Test applicability

This test applies to all types of E-UTRA HD-FDD UE Release 13 and forward of UE category M1.

#### 8.1.28.3	Minimum conformance requirements

When DRX is in use the UE shall be able to identify a new detectable HD-FDD intra frequency cell within Tidentify_intra_UE cat M1 as shown in Table 8.1.28.3-1.

When eDRX_CONN is in use the UE shall be able to identify a new detectable HD-FDD intra frequency cell within Tidentify_intra_UE cat M1 as shown in Table 8.1.28.3-2.

Table 8.1.28.3-1: Requirement to identify a newly detectable HD-FDD intrafrequency cell

| Gap pattern ID | DRX cycle length (s) | Tidentify_intra_UE cat M1 (s) (DRX cycles) |
| --- | --- | --- |
| 0 | ≤0.04 | 1.44 (Note1) |
|  | 0.04<DRX-cycle≤0.08 | Note2 (40) |
|  | 0.128 | 3.2 (25) |
|  | 0.128<DRX-cycle≤2.56 | Note2(20) |
| 1 | ≤0.08 | 2.88 (Note1) |
|  | 0.128 | 3.2 (32) |
|  | 0.128<DRX-cycle≤2.56 | Note2(25) |
| Note1: Number of DRX cycle depends upon the DRX cycle in use Note2: Time depends upon the DRX cycle in use |  |  |

Table 8.1.28.3-2: Requirement to identify a newly detectable HD-FDD intrafrequency cell when eDRX_CONN cycle is used

| eDRX_CONN cycle length (s) | Tidentify_intra_UE cat M1 (s) (eDRX_CONN cycles) |
| --- | --- |
| 2.56<eDRX_CONN cycle≤10.24 | Note (25) |
| NOTE: Time depends upon the eDRX_CONN cycle in use |  |

A cell shall be considered detectable when

-	RSRP related side conditions given in TS 36.133 [4] Sections 9.1.21.1 and 9.1.21.2 are fulfilled for a corresponding Band,

-	SCH_RP and SCH Ês/Iot according to TS 36.133 [4] Table B.2.14-2 for a corresponding Band

In the RRC_CONNECTED state the measurement period for intra frequency measurements is Tmeasure_intra_UE cat M1. When DRX is used, Tmeasure_intra_UE cat M1 is as specified in Table 8.1.28.3-3. When eDRX_CONN is used, Tmeasure_intra_UE cat M1 is as specified in Table 8.1.28.3-4. The UE shall be capable of performing RSRP measurements for 6 identified-intra-frequency cells, and the UE physical layer shall be capable of reporting measurements to higher layers with the measurement period of Tmeasure_intra_UE cat M1.

Table 8.1.28.3-3: Requirement to measure HD-FDD intrafrequency cells

| Gap pattern ID | DRX cycle length (s) | Tmeasure_intra_UE cat M1 (s) (DRX cycles) |
| --- | --- | --- |
| 0 | <0.08 | 0.48 (Note1) |
|  | 0.08≤DRX-cycle≤0.16 | Note2 (7) |
|  | 0.16<DRX-cycle≤2.56 | Note2(5) |
| 1 | <0.16 | 0.96 (Note1) |
|  | DRX-cycle=0.16 | 1.12 (7) |
|  | 0.16<DRX-cycle≤2.56 | Note2(5) |
| Note1: Number of DRX cycle depends upon the DRX cycle in use Note2: Time depends upon the DRX cycle in use |  |  |

Table 8.1.28.3-4: Requirement to measure HD-FDD intrafrequency cells when eDRX_CONN cycle is used

| eDRX_CONN cycle length (s) | Tmeasure_intra_UE cat M1 (s) (eDRX_CONN cycles) |
| --- | --- |
| 2.56<eDRX_CONN cycle≤10.24 | Note (5) |
| NOTE: Time depends upon the eDRX_CONN cycle in use |  |

Reported RSRP measurement contained in event triggered measurement reports shall meet the requirements in TS 36.133 [4] sections 9.1.21.1 and 9.1.21.2.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH.This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than T identify_intra, UE cat M1  defined in Clause 8.1.28.3. When L3 filtering is used or IDC autonomous denial is configured an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_intra_UE cat M1  defined in clause 8.1.28.3 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers an event, the event triggered measurement reporting delay shall be less than Tmeasure_intra_UE cat M1 provided the timing to that cell has not changed more than  50 Ts and the L3 filter has not been used. When L3 filtering is used or IDC autonomous denial is configured, an additional delay can be expected.

The normative reference for this requirement is TS 36.133 [4] clause 8.13.2.1.1.2 and A.8.1.28.

#### 8.1.28.4	Test description

##### 8.1.28.4.1	Initial conditions

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E Table E-1 and TS 36.508 [7] clauses 4.4.2 and 4.3.1.

Channel Bandwidth to be tested: 10 MHz as defined in TS 36.508 [7] clause 4.3.1.

1.	Connect the SS (node B emulator), faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A Figure A.15 using only main UE Tx/Rx antenna.

2.	The general test parameter settings are set up according to Table 8.1.28.4.1-1.

3.	Propagation conditions are set according to Annex B.0.

4.	Message contents are defined in clause 8.1.28.4.3.

5.	There is one E-UTRA FDD carrier and two cells specified in the test. Cell 1 is the cell used for connection setup with the power level set according to Annex C.0 and C.1 for this test.

Table 8.1.28.4.1-1: General test parameters for E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE when DRX is used

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test1 | Test2 |  |
| E-UTRA RF Channel Number |  |  | 1 | 1 | One radio channel is used. |
| Active cell |  |  | Cell 1 | Cell 1 |  |
| Neighbour cell |  |  | Cell 2 | Cell 2 | Cell to be identified. |
| CP length |  |  | Normal | Normal |  |
| DRX |  |  | ON | ON | DRX related parameters are defined in Table 8.1.28.5-2 |
| A3 | Offset | dB | -6 | -6 |  |
|  | Hysteresis | dB | 0 | 0 |  |
|  | Time To Trigger | s | 0 | 0 |  |
| Filter coefficient |  |  | 0 | 0 | L3 filtering is not used |
| Gap pattern ID |  |  | 0 | 0 | As specified in TS 36.133 [4] clause 8.1.2.1. |
| T1 |  | s | 5 | 5 |  |
| T2 |  | s | 5 | 35 |  |

##### 8.1.28.4.2	Test procedure

The test consists of one active cell and one neighbour cell. In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time durations of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

In Test 1 when DRX = 40 ms is used, UE needs to be provided at least once every 500 ms with new Timing Advance Command MAC control element to restart the Timer Alignment Timer to keep the UE uplink time alignment. Furthermore, the UE is allocated with PUSCH resource at every DRX cycle. In Test 2 when DRX = 1280 ms is used, the uplink time alignment is not maintained and the UE needs to use RACH to obtain uplink allocation for measurement reporting.

1.	Ensure the UE is in State 3A-RF-CE with the condition CEModeA according to TS 36.508 [7] clause 7.2A.3AA.

2.	Set the parameters according to T1 in Table 8.1.28.5-1. Propagation conditions are set according to Annex B.1. T1 starts.

3.	The SS shall transmit the RRCConnectionReconfiguration message.

4.	The UE shall transmit the RRCConnectionReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Tables 8.1.28.5-1 and 8.1.28.5-2.

6.	The UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than [1480+TT] ms for Test 1 or less than [26880+TT] ms for Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7.	After the SS receives the MeasurementReport message in step 6) or when T2 expires, the SS shall transmit the RRCConnectionRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9.	After the RRC connection release, the SS:
- transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in State 3A-CE with the condition CEModeA according to TS 36.508 [7] clause 4.5.3AB (if the paging fails, switches off and on the UE and ensures the UE is in State 3A-RF-CE with the condition CEModeA according to TS 36.508 [7] clause 7.2A.3AA),
or
- switches off and on the UE and ensures the UE is in State 3A-RF-CE with the condition CEModeA according to TS 36.508 [7] clause 7.2A.3AA.

10.	Repeat steps 2-9 until the confidence level according to Table G.2.3-1 in Annex G.2 is achieved.

11. Repeat steps 1-10 for each sub-test Table 8.1.28.4.1-1 as appropriate.

##### 8.1.28.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6 with the condition CEModeA and the following exceptions:

Table 8.1.28.4.3-1: Common Exception messages for E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CE in DRX test 1 and 2 requirements

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-2a Table H.3.1-7 Table H.3.7-1 Table H.3.7-2 Table H.3.7-3 |

Table 8.1.28.4.3-2: RRCConnectionReconfiguration: Additional E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CE in DRX test 1 and 2 requirements

| Derivation Path: TS 36.508 [7] clause 4.6.1, Table 4.6.1-8 RRCConnectionReconfiguration |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |
| measConfig |  | MeasConfig -DEFAULT |  |  |
| radioResourceConfigDedicated |  | RadioResourceConfigDedicated-HO |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.28.4.3-3: ReportConfigEUTRA-A3: Additional E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CE in DRX test 1 and 2 requirements

| Derivation Path: TS 36.508 [7] clause 4.6.6, Table 4.6.6-6 ReportConfigEUTRA-A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-A3 ::= SEQUENCE { |  |  |  |
| triggerType CHOICE { |  |  |  |
| event SEQUENCE { |  |  |  |
| eventide CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset | -12 (-6 dB) | -6 is actual value in dB (-12 * 0.5 dB) |  |
| reportOnLeave | FALSE |  |  |
| } |  |  |  |
| } |  |  |  |
| hysteresis | 0 (0 dB) | 0 is actual value in dB (0 * 0.5 dB) |  |
| timeToTrigger | 0 (0 ms) |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.28.4.3-4: SchedulingRequest-Config-DEFAULT: Additional E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CE in DRX test 1 and 2 requirements

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-20 SchedulingRequest-Config-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-PUCCH-ResourceIndex | 41 | 10 MHz channel bandwidth parameter |  |
| sr-ConfigIndex | 0 |  |  |
| dsr-TransMax | n4 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.28.4.3-5: MeasResults: Additional E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CE in DRX test 1 and 2 requirements

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResults ::= SEQUENCE { |  |  |  |
| measId | 1 | Identifies the measurement id for the reporting being performed |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult |  | Set according to specific test |  |
| rsrqResult |  | Set according to specific test |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA | MeasResultListEUTRA |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.28.4.3-6: MeasResultListEUTRA: Additional E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CE in DRX test 1 and 2 requirements

| Derivation Path: 36.331 clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResultListEUTRA ::= SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultEUTRA { |  |  |  |
| physCellId | PhysCellId |  |  |
| measResult SEQUENCE { |  |  |  |
| rsrpResult |  | Set according to specific test INTEGER(0..97) |  |
| rsrqResult |  | Set according to specific test INTEGER(0..34) |  |
| } |  |  |  |
| } |  |  |  |

#### 8.1.28.5	Test requirement

Tables 8.1.28.4.1-1, 8.1.28.5-1, 8.1.28.5-2, and 8.1.28.5-3 define the primary level settings including test tolerances for E-UTRAN HD-FDD event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CE in DRX.

Table 8.1.28.5-1: Cell specific test parameters for E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE when DRX is used

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |
| BWchannel | MHz | 10 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.10 HD-FDD |  | - |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.6 HD-FDD |  | R.6 HD-FDD |  |
| OCNG Patterns |  | OP.21 FDD |  | OP.2 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  |  |  |
|  | dB | 4+TT | 4+TT | -Infinity | 4+TT |
| Note 3 | dB | 4+TT | -1.46+TT | -Infinity | -1.46+TT |
| RSRP Note 3 | dBm/15 KHz | -94+TT | -94+TT | -Infinity | -94+TT |
| SCH_RP Note 3 | dBm/15 KHz | -94+TT | -94+TT | -Infinity | -94+TT |
| Io Note 3 | dBm/9MHz | -64.76+TT | -62.42+Tt | Specified in Cell 1 columns |  |
| Propagation Condition |  | ETU70 |  | ETU70 |  |
| Antenna Configuration |  | 1x1 |  | 1x1 |  |
| Timing offset to Cell 1 | s | - |  | 3 |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |

Table 8.1.28.5-2: DRX-Configuration for E-UTRAN HD-FDD intra-frequency event triggered reporting in DRX under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf40 | sf1280 |  |
| shortDRX | disable | disable |  |

Table 8.1.28.5-3: TimeAlignmentTimer -Configuration for E-UTRAN HD-FDD intra-frequency event triggered reporting in DRX under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

In Test 1 when DRX cycle length = 40 ms is used, the overall delay measured is defined as the time from the beginning of time period T2, to the moment the UE send one Event A3 triggered measurement report to Cell 2 on PUSCH.

In Test 2 when DRX cycle length = 1280 ms is used, the overall delay measured is defined as the time from the beginning of time period T2, to the moment the UE starts to send preambles on the PRACH for Scheduling Request (SR) to obtain allocation to send the measurement report to Cell 2 on PUSCH.

For both tests:

The overall delay measured may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A3 measurement report.

The overall delay measured when DRX cycle length is 40 ms test requirement is expressed as:

Overall delay measured = measurement reporting delay + TTI insertion uncertainty + DRX cycle length

Measurement reporting delay = Tidentify_intra_UE cat M1

Tidentify_intra_UE cat M1   = 1440 ms. When DRX cycle length is 40 ms then the Tidentify_intra_UE cat M1 is 1440 ms.

TTI insertion uncertainty = [TT] ms

DRX cycle length = 40 ms

The overall delay measured when DRX cycle length is 40 ms shall be less than a total of 1480+TT ms.

The overall delay measured when DRX cycle length is 1280 ms test requirement is expressed as:

Overall delay measured = measurement reporting delay + TTI insertion uncertainty + DRX cycle length

Measurement reporting delay = Tidentify_intra_UE cat M1

Tidentify_intra_UE cat M1   = 25600 ms. When DRX cycle length is 1280 ms then the Tidentify_intra_UE cat M1 is 20 x 1280 ms.

TTI insertion uncertainty = [TT] ms

DRX cycle length = 1280 ms

The overall delay measured when DRX cycle length is 1280 ms shall be less than a total of [26880+TT] ms.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

<< Unchanged sections omitted >>