3GPP TSG-RAN WG5 Meeting #101	R5-236016r1

Chicago, USA, 13th – 17th Nov, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-1 | CR | 0436 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update TC 15.3.6 with TT analysis results |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The TT analysis for positioning test case 15.3.6 is still missing. - The test requirement are not finished and the configuration tables are missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Editor’s Note related to TT analysis is removed. - The test requirement tables were added according to the latest TS 38.133 - The LPP response time calculation procedure was added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The positioning test case 15.3.6 remains incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R1:Updated the Es/Iot value to align with the TT analysis result. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

15.3.6	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer with RxTx TEG in FR2 SA

Editor’s Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

15.3.6.1	Test purpose

The purpose of the test is to verify that the relative UE Rx-Tx time difference measurement accuracy when the two measurements are within the same RxTx TEG is within the specified limits. This test will verify the requirements in TS 38.133 [50] clause 10.1.25.2. The test is conducted in AWGN propagation condition in FR2 in standalone scenario when single positioning frequency layer is configured.

15.3.6.2	Test applicability

This test applies to all types of NR UE release 17 onwards that supports Multi-RTT positioning and nr-UE-RxTxTEG-ID-MaxSupport-r17.

15.3.6.3	Minimum conformance requirements

The relative accuracy requirements in Table 10.1.25.2-3b for FR2 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

PRP|dBm according to Annex B.2.14 for a corresponding Band

AWGN propagation condition.

the two UE Rx-Tx time difference measurements are associated with the same RxTx TEG

Table 10.1.25.2-3b: UE Rx-Tx time difference relative measurement accuracy in FR2 in AWGN with TEG reporting

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |
|  |  |  |  |  | Minimum Io | Maximum Io |
| Tc Note 4 | dB | kHz | RB |  | dBm/SCS | dBm/BWChannel |
| 35 +ΔNote 6 | (PRS Ês/Iot)j ≥-6dB (PRS Ês/Iot)i ≥-13dB | 60 | ≥ 24 | ≥ 4 | Same value as PRS_RP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| 24 +Δ |  |  | ≥ 64 | ≥ 1 | Note 5 | Note 5 |
| 11 +Δ |  |  | ≥ 132 | ≥ 1 | Note 5 | Note 5 |
| 24+Δ |  | 120 | ≥ 32 | ≥ 4 | Same value as PRS_RP in Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| 13 +Δ |  |  | ≥ 64 | ≥ 1 | Note 5 | Note 5 |
| 6 +Δ |  |  | ≥ 128 | ≥ 1 | Note 5 | Note 5 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of resource j and resource i. NOTE 2: Minimum number of PRS resource repetitions among resource j and resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 6: Δis the value of the timing error margin for the RxTx TEG, reported via nr-UE-RxTxTEG-TimingErrorMargin. Δ cannot be larger than the sum of the margins in table 10.1.25.2-6 (dependent on PRS/SRS BW) for any pair of individual UE Rx-Tx time difference measurements associated with the RxTx TEG. |  |  |  |  |  |  |

…

Table 10.1.25.2-6: Margin for UE Rx-Tx time difference measurement accuracy in FR2

| Min(PRS BW, SRS BW) (MHz) |  | Margin (Tc Note 1) |
| --- | --- | --- |
| SCS = 60 kHz | SCS = 120 kHz |  |
| ≥ 24 | N/A | 76 |
| ≥ 64 | ≥ 32 | 32 |
| ≥ 132 | ≥ 64 | 24 |
| N/A | ≥ 128 | 20 |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 2: If SRS and PRS have different SCS, the margin corresponding to the smallest RS BW in MHz applies. |  |  |

15.3.6.4	Test description

15.3.6.4.1	Initial conditions

The supported test configuration is listed in Table 15.3.6.4-1.

Table 15.3.6.4-1: Test Configuration

| Test Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB and PRS SCS, 100 MHz bandwidth, TDD duplex mode |

Test Environment: Normal, as defined in TS 38.508-1 [45] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [45] clause 4.3.1.

Channel bandwidth to be tested: are specified in Table 15.3.6.4-1.

1.	Connect the SS and AWGN noise sources to the UE antenna connector or antenna connectors as shown in Annex A, Figure A.14.

2.	The general test parameter settings are set up according to Table 15.3.6.5-1.

3.	Propagation conditions are set according to clause 4.14.2.

4.	Message contents are defined in clause 15.3.6.4.3.

5.	In the test there are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). All cells are on the same RF channel in FR2.

15.3.6.4.2	Test procedure

The test consists of two sub-tests; the difference between the sub-tests is the PRS configuration, PRS.1.1 FR2 and PRS.1.2 FR2. The test consists of a set-up period and a measurement period. Cell 1 and Cell 2 are both active during the complete test. The beginning of the measurement period shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell. The NR-Multi-RTT-ProvideAssistanceData and NR-Multi-RTT-RequestLocationInformation shall be provided to the UE during the set-up period.

The UE is configured with measurement gap pattern ID # 24 or #13 before T2.

- Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, according to TS 38.508-1 [45] clause 4.5. Note that in the remainder of the test the timing advance command TA = 31 which indicates a timing advance adjustment value NTA = 0 Tc.

2.	The SS shall send a RESET UE POSITIONING STORED INFORMATION message.

3.	The SS adjusts the downlink timing for Cell 1 to a delay of +8 Tc, compared to the current value.

4.	Wait for 1.6 s to allow for the possibility that the UE makes autonomous timing adjustments.

5.	The SS shall transmit an RRCReconfiguration message with the SRS configuration.

6.	The UE shall transmit RRCReconfigurationComplete message.

7.	The SS shall transmit an LPP REQUEST CAPABILITIES message.

8.	The UE shall transmit an LPP PROVIDE CAPABILITIES message indicating the Multi-RTT capabilities supported by the UE in the NR-Multi-RTT-ProvideCapabilities IE.

9.	The SS shall send a LPP PROVIDE ASSISTANCE DATA message, including the NR-Multi-RTT-ProvideAssistanceData IE. The position of neighbour Cell 2 is described in 3GPP TS 37.571-5 [20]. If the UE message at step 8 includes the ackRequested IE set to TRUE, then the SS shall send an acknowledgment in the LPP PROVIDE ASSISTANCE DATA message.

10.	The SS shall send a LPP REQUEST LOCATION INFORMATION message, including the nr-UE-RxTxTEG-Request-r17 in the NR-Multi-RTT-RequestLocationInformation IE such that the UE receives the message T ms before the start of the measurement period, where T = 50 ms.

11.	The UE shall transmit a LPP PROVIDE LOCATION INFORMATION message including the NR-Multi-RTT-ProvideLocationInformation IE within the response time (see clause 15.3.6.4.3).

12.	If the UE message at step 11 includes the ackRequested IE set to TRUE, the SS shall send a LPP acknowledgement message.

13.	The SS shall check nr-UE-RxTxTimeDiff-r16 value for Cell 2 in the NR-Multi-RTT-SignalMeasurementInformation according to Table 15.3.6.5-3.

14.	Repeat steps 1-13 until the confidence level according to Annex D is achieved.

15.	Repeat step 1-14for the other sub-test defined in Table 15.3.6.5-1 as appropriate.

If all (applicable) sub-tests pass, the whole test passes. If one (applicable) sub-test fails, the whole test fails.

15.3.6.4.3	Message contents

Table 15.3.6.4.3-1: RESET UE POSITIONING STORED INFORMATION

| Derivation Path: 38.509 [44] clause 6.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE Positioning Technology | 0 0 0 0 0 1 1 0 | Multi-RTT |  |

Table 15.3.6.4.3-2: RRCReconfiguration

| Derivation Path: TS 38.508-1 [45],, table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup SEQUENCE { |  | CellGroupConfig |  |  |
| spCellConfig SEQUENCE { |  |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |  |
| srs-Config CHOICE { |  |  |  |  |
| setup |  | As defined in Table 15.3.6.4.3-3 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 15.3.6.4.3-3: SRS-Config (Table 15.3.6.4.3-2)

| Derivation Path: TS 38.508-1 [45], Table 4.6.3-182 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRS-Config ::= SEQUENCE { |  |  |  |
| srs-ResourceSetToReleaseList | Not present |  |  |
| srs-ResourceSetToAddModList | Not present |  |  |
| srs-ResourceToReleaseList | Not present |  |  |
| srs-ResourceToAddModList | Not present |  |  |
| tpc-Accumulation | Not present |  |  |
| srs-RequestDCI-1-2-r16 | Not present |  |  |
| srs-RequestDCI-0-2-r16 | Not present |  |  |
| srs-ResourceSetToAddModListDCI-0-2-r16 | Not present |  |  |
| srs-ResourceSetToReleaseListDCI-0-2-r16 | Not present |  |  |
| srs-PosResourceSetToReleaseList-r16 | Not present |  |  |
| srs-PosResourceSetToAddModList-r16 SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSet-r16 { | 1 entry |  |  |
| SRS-PosResourceSet-r16[1] SEQUENCE { |  | entry 1 |  |
| srs-PosResourceSetId-r16 | 0 |  |  |
| srs-PosResourceIdList-r16 SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-PosResourceId-r16 { | 1 entry |  |  |
| SRS-PosResourceId-r16 | 0 | 1 entry |  |
| } |  |  |  |
| resourceType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |
| alpha-r16 | alpha0 |  |  |
| p0-r16 | 0 |  |  |
| pathlossReferenceRS-Pos-r16 | Not present |  |  |
| } |  |  |  |
| srs-PosResourceToReleaseList-r16 | Not present |  |  |
| srs-PosResourceToAddModList-r16 SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResource-r16 { | 1 entry |  |  |
| SRS-PosResource-r16[1] SEQUENCE { |  | entry 1 |  |
| srs-PosResourceId-r16 | 0 |  |  |
| transmissionComb-r16 CHOICE { |  |  |  |
| n4-r16 SEQUENCE { |  |  |  |
| combOffset-n4-r16 | 0 |  |  |
| cyclicShift-n4-r16 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceMapping-r16 SEQUENCE { |  |  |  |
| startPosition-r16 | 0 |  |  |
| nrofSymbols-r16 | n4 |  |  |
| } |  |  |  |
| freqDomainShift-r16 | 0 |  |  |
| freqHopping-r16 SEQUENCE { |  |  |  |
| c-SRS-r16 | Matches NRB,c Table 15.3.6.5-1 |  |  |
| } |  |  |  |
| groupOrSequenceHopping-r16 | neither |  |  |
| resourceType-r16 CHOICE { |  |  |  |
| periodic-r16 SEQUENCE { |  |  |  |
| periodicityAndOffset-p-r16 CHOICE { |  |  |  |
| sl1280 | 160 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sequenceId-r16 | 0 |  |  |
| spatialRelationInfoPos-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 15.3.6.4.3-4: LPP Request Capabilities

| Information Element | Value/remark |
| --- | --- |
| nr-Multi-RTT-RequestCapabilities-r16 | TRUE |

Table 15.3.6.4.3-5: LPP RequestLocationInformation

| Derivation Path: TS 37.355 [49] clause 6.2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| LPP-Message ::= SEQUENCE { |  |  |  |  |  |  |  |
| transactionID SEQUENCE { |  |  |  |  |  |  |  |
| initiator |  | locationServer |  |  |  |  |  |
| transactionNumber |  | 1 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| endTransaction |  | FALSE |  |  |  |  |  |
| sequenceNumber |  | Not present |  |  |  |  |  |
| acknowledgement |  | Not present |  |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE { |  |  |  |  |  |  |  |
| requestLocationInformation SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE { |  |  |  |  |  |  |  |
| requestLocationInformation-r9 SEQUENCE { |  |  |  |  |  |  |  |
| commonIEsRequestLocationInformation SEQUENCE { |  |  |  |  |  |  |  |
| locationInformationType |  | locationMeasurementsPreferred |  |  |  |  |  |
| triggeredReporting |  | Not present |  |  |  |  |  |
| periodicalReporting |  | Not present |  |  |  |  |  |
| additionalInformation |  | onlyReturnInformationRequested |  |  |  |  |  |
| qos SEQUENCE { |  |  |  |  |  |  |  |
| horizontalAccuracy |  | Not present |  |  |  |  |  |
| verticalCoordinateRequest |  | FALSE |  |  |  |  |  |
| verticalAccuracy |  | Not present |  |  |  |  |  |
| responseTime SEQUENCE { |  |  |  |  |  |  |  |
| time |  | See 15.3.6.5 |  | Result of the response time calculation rounded up to the next second if response time <= 128s. Result of the response time calculation rounded up to the next multiple of ten seconds if response time > 128s |  |  |  |
| responseTimeEarlyFix-r12 |  | Not present |  |  |  | Rel-12 onwards |  |
| unit-r15 |  | Not present |  |  |  |  |  |
|  |  | ten-seconds |  |  |  | Calculated response time >128s |  |
| } |  |  |  |  |  |  |  |
| velocityRequest |  | FALSE |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| environment |  | Not present |  |  |  |  |  |
| locationCoordinateTypes |  | Not present |  |  |  |  |  |
| velocityTypes |  | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| a-gnss-RequestLocationInformation |  | Not present |  |  |  |  |  |
| otdoa-RequestLocationInformation |  | Not present |  |  |  |  |  |
| ecid-RequestLocationInformation |  | Not present |  |  |  |  |  |
| epdu-RequestLocationInformation |  | Not Present |  |  |  |  |  |
| sensor-RequestLocationInformation-r13 |  | Not present |  |  |  |  |  |
| tbs-RequestLocationInformation-r13 |  | Not present |  |  |  |  |  |
| wlan-RequestLocationInformation-r13 |  | Not present |  |  |  |  |  |
| bt-RequestLocationInformation-r13 |  | Not present |  |  |  |  |  |
| nr-ECID-RequestLocationInformation-r16 |  | Not present |  |  |  |  |  |
| nr-Multi-RTT-RequestLocationInformation-r16 SEQUENCE { |  |  |  |  |  |  |  |
| nr-UE-RxTxTimeDiffMeasurementInfoRequest-r16 |  | Not present |  |  |  |  |  |
| nr-RequestedMeasurements-r16 |  | bit 0 = 1 (prsrsrpReq) |  |  |  |  |  |
| nr-AssistanceAvailability-r16 |  | FALSE |  |  |  |  |  |
| nr-Multi-RTT-ReportConfig-r16 SEQUENCE { |  |  |  |  |  |  |  |
| maxDL-PRS-RxTxTimeDiffMeasPerTRP-r16 |  | Not present |  |  |  |  |  |
| timingReportingGranularityFactor-r16 |  | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| additionalPaths-r16 |  | Not present |  |  |  |  |  |
| nr-UE-RxTxTEG-Request-r17 |  | case1 |  |  |  |  |  |
| measureSameDL-PRS-ResourceWithDifferentRxTxTEGs-r17 |  | Not present |  |  |  |  |  |
| measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 |  | n2 |  |  |  |  |  |
| reducedDL-PRS-ProcessingSamples-r17 |  | Not present |  |  |  |  |  |
| nr-los-nlos-IndicatorRequest-r17 |  | Not present |  |  |  |  |  |
| additionalPathsExt-r17 |  | Not present |  |  |  |  |  |
| additionalPathsDL-PRS-RSRP-Request-r17 |  | Not present |  |  |  |  |  |
| multiMeasInSameReport-r17 |  | Not present |  |  |  |  |  |
| lowerRxBeamSweepingFactor-FR2-r17 |  | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| nr-DL-AoD-RequestLocationInformation-r16 |  | Not present |  |  |  |  |  |
| nr-DL-TDOA-RequestLocationInformation-r16 |  | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 15.3.6.4.3-6: LPP ProvideAssistanceData

| Derivation Path: 37.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | (0..255) |  |  |
| } |  |  |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | Not present |  |  |
| acknowledgement | Not present |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData-r9 SEQUENCE { |  |  |  |
| commonIEsProvideAssistanceData | Not present |  |  |
| a-gnss-ProvideAssistanceData | Not present |  |  |
| otdoa-ProvideAssistanceData | Not present |  |  |
| epdu-Provide-AssistanceData | Not present |  |  |
| sensor-ProvideAssistanceData-r15 | Not present |  |  |
| tbs-ProvideAssistanceData-r15 | Not present |  |  |
| wlan-ProvideAssistanceData-r15 | Not present |  |  |
| nr-Multi-RTT-ProvideAssistanceData-r16 SEQUENCE { |  |  |  |
| nr-DL-PRS-AssistanceData | As defined in Table 15.3.6.4.3-7 |  |  |
| nr-SelectedDL-PRS-IndexList-r16 | Not present |  |  |
| nr-Multi-RTT-Error-r16 | Not present |  |  |
| nr-On-Demand-DL-PRS-Configurations-r17 | Not present |  |  |
| nr-On-Demand-DL-PRS-Configurations-Selected-IndexList-r17 | Not present |  |  |
| assistanceDataValidityArea-r17 | Not present |  |  |
| } |  |  |  |
| nr-DL-AoD-ProvideAssistanceData-r16 | Not present |  |  |
| nr-DL-TDOA-ProvideAssistanceData-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 15.3.6.4.3-7: NR-DL-PRS-AssistanceData

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ReferenceInfo-r16 SEQUENCE { |  |  |  |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-DL-PRS-ResourceID-List-r16 | Not present |  |  |
| nr-DL-PRS-ResourceSetID-r16 | Not present |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataList-r16 SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-DL-PRS-AssistanceDataPerFreq-r16 { | 1 entry |  |  |
| NR-DL-PRS-AssistanceDataPerFreq-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-PositioningFrequencyLayer-r16 SEQUENCE { |  |  |  |
| dl-PRS-SubcarrierSpacing-r16 | kHz120 |  |  |
| dl-PRS-ResourceBandwidth-r16 | 3 | 32 PRBs | Sub-test 1 |
|  | 27 | 128 PRBs | Sub-test 2 |
| dl-PRS-StartPRB-r16 | 0 |  |  |
| dl-PRS-PointA-r16 | absoluteFrequencyPointA as defined for the DL frequency of the Cell 1 |  |  |
| dl-PRS-CombSizeN-r16 | n2 |  | Sub-test 1 |
|  | n4 |  | Sub-test 2 |
| dl-PRS-CyclicPrefix-r16 | normal |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-AssistanceDataPerTRP-r16{ | 2 entries |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[1] SEQUENCE { |  | entry 1 | Cell 1 |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-PhysCellID-r16 | Cell 1 |  |  |
| nr-CellGlobalID-r16 | Cell 1 |  |  |
| nr-ARFCN-r16 | Cell 1 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 0 |  |  |
| nr-DL-PRS-Info-r16 | As specified in Table 15.3.6.4.3-8 |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[2] SEQUENCE { |  | entry 2 | Cell 2 |
| dl-PRS-ID-r16 | 1 |  |  |
| nr-PhysCellID-r16 | Cell 2 |  |  |
| nr-CellGlobalID-r16 | Cell 2 |  |  |
| nr-ARFCN-r16 | Cell 2 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 23 | About 3 s |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 154 | About 5 s |  |
| nr-DL-PRS-Info-r16 | As specified in Table 15.3.6.4.3-8 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 15.3.6.4.3-8: NR-DL-PRS-Info

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-Info-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ResourceSetList-r16 SEQUENCE (SIZE (1..nrMaxSetsPerTrp-r16)) OF NR-DL-PRS-ResourceSet-r16 { | 1 entry |  |  |
| NR-DL-PRS-ResourceSet-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceSetID-r16 | 0 |  |  |
| dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16 CHOICE { |  |  |  |
| scs120-r16 CHOICE { |  |  |  |
| n160-r16 | 80 |  |  |
| } |  |  |  |
| dl-PRS-ResourceRepetitionFactor-r16 | n2 |  | Sub-test 1 |
|  | Not present |  | Sub-test 2 |
| dl-PRS-ResourceTimeGap-r16 | s1 |  |  |
| dl-PRS-NumSymbols-r16 | n4 |  |  |
| dl-PRS-MutingOption1-r16 | Not present |  |  |
| dl-PRS-MutingOption2-r16 | Not present |  |  |
| dl-PRS-ResourcePower-r16 | 27 |  |  |
| dl-PRS-ResourceList-r16 SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF NR-DL-PRS-Resource-r16 { | 1 entry |  |  |
| NR-DL-PRS-Resource-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceID-r16 | 0 |  |  |
| dl-PRS-SequenceID-r16 | 0 |  |  |
| dl-PRS-CombSizeN-AndReOffset-r16 CHOICE { |  |  |  |
| n2-r16 | 0 |  | Sub-test 1 |
| n4-r16 | 0 |  | Sub-test 2 |
| } |  |  |  |
| dl-PRS-ResourceSlotOffset-r16 | 0 |  | Sub-test 1 |
|  | 4 |  | Sub-test 2 |
| dl-PRS-ResourceSymbolOffset-r16 | 0 |  |  |
| dl-PRS-QCL-Info-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 15.3.6.4.3-9: LPP ProvideLocation Information

| Derivation Path: 37.355 clause 6.5.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| transactionID SEQUENCE { |  |  |  |  |
| initiator |  | locationServer |  |  |
| transactionNumber |  | 1 |  |  |
| } |  |  |  |  |
| endTransaction |  | TRUE |  |  |
| sequenceNumber |  | (0..255) |  |  |
| acknowledgement |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideLocationInformation-r9 SEQUENCE { |  |  |  |  |
| commonIEsProvideLocationInformation |  | Not present. |  |  |
| a-gnss-ProvideLocationInformation |  | Not present |  |  |
| otdoa-ProvideLocationInformation |  | Not present |  |  |
| ecid-ProvideLocationInformation |  | Not present |  |  |
| epdu-ProvideLocationInformation |  | Not present |  |  |
| sensor-ProvideLocationInformation-r13 |  | Not present |  |  |
| tbs-ProvideLocationInformation-r13 |  | Not present |  |  |
| wlan-ProvideLocationInformation-r13 |  | Not present |  |  |
| bt-ProvideLocationInformation-r13 |  | Not present |  |  |
| nr-ECID-ProvideLocationInformation-r16 |  | Not present |  |  |
| nr-Multi-RTT-ProvideLocationInformation-r16 SEQUENCE { |  |  |  |  |
| nr-Multi-RTT-SignalMeasurementInformation-r16 SEQUENCE { |  |  |  |  |
| nr-Multi-RTT-MeasList-r16 SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-Multi-RTT-MeasElement-r16 { |  | 2 entries |  |  |
| NR-Multi-RTT-MeasElement-r16[1] SEQUENCE { |  |  | entry 1 |  |
| dl-PRS-ID-r16 |  | INTEGER (0..255) |  |  |
| nr-PhysCellID-r16 |  |  |  |  |
| nr-CellGlobalID-r16 |  |  |  |  |
| nr-ARFCN-r16 |  |  |  |  |
| nr-DL-PRS-ResourceID-r16 |  |  |  |  |
| nr-DL-PRS-ResourceSetID-r16 |  |  |  |  |
| nr-UE-RxTxTimeDiff-r16 |  | Present. Any value acceptable. |  |  |
| nr-AdditionalPathList-r16 |  |  |  |  |
| nr-TimeStamp-r16 |  |  |  |  |
| nr-TimingQuality-r16 |  |  |  |  |
| nr-DL-PRS-RSRP-Result-r16 |  |  |  |  |
| nr-Multi-RTT-AdditionalMeasurements-r16 |  |  |  |  |
| nr-UE-RxTx-TEG-Info-r17 CHOICE { |  |  |  |  |
| case1-r17 SEQUENCE { |  |  |  |  |
| nr-UE-RxTx-TEG-ID-r17 |  | INTEGER (0..255) |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nr-DL-PRS-FirstPathRSRP-Result-r17 |  |  |  |  |
| nr-los-nlos-Indicator-r17 |  |  |  |  |
| nr-AdditionalPathListExt-r17 |  |  |  |  |
| nr-Multi-RTT-AdditionalMeasurementsExt-r17 |  |  |  |  |
| } |  |  |  |  |
| NR-Multi-RTT-MeasElement-r16[2] SEQUENCE { |  |  | entry 2 |  |
| dl-PRS-ID-r16 |  | INTEGER (0..255) |  |  |
| nr-PhysCellID-r16 |  |  |  |  |
| nr-CellGlobalID-r16 |  |  |  |  |
| nr-ARFCN-r16 |  |  |  |  |
| nr-DL-PRS-ResourceID-r16 |  |  |  |  |
| nr-DL-PRS-ResourceSetID-r16 |  |  |  |  |
| nr-UE-RxTxTimeDiff-r16 |  | Present. Any value acceptable. |  |  |
| nr-AdditionalPathList-r16 |  |  |  |  |
| nr-TimeStamp-r16 |  |  |  |  |
| nr-TimingQuality-r16 |  |  |  |  |
| nr-DL-PRS-RSRP-Result-r16 |  |  |  |  |
| nr-Multi-RTT-AdditionalMeasurements-r16 |  |  |  |  |
| nr-UE-RxTx-TEG-Info-r17 CHOICE { |  |  |  |  |
| case1-r17 SEQUENCE { |  |  |  |  |
| nr-UE-RxTx-TEG-ID-r17 |  | INTEGER (0..255) |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nr-DL-PRS-FirstPathRSRP-Result-r17 |  |  |  |  |
| nr-los-nlos-Indicator-r17 |  |  |  |  |
| nr-AdditionalPathListExt-r17 |  |  |  |  |
| nr-Multi-RTT-AdditionalMeasurementsExt-r17 |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nr-NTA-Offset-r16 |  |  |  |  |
| nr-SRS-TxTEG-Set-r17 |  |  |  |  |
| nr-UE-RxTEG-TimingErrorMargin-r17 |  |  |  |  |
| nr-UE-TxTEG-TimingErrorMargin-r17 |  |  |  |  |
| nr-UE-RxTxTEG-TimingErrorMargin-r17 |  |  |  |  |
| } |  |  |  |  |
| nr-Multi-RTT-Error-r16 |  |  |  |  |
| nr-Multi-RTT-SignalMeasurementInstances-r17 |  |  |  |  |
| } |  |  |  |  |
| nr-DL-AoD-ProvideLocationInformation-r16 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

15.3.6.5	Test requirement

Table 15.3.6.5-1 defines the primary level settings including test tolerances for the test.

Table 15.3.6.5-1: UE Rx-Tx time difference measurement accuracy test parameters

| Parameter | Unit | Test configuration | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| AoA setup |  | 1 | Setup 1 as specified in clause A.3.15 |  | Setup 1 as specified in clause A.3.15 |  |
| Beam AssumptionNote 7 |  | 1 | Rough | Rough | Rough | Rough |
| Measurement gap |  | 1 | GP#24 or GP#13 Note 8 |  | GP#24 or GP#13 Note 8 |  |
| DRX |  | 1 | OFF |  | OFF |  |
| Time offset with Cell 1 | s | 1 | N/A | 3 | N/A | 3 |
| TDD configuration |  | 1 | TDDConf.3.1 | TDDConf.3.1 | TDDConf.3.1 | TDDConf.3.1 |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD | N/A | SR.3.1 TDD | N/A |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD | N/A | CR.3.1 TDD | N/A |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD | N/A | CCR.3.1 TDD | N/A |
| OCNG Patterns |  | 1 | OP.1 | OP.1 | OP.1 | OP.1 |
| EPRE ratio of PSS to SSS | dB | 1 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |  |  |
| TRS Configuration |  | 1 | TRS.2.1 TDD | N/A | TRS.2.1 TDD | N/A |
| Initial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 | N/A | DLBWP.0.1 ULBWP.0.1 | N/A |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 | N/A | DLBWP.1.1 | N/A |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 | N/A | ULBWP.1.1 | N/A |
| PRS configuration |  | 1 | PRS.1.1 FR2 | PRS.1.1 FR2 | PRS.1.2 FR2 | PRS.1.2 FR2 |
| PRS Resource slot offset | slot | 1 | 0 | 4 | 0 | 4 |
| SRS configuration |  | 1 | POS-SRS.3 | N/A | POS-SRS.3 | N/A |
| Note 2 | dBm/SCS | 1 | -89 |  | -89 |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  | -98 |  |
| PRS | dB | 1 | -5.54 | -12.37 | -5.54 | -12.37 |
| PRS | dB | 1 | -5 | -11 | -5 | -11 |
| PRP Note 3 | dBm/SCS kHz | 1 | -94 | -100 | -94 | -100 |
| Io | dBm/95.04 MHz | 1 | -61.68 | -61.68 | -55.66 | -55.66 |
| Propagation Condition |  | 1 | AWGN |  | AWGN |  |
| Note 1: Void. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: PRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in TS 38.133 [50] B.2.1.3, and does not limit UE implementation or test system implementation Note 8: GP#24 is configured if UE supports MG#24, otherwise GP#13 is configured. Note 9: Calculation of Es/Iot includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [55], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [55] Table 6.2.1.3-4. |  |  |  |  |  |  |

The UE Rx-Tx time difference measurement accuracy for Cell 2 shall fulfil the absolute requirement in clause 15.3.6.3.

The value of the LPP Time IE depends on the UE capabilities. The calculation is the same as in clause 15.2.10.5 Therefore, the value of the LPP Time IE is given by the following equation:

kmultiTEG*8*2N'0.071N*4-1

Where the remaining parameters depend on the UE capabilities. The LPP time IE ranges between 10.248s and 164s.

Table 15.3.6.5-2: UE Rx – Tx time difference measurement accuracy requirements

| Test Configuration | Lowest reported value | Highest reported value |
| --- | --- | --- |
| Sub-test 1 | (1966.08 – 48) Tc converted to UE Rx-Tx time difference measurement according to clause 4.14.6 | (1966.08 + 58) Tc converted to UE Rx-Tx time difference measurement according to clause 4.14.6 |
| Sub-test 2 | (1966.08 – 30) Tc converted to UE Rx-Tx time difference measurement according to clause 4.14.6 | (1966.08 + 30) Tc converted to UE Rx-Tx time difference measurement according to clause 4.14.6 |

The test tolerances are defined in clauses C.1.6 and C.2.5.

The rate of successful tests during repeated tests shall be at least 90% with a confidence level of 95%.