3GPP TSG-RAN5 Meeting #101	R5-236034r1

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 2711 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Deletion of brackets editors notes and core spec alignment update for IoT NTN RA test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek (Hefei) Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Core spec alignment. [ ] are removed in TS36.133. - Core spec alignment. Adding contents in Test Purpose and Environment. - Ran4 dependency. Missing clarification on timing pre-compensation in 13.3.2.3. - Typo in 13.3.2.3.4.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the corresponding editor’s notes for 13.3.2.1, 13.3.2.2 and 13.3.2.3. And correct related contents. - Adding contents in test purpose according to core spec. - 13.3.2.3: align clarification on timing pre-compensation with 13.3.2.1/2 - Correct the typo in 13.3.2.3.4.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IoT NTN RA cases remain uncompleted and not aligned with core spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.3.2.1, 13.3.2.2, 13.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depending on RAN4 CR R4-2318070 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: to fix the 3GU issue, delete Invalid characters ‘ ‘ ’ in CR title and file name. |  |  |  |  |  |  |  |  |

<<< START OF CHANGES 1>>>

13.3.2.1	Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in normal coverage

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- MU/TT is TBD

- Initial condition and call setup procedure support satellite access are TBD

- Different configuration between NGSO and GSO is FFS

13.3.2.1.1	Test purpose

The purpose of this test is to verify whether the behavior of the random access procedure of a category NB1 UE in Normal Coverage is according to the requirements when connected to a NTN NB-IoT cell, whether the NPRACH power settings and timing are within specified limits, and whether the UE determines properly the enhanced coverage level based on the NRSRP measurement and the configured criterion in NRSRP-ThresholdsPrach[5]. This test will verify the requirements in 3GPP TS 36.133[4] Clause 6.6A.2, Clause 6.6A.3 and Clause 7.20A.2 in an AWGN model.

For this test a single NB-IoT cell is used. The test parameters are given in tables 13.3.2.1.5-1, 13.3.2.1.5-2 and 13.3.2.1.5-3. The UE shall perform timing pre-compensation before the initial NPRACH transmission using AT command-based test approach.

<< END OF CHANGES 1>>

<<< START OF CHANGES 2>>>

Table 13.3.2.1.5-1: nCell specific test parameters for HD-FDD contention based random access test for UE category NB1 Standalone mode in Normal Coverage

| Parameter | Unit | Value | Comments |
| --- | --- | --- | --- |
| NB-IOT operational mode |  | Standalone |  |
| BWchannel | kHz | 200 |  |
| NPDSCH parameters Note 2 |  | R.18 HD-FDD | As defined in TS36.133 A.3.1.5.3 |
| NPDCCH parameters Note 2 |  | R.30 HD-FDD | As defined in TS36.133 A.3.1.6.3 |
| NPBCH_RA | dB | -3 |  |
| NPBCH_RB | dB | -3 |  |
| NPSS_RA | dB |  |  |
| NSSS_RA | dB |  |  |
| NPDCCH_RA | dB |  |  |
| NPDCCH_RB | dB |  |  |
| NPDSCH_RA | dB |  |  |
| NPDSCH_RB | dB |  |  |
| NOCNG_RA Note 1 | dB |  |  |
| NOCNG_RB Note 1 | dB |  |  |
| DRX |  | OFF |  |
|  | dBm/15 kHz | -98+TT |  |
|  | dB | 3+TT |  |
| Note 3 | dB | 3 |  |
| NRSRP Note 3 | dBm/15 kHz | -95 |  |
| Io Note 3 | dBm/180 KHz | -82.45 |  |
| Propagation Condition | - | AWGN |  |
| Antenna Configuration |  | 1x1 |  |
| Note 1: NOCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The NPDSCH and NPDCCH reference measurement channels are used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: Es/Iot, NRSRP and Io level has been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |

<< END OF CHANGES 2>>

<<< START OF CHANGES 3>>>

Table 13.3.2.1.5-6: Test requirements for Te Timing Error Limit for HD-FDD contention based random access test for UE category NB1

| Downlink Bandwidth (MHz) | Te_ |
| --- | --- |
| 0.18 | 97*TS + TT |
| NOTE: TS is the basic timing unit defined in 3GPP TS 36.211 |  |

<< END OF CHANGES 3>>

<<< START OF CHANGES 4>>>

13.3.2.2	Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in Enhanced Coverage

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- MU/TT is TBD

- Initial condition and call setup procedure support satellite access are TBD

- Different configuration between NGSO and GSO is FFS

13.3.2.1.1	Test purpose

The purpose of this test is to verify whether the behavior of the random access procedure of a category NB1 UE in Normal Coverage is according to the requirements when connected to a NTN NB-IoT cell, whether the NPRACH power settings and timing are within specified limits, and whether the UE determines properly the enhanced coverage level based on the NRSRP measurement and the configured criterion in NRSRP-ThresholdsPrach[5]. This test will verify the requirements in 3GPP TS 36.133[4] Clause 6.6A.2, Clause 6.6A.3 and Clause 7.20A.2 in an AWGN model.

For this test a single NB-IoT cell is used. The test parameters are given in tables 13.3.2.1.5-1, 13.3.2.1.5-2 and 13.3.2.1.5-3. The UE shall perform timing pre-compensation before the initial NPRACH transmission using AT command-based test approach.

<< END OF CHANGES 4>>

<<< START OF CHANGES 5>>>

Table 13.3.2.2.5-1: nCell specific test parameters for HD-FDD contention based random access test for UE category NB1 Standalone mode in Enhanced Coverage

| Parameter | Unit | Value | Comments |  |
| --- | --- | --- | --- | --- |
| NB-IOT operational mode |  | Standalone |  |  |
| BWchannel | kHz | 200 |  |  |
| NPDSCH parameters Note 2 |  | R.18 HD-FDD | As defined in TS36.133 A.3.1.5.3 |  |
| NPDCCH parameters Note 2 |  | R.30 HD-FDD | As defined in TS36.133 A.3.1.6.3 |  |
| NPBCH_RA | dB | -3 |  |  |
| NPBCH_RB | dB |  |  |  |
| NPSS_RA | dB |  |  |  |
| NSSS_RA | dB |  |  |  |
| NPDCCH_RA | dB |  |  |  |
| NPDCCH_RB | dB |  |  |  |
| NPDSCH_RA | dB |  |  |  |
| NPDSCH_RB | dB |  |  |  |
| NOCNG_RA Note 1 | dB |  |  |  |
| NOCNG_RB Note 1 | dB |  |  |  |
| DRX |  | OFF |  |  |
|  | dBm/15 kHz | -98+TT |  |  |
|  | dB | -12.5+TT |  |  |
| Note 3 | dB | -12.5 |  |  |
| NRSRP Note 3 | dBm/15 kHz | -110.5 |  |  |
| Io Note 3 | dBm/180KHz | -86.97 |  |  |
| Propagation Condition | - | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  |
| Note 1: NOCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The NPDSCH and NPDCCH reference measurement channels are used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: Es/Iot, NRSRP and Io level has been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |

<< END OF CHANGES 5>>

<<< START OF CHANGES 6>>>

Table 13.3.2.2.5-6: Test requirements for Te Timing Error Limit for HD-FDD contention based random access test for UE category NB1

| Downlink Bandwidth (MHz) | Te_ |
| --- | --- |
| 0.18 | 97*TS + TT |
| NOTE: TS is the basic timing unit defined in 3GPP TS 36.211 |  |

<< END OF CHANGES 6>>

<<< START OF CHANGES 7>>>

13.3.2.3	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs Standalone mode in Enhanced Coverage

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- MU/TT is TBD

- Initial condition and call setup procedure support satellite access are TBD

- Different configuration between NGSO and GSO is FFS

13.3.2.3.1	Test purpose

The purpose of this test is to verify whether the behavior of the random access procedure of a category NB1 UE in Enhanced Coverage is according to the requirements, whether the NPRACH power settings and timing are within specified limits, and whether the UE determines properly the enhanced coverage level based on the NRSRP measurement and the configured criterion in NRSRP-ThresholdsPrach [5]. This test will verify the requirements in TS36.133 Clause 6.6A.2, Clause 6.6A.3 and Clause 7.20A2 in an AWGN model.

For this test a single NB-IoT cell is used. The test parameters are given in tables 13.3.2.3.5-1, 13.3.2.3.5-2 and 13.3.2.3.5-3. The UE shall perform timing pre-compensation before the initial NPRACH transmission using AT command-based test approach.

<< END OF CHANGES 7>>

<<< START OF CHANGES 8>>>

13.3.2.3.4.2	Test procedure

The test scenario comprises of 2 NB-IoT carriers with 1 Ncell.

The test procedures are the same as test procedures defined in 13.3.2.2.4.2, with step 4 and forward performed on the non-anchor carrier.

<< END OF CHANGES 8>>

<<< START OF CHANGES 9>>>

Table 13.3.2.3.5-1: nCell specific test parameters for HD-FDD contention based random access on non-achor carrier test for UE category NB1 Standalone mode in Enhanced Coverage

| Parameter | Unit | Value | Comments |
| --- | --- | --- | --- |
| NB-IOT operational mode |  | Standalone |  |
| BWchannel | kHz | 200 |  |
| NPDSCH parameters Note 2 |  | R.18 HD-FDD | As defined in TS36.133 A.3.1.5.3 |
| NPDCCH parameters Note 2 |  | R.30 HD-FDD | As defined in TS36.133 A.3.1.6.3 |
| NPBCH_RA | dB | -3 |  |
| NPBCH_RB | dB |  |  |
| NPSS_RA | dB |  |  |
| NSSS_RA | dB |  |  |
| NPDCCH_RA | dB |  |  |
| NPDCCH_RB | dB |  |  |
| NPDSCH_RA | dB |  |  |
| NPDSCH_RB | dB |  |  |
| NOCNG_RA Note 1 | dB |  |  |
| NOCNG_RB Note 1 | dB |  |  |
| DRX |  | OFF |  |
|  | dBm/15 kHz | -98+TT |  |
|  | dB | -12.5+TT |  |
| Note 3 | dB | -12.5 |  |
| NRSRP Note 3 | dBm/15 kHz | -110.5 |  |
| Io Note 3 | dBm/180 KHz | -86.97 |  |
| Propagation Condition | - | AWGN |  |
| Antenna Configuration |  | 1x1 |  |
| Note 1: NOCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The NPDSCH and NPDCCH reference measurement channels are used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: Es/Iot, NRSRP and Io level has been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |

<< END OF CHANGES 9>>

<<< START OF CHANGES 10>>>

Table 13.3.2.3.5-6: Test requirements for Te Timing Error Limit for HD-FDD contention based random access test for UE category NB1

| Downlink Bandwidth (MHz) | Te_ |
| --- | --- |
| 0.18 | 97*TS + TT |
| NOTE: TS is the basic timing unit defined in 3GPP TS 36.211 |  |

<< END OF CHANGES 10>>