3GPP TSG WG RAN5 Meeting #100	R5-236048r1

# Chicago, USA, 13 – 17 Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Text Proposal |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | N/A | rev | - | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding test case 6.4A.1 Frequency Error for category M1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc. , Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Adding test case 6.4A.1 Frequency Error for category M1 Implement proposal 1,2,3.2 and proposal 4 of R5-236027 in test procedure and initial condition description |  |  |  |  |  |  |  |  |
| Summary of change: |  | Complete test case for 6.4A.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR depends on R5-236027 TP analyses for frequency error test case in R5-237123 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: removed the changing of minimum requirement and keep align with core Spec, removed RAN4 dependency comments from coversheet |  |  |  |  |  |  |  |  |

<<Start of changes >>

6.4A.1	Frequency error for UE category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

6.4A.1.1	Test purpose

This test verifies the ability of both, the receiver and the transmitter, to process frequency correctly.

Receiver: to extract the correct frequency from the stimulus signal, offered by the System simulator, under ideal propagation conditions and low level.

Transmitter: to derive the correct modulated carrier frequency from the results, gained by the receiver.

6.4A.1.2	Test applicability

This test case applies to all types of NB-IoT FDD UE release 17 and forward of UE category M1 that support satellite access operation.

6.4A.1.3	Minimum conformance requirements

For category M1 UE, the basic measurement interval of modulated carrier frequency is 1 UL timeslot (0.5ms). The UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift based on received ephemeris information of the SAN in IE EphemerisInfo (TS 36.331 [6]), its own location and UL carrier frequency signalled to the UE by the SAN (according to TS36.300 [8] clause 16.14.2).

For category M1 FD-FDD UEs and for category M1 HD-FDD UEs with continuous uplink transmissions of duration ≤ 64 ms, the mean value of basic measurements of UE pre-compensated modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of one time slot (0.5 ms) compared with the ideally pre-compensated UL carrier frequency.

For category M1 HD-FDD UEs with continuous uplink transmissions of duration > 64 ms, the mean value of basic measurements of UE pre-compensated modulated carrier frequency shall be accurate within the limits in Table 6.4A.1-1 observed over a period of one time slot (0.5 ms) compared with ideally pre-compensated UL carrier frequency.

When a repetition period is configured on the uplink for which repetition period (R ) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3]. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission.

[NOTE:	The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift corresponding to the estimated Doppler frequency at the beginning of the transmission. For the test case, the location of the UE is explicitly provided to the UE from the test equipment.]

Requirement will be verified for at least two cases of which one has zero Doppler conditions

Table 6.4A.1.3-1: Frequency error requirement for HD-FDD UE category M1

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

The normative reference for this requirement is TS 36.102 [11] clause 6.4A.1.

6.4A.1.4	Test description

6.4A.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, channel bandwidths based on E-UTRA bands specified in sub-clause 5.2A.  All of these configurations shall be tested with applicable test parameters for each channel bandwidth are shown in table 6.4A.1.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.4A.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[12] subclause 4.1 |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS36.508 [12] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 36.508 [12] subclause 4.3.1 |  |  | 1.4MHz |  |
|  | Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |
| Ch BW | Modulation | RB allocation | Modulation | RB allocation |
| 1.4MHz | QPSK | 2 | QPSK | 2 |
| 1.4MHz | QPSK | 2 | QPSK | 2 |

1.	Connect the SS to the UE to the UE antenna connectors as shown in TS 36.508 [12] Annex A, Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508[12] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and DL Reference Measurement channels are set according to Table 6.4A.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

6.4A.1.4.2	Test procedure

1.	Test equipment shall emulate Zero Doppler conditions and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31 configuration in TS 36.508 [12]. Test system shall send same SIB31 information during the duration of each frequency error measurement

2.	Deactivate UE prediction of satellite trajectory and by AT commands or any other preconfigured means

3.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.4A.1.4.3.

4.	SS transmits PDSCH via MPDCCH DCI format 6-1A for C_RNTI to transmit the DL RMC according to Table 6.4A.1.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 6.4A.1.4.1-1, since the UE has no payload data to send, the UE transmit uplink MAC padding bits on the UL RMC

6.	Set the Downlink signal level to the appropriate REFSENS value defined in Table 7.3A.5-1. Send continuously uplink power control "up" commands to the UE in every uplink scheduling information to the UE so that the UE transmits at PUMAX level for the duration of the test.

7.	Measure the Frequency Error using Global In-Channel Tx-Test (Annex E) according to the UE’s declaration on the position of carrier centre frequency. For HDD-FD slots with transient periods are not under test. Half-duplex guard sub frame is not under test

8.	Repeat from test procedure steps 1-7 using maximum Doppler and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31 configuration in TS 36.508 [12]. Test system shall send same SIB31 information during the duration of each frequency error measurement

9.	Repeat from test procedure steps 1-7 using half of maximum Doppler and constant non-zero delay in service link for the SAN type supported by the UE (in case only one is supported) or for GSO (in case both SAN types are supported) by selecting the proper and fixed ephemeris snapshot from ephemeris file defined in TS 36.508 [12] clause 5.6.2. Common TA delay shall be configured as indicated in SIB31 configuration in TS 36.508 [12]. Test system shall send same SIB31 information during the duration of each frequency error measurement

10.	In case the UE supports both SAN types (GSO and NGSO), repeat test procedure steps 1-9 for NGSO (LEO-600)

6.4A.1.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 4.6 with the condition CEModeA and 5.6.2, the exceptions in subclause 7.3A.4.3.

6.4A.1.5	Test requirement

The 20 frequency error Δf results must fulfil the test requirement:

|Δf| ≤ (0.1 PPM + 15 Hz) (Carrier frequency >1 GHz)

|Δf| ≤ (0.2 PPM + 15 Hz) (Carrier frequency ≤1 GHz)

<< End of changes >>