3GPP TSG-RAN5 Meeting #101	R5-236066r1

# Chicago, US, 13th Nov 2023 – 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0991 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Defined MU and TT for 6.2D.1.1 and 6.2D.1.2 MOP FR2 UL MIMO tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MU and TT for FR2 MOP UL MIMO tests 6.2D.1.1 and 6.2D.1.2 are not defined. As previously agreed for FR2 MPR UL MIMO test, MU and TT can be the same as defined for the equivalent non UL MIMO tests 6.2.1.1 and 6.2.1.2. Those test cases have MU and TT already defined for PC1, PC3 and PC5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For 6.2D.1.1 and 6.2D.1.2, defined MU and TT in annex F same as defined for 6.2.1.1 and 6.2.1.2 respectively. Editor’s notes, minimum requirements and test requirements tables uppdated accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2D.1.1, 6.2D.1.2, F.1.2, F.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: -r1: -Reverted change in Table 6.2D.1.2.5-3b to minimize overlap with R5-237257. -In section 6.2D.1.1.5, updated values for FR2c aligned to latest proposal in R5-236064r2 covering corresponding single layer test case. |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

#### 6.2D.1.1	UE maximum output power - EIRP and TRP for UL MIMO

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- 	No test points are defined for 2-layer UL MIMO since there is no configuration satisfying MPR=0dB requirements in RAN4.

-	Measurement Uncertainties and Test Tolerances are FFS for power classes other than 1, 3 and 5.

-	The test case is incomplete for band n259.

-	Test Procedures for EIRP beam peak Extreme Conditions are FFS.

6.2D.1.1.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth for UL MIMO under the deployment scenarios where additional requirements are specified.

6.2D.1.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports UL MIMO.

6.2D.1.1.3	Minimum conformance requirements

6.2D.1.1.3.1	UE maximum output power for UL MIMO for power class 1

The following requirements define the maximum output power radiated by the PC1 UE. Requirements apply to UEs when configured for 2-layer transmission as well as when configured for single layer uplink full power transmission (ULFPTx), with configuration per clause 6.2D.1.0.

The minimum peak EIRP requirements are found in Table 6.2D.1.1.3.1-1 below. The period of measurement shall be at least one sub frame (1ms). The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle). Power class 1 UE is used for fixed wireless access (FWA).

Table 6.2D.1.1.3.1-1: UE minimum peak EIRP for UL MIMO for power class 1

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 40.0 |
| n258 | 40.0 |
| n260 | 38.0 |
| n261 | 40.0 |
| n262 | 34.2 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance. |  |

Table 6.2D.1.1.3.1-2: (void)

The maximum output power values for TRP and EIRP are found in Table 6.2D.1.1.3.1-3 below for UE with UL MIMO. The maximum allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode and EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2D.1.1.3.1-3: UE maximum output power limits for UL MIMO for power class 1

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 35 | 55 |
| n258 | 35 | 55 |
| n260 | 35 | 55 |
| n261 | 35 | 55 |
| n262 | 35 | 55 |

The minimum EIRP at the 85th percentile of the distribution of radiated power measured over the full sphere around the UE with UL MIMO is defined as the spherical coverage requirement and is found in Table 6.2D.1.1.3.1-4 below. The requirement is verified with the test metric of EIRP (Link=Spherical coverage grid, Meas=Link angle).

Table 6.2D.1.1.3.1-4: UE spherical coverage for UL MIMO for power class 1

| Operating band | Min EIRP at 85 %-tile CDF (dBm) |
| --- | --- |
| n257 | 32.0 |
| n258 | 32.0 |
| n260 | 30.0 |
| n261 | 32.0 |
| n262 | 26.0 |
| NOTE 1: Minimum EIRP at 85 %-tile CDF is defined as the lower limit without tolerance. |  |

6.2D.1.1.3.2	UE maximum output power for UL MIMO for power class 2

The following requirements define the maximum output power radiated by the PC2 UE. Requirements apply to UEs when configured for 2-layer transmission as well as when configured for single layer uplink full power transmission (ULFPTx), with configuration per clause 6.2D.1.0.

The minimum peak EIRP requirements are found in Table 6.2D.1.1.3.2-1 below. The period of measurement shall be at least one sub frame (1ms). The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2D.1.1.3.2-1: UE minimum peak EIRP for UL MIMO for power class 2

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 29 |
| n258 | 29 |
| n261 | 29 |
| n262 | 22.9 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance. NOTE 2: Min Peak EIRP refers to the total EIRP for the UL beams peaks. |  |

The maximum output power values for TRP and EIRP are found in Table 6.2D.1.1.3.2-2 below. The maximum allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode and EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2D.1.1.3.2-2: UE maximum output power limits for UL MIMO for power class 2

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 23 | 43 |
| n258 | 23 | 43 |
| n261 | 23 | 43 |
| n262 | 23 | 43 |

Table 6.2D.1.1.3.2-3: (void)

The minimum EIRP at the 60th percentile of the distribution of radiated power measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 6.2D.1.1.3.2-4 below. The requirement is verified with the test metric of EIRP (Link=Spherical coverage grid, Meas=Link angle).

Table 6.2D.1.1.3.2-4: UE spherical coverage for UL MIMO for power class 2

| Operating band | Min EIRP at 60 %-tile CDF (dBm) |
| --- | --- |
| n257 | 18.0 |
| n258 | 18.0 |
| n261 | 18.0 |
| n262 | 11.0 |
| NOTE 1: Minimum EIRP at 60 %-tile CDF is defined as the lower limit without tolerance |  |

6.2D.1.1.3.3	UE maximum output power for UL MIMO for power class 3

The following requirements define the maximum output power radiated by the PC3 UE.. Requirements apply to UEs when configured for 2-layer transmission as well as when configured for single layer uplink full power transmission (ULFPTx), with configuration per clause 6.2D.1.0.

The minimum peak EIRP requirements are found in Table 6.2D.1.1.3.3-1 below. The period of measurement shall be at least one sub frame (1 ms). The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2D.1.1.3.3-1: UE minimum peak EIRP for UL MIMO for power class 3

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 22.4 |
| n258 | 22.4 |
| n259 | 18.7 |
| n260 | 20.6 |
| n261 | 22.4 |
| n262 | 16.0 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance. NOTE 2: Min Peak EIRP refers to the total EIRP for the UL beams peaks. |  |

The maximum output power values for TRP and EIRP are found in Table 6.2D.1.1.3.3-2 below. The maximum allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode and EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2D.1.1.3.3-2: UE maximum output power limits for UL MIMO for power class 3

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 23 | 43 |
| n258 | 23 | 43 |
| n259 | 23 | 43 |
| n260 | 23 | 43 |
| n261 | 23 | 43 |
| n262 | 23 | 43 |

Table 6.2D.1.1.3.3-3: (void)

The minimum EIRP at the 50th percentile of the distribution of radiated power measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 6.2D.1.1.3.3-4 below. The requirement is verified with the test metric of EIRP (Link=spherical coverage grid, Meas=Link angle).

Table 6.2D.1.1.3.3-4: UE spherical coverage for UL MIMO for power class 3

| Operating band | Min EIRP at 50 %-tile CDF (dBm) |
| --- | --- |
| n257 | 11.5 |
| n258 | 11.5 |
| n259 | 5.8 |
| n260 | 8 |
| n261 | 11.5 |
| NOTE 1: Minimum EIRP at 50 %-tile CDF is defined as the lower limit without tolerance NOTE 2: The requirements in this table are only applicable for UE which supports single band in FR2 |  |

6.2D.1.1.3.4	UE maximum output power for UL MIMO for power class 4

The following requirements define the maximum output power radiated by the PC4 UE. Requirements apply to UEs configured for 2-layer transmission as well as UEs configured for single layer uplink full power transmission (ULFPTx), with configuration per clause 6.2D.1.0.

The minimum peak EIRP requirements are found in Table 6.2D.1.1.3.4-1 below. The period of measurement shall be at least one sub frame (1ms). The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2D.1.1.3.4-1: UE minimum peak EIRP for UL MIMO for power class 4

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 34 |
| n258 | 34 |
| n260 | 31 |
| n261 | 34 |
| n262 | 28.3 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance. NOTE 2: Min Peak EIRP refers to the total EIRP for the UL beams peaks. |  |

The maximum output power values for TRP and EIRP are found in Table 6.2D.1.1.3.4-2 below. The maximum allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode and EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2D.1.1.3.4-2: UE maximum output power limits for UL MIMO for power class 4

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 23 | 43 |
| n258 | 23 | 43 |
| n260 | 23 | 43 |
| n261 | 23 | 43 |
| n262 | 23 | 43 |

Table 6.2D.1.1.3.4-3: (void)

The minimum EIRP at the 20th percentile of the distribution of radiated power measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 6.2D.1.1.3.4-4 below. The requirement is verified with the test metric of EIRP (Link=Spherical coverage grid, Meas=Link angle).

Table 6.2D.1.1.3.4-4: UE spherical coverage for UL MIMO for power class 4

| Operating band | Min EIRP at 20 %-tile CDF (dBm) |
| --- | --- |
| n257 | 25 |
| n258 | 25 |
| n260 | 19 |
| n261 | 25 |
| n262 | 16.2 |
| NOTE 1: Minimum EIRP at 20 %-tile CDF is defined as the lower limit without tolerance |  |

The normative reference for this requirement is TS 38.101-2 [3] clause 6.2.1.

6.2D.1.1.3.6	UE maximum output power for UL MIMO for power class 5

The following requirements define the maximum output power radiated by the PC4 UE. Requirements apply to UEs configured for 2-layer transmission as well as UEs configured for single layer uplink full power transmission (ULFPTx), with configuration per clause 6.2D.1.0.

The minimum peak EIRP requirements are found in Table Table 6.2D.1.1.3.5-1 below. The period of measurement shall be at least one sub frame (1ms). The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle). Power class 5 UE is used for fixed wireless access (FWA).

Table 6.2D.1.1.3.5-1: UE minimum peak EIRP for UL MIMO for power class 5

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 30 |
| n258 | 30.4 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance |  |

The maximum output power values for TRP and EIRP are found in Table Table 6.2D.1.1.3.5-2 below for UE with UL MIMO. The maximum allowed EIRP is derived from regulatory requirements. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode and EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2D.1.1.3.5-2: UE maximum output power limits for UL MIMO for power class 5

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 23 | 43 |
| n258 | 23 | 43 |

The minimum EIRP at the 85th percentile of the distribution of radiated power measured over the full sphere around the UE with UL MIMO is defined as the spherical coverage requirement and is found in Table Table 6.2D.1.1.3.5-3 below. The requirement is verified with the test metric of EIRP (Link=Spherical coverage grid, Meas=Link angle).

Table 6.2D.1.1.3.5-3: UE spherical coverage for UL MIMO for power class 5

| Operating band | Min EIRP at 85 %-tile CDF (dBm) |
| --- | --- |
| n257 | 22 |
| n258 | 22.4 |
| NOTE 1: Minimum EIRP at 85 %-tile CDF is defined as the lower limit without tolerance |  |

6.2D.1.1.3.6	UE maximum output power for UL MIMO for power class 6

The following requirements define the maximum output power radiated by the PC6 UE. Requirements apply to UEs configured for 2-layer transmission as well as UEs configured for single layer uplink full power transmission (ULFPTx), with configuration per clause 6.2D.1.0.

The minimum peak EIRP requirements are found in Table 6.2D.1.1.3.6-1 below. The period of measurement shall be at least one sub frame (1ms). The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2D. 1.1.3.6-1: UE minimum peak EIRP for UL MIMO for power class 6

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 30 |
| n258 | 30.4 |
| n261 | 30 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance |  |

The maximum output power values for TRP and EIRP are found in Table 6.2D. 1.1.3.6-2 below for UE with UL MIMO. The maximum allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode and EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2D. 1.1.3.6-2: UE maximum output power limits for UL MIMO for power class 6

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 23 | 43 |
| n258 | 23 | 43 |
| n261 | 23 | 43 |

The minimum EIRP measured over the spherical coverage evaluation areas is defined as the spherical coverage requirement and is found in Table 6.2D. 1.1.3.6-3 below. UE spherical coverage evaluation areas are found in Table 6.2.1.1.3.6-3a in clause 6.2.1.1.3.6, by consisting of Area-1 and Area-2, in the reference coordinate system in Annex J.1. The requirement is verified with the test metric of EIRP (Link= Spherical coverage grid, Meas=Link angle).

Table 6.2D. 1.1.3.6-3: UE spherical coverage for UL MIMO for power class 6

| Operating band | Min EIRP over UE spherical coverage evaluation areas (dBm) |
| --- | --- |
| n257 | 20 |
| n258 | 20.4 |
| n261 | 20 |
| NOTE 1: Minimum EIRP over UE spherical coverage evaluation areas is defined as the lower limit without tolerance NOTE 2: The requirements in this table are verified only under normal temperature conditions as defined in Annex E.2.1. NOTE 3: The requirements in this table are applicable to FR2 PC6 UE with the network signalling [highSpeedMeasFlag-r17] configured as [set2]. |  |

6.2D.1.1.4	Test description

6.2D.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.2D.1.1.4.1-1 and Table 6.2D.1.1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2D.1.1.4.1-1: Test Configuration Table for 2-layer UL MIMO

NOTE:	No test points are defined since there is no configuration satisfying MPR=0dB requirements in RAN4.

Table 6.2D.1.1.4.1-2: Test Configuration Table for uplink full power transmission (ULFPTx)

| Default Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid Range, High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest, 100 MHz, Highest |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120 kHz |  |  |
| Test Parameters |  |  |  |  |  |  |  |
| Test ID | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |  |
|  |  | Default | N/A | Modulation |  | RB allocation (NOTE 1) |  |
| 1 | 50 |  |  | DFT-s-OFDM QPSK |  | Inner_Full for PC2, PC3 |  |
| 2 | 100 |  |  |  |  | , PC4 and PC6 |  |
| 3 | 200 |  |  |  |  | Inner_Full_Region1 for |  |
| 4 | 400 |  |  |  |  | PC1 |  |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3, PC4 and PC6 or Table 6.1-2 for PC1. |  |  |  |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement channels are set according to Table 6.2D.1.1.4.1-2.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.2D.1.1.4.3

6.2D.1.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2D.1.1.4.1-2. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Messages to configure the appropriate uplink modulation in section 6.2D.1.1.4.3.

2.	Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

3.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 msec starting from the first TPC command in this step to ensure that the UE transmits at its maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

4.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

5.	Measure UE EIRP in the Tx beam peak direction in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Tables 6.2D.1.1.5-1 to 6.2D.1.1.5-4. EIRP test procedure is defined in Annex K.1.3. The measuring duration is one active uplink subframe. EIRP is calculated considering both polarizations, theta and phi.

6.	Measure TRP of the transmitted signal for the assigned NR channel with a rectangular measurement filter with bandwidths according to Table 6.5.2.3.5-1. Total radiated power is measured according to TRP measurement procedure defined in Annex K.1.7 and measurement grid specified in Annex M.4. TRP is calculated considering both polarizations, theta and phi.

7.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

8.	If UE supports ULFPTx, repeat test steps 1~7 with UL RMC according to Table 6.2D.1.1.4.1-2. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1, ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability. Message contents are according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.

6.2D.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.

6.2D.1.1.5	Test requirement

The EIRP derived in step 4, TRP derived in step 5, and EIRP and TRP derived in step 8 shall not exceed the values specified in Table 6.2D.1.1.5-1 to Table 6.2D.1.1.5-4.

Table 6.2D.1.1.5-1: UE maximum output test requirements for power class 1

| Operating band | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| n257 | 35+TT | 55 | 40.0-TT |
| n258 | 35+TT | 55 | 40.0-TT |
| n260 | 35+TT | 55 | 38.0-TT |
| n261 | 35+TT | 55 | 40.0-TT |

Table 6.2D.1.1.5-1a: Test Tolerance (Max TRP for Power class 1)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | 2.78 dB, NTC 2.94 dB, ETC | 2.87 dB, NTC 3.03 dB, ETC |

Table 6.2D.1.1.5-1b: Test Tolerance (Min peak EIRP for Power class 1)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | 3.12 dB, NTC 3.28 dB, ETC | 3.12 dB, NTC 3.28 dB, ETC |

Table 6.2D.1.1.5-2: UE maximum output test requirements for power class 2

| Operating band | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| n257 | 23+TT | 43 | 29-TT |
| n258 | 23+TT | 43 | 29-TT |
| n260 |  |  |  |
| n261 | 23+TT | 43 | 29-TT |
| n262 | 23+TT | 43 | 22.9-TT |

Table 6.2D.1.1.5-3: UE maximum output test requirements for power class 3

| Operating band | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| n257 | 23+TT | 43 | 22.4-TT |
| n258 | 23+TT | 43 | 22.4-TT |
| n260 | 23+TT | 43 | 20.6-TT |
| n261 | 23+TT | 43 | 22.4-TT |
| n262 | 23+TT | 43 | 16.0-TT |

Table 6.2D.1.1.5-3a: Test Tolerance (Max TRP for Power class 3)

| Test Metric | FR2a | FR2b | FR2c |
| --- | --- | --- | --- |
| Max device size ≤ 30 cm | 2.77 dB, NTC 2.91 dB, ETC | 2.89 dB, NTC 3.04 dB, ETC | 3.70 dB, NTC TBD dB, ETC |

Table 6.2D.1.1.5-3b: Test Tolerance (Min peak EIRP for Power class 3)

| Test Metric | FR2a | FR2b | FR2c |
| --- | --- | --- | --- |
| Max device size ≤ 30 cm | 2.99 dB, NTC 3.15 dB, ETC | 2.99 dB, NTC 3.15 dB, ETC | 3.80 dB, NTC 3.89 dB, ETC |

Table 6.2D.1.1.5-4: UE maximum output power test requirements for power class 4

| Operating band | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| n257 | 23+TT | 43 | 34-TT |
| n258 | 23+TT | 43 | 34-TT |
| n260 | 23+TT | 43 | 31-TT |
| n261 | 23+TT | 43 | 34-TT |

Table 6.2D.1.1.5-5:

UE maximum output power test requirements for power class 5

| Operating band | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| n257 | 23+TT | 43 | 30.0-TT-MBP,n |
| n258 | 23+TT | 43 | 30.4-TT-MBP,n |
| Note 1: MBP,n = 0 for single band UE. For multi-band UEs, MBP,n is defined in table 6.2.1.1.3.5-4. |  |  |  |

Table 6.2D.1.1.5-5a: Test Tolerance (Max TRP for Power class 5)

| Test Metric | FR2a |
| --- | --- |
| Max device size ≤ 30 cm | 2.78 dB, NTC 2.94 dB, ETC |

Table 6.2D.1.1.5-5b: Test Tolerance (Min peak EIRP for Power class 5)

| Test Metric | FR2a |
| --- | --- |
| Max device size ≤ 30 cm | 3.12 dB, NTC 3.28 dB, ETC |

Table 6.2D.1.1.5-6: UE maximum output power test requirements for power class 6

| Operating band | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| n257 | 23+TT | 43 | 30+TT |
| n258 | 23+TT | 43 | 30.4+TT |
| n261 | 23+TT | 43 | 30+TT |

#### 6.2D.1.2	UE maximum output power - Spherical coverage for UL MIMO

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- 	No test points are defined for 2-layer UL MIMO since there is no configuration satisfying MPR=0dB requirements in RAN4.

- 	Measurement Uncertainties and Test Tolerances are FFS for power classes other than 1, 3 and 5.

- 	The test case is incomplete for band n259.

6.2D.1.2.1	Test purpose

To verify that the spatial coverage of the UE in expected directions is acceptable.

6.2D.1.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support beam correspondence without UL beam sweeping.

6.2D.1.2.3	Minimum conformance requirements

Minimum conformance requirements are defined in clause 6.2D.1.1.3.

6.2D.1.2.4	Test description

6.2D.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.2D.1.2.4.1-1 and Table 6.2D.1.2.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2D.1.2.4.1-1: Test Configuration Table for 2-layer UL MIMO

NOTE:	No test points are defined since there is no configuration satisfying MPR=0dB requirements in RAN4.

Table 6.2D.1.2.4.1-2: Test Configuration Table for uplink full power transmission (ULFPTx)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid Range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120 kHz |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Default | N/A | Modulation |  | RB allocation (NOTE 1) |
| 1 | 50 |  |  | DFT-s-OFDM QPSK |  | Inner_Full for PC2, PC3 |
| 2 | 100 |  |  |  |  | and PC4 |
| 3 | 200 |  |  |  |  | Inner_Full_Region1 for |
| 4 | 400 |  |  |  |  | PC1 |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. |  |  |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.2 and TS 38.508-1 [10] subclause 5.2.1.1.1, and uplink signals according to Annex G.0, G.1 and G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.2D.1.2.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.2D.1.2.4.3

6.2D.1.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2D.1.2.4.1-2. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Messages to configure the appropriate uplink modulation in section 6.2D.1.2.4.3.

2.	Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

3.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 msec to ensure that the UE transmits at its maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

4.	Through its beam correspondence procedure, DUT refines its TX beam toward that direction depending on DUT’s beam correspondence capability which shall match OEM declaration:

4a 	If the DUT’s beam correspondence capability beamCorrespondenceWithoutUL-BeamSweeping is supported, then DUT autonomously chooses the corresponding TX beam for PUSCH transmission using downlink reference signals to transmit in the direction of the incoming DL signal, which is based on beam correspondence without relying on UL beam sweeping;

4b 	If the DUT’s beam correspondence capability beamCorrespondenceWithoutUL-BeamSweeping is not present, then DUT chooses the TX beam for PUSCH transmission which is based on beam correspondence with relying on both DL measurements on downlink reference signals and network-assisted uplink beam sweeping:

4b.1) DUT uses downlink reference signals to select proper RX beam and uses autonomous beam correspondence to select the TX beam.

4b.2) SS configures M=8 SRS resources to DUT, with the field spatialRelationInfo omitted and the field usage set as ‘beamManagement’. In case DUT supports less than 8 SRS resources, SS configures the number of SRS resources according to the maximum number of SRS resources indicated by UE capability signalling. Additionally, for codebook based PUSCH transmission, SS configures a semi-persistent SRS resource set with the field usage as 'codebook'.

4b.3) Based on the TX beam autonomously selected by DUT, DUT chooses TX beams to transmit SRS-resources configured by SS.

4b.4) Based on measurement of the received beamManagement SRS, SS chooses the best SRS beam and, if needed, updates the spatial relation information between the semi-persistent codebook SRS resources and the SS selected beamManagement SRS resource in the activation MAC CE of the semi-persistent SRS resource. The SS indicates in the SRS Resource Indicator (SRI) field in the scheduling grant for PUSCH, if present, the SRS resource within the semi-persistent SRS resource set whose spatial relation is linked to the best detected SRS beam.

4b.5) DUT transmits PUSCH corresponding to the SRS resource indicated by the SRI.

5. Measure UE EIRP value for each grid point according to the EIRP spherical coverage procedure defined in Annex K.1.5.0, and obtain a cumulative distribution function (CDF) of all EIRP dBm values. Alternatively, UE EIRP measurement for each grid point could be done according to Tx Fast spherical coverage procedure defined in Annex K.1.5.1. After a rotation, allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for UE to find the best beam to use. The measuring duration is one active uplink subframe. EIRP is calculated considering both polarizations, theta and phi.

6.	Identify the EIRP dBm value corresponding to %-tile (UE power class dependent) value in the applicable test requirement table in section 6.2D.1.2.5.

7.	If UE supports ULFPTx, repeat test steps 1~6 with UL RMC according to Table 6.2D.1.2.4.1-2. The PDCCH DCI format 0_1 is specified with the condition ULFPTx_Mode1, ULFPTx_Mode2 or ULFPTx_ModeFull in 38.508-1 [5] subclause 4.3.6.1.1.2 depending on UE reported capability. Message contents are according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.

6.2D.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.

6.2D.1.2.5	Test requirement

The defined %-tile EIRP in measurement distribution derived in step 5 and step 6 shall exceed the values specified in Table 6.2D.1.2.5-1 to Table 6.2D.1.2.5-6.

Table 6.2D.1.2.5-1: UE spherical coverage for power class 1

| Operating band | Min EIRP at 85%-tile CDF (dBm) |
| --- | --- |
| n257 | 32.0-TT |
| n258 | 32.0-TT |
| n260 | 30.0-TT |
| n261 | 32.0-TT |
| n262 | 26.0-TT |

Table 6.2D.1.2.5-1a: Test Tolerance (UE spherical coverage for Power class 1)

| Test Metric | FR2a | FR2b |
| --- | --- | --- |
| Max device size ≤ 30 cm | 2.69 dB | 2.69 dB |

Table 6.2D.1.2.5-2: UE spherical coverage for power class 2

| Operating band | Min EIRP at 60%-tile CDF (dBm) |
| --- | --- |
| n257 | 18.0-TT |
| n258 | 18.0-TT |
| n260 |  |
| n261 | 18.0-TT |
| n262 | 11.0-TT |

Table 6.2D.1.2.5-3: UE spherical coverage for power class 3

| Operating band | Min EIRP at 50t%-tile CDF (dBm) |
| --- | --- |
| n257 | 11.5-TT |
| n258 | 11.5-TT |
| n259 | 5.8-TT |
| n260 | 8-TT |
| n261 | 11.5-TT |

Table 6.2D.1.2.5-3b: Test Tolerance (UE spherical coverage for Power class 3)

FFS

Table 6.2D.1.2.5-4: UE spherical coverage for power class 4

| Operating band | Min EIRP at 20%-tile CDF (dBm) |
| --- | --- |
| n257 | 25-TT |
| n258 | 25-TT |
| n260 | 19-TT |
| n261 | 25-TT |
| n262 | 16.2-TT |

Table 6.2D.1.2.5-5: UE spherical coverage for power class 5

| Operating band | Min EIRP at 85 %-tile CDF (dBm) |
| --- | --- |
| n257 | 22-TT |
| n258 | 22.4-TT |

Table 6.2D.1.2.5-5a: Test Tolerance (UE spherical coverage for Power class 5)

| Test Metric | FR2a |
| --- | --- |
| Max device size ≤ 30 cm | 2.69 dB |

Table 6.2D.1.2.5-6: UE spherical coverage for power class 6

| Operating band | Min EIRP over UE spherical coverage evaluation areas (dBm) |
| --- | --- |
| n257 | 20-TT |
| n258 | 20.4-TT |
| n261 | 20-TT |

## <<< Skip unchanged sections >>>

## F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty (MTSU) for transmitter tests

| Sub clause | Maximum Test System Uncertainty | Derivation of MTSU |
| --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) | PC3 Minimum peak EIRP, Max EIRP Max Device size ≤ 30 cm ±5.08 dB (FR2a, NTC testing) ±5.28 dB (FR2b, NTC testing) [±5.91] dB (FR2c, NTC testing) ±5.35 dB (FR2a, ETC testing) ±5.55 dB (FR2b, ETC testing) TBD (FR2c, ETC testing) PC1 Minimum peak EIRP, Max EIRP Max Device size ≤ 30 cm ±5.33 dB (FR2a, NTC testing) ±5.40 dB (FR2b, NTC testing) ±5.60 dB (FR2a, ETC testing) ±5.67 dB (FR2b, ETC testing) PC5 Minimum peak EIRP, Max EIRP Max Device size ≤ 30 cm ±5.33 dB (FR2a, NTC testing) ±5.60 dB (FR2a, ETC testing) | MTSU = 1.00 x MU (from Table B.3-1 in TR 38.903) |
| 6.2.1.1 UE maximum output power (TRP) | PC3 Max TRP Max Device size ≤ 30 cm ±4.61 dB (FR2a, NTC testing) ±4.81 dB (FR2b, NTC testing) [±5.44] dB (FR2c, NTC testing) ±4.85 dB (FR2a, ETC testing) ±5.07 dB (FR2b, ETC testing) TBD (FR2c, ETC testing) PC1 Max TRP Max Device size ≤ 30 cm ±4.64 dB (FR2a, NTC testing) ± 4.78 dB (FR2b, NTC testing) ± 4.90 dB (FR2a, ETC testing) ± 5.04 dB (FR2b, ETC testing) PC5 Max TRP Max Device size ≤ 30 cm ±4.64 dB (FR2a, NTC testing) ± 4.90 dB (FR2a, ETC testing) | MTSU = 1.00 x MU (from Table B.3-2 in TR 38.903) |
| 6.2.1.1_1 UE maximum output power – EIRP (Rel-16 and forward) | Same as 6.2.1.1 |  |
| 6.2.1.2 UE maximum output power (Spherical coverage) | PC3 Max Device size ≤ 30 cm ±4.78 dB (FR2a) ±5.38 dB (FR2b) TBD (FR2c) PC1 Max Device size ≤ 30 cm ±4.69 dB (FR2a) ±4.84 dB (FR2b) | MTSU = 1.00 x MU (from Table B.3-3 in TR 38.903) |
| 6.2.1.2_1 UE maximum output power – Spherical coverage (Rel16 and forward) | Same as 6.2.1.2 |  |
| 6.2.2 UE maximum output power reduction | PC3 Max Device size ≤ 30 cm ±5.11 dB (FR2a, NTC testing) ±5.29 dB (FR2b, NTC testing) ±5.38 dB (FR2a, ETC testing) ±5.56 dB (FR2b, ETC testing) PC1 Max Device size ≤ 30 cm ±5.33 dB (FR2a, NTC testing) ±5.50 dB (FR2b, NTC testing) ±5.60 dB (FR2a, ETC testing) ±5.77 dB (FR2b, ETC testing) PC5 Max Device size ≤ 30 cm ±5.33 dB (FR2a, NTC testing) ±5.60 dB (FR2a, ETC testing) | MTSU = 1.00 x MU (from Table B.4-1 in TR 38.903) |
| 6.2.2_1 UE maximum output power reduction enhancements | Same as 6.2.2 for FR2a, FR2b PC3 Max Device size ≤ 30 cm TBD (FR2c, NTC testing) TBD (FR2c, ETC testing) | MTSU = 1.00 x MU (from Table B.4-1 in TR 38.903) |
| 6.2.3 UE maximum output power with additional requirements | Same as 6.2.2 |  |
| 6.2.4 Configured transmitted power | TBD |  |
| 6.2.4_1 Configured transmitted power with Power Boost | Same as 6.2.1.1 |  |
| 6.2A.1.1.1 UE maximum output power - EIRP and TRP for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.1.2 UE maximum output power - EIRP and TRP for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.1.3 UE maximum output power - EIRP and TRP for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.1.4 UE maximum output power - EIRP and TRP for CA (5UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.1.5 UE maximum output power - EIRP and TRP for CA (6UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.1.6 UE maximum output power - EIRP and TRP for CA (7UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.1.7 UE maximum output power - EIRP and TRP for CA (8UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.2.1 Spherical coverage for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.2.2 Spherical coverage for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.2.3 Spherical coverage for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.2.4 Spherical coverage for CA (5UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.2.5 Spherical coverage for CA (6UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.2.6 Spherical coverage for CA (7UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.2.7 Spherical coverage for CA (8UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.2.1 UE maximum output power reduction for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD | MTSU = 1.00 x MU (from Table B.4-1 in TR 38.903) |
| 6.2A.2.2 UE maximum output power reduction for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.2.3 UE maximum output power reduction for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.2.4 UE maximum output power reduction for CA (5UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.2.5 UE maximum output power reduction for CA (6UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.2.6 UE maximum output power reduction for CA (7UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.2.7 UE maximum output power reduction for CA (8UL CA) | Intra-band contiguous CA TBD |  |
| 6.2D.1.1 UE maximum output power (EIRP) for UL MIMO | Same as 6.2.1.1 (EIRP) |  |
| 6.2D.1.1 UE maximum output power (TRP) for UL MIMO | Same as 6.2.1.1 (TRP) |  |
| 6.2D.1.2 UE maximum output power (Spherical coverage) for UL MIMO | Same as 6.2.1.2 |  |
| 6.2D.2 UE maximum output power reduction for UL MIMO | Same as 6.2.2 |  |
| 6.2D.3 UE maximum output power with additional requirements for UL MIMO | Same as 6.2.3 |  |

## <<< Skip unchanged sections >>>

## F.3.2	Measurement of transmitter

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Influence of noise is subtracted from MTSU before calculating the TT for lower limit Tx test cases.

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) | PC3 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 2.99 dB (FR2a, NTC) 2.99 dB (FR2b, NTC) [3.37]dB (FR2c, NTC) 3.15 dB (FR2a, ETC) 3.15 dB (FR2b, ETC) TBD (FR2c, ETC) PC1 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 3.12 dB (FR2a, NTC) 3.12 dB (FR2b, NTC) 3.28 dB (FR2a, ETC) 3.28 dB (FR2b, ETC) PC5 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 3.12 dB (FR2a, NTC) Max EIRP 0 dB | PC3 Minimum peak EIRP TT = 0.60 x (MTSUIFF - 0.1) (FR2a) TT = 0.60 x (MTSUIFF - 0.3) (FR2b) TT = 0.60 x (MTSUIFF - 0.3) (FR2c) PC1 Minimum peak EIRP TT = 0.60 x (MTSUIFF – 0.13) (FR2a) TT = 0.60 x (MTSUIFF – 0.20) (FR2b) PC5 Minimum peak EIRP TT = 0.60 x (MTSUIFF – 0.13) (FR2a) |
| 6.2.1.1 UE maximum output power (TRP) | PC3 Max TRP IFF (Max Device size ≤ 30 cm) 2.77 dB (FR2a, NTC) 2.89 dB (FR2b, NTC) [3.26] dB (FR2c, NTC) 2.91 dB (FR2a, ETC) 3.04 dB (FR2b, ETC) TBD (FR2c, ETC) PC1 Max TRP IFF (Max Device size ≤ 30 cm) 2.78 dB (FR2a, NTC) 2.87 dB (FR2b, NTC) 2.94 dB (FR2a, ETC) 3.03 dB (FR2b, ETC) PC5 Max TRP IFF (Max Device size ≤ 30 cm) 2.78 dB (FR2a, NTC) 2.94 dB (FR2a, ETC) | Max TRP TT = 0.60 x MTSUIFF |
| 6.2.1.1_1 UE maximum output power – EIRP (Rel-16 and forward) | Same as 6.2.1.1 |  |
| 6.2.1.2 UE maximum output power (Spherical coverage) | PC1 IFF (Max Device size ≤ 30 cm) 2.69 dB (FR2a) 2.69 dB (FR2b) PC2 TBD PC3 IFF (Max Device size ≤ 30 cm) 2.69 dB (FR2a) 2.69 dB (FR2b) TBD (FR2c) PC4 TBD PC5 IFF (Max Device size ≤ 30 cm) 2.69 dB (FR2a) | PC3 TT = 0.60 x (MTSUIFF - 0.3) (FR2a) TT = 0.60 x (MTSUIFF - 0.9) (FR2b) PC1 TT = 0.60 x (MTSUIFF - 0.20) (FR2a) TT = 0.60 x (MTSUIFF - 0.35) (FR2b) PC5 TT = 0.60 x (MTSUIFF - 0.20) (FR2a) |
| 6.2.1.2_1 UE maximum output power – Spherical coverage (Rel16 and forward) | Same as 6.2.1.2 |  |
| 6.2.2 UE maximum output power reduction | PC3 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 3.24 dB (FR2a, NTC) 3.24 dB (FR2b, NTC) 3.41 dB (FR2a, ETC) 3.41 dB (FR2b, ETC) PC1 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 3.38 dB (FR2a, NTC) 3.38 dB (FR2b, NTC) 3.56 dB (FR2a, ETC) 3.56 dB (FR2b, ETC) PC5 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 3.38 dB (FR2a, NTC) 3.56 dB (FR2a, ETC) | Minimum peak EIRP PC3 TT = 0.65 x (MTSUIFF - 0.13) (FR2a) TT = 0.65 x (MTSUIFF - 0.31) (FR2b) PC1 TT = 0.65 x (MTSUIFF - 0.13) (FR2a) TT = 0.65 x (MTSUIFF - 0.3) (FR2b) PC5 TT = 0.65 x (MTSUIFF - 0.13) (FR2a) |
| 6.2.2_1 UE maximum output power reduction enhancements | Same as 6.2.2 for FR2a, FR2b PC3 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) TBD (FR2c, NTC) TBD (FR2c, ETC) |  |
| 6.2.3 UE maximum output power with additional requirements | Same as 6.2.2 |  |
| 6.2.4 Configured transmitted power | TBD |  |
| 6.2.4_1 Configured transmitted power with Power Boost | Same as 6.2.1.1 |  |
| 6.2A.1.1.1 UE maximum output power - EIRP and TRP for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous TBD |  |
| 6.2A.1.1.2 UE maximum output power - EIRP and TRP for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous TBD |  |
| 6.2A.1.1.3 UE maximum output power - EIRP and TRP for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous TBD |  |
| 6.2A.1.1.4 UE maximum output power - EIRP and TRP for CA (5UL CA) | Intra-band contiguous CA, Intra-band non-contiguous CA TBD |  |
| 6.2A.1.1.5 UE maximum output power - EIRP and TRP for CA (6UL CA) | Intra-band contiguous CA, Intra-band non-contiguous CA TBD |  |
| 6.2A.1.1.6 UE maximum output power - EIRP and TRP for CA (7UL CA) | Intra-band contiguous CA, Intra-band non-contiguous CA TBD |  |
| 6.2A.1.1.7 UE maximum output power - EIRP and TRP for CA (8UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.2.1 Spherical coverage for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.2.2 Spherical coverage for CA (3UL CA) | Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.2.3 Spherical coverage for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.2.4 Spherical coverage for CA (5UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.2.5 Spherical coverage for CA (6UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.2.6 Spherical coverage for CA (7UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.2.7 Spherical coverage for CA (8UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.2.1 UE maximum output power reduction for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.2.2 UE maximum output power reduction for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.2.3 UE maximum output power reduction for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.2.4 UE maximum output power reduction for CA (5UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.2.5 UE maximum output power reduction for CA (6UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.2.6 UE maximum output power reduction for CA (7UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.2.7 UE maximum output power reduction for CA (8UL CA) | Intra-band contiguous CA TBD |  |
| 6.2D.1.1 UE maximum output power (EIRP) for UL MIMO | Same as 6.2.1.1 (EIRP) |  |
| 6.2D.1.1 UE maximum output power (TRP) for UL MIMO | Same as 6.2.1.1 (TRP) |  |
| 6.2D.1.2 UE maximum output power (Spherical coverage) for UL MIMO | Same as 6.2.1.2 |  |
| 6.2D.2 UE maximum output power reduction for UL MIMO | Same as 6.2.2 |  |
| 6.2D.3 UE maximum output power with additional requirements for UL MIMO | Same as 6.2.3 |  |

## <<< END OF CHANGES >>>