3GPP TSG-RAN5 Meeting #101	R5-236089r1

# Chicago, US, 13 - 17 Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2664 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification to CG-SDT RRM test case for FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NR FR2 CG-SDT RRM test case needs to be completed |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removed some of the editor’s notes - Revised the Test Description and Test Requirements sections to align with RAN4 CR R4-2317398 (endorsed in RAN4#108-bis meeting). - Added Message Contents. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Existing NR FR2 CG-SDT RRM test case remains incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 reply LS to RAN5 in R5-236042 (R4-2317399) Depending on the following RAN4 CR: - R4-2317398 (Draft CR endorsed in RAN4#108-bis) - R4-2321012 (CR 3946 in RAN4#109). Cat A R4-2321013 (CR 3947) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: RAN4 dependecy updated in the cover sheet |  |  |  |  |  |  |  |  |

-------- Start of changes --------

#### 7.2.1.1	TA Validation for CG-SDT in FR2

Editor’s note: This test case is incomplete. The following aspects are remaining

- Some parameters in brackets- TT analysis is missing

- Annex F

7.2.1.1.1	Test Purpose

The purpose of this test is to verify that the UE correctly performs TA validation for CG-SDT. This test will partly verify the TA validation requirements in clause 5.5.3 of TS 38.133 [6].

7.2.1.1.2	Test Applicability

This test applies to all types of NR UE from Release 17 onwards that supports transmission of data and/or signalling over allowed radio bearers in RRC_INACTIVE state via configured grant type 1 (i.e., CG-SDT), as specified in TS 38.331 [13].

7.2.1.1.3	Minimum Conformance Requirements

The minimum conformance requirements are specified in clause 7.2.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.2.1.1.

7.2.1.1.4	Test Description

7.2.1.1.4.1	Initial conditions

This test can be run in the configuration defined in Table 7.2.1.1.4.1-1.

Table 7.2.1.1.4.1-1: Supported test configurations for FR2 PCell

| Test Case ID | Test Config Index | Description |
| --- | --- | --- |
| 7.2.1.1-1 | 1 | TDD, SSB SCS 120 KHz, data SCS 120KHz, BW 100 MHz |

Configure the test equipment and the DUT according to the parameters in Table 7.2.1.1.4.1-2.

Table 7.2.1.1.4.1-2: Initial conditions for TA Validation for CG-SDT in FR2

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration from Table 7.2.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1-1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 7.2.1.1.4.1-3.

2.	Message contents are defined in clause 7.2.1.1.4.3.

3. There is only one NR Cell (Cell 1) specified in the test, configured as PCell in FR2. The Cell 1 is configured according to the settings in Annex C.1.1 and C.1.2.

Table 7.2.1.1.4.1-3: General test parameters for TA validation for CG-SDT in FR2

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode | Config 1 |  | TDD |
| BWchannel | Config 1 | MHz | 100: NRB,c = 66 |
| DL initial BWP configuration | Config 1 |  | DLBWP.0.1 |
| UL initial BWP configuration | Config 1 |  | ULBWP.0.1 |
| TDD Configuration | Config 1 |  | TDDConf.3.1 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.3.1 DD |
| SSB Configuration | Config 1 |  | SSB.1 FR2 |
| SMTC Configuration | Config 1 |  | SMTC.X |
| PDSCH/PDCCH subcarrier spacing | Config 1 |  | 120 kHz |
| PRACH Configuration | Config 1 |  | Table A.3.8.3.4 [6] |
| OCNG parameters |  |  | OP.5 |
| CP length |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |
| DRX |  | s | 0.64 |
| cg-SDT-RSRP-ThresholdSSB |  | dBm | -110 |
| cg-SDT-RSRP-ChangeThreshold |  | dB | [8] |
| cg-SDT-TimeAlignmentTime |  |  | infinity |
| CG-SDT resource period |  | ms | 320 |
| T1 |  | s | [0.8] |
| T2 |  | s | [0.96] |
| T3 |  | s | [3.04] |
| T4 |  | s | [1.54] |
| T5 |  | s | [1,76] |
| T6 |  | s | [4.58] |

7.2.1.1.4.2	Test procedure

The test consists of a single cell (Cell 1), which is the active NR cell configured as PCell in FR2 and includes two sub-tests. Sub-test#1 is for testing valid TA where UE can initiate CG-SDT transmission and Sub-test#2 is for testing invalid TA where UE does not initiate CG-SDT transmission. Subtest#2 is only tested if Sub-test#1 is passed. For each sub-test, the DUT is configured with CG-SDT configurations when entering RRC Inactive state. Sub-test#1 consists of four successive time periods, with time duration of T1, T2, T3, and T4 respectively. Sub-test#2 consists of two successive time periods, with time duration of T5 and T6 repectively. Figure 7.2.1.1.4.2-1 shows the variation of the RSRP level in the active cell  over the duration of Sub-test#1, and Figure 7.2.1.1.4.2-2 shows the variation of the RSRP level over the duration of Sub-test#2.

Figure 7.2.1.1.4.2-1: RSRP variation for TA validation for CG-SDT Sub-test#1

Figure 7.2.1.1.4.2-2: RSRP variation for TA validation for CG-SDT Sub-test#2

The derivation of supplementary timeline events is given in Table 7.2.1.1.4.2-1.

Table 7.2.1.1.4.2-1: Derivation of Supplementary Timeline Events

| Time Period | Unit | Test configuration | Value | Derivation |
| --- | --- | --- | --- | --- |
| W1 | ms | 1 | 480 | Test configuration |
| W2 | ms | 1 | 480 | Test configuration |
| W3 | ms | 1 | 1060 | Test configuration |
| Subtest#1 |  |  |  |  |
| TAtoTB | ms | 1 | [800] | TAtoTB = T1 |
| TBtoTC | ms | 1 | 480 | TBtoTC = W1 |
| TCtoTD | ms | 1 | 480 | TCtoTD = W1 |
| TDtoTE | ms | 1 | [3040] | TDtoTE = T3 |
| TEtoTF | ms | 1 | 480 | TEtoTF = W2 |
| TFtoTG | ms | 1 | 1060 | TFtoTG = W3 |
| IP PDU Delay (T_delay_modeB) | ms | 1 | [4000] | Test configuration for step 5 & 16 |
| Subtest#2 (to be used only if Subtest#1 is passed) |  |  |  |  |
| TAtoTB | ms | 1 | [800] | TAtoTB = T5 – TBtoTC – TCtoTD = T5 – 2*W1 |
| TBtoTC | ms | 1 | 480 | TBtoTC = W1 |
| TCtoTD | ms | 1 | 480 | TCtoTD = W1 |
| TDtoTE | ms | 1 | [3040] | TDtoTE = T6 – TEtoTF – TFtoTG = T6 – W2 - W3 |
| TEtoTF | ms | 1 | 480 | TEtoTF = W2 |
| TFtoTG | ms | 1 | 1060 | TFtoTG = W3 |
| IP PDU Delay (T_delay_modeB) | ms | 1 | [4000] | Test configuration for step 5 & 16 |
| Note 1: These derivations are useful to understand different timeline events other than T1, T2, T3, T4, T5 and T6. Calculations are informative. |  |  |  |  |

1. Ensure the UE is fully synchronized to PCell (Cell 1), be registered to the cell and is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5 (Table 4.5.1-1).

2. Set the parameters according to Table 7.2.1.1.5-1.

3. The test starts at time-point TA before which SS sets the transmit power to P0. Sub-test#1 starts (see Figure 7.2.1.1.4.2-1).

4. At time-point TB, the SS changes the power level from P0 to P1.

5. Between time-points TB and TC, SS sends CLOSE UE TEST LOOP message, containing UE test loop mode B indication and the configuration for T_delay_modeB timer, to the UE.

6. UE closes the test loop mode B and sends CLOSE UE TEST LOOP COMPLETE message to the SS.

7. At time point TC, the SS sends an SDAP PDU and RRCRelease message with CG-SDT configuration to the UE. The UE buffers the received SDAP SDU (IP PDU), starts T_delay_modeB timer and enters RRC_INACTIVE state.

8. At time-point TD, the SS changes the power level from P1 to P0.

9. At time-point TE, the SS changes the power level from P0 to P2.

10. At time-point TF, the T_delay_modeB timer expires which trigger UL data arrival at UE lower layer.

11. After time point TF, SS observes whether UE transmits with CG-SDT no later than TG

12. Check 1: If UE transmits an RRCResumeRequest and the looped back SDAP SDU in a CG PUSCH occasion occuring anywhere from time-points TF to TG, declare Sub-test#1  as passed.

13. If Sub-test#1 is passed in Step 12, go to Step 14 else declare the test iteration verdict as ‘FAIL’ and go to Step 26. Sub-test#1 ends.

14. After time point TG, SS initiates the generic procedure given in Table 4.5.4.2-3 (Steps 1-8) of TS 38.508-1 [14] and brings the UE from RRC INACTIVE to RRC CONNECTED state.

15. SS sends OPEN UE TEST LOOP message to the UE.

16. UE sends OPEN UE TEST LOOP COMPLETE message to the SS.

17. Sub-test#2 starts; SS sets the transmit power to P2 (see Figure 7.2.1.1.4.2-2) and this time is considered as time-point TA for Sub-test#2. From time-point TA to time-point TD, power level is fixed at P2.

18. Between the time-points TB and TC of Sub-test#2, SS sends CLOSE UE TEST LOOP message, containing UE test loop mode B indication and the configuration for T_delay_modeB timer, to the UE.

19. UE closes the test loop mode B and sends CLOSE UE TEST LOOP COMPLETE message to the SS.

20. At time-point TC, the SS sends an SDAP PDU and RRCRelease message with CG-SDT configuration to UE. The UE buffers the received SDAP SDU (IP PDU), starts T_delay_modeB timer and enters in RRC_INACTIVE state.

21. At time-point TD, the SS changes the power level from P2 to P0.

22. At time-point TF, the T_delay_modeB timer expires which trigger UL data arrival at UE lower layer.

23. After time point TF, SS observes whether UE transmits with CG-SDT no later than TG.

24. Check 2: If UE transmit an RRCResumeRequest and the looped back SDAP SDU in a CG PUSCH occasion occuring anywhere from time-points TF to TG, Sub-test#2 is failed else Sub-test#2 is passed. Sub-test#2 ends..

25. If Sub-test#2 is passed, declare the test iteration verdict as ‘PASS’ else ‘FAIL’.

26. After time point TG, SS initiates the generic procedure given in Table 4.5.4.2-3 (Steps 1-8) of TS 38.508-1 [14] and brings the UE from RRC INACTIVE to RRC CONNECTED state.

27. SS sends OPEN UE TEST LOOP message to the UE.

28. UE opens the test loop and sends OPEN UE TEST LOOP COMPLETE message to the SS.

29. Repeat steps 1-28 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.2.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clauses 4.6.1 and 7.3 with the following exceptions:

Table 7.2.1.1.4.3-1: SIB1 (All steps, clause 7.2.1.1.4.2)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sdt-ConfigCommon-r17 SEQUENCE { |  |  |  |
| sdt-RSRP-Threshold-r17 | Not present |  |  |
| sdt-LogicalChannelSR-DelayTimer-r17 | sf512 |  |  |
| sdt-DataVolumeThreshold-r17 | byte100 |  |  |
| t319a-r17 | ms4000 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.2.1.1.4.3-2: CLOSE UE TEST LOOP (Step 5 and 16, clause 7.2.1.1.4.2)

| Derivation path: 38.508-1 [4] Table 4.7A-3 condition UE test loop mode B |  |  |
| --- | --- | --- |
| Information Element | Value/remark | Comment |
| UE test loop mode B_Enhanced LB setup |  |  |
| IP PDU delay | 0 0 0 0 0 1 0 0 | T_delay_modeB duration in seconds (4 seconds) |

Table 7.2.1.1.4.3-3: RRCRelease (Steps 5 and 16, clause 7.2.1.1.4.2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-16 with condition NR_RRC_INACTIVE and SDT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| suspendConfig SEQUENCE { |  |  |  |
| sdt-Config-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sdt-MAC-PHY-CG-Config-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cg-SDT-ConfigInitialBWP-NUL-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| pusch-Config-r17 CHOICE { |  |  |  |
| setup | PUSCH-Config | Derivation Path: TS 38.508-1 [4], Table 4.6.3-118 |  |
| } |  |  |  |
| configuredGrantConfigToAddModList-r17 SEQUENCE (SIZE (1..maxNrofConfiguredGrantConfig-r16)) OF ConfiguredGrantConfig { |  |  |  |
| ConfiguredGrantConfig | ConfiguredGrantConfig | Table 7.2.1.1.4.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cg-SDT-ConfigInitialBWP-DL-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| pdcch-Config-r17 CHOICE { |  |  |  |
| setup | PDCCH-Config | Derivation Path: TS 38.508-1 [4], Table 4.6.3-95 |  |
| } |  |  |  |
| pdsch-Config-r17 CHOICE { |  |  |  |
| setup | PDSCH-Config | Derivation Path: TS 38.508-1 [4], Table 4.6.3-100 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cg-SDT-TimeAlignmentTimer-r17 | infinity | Time Alignment Timer value |  |
| cg-SDT-RSRP-ThresholdSSB-r17 | 46 | (IE value – 156) dBm = - 110 dBm |  |
| cg-SDT-TA-ValiditationConfig-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cg-SDT-RSRP-ChangeThreshold-r17 | [dB8] | [8] dB |  |
| } |  |  |  |
| } |  |  |  |
| cg-SDT-CS-RNTI-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.2.1.1.4.3-4: ConfiguredGrantConfig (Table 7.2.1.1.4.3-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-26, condition CG_Config_Type1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ConfiguredGrantConfig ::= SEQUENCE { |  |  |  |
| periodicity | Sym2560x14 | 320ms | SCS120 |
| rrc-ConfiguredUplinkGrant SEQUENCE{ |  |  |  |
| frequencyDomainAllocation | BIT STRING (SIZE(18) | Equals to NBWPsize * (NBWPsize – LRB + 1) + (NBWPsize -1 - RBstart), where LRB = 24 PRB, RBstart = 0, NBWPsize is the size [PRBs] of the carrier bandwidth and contained in TS.38.508-1 [4] clause 6.2.3.1 | SCS15 AND CBW5, SCS30 AND CBW10 |
|  | BIT STRING (SIZE(18) | Equal to NBWPsize * (LRB-1) + RBstart), where LRB = 24 PRB, RBstart = 0, NBWPsize is the size [PRBs] of the carrier bandwidth and contained in TS.38.508-1 [4] clause 6.2.3.1 |  |
| dmrs-SeqInitialization | 0 |  |  |
| mcsAndTBS | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| Note : Configured UL grant for CG-SDT is 888 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen to enable UE to transmit RRCResumeRequest and Loop backed RLC PDU in a MAC PDU ( 97 bytes for Loop backed RLC PDU + 2 bytes for MAC Sub Header + 6 bytes for RRCResumeRequest message + 2 bytes for MAC Sub Header + 4 bytes for BSR and Padding ) |  |  |  |

Table 7.2.1.1.4.3-5: RRCResumeRequest (Steps 10 and 20, clause 7.2.1.1.4.2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCResumeRequest ::= SEQUENCE { |  |  |  |
| rrcResumeRequest SEQUENCE { |  |  |  |
| resumeCause | mo-data |  |  |
| } |  |  |  |
| } |  |  |  |

7.2.1.1.5	Test Requirements

Tables 7.2.1.1.4.1-3 and 7.2.1.1.5-1 define the general parameters and cell specific parameters for NR SA FR2 CG-SDT TA validation test.

Table 7.2.1.1.5-1: Cell specific test parameters TA validation for CG-SDT in FR2

| Parameter |  | Unit | Test 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 | T6 |
| AoA setup |  |  | Setup 1 defined in A.3.15 |  |  |  |  |  |
| Assumption for UE beams Note 4 |  |  | Rough |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |  |
|  | Config 1 | dBm/15kHz | -100 + TT |  |  |  |  |  |
|  | Config 1 | dBm/SCS | -100 + TT |  |  |  |  |  |
|  | Config 1 | dB | [0] + TT | [13] + TT | [0] + TT | [24.5] + TT | [24.5] + TT | [0] + TT |
|  | Config 1 | dB | [0] + TT | [13] + TT | [0] + TT | [24.5] + TT | [24.5] + TT | [0] + TT |
| SS-RSRP | Config 1 | dBm/SCS | [-100] + TT | [-87] + TT | [-100] + TT | [-75.5] + TT | [-75.5] + TT | [-100] + TT |
| Io | Config 1 | dBm/95.04 MHz | [-68] + TT | [-57.8] + TT | [-68] + TT | [-46.50] + TT | [-46.50] + TT | [-68] + TT |
| Propagation condition |  |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |  |

The UE behaviour in each test during time durations shall be as follows:

During Sub-test#1, UE shall transmit UL data with CG-SDT within 1060ms after time point TF.

During Sub-test#2, after passing Sub-test#1, UE shall not transmit UL data with CG-SDT.

The rate of correct events observed during repeated tests shall be at least 90%.

-------- End of changes --------