3GPP TSG-RAN5 Meeting #101	R5-236108r1

# Chicago, USA 13th – 17th November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2929 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update IE RACH-ConfigDedicated |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_2step_RACH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To align the IE RACH-ConfigDedicated with the core specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The local IE has been expanded. Missing ASN.1 field value has been assigned. Editorial updates. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will not be aligned with the core specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision R5-236108r1. The coversheet is updated. ASN.1 fields added. |  |  |  |  |  |  |  |  |

<Start of first modified section>

#### –	RACH-ConfigDedicated

Table 4.6.3-129: RACH-ConfigDedicated

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigDedicated ::= SEQUENCE { |  |  |  |
| cfra SEQUENCE { |  |  |  |
| occasions SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric |  |  |
| ssb-perRACH-Occasion | one |  |  |
| } |  |  |  |
| resources CHOICE { |  |  |  |
| ssb SEQUENCE { |  |  |  |
| ssb-ResourceList SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF CFRA-SSB-Resource { | 1 entry |  |  |
| CFRA-SSB-Resource[1] SEQUENCE { |  | entry 1 |  |
| ssb | SSB-Index |  |  |
| ra-PreambleIndex | 8 |  |  |
| } |  |  |  |
| } |  |  |  |
| ra-ssb-OccasionMaskIndex | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ra-Prioritization | Not present |  |  |
| ra-PrioritizationTwoStep-r16 | Not present |  |  |
| cfra-TwoStep-r16 | Not present |  |  |
| cfra-TwoStep-r16 SEQUENCE { |  |  | 2-step RA |
| occasionsTwoStepRA-r16 SEQUENCE { |  |  |  |
| rach-ConfigGenericTwoStepRA-r16 | RACH-ConfigGenericTwoStepRA |  |  |
| ssb-PerRACH-OccasionTwoStepRA-r16 | oneEighth |  |  |
| } |  |  |  |
| msgA-CFRA-PUSCH-r16 SEQUENCE { |  |  |  |
| msgA-MCS-r16 | 1 |  |  |
| nrofSlotsMsgA-PUSCH-r16 | 1 |  |  |
| nrofMsgA-PO-PerSlot-r16 | one |  |  |
| msgA-PUSCH-TimeDomainOffset-r16 | 4 |  |  |
| msgA-PUSCH-TimeDomainAllocation-r16 | Not present |  |  |
| startSymbolAndLengthMsgA-PO-r16 | 32 |  |  |
| mappingTypeMsgA-PUSCH-r16 | typeA |  |  |
| guardPeriodMsgA-PUSCH-r16 | 0 |  |  |
| guardBandMsgA-PUSCH-r16 | 0 |  |  |
| frequencyStartMsgA-PUSCH-r16 | 1 |  |  |
| nrofPRBs-PerMsgA-PO-r16 | 1 |  |  |
| nrofMsgA-PO-FDM-r16 | one |  |  |
| msgA-IntraSlotFrequencyHopping-r16 | Not present |  |  |
| msgA-HoppingBits-r16 | Not present |  |  |
| msgA-DMRS-Config-r16 SEQUENCE { |  |  |  |
| msgA-DMRS-AdditionalPosition-r16 | pos0 |  |  |
| msgA-MaxLength-r16 | len2 |  |  |
| msgA-PUSCH-DMRS-CDM-Group-r16 | 0 |  |  |
| msgA-PUSCH-NrofPorts-r16 | 0 |  |  |
| msgA-ScramblingID0-r16 | Not present |  |  |
| msgA-ScramblingID1-r16 | Not present |  |  |
| } |  |  |  |
| nrofDMRS-Sequences-r16 | 1 |  |  |
| msgA-Alpha-r16 | alpha0 |  |  |
| interlaceIndexFirstPO-MsgA-PUSCH-r16 | 1 |  |  |
| nrofInterlacesPerMsgA-PO-r16 | 1 |  |  |
| } |  |  |  |
| msgA-TransMax-r16 | n10 |  |  |
| resourcesTwoStep-r16 SEQUENCE { |  |  |  |
| ssb-ResourceList SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF CFRA-SSB-Resource { | 1 entry |  |  |
| CFRA-SSB-Resource SEQUENCE { |  | entry 1 |  |
| ssb[1] | 0 |  |  |
| ra-PreambleIndex[1] | 52 | Randomly selected |  |
| } |  |  |  |
| } |  |  |  |
| ra-ssb-OccasionMaskIndex | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2-step RA | 2-step RA type Random Access |

<End of modified section>