3GPP TSG-RAN5 Meeting #101	R5-236219r1

# Chicago, USA, 13th – 17th November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2682 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Completion of NR SA FR1 intra-frequency CGI identification of NR neighbour cell for 2 Rx UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test case 16.6.7.2 is incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are made: - Removed some of the editor’s note, - updated TT values in Table 16.6.7.2.5-1, F.1.1.2-9 and F.1.3.2-7. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case 16.6.7.2 is complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.6.7.2, Annex F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.508-1 CR 2933 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated TR 38.903 TT analysis Tdoc number: R5-236221 Depending on RAN4 CR R4-2320859 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: - Updated the RAN4 TDoc details. - Resolved 3GU issue in the title of the cover sheet. - Removed Annex F changes to address the overlap with R5-236738 |  |  |  |  |  |  |  |  |

-------- Start of Change --------

#### 16.6.7.2	NR SA FR1 intra-frequency CGI identification of NR neighbour cell in FR1 for 2 Rx UE

16.6.7.2.1	Test Purpose

The purpose of this test is to verify that the UE makes correct reporting of intra-frequency CGI identification of an NR neighbour cell in FR1 with autonomous gaps. This test shall partly verify the measurement requirements in clause 9.11A of TS 38.133 [6].

16.6.7.2.2	Test Applicability

This test applies to all types of NR RedCap UE with 2Rx from release 17 onwards.

16.6.7.2.3	Minimum Conformance Requirements

The minimum conformance requirements are specified in clause 16.6.7.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.7.2.

16.6.7.2.4	Test Description

16.6.7.2.4.1	Initial conditions

This test can be run in the configuration defined in Table 16.6.7.2.4.1-1.

Table 16.6.7.2.4.1-1: Supported test configurations

| Test Case ID | Test Config Index | Description |
| --- | --- | --- |
| 16.6.7.2-1 | 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.7.2-2 | 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.7.2-3 | 3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.7.2-4 | 4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Configure the test equipment and the DUT according to the parameters in Table 16.6.7.2.4.1-2.

Table 16.6.7.2.4.1-2: Test Environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration from Table 16.6.7.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 16.6.7.2.4.1-3.

2.	Message contents are defined in clause 16.6.7.2.4.3.

3.	There are two cells: Cell 1 and Cell 2 specified in the test. Cell 1 is the FR1 PCell and Cell 2 is an FR1 neighbour cell on the same frequency as the PCell. The Cell 1 and Cell 2 are configured according to the settings in Annex C.1.1 and C.1.2.

Table 16.6.7.2.4.1-3: General test parameters for SA intra-frequency CGI identification of NR neighbour cell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3, 4 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3, 4 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3, 4 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1, 4 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.1 RedCap FR1 |  |
| SMTC configuration |  | 1, 4 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A3-Offset | dB | 1, 2, 3, 4 | -4.5 |  |
| CP length |  | 1, 2, 3, 4 | Normal |  |
| Hysteresis | dB | 1, 2, 3, 4 | 0 |  |
| Time To Trigger | s | 1, 2, 3, 4 | 0 |  |
| Filter coefficient |  | 1, 2, 3, 4 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3, 4 |  | OFF |
| Time offset between serving and neighbour cells |  | 1, 4 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 ms | Synchronous cells |
|  |  | 3 | 3 ms | Synchronous cells |
| T1 | s | 1, 2, 3, 4 | 5 |  |
| T2 | s | 1, 2, 3, 4 | 5 |  |
| T3 | s | 1, 2, 3, 4 | 5 |  |

16.6.7.2.4.2	Test procedure

Same as in clause 16.6.7.1.4.2

16.6.7.2.4.3	Message contents

Same as in clause 16.6.7.1.4.3

16.6.7.2.5	Test Requirements

Tables 16.6.7.2.4.1-3 and 16.6.7.2.5-1 define the general parameters and cell specific parameters for this test.

Table 16.6.7.2.5-1: NR Cell specific test parameters for SA intra-frequency CGI identification of NR neighbour cell in FR1

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 4 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1, 4 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1, 4 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1, 4 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3, 4 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1, 4 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3, 4 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3, 4 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3, 4 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3, 4 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1, 4 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1, 4 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1, 4 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1, 4 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1, 4 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 + TT | -Infinity | -91 + TT |
| Io | dBm/9.36 MHz | 1, 4 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/18.36 MHz | 3 | -61.67 | -59.32 | -61.67 | -59.32 |
| Propagation Condition |  | 1, 2, 3, 4 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall send a measurement report containing the CGI of cell 2 within 252 ms from the start of time period T3.

Test requirement = RRC Procedure delay + Tidentify_CGI + reporting delay = 10 + 240 + 2ms from the start of T3 = 252 ms

The UE shall be scheduled continuously throughout the test. From the start of T3 until 252 ms, the interruption on PCell shall not be more than the values specified for SA in TS 38.133 [6] clause 8.2.2.2.14.

The rate of correct events observed during repeated tests shall be at least 90%.

-------- End of Change --------