3GPP TSG-RAN WG5 Meeting #101                                                       R5-236239r1

# Chicago, USA, 13st – 17 th Nov 2023

Source:	MediaTek Inc., China Mobile, China Telecom, China Unicom, AT&T, FirstNet

Title:		Discussion paper for selecting optional Rel-16 features and test case for Redcap Only UE

Agenda item:	6.3.9.8

# Document for: Discussion and Decision

Agenda Item:

## Introduction

## At RAN5#100 [1] “Way forward for Rel-16 and Rel-17 legacy test cases applicability to RedCap UE” was agreed that introduces the way forward on how to handle Rel-16 and Rel-17 protocol (TS 38.523-1) legacy test cases applicability to Redcap Only UEs.

## Discussion

Mandatory Rel-16 legacy features that are optional for RedCap only UEs:

- SON (Self-Organising Networks) and MDT (Minimization of Drive Tests) support for NR/Automatic  Neighbour relations part:

  - TS 38.306/ nr-CGI-Reporting-NPN-r16

  - TS 38.306/ reportAddNeighMeasForPeriodic-r16

Mandatory Rel-16 legacy test cases:

- 8.1.1.2.4

- SON/MDT test cases

Below is a list of optional Rel-16 legacy features with protocol test cases. Features excluded from the below list:

- features with no test cases

- features not applicable to RedCap:DC,CA,ENDC and multi-layer MIMO

- features that not specifically adapted to RedCap: 5G V2X with NR sidelink

- IMS related features as IMS test case should be handled separately

| Name | Acronym |
| --- | --- |
| UE Conformance Test Aspects -UE Power Saving in NR | NR_UE_pow_sav-UEConTest |
| UE Conformance Test Aspects - Physical Layer Enhancements for NR URLLC | NR_L1enh_URLLC-UEConTest |
| UE Conformance Test Aspects – Support of NR Industrial Internet of Things (IoT) | NR_IioT-UEConTest |
| UE Conformance Test Aspects - NR-based access to unlicensed spectrum | NR_unlic-UEConTest |
| UE Conformance Test Aspects - 5G V2X with NR sidelink | 5G_V2X_NRSL_eV2XARC-UEConTest |
| UE Conformance Test Aspects - NR positioning support | NR_pos-UEConTest |
| UE Conformance Test Aspects - Optimisations on UE radio capability signalling - NR/E-UTRA Aspects | RACS-UEConTest |
| UE Conformance Test Aspects - Enhancement of Network Slicing | eNS-UEConTest |
| UE Conformance Test Aspects - Access Traffic Steering, Switch and Splitting support in 5G system | ATSSS-UEConTest |
| UE Conformance Test Aspects - Private Network Support for NG-RAN | NG_RAN_PRN_Vertical_LAN-UEConTest |
| UE Conformance Test Aspects - Enhancements on MIMO for NR | NR_eMIMO-UEConTest |
| UE Conformance Test Aspects - NR Mobility Enhancements | NR_Mob_enh-UEConTest |
| UE Conformance Test Aspects - 2-step RACH for NR | NR_2step_RACH-UEConTest |
| UE Conformance Test Aspects - SON (Self-Organising Networks) and MDT (Minimization of Drive Tests) support for NR | NR_SON_MDT-UEConTest |
| UE Conformance Test Aspects - B1C Signal in BDS Positioning System Support for LTE and NR | B1C_BDS_pos_UEConTest |
| UE Conformance Test Aspects - Single Radio Voice Call Continuity from 5G to 3G | SRVCC_NR_to_UMTS-UEConTest |

## 3.	Proposal

Proposal 1:	Keep test case 8.1.1.2.4 as not applicable to RedCap UEs.

Proposal 2:	Update MDT/SON test case applicability for RedCap [2],[3]

Proposal 3:	Track test cases applicable to RedCap in TS 38.523-2

Proposal 4:	Request wider industry input regarding which of the optional Rel-16 features will be deployed.

Proposal 5: The applicability/prose and TTCN for the protocol test cases belonging to the below optional Rel-16 features should be updated to support RedCap only UEs:

| Name | Acronym |
| --- | --- |
| UE Conformance Test Aspects -UE Power Saving in NR | NR_UE_pow_sav-UEConTest |
| UE Conformance Test Aspects - NR Mobility Enhancements | NR_Mob_enh-UEConTest |
| UE Conformance Test Aspects - 2-step RACH for NR | NR_2step_RACH-UEConTest |
| UE Conformance Test Aspects - Physical Layer Enhancements for NR URLLC | NR_L1enh_URLLC-UEConTest |
| UE Conformance Test Aspects – Support of NR Industrial Internet of Things (IoT) | NR_IioT-UEConTest |
| UE Conformance Test Aspects - NR positioning support | NR_pos-UEConTest |
| UE Conformance Test Aspects - Enhancement of Network Slicing | eNS-UEConTest |
| UE Conformance Test Aspects - Private Network Support for NG-RAN | NG_RAN_PRN_Vertical_LAN-UEConTest |
| UE Conformance Test Aspects - SON (Self-Organising Networks) and MDT (Minimization of Drive Tests) support for NR | NR_SON_MDT-UEConTest |

## 4.	References

[1]	R5-235348: Way forward for Rel-16 and Rel-17 legacy test cases applicability to RedCap UE

[2]	R5-237088: Adding SON/MDT PICS for RedCap

[3]	R5-237089: Updating SON/MDT test case applicability for RedCap

[4]	R5-237026: Update 2-step RACH TC 7.1.1.1.8 for HD-FDD UE-PRACH

[5]	R5-237027: Update 2-step RACH TC 7.1.1.1.9 for HD-FDD UE-PRACH

[6]	R5-237028: Update URLLC TC 7.1.1.4.1.5 to test RedCap UE

[7]	R5-237029: Update URLLC TC 7.1.1.4.2.6 to test RedCap UE